3GPP TSG RAN WG1 #112bis-e		R1-2302518

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on Rel-18 Multi-carrier enhancements, eDSS, NCR and BWP without restriction

Agenda Item:	9.18

Document for:	Discussion and Decision

- Introduction

In this contribution, we discussed some remaining issues on the following aspects:

- Rel-18 Multi-carrier enhancements

- eDSS

- NCR

- BWP without restriction

- Discussion on Multi-carrier enhancements

Multi-cell scheduling

Type1A field

Agreement - Priority indicator in DCI format 0_X belongs to Type-1A field. - The indicated priority is applied to all the co-scheduled PUSCH(s) - Priority indicator in DCI format 1_X belongs to Type-1A field. - The indicated priority indicator is applied to the PUCCH. - RRC parameters is introduced to configure the presence of priority indicator in DCI format 0_X/1_X - This parameter is per set of cells

priorityIndicatorDCI-1-1-r16             ENUMERATED {enabled}                                               OPTIONAL,   -- Need S

A remaining issue for the priority indicator in mc-DCI format is the interpretation of this field. In R16, the presence of a priority indicator in a UL-DCI or a DL-DCI is per BWP configured. Therefore, it could be the case that the active BWPs of some cells in a cell combination do not have a priority indicator, while some other cells are configured with a priority indicator. In this case, how to interpret the priority indicator should be clarified.

For multi-cell PDSCH scheduling, as the priority indicator is applied to the PUCCH, there is no ambiguity for the PUCCH. For the scheduled PDSCHs, all the PDSCHs should follow the same priority index determined by the priority indicator, otherwise, additional work for either multiplexing HARQ-ACK with different priority indexes or dropping HARQ-ACK with different priority indexes is needed.

For multi-cell PUSCH scheduling, one interpretation is that each cell in the cell set must support both eMBB and URLLC services and have the corresponding priority indicator configuration, e.g., priorityIndicatorDCI-0-1-r16, if the mc-DCI format includes a priority indicator. The second interpretation is that the priority indicator is valid only for cells that have priorityIndicatorDCI-0-1-r16. For cells without priorityIndicatorDCI-0-1-r16, this field is ignored and a priority index of 0 is applied. This interpretation allows a mc-DCI format to schedule PUSCHs with different priorities. The third interpretation is that this priority indicator field in a mc-DCI format only takes effect when all co-scheduled cells have the corresponding per-BWP priority indicator configuration. When at least one co-scheduled cell in the scheduled cell combination does not support the priority bit, the value of the priority indicator field in the mc-DCI format is expected to be 0, and only eMBB PUSCHs are scheduled by the mc-DCI format. The third interpretation has greater flexibility than interpretation1 by offering eMBB service to some cell combinations while providing both eMBB and URLLC services to some other combinations. Moreover, this interpretation is consistent with that of the DL mc-DCI as it does not support the mc-DCI format to schedule PUSCHs with mixed priorities.

Proposal 1. All PDSCHs scheduled by a DCI format 1_X have the same priority index determined by the priority indicator in DCI format 1_X, and all PUSCHs scheduled by a DCI format 0_X have the same priority index determined by the priority indicator in DCI format 0_X.

Proposal 2. For a cell combination scheduled by a DCI format 0_X/1_X, if all co-scheduled cells in the cell combination are configured with priorityIndicatorDCI-0-1-r16 or priorityIndicatorDCI-1-1-r16 for the corresponding active BWPs, the value of priority indicator can be 1 or 0.

Proposal 3. For a cell combination scheduled by a DCI format 0_X/1_X, if there is at least one co-scheduled cell without priorityIndicatorDCI-0-1 or priorityIndicatorDCI-1-1-r16 configured for its active BWP, the value of priority indicator is expected to be 0, and the priority index 0 is applied to all scheduled PUSCHs/PDSCHs.

- TPs

===212 TP1===

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-0-X is not configured for the set of cell; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

===TP end===

===212 TP2===

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-1-X is not configured for the set of cell; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

===TP end===

===213 TP3===

If in an active DL BWP a UE monitors PDCCH for detection of DCI format that includes a priority indicator field, a priority index can be provided by the priority indicator field. If there is a cell that is not configured with priorityIndicatorDCI-0-1 or priorityIndicatorDCI-1-1 in the cell combination scheduled by a DCI format 0_X or DCI format 1_X that includes a priority indicator field, UE expects that priority index 0 is indicated by the priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format that includes a priority indicator field, the DCI format can schedule PUSCH transmissions of any priority, or PDSCH receptions and/or trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

===TP end===

Type1C field

Agreement CSI request in DCI format 0_X belongs to Type-1C field. - This field is applied to the cell with smallest serving cell index among the co-scheduled cells. Agreement UL-SCH indicator in DCI format 0_X belongs to Type-1C field. - This field is applied to the cell with smallest serving cell index among the co-scheduled cells.

CSI request and UL-SCH are agreed as Type-1C, but how to determine the field size of Type-1C field is not defined yet.

In DCI format 0-1, CSI request field size is determined based on reportTriggerSize configured for a serving cell, and the bit size of UL-SCH indicator in a DCI format 0-1 scheduling a single PUSCH is fixed as 1. It is natural to reuse the design of CSI request and UL-SCH in DCI format 0-1 as the baseline.

38.212 - CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize. - UL-SCH indicator – 0 or 1 bit as follows - 0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; - 1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s). - SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214]. Table 7.3.1.1.2-24: SRS request Value of SRS request field; Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 1_1, 1_2, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB'; Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'. 00; No aperiodic SRS resource set triggered; No aperiodic SRS resource set triggered. 38.214 A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

If the cell with the smallest serving cell index among the co-scheduled cells in a cell combination is not configured with reportTriggerSize, the CSI request field for the cell combination requires 0 bits. To avoid varying field size, the CSI request field size can be determined in a similar way as Type-1A, i.e., the size is determined as the maximum field size of active BWP among all cell combinations within the set of cells. If the field size of the CSI request for active BWP of a scheduled cell combination is smaller than the determined field size in the DCI format 0_X, LSB of the field is applied.

Proposal 4. For a set of cells, the size of the CSI request in the DCI format 0_X is determined as the maximum field size of active BWP among all cell combinations within the set of cells.
- For a scheduled cell combination, the field size of the CSI request is determined by the higher layer parameter reportTriggerSize for the cell with the smallest serving cell index among the co-scheduled cells. 
- If the field size of the CSI request for a scheduled cell combination is smaller than the determined field size in the DCI format 0_X, LSB of the field is applied.

Proposal 5. For a set of cells, the size of the UL-SCH indicator in DCI format 0_X is 1 bit.

Secondly, the purpose of the blue-highlighted text above is to avoid the error cases of wasting PUSCH grants. Noted that triggering SRS only vai DCI is an optional feature, i.e., a per-band capability srs-TriggeringDCI-r17 is introduced to indicate whether UE supports SRS only in DCI.

| Features | Index | Feature group | Components | Field name in TS 38.331 | Parent IE in TS 38.331 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | srs-TriggeringDCI-r17 | MIMO-ParametersPerBand | Optional with capability signalling |

-- R1 23-8-2    Triggering SRS only in DCI 0_1/0_2

srs-TriggeringDCI-r17                       ENUMERATED {supported}                                         OPTIONAL,

When it comes to DCI format 0_X, it should be first clarified whether UE support triggering SRS only in DCI format 0_X for a scheduled cell. As we discussed in our companion[2], the legacy UE feature can be reused and extended for DCI format 0_X/1_X. Thus, if a R18 mc-scheduling UE reports srs-TriggeringDCI-r17 for the smallest serving cell index among the co-scheduled cells, it means the UE supports triggering SRS only in DCI format 0_X for the co-scheduled cell. While if a UE only reports the support of the R18 mc-scheduling basic feature but does not support srs-TriggeringDCI-r17, the UE does not support mc-DCI format to schedule SRS-only transmission and it is not expected to receive a DCI format 0_X with UL-SCH indicator of "0" and CSI request of all zero(s).

It is also worth noting that the SRS request is agreed as Type-1B, the size of which is dependent on the number of rows in the configured table which contains combinations of indications for each cell in the set configured for the DCI format 0_X. According to the table, SRS request of all zeros in a mc-DCI format may point to a row of joint SRS resource set combinations for a cell combination instead of the same meaning, i.e., "No aperiodic SRS resource set triggered", as DCI format 0_1/1_1. Thus, if a UE supports SRS-only transmission for a co-scheduled cell, the UE is not expected to receive a DCI format 0_X with UL-SCH indicator of "0", CSI request of all zero(s), and SRS request indicating “No aperiodic SRS resource set triggered” for the co-scheduled cell.

Proposal 6. For a UE not supporting triggering SRS only in DCI for the smallest serving cell index among the co-scheduled cells, UE is not expected to receive a DCI format 0_X with UL-SCH indicator of "0" and CSI request of all zero(s).

Proposal 7. For a UE not supporting triggering SRS only in DCI for the smallest serving cell index among the co-scheduled cells, the UE is not expected to receive a DCI format 0_ X with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request indicating a codepoint associated with “No aperiodic SRS resource set triggered” for the serving cell.

- TPs

====212 TP4====

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by the maximum field size of CSI request required for the cells with the smallest cell index among all cell combinations within the set of cells

-	CSI request is applied to the cell with the smallest cell index in the scheduled cell combination.

-	X LSBs of CSI request provide the CSI request for a cell with the smallest cell index in a cell combination, X is determined by higher layer parameter reportTriggerSize configured for the cell with the smallest cell index.

===TP end===

====212 TP5====

-	UL-SCH indicator –1 bit as follows

-	1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, the UE is not expected to receive a DCI format 0_X with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, the UE is not expected to receive a DCI format 0_X with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request indicating a codepoint associated with “No aperiodic SRS resource set triggered” for the serving cell with the smallest cell index in the scheduled cell combination.

===TP end===

Type2 Field

Proposal 3-15rev3: - For a set of cells configured for multi-cell scheduling using DCI format 0_X/1_X, - the size of a Type-1A field in the DCI format 0_X/1_X is determined as maximum field size of active BWP among all cells within the set of cells. - the size of a Type-1B field in the DCI format 0_X/1_X is equal to ceiling(log2(N)), where N is the number of rows in RRC-configured table with each row containing multiple indexes for all cells within the set of cells. - The Type-1B field indicates one row of the configured table - The Type-1B index for a cell points to a corresponding index in a RRC configured table applicable for DCI format 0_1/1_1 or MAC CE activated values. - the size of a per cell Type-2 field in the DCI format 0_X/1_X is determined based on active BWP for each cell. Agreement For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following: - If table defining combinations of co-scheduled cells for the set of cells is configured, - an indicator in the DCI is included and points to one row of the table. - The table is configured by RRC signaling for the set of cells. - Separate tables are configured for downlink scheduling and uplink scheduling - The size of the indicator is equal to ceil(log2(N)), where N is the number of rows in the table. - The max number of rows in the table is 16 - The size of the per-cell Type 2 fields for each co-scheduled cell does not change according to the indicated co-scheduled cell combination - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 1_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 0_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - Otherwise, - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells. - For Type 0 FDRA, all 0s indicates the cell is not scheduled. - For Type 1 FDRA, all 1s indicates the cell is not scheduled. - The size of the Type 2 fields for each cell does not change according to actually co-scheduled cells. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells. - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

For the ‘if’ branch, it is agreed that the size of the per-cell Type-2 fields for each co-scheduled cell does not change according to the indicated co-scheduled cell combination. However, how to determine the total size of a Type-2 field for a co-scheduled cell combination should be clarified.

One way is to determine the total size of a Type-2 field as the maximum field size for active BWP among all cell combinations. If the bits required for a cell combination are less than the total field size, zeros are padded to match the total field size. Another way is that the total size of a Type-2 field is depending on the scheduled cell combination, e.g., the total size is determined as the sum of bits required for each cell in a cell combination. If the sum of all fields for a cell combination is less than the determined DCI format size, zeros are padded to match the determined DCI format size.

Proposal 8. Clarify how to determine the total field size of a Type-2 field when the table defining combinations of co-scheduled cells for the set of cells is configured, including whether the total size of a Type-2 field is determined as the field size required for active BWP of the scheduled cell combination or the maximum field size for active BWP among all cell combinations and whether the total size changes according to the scheduled cell combination.

Type3 Field

It should be noted that although ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ are existing fields, they are different from the legacy sc-scheduling in that the types of these fields are configurable between Type-1A and Type-2. Since Type-1A and Type-2 lead to different field structures and interpretations, it is unreasonable to force basic UEs to support both types for a single field. The support of type configurability should be an optional feature. Basic UE may only support these three fields as Type-2. If UE is not informed by NW of the field type for these fields, Type-2 is assumed by default.

Proposal 9. The default type for ‘Antenna port(s)’, ‘Precoding information and number of layers’ or ‘SRS resource indicator’ is Type-2, if the type field is not configured for these fields by NW, Type-2 is assumed for these fields.

PDCCH monitoring

Agreement For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells by RRC signaling. Agreement Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - When multiple sets of cells are configured, - a cell in one set of cells can’t be included in another set of cells. - n_CI value is independently configured for each set of cells. - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells. - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells. - DCI size of DCI format 0_X is independently determined for each set of cells. - DCI size of DCI format 1_X is independently determined for each set of cells. - The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability. - An indicator is included in the DCI to indicate the scheduled set of cells, - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells. - Unique n_CI value is configured for each set of cells. Agreement - Separate search space sets for DCI format 0_X/1_X and legacy DCI formats are independently configured - Separate search space sets for DCI format 0_X and 1_X can be independently configured

The n_CI value for mc-DCI format for a cell set can be determined in the same way as the n_CI value (CIF value) for legacy DCI configured for the reference cell of the cell set. In this case, mc-scheduling is to some extent considered to be part of the scheduling of the reference cell, and the legacy framework and spec can be reused when determining the corresponding CCE index. It was agreed that separate search space sets for DCI format 0_X/1_X and legacy DCI formats are independently configured, however, to avoid confusion and PDCCH blocking between legacy DCI format and mc-DCI format, it is desirable to clarify that separate SS sets should be configured for legacy DCI format and mc-DCI format, respectively.

Proposal 10. From the perspective of the reference cell, n_CI for mc-DCI format is determined in the same way as legacy DCI format, moreover, mc-DCI format and legacy DCI format should be configured in different SS sets if both mc-DCI format and legacy DCI format are configured to schedule the same cell.

Agreement For a set of cells which is configured for multi-cell scheduling using DCI format 0_X/1_X, if DCI size budget on the reference cell can’t be maintained after performing Rel-17 DCI size alignment procedures for legacy DCI formats (after step 4C), UE applies zero padding to whichever of DCI formats 0_X or 1_X that has a smaller size to have equal size.

In R17, DCI format 0_0/0_1/0_2 must have distinct sizes, and DCI format 1_0/1_1/1_2 must also have distinct sizes. Thus, if the sizes of a mc-DCI format and a legacy DCI format monitored on the same PDCCH candidate are the same, 2 bits are needed for 'Identifier for DCI formats' in a mc-DCI format, i.e., 1 bit indicating uplink or downlink grant and 1 bit indicating whether the DCI format is legacy DCI format or mc-DCI format. However, since the legacy DCI format only has 1 bit of 'Identifier for DCI formats', UE cannot correctly identify the bit size of 'Identifier for DCI formats' when a mc-DCI format and a legacy DCI format share the same payload size, and thus still cannot recognize the DCI format. To resolve this issue, one feasible solution is for the NW to ensure that the size of mc-DCI format is always different from any configured legacy DCI format. Another solution is for the NW to ensure non-overlapped MO/PDCCH candidates for a mc-DCI format and any legacy DCI format.

Proposal 11. For a set of cells, the following solutions can be considered:
- Alt1. NW should ensure that the size of mc-DCI format(s) is always different from any legacy single-cell scheduling DCI format(s) (i.e., DCI format 0_0/0_1/0_2/1_0/1_1/1_2) configured for the mc-DCI format.
- Alt2. NW should ensure that mc-DCI format(s) and legacy single-cell scheduling DCI format(s) (i.e., DCI format 0_0/0_1/0_2/1_0/1_1/1_2) are configured with non-overlapped PDCCH monitoring occasion or PDCCH candidates.

- TPs

===212 TP6 for alt1===

The UE is not expected to handle a configuration that, after applying the above steps, results in

-	the total number of different DCI sizes configured to monitor is more than 4 for the cell; or

-	the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 0_0 and 0_2 are mapped to the same resource; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 1_0 and 1_2 are mapped to the same resource; or

-	the size of DCI format 0_1 in a UE-specific search space is equal to DCI format 0_2 in the same or another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 0_1 and 0_2 are mapped to the same resource; or

-	the size of DCI format 1_1 in a UE-specific search space is equal to DCI format 1_2 in the same or another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 1_1 and 1_2 are mapped to the same resource.

-	the size of DCI format 0_0 in a UE-specific search space, or the size of DCI format 0_1 in a UE-specific search space, or the size of DCI format 0_2 in a UE-specific search space is equal to DCI format 0_X in another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 0_0 or 0_1 or 0_2 and DCI format 0_X are mapped to the same resource; or

-	the size of DCI format 1_0 in a UE-specific search space, or the size of DCI format 1_1 in a UE-specific search space, or the size of DCI format 1_2 in a UE-specific search space is equal to DCI format 1_X in another UE-specific search space when at least one pair of the corresponding PDCCH candidates of DCI formats 1_0 or 1_1 or 1_2 and DCI format 1_X are mapped to the same resource.

===TP end===

===212 TP7 for alt2===

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format scheduling a single cell with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, MCS-RNTI, MCCH-RNTI, G-RNTI, or G-CS-RNTI and a DCI format 0_X or a DCI format 1_X with CRC scrambled by a CS-RNTI for scheduling respective transmission or reception on a same serving cell.

===TP end===

Dormancy/deactivation

Another aspect that needs clarification is whether a mc-DCI format can be used to schedule a cell combination including an inactive or dormant cell. Since the cell deactivation procedure/dormancy procedure and mc-scheduling procedure are decoupled, if the mc-DCI format is not permitted to schedule a cell combination with inactive cell/dormant cell, the other cells in the combination may no longer be schedulable together. Worse still, co-scheduled cells without legacy DCI format may also become not schedulable. Hence, mc-DCI format should be allowed to schedule cell combinations that contain inactive or dormant cells.

For DCI format size determination in this case, UE assumes the active BWP of the inactive cell is the first active BWP. UE simply ignores the scheduling information for the deactivated/dormant cell.

Proposal 12. If a co-scheduled cell becomes inactive or dormant, gNB is still allowed to indicate a cell combination including that cell in DCI format 0_X/1_X, UE simply ignores the scheduling information for the inactive/dormant cell.

Proposal 13. If a co-scheduled cell becomes inactive, the active BWP is assumed to be the BWP with firstActiveDownlinkBWP-Id for that cell when determining the mc-DCI format size and field size.

Another issue is the BD/CCE counting for a reference cell when the reference cell is deactivated/dormant. According to current spec, when a Scell is dormant or deactivated, BD/CCE budget for the scheduling of the Scell still holds. If the reference cell is configured with single-cell scheduling (i.e., self-scheduling or CCS ) in addition to mc-scheduling , when it is indicated to switch to a dormant DL BWP or inactive state, there is no need to monitor DCI format 0_X/1_X anymore, and thus mc-scheduling can be considered as ‘disabled’. The BD/CCE counting for scheduling of the reference cell should fall back to single-cell scheduling only.

Proposal 14. If legacy CCS or self-scheduling is configured for the reference cell of a set of cells, and when the reference cell is dormant or deactivated, the BD/CCE budget for the legacy CCS or self-scheduling is the same as R17.

Minimum applicable scheduling offset

Agreement Inclusion of minimum applicable scheduling offset indicator in DCI format 0_X/1_X is configurable

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

MinSchedulingOffsetK2-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r16)

The field type for the minimum applicable scheduling offset indicator is not defined. Since all scheduled cells have the same FR/SCS, Type-1A can be considered for the minimum applicable scheduling offset indicator. This field only has two bits, the 1st value of min scheduling offset (if configured) is applied if the field indicates 0, and the 2nd value of min scheduling offset (if configured) is applied when the field indicates 1. For cells without MinSchedulingOffsetK0 or MinSchedulingOffsetK1, this field is ignored.

Proposal 15. Minimum applicable scheduling offset indicator is Type1A

Proposal 16. For co-scheduled cells with MinSchedulingOffsetK0 or MinSchedulingOffsetK1, when the minimum applicable scheduling offset indicator indicates 0, the 1st value of min scheduling offset indicator is applied; when the field indicates 1, the 2nd value of min scheduling offset indicator is applied. For co-scheduled cells without MinSchedulingOffsetK0 or MinSchedulingOffsetK1, this field is ignored.

DWS

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI.

It is agreed that a DWS indicator is introduced to indicate the dynamic waveform switching in a UL scheduling DCI. For the case when multiple PUSCHs are scheduled by DCI format 0_X for different cells, there is no need to force all co-scheduled cells to use the same waveform, and thus per-cell level DWS indication can be considered. If there’s concern on the overhead of mc-DCI format due to separate DWS indication for each serving cell, Type-2 DWS can be considered at least for inter-band CA cases, while for intra-band CA case, a single DWS bit that is common for all co-scheduled cell can be considered. Thus, we have the following proposals:

Proposal 17. The inclusion of dynamic waveform indication in DCI format 0_X is supported and can be configurable.

Proposal 18. If dynamic waveform indication is present in DCI format 0_X, it is Type-2 at least when the co-scheduled cells are inter-band CA cells.

Configuration of DCI fields

Agreement For a set of cells configured for multi-cell scheduling using DCI format 0_X/1_X, - the size of a Type-1A field in the DCI format 0_X/1_X is determined as maximum field size of active BWP among all cells within the set of cells. - the size of a Type-1B field in the DCI format 0_X/1_X is equal to ceiling(log2(N)), where N is the number of rows in RRC-configured table with each row containing multiple indexes for all cells within the set of cells. - The Type-1B field indicates one row of the configured table - The Type-1B index for a cell points to a corresponding index in a RRC configured table applicable for DCI format 0_1/1_1 or MAC CE activated values. - the size of a per cell Type-2 field in the DCI format 0_X/1_X is determined based on active BWP for each cell.

In the last meeting, RAN1 discussed which fields require new RRC parameters. Some companies believe that to reduce RRC signaling overhead, most of the per cell/per CG configured fields in mc-DCI format can reuse existing parameters for the corresponding fields in DCI format 1_1/0_1. However, for some newly introduced configurable fields or fields with new types (such as Type-1B, Type-3), new parameters need to be introduced. Our views on the introduction of new parameters are summarized in Table 1.

Considering that mc-scheduling can be seen as part of scheduling for the reference cell, these new parameters can be placed in ServingCellConfig of the reference cell. Alternatively, they can be carried by ServingCellConfig of the scheduling cell, or PhysicalCellGroupConfig. It should be noted that if these configurations are carried in the scheduling cell or PhysicalCellGroupConfig, the relationship between the configurations and the cell set needs to be clearly indicated. Having said that, the signalling design is up to RAN2.

Proposal 19. Regarding the introduction of new RRC parameters for the mc-DCI format, Table 1 can be considered.

Table 1 Introduction of new RRC parameter

| Field | Field Type | New RRC parameter | Details |
| --- | --- | --- | --- |
| Identifier for DCI formats | Type 1A | No need | always 1 bit in mc-DCI |
| Downlink assignment index | Type 1A | No need |  |
| TPC for scheduled PUCCH | Type 1A | No need |  |
| PUCCH resource indicator | Type 1A | No need |  |
| PDSCH-to-HARQ timing indicator | Type 1A | No need |  |
| One-shot HARQ-ACK request | Type 1A | No need |  |
| BWP indicator | Type 1A | No need |  |
| VRB-to-PRB mapping | Type 1A | No need if reuse the parameter for DCI format 1_1, new parameter is needed if not to reuse the parameter for DCI format 1_1 |  |
| PRB bundling size indicator | Type 1A | No need if reuse the parameter for DCI format 1_1, new parameter is needed if not to reuse the parameter for DCI format 1_1 |  |
| DMRS sequence initialization | Type 1A | No need | DMRS sequence initialization field of DCI format 1_X is always present, DMRS sequence initialization field of DCI format 0_X is determined by whether there is a co- scheduled cell in the set enabling transform precoder. |
| Frequency hopping flag | Type 1A | No need if reuse the parameter for DCI format 0_1, new parameter is needed if not to reuse the parameter for DCI format 0_1 |  |
| Open-loop power control parameter set indication | Type 1A | No need |  |
| Enhanced Type-3 codebook indicator | Type 1A | No need |  |
| HARQ-ACK retransmission indicator | Type 1A | No need |  |
| PUCCH Cell indicator | Type 1A | No need |  |
| Priority indicator | Type 1A | New RRC parameter: Presence of priority indicator | The configuration can be in the RRC configuration of the reference cell |
| ChannelAccess-Cpext | Type 1A | No need |  |
| ChannelAccess-Cpext-CAPC | Type 1A | No need |  |
| beta offset indicator | Type 1A | No need |  |
| Rate matching indicator | Type 1B (up to 4 bits) | New RRC parameter: Table of joint RM indication | The configuration can be in ServingCellConfig of reference cell or scheduling cell , or in PhysicalCellGroupConfig |
| ZP CSI-RS trigger | Type 1B (up to 3 bits) | New RRC parameter: Table of joint CSI-RS triggering | The configuration can be in ServingCellConfig of reference cell or scheduling cell , or in PhysicalCellGroupConfig |
| TCI | Type 1B (up to 4 bits) | New RRC parameter: Table of joint CSI-RS triggering | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| SRS request | Type 1B (up to 4 bits) | New RRC parameter: Table of joint SRS request | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| SRS offset indicator | Type 1B (up to 3 bits) | New RRC parameter: Table of joint SRS offset indicator | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| TDRA | Type 1B | New RRC parameter: Table of TDRA index | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| CSI request | Type 1C | No need |  |
| UL-SCH indicator | Type 1C | No need |  |
| HARQ process number | Type 2 | No need |  |
| MCS | Type 2 | No need |  |
| New data indicator per TB | Type 2 | No need |  |
| Redundancy version per TB | Type 2 | No need |  |
| FDRA | Type 2 | New RRC parameter: New RBG size “Configuration 3” for RA type 0 New RRC parameter of RBG granularity for RA type 1 | New RBG size “Configuration 3” for RA type 0 is per BWP per co-scheduled cell New RRC parameter of RBG granularity for RA type 1 is per BWP per co-scheduled cell |
| TPC command for scheduled PUSCH | Type 2 | No need |  |
| PTRS-DMRS association | Type 2 | No need |  |
| Antenna port(s) | Type 3: Configurable between Type 1A and Type-2 | New RRC parameter: Type configuration | per BWP per co-scheduled cell |
| Precoding information and number of layers | Type 3: Configurable between Type 1A and Type-2 | New RRC parameter: Type configuration | per BWP per co-scheduled cell |
| SRS resource indicator | Type 3: Configurable between Type 1A and Type-2 | New RRC parameter: Type configuration | per BWP per co-scheduled cell |
| Indicator of cell sets | Reuse carrier indicator | New RRC parameter: configuration of cell sets | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| Indicator of co-scheduled cells | N/A | New RRC parameter: Table of cell combinations | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| SCell dormancy indication | N/A | New RRC parameter: Presence of SCell dormancy indication | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| PDCCH monitoring adaptation indication | N/A | New RRC parameter: Presence of PDCCH monitoring adaptation indication | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| Minimum applicable scheduling offset indicator | N/A | New RRC parameter: Presence of minimum applicable scheduling offset indicator | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |
| Dynamic waveform indication | Type-2 | New RRC parameter: Presence of dynamic waveform indication | The configuration can be in ServingCellConfig of reference cell or scheduling cell, or in PhysicalCellGroupConfig |

Remaining issues on HARQ-ACK codebook

In previous meetings, the enhancement of Type-2 HARQ-ACK codebook and multiple sets of cells have been agreed.

Agreement in 110 - For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell. - Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell. - FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one. - Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook. - If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell; - Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. - HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs. - HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling. Agreement in 110b-e For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. Agreement in 112 Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - When multiple sets of cells are configured, - a cell in one set of cells can’t be included in another set of cells. - n_CI value is independently configured for each set of cells. - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells. - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells. - DCI size of DCI format 0_X is independently determined for each set of cells. - DCI size of DCI format 1_X is independently determined for each set of cells. - The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability. - An indicator is included in the DCI to indicate the scheduled set of cells, - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells. - Unique n_CI value is configured for each set of cells.

According to the agreements, for the Type-2 HARQ-ACK codebook, the number of HARQ-ACK information bits for each DCI format 1_X can be determined by the configuration of codeword and spatial bundling for a set of cells. Specifically, if at least one cell of a set of cells is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules the cell set is equal to N, where N is the maximum number of cells that can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

However, when multiple sets of cells are configured, how to determine the HARQ-ACK information bits for DCI format 1 X needs further study. For example, there are two cell sets (i.e. set1 and set2), set1 has A cells, and one cell in set1 is configured with maximum 2 codewords per PDSCH without spatial bundling, the maximum number of TBs which can be co-scheduled by a DCI format 1_X for set1 is M_A; set2 has B cells, all cells in set2 are configured with a single codeword, the maximum number of cells which can be co-scheduled by a DCI format 1_X for set2 is N_B, M_A>N_B. The following alternatives can be considered:

- Alt.1: the HARQ-ACK information bits corresponding to a DCI format 1_X are determined according to the multiple sets of cells in the same PUCCH group. In the above example, the number of HARQ-ACK information bits corresponding to a DCI format 1_X scheduling a cell combination in set1 or set2 is determined as max (M_A, N_B).

- Alt.2: the HARQ-ACK information bits for different cell sets are separately determined. In the above example, for set 1 and set 2, the number of HARQ-ACK information bits are M_A and N_B separately.

Compare Alt.1 with alt 2, alt.1 is simpler because the DAI can be counted across all the cell sets, and HARQ-ACK information bits for different cell sets can be collected in the same sub-codebook.

Alt.2 is more complex. In this case, when the number of HARQ information bits varies for different sets, the DAI counting has to be separately performed for each cell set, and the corresponding HARQ-ACK information bits for different sets cannot be carried by a single sub-codebook. Otherwise, some restrictions on NW scheduling are needed. For example, UE does not expect the multi-cell scheduling of different sets of cells to result in the HARQ-ACK feedback for more than one set at the same time or in the same resource. Therefore, Alt.1 is preferred.

Proposal 20. When multiple sets of cells are configured, for Type-2 codebook, the number of HARQ-ACK information bits is determined as follows.

- if at least one cell in the multiple sets of cells is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of any set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE. Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

It has been agreed that for Type-2 HARQ-ACK codebook, HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X are ordered based on serving cell indices associated with co-scheduled PDSCHs. However, whether the ordering is performed first in order of cell index, or first in order of TB index should be clarified. Since there is no distinct difference between the two ordering rules, the HARQ-ACK bits can be ordered first in ascending order of cell index and then second in ascending order of TB index similar to the legacy rule.

Proposal 21. For Type-2 codebook, HARQ-ACK bits can be ordered first in ascending order of cell index and then second in ascending order of TB index similar as the legacy rule.

If a deactivated or a dormant cell can be scheduled by mc-DCI format, the Type-2 HARQ-ACK information bits should still be generated although no actual PDSCH transmission takes place in the deactivated or dormant cell(s). Similarly, the HARQ-ACK information bits for the candidate PDSCH occasion which collides with semi-static TDD UL/DL configuration should also be generated. For example, the HARQ-ACK information bits of PDSCH in the deactivated cell or dormant cell or an occasion with TDD UL/DL collision can be placed in the ACK/NACK bit position determined according to the order of cell indices and TB indices in the codebook. For Type-1 codebook, according to the legacy procedure, UE feedbacks HARQ-ACK based on the first active BWP for a deactivated cell, and, UE is not expected to feedback HARQ-ACK for the dormant cell. While for Type-3, UE should feedback HARQ-ACK for all the configured cells. Thus, there is no need for enhancement for Type-1 or Type-3 codebook when a co-scheduled cell becomes deactivated or a dormant cell.

Proposal 22. For Type-2 codebook, the HARQ information bits of PDSCH occasion in the deactivated cell or dormant cell or an occasion with TDD UL/DL collision should be placed in the ACK/NACK bit position determined according to the order of the cell indices and TB indices.

Remaining issues on UL TX switching

Ambiguity handling for dual UL

In previous meetings, the following agreements were made.

Agreement in 111 Following working assumption is confirmed with updates. Working Assumption for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases - Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported - if twoT is indicated, both of two Tx chains are switched to band B - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A - Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported - if twoT is indicated, both of two Tx chains are switched to band C - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is - based on new RRC parameter Agreement in 111 In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives. - An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band) - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}. - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C) - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D) If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled.

It is proposed by some companies to discuss the case that concurrent transmission on band pair A+B is not supported but oneT is indicated and the switching is performed from 2T on band A to 1 port transmission on band B. The following proposal has been discussed in last meeting but is not converged.

Proposal 6-4[2]: Down-select from following solutions for the issue in case that concurrent transmission on band pair A+B is not supported but oneT is indicated and the switching is performed from 2T on band A to 1 port transmission on band B. - Alt.1: another Tx chain is switched to configured associated band for band B as in case#2 - Alt.2: another Tx chain is also switched to band B even though oneT is configured

Per the agreement for case #2, if there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled. When the concurrent transmission on band pair A+B is not supported, band A should not be configured as the associated band for band B. If oneT is indicated and the switching is performed from 2T on band A to 1 port transmission on band B, and if band B is configured with another associated band (e.g., band C), either Alt.1 or Alt.2 works. However, if band B is not configured with any other associated bands, the issue in proposal 6-4 above becomes quite similar to case#2, and Alt.2 should be used. Thus, Alt.2 is preferred as it is applicable to all cases.

Proposal 23. In case concurrent transmission on band pair A+B is not supported but oneT is indicated and the switching is performed from 2T on band A to 1 port transmission on band B, another Tx chain is also switched to band B even though oneT is configured.

Ambiguity between one switching and two switchings

### Proposal 5-1[2]: - When a UE is triggered to perform TX switching between a band pair, and the start of the UL transmission after TX switching is T0, UE uses grants received before T0-Toffset to determine how to perform switching, where Toffset is the UE processing procedure time defined for the uplink transmission triggering. - To determine the Toffset which is composed of N2 and Tswitch, the minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL and the minimum SCS among the UL carriers after Tx switching is used as µUL to determine N2, additionally, the minimum SCS among the UL carriers involved in Tx switching is used as µUL to determine Tswitch - If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band C->band B + band D), when the two UL transmissions after TX switching are at least partially overlapped in time domain, UE perform it as one TX switching involving more than 2 bands, otherwise as twice of TX switching. - For UE having capability to use Tx chain for transmission even during the switching performed at another Tx chain, if the two Tx chains are triggred to switch between two different band pairs (e.g., band A + band C->band B + band D), when there is time domain overlapping between two bands Tx switching period, UE perform it as one TX switching involving more than 2 bands, otherwise as twice of TX switching. Comments from FL It seems majority supports intention of the proposal, while some further discussion is necessary e.g., for following points. ・definition of T0 and Toffset, especially when starting timing of two uplink transmissions after uplink switching are different ・Tadd is necessary or not ・condition on “partially overlapped in time domain” is necessary or not ・how to handle UE with advanced capability agreed in RAN4

The above proposal has been discussed and most companies think that the ambiguity issue of Tx switching needs to be resolved in last meeting, however, there are still some points needs further clarification.

- Firstly, for the 2nd sub-bullet, it is not clear how to derive T0 when the two UL transmissions after the Tx switching have different starting points.

Assuming the starting points of the earlier transmission and the later transmission after the Tx switching are T0_1, T0_2, respectively. After receiving DCI#1 scheduling the earlier transmission on B, UE may decide to perform A+C->B+C if the associated band of B is C. If the DCI#2 scheduling the later transmission on D is later than T0_1-Toffset, it would be too late for UE to change the switching case to A+C->B+D. Thus, if NW wants to schedule overlapped UL transmission on B+D, it should ensure that DCI#2 is no later than T0_1 – Toffset . In other words, in this case, T0 is the start of the earlier UL transmission of the two UL transmissions. The Toffset is the UE processing procedure time defined for the uplink transmission triggering as  Tproc,2 in the following formula, the related specification description is pasted here:

- N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211]. - If uplink switching gap is triggered as defined in subclause 6.1.6, equals to the switching gap duration and for the UE configured with uplinkTxSwitchingOption2 µUL=min(µUL,carrier1, µUL,carrier2), otherwise .

From the highlighted part, it can be seen that N2  is related with the µUL and µDL, and is related with µUL. For “one Rel-18 Tx switching involving 3 or 4 bands”, this related SCS should be specified. The minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL and the minimum SCS among the UL carriers after Tx switching is used as µUL to determine N2, additionally, the minimum SCS among the UL carriers involved in Tx switching is used as µUL to determine .

- Secondly, whether the condition on “partially overlapped in time domain” is necessary or not.

Here is an example as shown in Figure 1 and Figure 2, the preceding uplink transmissions are a 1-port transmission on band A and 1-port transmission on band B, and the UE is indicated by DCI#1 and DCI#2 to transmit 1-port UL transmission on band C and 1-port UL transmission on band D, respectively. There is a gap between UL transmissions on band C and band D in Figure 1. While in Figure 2, the UL transmissions on band C and band D are partially overlapped in the time domain. The switching period reported for band pair 1 (band A and band C) and band pair 2(band B and band D) is Tswitch1 and Tswitch2 respectively, and we assume Tswitch1 > Tswitch2.

Figure 1 Switching period applies to two Rel-18 Tx switching vs one Rel-18 Tx switching when a gap is between the starting time of the two succeeding transmissions

Figure 2 Switching period applies to twice of Tx switching vs one Rel-18 Tx switching when the transmissions are overlapped in the time domain

If a UE performs the switching as twice of Tx switching, as shown in the left part of Figure 1 and Figure 2, according to DCIs received before T0,1-Toffset and T0,2-Toffset for each band pair respectively, the UL transmission on band C would be interrupted by Tx switching for another band pair(band B to band D) in Figure 2, while it does not happen in Figure 1. T0,1 and T0,2 correspond to the starting time of UL transmission on band C and band D respectively, while Toffset is the UE processing procedure time defined for the uplink transmission triggering.

In contrast, if UE performs the switching as “one Rel-18 Tx switching” involving 4 bands, the larger one between the two reported switching period values for the two band pairs A+C and B+D should be applied. As shown in the right part of Figure 1 and Figure 2, the UL transmissions on band A and band B are interrupted by the larger switching period but the UL transmission on C is not.

Comparing the left part and right part in Figure 1, the victim bands of the switching periods are the same. Thus, when there is a gap between UL transmission on band C and band D, UE can implement the two Tx switchings separately. While in Figure 2, the left part (‘twice of Tx switching’) leads to more victim bands and more UL interruption compared with the right part. Therefore, UE should perform the “one Rel-18 Tx switching” involving 3 or 4 bands when the two UL transmissions with Tx switching are triggered and the two UL transmissions on the target bands that are at least partially overlapped in time domain.

- Thirdly, how to handle UE with advanced capability agreed in RAN4.

The advanced capability agreed in RAN4 is as follows:

LS from R4-2303507 Issue 3: Impact from switching of one Tx chain on the other Tx chain Scenario of one band with the number of Tx chain unchanged due to switching When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged during UL switching as follows: - Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination.

The last sub-sub-bullet in Proposal 5-1 is trying to define different UE behaviors in cases where there is time domain overlap between two Tx switching periods for two different band pairs, compared to those where such overlap does not occur. This is because, for switching A->C and B->D, advanced UE may perform the two switchings separately as the switching period for one band pair will not impact the ongoing UL transmission on a third band, the situation would be slightly different from the above case in Figure 2.

However, it is still not clear whether the network can determine when UE decides to perform two TX switching, where the switching period location is, and whether there is such overlapping between the two switching periods of the two different band pairs, as currently the time location of the switching period is up to UE when there is a sufficient gap. Even if the Rel-16 discussion on specifying a fixed switching period location is agreed and the switching period has to be completed right before the following UL transmission, there is no clear benefit to having two non-overlapped switching periods. Moreover, different implementations are required for the above cases, which also lead to larger spec efforts. Thus, no special treatment for advanced UE is not preferred.

Observation 1: It is still not clear whether the network can identify when UE decides to perform two TX switching, where the switching period location is, as well as whether there is such overlapping between the two switching periods of the two different band pairs, as currently the time location of the switching period is up to UE when there is a sufficient gap.

Consequently, some refinements to the proposal are made to make it clearer.

Proposal 5: When a UE is triggered to perform TX switching between a band pair, and the start of the UL transmission after TX switching is T0, UE uses grants received before T0-Toffset to determine how to perform switching, where Toffset is the UE processing procedure time defined for the uplink transmission triggering.

  - To determine the N2 of Toffset, the minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL, and the minimum SCS among the UL carriers after Tx switching is used as µUL

  - To determine the Tswitch of Toffset, the minimum SCS among the UL carriers involved in Tx switching is used as µUL

  - If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band C->band B + band D), and when the two UL transmissions after TX switching are at least partially overlapped in time domain, one TX switching involving more than 2 bands should be performed, and T0 is the start of the earlier UL transmission of the two UL transmission; otherwise, twice of TX switching should be performed.

- Discussion on eDSS

In the previous meeting, the following agreements were made for PDCCH reception on CRS symbol. In the section, we provide our views on some remaining issues regarding this aspect.

Agreement If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs” is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

RRC configuration

Alt-1: A new RRC parameter to enable reception of NR PDCCH candidates overlapped with LTE CRS REs Alt-2: Specify configuration of PDCCH candidates/aggregation levels of a search space that is to be monitored when overlapped with LTE CRS REs Alt-3: Specify that PDCCH aggregation levels higher than the aggregation levels employed in the CSS during the transition from RRC Idle to RRC connected state are assumed to be monitored

It was discussed in the previous meeting whether a new RRC parameter is needed to enable the reception of NR PDCCH candidates overlapped with LTE CRS REs. Some companies commented that UE will automatically determine whether the R18 DSS feature is enabled based on CORESET configuration. However, this would lead to some issues. First, it is worth noting that RAN2 provides the following guideline in LS R2-2002378, explaining that gNB should always configure the UE explicitly by DL RRC signaling, respecting the reported capabilities.

5 Avoid defining functionality that has no RRC configuration but is dependent on capability bits. The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting the reported capabilities. A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

Second, starting from R15, NW is allowed to configure a CORESET with precoderGranularity = sameAsREG-bundle overlapped with CRS symbol. The UE drops the candidate overlapped with CRS and decodes the clean ones. Without an explicit indication of R18 DSS, the R18 UE has no information on whether the NW support R18 DSS or not, then it does not know whether to drop or decode the PDCCH candidate overlapped with CRS. If the R18 UE always buffers the overlapping part and performs PDCCH reception, it consumes unnecessary power when NW is not deploying R18 DSS because NW may never use the overlapping part for PDCCH transmission.

Furthermore, when the NW enables R18 DSS and the UE supports both CE mechanisms, which CE mechanism is used should be determined. According to the previous simulation results, the PDCCH performance not only depends on the receiver type, but also heavily relies on the PDCCH decoding and CE mechanisms. When the PSD ratio of CRS and PDCCH is 1:1, the PDCCH performance of legacy receivers (i.e., legacy PDCCH decoding+legacy CE) deteriorates significantly compared with R17. Moreover, it suffers from unacceptable performance losses especially when the AL is small, whereas the receivers capable of PDCCH puncturing and CE based on clean symbol can achieve an attractive gain in most cases. If the UE performs PDCCH puncturing together with legacy CE, the PDCCH performance is worse than the case with PDCCH puncturing with advanced CE, but it still outperforms the legacy receiver. When the CRS PSD is relatively low, legacy receivers may have better performance. Thus, it is important to apply a suitable CE mechanism to achieve the best performance, which is highly dependent on the network deployment that the UE is not aware of.

Consequently, it was also a discussion on whether an RRC indication should be used to provide some guidance to UE side, e.g., explicitly indicating which CE mechanism to use. Some companies commented that it should be up to UE implementation to use the CE mechanism with the best performance. However, it is questionable whether the UE can identify which CE mechanism has the best performance without any guidance from NW. Furthermore, leaving the selection of CE mechanism entirely to the UE implementation also means that the gNB shall guarantee PDCCH decoding performance to meet the requirements in the worst case. Additionally, from the NW side, it may not transmit PDCCH/DMRS on CRS REs to protect the LTE RAT and in this case ‘CE on clean symbol(s) only’ is preferred. Therefore, RRC configuration should be introduced to suggest the preferred CE to be used. If the gNB indicates two CE mechanisms, the UE can select one of them. Nevertheless, it is worth noting that if the RRC indication is introduced, the UE is not expected to be indicated with 'clean symbol based CE' by the gNB for a single symbol CORESET overlapped with a CRS symbol.

The RRC indicator to enable R18 DSS can be also used to indicate the preferred CE mechanism, thus we have the following proposal

Proposal 24. Introduce a RRC indicator to inform that R18 DSS is enabled.
-  if the RRC indicator is provided, R18 DSS is enabled;

-  if the RRC indicator is not provided, R18 DSS is not enabled;

Proposal 25. The RRC indicator of the enabling of R18 DSS also indicates how to apply R18 DSS CE mechanism:
-  the RRC indicator indicates the CE mechanism to be used by UE.

Proposal 26. For 1-symbol CORESET that is overlapped with CRS, UE is not expected to be indicated to perform 'clean symbol-based CE' by the RRC indicator.

CORESET for PDCCH reception on CRS symbol

One issue that needs to be discussed is whether this R18 DSS feature can be used for any CORESETs. Common CORESETs are shared by idle/inactive UEs and connected R18 UEs, thus they should not overlap with CRS symbols. Otherwise, there would be a negative impact on the PDCCH reception performance of idle UEs.

Proposal 27. Reception of NR PDCCH candidates that overlap with LTE CRS REs is not supported for common CORESET that is shared by idle/inactive UE.

Clarification on UE behavior when two CRS lists are configured

In R17, if a PDCCH candidate is overlapped with the CRS pattern indicated by either lte-CRS-ToMatchAround (included in ServingCellConfigCommon or ServingCellConfig) or lte-CRS-PatternList1/lte-CRS-PatternList2 (included in ServingCellConfig), UE is not expected to monitor the PDCCH candidate. Moreover, CORESET with precoderGranularity = allContiguousRBs should not be overlapped with CRS pattern indicated by lte-CRS-ToMatchAround or lte-CRS-PatternList1/lte-CRS-PatternList2.

TS 38.213 h.3.0 If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. ==omitted== When precoderGranularity = allContiguousRBs, a UE does not expect - to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency - any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList, or with any RE of a SS/PBCH block.

In R18 DSS, the UE may be configured to receive the PDCCH overlapping with one CRS pattern list, and at the same time may also be configured to receive PDSCH overlapping with two CRS pattern lists. If two CRS pattern lists are configured and both lists are configured with 4 CRS ports, there can be up to 8 CRS REs on one RB of PDCCH symbol overlapping with CRS, in which case UE can hardly decode any PDCCH properly. Therefore,  receiving a PDCCH overlapping with two CRS pattern lists is not supported in R18 DSS, which was the common understanding during the WID discussion. Nevertheless, when two CRS pattern lists are configured, it is still not clear whether a UE is required to receive a PDCCH overlapped with only one CRS pattern list, or alternatively, the two features cannot be enabled simultaneously. Further, even if the UE is required to receive the PDCCH, it is not clear whether that PDCCH is expected to overlap with lte-CRS-PatternList1 only, lte-CRS-PatternList2 only, lte-CRS-PatternList3 only, lte-CRS-PatternList4 only or any one of them.

To address the aforementioned issue, the following alternatives were proposed by FL in the previous meeting. In our understanding, alt-2 leads to serious performance loss when more than one CRS pattern are overlapped with NR PDCCH on the same set of RBs. In contrast, alt-1 is a much more realistic approach – for the overlapping RB between a LTE carrier and NR PDCCH, only one of the CRS pattern lists is expected to be overlapped with NR PDCCH if UE is required to monitor PDCCH candidates in a CORESET overlapping with CRS. Nevertheless, further clarification is needed that alt-1 is for a LTE frequency instead of all LTE frequencies.

- Alt-1: Reception of NR PDCCH candidates overlapped with LTE CRS REs from only one configured LTE CRS pattern is supported - Alt-2: Reception of NR PDCCH candidates overlapped with LTE CRS REs from one or multiple LTE CRS patterns of either one or two configured LTE CRS pattern lists is supported

Proposal 28. Reception of NR PDCCH candidates that overlap with LTE CRS REs of a LTE carrier can be supported only when the PDCCH candidates are overlapped with LTE CRS REs from one LTE CRS pattern from one CRS pattern list for that LTE carrier.

Clarification on narrow/wideband CORESET

In R15, CORESET configured with wideband DMRS is not expected to overlap with CRS while CORESET with narrowband DMRS can partially overlap with CRS. This principle should be retained in R18 DSS. With the newly introduced CRS pattern list3/4, wideband DMRS CORESET is still not expected to collide with CRS of either CRS pattern list, while narrowband DMRS CORESET is not expected to overlap with more than one CRS pattern list. Since lte-CRS-PatternList3 and lte-CRS-PatternList4 are overlapped in RB level, if a narrowband CORESET overlaps with one of them, it must overlap with the other. Thus, a narrowband CORESET can be configured for PDCCH reception on CRS symbol when only lte-CRS-PatternList1 or only lte-CRS-PatternList3 is configured.

Proposal 29. For CORESET for PDCCH reception on CRS symbol in R18 DSS, CORESET with precoderGranularity = sameAsREG-bundle can be overlapped with RE determined from lte-CRS-ToMatchAround, or LTE-CRS-PatternList of up to one CRS pattern list (i.e., lte-CRS-PatternList1 or lte-CRS-PatternList3) per LTE carrier.

Proposal 30. CORESET with precoderGranularity = allContiguousRBs is not expected to overlap with CRS RE determined from any LTE-CRS-PatternList.

- Discussion on NCR

AL beam indication conflict

Agreement A priority flag is introduced per list of periodic and semi-persistent indications. The flag gives priority to periodic and semi-persistent indications over aperiodic indications. Additionally, the following applies: - If there is conflict among beam indication from different type of indication, the order of priority is defined as: Aperiodic beam indication > semi-persistent beam indication > periodic beam indication. - No conflict is expected between periodic beam indications - No conflict is expected between semi-persistent indications - If there is conflict between two aperiodic indications, the latest indication is prioritized.

In RAN1#112 meeting, the agreements regarding the resolution of AL beam indication conflict have been reached. In the conflict resolution rule, a priority flag is introduced to give priority to periodic and semi-persistent indications over aperiodic indications. If priority flag is given to periodic indication, the order of priority is periodic beam indication > aperiodic beam indication > semi-persistent beam indication; If the priority flag is given to semi-persistent indication, the order of priority is semi-persistent beam indication > aperiodic beam indication > periodic beam indication; However, if there is conflict between periodic indication and semi-persistent indication, and the priority flag is given to both of the periodic indication and semi-persistent indication, the priority between periodic indication and semi-persistent indication is not clear.

To complete the rule to handle periodic indication and semi-persistent indication conflict, further rule should be defined, e.g., if priority flag is given to both periodic indication and semi-persistent indication, semi-persistent indication has higher priority than periodic indication.

Proposal 31. If priority flag is given to both periodic indication and semi-persistent indication, semi-persistent indication has higher priority than periodic indication.

Remaining issues on backhaul link

In RAN1#112 meeting, the details of the relationship between different beam indication framework has been discussed, and the signalling structure of both separate and unified beam indication has been discussed and formed an LS to RAN2 [1].

Agreement In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-17 beam indication framework is used for C-link, - If no unified TCI is applied for C-link, the beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. (i.e. same as the default beam defined for Rel-15/16 beam indication framework) - If there is unified TCI applied for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Agreement The RRC parameters and MAC CE parameters in R1-2302113 are agreed. LS to RAN2 is approved in R1-2302227.

There are still some remaining issues mentioned in LS to be discussed before the end of NCR WI. The necessary parameters for backhaul beam indication, including the active/deactive bit and BWP selection, should be determined from RAN1 side to help RAN2 for the design of MAC CE signalling.

In RAN1#111, it has been agreed that the semi-static beam indication is used for backhaul link following the beam indication framework in Rel-15 or Rel-17. For Rel-15 beam indication framework, DL beam and UL beam are indicated separately. The DL beam of PDCCH is indicated by MAC CE, as defined in section 6.1.3.15 in TS 38.321, in a semi-static manner. The DL beam of PDSCH is indicated by DCI dynamically selected from a candidate beam set, which is configured by MAC CE defined in section 6.1.3.14 in TS 38.321. According to the agreement in RAN1#111, it is preferred to reuse the signalling structure of PDCCH beam indication as the baseline of DL backhaul beam indication, and the bit field “CORESET ID” could be refarmed as “A/D” bitfield and/or “BWP ID” bitfield if the explicit indication is supported.

Figure 3 TCI State Indication for UE-specific PDCCH MAC CE defined in TS 38.321

Similarly, the signalling design of UL backhaul beam could follow the MAC CE structure of PUCCH defined in section 6.1.3.18 in TS 38.321. The bitfield “PUCCH Resource ID” could be reserved considering that bitfields S7~S0 have been supported for active/deactive indication. It should be noted that only a single PUCCH Spatial Relation Info S7~S0 can be active for a PUCCH Resource at a time, which equals to the beam indication using a bitmap.

Figure 4 PUCCH spatial relation Activation/Deactivation MAC CE defined in TS 38.321

For Rel-17 beam indication framework, the joint UL/DL beam indication of backhaul link could follow the MAC CE structure of Unified TCI States Activation/Deactivation MAC CE defined in section 6.1.3.47 in TS 38.321. The reserved bits could be refarmed as A/D bitfield if supported.

Figure 5 Unified TCI state activation/deactivation MAC CE defined in TS 38.321

In the description of LS, A/D bitfield is used to indicate whether to active or deactive the indicated TCI state. The further issue is which beam should be used for backhaul link transmission after TCI state is indicated to be deactive. One possible scheme is to use the implicit rule (i.e. QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link) after the deactive indication is applied. On the other hand, none of the beam indication signalling mentioned above supports the beam deactive operation in semi-static beam indication in the current specification. To minimize the specification effort, A/D bitfield should not be included in the MAC CE structure.

Proposal 32. A/D bitfield is not included in the MAC CE .

Considering different BWP could be configured with different beam set, BWP should be determined before beam indication. In an implicit way, BWP for backhaul beam indication could be same as the active BWP of the control link, and the TCI state set configured for control link could be reused for backhaul beam indication. Considering that UE connected via NCR could be on a different BWP other than control link BWP, an explicit BWP ID bitfield could be included in the backhaul beam indication signalling to choose a more sufficient TCI state set for the backhaul link.

Proposal 33. BWP ID bitfield is included in the backhaul beam indication signalling considering that UE connected via NCR can be on a different BWP from the control link BWP.

The creation of backhaul beam determination has been agreed in the WI stage, i.e. the backhaul beam is determined in an implicit rule or an explicit indication when control link does not work, otherwise, the beam of control link is used when the control link and backhaul link work simultaneously. However, the beam switching timing between the two cases is not discussed. Considering that there is no restriction on the relation of control link BWP and backhaul link BWP, we can assume that different BWP and different SCS is configured for control link and backhaul link (More precisely, backhaul link BWP refers to the active BWP configured for a UE which is connected to gNB with the help of NCR). For example, SCS of control link BWP is 120kHz, and SCS of backhaul link BWP is 60kHz. As shown in Figure 6, there is a risk that the backhaul beam switches between the control link beam and the default/indicated backhaul beam during the PDSCH transmission which would result in the channel estimation error and PDSCH transmission failure. Therefore, a proper beam switching timing should be considered for backhaul beam switching. To minimize the specification impact, the backhaul beam switching timing could be the slot boundary derived from the reference SCS which is defined in DL/UL slot format configuration.

Proposal 34. The backhaul beam switching timing is the slot boundary derived from the reference SCS which is defined in DL/UL slot format configuration.

Figure 6 An example of the transmission failure when different SCS is configured for control link and backhaul link.

- Discussion on BWP without restriction

In RAN#99, a new WI of “Complete the specification support for BandWidth Part operation without restriction in NR” was approved in [3] with the following objectives.

### The work item includes following objectives: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

For Option B-1-1 and OptionB-1-2 that UE support BM/RLM/BFD based on SSB outside the active DL BWP, from RAN1 perspective, the necessary correction is to allow UE perform BM/RLM/BFD based on SSB outside the active DL BWP in Clause 5 of TS 38.213. The draft CR is also provided in the document of “draft CR 38.213-BWP-wor-Option B-1” that zipped together with this discussion paper.

For Option C that non-RedCap UEs support BM/RLM/BFD based on NCD-SSB within active BWP, following corrections are identified and the corresponding draft CRs can be found in the documents of “draft CR 38.213-BWP-wor-Option C” and “draft CR 38.214-BWP_wor-Option C”that zipped together with this discussion paper.

- For TS 38.213, collison handling for monitoring PDCCH candidate in clasue 10, UL transmissions in clasue 11.1 and slot format in clause 11.1.1 should take the NCD-SSB into account for non-RedCap UEs.

- For TS 38.214, include the NCD-SSB as one factor to determine the invalid symbol for PUSCH repetition type B in clause 6.1.2.1 for non-RedCap UEs supporting half-duplex TDD CA.

Proposal 35. Adopt the draft CR of R1-2301518 of “38.213-BWP-wor-Option B-1” for Option B-1-1 and B-1-2.

Proposal 36. Adopt the draft CR of R1-2301518 of “draft CR 38.213-BWP-wor-Option C” and “draft CR 38.214-BWP_wor-Option C” for Option C.

- Conclusion

In this contribution, we have the following proposals.

For Multi-cell scheduling:

Proposal 1. All PDSCHs scheduled by a DCI format 1_X have the same priority index determined by the priority indicator in DCI format 1_X, and all PUSCHs scheduled by a DCI format 0_X have the same priority index determined by the priority indicator in DCI format 0_X.

Proposal 2. For a cell combination scheduled by a DCI format 0_X/1_X, if all co-scheduled cells in the cell combination are configured with priorityIndicatorDCI-0-1-r16 or priorityIndicatorDCI-1-1-r16 for the corresponding active BWPs, the value of priority indicator can be 1 or 0.

Proposal 3. For a cell combination scheduled by a DCI format 0_X/1_X, if there is at least one co-scheduled cell without priorityIndicatorDCI-0-1 or priorityIndicatorDCI-1-1-r16 configured for its active BWP, the value of priority indicator is expected to be 0, and the priority index 0 is applied to all scheduled PUSCHs/PDSCHs.

Proposal 4. For a set of cells, the size of the CSI request in the DCI format 0_X is determined as the maximum field size of active BWP among all cell combinations within the set of cells.
- For a scheduled cell combination, the field size of the CSI request is determined by the higher layer parameter reportTriggerSize for the cell with the smallest serving cell index among the co-scheduled cells. 
- If the field size of the CSI request for a scheduled cell combination is smaller than the determined field size in the DCI format 0_X, LSB of the field is applied.

Proposal 5. For a set of cells, the size of the UL-SCH indicator in DCI format 0_X is 1 bit.

Proposal 6. For a UE not supporting triggering SRS only in DCI for the smallest serving cell index among the co-scheduled cells, UE is not expected to receive a DCI format 0_X with UL-SCH indicator of "0" and CSI request of all zero(s).

Proposal 7. For a UE not supporting triggering SRS only in DCI for the smallest serving cell index among the co-scheduled cells, the UE is not expected to receive a DCI format 0_ X with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request indicating a codepoint associated with “No aperiodic SRS resource set triggered” for the serving cell.

Proposal 8. Clarify how to determine the total field size of a Type-2 field when the table defining combinations of co-scheduled cells for the set of cells is configured, including whether the total size of a Type-2 field is determined as the field size required for active BWP of the scheduled cell combination or the maximum field size for active BWP among all cell combinations and whether the total size changes according to the scheduled cell combination.

Proposal 9. The default type for ‘Antenna port(s)’, ‘Precoding information and number of layers’ or ‘SRS resource indicator’ is Type-2, if the type field is not configured for these fields by NW, Type-2 is assumed for these fields.

Proposal 10. From the perspective of the reference cell, n_CI for mc-DCI format is determined in the same way as legacy DCI format, moreover, mc-DCI format and legacy DCI format should be configured in different SS sets if both mc-DCI format and legacy DCI format are configured to schedule the same cell.

Proposal 11. For a set of cells, the following solutions can be considered:
- Alt1. NW should ensure that the size of mc-DCI format(s) is always different from any legacy single-cell scheduling DCI format(s) (i.e., DCI format 0_0/0_1/0_2/1_0/1_1/1_2) configured for the mc-DCI format.
- Alt2. NW should ensure that mc-DCI format(s) and legacy single-cell scheduling DCI format(s) (i.e., DCI format 0_0/0_1/0_2/1_0/1_1/1_2) are configured with non-overlapped PDCCH monitoring occasion or PDCCH candidates.

Proposal 12. If a co-scheduled cell becomes inactive or dormant, gNB is still allowed to indicate a cell combination including that cell in DCI format 0_X/1_X, UE simply ignores the scheduling information for the inactive/dormant cell.

Proposal 13. If a co-scheduled cell becomes inactive, the active BWP is assumed to be the BWP with firstActiveDownlinkBWP-Id for that cell when determining the mc-DCI format size and field size.

Proposal 14. If legacy CCS or self-scheduling is configured for the reference cell of a set of cells, and when the reference cell is dormant or deactivated, the BD/CCE budget for the legacy CCS or self-scheduling is the same as R17.

Proposal 15. Minimum applicable scheduling offset indicator is Type1A

Proposal 16. For co-scheduled cells with MinSchedulingOffsetK0 or MinSchedulingOffsetK1, when the minimum applicable scheduling offset indicator indicates 0, the 1st value of min scheduling offset indicator is applied; when the field indicates 1, the 2nd value of min scheduling offset indicator is applied. For co-scheduled cells without MinSchedulingOffsetK0 or MinSchedulingOffsetK1, this field is ignored.

Proposal 17. The inclusion of dynamic waveform indication in DCI format 0_X is supported and can be configurable.

Proposal 18. If dynamic waveform indication is present in DCI format 0_X, it is Type-2 at least when the co-scheduled cells are inter-band CA cells.

Proposal 19. Regarding the introduction of new RRC parameters for the mc-DCI format, Table 1 can be considered.

Proposal 20. When multiple sets of cells are configured, for Type-2 codebook, the number of HARQ-ACK information bits is determined as follows.

Proposal 21. For Type-2 codebook, HARQ-ACK bits can be ordered first in ascending order of cell index and then second in ascending order of TB index similar as the legacy rule.

Proposal 22. For Type-2 codebook, the HARQ information bits of PDSCH occasion in the deactivated cell or dormant cell or an occasion with TDD UL/DL collision should be placed in the ACK/NACK bit position determined according to the order of the cell indices and TB indices.

For TX switching:

Proposal 23. In case concurrent transmission on band pair A+B is not supported but oneT is indicated and the switching is performed from 2T on band A to 1 port transmission on band B, another Tx chain is also switched to band B even though oneT is configured.

Proposal 24. Introduce a RRC indicator to inform that R18 DSS is enabled.
-  if the RRC indicator is provided, R18 DSS is enabled;

For eDSS:

Proposal 25. The RRC indicator of the enabling of R18 DSS also indicates how to apply R18 DSS CE mechanism:
-  the RRC indicator indicates the CE mechanism to be used by UE.

Proposal 26. For 1-symbol CORESET that is overlapped with CRS, UE is not expected to be indicated to perform 'clean symbol-based CE' by the RRC indicator.

Proposal 27. Reception of NR PDCCH candidates that overlap with LTE CRS REs is not supported for common CORESET that is shared by idle/inactive UE.

Proposal 28. Reception of NR PDCCH candidates that overlap with LTE CRS REs of a LTE carrier can be supported only when the PDCCH candidates are overlapped with LTE CRS REs from one LTE CRS pattern from one CRS pattern list for that LTE carrier.

Proposal 29. For CORESET for PDCCH reception on CRS symbol in R18 DSS, CORESET with precoderGranularity = sameAsREG-bundle can be overlapped with RE determined from lte-CRS-ToMatchAround, or LTE-CRS-PatternList of up to one CRS pattern list (i.e., lte-CRS-PatternList1 or lte-CRS-PatternList3) per LTE carrier.

Proposal 30. CORESET with precoderGranularity = allContiguousRBs is not expected to overlap with CRS RE determined from any LTE-CRS-PatternList.

For NCR:

Proposal 31. If priority flag is given to both periodic indication and semi-persistent indication, semi-persistent indication has higher priority than periodic indication.

Proposal 32. A/D bitfield is not included in the MAC CE .

Proposal 33. BWP ID bitfield is included in the backhaul beam indication signalling considering that UE connected via NCR can be on a different BWP from the control link BWP.

Proposal 34. The backhaul beam switching timing is the slot boundary derived from the reference SCS which is defined in DL/UL slot format configuration.

For BWP without restriction:

Proposal 35. Adopt the draft CR of R1-2301518 of “38.213-BWP-wor-Option B-1” for Option B-1-1 and B-1-2.

Proposal 36. Adopt the draft CR of R1-2301518 of “draft CR 38.213-BWP-wor-Option C” and “draft CR 38.214-BWP_wor-Option C” for Option C.

- Reference

- R1-2302114, Summary#2 of discussion on multi-carrier UL Tx switching scheme	Moderator (NTT DOCOMO, INC.)

- R1-2302515, Discussion on UE features for Multi-carrier enhancements,vivo

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Moderator (Vodafone), vivo