3GPP TSG-RAN WG1 #112bis-e	R1-2302518

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on invalid symbol determination for PUSCH repetition type B for BWP operation without restriction using Option C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | BWP_wor |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For BWP operation without restriction using Option C (i.e., support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs), NCD-SSB should also be used to determine the invalid symbol for PUSCH repetition type B for non-RedCap UEs supporting half-duplex TDD CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the NCD-SSB for invalid symbol determination for PUSCH repetition type B in TS 38.214 clause 6.1.2.1 for non-RedCap UEs supporting half-duplex TDD CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification does not support BWP operation without restriction for Option C. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the Release 17 specification version 17.5.0, it will be based on the Release 18 specification when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.2.1	Resource allocation in time domain

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

*** unchanged text omitted ***

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by [NonCellDefiningSSB], or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

and

-	a symbol is considered as an invalid symbol in another cell among the set of serving  cell(s) provided with directionalCollisionHandling-r16 for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell as defined in Clause 11.1 of [6, TS 38.213], or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

*** unchanged text omitted ***