3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302520

e-Meeting, April 17th – April 26th, 2023

Source: 	National Spectrum Consortium

Title: 	Sidelink Physical Channel Design Considerations

Agenda item:	9.4.1.2

Document for:	Discussion and Decision

# Background

The WI on NR sidelink evolution [1] includes an objective to study and specify support for sidelink operation in the unlicensed bands with focus on the FR1 unlicensed bands, n46 and n96/n102. The agreements related to SL-U physical channel design at the 3GPP RAN WG1 #112 meeting are listed below [2].

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

In this contribution we present our views on open and unaddressed issues on this topic.

# Channel Design

## PSFCH Aspects

Multiple alternatives have been listed for PSFCH resource allocation with 15 kHz and 30 kHz SCS with potential down selection. We believe Alt 1-1a (each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)) offers the optimal combination of flexible capacity and adherence to OCB/PSD limitations. K3 should be configurable between 1, 2, 4.

Proposal 1: Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, support Alt 1-1a.

Another open issue is details of PSFCH transmission dropping due to LBT failure. NR-U greatly expanded the flexibility and codebook construction of HARQ ACK/NACK feedback, including on-demand dynamic HARQ codebooks, precisely to make it robust to transmission dropping due to PUCCH/PUSCH LBT failure. We propose to augment the single R16/17 pre-configured PSFCH resource following a PSSCH with an additional PSFCH occasion that is preconfigured and dynamically indicated to avoid unnecessary PSFCH transmissions.

Proposal 2: To address PSFCH transmission dropping due to LBT failure when more than 1 PSFCH occasion per PSCCH/PSSCH transmission is available, support Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated.

## S-SSB Aspects

The support of additional candidate S-SSB occasions was agreed in the last meeting. The legacy S-SSB is 13 symbols in duration for normal CP and 12 symbols for ECP. Within a S-SSB period of 16 frames, 1, 2, 4, 8, 16, 32, or 64 S-SSB transmissions can currently be indicated to the UE. Whether the additional candidate occasions are excluded from the resource pool or not remains open. Including them in the resource pool increases the risk of channel access contention between S-SSB and PSCCH/PSSCH, which is undesirable.

Proposal 3: Support Option 1: Additional candidate S-SSB occasions are excluded from resource pool.

Next, we examine frequency-domain allocation. Legacy S-SSB spans 132 subcarriers, which equates to 1.98 MHz and 3.96 MHz for 15 kHz and 30 kHz SCS, respectively. From previous meetings we have a combination or permutations of the following choices

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

Option 1-1 is not preferred due to the need for link-level performance analysis, impact on legacy S-SSB acquisition implementation, and there is no precedent for interlaced SSB in NR-U. Option 3-1 has a PAPR problem but is otherwise the simpler approach if temporary OBW relaxation allowance and multiplexing by scheduling are not sufficient.

Proposal 4: Support Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement.

Finally, consider Tx UE behaviour on additional S-SSB occasions. The options under discussion are:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions.

Alt. 2 seems to increase the level of contention in the network. Alt. 3 provides the most flexibility in terms of channel access, CO management, coverage enhancement, and network discoverability by new UEs. Alt. 4 introduces unnecessary restrictions on S-SSB; if LBT conditions are satisfied then no duty cycle limit is needed.

Proposal 5: Support Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

## Power Control Enhancements

Baseline SL power control is open loop for PSSCH/PSCCH/PSFCH transmissions. NR-U utilizes closed-loop power control in comparison. However, the UL power control mechanisms for NR-U are the same as power control for operation without shared spectrum access. Since interlace-based transmissions have already been introduced in SL-U to overcome PSD limitations, it is not clear what additional enhancements are necessary.

Proposal 6: Clarify the extent and necessity of power control enhancements due to PSD limits in unlicensed spectrum.

# Summary of Proposals

This contribution presented the following proposals for SL-U physical channel design.

Proposal 1: Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, support Alt 1-1a.

Proposal 2: To address PSFCH transmission dropping due to LBT failure when more than 1 PSFCH occasion per PSCCH/PSSCH transmission is available, support Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated.

Proposal 3: Support Option 1: Additional candidate S-SSB occasions are excluded from resource pool.

Proposal 4: Support Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement.

Proposal 5: Support Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

Proposal 6: Clarify the extent and necessity of power control enhancements due to PSD limits in unlicensed spectrum.

# References

- 3GPP RP-230077: "WID revision: NR sidelink evolution ".

- Chairman’s Notes, RAN WG1 #112

- 3GPP TS 37.213 V17.3.0 (2022-09), Physical layer procedures for shared spectrum channel access (Release 17)