3GPPTSG RAN WG1 #112bis-e                                   R1-2302524

e-Meeting, April 17th – April 26th, 2023

Source:        BUPT

Title:           Discussions on Carrier Phase Measurement for NR Positioning

Agenda item:    9.5.2

Document for:	Discussion

Introduction

At the RAN #94 meeting, representatives unanimously agreed to expand and improve the SID research of NR positioning RP-213588 [1], including the following goals related to carrier phase measurements:

- Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

At the RAN1 #112 meeting, attendees reached a series of agreements. In this article, we will elaborate on some viewpoints regarding carrier phase measurements.

At the RAN #112 meeting, some issues related to carrier phase measurements and positioning were discussed, and a series of consensus were reached. The following achievements were made [2]:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP

In this article, we provide our perspectives on the definition of CP.

Carrier Phase

## 2.1 Downlink Carrier Phase

Carrier phase measurement refers to the phase of the base station signal received relative to the carrier phase generated in the receiving device at the time of reception. It represents a measurement of the distance between the base station and the receiving device in units of carrier frequency cycles. Therefore, by offsetting the signal carrier phase generated and matching it with the carrier phase received from the base station, observable values can be obtained.

Assuming there is no error in an ideal scenario, the measured PRS phase φUETRPi-prs follows the following expression:

φUETRPi-prs=φUEtR-φTRPi-prstR+NUETRPi-prs#1

where φUE is the phase of the receiving device, φTRPi-prs is the phase of the PRS signal received from base station i, NUETRPi-prs is the integer ambiguity between base station i and the receiving device, and t_R represents the time of reception. Considering the error factors in practical scenarios, the measured phase ϕUETRPi-prs can be expressed as:

ϕUETRPi-prs=φUETRPi-prs+δi-FO+δi-RPE+δi-ARP+δi-PCO+δi-mp#2

where δi-FO represents the carrier frequency offset of base station i, δi-RPE represents the random initial phase error, δi-ARP represents the antenna reference point (ARP) position error, δi-PCO represents the phase center offset, and δi-mp represents the multipath error between base station i and the receiving device.

Proposal 1: The definition of DL CP can be represented by Equation 2.

The downlink carrier phase difference is obtained from the downlink carrier phases from base stations i and j, respectively.

CDOAUETRPij-prs=ϕUETRPi-prs-ϕUETRPj-prs#3

Proposal 2: The definition of DL CPD can be represented by Equation 3.

## 2.2 Uplink Carrier Phase

The uplink carrier phase is obtained through TRP measurement using UL SRS for positioning or MIMO SRS from the UE. In an ideal scenario, the measured SRS phase φUEsrsTRPi follows the following expression:

φUEsrsTRPi=φUEsrstR-φTRPitR+NUEsrsTRPi#4

The variable φUEsrs represents the phase of the SRS signal received by base station i, while φTRPi represents the phase of base station i. NUEsrsTRPi represents the ambiguity between base station i and the receiving device. t_R represents the receiving time. Taking into account measurement errors in practical situations, the measured phase ϕUEsrsTRPi can be expressed as:

ϕUEsrsTRPi=φUEsrsTRPi+δi-FO+δi-RPE+δi-ARP+δi-PCO+δi-mp#5

Proposal 3: The definition of the UL CP can be expressed by Equation 5.

Conclusion

Proposal 1: The definition of DL CP can be represented by Equation 2.

Proposal 2: The definition of DL CPD can be represented by Equation 3.

Proposal 3: The definition of the UL CP can be expressed by Equation 5.

Reference

- RP-213588 Revised SID on Study on expanded and improved NR positioning, RAN#94e.

- Chair’s Notes RAN1#112 v20, February 27th – March 3rd, 2023.