3GPP TSG RAN WG1 #112bis-e		R1-2302525

e-Meeting, April 17th – April 26th, 2023

Source:	OPPO

Title:	Discussion on comparison of SL-RSRP and SD-RSRP measurements

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

A LS from RAN2 in [1] is sent to RAN1 and RAN4 and asking whether it can be supported for the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report.

During the work on session continuity enhancements within NR_SL_relay_enh work item a new measurement report trigger has been proposed yet with no consensus in RAN2: Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE This new event requires either the comparison of SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages) measurements from different UEs, or the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements. During the discussion some companies expressed concerns on direct comparison of SD-RSRP and SL-RSRP due to the power control mechanism applied on PC5 unicast links, while other companies think the network can handle such comparison by using an appropriate offset in the measurement event. Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements: Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

This contribution discusses the question above from RAN1’s perspective, and the corresponding proposal is provided based on the analysis. The draft LS reply can be found in [2].

Discussion

From RAN1’s perspective, it is not suitable to compare directly of SL-RSRP and SD-RSRP measurement be used for purpose of triggering a measurement report.

In NR Sidelink unicast, SL-RSRP is measured by Rx UE and reported to Tx UE which applies the measurements for Sidelink open-loop power control. From the Tx UE’s perspective, transmission power is dynamically adjusted based on DL pathloss and/or SL pathloss based on the reported measurement from Rx UE. In NR Sidelink broadcast, OLPC is based on DL pathloss. Sidelink discovery messages are transmitted over broadcast, which means that Tx UE may adjust its transmission power for discovery messages based on DL pathloss. Therefore, it is not suitable to directly compare SD-RSRP measurement and SL-RSRP measurement, since transmission powers may not be the same.

On the other hand, comparison of SD-RSRP measurements from different relay UEs can be considered, because transmission power over broadcast for discovery messages are aligned. There is not necessary to additionally introduced a comparison of SD-RSRP and SL-RSRP measurement.

From RAN1’s perspective, it is not supported to compare directly of SL-RSRP and SD-RSRP measurement be used for purpose of triggering a measurement report.

Proposal 1: From RAN1’s perspective, it is not supported for the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report.

Conclusion

In this contribution, whether it is suitable to compare SL-RSRP and SD-RSRP measurement to be used of triggering a measurement report is discussed and analyzed from RAN1’s perspective. The corresponding proposed reply on the LS can be as follows:

Proposal 1: From RAN1’s perspective, it is not supported for the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report.

Reference

- R1-2302280 (R2-2302234), LS on Comparison of SL-RSRP and SD-RSRP measurements, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2302526, Draft reply LS on comparison of SL-RSRP and SD-RSRP measurements, RAN1#112bis-e, e-meeting, April 17th – April 26th, 2023.