3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302528

e-Meeting, April 17th - April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | - | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CWS adjustment for channels without explicit HARQ feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 4.1.4.2, regarding CWS adjustment for channels without explicit HARQ feedback, the gNB “using the latest CWp used for any DL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p”, so this procedure can be interpreted as: the gNB uses the same CWS that latest used for DL transmission using Type 1 channel access and using the same CAPC. However, in RAN1 #99 meeting, the following agreement was made: Agreement: Channels without explicit feedback use the CWS last updated by channels with explicit feedback and using the same CAPC if such channels exist; otherwise they use the minimum CWS corresponding to the CAPC. According to the agreement, the above procedure should be: the gNB uses the same CWS that latest updated by DL transmission associated with explicit HARQ feedback and using the same CAPC. Since CWS for all the priority classes are adjusted together once the explicit HARQ feedbacks for any DL transmission are received, the above two procedures are different. This draft CR is trying to correct this procedure according to the agreement to avoid misunderstanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct CWS adjustment procedure for channels without explicit HARQ feedback according to agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect CWS adjustment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.1.4.2	Contention window adjustment procedures for DL transmissions by gNB

<Unchanged parts are omitted>

If a gNB transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit HARQ-ACK feedbacks by the corresponding UE(s), the gNB adjusts CWp before step 1 in the procedures described in clause 4.1.1, using the latest CWp associated with the channel access priority class p updated by any DL transmissions on the channel associated with explicit HARQ-ACK feedbacks using Type 1 channel access proceduresp. If the corresponding channel access priority class p  has not been used for any DL transmissions on the channel, CWp=CWmin,p is used.

<Unchanged parts are omitted>