3GPP TSG RAN WG1 #112bis-e	 	                                               R1-2302532

e-meeting, April 17th – 26th, 2023

Source:	OPPO

Title:	Unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion

Introduction

The WID [1] of MIMO evolution for downlink and uplink was agreed in RAN#94e meeting. According to the arrangement, the objectives related to this agenda item are collected and highlighted as below

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we present our view on the aspects of a) extending unified TCI state for multi-TRP operation, b) UL beam indication for simultaneous multi-panel transmission (STxMP), c) power control for sDCI based multi-TRP operation and d) other aspects for multi-TRP operation with low priority.

Unified approach for extending unified TCI state for mTRP

From Rel.16 to Rel.17, the multi-TRP transmission schemes have been studied and continuously specified. In Rel.18, STxMP in UL was supported with validated performance benefits and PDSCH CJT in DL was supported at FR1 with up to 2 indicated DL/joint TCI states.

In a short summary, these transmission schemes can be listed for DL and UL separately as below and illustration can be found in Figure 1.

- DL transmission

  - Rel.16 mTRP PDSCH (sDCI-mPDSCH, mDCI-mPDSCH, 1a/2a/2b/3/4)

  - Rel.17 inter-cell multi-TRP (mDCI-mPDSCH)

  - Rel.17 mTRP PDCCH repetition

  - Rel.17 SFN PDCCH and SFN PDSCH (for HST)

  - Rel.18 PDSCH CJT at FR1

- UL transmission

  - Rel.17 mTRP PUCCH repetition

  - Rel.17 mTRP PUSCH repetition

  - Rel.18 simultaneous transmission of multi-panel (STxMP)

- : MTRP operation with unified TCI framework extension where two PDCCHs (including solid line and dashed line) and single PDCCH (including only solid line) stands for M-DCI and S-DCI MTRP, respectively

- Specify the extended unified TCI framework to MTRP for beam indication/update with high priority.

Foundation of extending unified TCI state framework

Regarding on whether the RRC parameter CORESETPoolIndex is presented or absent, the difference between S-DCI and M-DCI based MTRP operation can be identified. Given the current available solutions for both operations, it seems unified solutions for S-DCI and M-DCI MTRP cannot be easily achieved.

- Unified solution for S-DCI and M-DCI MTRP beam indication cannot be easily achieved.

For MTRP operation, up to 2 TRPs transmission are supported until Rel.17. For Rel.17 STRP operation, unified TCI state should be shared among DL channels/signals (UE-dedicated PDCCH/PDSCH and AP-CSI-RS) and UL channels/signals (PUCCH/PUSCH/SRS (except SRS for aperiodic beam management purpose)). For MTRP operation, each unified TCI state should be shared within its associated TRP.

Consider that two types of unified TCI state defined in Rel.17, i.e. joint TCI state and separate DL/UL TCI state. Each TRP can be associated with a set of unified TCI state(s). The set of unified TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states. To differentiate two TRPs in specification, beam indication can be carried by the 1st set of indicated TCI state(s) and the 2nd set of indicated TCI states. In our view, that’s the foundation for the extension of unified TCI framework to MTRP.

- Support the 1st set of indicated TCI state(s) for one TRP and the 2nd set of indicated TCI state(s) for the other TRP

- One set of indicated TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states

TCI state(s) activation and update

Maximum number of indicated TCI state(s)

In RAN1#110, the following agreement on maximum number of indicated unified TCI states was reached. But it lacks of clarity on which types of TCI states it refers to.

Agreement

On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation

- FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP

- Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1

- Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17

Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI

For the maximum number of indicated TCI states, i.e. M (for DL) and/or N (for UL), it is natural to align with single beam operation of STRP, i.e. limiting M<=2 and/or N<=2. Each TRP can be indicated with either 2 separate DL/UL TCI states (1 for DL and 1 for UL) or 1 joint TCI state. In our understanding, in above-mentioned agreement, up to 4 TCI states refer to separate DL/UL states for two TRPs. If the TCI states indicated are joint TCI states, then the maximum number should be up to 2 accordingly. We have to note that this applies to the target use cases agreed in RAN1#109-e in AI 9.1.1.1 including STxMP. As for CJT, there was a separate discussion to support up to 2 DL/joint TCI state(s) for time/freq. domain QCL indication at FR1.

- On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, support up to 2 joint TCI states and 4 separate DL/UL TCI states per codepoint.

Signaling medium for S-DCI MTRP

In RAN1#109e, one agreement for at least S-DCI MTRP was achieved as below.

Agreement @ 109e

On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list

- FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint

- FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints

- FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits

- Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded

Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

From signaling perspective, there could be up to 4 separate DL/UL TCI states or 2 joint TCI states to be carried by MAC CE and/or DCI. Then the total number of TCI states per TCI codepoint reaches up to 4 for separate DL/UL TCI states and up to 2 for joint TCI states. In addition, given the conclusion made in RAN1#110bis-e that only joint or separate TCI states can be configured per serving cell. Correspondingly, MAC CE should be extended to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

Conclusion

On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell.

- For S-DCI MTRP, enhance MAC CE to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

Meanwhile, when NW intends to partially update the indicated TCI states for one particular TRP, e.g. one out of two joint TCI states updated, the MAC CE to be enhanced should clearly indicate which joint/DL/UL TCI state(s) should be updated, either 1st set or 2nd set of TCI state(s). The detailed signaling design can be up to RAN2.

- For S-DCI MTRP, MAC CE should clearly indicate which part of separate DL/UL TCI state(s) or joint TCI state(s) (i.e. 1st set, 2nd set or both sets of TCI states) would be updated.

Recall the MAC CE design for S-DCI based M-PDSCH in Rel.16, there are up to 8 codepoints and each codepoint may include one or two legacy TCI state(s). In Rel.18, at least for joint TCI states, there seems no strong motivation to increase the maximum number of TCI codepoints. As for separate DL/UL TCI state(s), since the indicated TCI states can be partially updated, it seems a flexible mechanism to combined indicated TCI states, if NW indicates separate DL/UL TCI state properly.

- For both joint and separate DL/UL TCI states, support up to 8 codepoints for TCI state activation (same as in legacy MAC CE).

TCI state updating mechanism

In Rel.15/16, beam indication for PDSCH (either STRP or MTRP) can be based on dynamic TCI state indication in DL DCI. The indicated TCI state(s) is (are) applicable for the scheduled PDSCH on its occasion(s). The indicated TCI state will not last for the upcoming slots. Different from the legacy TCI indication, the unified TCI state in Rel.17 will only be applicable after beam application time (BAT). The indicated unified TCI state will last for an uncertain period until next update from NW.

For S-DCI, one set or two sets of unified TCI state(s) can be indicated to update previously applicable one(s). For the case (as shown in Figure 2) when one set of TCI state is indicated/updated and previously two sets of indicated TCI states are applicable, the issue merging up is whether UE should maintain the other set of TCI state(s) to facilitate other MTRP operation as configured. If yes, other MTRP operation, e.g. PUCCH repetition, could be maintained. Otherwise, all the channel(s) even configured for MTRP operation should fall back to STRP operation. If the latter is the case, it seems the dynamic indicated TCI state(s) conflicting with RRC configured MTRP/STRP operation.

- : DCI partially updating the set of unified TCI state(s)

- When unified TCI states are partially updated, UE should update the indicated TCI state(s) and maintain the TCI state(s) not in the set of updated TCI state(s).

In previous meetings, the following FL proposal has been widely discussed and slightly revised as final version in FL summary. It seems aligned well with our corresponding proposals. Hence, from our point of view, we would like to have following proposal. That’s the essential part of unified TCI state indication for S-DCI MTRP. Without consensus on it, the UTCI extension for S-DCI MTRP would be incomplete feature.

Proposal 2.1: On unified TCI framework extension, support the followings when S-DCI based MTRP operation is configured:

- For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {1st joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

  - If the UE receives a TCI codepoint mapped with a full-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI states in the full-set

  - If the UE receives a TCI codepoint mapped with a sub-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI state in the sub-set, and keep current joint TCI state not in the sub-set

  - TCI state activation command (MAC-CE) should indicate that each activated joint TCI state in the TCI state activation command is mapped to the 1st or 2nd joint TCI state of a TCI codepoint, and how to indicate is up to RAN2 design

  - Note: The 1st joint TCI sate corresponds to one TRP, and the 2nd joint TCI state corresponds to another TRP

- For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

  - If the UE receives a TCI codepoint mapped with a full-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI states in the full-set.

  - If the UE receives a TCI codepoint mapped with a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI state(s) in the sub-set, and keep current DL/UL TCI state(s) not in the sub-set

  - TCI state activation command (MAC-CE) should indicate that each activated DL/UL TCI state in the TCI state activation command is mapped to the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state of a TCI codepoint, and how to indicate is up to RAN2 design

  - Note: The 1st DL and UL TCI sates correspond to one TRP, and the 2nd DL and UL TCI states correspond to another TRP

[FFS: How to determine that one indicated joint/DL/UL TCI state or two indicated joint/DL/UL states should be maintained by the UE, e.g., based on RRC configuration or MAC-CE TCI state activation?]

- On unified TCI state extension for S-DCI MTRP, support the FL Proposal 2.1 to fully enable unified TCI state indication.

Association between M-DCI MTRP and indicated TCI state(s)

In RAN1#110bis-e and RAN1#111, the following agreements on TCI state indication for PDCCH and scheduled or activated PDSCH/PUSCH was achieved respectively.

Agreement

On unified TCI framework extension for M-DCI based MTRP:

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value

Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state

FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework

Agreement

On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

Next, we discuss other channel(s)/signal(s) depending on whether the key factor of M-DCI MTRP, i.e. CORESETPoolIndex can be associated.

Channel(s)/signal(s) associated CORESETPoolIndex

In RAN1#112, the following agreement on PUCCH was achieved for further down selection.

Agreement

On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission:

- Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

- Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted.

  - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint

- Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted.

Note: Either Opt1 or Opt2 must be supported

In our understanding, Opt.1 is to configure TRP-specific PUCCH resource/resource group via existing CORESETPoolIndex, whereas Opt.2 is to associate 1st or 2nd indicated UL/joint TCI state with CORESETPoolIndex by using a new RRC parameter. The main difference lies in whether the DL related parameter CORESETPoolIndex can be directly used for UL channel(s). As in agenda item 9.1.4.1, RAN1 agreed that CORESETPoolIndex can be associated with SRS resource set. Then we don’t think it’s necessary to introduce a new RRC parameter for the same purpose.

- Within PUCCH resource/resource group of M-DCI MTRP, at least support to include the existing CORESETPoolIndex (Opt1), rather than introducing a new RRC parameter for the same purpose (Opt2).

However, in legacy release, PUCCH resource/resource group are not explicitly associated with any specific TRP in order to main flexibility. For PUCCH carrying separate HARQ-ACK for scheduled PDSCH, the flexible way to use it is not to pre-configure the CORESETPoolIndex or an new RRC parameter. The UL/joint TCI state can be selected by according to the CORESETPoolIndex from which the corresponding PDCCH is transmitted.

- For PUCCH resource/resource group of M-DCI MTRP, additionally support Opt.3 to maintain the flexibility of PUCCH.

For AP CSI-RS for CSI/BM when qcl-Info is not configured per resource, there should be a rule to determine which indicated DL/joint TCI state should be applied for it. Consider the PDCCH triggered DL RS, the UE should apply the indicated DL/joint TCI state corresponding to a CORESETPoolIndex from which the AP CSI-RS resource set is triggered.

- For AP CSI-RS of M-DCI MTRP, apply the indicated DL/joint TCI state specific to a CORESETPoolIndex for the triggered AP CSI-RS by PDCCH from the same CORESETPoolIndex.

For SRS for CB/NCB, we notice that in agenda item 9.1.4.1, the following agreement was made in RAN1#111. For M-DCI based STxMP, each SRS resource set is associated with a CORESETPoolIndex. Specifically, one following-up agreement confirms that the SRS resource set with lower ID is associated with CORESETPoolIndex #0. Hence, regardless STxMP, if the SRS resource set configured to follow unified TCI state, the UE should apply the indicated UL/joint TCI state specific to a CORESETPoolIndex to the associated SRS resource set.

Agreement @ RAN1#111

- For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission,

  - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1

    - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex

  - FFS: Which is the first SRS resource set, e.g., the set with lower set ID.

For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

- For CB/NCB SRS of M-DCI MTRP, apply the indicated UL/joint TCI state specific to a CORESETPoolIndex to the SRS resource set associated with the same CORESETPoolIndex.

Other channel(s)/signal(s) not associated CORESETPoolIndex

For channel(s)/signal(s), some of them cannot be associated with a CORESETPoolIndex. For instance, Type1 SPS PDSCH and Type 1 CG PUSCH are purely based on RRC configuration. For P/SP CSI-RS and AS/BM SRS, the signaling relies on RRC configuration and/or MAC CE activation, hence not related to any PDCCH.

Under such circumstances, the RRC configuration can be a good solution. For instance, NW could configure the above-mentioned channel(s)/signal(s) to follow either the 1st set or the 2nd set of indicated TCI state(s).

- For channel(s)/signal(s) those are not associated with a CORESETPoolIndex, use RRC signaling to configure to apply the 1st set or 2nd set of indicated TCI state(s).

Association between S-DCI MTRP and indicated TCI state(s)

Since we have defined the 1st set and 2nd set of indicated TCI states, next it’s necessary to associate/map the sets of indicated TCI states with/to DL or UL channels. Let’s next present our view per each channel when considering previous agreements.

PDCCH

Consider the case that when S-DCI schedules M-PDSCH or M-PUSCH. Two sets of TCI states in the S-DCI are indicated for two PDSCHs or associated with two PUSCHs. Then which one of the two sets of indicated TCI states would be applied to PDCCH itself seems unclear.

In RAN1#110bis-e, the agreement for down selection on TCI state indication for PDCCH was made. RAN1 agreed to adopt RRC signaling to indicate UE’s behavior on PDCCH reception. Specifically, for one specific CORESET, NW could configure the indicated TCI state(s) to be applied, i.e. the 1st one, 2nd one, both or none.

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following:

- Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

One remaining issue to be handle is whether to apply such RRC configuration on a per CORESET basis or per CORESET group. For S-DCI MTRP, one clear boundary from M-DCI MTRP is whether the implicit downlink “TRP ID”, i.e. CORESETPoolIndex can be configured or not. If CORESETs can be grouped for S-DCI MTRP, we then see no major difference from M-DCI MTRP. Moreover, configuration a group of CORESETs with the same indicated TCI state(s) may lack of flexibility but provide no additional benefits.

- For PDCCH of S-DCI MTRP, RRC informs which TCI state(s) to apply per CORESET, rather than per CORESET group.

PDSCH

In RAN1#112, the following agreement was achieved to specify a new indicator field, i.e. TCI selection field, in DCI format 1_1/1_2 to dynamically select the 1st, 2nd or both indicated unified TCI state(s) for scheduled PDSCH.

Agreement

On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings:

- If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2

- FFS: Whether and how to use the codepoint "11" of the [TCI selection field]

If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2.

- Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

- FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2

FFS: Detail of the capability of two default beams for S-DCI based MTRP

FFS: The threshold value

The legacy feature of dynamic switching between STRP/MTRP PDSCH could be facilitated given the codepoint design of the TCI selection field. However, the same issue of RF retuning for beam change as in legacy TCI framework has also been introduced as well.

Consider the case as depicted in Figure 3, the UE that doesn’t support two default beams may need a period of time to prepare its Rx beam for PDSCH reception, since the UE doesn’t know which indicated DL/joint TCI state(s) would be applied before decoding the TCI selection field of scheduling DCI. If the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold, there should be a need to specify default beam for PDSCH reception. In other words, even UE knows the two indicated TCI states prior to PDSCH transmission, it still may need some time to dynamically switch from one to another or to two both. Analogously, the period could be as long as the UE capability on DL scheduling gap, i.e. TimeDurationForQCL, with 7/14/28 OFDM symbols to prepare PDSCH reception.

- : TCI selection gap between scheduling DCI and scheduled PDSCH

- For S-DCI MTRP PDSCH, introduce new or reuse UE capability on scheduling gap when UE cannot be enabled with two default beams for PDSCH reception at FR2.

Moreover, when the scheduling gap is smaller than the threshold, default beam(s) might be needed. Consider the difference between unified TCI state and legacy TCI state, it seems not necessary to reuse the complicated Rel.15/16 rule. In RAN1#112, the following agreement was made to address the issue when the TCI selection field is not presented in DCI format 1_1/1_2.

Specifically, Alt.1 is to use RRC signaling to control UE’s behavior on PDSCH reception. It could be simple, by pre-configuring the 1st, the 2nd, or both indicated DL/joint TCI state(s). Alt.2 and Alt.3 is via fixed rule(s) to apply either one out of two or both of indicated DL/joint TCI states.

As for Alt.3A, PDSCH follows the indicated DL/joint TCI state of PDCCH. However, it could be problematic that different CORESETs may have different DL/joint TCI states, e.g. some of CORESETs configured to not follow the indicated DL/joint TCI states whereas others configured to follow it. In this sense, depending on to which CORESET the scheduling DCI belongs, the Rx beam for PDSCH may also change dynamically. For Alt.4, it seems not default beam as the most recently applied DL/joint TCI state(s) may also dynamically change.

Agreement

On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]:

- Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception

- Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception

- Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI

Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

- For S-DCI MTRP PDSCH, support either Alt.1 (RRC configured) or Alt.2 (1st indicated TCI state)/Alt3 (both indicated TCI states) as the default beam for the cases when TCI selection field is absent in DCI format 1_1/1_2 or the scheduling gap is smaller than a threshold if necessary.

PUSCH

In Rel.17, the TDM-based PUSCH repetition toward different TRPs was specified to enhance UL data reliability. Specifically, two SRS resource sets can be signaled with different orders for MTRP UL transmission. The association between TRPs and UL scheduling information (e.g. TPMIs, SRIs) is signaled to UE as well. STRP transmission can also be dynamically switched ON by indicating one SRS resource set. The beam indication for PUSCH repetition is based on legacy spatial relation information.

In RAN1#112, the following agreement was achieved on UTCI extension for S-DCI MTRP PUSCH. By reusing the codepoints, i.e. SRS resource set indicator, the association between indicator UL/joint TCI state and PUSCH transmission occasion(s)/antenna port(s) can be established.

Agreement

On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator:

  - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214)

  - FFS: SDM and SFN based PUSCH Tx schemes

FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

One remaining issue lies in the last codepoint design, i.e. “11”, for SDM and SFN based PUSCH Tx schemes. As for both SDM and SFN, there is no sequential order in time domain as TDM based PUSCH repetition. Codepoint “10” can be used to indicator two UL/joint TCI states, whereas codepoint “11” could be reserved, rather than introducing additional complexity at UE side.

- For S-DCI STxMP (e.g. SDM and SFN), the last codepoint “11” can be reserved.

Another timeline issue as listed in the last FFS already existed in Rel.17 STRP based on UTCI. It is up to NW to update the UL/joint TCI state(s) before actual SRS transmission. In such case, PUSCH antenna port(s) and SRS antenna port(s) can be always with the same Tx beam (i.e. the same indicated UL/joint TCI state(s)).

One may argue that this implementation may bring additional constraint at NW side. But we would also like to mention that SRS resource set could even be configured not to follow the indicated UL/joint TCI state which should apply to PUSCH. Hence there could always be a way to diverge the UL Tx beam of SRS and PUSCH if NW would like to do so.

- For S-DCI MTRP PUSCH, it is possible to apply the same indicated UL/joint TCI state(s) for PUSCH antenna port(s) and SRS port(s) via implementation.

AP CSI-RS

For AP CSI-RS triggered by S-DCI, there is no CORESETPoolIndex value to associate one CSI-RS resource set to a specific TRP. Hence a simple solution could be based on RRC configuration. Specifically, each AP CSI-RS resource set can be configured to follow either the 1st or the 2nd indicated DL/joint TCI state. For the case when there is only one indicated DL/joint TCI state applicable, the UE may apply its maintained DL/joint TCI state to the corresponding AP CSI-RS resource set.

- For S-DCI MTRP, AP CSI-RS can be configured to follow either the 1st or the 2nd indicated DL/joint TCI state.

AP SRS

Similar issue of AP CSI-RS also exists for AP SRS when S-DCI MTRP operates. But solution could be somehow different. Instead of RRC configuration on which indicated UL/joint TCI state to follow for each SRS resource set, even more simple rule could be used. Allow us to borrow the concept of the 1st or 2nd SRS resource set created in AI 9.1.4.1 for M-DCI MTRP. The SRS resource set ordering can be reused for S-DCI as well. That is the 1st SRS resource set (with lower ID) and the 2nd SRS resource set (with higher ID) are to follow the 1st and the 2nd indicated UL/joint TCI state respectively. In this setting, it seems not necessary to invoke higher layer signaling.

- For S-DCI MTRP, the 1st and the 2nd AP SRS resource set are to follow the 1st and 2nd indicated UL/joint TCI state respective.

PDSCH-CJT

The indicated TCI state(s) for PDSCH-CJT

As for CJT MTRP operation, there could be up to 4 TRPs simultaneously transmitting PDSCH to UE at FR1. TCI states carries DL RS (typically TRS) mainly for QCL-Type1 indication, i.e. for time/frequency tracking. To simply UE’s measurement/tracking on TRS, NW can send the same TRS using all the involved TRPs. And it can be transparent to UE which TRPs are sending the TRS. This operation could be viewed as UE-specific SFN TRS and facilitate QCL assumption for CJT operation.

On the other hand, to balance the flexibility of TRS configuration and UE reception complexity, one compromised agreement to support up to 2 indicated joint TCI states (with respective to UE capability) was achieved as below.

Agreement

On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode

- Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability

- FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT

- Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1

In RAN1#111, the following 1st agreement says that PDSCH-CJT can be treated as one of the S-DCI based MTRP schemes. The 2nd agreement continues by providing alternatives on the conveyed QCL type information.

Agreement

On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme

Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework

Agreement

On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

In our understanding, the involved multiple TRPs for PDSCH-CJT normally located with similar distance to a UE, therefore the average delays of DL Tx to the UE are in the same range, i.e. smaller than a CP at FR1. If that’s the common case, there is no strong motivation adjust DL Tx timing per each TRP for a specific UE. As for the frequency domain, we note that in Rel.17 HST-SFN, UE can drop the Doppler-related QCL parameters, but that’s for the high-speed train scenario. For PDSCH-CJT, it seems UEs moving with high velocity are not commonly assumed.

- For PDSCH-CJT, there seems no strong motivation to adopt frequency-domain and/or time-domain pre-compensation at TRP side, and corresponding UE behavior.

With above being said, we didn’t identify any strong reason to modify the UE behavior on existing QCL rule, i.e. reusing QCL-TypeA for PDSCH-CJT at FR1.

- When 1 or 2 joint TCI state(s) indicated for PDSCH-CJT at FR1, the PDSCH DMRS port(s) is QCLed with respect to QCL-TypeA only (Alt1).

Unified TCI state for UL power control

In Rel.17, the UL power control parameters, i.e. PL-RS, and the set of alpha, P0 and CLI, can be optionally associated with UL/joint TCI state. This association can be considered as beam-level power control, which can be simply extended from STRP to MTRP by reusing similar approach as in Rel.17 unified TCI state design but on a per TRP basis. In RAN #109e, the following agreement was reached to associate the indicated UL/joint TCI state with UL PC parameters.

Agreement

On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion.

- FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported

- FFS: UL PC enhancement for CB and non-CB SRS in above case

FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

In Rel.17 for MTRP PUSCH repetition, two SRS resource sets can be mapped to two SRIs in UL scheduling DCI. The SRI field in DCI format 0_1/0_2 can dynamically indicate the UL PC parameters. The SRS resource set(s) can also be associated with UL/joint TCI state(s) to determine the indicated/updated UL beam(s) for transmission. Hence, the power control association chains can be descripted as in Figure 4 where the UL PC parameter set associated with UL/joint TCI state could be different from the one indicated by SRI. Since at each end of this chain, the UL PC parameter set is optionally managed (configured/activated) by NW. This potential power control parameter collision can be handled by NW via implementation.

- : UL power control parameter set chains

- Potential UL PC parameter set collision (one set from associated UL/joint TCI state and another set from indicated SRI) can be and should be addressed by NW via implementation.

It is one possible configuration that one (but not both) indicated UL/joint TCI state(s) is not associated with UL PC parameters for PUCCH/PUSCH, and the other indicated UL/joint TCI state is associated with UL PC parameter set for the other TRP. For the one not associated with UL PC parameter set, the default UL PC parameter set defined in Rel.17 can be reused, but this default one is on a per channel basis, rather than per TRP.

For the case when both indicated UL/joint TCI states are not associated with UL PC for PUCCH/PUSCH, apparently the only one UL PC parameter set cannot properly serve transmission toward to two TRPs. It seems natural to have two default UL PC parameter sets for two TRPs.

Moreover, for PUSCH 1 or 2 UL PC parameter set(s) can be associated with the indicated SRS resource set(s) with usage of CB/NCB. However, for PUCCH, the Rel.15/16 UL PC parameter configured in PUCCH-SpatialRelationInfo cannot be used, since spatial relation is not allowed to be co-existed with UL/joint TCI state in the same serving cell.

- For the cases when 1 or 2 indicated UL/joint TCI state(s) is (are) not associated with UL PC parameter set(s), support two default UL PC parameter sets.

BFR enhancement using unified TCI states

In Rel.17, when unified TCI states configured and activated, the procedure of beam failure recovery can recover DL and UL new beam for PDCCH/PDSCH/AP-CSI-RS and PUCCH/PUSCH/SRS respectively. That results in the unified new beam for DL and/or UL. Similarly, for extending unified TCI states to MTRP, the similar UE behavior on beam setting can be studied and investigated as well. But given current status, this issue can be with low priority.

In RAN1#110bis-e, the following agreement was achieved for TRP-specific BFR using unified TCI state

Agreement

On unified TCI framework extension, study the following enhancements for TRP-specific BFR:

- Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP

- Enhancement to beam update after NW response to TRP-specific BFR request

- When unified TCI state applied, support to extend the Rel.17 beam reset/update rule for TRP-specific BFR.

Conclusion

In this section, allow us to repeat our proposals and observations

- Specify the extended unified TCI framework to MTRP for beam indication/update with high priority.

- Support the 1st set of indicated TCI state(s) for one TRP and the 2nd set of indicated TCI state(s) for the other TRP

- One set of indicated TCI state(s) can be either 1 joint TCI state or 2 separate DL/UL TCI states

- On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, support up to 2 joint TCI states and 4 separate DL/UL TCI states per codepoint.

- For S-DCI MTRP, enhance MAC CE to facilitate up to 4 separate DL/UL TCI states or 2 joint TCI states per codepoint.

- For S-DCI MTRP, MAC CE should clearly indicate which part of separate DL/UL TCI state(s) or joint TCI state(s) (i.e. 1st set, 2nd set or both sets of TCI states) would be updated.

- For both joint and separate DL/UL TCI states, support up to 8 codepoints for TCI state activation (same as in legacy MAC CE).

- When unified TCI states are partially updated, UE should update the indicated TCI state(s) and maintain the TCI state(s) not in the set of updated TCI state(s).

- On unified TCI state extension for S-DCI MTRP, support the FL Proposal 2.1 to fully enable unified TCI state indication.

- Within PUCCH resource/resource group of M-DCI MTRP, at least support to include the existing CORESETPoolIndex (Opt1), rather than introducing a new RRC parameter for the same purpose (Opt2).

- For PUCCH resource/resource group of M-DCI MTRP, additionally support Opt.3 to maintain the flexibility of PUCCH.

- For AP CSI-RS of M-DCI MTRP, apply the indicated DL/joint TCI state specific to a CORESETPoolIndex for the triggered AP CSI-RS by PDCCH from the same CORESETPoolIndex.

- For CB/NCB SRS of M-DCI MTRP, apply the indicated UL/joint TCI state specific to a CORESETPoolIndex to the SRS resource set associated with the same CORESETPoolIndex.

- For channel(s)/signal(s) those are not associated with a CORESETPoolIndex, use RRC signaling to configure to apply the 1st set or 2nd set of indicated TCI state(s).

- For PDCCH of S-DCI MTRP, RRC informs which TCI state(s) to apply per CORESET, rather than per CORESET group.

- For S-DCI MTRP PDSCH, introduce new or reuse UE capability on scheduling gap when UE cannot be enabled with two default beams for PDSCH reception at FR2.

- For S-DCI MTRP PDSCH, support either Alt.1 (RRC configured) or Alt.2 (1st indicated TCI state)/Alt3 (both indicated TCI states) as the default beam for the cases when TCI selection field is absent in DCI format 1_1/1_2 or the scheduling gap is smaller than a threshold if necessary.

- For S-DCI STxMP (e.g. SDM and SFN), the last codepoint “11” can be reserved.

- For S-DCI MTRP, AP CSI-RS can be configured to follow either the 1st or the 2nd indicated DL/joint TCI state.

- For S-DCI MTRP, the 1st and the 2nd AP SRS resource set are to follow the 1st and 2nd indicated UL/joint TCI state respective.

- When 1 or 2 joint TCI state(s) indicated for PDSCH-CJT at FR1, the PDSCH DMRS port(s) is QCLed with respect to QCL-TypeA only (Alt1).

- For the cases when 1 or 2 indicated UL/joint TCI state(s) is (are) not associated with UL PC parameter set(s), support two default UL PC parameter sets.

- When unified TCI state applied, support to extend the Rel.17 beam reset/update rule for TRP-specific BFR.

- Unified solution for S-DCI and M-DCI MTRP beam indication cannot be easily achieved.

- For S-DCI MTRP PUSCH, it is possible to apply the same indicated UL/joint TCI state(s) for PUSCH antenna port(s) and SRS port(s) via implementation.

- For PDSCH-CJT, there seems no strong motivation to adopt frequency-domain and/or time-domain pre-compensation at TRP side, and corresponding UE behavior.

- Potential UL PC parameter set collision (one set from associated UL/joint TCI state and another set from indicated SRI) can be and should be addressed by NW via implementation.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94e, Dec. 2021