3GPP TSG-RAN WG1 Meeting #112bis-e                                                 	 R1-2302540

e-meeting, April 17th- 26th, 2023

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1 #112, some agreements on the evaluation on AI/ML based CSI feedback enhancement have been made as follows [1]. In this contribution, we present our views on various aspects, including evaluation methodology, KPI and some preliminary results on AI/ML for CSI feedback enhancement based on [1][2].

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Agreement

Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

Conclusion

For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Agreement

- For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

    - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, ect.

Agreement

For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison.

- FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

  - FFS: input/output type

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is

    - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

    - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is

    - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

    - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

- Other options not precluded.

Agreement

The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

- For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank.

- For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB

  - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank

Discussion on CSI compression

Quantization issue

As agreed in RAN 1#111 the quantization issue of CSI feedback for CSI compression sub use case should be studied. From our understanding, a basic principle is the quantization/dequantization method should be aligned between UE and NW.  According to whether the quantization/dequantization function is involved in the AI model training phase, Case 1 and Case 2 are agreed for quantization non-aware training and quantization aware training, respectively. Moreover, Case 2-1 and Case 2-2 are further defined based on whether the quantization method/parameters are updated together during the AI model training.

From our understanding, different training types on the use of quantization schemes should also be considered. For training type 1, it is quantization non-aware and the alignment of quantization modules can be solved through implementation. For training type 2 and type 3, they are quantization aware training, especially when quantization modules are involved in the CSI model training process, it may be necessary to specify the quantization methods to ensure the encoder and encoder to be well trained and could work together.

Proposal 1: For training collaborative Type3, study whether/how to align the quantization and dequantization method between UE and NW

Generalization

As for generalization issue, the input/output CSI dimension keeps the same with different configuration(s)/scenario(s) for both training and inference stages. Therefore, the AI/ML model trained on one dataset with Scenario#A/Configuration#A or mixing dataset from multiple scenarios/configuraions can be directly inferenced/tested on dataset from Scenario#A/Configuration#B or Scenario#B/Configuraion#B.

Firstly, the generalization performance on different scenarios, such as various deployment scenarios (e.g., UMa, UMi, InH), various outdoor/indoor UE distributions for UMa/UMi (e.g. 10:0, 8:2, 5:5, 2:8, 0:10), various carrier frequencies (e.g. 2GHz, 3.5GHz) and other aspects (e.g. antenna spacing, antenna virtualization, ISDs, UE speeds, etc.) can be evaluated.

Secondly, the generalization performance on different configurations, such as various bandwidths (e.g. 10MHz, 20MHz), various antenna layouts, e.g., (N1/N2/P), other aspects of configurations (e.g. various numerologies, various rank numbers/layers, etc.) which have no effect on the input/output dimension of AI/ML model, can be evaluated.

However, although the baseline SLS EVM has been agreed to construct the dataset, there are still multiple kinds of combinations of scenarios and configurations to generate different mixing datasets for generalization evaluation. From our opinion, in the initial stage, companies are encouraged to provide generalization performance results on various kinds of datasets as diverse as possible to get more insights about AI-based CSI compression. In the second stage, it would be better to construct one or several typical dataset(s) with aligned mixed configuration(s)/scenario(s) to draw the conclusion on generalization performance in this SI.

Scalability

For scalability issue, different input/output dimensions for various configurations with one AI/ML model should be studied, including various frequency granularities (e.g. size of sub-band), various size of CSI feedback payloads, various antenna port numbers (e.g. 32ports, 16ports) and other aspects of configurations which lead to different input/output CSI dimensions.

In current stage, since the scalability issue has not been well studied, companies are encouraged to provide the details of methodologies to achieve the scalability of AI/ML model, including the pre-processing on the input and post-processing on the output of the AI/ML model, and the advanced training method to obtain the AI/ML model with good scalability.

Figure 1: Zero-padding pre-processing on the input CSI of the encoder: (a) zero-padding on antenna port dimension; (b) zero-padding on sub-band dimension

Specifically, the zero-padding pre-processing on the input CSI of the encoder at the UE side can be utilized. For example, as shown in Figure 1 (a), the AI/ML model trained on dataset from Configuration#A with 32 port can be inferenced/tested on dataset from Configuration#B with 16 port and zero-padding on the antenna port domain. Similarly, as shown in Figure 2 (a), the AI/ML model trained on dataset from Configuration#A with 13 sub-band can be inferenced/tested on dataset from Configuration#B with 8 sub-band and zero-padding on the sub-band domain.

Meanwhile, the clipping post-processing on the output CSI of the decoder at the NW side should be utilized correspondingly. For example, for Figure 1 (a), the output CSI on the first 16 port should be reserved with real CSI input and another part of output CSI on the latter 16 port should be clipped with zero-padding input. For Figure 1 (b), the output CSI on first 8 sub-band should be reserved with real CSI input and another part of output CSI on the latter 5 sub-band should be clipped with zero-padding input.

For different CSI feedback payloads, referred to as the output of encoder and the input of decoder, the bitstream truncation operation can be considered. Specifically, we have two kinds of datasets from Configuration#A with CSI feedback payload LA bits and Configuration#B with CSI feedback payload LB bits with LA>LB. When the AI/ML model trained on Configuration#A is adopted on Configuration#B, the first LB bits can be reserved, and the latter LA-LB bits can be truncated during the interface feedback. Then, the truncated  LA-LB bits can be regarded as default 0 or 1 for the decoder input.

For the scalability evaluation, we also suggest that companies to report the details of methods and configurations in current stage. Next, it would be better to construct one or several typical dataset(s) with aligned different configurations to draw the conclusion on scalability performance in this SI.

Based on the above discussions, we have the following proposals:

Proposal 2: For scalability evaluation, zero-padding, clipping and truncation can be considered for pre-processing and post-processing.

Fine-tuning

For generalization evaluation, Case 1, Case 2 and Case 3 have been agreed in RAN1 #110bis-e to evaluate the generalization performance of AI/ML based CSI feedback. And the potential performance benefits of fine-tuning for CSI feedback can be optionally considered by companies. However, we think some important points about fine-tuning should be studied before performance evaluation and drawing conclusions.

First of all, the EVM for fine-tuning should be defined. In Case 2A, the AI/ML model is trained on dataset from Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g. Scenario#B/Configuration#B, Scenario#A/Configuration#B. Here, we denote the dataset from Scenario#A/Configuration#A as original dataset, and the dataset different than Scenario#A/Configuration#A used for AI/ML model updating as fine-tuning dataset. The following aspects should be considered:

- Size of fine-tuning dataset: different size of fine-tuning dataset may lead to different performance. For example, using 2k samples may achieves better fine-tuning performance than using 200 samples. Generally, the fine-tuning performance may improve with more samples in fine-tuning dataset for AI/ML model update. Therefore, the size of fine-tuning dataset should be considered in EVM.

- Sampling distribution of fine-tuning dataset: the sampling distribution is also very critical. Since the size of fine-tuning dataset is relatively smaller, when the limited samples are badly selected, e.g. non-uniformly selected from limited drops or biased UE distributions, the AI/ML may be overfitted on the limited samples in fine-tuning dataset, and the fine-tuning performance may even degrade compared to the performance without fine-tuning.

- Diversity between fine-tuning dataset and original dataset: the diversity between fine-tuning dataset and original dataset also affects the fine-tuning performance. If there is large difference between fine-tuning dataset and original dataset, the AI/ML model trained on original dataset may be not a good starting point for updating on fine-tuning dataset. Therefore, given a fine-tuning dataset, a proper original dataset should be selected for better fine-tuning performance.

- Fine-tuning delay: from our understanding, fine-tuning is more likely to be performed in online training. Therefore, the time cost for fine-tuning, defined as fine-tuning delay should also be considered.

Moreover, to evaluate the performance of fine-tuning, direct training on fine-tuning dataset from random initialization of AI/ML model without Scenario#A/Configuration#A as the original dataset and inference on the testing dataset, e,g. from Scenario#A/Configuration#B, Scenario#B/Configuration#B should also be considered as a baseline besides the Rel-16 eType II baseline. Therefore, based on the above discussions, we have the following proposals:

Proposal 3: Regarding the EVM for fine-tuning, at least the following factors should be considered:

- Size of fine-tuning dataset

- Sampling distribution of fine-tuning dataset

- Diversity between fine-tuning dataset and original dataset

- Fine-tuning delay

- Performance gain

- Other aspects related to fine-tuning

Proposal 4: For the baseline of fine-tuning evaluation, direct training on fine-tuning dataset from random initialization and inference on the testing dataset should be considered as a benchmark.

AI/ML model

In this contribution, both the encoder at UE side and decoder at NW side are realized based on Transformer backbone. As agreed in RAN1 #112, for the AI/CSI compression sub use cases with rank>=1, we select Option 3 with layer common AI/ML model. Moreover, the Option 3-1 is considered, where a unified AI/ML model is applied for each layer under any rank value to perform individual inference. Therefore, for each layer in different ranks, only one AI/ML model is required. The complexity with FLOPs and model size with trainable parameters for the unified Transformer based model are listed in Table 1.

Table 1 FLOPs and trainable parameters

| AI/ML model | FLOPs | Trainable parameters |
| --- | --- | --- |
| Encoder | ~21.4M | ~10.7M |
| Decoder | ~21.4M | ~10.7M |
| Total | ~42.8M | ~21.4M |

We use a model named EVCsiNet-T (as shown in Figure 2), in which each vector of sub-band in the input CSI matrix is firstly processed by an embedding layer and then 6 self-attention based blocks are sequentially introduced before a mixed 3bits/2bits quantization layer. As for the decoder part, after dequantization layer, a dense layer with 64 nodes is employed, followed by 6 self-attention based blocks as well. Finally, a reshape layer is implemented to obtain the output with the shape of the original CSI. The SGCS loss function is used to train the EVCsiNet-T.

Figure 2: Illustration of EVCsiNet-T model for CSI feedback compression

From our opinion, companies are encouraged to disclose their utilized dataset(s) and/or reference model(s), which would be very helpful for crosscheck between companies.  Furthermore, common dataset(s) and/or reference model(s) would be more efficient for performance calibration and drawing final conclusions. The reference model in our simulations for AI/ML based CSI feedback enhancement can be find in https://wireless-intelligence.com/#/download. However, how to establish common dataset(s) and/or reference model(s) in 3GPP frame remains further study.

Proposal 5: Companies are encouraged to disclose their utilized dataset(s) and reference model(s)

- FFS: to establish common dataset(s) and/or reference model(s) for performance calibration and drawing final conclusions.

Performance evaluation

SLS throughput

In this section, we give some performance evaluation results with 32Tx4Rx antenna configuration. The SGCS for rank 1 and rank 2 on sub-band level for different CSI feedback overhead are provided in Figure 3 and Figure 4, where the R16 eType II is used as the comparable baseline. For rank 1, the CSI feedback overhead is (67, 92, 120, 174, 231, 250, 285, 335) bit for EVCsiNet-T-layer 1 and eType II. For rank 2, the CSI feedback overhead is (67, 92, 120, 174, 231, 250, 285, 335) bit for both EVCsiNet-T-layer 1 and EVCsiNet-T-layer 2, where the AI/ML model is trained on dataset for layer 1 and dataset for layer 2 with the same architecture EVCsiNet-T, respectively.  And the SGCS for rank 2 is calculated by the average of SGCS of layer 1 and layer 2. The training and testing dataset for each layer is 600k and 5k, respectively.

Please note that “rank2” refers to the dynamic transmission of single-layer and 2-layer MIMO scheduled by NW in the evaluation of SLS throughput.

Figure 3. Comparison of SGCS between AI based CSI feedback and R16 eType II baseline (rank 1)

Figure 4. Comparison of SGCS between AI based CSI feedback and eType II baseline (rank 2)

Observed from Figure 3 and Figure 4 AI based CSI feedback with EVCsiNet-T can achieve higher SGCS performance compared with R16 eType II baseline for both rank 1 (5%~8%) and rank 2 (8%~16%), especially with lower feedback payload. Meanwhile, compared with rank 1, the AI based CSI feedback has larger performance gain in rank 2.

Figure 5: Comparisons of SLS throughput between AI based CSI feedback and R16 eType II baseline with ideal channel estimation

Figure 6: Comparisons of SLS throughput between AI based CSI feedback and R16 eType II baseline with realistic channel estimation

Observed from Figure 5 and Figure 6, AI based CSI feedback can achieve higher SLS throughput for all conditions, such as rank 1/2, FTP/Full buffer and ideal/realistic channel estimation, especially with lower CSI feedback overhead. Specifically, for rank 1 with realistic channel estimation, full buffer model can provide about 3%~6% and FTP model can provide about 1%~3% performance gain. Similar to SGCS comparison results, the performance gain is also larger for rank 2 configuration, about 4%~10% performance gain with FTP model. Based on the above evaluation results, we have the following observations:

Observation 1: Compared to rank 1 achieving 5%~8% SGCS gain and 1%~3% SLS throughput gain, AI based CSI feedback achieves 8%~16% SGCS gain and 4%~10% SLS throughput gain for rank 2.

Observation 2: Compared to higher feedback overhead achieving 1% for rank 1 and 4% for rank 2 SLS throughput gain, AI based CSI feedback achieves larger SLS throughput gain with lower feedback overhead, about 3% for rank 1 and 10% for rank 2.

Observation 3: Compared to FTP model achieving 1%~3% SLS throughput gain for rank 1, AI based CSI feedback achieves larger SLS throughput gain for full buffer model about 3%~6%.

Generalization evaluation

The generalization performance is presented in this part. As shown in Table 2, the intermediate KPI with SGCS trained on UMa/UMi/mixing datasets and inferenced on UMa/UMi with 67/174bit CSI feedback payload is given. Each training data consists of 600k samples, and each testing set includes 5k samples. For the mixing dataset, it includes 300k UMa samples and 300k UMi samples.

Table 2 Generalization performance evaluation

| Training set | Testing set: payload |  |  |  |
| --- | --- | --- | --- | --- |
|  | UMa:67bit | UMi:67bit | UMa:174bit | UMi:174bit |
| UMa#600k | 0.786 | 0.773 | 0.878 | 0.850 |
| UMi#600k | 0.772 | 0.784 | 0.844 | 0.860 |
| UMa#300k + UMi#300k | 0.781 | 0.775 | 0.875 | 0.857 |

Obviously, the SGCS of the AI/ML model trained on UMa degrades slightly when testing on UMi and vice versa. Meanwhile, the AI/ML model trained on mixing dataset can achieve relatively higher SGCS compared with the condition when training set and testing set are mismatching. Therefore, based on the results above, we have the following observations:

Observation 4: For different scenarios, the SGCS degradation is slight (about 1%~3%) when training set and testing set are mismatching.

Observation 5: For different scenarios, training on mixing dataset can improve the generalization performance of AI/ML model.

Fine-tuning evaluation

The fine-tuning performance is presented in this part, where the UMa is used as the training set to obtain the initial AI model. Then the AI model is fine-tuned and tested at the UMi and CDL-C, respectively. The SGCS performance evaluation is given in Table 3. The Epoch=50 indicates that the AI model fine-tuning has been finished, the corresponding SGCS is the final fine-tuning performance. The upper-bound for UMi means to use UMi#600k as the training set to directly train the AI model for UMi scenario. The upper-bound for CDL-C means to use CDL-C#100k as the training set to directly train the AI model for CDL-C scenario.

Table 3 Fine-tuning SGCS performance evaluation

| Training set | Fine-tuning set | SGCS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Epoch = 1 | Epoch=50 | Upper-bound | eTypeII |
| UMa #600k | UMi#1k | 0.773 | 0.774 | 0.784 | 0.713 |
|  | UMi#10k | 0.775 | 0.776 |  |  |
|  | UMi#100k | 0.777 | 0.779 |  |  |
|  | CDL-C#200 | 0.772 | 0.772 | 0.854 | 0.668 |
|  | CDL-C#2k | 0.772 | 0.780 |  |  |
|  | CDL-C#20k | 0.779 | 0.807 |  |  |

For both the fine-tuning on UMi and CDL-C, the fine-tuning with insufficient datasets can not achieve the equivalent SGCS performance as upper-bound. The fine-tuning performance also increases with larger fine-tuning datasets.

Generally, the difference between UMi and UMa is moderate, both these two scenarios include 80% indoor UEs and 20% outdoor UEs, wherein the major 80% indoor UEs follow similar distribution and characteristics. Therefore, the initial AI model trained on UMa can perform well on UMi scenario with good generalization performance. Hence, the fine-tuning gain on SGCS performance is very small with insufficient UMi fine-tuning dataset.

For the LLS CDL-C channel, it can be regarded as one typical case in UMa scenario with given cluster and delay distribution. The CDL-C possess more concentrated characteristics in spatial and frequency domain, hence the upper-bound SGCS performance for CDL-C is higher than UMa and UMi. And the fine-tuning gain on SGCS performance is visible with relatively larger fine-tuning dataset (CDL-C#20k), which vanishes when fine-tuning dataset is small (CDL-C#200). Based on Table 3, we have the following observations:

Observation 6: For AI model trained on UMa and fine-tuned on UMi, the fine-tuning gain on SGCS performance is small with insufficient UMi fine-tuning dataset.

Observation 7: For AI model trained on UMa and fine-tuned on CDL-C, the fine-tuning gain on SGCS performance is about 3.6% for larger fine-tuning dataset, which vanishes when fine-tuning dataset is small.

Observation 8: For AI model trained on UMa and fine-tuned on UMi and CDL-C, the fine-tuning with insufficient datasets cannot achieve the equivalent SGCS performance as upper-bound.

- Note: upper-bound indicates using sufficient training set, which is from the same scenario as testing set.

Furthermore, the convergence speed for fine-tuning is also evaluated in Table 4. Here fine-tuning solution means trained on UMa and then fine-tuned on set CDL-C#2k and CDL-C#20k. The compared baseline is directly trained on set CDL-C#2k and CDL-C#20k with the random AI model initialization as the starting point.

Table 4: Fine-tuning convergence speed evaluation

| Training set | Solution | SGCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Epoch=0 | Epoch = 1 | Epoch=5 | Epoch=10 | Epoch=20 | Epoch=50 |
| CDL-C#2k | Fine-tuning | 0.742 | 0.772 | 0.778 | 0.779 | 0.780 | 0.780 |
|  | Baseline | / | 0.100 | 0.277 | 0.414 | 0.481 | 0.610 |
| CDL-C#20k | Fine-tuning | 0.742 | 0.772 | 0.789 | 0.792 | 0.798 | 0.807 |
|  | Baseline | / | 0.351 | 0.605 | 0.639 | 0.715 | 0.804 |

Obviously, for both 2k and 20k dataset, the fine-tuning solution converges faster than baseline. Specifically, for smaller dataset CDL-C#2k, the fine-tuning can achieve about 27.9% SGCS performance gain compared with baseline after 50 epochs. While for larger dataset CDL-C#20k, fine-tuning and baseline achieve similar SGCS performance after 50 epochs.  Based on Table 4, we have the following observations:

Observation 9: Fine-tuning converges faster than baseline for both CDL-C#2k and CDL-C#20k datasets. Compared with baseline, fine-tuning achieves about 27.9% SGCS performance gain on smaller dataset CDL-C#2k and similar SGCS performance gain on larger dataset CDL-C#20k.

- Note: baseline indicates using directly trained on CDL-C#2k and CDL-C#20k with random AI model initialization as starting point.

Scalability evaluation

The scalability performance is presented in this part, where the CDL-C channels with 300ns delay spread are utilized. Here, three kinds of configurations, including 32port#67bit, 16port#67bit and 16port#49bit are considered. Each dataset includes 100k samples, where 95k for training and 5k for testing. The intermediate KPI with SGCS for different training sets and testing sets are listed in Table 5.

Table 5 Scalability performance evaluation

| Training set | Testing set |  |  |
| --- | --- | --- | --- |
|  | 32 port#67bit | 16 port#67bit | 16 port#49bit |
| 32 port#67bit | 0.855 | 0.693 | / |
| 16 port#67bit | / | 0.886 | 0.734 |
| 16 port#49bit | / | / | 0.861 |
| 32 port#67bit + 16 port#67bit | 0.853 | 0.898 | / |
| 32 port#67bit + 16 port#49bit | 0.834 | / | 0.821 |

Here, when AI/ML model trained on 32-port dataset and tested on 16-port dataset, or trained on mixing datasets with 32 port and 16 port, the zero-padding pre-processing and clipping post-processing is utilized for 16-port dataset. When AI/ML model trained on 67bit and tested on 49bit, or trained on mixing datasets with 67bit and 49bit, the bitstream truncation and default 0 is utilized for 49bit dataset.

Obviously, for the scalability on various number of antenna ports, the AI/ML model only trained on 32 port will suffer considerable performance degradation in the testing with 16 port, while the AI/ML model trained on mixing datasets with 32 port and 16 port can improve the SGCS significantly when testing on 16 port. Meanwhile, for the scalability on various CSI feedback payloads, the AI/ML model only trained on 67bit performs will suffer significant performance degradation in the testing with 49bit, while the AI/ML model trained on mixing datasets with 32 port#67bit and 16 port#49bit can improve the SGCS when testing on 49bit. Therefore, the scalability performance of AI/ML model can be improved by trained on mixing datasets including various configurations. Based on the results above, we have the following observations:

Observation 10: The scalability performance of AI/ML model for various number of antenna ports and CSI feedback payloads can be improved by trained on mixing datasets.

We simulate the performance with 1 common decoder at NW and 2 UE-specific encoders at different UE sides with different configurations. Specifically, 32 port#67bit is configured for UE 1 and 16 port#49bit is configured for UE 2. The same encoder structure with EVCsiNet-T is utilized for both two UEs, and the aligned decoder structure with EVCsiNet-T is utilized for NW. The AI/ML model is trained on the mixing datasets with 32 port#67bit and 16 port#49bit, Table 6 shows the testing results for both two configurations.

Table 6 Multi-encoder performance

| Training set | Encoder 1, 32 port#67bit | Encoder 2, 16 port#49bit |
| --- | --- | --- |
| 32 port#67bit + 16 port#49bit | 0.835 | 0.858 |

As given in Table 6, AI/ML model with common decoder and multi-encoder at different UE sides with different configurations can achieve good performance for both two UEs. Furthermore, training on the mixing datasets with UE-specific encoder can achieve 0.858 for UE 2, which is larger than 0.821 with common encoder in Table 5.

Observation 11: Using common decoder with UE-specific encoder achieves higher SGCS than using common decoder with common encoder.

Similarly, we also simulate the performance with 1 common encoder at UE side and 2 gNB-specific decoders at NW side with different configurations. Specifically, 32 port#67bit is configured for gNB 1 and 16 port#49bit is configured for gNB 2. The same decoder structure with EVCsiNet-T is utilized for both two gNBs, and the aligned encoder structure with EVCsiNet-T is utilized for UE. The AI/ML model is trained on the mixing datasets with 32 port#67bit and 16 port#49bit, Table 7 shows the testing results for both two configurations.

Table 7 Multi-decoder performance

| Training set | Decoder 1, 32 port#67bit | Decoder 2, 16 port#49bit |
| --- | --- | --- |
| 32 port#67bit + 16 port#49bit | 0.848 | 0.867 |

As given in Table 7, AI/ML model with common encoder and multi-decoder at different NW sides with different configurations can achieve good performance for both two configurations. Furthermore, training on the mixing datasets with gNB-specific decoder can achieve 0.858/0.867 for UE 1/2, which is larger than 0.834/0.821 with common decoder in Table 5.

Observation 12: Using common encoder with gNB-specific decoder achieves higher SGCS than using common encoder with common decoder.

Type 3 training evaluation

The performance of type 3 training for CSI compression is evaluated in Table 8. Here type 1 training with UMa#600k is used as the baseline for comparison. The overhead indicates the size of transmitted training data, including the CSI eigenvector dataset from UE to NW, and the bitstream dataset from NW to UE in NW-first training and UE to NW in UE-first training. For CSI eigenvector transmission, the float32 format is used to transmit the CSI label, thus totally 600k×32×13×2×32/8≈1904M is required for 600k size dataset. For bitstream transmission with 67bit CSI feedback payload, totally  600k*67/8≈4.8M is required. Therefore, for 600k, 300k, 100k and 50k training data, the total type 3 training overhead is about 1909M, 954M, 319M and 160M.

Table 8: Type 3 training evaluation

| Solution | Type 1 training | Type 3 training overhead (Mbytes) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1909 | 954 | 319 | 160 |
| NW-first | 0.787 | 0.780 | 0.772 | 0.751 | 0.735 |
| UE-first |  | 0.785 | 0.779 | 0.768 | 0.759 |

Compared with baseline using type 1 training, NW-first/UE-first type 3 training has 0.9%/0.3% SGCS performance loss with 1909Mbytes overhead and 6.6%/3.6% SGCS performance loss with 159Mbytes overhead. For both of NW-first and UE-first type 3 training, the SGCS performance decreases with smaller overhead over the air interface. The UE-first method achieves higher SGCS performance compared with NW-first method, since a better decoder can be trained with fixed input bitstream from well trained encoder at the UE-side. Based on Table 8, we have the following observations:

Observation 13: Compared with type 1 training, NW-first type 3 training has 0.9%~6.6% and UE-first type 3 training has 0.3%~3.6% SGCS performance loss with 160M~1909M overhead, respectively.

Next, the Type 3 training performance with matched/unmatched encoder at the UE side and decoder at the NW side is presented in Table 9. Here the Type 1 1-on-1 joint training with UMa#600k and Transformer based matched encoder and decoder is marked as baseline-Transformer, the Type 1 1-on-1 joint training with UMa#600k and ResNet based matched encoder and decoder is marked as baseline-ResNet. For all solutions, 67bits CSI feedback payload is adopted.

Table 9: Type 3 training evaluation for matched and unmatched model pairing

| Solution | UE-first (encoder + decoder) |  | NW-first (encoder + decoder) |  |
| --- | --- | --- | --- | --- |
| Baseline-Transformer | Transformer + Transformer | Transformer + ResNet | Transformer + Transformer | ResNet + Transformer |
| 0.787 | 0.785 | 0.774 | 0.780 | 0.690 |
| Baseline-ResNet | ResNet + ResNet | ResNet + Transformer | ResNet + ResNet | Transformer + ResNet |
| 0.719 | 0.716 | 0.719 | 0.717 | 0.719 |

Firstly, comparing baseline-Transformer and baseline-ResNet, the Transformer backbone achieves higher SGCS performance than ResNet. Compared with baseline-Transformer, both UE-first and NW-first matched model pairing can approach similar SGCS performance with only 0.3% and 0.9% loss. For UE-first Type 3 training with unmatched AI/ML model pairing using Transformer encoder and ResNet decoder, the SGCS performance loss is about 1.7%. As comparison, for the NW-first Type 3 training with unmatched ResNet encoder and Transformer decoder, the SGCS performance loss is about 12.3%, which indicates that using a worse AI/ML model architecture for encoder at the UE side in NW-first Type 3 training may result large performance loss.

Observation 14: NW-first Type 3 training with unmatched AI/ML model pairing using ResNet encoder and Transformer decoder has 12.3% SGCS performance loss compared with Type 1 1-on-1 joint training Transformer baseline.

Moreover, the results of multi-vendor Type 3 training are presented in Table 10. For UE-first training, M=2 encoders at M=2 UE vendors and N=1 decoder at the NW side are considered, where each UE possesses UMa#300k dataset to train its encoder model individually, and then NW combines the output information from M=2 encoders as total UMa#600k dataset to train the decoder at the NW side.  For NW-first training, N=2 decoders at N=2 gNB vendors and M=1 encoder at the UE side are considered, where each gNB possesses UMa#300k dataset to train its decoder model individually, and then UE combines them as total UMa#600k dataset to train the encoder at the UE side.

Specifically, for UE-first Type 3 training, we consider Transformer-6 as the decoder. At the UE side, three kinds of encoder combinations are assumed for M=2 UE vendors, including (model A,model B) = (Transformer-6, Transformer-6), (Transformer-6, Transformer-3) and (Transformer-6, ResNet), where -6 and -3 denotes the number of self-attention layers in the Transformer model. Similarly, for NW-first Type 3 training, we consider Transformer-6  as the encoder. At the NW side, three kinds of decoder combinations are assumed for N=2 gNB vendors, including  (model A,model B) = (Transformer-6, Transformer-6), (Transformer-6, Transformer-3) and (Transformer-6, ResNet).

Table 10: Type 3 training evaluation for multi-vendor training

| Solution |  | (Transformer-6, Transformer-6) | (Transformer-6, Transformer-3) | (Transformer-6, ResNet) |
| --- | --- | --- | --- | --- |
| UE-first | Model A | 0.738 | 0.739 | 0.748 |
|  | Model B | 0.741 | 0.739 | 0.696 |
| NW-first | Model A | 0.427 | 0.355 | 0.330 |
|  | Model B | 0.471 | 0.398 | 0.390 |

Observed from Table 10, we find that NW-first training with N=2 decoders and M=1 encoder cannot work with very large SGCS performance loss, even though with the same AI/ML model architecture (Transformer-6, Transformer-6). For UE-first training with M=2 encoders and N=1 decoder, both three encoder combinations can achieve comparable SGCS performance. Using different numbers of self-attention layers in the Transformer backbone, similar SGCS performance can be realized. Moreover, for (Transformer-6, ResNet) combination, the unique decoder can be adopted for both two encoders with decent SGCS performance.

Observation 15: Regarding UE-first Type 3 multi-vendor training, N=1 decoder and M=2 encoders with (Transformer-6, Transformer-6), (Transformer-6, Transformer-3) and (Transformer-6, ResNet) achieve comparable SGCS performance.

Observation 16: Regarding NW-first Type 3 multi-vendor training with M=1 encoder and N=2 decoders, the SGCS performance is very large.

For UE-first Type 3 multi-vendor training, we further evaluate the influence of UE vendor numbers with M = 2,3,4 and 10. The encoders for all UE vendors are realized with Transformer-6. The results are listed in Table 11, where the baseline is given by individual 1-on-1 joint training with each pair of encoder and decoder model.

Table 11: UE-first Type 3 multi-vendor training evaluation for different M

| Solution | M=2 | M=3 | M=4 | M=10 |
| --- | --- | --- | --- | --- |
| Baseline | 0.764 | 0.754 | 0.746 | 0.721 |
| Type 3 training | 0.739 | 0.715 | 0.712 | 0.673 |

We find that as the number of UE vendors increases from M=2 to M=10, the UE-first Type 3 training has larger SGCS performance loss from 3.3% to 6.6%, which may result from that one unique decoder at the NW side is required to adapt more encoders. Therefore, we have the following observation and proposal:

Observation 17: Regarding UE-first Type 3 multi-vendor training with M>1 encoders and N=1 decoder, the SGCS performance loss increases as M increases.

Proposal 6: Suggest to evaluate UE-first Type 3 multi-vendor training with M>1 UE part models and N=1 NW part model, especially when M is large (e.g. M>10).

Discussion on CSI prediction

In 3GPP RAN1 #111, the following working assumption about CSI prediction is achieved:

Agreement

Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

Evaluation methodology

As for CSI prediction for time domain, if it is selected as one sub use case, the EVM used for CSI compression sub use case should be the baseline for simplicity. Then, other specific parameters for CSI prediction (e.g. UE speed, outdoor/indoor UE distribution, CSI feedback periodicity) should be further considered.

Regarding the input of AI model for CSI prediction, both raw channel matrixes and eigenvectors can be used. For raw channel prediction, the input/output of AI/ML model would be the channel matrix H∈CNt×Nr×Nsc, where Nt,Nr,Nsc are the numbers of Tx port, Rx port and subcarrier. For eigenvector prediction, the input/output of AI/ML model would be the eigenvector W∈CNt×Nsb, where Nsb is the number of sub-band.  Generally, selecting raw channel as the input/output may lead to a much huger dataset with massive storage space compared to eigenvector, which is inconvenient for dataset construction and AI/ML model training. Moreover, as for the intermediate KPI, normalized mean square error (NMSE) and SGCS can be used for raw channel and eigenvector, respectively. But using eigenvector for CSI prediction and SGCS as the intermediate KPI would be more intuitive since it has been widely used in CSI compression sub use case. In our following text, both the eigenvector input and raw channel input are e

Figure 7 CSI prediction assumption 1

Take eigenvector input as an example, two aspects of CSI prediction assumptions can be considered. As shown in Figure 7, the input of CSI model includes K historic eigenvectors [Wt-5(K-1),…Wt-5,Wt] from K CSI-RS measurement (in red color). The output of CSI prediction model includes 4 future eigenvectors Wt+1,Wt+2,Wt+3,Wt+4 on the next 4 interval slots (in yellow color). Then the observation window is K and the prediction window is 4. For the interval slots in prediction window, the predicted CSI eigenvectors from AI/ML model output can be used for transmission. This is considered to deal with the impact of scheduling delay and outdated CSI on the use of CSI information, the CSI accuracy on the interval slots can be improved using AI/ML model by compensating the difference of CSI resulted from the scheduling delay. Similarly, different UE speeds (e.g. 30km/h, 60km/h, 90km/h and 120km/h) and CSI-RS periods (e.g. 5ms, 10ms and 20ms) can be considered with various lengths of observation window and prediction window.

(a)

(b)

Figure 8 CSI prediction assumption 2

Secondly, as shown in Figure 8, it is proposed to reduce the CSI-RS overhead. In Figure 8 (a), the K historic eigenvectors CSI-RS with long periodicity (e.g. 40ms/80ms/160ms) can be used as the input of CSI prediction model (in red color). In Figure 8 (b), the input of CSI prediction model consists of K historic eigenvectors [Wt-5(K-1),…Wt-5,Wt] from K CSI-RS measurement (in red color). For both (a) and (b) configurations, the output of CSI prediction model includes T future eigenvectors [Wt+5,Wt+10,…Wt+5T] on T CSI-RS transmission instances (in yellow color). Then the observation window is K and the prediction window is T. For the interval slots in prediction window, the nearest predicted CSI eigenvector is directly utilized for transmission. For this assumption, the CSI-RSs in prediction window are not required, and the CSI-RS overhead can be reduced with proportion of T/(K+T). For different UE speeds (e.g. 30km/h, 60km/h, 90km/h and 120km/h) and CSI-RS periods (e.g. 5ms, 10ms and 20ms), we can select different lengths of observation window and prediction window, therefore obtain different CSI-RS overhead reduction proportions using AI/ML based CSI prediction in time domain.

Actually, both two assumptions for CSI prediction can be studied for different purposes. Furthermore, in the training stage of assumption 1, the CSI eigenvectors for every slot should be stored, which increases the complexity and storage load of dataset construction. As comparison, for assumption 2, only eigenvectors on CSI-RS instances are required as the input and label, the CSI eigenvectors for interval slots are not required, which decreases the complexity and storage load of dataset construction.

Based on above discussions, we have the following proposals:

Proposal 7: In R18 time domain CSI prediction, following two aspects should be studied to evaluate the performance gain and identify the potential spec impacts.

- Impact on throughput caused by scheduling delay and outdated CSI

- Reduction of CSI-RS overhead

Proposal 8: Regarding the EVM on CSI prediction, evaluate the performance with different numbers of observation window K and prediction window T according to CSI-RS period

UE-side model

As discussed in RAN1 #111, the UE-sided CSI prediction is agreed as the sub use case. For UE-sided AI model, the input and output should be considered as the following.

The input of AI/ML model can be the estimated raw channel(s) or eigenvector(s) in the observation window. The output of AI/ML model should be the predicted raw channel(s) or eigenvector(s) in the prediction window. Then, during CSI feedback phase, the codebook based eType II (with possible enhancement) and AI based CSI feedback can be used to report the eigenvector(s) in prediction window.  However, for raw channel(s) feedback, it seems conventional codebook based algorithms cannot be used currently. Therefore, only AI based raw channel compression and feedback is workable, or the post-processing by calculating the eigenvector(s) from the predicted raw channel(s) is required for CSI feedback.

For UE-side model training, similar to the CSI compression sub use case, the ideal channel estimation can be used to construct training dataset and the ideal CSI can also be used as the target CSI for training. Then the ideal channel estimation can be used for CSI prediction performance calibration with intermediate KPI, and the realistic channel estimation should be used for SLS throughput evaluation for drawing the conclusions. Moreover, in SLS evaluation, the CSI compression and recovery error in prediction window should be considered as well.

Based on the discussions above, we have the following proposal:

Proposal 9: For UE-sided CSI prediction, the following items should be considered

- Ideal channel estimation for training stage and intermediate KPI calibration

- Realistic channel estimation together with CSI feedback error for SLS performance evaluation

- Possible CSI feedback enhancement for the output of CSI prediction (e.g. eType II based, AI based CSI feedback)

- FFS: how to perform raw channel feedback in observation and prediction window

AI/ML model

In this contribution, we use the MLP-Mixer backbone with ~46.02M FLOPs and model size with ~23.07M trainable parameters for eigenvector based CSI prediction and Conv-mixer backbone with ~1.7M FLOPs and model size with ~0.86M trainable parameters for raw channel based CSI prediction. The corresponding AI/ML structures are illustrated in Figure 9(a) and Figure 9(b).

(a)

(b)

Figure 9: structure of MLP-Mixer (a) and Conv-Mixer (b)

The MLP-Mixer model is composed of dimensional transform block, 10 blocks of mixer layer and a fully-connected layer. The Conv-Mixer model is composed of 4 convolution block and 1 depth-to-space reshape layer. The output dimension of the full-connected layer in MLP-Mixer and the depth-to-space layer in Conv-Mixer can be adjusted according to the different lengths of prediction window.

Performance evaluation

Multi-UE evaluation

We give some performance evaluation results for CSI prediction in Table 12 to evaluate the availability of AI/ML based CSI prediction for multi-UE scenario. The UE-side model with eigenvector as the input and the output is adopted. The observation window is set as K=4, the prediction window is set as T = [5,10,15,20]ms. The intermediate KPI with SGCS of the AI based CSI prediction is compared with the sample-and-hold non-AI baseline. The dataset includes 5 drops CSI eigenvector samples, where each drop includes 570 UE and each UE includes 397 slots with CSI-RS interval 5ms. For each drop, the first 90% UE and 90% slots are utilized for training. For testing set, we have Set1 and Set2, respectively. For Set1, the 10% UE of each drop is used for testing. For Set2, the 10% samples of each UE of each drop is used for testing.

Table 12 AI based CSI prediction evaluation for multi-UE scenario

| SGCS | 5ms |  | 10ms |  | 15ms |  | 20ms |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set1 | Set2 | Set1 | Set2 | Set1 | Set2 | Set1 | Set2 |
| Non-AI (sample-and-hold) | 0.870 | 0.871 | 0.805 | 0.793 | 0.730 | 0.720 | 0.675 | 0.667 |
| AI | 0.922 | 0.930 | 0.843 | 0.839 | 0.767 | 0.764 | 0.709 | 0.706 |
| Gain | 0.052 (6.0%) | 0.059 (6.8%) | 0.038 (4.7%) | 0.046 (5.8%) | 0.037 (5.1%) | 0.044 (6.1%) | 0.034 (5.0%) | 0.039 (5.8%) |

For all T=1,2,3,4 and both Set1 and Set2, AI based CSI prediction can achieve higher SGCS compared with non-AI solution. The performance gain decreases with farther prediction instances.

Observation 18: Regarding AI based CSI prediction in multi-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

Single-UE evaluation

Considering that UE-sided CSI prediction can be realized with different AI models at different UEs, the CSI prediction for single-UE is further evaluated besides the multi-UE scenario. Both eigenvector based CSI prediction and raw channel based CSI prediction are evaluated with SGCS and NMSE intermediate KPI, respectively.

Here, two different UEs with 80k samples for each UE are provided. The channel variation in time domain is fast for UE 0 and slow for UE 1. The CSI prediction result with prediction window T=[1,2,3,4]ms is presented in Figure 10 (a). The CSI prediction result with prediction window T=[5,10,15,20]ms is presented in Figure 10 (b). For both two conditions, the observation window is set as K=4. Here the legend BL-W and AI-W represent the baseline and AI based CSI prediction for eigenvector input with SGCS intermediate KPI in the left y-axis, respectively.  The legend BL-H and AI-H represent the baseline and AI based CSI prediction for raw channel input with NMSE intermediate KPI in the right y-axis, respectively.

(a)                                                                             (b)

Figure 10: CSI prediction for single-UE

Table 13 AI CSI prediction gain for Figure 10 (a)

| Prediction instance | 1ms |  | 2ms | 3ms | 4ms |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SGCS Gain (Relative SGCS Gain) |  |  |  |  |  |
| UE 0 | 0.017 (1.7%) |  | 0.051 (5.8%) | 0.109 (14.5%) | 0.197 (31.6%) |  |
| UE 1 | 0.001 (0.1%) |  | 0.003 (0.3%) | 0.010 (1.0%) | 0.017 (1.7%) |  |
|  | NMSE Gain (dB) |  |  |  |  |  |
| UE 0 | -19.873 | -25.380 |  | -26.195 |  | -25.339 |
| UE 1 | -19.049 | -25.000 |  | -27.380 |  | -29.804 |

In Table 13, for both SGCS and NMSE KPI, the CSI prediction performance gain increases with farther prediction instances for both UE 0 and UE 1. Specifically, the performance for both AI based CSI prediction and baseline deceases with farther CSI prediction instances. But AI based CSI prediction can achieve larger SGCS performance gain than baseline in farther CSI prediction instance.

For UE 0, the channel variation is fast in time domain, so the performance of baseline using sample-and-hold decreases rapidly with larger T. The AI based CSI prediction obtains 31.6% SGCS and -25.339dB NMSE performance gain at T=4ms.

As comparison, the channel variation for UE 1 is slow in time domain, hence the baseline using sample-and-hold still performs well at T=4ms, and the AI based CSI prediction only has 1.7% SGCS performance gain, but the NMSE performance gain is still very large about -29.804dB.

Table 14 AI CSI prediction gain for Figure 10(b)

| Prediction instance | 5ms | 10ms | 15ms | 20ms |
| --- | --- | --- | --- | --- |
|  | SGCS Gain (Relative SGCS Gain) |  |  |  |
| UE 0 | 0.239 (47.5%) | 0.223 (73.3%) | 0.089 (27.0%) | 0.030 (8.9%) |
| UE 1 | 0.027 (2.8%) | 0.052 (5.9%) | 0.068 (8.8%) | 0.034 (5.1%) |
|  | NMSE Gain (dB) |  |  |  |
| UE 0 | -19.959 | -19.536 | -15.795 | -11.082 |
| UE 1 | -27.352 | -30.966 | -31.049 | -29.380 |

In Table 14, for eigenvector input, AI based CSI prediction achieves highest about 73.3% performance gain at T=10ms for UE 0 and 8.8% performance gain at T=15ms for UE 1. For UE 0 with fast channel variation, the CSI prediction accuracy for both AI and baseline become very bad with SGCS<0.4. While for UE 1 with slow channel variation, the AI based CSI prediction can still achieve SGCS>0.7 at T=20ms.

As comparison for raw channel input with NMSE KPI, AI based CSI prediction achieves -19.536dB NMSE performance gain at T=10ms for UE 0. While for UE 1, -30.966dB NMSE performance gain is achieved.

Based on the results above, we have the following observation:

Observation 19: Regarding AI based CSI prediction in single-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

- AI achieves higher CSI prediction performance gain for UE with fast channel variation

- AI achieves higher CSI prediction performance gain with father CSI prediction instance within the same CSI-RS feedback period

Besides the above results, to better descript the channel variation in time domain, we also provide the NMSE of raw channel and SGCS of eigenvector comparison using sample-and-hold baseline.

Table 15 NMSE and SGCS comparison

| UE index | KPI | 1ms | 2ms | 3ms | 4ms |
| --- | --- | --- | --- | --- | --- |
| UE 0 | SGCS | 0.964 | 0.871 | 0.749 | 0.623 |
|  | NMSE (dB) | -12.84 | -6.91 | -3.54 | -1.25 |
| UE 1 | SGCS | 0.997 | 0.990 | 0.981 | 0.970 |
|  | NMSE (dB) | -17.85 | -11.85 | -8.36 | -5.90 |

As shown in Table 15, NMSE increases and SGCS decreases with farther CSI prediction instance. For UE 0 when NMSE increases from -12.84dB to -1.25dB, the SGCS decreases from 0.964 to 0.623. However, for UE 1, when NMSE increases from -17.85dB to -5.90dB, the SGCS only decreases slightly from 0.997 to 0.970. Compare the T=1ms for UE 0 and T=4ms for UE 1, we find that UE 1 has larger SGCS than UE 0 (0.970>0.964), while UE 1 has worse NMSE performance than UE 0.

Based on these results in Table 13, Table 14 and Table 15, we find that the NMSE performance cannot completely match to the SGCS performance, especially for different UEs. Considering that CSI eigenvector is finally utilized, we prefer to select CSI eigenvector as the input of AI model for CSI prediction.

Observation 20:  NMSE performance cannot completely match to the SGCS performance, especially for different UEs.

Proposal 10: Suggest to use multiple kinds of intermediate KPIs (e.g. SGCS, NMSE) for CSI prediction evaluation, conclusions should be drawn based on eventual KPI (e.g. SLS throughput) instead of intermediate KPIs.

Conclusion

In this contribution, we provide some discussions and preliminary results about the evaluation on AI/ML for CSI feedback enhancement. Based on the discussions and evaluations, we have following observations and proposals:

Observation 1: Compared to rank 1 achieving 5%~8% SGCS gain and 1%~3% SLS throughput gain, AI based CSI feedback achieves 8%~16% SGCS gain and 4%~10% SLS throughput gain for rank 2.

Observation 2: Compared to higher feedback overhead achieving 1% for rank 1 and 4% for rank 2 SLS throughput gain, AI based CSI feedback achieves larger SLS throughput gain with lower feedback overhead, about 3% for rank 1 and 10% for rank 2.

Observation 3: Compared to FTP model achieving 1%~3% SLS throughput gain for rank 1, AI based CSI feedback achieves larger SLS throughput gain for full buffer model about 3%~6%.

Observation 4: For different scenarios, the SGCS degradation is slight (about 1%~3%) when training set and testing set are mismatching.

Observation 5: For different scenarios, training on mixing dataset can improve the generalization performance of AI/ML model.

Observation 6: For AI model trained on UMa and fine-tuned on UMi, the fine-tuning gain on SGCS performance is small with insufficient UMi fine-tuning dataset.

Observation 7: For AI model trained on UMa and fine-tuned on CDL-C, the fine-tuning gain on SGCS performance is about 3.6% for larger fine-tuning dataset, which vanishes when fine-tuning dataset is small.

Observation 8: For AI model trained on UMa and fine-tuned on UMi and CDL-C, the fine-tuning with insufficient datasets cannot achieve the equivalent SGCS performance as upper-bound.

- Note: upper-bound indicates using sufficient training set, which is from the same scenario as testing set.

Observation 9: Fine-tuning converges faster than baseline for both CDL-C#2k and CDL-C#20k datasets. Compared with baseline, fine-tuning achieves about 27.9% SGCS performance gain on smaller dataset CDL-C#2k and similar SGCS performance gain on larger dataset CDL-C#20k.

- Note: baseline indicates using directly trained on CDL-C#2k and CDL-C#20k with random AI model initialization as starting point.

Observation 10: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets.

Observation 11: Using common decoder with UE-specific encoder achieves higher SGCS than using common decoder with common encoder.

Observation 12: Using common encoder with gNB-specific decoder achieves higher SGCS than using common encoder with common decoder.

Observation 13: Compared with type 1 training, NW-first type 3 training has 0.9%~6.6% and UE-first type 3 training has 0.3%~3.6% SGCS performance loss with 159M~1904M overhead, respectively.

Observation 14: NW-first Type 3 training with unmatched AI/ML model pairing using ResNet encoder and Transformer decoder has 12.3% SGCS performance loss compared with Type 1 1-on-1 joint training Transformer baseline.

Observation 15: Regarding UE-first Type 3 multi-vendor training, N=1 decoder and M=2 encoders with (Transformer-6, Transformer-6), (Transformer-6, Transformer-3) and (Transformer-6, ResNet) achieve comparable SGCS performance.

Observation 16: Regarding NW-first Type 3 multi-vendor training with M=1 encoder and N=2 decoders, the SGCS performance is very large.

Observation 17: Regarding UE-first Type 3 multi-vendor training with M>1 encoders and N=1decoder, the SGCS performance loss increases as M increases.

Observation 18: Regarding AI based CSI prediction in multi-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

Observation 19: Regarding AI based CSI prediction in single-UE scenario, AI achieves higher SGCS compared with sample-and-hold baseline

- AI achieves higher CSI prediction performance gain for UE with fast channel variation

- AI achieves higher CSI prediction performance gain with father CSI prediction instance within the same CSI-RS feedback period

Observation 20:  NMSE performance cannot completely match to the SGCS performance, especially for different UEs.

Proposal 1: For training collaborative Type3, study whether/how to align the quantization and dequantization method between UE and NW

Proposal 2: For scalability evaluation, zero-padding, clipping and truncation can be considered for pre-processing and post-processing.

Proposal 3: Regarding the EVM for fine-tuning, at least the following factors should be considered:

- Size of fine-tuning dataset

- Sampling distribution of fine-tuning dataset

- Diversity between fine-tuning dataset and original dataset

- Fine-tuning delay

- Performance gain

- Other aspects related to fine-tuning

Proposal 4: For the baseline of fine-tuning evaluation, direct training on fine-tuning dataset from random initialization and inference on the testing dataset should be considered as a benchmark.

Proposal 5: Companies are encouraged to disclose their utilized dataset(s) and reference model(s)

- FFS: to establish common dataset(s) and/or reference model(s) for performance calibration and drawing final conclusions.

Proposal 6: Suggest to evaluate UE-first Type 3 multi-vendor training with M>1 UE part models and N=1 NW part model, especially when M is large (e.g. M>10).

Proposal 7: In R18 time domain CSI prediction, following two aspects should be studied to evaluate the performance gain and identify the potential spec impacts.

- Impact on throughput caused by scheduling delay and outdated CSI

- Reduction of CSI-RS overhead

Proposal 8: Regarding the EVM on CSI prediction, evaluate the performance with different numbers of observation window K and prediction window T according to CSI-RS period

Proposal 9: For UE-sided CSI prediction, the following items should be considered

- Ideal channel estimation for training stage and intermediate KPI calibration

- Realistic channel estimation together with CSI feedback error for SLS performance evaluation

- Possible CSI feedback enhancement for the output of CSI prediction (e.g. eType II based, AI based CSI feedback)

- FFS: how to perform raw channel feedback in observation and prediction window

Proposal 10: Suggest to use multiple kinds of intermediate KPIs (e.g. SGCS, NMSE) for CSI prediction evaluation, conclusions should be drawn based on eventual KPI (e.g. SLS throughput) instead of intermediate KPIs.

References

- Chair’s notes for RAN1#112

- R1-2300280 Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement, OPPO

Appendix

Table A1. Evaluation results for CSI compression without model generalization/scalability, [FTP], [Max rank value = 2], [RU=70%] [type 1 training]

|  |  | OPPO-FTP/Full buffer |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone | Transformer |
|  | Pre-processing | / |
|  | Post-processing | / |
|  | FLOPs/M | 21.4 |
|  | Number of parameters/M | 10.7 |
|  | [Storage /Mbytes] | / |
| CSI reconstruction part | AL/ML model backbone | Transformer |
|  | [Pre-processing] | / |
|  | [Post-processing] | / |
|  | FLOPs/M | 21.4 |
|  | Number of parameters/M | 10.7 |
|  | [Storage /Mbytes] | / |
| Common description | Input type | CSI eigenvector |
|  | Output type | CSI eigenvector |
|  | Quantization /dequantization method | Scalar quantization, 3bit&2bit |
| Dataset description | Train/k | 595 |
|  | Test/k | 5 |
|  | Ground-truth CSI quantization method | Float32 |
| [Other assumptions/settings agreed to be reported] |  | / |
| Benchmark |  | R16 eType II |
| Intermediate KPI I#1 (SGCS) of benchmark, [layer 1] | CSI feedback payload 67bit | 0.71 |
|  | CSI feedback payload 96bit | 0.75 |
|  | CSI feedback payload 117bit | 0.79 |
|  | CSI feedback payload 174bit | 0.83 |
|  | CSI feedback payload 231bit | 0.85 |
|  | CSI feedback payload 250bit | 0.87 |
|  | CSI feedback payload 285bit | 0.87 |
|  | CSI feedback payload 335bit | 0.88 |
| Intermediate KPI I#1 (SGCS) of benchmark, [layer 2] | CSI feedback payload 67bit | 0.53 |
|  | CSI feedback payload 96bit | 0.57 |
|  | CSI feedback payload 117bit | 0.65 |
|  | CSI feedback payload 174bit | 0.73 |
|  | CSI feedback payload 231bit | 0.75 |
|  | CSI feedback payload 250bit | 0.77 |
|  | CSI feedback payload 285bit | 0.77 |
|  | CSI feedback payload 335bit | 0.80 |
| Gain for intermediate KPI I#1 (SGCS), [layer 1] | CSI feedback payload 67bit | 0.06 |
|  | CSI feedback payload 96bit | 0.05 |
|  | CSI feedback payload 117bit | 0.03 |
|  | CSI feedback payload 174bit | 0.04 |
|  | CSI feedback payload 231bit | 0.03 |
|  | CSI feedback payload 250bit | 0.02 |
|  | CSI feedback payload 285bit | 0.03 |
|  | CSI feedback payload 335bit | 0.03 |
| Gain for intermediate KPI#1 (SGCS), [layer 2] | CSI feedback payload 67bit | 0.16 |
|  | CSI feedback payload 96bit | 0.23 |
|  | CSI feedback payload 117bit | 0.15 |
|  | CSI feedback payload 174bit | 0.12 |
|  | CSI feedback payload 231bit | 0.13 |
|  | CSI feedback payload 250bit | 0.10 |
|  | CSI feedback payload 285bit | 0.11 |
|  | CSI feedback payload 335bit | 0.11 |
| Gain for Mean UPT, [rank 1] (FTP/Full buffer) | CSI feedback payload 67bit | 3%/6% |
|  | CSI feedback payload 96bit | 2%/5% |
|  | CSI feedback payload 117bit | 1%/3% |
|  | CSI feedback payload 174bit | 1%/4% |
|  | CSI feedback payload 231bit | 1%/3% |
|  | CSI feedback payload 250bit | 1%/2% |
|  | CSI feedback payload 285bit | 1%/4% |
|  | CSI feedback payload 335bit | 1%/3% |
| Gain for Mean UPT, [rank 2] | CSI feedback payload 67bit | 11% |
|  | CSI feedback payload 96bit | 9% |
|  | CSI feedback payload 117bit | 3% |
|  | CSI feedback payload 174bit | 4% |
|  | CSI feedback payload 231bit | 4% |
|  | CSI feedback payload 250bit | 1% |
|  | CSI feedback payload 285bit | 4% |
|  | CSI feedback payload 335bit | 3% |
| Gain for 5% UPT [rank 1] | CSI feedback payload 67bit | 4% |
|  | CSI feedback payload 96bit | 4% |
|  | CSI feedback payload 117bit | <1% |
|  | CSI feedback payload 174bit | 4% |
|  | CSI feedback payload 231bit | <1% |
|  | CSI feedback payload 250bit | 1% |
|  | CSI feedback payload 285bit | 2% |
|  | CSI feedback payload 335bit | 1% |
| FFS others | / | / |