3GPP TSG-RAN WG1 Meeting #112bis-e			R1-2302549

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.4.1.1

Title:	On channel access mechanism and resource allocation for SL-U

Source:	OPPO

Document for:	Discussion and Decision

# 1. Introduction

In RAN#99-e, WID for the R18 NR sidelink evolution project was revised [1], but the objective for SL-U was not changed. On the progress for channel access and resource allocation in SL-U, the following agreements were reached in the last RAN1#112 meeting [2].

Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2 Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

In this contribution, we will provide discussions and our views on the following technical topics:

- Further details of channel access mechanisms for SL-U

  - Type 1 and Type 2 (2A/2B/2C) channel access procedures

  - Contention window adjustment

  - UE-to-UE COT sharing

  - CP extension (CPE)

  - Multi-channel access for S-SSB and PSFCH

- Multi-consecutive slots transmission (MCSt)

# 2. Discussion

## 2.1 Further details on Type 1 and Type 2 (2A/2B/2C) channel access procedures

Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs Agreement - Type 1 SL channel access procedure is applicable to the following transmissions by a UE: - PSSCH/PSCCH transmission(s) scheduled or configured by a gNB in SL Mode 1 resource allocation. - PSSCH/PSCCH transmission(s) from the UE in SL Mode 2 resource allocation. - Other SL transmissions including S-SSB and PSFCH transmissions from a UE - FFS: how to set CAPC for S-SSB and PSFCH - Note: Type 1 can be used to initiate a COT - A UE uses a channel access priority class applicable to the sidelink user plane data multiplexed in PSSCH for performing the Type 1 channel access procedures to transmit transmission(s) including PSSCH with user plane data and its associated PSCCH. - Note: how to set CAPC for MAC CE multiplexed in PSSCH is up to RAN2 - A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds the maximum COT duration where the channel access procedures are performed based on a channel access priority class p passociated with the UE transmissions, as given in CAPC table for SL. Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: For p=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms.. Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

As part of the remaining details for Type 1 SL channel access procedures, since RAN1 has agreed on the CAPC table for sidelink and the NR-U DL CW adjustment mechanism is used as the baseline for SL-U when SL-HARQ feedback is enabled in SCI for unicast, it is also necessary to agree on the 6-step of the Type 1 channel access procedures from NR-U (including definition of Ninit, decrement of the random counter, defer duration, sensing slot duration, etc.) for the SL-U operation.

Proposal 1: To directly reuse the 6-step of Type 1 channel access procedures from NR-U (specified in TS37.213) for the SL-U Type 1 SL channel access procedures.

- FFS on the additional LBT sensing duration (e.g., the length is based on a (pre-)configured value)

Specifically, the following text for specification could be adopted for SL-U.

A UE may transmit a SL transmission using Type 1 SL channel access procedures after first sensing the channel to be idle during the sensing slot durations of a defer duration Td, and after the counter N is zero in step 4. The counter N is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps described below. 1) set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4; 2) if N>0 and the UE chooses to decrement the counter, set N=N-1; 3) sense the channel for an additional sensing slot duration, and if the additional sensing slot duration is idle, go to step 4; else, go to step 5; 4) if N=0, stop; else, go to step 2. 5) sense the channel until either a busy sensing slot is detected within an additional defer duration Td or all the sensing slots of the additional defer duration Td are detected to be idle; 6) if the channel is sensed to be idle during all the sensing slot durations of the additional defer duration Td, go to step 4; else, go to step 5; The defer duration Td consists of duration Tf=16μs immediately followed by mp consecutive sensing slot durations where each sensing slot duration is Tsl=9μs, and Tf includes an idle sensing slot duration Tsl at start of Tf. CWmin,p≤CWp≤CWmax,p is the contention window. CWp adjustment is described in clause x.x.x. CWmin,p and CWmax,p are chosen before step 1 of the procedure above. mp, CWmin,p, and CWmax,p are based on a channel access priority class p as shown in Table x.x.x. For p=3 and p=4 , if the absence of any other technology sharing the channel can be guaranteed on a long term basis (e.g. by level of regulation or (pre-)configuration), Tmcot,p=10ms, otherwise, Tmcot,p=6ms.

For the FFS issue on whether the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 can be provided, it is our understanding, this parameter can be provided by gNB when under a network gNB coverage (for both Mode 1 and Mode 2 RA) or pre-configured when the UE is dedicated to be used in a certain environment (e.g., in a private factory setting). Even if the semi-static FBE channel access procedure is not supported for SL-U in Rel-18, the use of a longer maximum COT duration for p=3, 4 is still applicable and beneficial.

Proposal 2: For p=3,4, the current NOTE1 in the agreed CAPC table for SL in the Type 1 SL channel access procedures should be supported. The higher layer parameter name should be changed to “absenceOfAnyOtherTechnology-r18”.

For the FFS issue on which conditions Type 2B or Type 2C is applied in case of a transmission gap is 16μs, first of all, it should be noted that there is already an additional condition according to the existing agreement that the duration of the corresponding transmission for SL is at most 584µs to apply Type 2C. So, specifically the case of ambiguity is when the transmission gap is exactly 16µs (e.g., when CPE is used before the next AGC symbol) and the corresponding SL transmission is equal to or less than 584µs. In such a case, the transmission gap of 16µs is always known to the UE in advanced. To save the SL UE from performing a Type 2B LBT sensing and the need to perform a RX-TX switching, in our view, the UE should perform Type 2C channel access procedure to save effort, processing and power.

Proposal 3: For the case when a transmission gap is 16us and the corresponding SL transmission is not more than 584µs, Type 2C channel access procedure should be performed by the UE to save effort, processing and power from performing a LBT sensing in Type 2B and RX/TX switching.

Another open issue is whether Type 2A channel access can be applied to PSFCH without a shared channel occupancy (just as S-SSB transmissions according to the above agreement from the last RAN1#111 meeting). As discussed earlier, PSFCH carrying SL-HARQ feedback control information is important for sidelink communication performance, and hence it should be treated with highest priority. Note that, from short control signaling exemption perspective, it is already allowed by ETSI regulation [4] for a device to perform this type of transmission as long as a certain duty cycle and Tx duration are followed/obeyed within an observation period of 50ms. Therefore, as long as channel is idle for at least 25us, it should be allowed for a UE to transmit PSFCH after performing a Type 2A LBT procedure (just as S-SSB). Note that, when UE is in network coverage operating in Mode 1 or Mode 2 and follows network timing for synchronization, the UE does not need to transmit S-SSB. Furthermore, a UE only transmit PSFCH when SL-HARQ feedback is enabled in a received SCI in groupcast and unicast. Although PSFCH resources can be configured periodically in a resource pool, the UE transmit SL-HARQ feedback only when it is triggered in SCI. As such PSFCH transmissions should consider to be aperiodic and the duty cycle constrain for S-SSB would not be applicable for PSFCH.

Proposal 4: Type 2A channel access should be applied to PSFCH transmissions without a shared channel occupancy with the same time duration constrain as S-SSB.

Proposal 5; Further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure could be:

- For each of S-SSB and PSFCH transmission from a UE, the combined duration of S-SSB and PSFCH transmissions using Type 2A channel access out of shared COT should not exceed 2500us and the combined number of S-SSB and PSFCH transmissions using Type 2A channel access out of shared COT should not exceed 50 in an observation period of 50ms.

- Otherwise, Type 1 channel access should be applied.

Another open issue is whether and how to apply channel access to groupcast SL feedback option 1 (i.e., NACK-only feedback). If channel access is applied to GC feedback option 1, LBT failure is possible even if more transmission occasions are configured for PSFCH. In this case, PSSCH RX UE will not perform NACK feedback if it does not detect PSSCH correctly. Then the PSSCH TX UE does not receive any feedback from RX UE. But it cannot differentiate whether it is because of LBT failure, or PSSCH correctly detected. That will cause ambiguous and PSSCH TX UE does not what to do. If the non-reception of PSFCH from the RX UE is due to LBT failure, which means RX UE want to transmit NACK, TX UE should perform retransmission. If due to RX UE have detected PSSCH correctly, TX UE should not perform retransmission. Since this ambiguity problem / question has been studied for 2 meetings already and no good solution has been found (besides the short control signaling mechanism to avoid the need to perform channel access procedure), it is proposed not to support SL-HARQ groupcast option 1 for SL-U in Rel-18.

Observation 1: For groupcast feedback option 1, if channel access procedure is applied (even for Type 2A without shared channel occupancy), it will cause ambiguity at PSSCH TX UE when no PSFCH is received. The TX UE cannot determine whether or not to perform PSCCH/PSSCH retransmission.

Proposal 6: To avoid the ambiguity problem when no PSFCH is received at PSCCH/PSSCH TX UE for groupcast option 1, it is proposed not to support SL-HARQ groupcast option 1 (NACK-only PSFCH feedback) for SL-U in Rel-18.

For Type 1 CA, it is a counter based random back-off channel access mechanism. In NR-U, when the UE/gNB starts the Type 1 procedure is not specified, and up to implementation. When will the counter equal to 0 is not predictable, and depends on the congestion level of shared spectrum. If the counter is equal to 0 and UE/gNB does not perform transmission, an additional short LBT (corresponding length is Td+Tsl) is performed before the transmission to sense whether the channel is idle or not. The motivation of the additional short LBT is to avoid potential collision with other RATs since other RAT have opportunity to access to the channel during the gap between the time when counter is equal to 0 and slot boundary for transmission.

The description of Type 1 CA for UE is copied below [5].

“If a UE has not transmitted a UL transmission on a channel on which UL transmission(s) are performed after step 4 in the procedure above, the UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the UE is ready to transmit the transmission and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before the transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the UE first senses the channel after it is ready to transmit, or if the channel has not been sensed to be idle during any of the sensing slot durations of a defer duration Td immediately before the intended transmission, the UE proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td. ”

Observation 2: For Type 1 channel access in NR-U, when the UE/gNB starts the Type 1 procedure is not specified, and up to implementation.

Observation 3: For Type 1 channel access in NR-U, an additional short LBT is performed before transmission if there is gap between the time when counter is equal to 0 and the starting time of the transmission.

In NR-U, the duration of the additional short LBT is  Td+Tsl, where Td=Tf+mp∙Tsl, and Tf=16us, Tsl=9us, mp is determined by UL or DL CAPC level. For SL-U, we have agreed to reuse the following table which is based on NR-U UL CAPC table in RAN1#110-bis.

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

For different CAPC level, the duration of additional short LBT is 43µs, 43µs, 52µs, 88µs corresponds to different CAPC levels.  Furthermore, UE needs to perform SL transmission after successful short LBT so that RX/TX switching time is needed. For FR1, the TX/RX switching is about 13µs [7]. The total required duration for additional short LBT and RX/TX switching time is summarized in the table.

Table 1: Required time duration to perform additional short LBT in Type 1 channel access procedures and RX/TX switching for different CAPC level.

| CAPC level | Required duration for additional short LBT (µs) | Required duration for short LBT and RX/TX switching (µs) |
| --- | --- | --- |
| 1 | 43µs | 56µs |
| 2 | 43µs | 56µs |
| 3 | 52µs | 65µs |
| 4 | 88µs | 101µs |

If the UE needs to perform SL transmission in slot n and there is already another SL transmission in slot n-1, UE can only perform additional short LBT within GP symbol of slot n-1. The GP symbol length is about 71µs/35µs/16µs for 15kHz/30kHz/60kHz SCS respectively. For 15kHz SCS, the required duration of additional short LBT can be fulfilled only for CAPC level 1, level 2 and level 3, not for CAPC level 4. For 30kHz and 60kHz SCS, the required duration of additional short LBT cannot be fulfilled for all CAPC levels. If additional short LBT fails, the whole Type 1 channel access procedure is seen as failure. If there is SL transmission in previous slot, UE cannot have enough time to finish the additional short LBT procedure which will cause Type 1 LBT failure. In this case, it is preferred that UE can finish the additional short LBT procedure within GP symbol.

Observation 4: The duration of additional short LBT in Type 1 and RX/TX switching is larger than duration of GP symbol in most cases.

Observation 5: If there is SL transmission in previous slot, UE cannot finish additional LBT failure within GP symbol.

Proposal 6: For Type 1 channel access in SL-U, the duration of additional short LBT is shorter than GP symbol is preferred.

One potential solution to solve the issue is to support (pre-)configured duration of additional short LBT. For example, the duration of additional short LBT is Tsl, which corresponds to one sensing slot.

Proposal 7: For Type 1 channel access in SL-U, (pre-)configure the duration of additional short LBT can be supported.

One FFS point of Type 2B is whether the gap is between 16us and 25us. In our view, we think Type 2B can be applied in case the gap is between 16us and 25us. Considering RX/TX switching time in FR1 is about 13us, if the gap is exactly 16us, there is only 3us left for UE to perform sensing. UE cannot perform sensing within such short duration.

Proposal 8: Type 2B channel access procedure is applicable when the gap is equal to or larger than 16μs and less than 25μs in a shared channel occupancy.

## 2.2 Further details on contention window adjustment

During the last few meetings, it was agreed that NR-U DL CW adjustment is used as the baseline for SL-U CW adjustment. In NR-U, it is supported to adjust contention window based on the HARQ feedback(s) corresponding to the PDSCH(s) within DL reference duration. As a key point for CW adjustment, the definition of SL-U reference duration was extensively discussed and the following agreement was achieved:

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

According to the above agreement, the SL reference duration is determined based on PSSCH with ACK/NACK HARQ-ACK enabled and the SL reference duration is not used when such PSSCH cannot be found. However, whether or how to define the SL reference duration for NACK-only case can still be further discussed. In SL-U, RX UE is not able to transmit HARQ feedback(s) to TX UE when LBT failure occurs. Correspondingly, it is also hard for TX UE to judge whether the data was decoded successfully or not when NACK was not detected in NACK-only based operation. In our view, it is still unclear how to support NACK-only based feedback in SL-U. Thus, we think NACK-only based feedback should not be taken into consideration for CW adjustment.

Proposal 9: NACK-only based feedback is not suitable for CW adjustment.

The feature of multi-consecutive slots transmission (MCSt) was already agreed in SL-U, which is similar with the concept of transmission burst in NR-U. Based on the above agreements, it is open to discuss another ending time of SL reference duration especially for the case of MCSt. In NR-U, DL reference duration may terminate at the end of the first transmission burst that contains unicast PDSCH(s). By following the same principle, the end of the first MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled can be regarded as another ending time of SL reference duration.

Proposal 10: For MCSt, SL reference duration can terminate at the end of the first MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled.

In NR-U, when DL transmissions are not associated with explicit HARQ-ACK feedbacks, the contention window keeps the same as previous adjusted window size of the same CAPC level. This mechanism can be reused in SL-U. If a UE transmits SL transmissions using Type 1 channel access procedure and the transmissions are not associated with explicit SL-HARQ feedback, the contention window keeps the same as previous adjusted window size of the same CAPC level. As we mentioned in the above, it is unclear whether and how to support NACK-only based feedback in SL-U. Therefore, we prefer to consider PSSCH transmissions with NACK-only based feedback same as blind transmissions during CW adjustment.

Proposal 11: For PSSCH transmission not associated with explicit SL-HARQ feedback or associated with NACK-Only based feedback:

- For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures.

For the case of groupcast option 2 (A/N based SL feedback), the preferred approach is to calculate the ratio of received ACK such that the state of interference and quality of channel for groupcast option 2 can be reflected and further used for CW adjustment. If the ratio of received ACK is larger or equal than a (pre-)configured ratio, UE can set the contention window size to the minimum allowed value, otherwise the contention window is adjusted to next higher allowed value. More specifically, the ratio of received ACK is the number of actually received ACK divided by the number of HARQ feedbacks that are expected to be received by TX UE.

Proposal 12: CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (at least in case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration):

- Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks corresponding to the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

- The ratio of received SL HARQ-ACK feedbacks refers to the ratio of received ACK.

Furthermore, it is also unclear how MCSt works at this stage (e.g., MCSt is for the transmission of same TB or different TBs) so we think CW adjustment for MCSt can be discussed after RAN1 works out some details of MCSt.

## 2.3 Further details on UE-to-UE COT sharing

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE. Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

In RAN1#109-e meeting [6], it was agreed to support UE-to-UE COT sharing. While the details need FFS. In NR-U, a COT can be initiated or shared only when a device (gNB or UE) accesses the channel through Type 1 channel access. This principle can also be applied to SL-U: only when a UE access the channel through Type 1 channel access, it can share a COT to other UEs.

When a responding UE receives a shared COT from another UE, it should not forward or relay the shared COT to other SL UE since the COT is not initiated by the responding UE. Furthermore, the interference experienced at COT initiating UE and responding UE are different. The COT initiating UE can perform channel access procedure successfully, which does not mean the responding UE will. Therefore, COT forwarding / relaying violets the principle of that LBT sensing outcome is naturally restricted and only valid within a geographical area. By forwarding / relaying an initiated COT, UEs that are far away from the COT initiator who would naturally sense a different channel environment will cause an unfairness access to the channel to other RAT devices.

Proposal 13: Information relating to an initiated COT can only be shared by the COT initiator UE who has successfully completed Type 1 SL channel access procedure; A responding UE forwarding/relaying information about a COT initiated by another UE is not allowed in SL-U.

When an initiator UE shares its COT to other UEs, the following information should be carried in the COT sharing information:

- CAPC level: this is used to determine which UE can use the shared COT. Only when another UE’s CAPC priority is higher or equal to the indicated CAPC level, the UE can use the shared COT;

- RB set: this is used to indicate the RB set(s) on which the COT initiating UE performs channel access successfully. Only when a responding UE’s SL transmission is within the indicated RB sets, it can use the shared COT. That is, when the RB sets correspond to the SL transmission of the responding UE is a subset of the RB sets of the COT initiating UE’s transmission, the responding UE can use the shared COT, otherwise not.

- COT duration or COT length (remaining):  this is used to determine the remaining COT duration. A full COT duration starts from the moment when the initiator UE starts transmitting on the channel. By the time the initiator UE starts its transmission (e.g., at the slot boundary of the next slot or at the 2nd starting symbol within a slot), the UE may indicate the COT sharing information. In some cases, the initiator UE may only share its COT at the end of MCSt. As such, the initiator UE should indicate only the remaining COT duration.

- Additional ID(s): besides the source and destination IDs carried in the 2nd SCI from the COT initiating UE, other ID(s) of unicast/groupcast/broadcast session(s) to which the COT initiating UE also belongs could be also indicated as part of the COT sharing information. This is to support the case of a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating UE’s PSCCH/PSSCH transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

The COT sharing information is used to assist another UE to share the COT. It can be carried by SCI or MAC CE.  Considering the COT sharing can be supported for unicast/groupcast/broadcast, it cannot be carried in PC5-RRC. SCI or MAC CE is more suitable to carry the COT sharing information.

As mentioned several times previously, SL-HARQ feedback control information carried in PSFCH is vitally important to SL communication performance and resource utilization efficiency (i.e., UE performs retransmission only when necessary). Therefore, besides the case a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE, we would also like to support the case to allow a responding UE to transmit PSFCH(s) to UE(s) other than the initiator. That is, a responding UE can utilize a shared COT to transmit any PSFCH even when it is not intended for the COT initiating UE.

Proposal 14: For UE-to-UE COT sharing:

- At least the following information should be carried in COT sharing information: CAPC, RB set information, remaining COT duration, and optionally additional ID(s);

- Only when the RB sets correspond to the SL transmission of the responding UE is a subset of the RB sets of the COT initiating UE’s transmission, the responding UE can use the shared COT, otherwise not.

- In case of PSFCH, a responding UE is allowed to transmit PSFCH(s) to UE(s) other than the initiator when none of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

- The COT sharing information is carried in SCI or MAC CE.

When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s). The responding UE can perform any kind of SL transmissions, such as PSCCH/PSSCH, PSFCH and/or S-SSB.

For the last FFS about whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA, the benefit to support such feature is not clear. The maximal COT duration is 10ms. If UE report a COT to gNB, it needs to obtain UL resource for such kind of reporting which needs some time. And gNB can only schedule SL transmission within the remaining COT duration to make use of the shared COT. Whether it can provide benefit is not clear. On the other hand, it will result in more specification work to support such feature.

Proposal 15: Mode 1 UE reporting a COT or related information to gNB is not supported in Rel-18.

During the discussion in the last RAN1#112 meeting, source and destination ID matching for COT sharing was discussed. In Rel-16, the full 24-bit Layer-2 source and destination IDs for a TB associated to unicast are carried in MAC PDU subheader and SCI as (similar for groupcast and broadcast):

- the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI;

- the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI;

The discussion in RAN1#112 was whether the source and destination ID matching for COT sharing should consider only the bits carried in SCI or it should be the full 24-bit Layer-2 IDs. In our view, if the full 24-bit Layer-2 IDs should be considered and used, the ID matching process (for both the source/destination IDs in SCI and the additional IDs) for identifying whether a shared COT can be utilized or not will have the following impacts:

- A receiver UE needs to decode both L1 IDs carried in SCI and the remaining L2 IDs carried in the MAC PDU. The UE will need to additionally decode MAC PDU to extract such information and cause delay to the overall COT sharing determination due to the additionally processing. And the COT sharing process will no longer be just a L1 function.

- If the full 24-bit IDs are to be carried in SCI to minimize the processing latency, it will cause impact/changes to both existing TS28.212 and TS38.321 specifications and process, which in our view is highly undesirable. Furthermore, it will increase the payload in SCI significantly and degrade SCI decoding performance. Considering additional ID(s) could be carried in SCI as well, it will further restrict the number of additional IDs that can be carried in the COT sharing information.

Since the 8-bit source ID and 16-bit destination ID are deemed to be accurate enough in Rel-16 for V2X with potentially hundreds of UEs on the road in proximity (i.e., with minimal ID matching collision probability), in our view the existing source/destination ID format in SCI-2 is sufficient for the ID matching in COT sharing. That is, total of 24 bits for unicast and 16 bits for groupcast/broadcast per ID matching.

Proposal 16: For source/destination ID matching in COT sharing, without causing the processing delay at the receiver UE and/or changes to the existing 212 and 321 specifications, the existing L1 source/destination ID format should be reused.

## 2.4 Further details on CP extension (CPE)

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

During RAN1#112, the design on CPE transmission was extensively discussed. The discussion was very complicated since it relates to UE performing Type 1 LBT to gain access to the channel (i.e., the additional LBT sensing), UE performing Type 2 LBT to utilized a shared COT and at the same time how to support FDM of different SL transmissions (from multiple UEs) in the same slot. To make things even more complicated, the GP symbol lengths are different for different SCS and we should also consider both the 1st and 2nd candidate starting symbols within a slot for PSCCH/PSSCH transmission.

On the topic of the 2nd candidate starting symbol for PSCCH/PSSCH transmission, it should be noted that:

- There is no GP symbol before the next AGC symbol for the intended SL transmission. This means, it is not possible for a SL TX UE to perform Type 2B and 2C LBTs to gain access to the channel since there is no gap of 16µs from the previous PSCCH/PSSCH transmission to the next AGC symbol.

- However, it is still possible for a SL TX UE to perform a Type 2A LBT when there is no SL transmission from the 1st candidate starting symbol, since the LBT sensing requirement is “at least” 25µs.

- Of course, it is always possible for a UE to perform Type 1 LBT to gain access to the channel from the 2nd candidate starting symbol.

Based on the latest agreement from RAN1#112 (captured in the above), the intention of CPE design should allow SL TX UE to access the channel as early as possible within the time boundary of 1 symbol before the next AGC symbol in 15kHz SCS and 2 symbols in 30kHz and 60kHz SCSs. This means, if there is no other SL or WiFi transmission in the channel, then the earliest timing opportunity that a UE is allow to transmit CPE for an intended SL transmission (regardless PSFCH, S-SSB, PSCCH/PSSCH in the 1st / 2nd candidate starting symbols) should be 1 symbol (15kHz) or 2 symbols (30/60kHz) before the next AGC symbol.

Figure 1: A set of (pre-)configured CPE starting positions within 1 symbol (CPE window) for 15kHz SCS before the next AGC symbol of the intended SL transmission.

Figure 2: A set of (pre-)configured CPE starting positions within 2 symbols (CPE window) for 30kHz and 60kHz SCS before the next AGC symbol of the intended SL transmission.

Similarly, the latest timing opportunity for a UE to transmit CPE / access the channel would be at the beginning of the next AGC symbol (meaning the CPE length is zero). This should be possible assuming there is a WiFi transmission and it stops at some point before the next AGC symbol, where a SL TX UE could still perform and complete LBT sensing just before the next AGC symbol. Therefore, there is a window of opportunity for a SL TX UE to perform CPE transmission and this CPE window would be:

- The beginning of the CPE window in 15kHz SCS is 1 symbol before the next AGC symbol (max CPE length = 1 symbol) as shown in Fig. 1 for 15kHz SCS case;

- The beginning of the CPE window in 30kHz and 60kHz SCS is 2 symbols before the next AGC symbol (max CPE length = 2 symbols) as shown in Fig. 2 for 30kHz and 60kHz SCS cases;

- The ending of the CPE window in all SCS would be just before the next AGC symbol (min CPE length is zero);

  - If a UE RX/TX switching time should be taken into account (switching from LBT sensing to AGC transmitting), the ending time could be defined as RX/TX switching time before the next AGC symbol (e.g., 13µs according to R16 agreement [7]) as shown in Fig. 1 for 15kHz SCS case; If a UE is capable of performing RX/TX switching earlier than 13us, the UE may transmit CPE until the next AGC symbol;

Proposal 17: A CPE window can be defined as:

- In 15kHz SCS, the beginning of the CPE window 1 symbol before the next AGC symbol of the intended SL transmission (max CPE length = 1 symbol);

- In 30kHz and 60kHz SCS, the beginning of the CPE window 2 symbols before the next AGC symbol of the intended SL transmission (max CPE length = 2 symbols);

- The ending of the CPE window in all SCS is just before the next AGC symbol (min CPE length is zero);

  - A RX/TX switching can be taken into account for the ending of the CPE window (e.g., 13us). If a UE is capable of performing RX/TX switching earlier, a CPE may be transmitted until the next AGC symbol.

Taking into consideration of different SL TX UE may need to perform LBT sensing of different lengths (i.e., 43µs and 55µs for Type 1 LBT, 16µs and 25µs for Type 2A/2B/2C LBT), a set of CPE starting positions within the CPE window could be (pre-)configured, including assigning one of the positions to be the default position. When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, SL TX UE transmits CPE from the default CPE starting position. Otherwise (when no existing reservation is detected), a CPE starting position is selected base on UE’s priority and LBT sensing requirement for PSCCH/PSSCH transmission to avoid transmission collision among SL TX UEs.

For PSFCH and S-SSB, on the other hand, a default CPE starting position should be always used for CPE transmission for multiplexing different SL-HARQ feedbacks in the same symbol and avoiding inter-UE blocking for S-SSB. However, the earliest and the latest CPE transmission opportunity timings are still the beginning and the end of the CPE window, respectively.

Alternatively, instead of using/conforming to a (pre-)configured set of CPE starting positions, only a default CPE starting position is (pre-)configured within the CPE window. When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, a SL TX UE transmits CPE from the default CPE starting position. Otherwise (when no existing reservation is detected), the UE transmits its CPE as soon as it has completed its LBT sensing requirement (e.g., 25µs, 43µs, 52µs). The main benefit of not conforming to a set of preset CPE positions is to allow UE to start transmitting CPE as soon as the channel becomes idle and LBT sensing is completed. For example, SL TX UEs monitor the channel at the same time and start LBT sensing as soon as the channel becomes idle (e.g., after a SL or WiFi transmission has stopped). Then based on LBT sensing requirement of different UE, high priority transmissions would complete LBT first and start transmitting CPE earlier than lower priority ones.

Finally, another approach to determine a CPE starting position for a SL TX UE could be based on SL Mode 1 and Mode 2 RA separation. That is, when UE is under a network gNB control in a Mode 1 resource pool, all SL transmissions are scheduled by the gNB (just like NR-U). In this case, multiple CPE starting positions could be configured for the resource pool and the gNB indicates which one of the configured positions to be used by the UE. As such, there would be no inter-UE blocking issue and FDM for multiple SL transmissions in a same slot can be easily achieved.

When SL TX UE is operating in SL Mode 2 RA, a single default CPE starting position can be (pre-)configured and all UEs use the same CPE starting position to achieve FDM transmissions among different UEs. And this should be used regardless the UE is initiating or sharing a COT, or the SL transmission is partial or full RB set allocation.

Proposal 18: To determine a CPE starting position for a SL TX UE, one of the following approaches could be adopted.

- A set of CPE starting positions within the CPE window is (pre-)configured according to different LBT sensing requirement (e.g., 16µs, 25µs, 43µs, …) and one of which is assigned as the default CPE starting position.

  - When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, SL TX UE transmits CPE from the default CPE starting position.

  - Otherwise, a CPE starting position is selected base on UE’s priority and LBT sensing requirement.

- Only a default CPE starting position is (pre-)configured within the CPE window.

  - When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, SL TX UE transmits CPE from the default CPE starting position.

  - Otherwise, the SL TX UE transmits its CPE as soon as it has completed its LBT sensing requirement (e.g., 25µs, 43µs, 52µs) for Type 1 or one of Type 2 LBTs.

- In SL Mode 1, multiple CPE starting positions are configured within the CPE window and the gNB indicates which one of the configured positions to be used by the UE. In SL Mode 2, a single default CPE starting position can be (pre-)configured within the CPE window and all UEs use the same CPE starting position to achieve FDM transmissions among different UEs.

## 2.5 Further details on multiple-channel access

Multiple-channel access was discussed in RAN1#110-bis-e and the following agreements were achieved:

Agreement Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation Agreement For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation Agreement - For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

Regarding the first FFS, we need to discuss whether per RB set based PSFCH or S-SSB transmission is supported. For PSFCH, its frequency resource can be interlaced RB based which is confined within one RB set. It is possible that multiple PSFCH on multiple RB sets need to be transmitted by a UE in the same slot respectively if their corresponding PSSCH are within different RB sets. When multiple PSFCH within multiple RB sets to be transmitted, UE performs channel access on each RB set. For each RB set, if the channel access success, the corresponding PSFCH within the RB set can be transmitted. Otherwise not. The multiple-channel access schemes of NR-U DL (Type A) should be taken as baseline for multi-channel access of PSFCH, since SL-HARQ feedbacks should be considered as high priority. If ACK is not received by the TX UE due to LBT sensing failure, it will cause unnecessary retransmission of the same TB and increase channel congestion.

For S-SSB, its frequency resource can also be confined within one RB set. We think it is beneficial to support S-SSB transmission on multiple RB sets which can assist other SL UEs to perform synchronization using available RB sets. Which RB set can be used for S-SSB transmission depends on channel access procedure on these RB sets respectively. For each RB set, if the channel access success, the corresponding S-SSB within the RB set can be transmitted. Otherwise not. The multiple-channel access schemes of NR-U DL can be taken as baseline for multi-channel access of S-SSB. Similar to PSFCH, S-SSB transmissions on multiple channels can be also beneficial in terms of maintaining a shared COT. However, it is noted that the decision on whether to transmit S-SSB in more than one RB set should be made first in the PHY structure agenda.

Proposal 19: For PSFCH transmissions on multiple channels, NR-U DL Type A multi-channel access procedure should be used as the baseline. It is not necessary to support the Type B DL multi-channel access procedure from NR-U.

Proposal 20: For S-SSB transmissions on multiple channels (if agreed in the PHY structure agenda), NR-U DL Type A multi-channel access procedure should be used as the baseline. It is not necessary to support the Type B DL multi-channel access procedure from NR-U.

## 2.6 Multi-Consecutive Slots transmission (MCSt)

In RAN1#110, the following agreement was achieved for multi-consecutive slots transmission:

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

The main advantage of MCSt is to perform back-to-back transmissions such that the initiated/shared COT can be occupied and retained as long as possible and the technologies of other RATs (e.g., Wi-Fi users) cannot access the channel during the COT of SL transmissions (to this end MCSt should occupy the GP symbol of each slot). However, it is still unclear how the MCSt works. In our understanding, there are three possible options for the operation mode of MCSt:

- Option 1: Each slot in the consecutive slots is used for one transmission (initial Tx or re-Tx) of the same TB – same HARQ process.

- Option 2: Each slot in the consecutive slots is used for one transmission (initial Tx or re-Tx) of different TBs – different HARQ processes.

- Option 3: One transmission across the consecutive slots – same TB is channel encoded over multiple slots.

For simplicity, let us use a concept of MCSt group to represent the consecutive slots in MCSt. In Option 1, each MCSt group only contains the (re)transmissions of one TB from the same HARQ process, which helps to simplify the resource reservation and allocation procedure for MCSt. As for Option 2, each MCSt group may include the (re)transmissions of different TBs from different HARQ processes. According to current resource allocation procedure of Mode 1 and Mode 2, the resources of different SL grants are determined independently. In particular, the resource exclusion of PHY layer and the resource selection of MAC layer are performed for single SL grant in current Mode 2 RA. Hence, there could be a significant spec impact and structure change for both L1 and L2 if the above Option 2 is supported. For example, the resources of other existing SL grants should be taken into consideration when scheduling or selecting the resources for a current SL grant. Option 3 introduces a new continuous SL transmission across multiple slots. Obviously, it breaks the current physical structure of slot-based SL transmission, such as the mapping of PSCCH and PSSCH, TB size determination, AGC training and so on.  Therefore, considering the large spec impact from adopting both Option 2 and Option 3, we propose to further study and work on the above Option 1 with higher priority for MCSt.

Proposal 21: For the study and future work on MCSt, slot-based MCSt for the same TB is considered with higher priority.

To support the slot-based MCSt for the same TB (i.e., above Option 1), a new type of resource reservation needs to be studied and introduced. In legacy sidelink, both time domain and frequency domain resource reservation and indication are based on the design per slot. To support MCSt in SL-U, how to indicate the consecutive slots and sub-channels for these slots should be considered.

- Time domain

According to current retransmission scheme in sidelink, a TB may be configured to be (re-)transmitted multiple times which is no more than 32. For MCSt in SL-U, when a TB is configured to be (re-)transmitted multiple times, the consecutive slots may be allocated in one MCSt group or different MCSt groups. In the current indication mechanism, the Time resource assignment field with 5 or 9 bits in DCI/SCI can only indicate 2 or 3 slots, which cannot reflect the resource reservation information or avoid conflict by sensing UEs for the case of MCSt. How to indicate the time domain resources should be further studied and designed.

- Case 1: All slots for (re-)transmissions of one TB are in one MCSt group, which means all the potential transmission slots of the TB are selected to be consecutive. The current time resource indication mechanism may not be able to ensure all the reserved slots are excluded by other UEs. One example is provided in Figure 4 to clarify such issue. UE2 is performing sensing and resource selection procedure, while UE1 is transmitting a TB by reserving 7 consecutive slots. UE2 can only obtain the information of slot #1 by sensing and slot #2/3 by indication. Slots #6/7 are not excluded from candidate slots set unless UE2 can have the reservation information by decoding SCIs in slot #4/5. However, even enabling re-evaluation and pre-emption checking, there always exists processing time gap (i.e., Tproc,0 and Tproc,1) without sensing results. Therefore, the legacy sidelink resource indication mechanism may not be enough. One possible solution is to indicate the length of the MCSt group, or the number of the remaining consecutive slots in the MCSt group. The indication can be an information field included in DCI and 1st stage SCI.

- Case 2: The total slots for transmitting one TB are divided into a few non-consecutive MCSt groups, and each MCSt group may have the same or different number of consecutive slots. For an example, a TB is decided to be (re-)transmitted up to 10 times by MAC layer, and 10 slots are selected and allocated into three groups, where group #1, group #2 and group #3 has 4, 4 and 2 consecutive slots, respectively. To indicate these groups of slots, starting position and number of consecutive slots per group should be included in the indication information (e.g., in SCI). The Time resource assignment field in DCI/SCI can be reused to indicate the starting position of the second and third group. However, this information field can only indicate up to 2 groups besides the current group. If the number of groups is more than 3, the indication bits should be extended. For the number of slots allocated in each group, a simple way is to (pre-)configure a fixed number of slots per group. If the number of slots in each group is dynamically changing, an extra indication field is needed in SCI. Furthermore, the remaining slots in each group should also be indicated to help resource exclusion by other UEs that perform sensing and resource selection, which is similar to the issue in Case 1. From the analysis above, Case 2 needs much more standards effort and it has a larger specification impact (e.g. more indication bits) than that of Case 1.

- Frequency domain

For a MCSt group which contains multiple consecutive slots (e.g. 4 slots), although the number of occupied sub-channels would be the same for one TB, the starting position of sub-channel in each slot may be different. The current Frequency resource assignment field in DCI/SCI can be used only to indicate 2 more starting position of the following slots. Extra bits are needed to indicate frequency starting positions if the number of following consecutive slots is more than 2. To simplify the design, the frequency starting positions in all the consecutive slots of a MCSt group can be same. Even such design may have some restrictions on frequency resource selection, it simplifies the resource reservation mechanism by dramatically decreasing the indication bits in DCI/SCI.

Proposal 22: In Rel-18 SL-U, for multi-consecutive slots transmission (MCSt), all the slots for a TB’s (re-)transmission can be consecutive.

- Further study the consecutive slots in MCSt are divided into multiple discrete groups.

Proposal 23: In Rel-18 SL-U, an information field can be included in SCI to indicate the length or the remaining slots in the MCSt group of consecutive slots.

Proposal 24: In Rel-18 SL-U, for multi-consecutive slots transmission (MCSt), the frequency starting positions in all the consecutive slots of a MCSt group can be the same for simplicity.

Figure 4. MCSt resource reservation and indication

In addition, resource allocation in Mode 2 also needs to be enhanced for MCSt. In RAN1#110bis-e, the following agreements for resource allocation in MCSt was approved:

Agreement: On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

According to the above agreements, there are two options for the set of parameters provided by higher layer when L1 is triggered to report the candidate resource set SA. In option 1, only one set of parameters is provided for resource exclusion and such behaviour is more aligned with the existing design of current specification. As for option 2, it intends to introduce a new framework that the resource (re-)selections for multiple TBs are triggered simultaneously and one or more candidate resource set SA is used to determine the transmission resources for multiple SL grants. Obviously, it will cause a significant impact on current resource allocation procedure of both physical and mac layer. Furthermore, it may require higher layer to provide different sets of selected/reserved resources associated with different sets of parameters during re-evaluation and pre-emption checking. As a result, the condition for re-evaluation and pre-emption checking in physical layer becomes more complicated if option 2 is supported. Therefore, we think only option 1 ought to be adopted for the resource (re-)selection triggering in MCSt.

Proposal 25: When L1 is triggered for reporting a subset of candidate resources for MCSt,

- Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

As for resource exclusion in L1, the simplest way is to redefine the single-slot candidate resource Rx,y as a multiple-slots candidate resource. In the current specification, Rx,y denotes a set of LsubCH contiguous sub-channels starting from sub-channel x in slot t'ySL. In order to support MCSt, we can redefine the Rx,y as a set of LsubCH contiguous sub-channels starting from sub-channel x in Lslot consecutive slots starting from slot t'ySL, where Lslot can be provided by the higher layer. Based on the new defined Rx,y, the candidate resource set SA includes a number of multi-slots candidate resources such that the resource exclusion in L1 and resource selection in L2 can be reused for MCSt. Therefore, option A in above agreements should be supported when L1 reports the candidate resource set SA to higher layer. Simultaneously, higher layer can provide the selected multiple-slots resource to physical layer for re-evaluation and pre-emption checking. The conditions for the existing re-evaluation and pre-emption checking can be reused as well. For example, the higher layer provides resource r1 to physical layer for re-evaluation checking, r1 corresponds to LsubCH subchannels in frequency domain and Lslot consecutive slots in time domain. If r1 is not included within the remaining candidate resource set SA  after resource exclusion, in which the candidate resource Rx,y has the same size with r1, physical layer reports r1 to the higher layer and then triggers the corresponding resource re-selection in SA.

Proposal 26: The single-slot candidate resource Rx,y in the current specification should be redefined as multi-slots candidate resource for MCSt with a size of LsubCH and Lslot, which are to be provided by the higher layer in Mode 2 resource (re)selection procedure.

Proposal 27: When L1 reports a subset of candidate resources for MCSt,

- Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time.

Proposal 28: Higher layer provides the selected / reserved multiple-slots resource(s) to physical layer for re-evaluation or pre-emption checking.

In Figure 5, the simulation result for legacy single-slot based transmission and MCSt is provided. It should be noted that MCSt for single TB is simulated and it is implemented according to above Option 1+Option A. Other simulation assumptions can be found in the appendix. As can be observed in Figure 5, the UPT of MCSt is larger than the UPT of single-slot based transmission. It is mainly due to SL transmissions occupy the channel continuously in MCSt such that a low latency is achieved. For example, the retransmission resource may be located tens of slots after initial transmission in single-slot based transmission while the retransmission is next to initial transmission in MCSt. Thus, the data transmission can be delivered more quickly in MCSt.

Figure 5. UPT of single-slot based transmission and MCSt

Observation 6: MCSt can achieve a higher data rate than legacy single-slot based transmission in SL-U.

# 3. Conclusion

Proposal 1: To directly reuse the 6-step of Type 1 channel access procedures from NR-U (specified in TS37.213) for the SL-U Type 1 SL channel access procedures.

- FFS on the additional LBT sensing duration (e.g., the length is based on a (pre-)configured value)

Proposal 2: For p=3,4, the current NOTE1 in the agreed CAPC table for SL in the Type 1 SL channel access procedures should be supported. The higher layer parameter name should be changed to “absenceOfAnyOtherTechnology-r18”.

Proposal 3: For the case when a transmission gap is 16us and the corresponding SL transmission is not more than 584µs, Type 2C channel access procedure should be performed by the UE to save effort, processing and power from performing a LBT sensing in Type 2B and RX/TX switching.

Proposal 4: Type 2A channel access should be applied to PSFCH transmissions without a shared channel occupancy with the same time duration constrain as S-SSB.

Proposal 5; Further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure could be:

- For each of S-SSB and PSFCH transmission from a UE, the combined duration of S-SSB and PSFCH transmissions using Type 2A channel access out of shared COT should not exceed 2500us and the combined number of S-SSB and PSFCH transmissions using Type 2A channel access out of shared COT should not exceed 50 in an observation period of 50ms.

- Otherwise, Type 1 channel access should be applied.

Observation 1: For groupcast feedback option 1, if channel access procedure is applied (even for Type 2A without shared channel occupancy), it will cause ambiguity at PSSCH TX UE when no PSFCH is received. The TX UE cannot determine whether or not to perform PSCCH/PSSCH retransmission.

Proposal 6: To avoid the ambiguity problem when no PSFCH is received at PSCCH/PSSCH TX UE for groupcast option 1, it is proposed not to support SL-HARQ groupcast option 1 (NACK-only PSFCH feedback) for SL-U in Rel-18.

Observation 2: For Type 1 channel access in NR-U, when the UE/gNB starts the Type 1 procedure is not specified, and up to implementation.

Observation 3: For Type 1 channel access in NR-U, an additional short LBT is performed before transmission if there is gap between the time when counter is equal to 0 and the starting time of the transmission.

Observation 4: The duration of additional short LBT in Type 1 and RX/TX switching is larger than duration of GP symbol in most cases.

Observation 5: If there is SL transmission in previous slot, UE cannot finish additional LBT failure within GP symbol.

Proposal 6: For Type 1 channel access in SL-U, the duration of additional short LBT is shorter than GP symbol is preferred.

Proposal 7: For Type 1 channel access in SL-U, (pre-)configure the duration of additional short LBT can be supported.

Proposal 8: Type 2B channel access procedure is applicable when the gap is equal to or larger than 16μs and less than 25μs in a shared channel occupancy.

Proposal 9: NACK-only based feedback is not suitable for CW adjustment.

Proposal 10: For MCSt, SL reference duration can terminate at the end of the first MCSt transmission that contains at least one PSSCH with ACK/NACK HARQ-ACK enabled.

Proposal 11: For PSSCH transmission not associated with explicit SL-HARQ feedback or associated with NACK-Only based feedback:

- For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures.

Proposal 12: CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (at least in case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration):

- Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks corresponding to the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

- The ratio of received SL HARQ-ACK feedbacks refers to the ratio of received ACK.

Proposal 13: Information relating to an initiated COT can only be shared by the COT initiator UE who has successfully completed Type 1 SL channel access procedure; A responding UE forwarding/relaying information about a COT initiated by another UE is not allowed in SL-U.

Proposal 14: For UE-to-UE COT sharing:

- At least the following information should be carried in COT sharing information: CAPC, RB set information, remaining COT duration, and optionally additional ID(s);

- Only when the RB sets correspond to the SL transmission of the responding UE is a subset of the RB sets of the COT initiating UE’s transmission, the responding UE can use the shared COT, otherwise not.

- In case of PSFCH, a responding UE is allowed to transmit PSFCH(s) to UE(s) other than the initiator when none of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

- The COT sharing information is carried in SCI or MAC CE.

Proposal 15: Mode 1 UE reporting a COT or related information to gNB is not supported in Rel-18.

Proposal 16: For source/destination ID matching in COT sharing, without causing the processing delay at the receiver UE and/or changes to the existing 212 and 321 specifications, the existing L1 source/destination ID format should be reused.

Proposal 17: A CPE window can be defined as:

- In 15kHz SCS, the beginning of the CPE window 1 symbol before the next AGC symbol of the intended SL transmission (max CPE length = 1 symbol);

- In 30kHz and 60kHz SCS, the beginning of the CPE window 2 symbols before the next AGC symbol of the intended SL transmission (max CPE length = 2 symbols);

- The ending of the CPE window in all SCS is just before the next AGC symbol (min CPE length is zero);

  - A RX/TX switching can be taken into account for the ending of the CPE window (e.g., 13us). If a UE is capable of performing RX/TX switching earlier, a CPE may be transmitted until the next AGC symbol.

Proposal 18: To determine a CPE starting position for a SL TX UE, one of the following approaches could be adopted.

- A set of CPE starting positions within the CPE window is (pre-)configured according to different LBT sensing requirement (e.g., 16µs, 25µs, 43µs, …) and one of which is assigned as the default CPE starting position.

  - When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, SL TX UE transmits CPE from the default CPE starting position.

  - Otherwise, a CPE starting position is selected base on UE’s priority and LBT sensing requirement.

- Only a default CPE starting position is (pre-)configured within the CPE window.

  - When at least one existing reservation is detected for the slot of the intended PSCCH/PSSCH transmission, SL TX UE transmits CPE from the default CPE starting position.

  - Otherwise, the SL TX UE transmits its CPE as soon as it has completed its LBT sensing requirement (e.g., 25µs, 43µs, 52µs) for Type 1 or one of Type 2 LBTs.

- In SL Mode 1, multiple CPE starting positions are configured within the CPE window and the gNB indicates which one of the configured positions to be used by the UE. In SL Mode 2, a single default CPE starting position can be (pre-)configured within the CPE window and all UEs use the same CPE starting position to achieve FDM transmissions among different UEs.

Proposal 19: For PSFCH transmissions on multiple channels, NR-U DL Type A multi-channel access procedure should be used as the baseline. It is not necessary to support the Type B DL multi-channel access procedure from NR-U.

Proposal 20: For S-SSB transmissions on multiple channels (if agreed in the PHY structure agenda), NR-U DL Type A multi-channel access procedure should be used as the baseline. It is not necessary to support the Type B DL multi-channel access procedure from NR-U.

Proposal 21: For the study and future work on MCSt, slot-based MCSt for the same TB is considered with higher priority.

Proposal 22: In Rel-18 SL-U, for multi-consecutive slots transmission (MCSt), all the slots for a TB’s (re-)transmission can be consecutive.

- Further study the consecutive slots in MCSt are divided into multiple discrete groups.

Proposal 23: In Rel-18 SL-U, an information field can be included in SCI to indicate the length or the remaining slots in the MCSt group of consecutive slots.

Proposal 24: In Rel-18 SL-U, for multi-consecutive slots transmission (MCSt), the frequency starting positions in all the consecutive slots of a MCSt group can be the same for simplicity.

Proposal 25: When L1 is triggered for reporting a subset of candidate resources for MCSt,

- Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1.

Proposal 26: The single-slot candidate resource Rx,y in the current specification should be redefined as multi-slots candidate resource for MCSt with a size of LsubCH and Lslot, which are to be provided by the higher layer in Mode 2 resource (re)selection procedure.

Proposal 27: When L1 reports a subset of candidate resources for MCSt,

- Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time.

Proposal 28: Higher layer provides the selected / reserved multiple-slots resource(s) to physical layer for re-evaluation or pre-emption checking.

Observation 6: MCSt can achieve a higher data rate than legacy single-slot based transmission in SL-U.

# 4. References

[1] RP-230077, “WID revision: NR sidelink evolution”, RAN#99

[2] RAN1 Chairman’s Notes, RAN1#112

[3] R1-2300017, “LS to RAN1 on CAPC for SL-U”, RAN2

[4] ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

[5] 3GPP TS37.213 V17.3.0, “Physical layer procedures for shared spectrum channel access”.

[6] RAN1 Chairman’s Notes, RAN1#109-e

[7] RAN1 Chairman’s Notes, RAN1#94

# 5. Appendix

| Parameter | Value |
| --- | --- |
| Layout | Indoor Scenario 1 – option 1 |
| Carrier frequency | 5 GHz |
| Simulation bandwidth | 20 MHz |
| Sub-carrier spacing | 30 KHz |
| Channel model | InH mixed office specified in TR 38.901 |
| UE Tx power | 18 dBm |
| UE number | 6 SL-U pairs and 4 STAs per AP per 20 MHz The minimum received power for UE pairing is -82 dBm. |
| Traffic model | SL-U: Periodic traffic model 3 Wi-Fi: FTP model 3 |
| Resource allocation scheme | R16 resource allocation mode 2 (full sensing) for single-slot based transmission. Option 1+Option A for MCSt. |
| ED threshold | -82 dBm |