3GPP TSG RAN WG1 #112bis-e		R1-2302564

e-Meeting, April 17th – April 26th, 2023

Source:	OPPO

Title:	Discussion on coverage enhancement for NR NTN

Agenda Item:	9.9.1

Document for:	Discussion and Decision

Introduction

In RAN1#112 meeting, the necessity of DMRS bundling enhancement for PUSCH and potential solutions on Msg4 PUCCH repetition were extensively discussed, and the following agreements were made for further discussion [1]:

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design) Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, we continue to provide our analysis and view on the procedure and signaling to support Msg4 PUCCH repetition, including high-level procedure, repetition request or capability report, and reusing existing DCI field for dynamic indication. Then, we discuss the necessity of the enhancement on DMRS bundling when considering NTN-specific time/frequency pre-compensation.

Coverage enhancement for NR NTN

PUCCH enhancement for Msg4 HARQ-ACK

RSRP threshold and repetition factor configured in SIB

Regarding the procedures for Msg4 PUCCH repetition, two options of repetition request or capability report transmitted from UE to gNB has been discussed for several meetings, but no agreement is achieved. In the last meeting, some companies state that repetition request and capability report can be both supported and selected by gNB configuration. Then, a working assumption was proposed as follows:

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

According to the main bullet of the working assumption, the RSRP threshold and repetition factor are both optionally configured. In this case, the following 4 cases should be taken into account:

- Case 1: the RSRP threshold and repetition factor are configured in SIB

- Case 2: the RSRP threshold is not configured but the repetition factor is configured in SIB

- Case 3: the RSRP threshold is configured but the repetition factor is not configured in SIB

- Case 4: the RSRP threshold and repetition factor are not configured in SIB

Case 1 corresponds to the 1st sub-bullet of the working assumption, where repetition request is used. If the measured RSRP is lower than the configured RSRP threshold, the UE capable of Msg4 PUCCH repetition sends repetition request to the gNB to start up Msg4 PUCCH repetition procedure.

Regarding the maximum configurable value of the RSRP threshold in the FFS point, the currently defined RSRP-Range {0…127} in TS 38.331 can be reused to cover different performance requirement, where X = 127 corresponds to infinity. In addition, we provide the coverage performance evaluation results of one-shot Msg3 PUSCH and Msg4 PUCCH under the reference scenario defined in the WID [2]. It can be observed that the performance gap of Msg3 PUSCH is larger than that of Msg4 PUCCH, so a new RSRP threshold should be introduced for Msg4 PUCCH repetition.

Table 1. Performance gap of Msg3 PUSCH and Msg4 PUCCH without repetition under LEO-1200 set-1 satellite

| Physical channel | Required SNR [dB] | CNR [dB] | Performance gap [dB] |
| --- | --- | --- | --- |
| Msg3 PUSCH | -2.63 | -11.11 | 8.48 |
| PUCCH for Msg4 HARQ-ACK | -3.93 | -8.11 | 4.18 |

Proposal 1: The RSRP-Range {0…127} defined in current spec can be reused in Msg4 PUCCH repetition, and the maximum configurable value of the RSRP threshold is 127.

Proposal 2: A new RSRP threshold should be introduced for Msg4 PUCCH repetition.

Case 2 corresponds to the 2nd sub-bullet of the working assumption, where capability report is used. The UE capable of Msg4 PUCCH repetition should report its capability to the gNB regardless of channel quality. However, capability report may cause inappropriate PUCCH resource allocation. For example, when multiple repetition factors of {4,8} are configured via SIB, even with good channel condition, the UE capable of Msg4 PUCCH repetition has to perform at least 4 PUCCH repetitions, resulting in unnecessary PUCCH resource waste.

Observation 1: For Msg4 PUCCH repetition, UE indicating capability report regardless of channel quality may cause unnecessary PUCCH resource waste.

Case 3 is not excluded by the working assumption, so it should be considered for Msg4 PUCCH repetition and the specific UE behavior can be further discussed. Moreover, we note that even in R17 Msg3 PUSCH repetition, the Msg3 PUSCH repetition factor is optionally configured. If the UE sends repetition request but no repetition factor is configured in SIB, a default set of Msg3 PUSCH repetition factor {1,2,3,4} is applied. Thus, a similar UE behavior can be considered for Msg4 PUCCH repetition, i.e., if the UE requests Msg4 PUCCH repetition and no Msg4 PUCCH repetition factor is configured in SIB, a default set of Msg4 PUCCH repetition factor e.g., {1,2,4,8}, is applied.

BWP-UplinkCommon field descriptions

numberOfMsg3-RepetitionsList The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {n1, n2, n3, n4} (see TS 38.214 [19], clause 6.1.2.1).

Observation 2: In R17 Msg3 PUSCH repetition, the Msg3 PUSCH repetition factor is optionally configured.

Observation 3: In R17 Msg3 PUSCH repetition, if the UE sends repetition request but no repetition factor is configured in SIB, a default set of Msg3 PUSCH repetition factor {1,2,3,4} is applied.

Proposal 3: For Msg4 PUCCH repetition, the case that the RSRP threshold is configured but the repetition factor is not configured in SIB should be considered.

As for Case 4, if neither the RSRP threshold nor repetition factor is configured in SIB, the UE has no information of whether the network support Msg4 PUCCH repetition. In this case, the UE will not transmit repetition request or capability report to the gNB, and the legacy UE behavior is applied. In other words, if the gNB wants to enable Msg4 PUCCH repetition, either the RSRP threshold or repetition factor should be configured via SIB to implicitly inform the UE that Msg4 PUCCH repetition is supported in the network.

Proposal 4: When the RSRP threshold and repetition factor are not configured in SIB, the legacy UE behaviour is applied.

Signaling for repetition request or capability report

The signaling details of how to transmit repetition request or capability report from UE to gNB has been discussed in the last meeting, and three options are listed in the working assumption as follows:

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports

For Option A, the information can be conveyed by the configured PRACH resource which is associated with Msg4 PUCCH repetition. Table 1 shows that Msg4 PUCCH has better link budget than Msg3 PUSCH, so the request of Msg4 PUCCH repetition automatically triggers the request of Msg3 PUSCH repetition is reasonable. This can be achieved by associating a PRACH resource with the request of Msg3 PUSCH repetition and Msg4 PUCCH repetition simultaneously, which mitigates PRACH resource partitioning issue.

Observation 4: Associating a PRACH resource with the request of Msg3 PUSCH repetition and Msg4 PUCCH repetition simultaneously can mitigate PRACH resource partitioning issue.

For Option B, the seven reserved LCID codepoints (range 37-42) can be used to carry the information. Furthermore, four of the seven LCID codepoints are needed to support different types of CCCH, including CCCH size of 48/64bits for RedCap/non-RedCap UE, so the spec impact is non-trivial.

In addition, if the repetition request or capability report is transmitted by Msg3 PUSCH, there may be an ambiguity issue when UEs with and without Msg4 PUCCH repetition capability send Msg3 PUSCH simultaneously. For example, UE1 incapable of Msg4 PUCCH repetition and UE2 capable of Msg4 PUCCH repetition select the same PRACH resource to initiate a random access procedure, then the same TC-RNTI is applied and the same time-frequency resource is used for Msg3 and Msg4 transmission. In this case, when the Msg3 PUSCH of UE1 is successfully received by the gNB, UE2 cannot realize it and will assume its Msg3 PUSCH was received. Then, if the repetition factor is indicated via the existing field in DCI, UE2 may mistakenly perform Msg4 PUCCH repetition, resulting in continuous interference.

Option C raises the same ambiguity issue and blind detection is additionally needed to identify the request or capability indication of Msg4 PUCCH repetition, which increases the gNB complexity and should be deprioritized.

Fig. 1. An ambiguity issues occurs when repetition request or capability report is transmitted by Msg3 PUSCH

Proposal 5: For Option B and C, RAN1 discusses whether/how to solve the ambiguity issue when UEs with and without Msg4 PUCCH repetition capability send Msg3 PUSCH simultaneously.

When multiple PUCCH repetition factors from {1,2,4,8} are configured in SIB, the gNB should further determine and dynamically indicate the specific repetition factor to UE in DCI. Considering the PUCCH repetition factor varies for each Msg4 PUCCH transmission, DCI 1_0 scheduling the Msg4 PDSCH should be used for indication. In addition, for backward compatibility reasons, reusing 1 or 2 bits of the existing field in DCI 1_0, e.g., PRI field, HARQ process number field, MCS field and DAI field, can be further discussed.

Proposal 6: For dynamic indication, reusing 1 or 2 bits of the existing field in DCI 1_0, e.g., PRI field, HARQ process number field, MCS field and DAI field, can be considered.

DMRS bundling for PUSCH

To achieve common understanding on the necessity of enhancement for DMRS bundling in NTN, three RAN4 requirements, including timing error limit/frequency error limit/phase difference limit, are discussed and the following observation was made in the last meeting:

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit

Timing error limit

When the timing error limit of 29 Ts (Table 7.1C.2-1 in TS 38.133) is all used for managing the timing drift in NTN, the longest TA pre-compensation update period can be 13 slots in LEO 1200 with elevation angle 30 deg. However, there are also some other error sources should be taken into account. In our understanding, the timing error limit of 12 Ts (Table 7.12-1 in TS 38.133) in TN is large enough to accommodate the initial timing error. Thus, the remaining part, 29 Ts – 12 Ts = 17 Ts, can be used for managing the timing drift due to satellite movement. In this case, the longest TA pre-compensation update period is reduced to 8 slots in LEO 1200 with elevation angle 30 deg.

Observation 5: When timing error limit of 12 Ts in TN is considered as the initial timing error, the longest TA pre-compensation update period is reduced to 8 slots in LEO 1200 with elevation angle 30 deg.

In R17 DMRS bundling, a series events that violate power consistency and phase continuity are identified, including uplink timing adjustment in response to a TA command, where the time domain window used for DMRS bundling is interrupted as shown in Fig 2.

Fig 2. Uplink TA violates power consistency and phase continuity for DMRS bundling.

In NR-NTN, the UE is required to use the UE self-estimated TA and doppler shift to perform UL time and frequency pre-compensation for the service link, which is transparent for the network. In this case, phase continuity is not maintained anymore, but the network is not aware of the event. Therefore, the UL time/frequency pre-compensation should be defined as a new event which causes power inconsistency and phase discontinuity.

Proposal 7: For DMRS bundling in NTN, the UL time/frequency pre-compensation should be defined as a new event which causes power inconsistency and phase discontinuity.

When applying DMRS bundling in NTN, the existing procedure should be reused to the most extent. To ensure the gNB and UE have common understanding on the timing of TA pre-compensation update, it can be specified that the UE can only perform time/frequency pre-compensation at the boundary of the configured TDW. Besides, UE reporting information is not needed since the gNB can provide an appropriate TDW as in segment transmission in R17 IoT NTN.

Proposal 8: For DMRS bundling in NTN, the UE can only perform time/frequency pre-compensation at the boundary of the configured TDW, and UE reporting information is not needed.

Phase difference limit

In RAN1#112 meeting, companies’ views on using carrier bandwidth or allocated bandwidth to compute the phase difference are divergent. Fig 59.3 in [3] depicts the phase-frequency characteristics with timing drift, where the the line of “phase vs. subcarrier” of different symbols within a slot rotates around the direct current (DC) subcarrier. The characteristic can be naturally extended to multiple slots, so the phase difference due to timing drift should be calculated based on the subcarrier index relative to the directive current (DC) subcarrier.

Fig 3. Phase-frequency characteristics with timing drift and after timing drift compensation in [3].

Therefore, for DMRS bundling in LEO 1200 with elevation angle of 30 deg and SCS = 15kHz, the phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots if the PRB position is not within 6 PRBs from the directive current (DC) subcarrier (Δφ = 2π (rad) × 70.5 (us/s) × 1 (ms) × 180 × 6 (kHz) = 0.0797 (rad) × 6 = 4.57 (deg) × 6). To enable DMRS bundling over multiple slots in NTN, enhancement is necessary to mitigate the phase rotation impact, as shown in the right part in Fig 3. Considering the timing drifts within a satellite beam are close, the RX timing post-compensation at gNB is possible to improve the performance of channel estimation.

Observation 6: For DMRS bundling in LEO 1200 with elevation angle of 30 deg and SCS = 15kHz, the phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots if the PRB position is not within 6 PRBs from the directive current (DC) subcarrier.

Conclusion

In this contribution, we present our view and analysis on PUCCH enhancement for Msg4 HARQ-ACK and necessity of enhancement on PUSCH DMRS bundling. The following observations and proposals are made:

Observation 1: For Msg4 PUCCH repetition, UE indicating capability report regardless of channel quality may cause unnecessary PUCCH resource waste.

Observation 2: In R17 Msg3 PUSCH repetition, the Msg3 PUSCH repetition factor is optionally configured.

Observation 3: In R17 Msg3 PUSCH repetition, if the UE sends repetition request but no repetition factor is configured in SIB, a default set of Msg3 PUSCH repetition factor {1,2,3,4} is applied.

Observation 4: Associating a PRACH resource with the request of Msg3 PUSCH repetition and Msg4 PUCCH repetition simultaneously can mitigate PRACH resource partitioning issue.

Observation 5: When timing error limit of 12 Ts in TN is considered as the initial timing error, the longest TA pre-compensation update period is reduced to 8 slots in LEO 1200 with elevation angle 30 deg.

Observation 6: For DMRS bundling in LEO 1200 with elevation angle of 30 deg and SCS = 15kHz, the phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots if the PRB position is not within 6 PRBs from the directive current (DC) subcarrier.

Proposal 1: The RSRP-Range {0…127} defined in current spec can be reused in Msg4 PUCCH repetition, and the maximum configurable value of the RSRP threshold is 127.

Proposal 2: A new RSRP threshold should be introduced for Msg4 PUCCH repetition.

Proposal 3: For Msg4 PUCCH repetition, the case that the RSRP threshold is configured but the repetition factor is not configured in SIB should be considered.

Proposal 4: When the RSRP threshold and repetition factor are not configured in SIB, the legacy UE behaviour is applied.

Proposal 5: For Option B and C, RAN1 discusses whether/how to solve the ambiguity issue when UEs with and without Msg4 PUCCH repetition capability send Msg3 PUSCH simultaneously.

Proposal 6: For dynamic indication, reusing 1 or 2 bits of the existing field in DCI 1_0, e.g., PRI field, HARQ process number field, MCS field and DAI field, can be considered.

Proposal 7: For DMRS bundling in NTN, the UL time/frequency pre-compensation should be defined as a new event which causes power inconsistency and phase discontinuity.

Proposal 8: For DMRS bundling in NTN, the UE can only perform time/frequency pre-compensation at the boundary of the configured TDW, and UE reporting information is not needed.

Reference

[1]       RAN1#112 meeting chairman notes

[2]       RP-230728, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, RAN#99, March 20-23, 2023

[3]     J. Zhou et al., "Estimation and compensation of timing drift for NR-based NTN system," Advances in Communications Satellite Systems. Proceedings of the 37th International Communications Satellite Systems Conference (ICSSC-2019), Okinawa, Japan, 2019, pp. 1-12, doi: 10.1049/cp.2019.1265.

Appendix

Simulation assumptions

Table 2. PUCCH for Msg4 HARQ-ACK simulation assumption

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 1bit UCI. |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. |
| Number of UE transmit chains | 1 |
| Repetitions | wo repetition. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

Table 3. Msg3 PUSCH simulation assumption

| Parameter | Value |
| --- | --- |
| Number of UE transmit chains | 1 |
| Frequency hopping | wo frequency hopping |
| DMRS configuration | Type I, 2 DMRS symbols, no multiplexing with data. |
| Waveform | DFT-s-OFDM |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| MCS | MCS 0 |