3GPP TSG RAN WG1 #112bis-e		R1-2302566

e-Meeting, April 17th - April 26th, 2023

Source:	OPPO

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Agenda Item:	9.9.3

Document for:	Discussion and Decision

Introduction

In RAN1#112 meeting, the following conclusions and agreements regarding disabling of HARQ feedback for IoT NTN in Rel-18 were made:

Conclusion For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling. Agreement For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process. Agreement For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation - for the first SPS PDSCH after activation, - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise. Conclusion For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration. Agreement For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity - Alternative 1: applies to both semi-statically HARQ enabled and disabled processes - Alternative 2: only applied to semi-statically HARQ disabled processes - Alternative 3: only applied to semi-statically HARQ enabled processes Agreement Confirm the following working assumption with the following update: Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A. Agreement For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI - Option 2: Indication by reusing/reinterpreting existing field in DCI

In this contribution, we further provide our views on introducing disabling of HARQ feedback for IoT NTN.

Discussion

Disabling of HARQ feedback for DL transmission

### Indication of disabling HARQ feedback by DCI

For NB-IoT NTN and eMTC NTN for CE mode B, using DCI to indicate enabling/disabling of HARQ feedback can be considered in the following two cases:

- Case 1: only Option 3 is configured, i.e., DCI indicates enabling/disabling of HARQ feedback for HARQ process not configured with initial feedback state.

- Case 2: both Option 1 and Option 3 are configured, i.e., DCI indicates enabling/disabling of HARQ feedback for HARQ process configured with initial feedback state by RRC bitmap.

Regarding case 1, since HARQ processes are not semi-statically configured with enabled/disabled state, for a scheduled DL transmission, 1-bit indication can be introduced in the corresponding scheduling DCI to indicate enabling/disabling of HARQ feedback state directly if Option 3 is configured. For the case that one DCI scheduling multiple TBs corresponding to multiple HARQ processes, the simplest method is that the indicated enabled/disabled state by the 1-bit indication applies to all the HARQ processes corresponding to the same DCI.

Proposal 1: If only Option 3 is configured, introducing 1-bit indication in the scheduling DCI to indicate enabling/disabling of HARQ feedback for the scheduled DL transmission.

- For one DCI scheduling multiple TBs corresponding to multiple HARQ processes, the 1-bit indication in the DCI should apply to all the HARQ processes scheduled by the same DCI.

Regarding case 2, the first issue is how to interpret this DCI-based overridden mechanism, e.g., DCI signaling is used to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. In our view, if the DCI signaling is used to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration, then the initial configuration of HARQ feedback state by RRC bitmap is useless, and it seems no need to introduce this DCI-based overridden mechanism. For this reason, we think the DCI signaling should be used to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration.

Proposal 2: Support DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration for DCI-based overridden mechanism.

From DCI overhead perspective, when HARQ processes are configured with enabled/disabled state by RRC configuration, 1-bit indication can be introduced in the scheduling DCI to dynamically override the semi-statically configured HARQ process state for case 2. The DCI-based overridden mechanism can be restricted to be applied to only semi-statically HARQ disabled processes or only semi-statically HARQ enabled processes, especially for the case that one DCI scheduling multiple TBs corresponding to multiple HARQ processes. With such kind of restriction, the states of the scheduled multiple HARQ processes by a single DCI would either be the same as indicated by the 1-bit indication or follow the RRC configuration, which would facilitate the HARQ process state selection for scheduled TBs at eNB side.

Proposal 3: If both Option 1 and Option 3 are configured, the 1-bit indication in the scheduling DCI applies only to semi-statically HARQ disabled processes or only to semi-statically HARQ enabled processes for both single-TB and multi-TB scheduling cases.

No matter the HARQ process state is semi-statically configured by RRC signaling or dynamically indicated by DCI, it should be kept the same for initial transmission and retransmission corresponding to a same TB.

Proposal 4: The enabled/disabled state corresponding to a same TB for initial transmission and retransmission should be the same.

### DCI design for indicating enabling/disabling of HARQ feedback

The following two options are discussed for DCI design for indicating enabling/disabling of HARQ feedback.

- Option 1: indication by adding one field in DCI.

- Option 2: indication by reusing/reinterpreting existing field in DCI.

In our view, introducing 1-bit in DCI is enough to indicate/override enabling/disabling of HARQ feedback regardless of the TB number scheduled by the DCI. The overhead impact of the additional bit in DCI is negligible, we prefer to add one field in DCI for simplicity.

Proposal 5: Support adding one field in DCI to indicate enabling/disabling of HARQ feedback.

### HARQ-ACK feedback

Regarding HARQ-ACK feedback for disabled HARQ process in IoT NTN, Rel-17 NR NTN method can be reused as much as possible. In IoT NTN, HARQ-ACK feedback can be configured with non-bundling feedback and bundling feedback. For non-bundling feedback, NACK should be reported for a DL transmission associated with a disabled HARQ process, similar to NR NTN.

For HARQ-ACK bundling feedback for multiple TB scheduling with single DCI, how to perform the bundling operation depends on the disabled HARQ process configuration/indication method. If the disabled HARQ processes are configured by Option 1 or both Option 1 and Option 3, then the HARQ-ACK feedback information corresponding to a TB associated with a disabled HARQ process should not be considered to contribute to the HARQ-ACK bundling result, or ACK should be assumed for a TB associated with a disabled HARQ process for bundling operation. If the disabled HARQ processes are indicated only by Option 3, the indicated enabling/disabling HARQ feedback in the DCI should apply to all the HARQ processes scheduled by the same DCI for simplicity, and then for a DCI scheduling multiple TB for disabled HARQ processes, the corresponding HARQ-ACK transmission should be omitted.

Observation: For multiple TB scheduling with single DCI,

- If the disabled HARQ processes are configured by Option 1 or both Option 1 and Option 3, the HARQ-ACK feedback corresponding to a TB associated with a disabled HARQ process should not be considered to contribute to the HARQ-ACK bundling result.

- If the disabled HARQ processes are indicated only by Option 3, the indicated enabling/disabling HARQ feedback in the DCI should apply to all the HARQ processes, and the corresponding HARQ-ACK transmission should be omitted.

Based on the above discussion, we have the following proposals.

Proposal 6: NACK should be reported for a DL transmission associated with a disabled HARQ process for HARQ-ACK non-bundling feedback.

Proposal 7: For a DCI scheduling multiple TB,

- If the disabled HARQ processes are configured by Option 1 or both Option 1 and Option 3, the HARQ-ACK feedback corresponding to a TB associated with a disabled HARQ process should not be considered to contribute to the HARQ-ACK bundling result.

- If the disabled HARQ processes are indicated only by Option 3, the corresponding HARQ-ACK transmission should be omitted.

Disabling of HARQ feedback for UL transmission

In RAN2#119e meeting, it was agreed that an UL HARQ process can be configured with HARQ mode A or HARQ mode B in IoT NTN, similar to NR NTN:

Agreements: - Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability. - For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN. - From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

HARQ mode A can be considered as the legacy HARQ mode, while HARQ mode B is introduced to resolve the HARQ stalling issue due to long propagation delays, and it is similar to disabled HARQ feedback for a DL HARQ process. In Rel-17 NR NTN, this configuration mainly has impacts on UE’s PDCCH monitoring behavior when UE is configured with DRX cycle. The same UL HARQ mechanism can be reused in Rel-18 IoT NTN. In addition, unlike NR NTN, the gNB may transmit a PDCCH carrying ACK information corresponding to PUSCH transmissions to a UE in an IoT NTN system, e.g., using DCI format 6-0A or DCI format 6-0B in an eMTC system. How to enhance HARQ-ACK feedback for PUSCH transmissions associated with an UL HARQ mode B process should also be discussed.

Proposal 8: Discuss enhancements on PDCCH carrying HARQ-ACK feedback when UE is configured with UL HARQ mode B.

Conclusion

In this contribution, we provide our views on introducing disabling of HARQ feedback for IoT NTN.

The following observation and proposals are made.

Observation: For multiple TB scheduling with single DCI,

- If the disabled HARQ processes are configured by Option 1 or both Option 1 and Option 3, the HARQ-ACK feedback corresponding to a TB associated with a disabled HARQ process should not be considered to contribute to the HARQ-ACK bundling result.

- If the disabled HARQ processes are indicated only by Option 3, the indicated enabling/disabling HARQ feedback in the DCI should apply to all the HARQ processes, and the corresponding HARQ-ACK transmission should be omitted.

The following proposals are made.

Proposal 1: If only Option 3 is configured, introducing 1-bit indication in the scheduling DCI to indicate enabling/disabling of HARQ feedback for the scheduled DL transmission.

- For one DCI scheduling multiple TBs corresponding to multiple HARQ processes, the 1-bit indication in the DCI should apply to all the HARQ processes scheduled by the same DCI.

Proposal 2: Support DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration for DCI-based overridden mechanism.

Proposal 3: If both Option 1 and Option 3 are configured, the 1-bit indication in the scheduling DCI applies only to semi-statically HARQ disabled processes or only to semi-statically HARQ enabled processes for both single-TB and multi-TB scheduling cases.

Proposal 4: The enabled/disabled state corresponding to a same TB for initial transmission and retransmission should be the same.

Proposal 5: Support adding one field in DCI to indicate enabling/disabling of HARQ feedback.

Proposal 6: NACK should be reported for a DL transmission associated with a disabled HARQ process for HARQ-ACK non-bundling feedback.

Proposal 7: For a DCI scheduling multiple TB,

- If the disabled HARQ processes are configured by Option 1 or both Option 1 and Option 3, the HARQ-ACK feedback corresponding to a TB associated with a disabled HARQ process should not be considered to contribute to the HARQ-ACK bundling result.

- If the disabled HARQ processes are indicated only by Option 3, the corresponding HARQ-ACK transmission should be omitted.

Proposal 8: Discuss enhancements on PDCCH carrying HARQ-ACK feedback when UE is configured with UL HARQ mode B.

Reference

[1]     R1-2300268, “Discussion on disabling of HARQ feedback for IoT NTN”, OPPO.

[2]     R1-2301804, “FLS#2 on disabling of HARQ feedback for IoT NTN”, Moderator (Lenovo).