3GPP TSG RAN WG1 #112 bis-e		                                             R1-2302570

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.11.1

Source: 	OPPO

Title:	Evaluation for lower power wake-up signal

Document for:	Discussion & Decision

Introduction

In RAN1#112 and post email discussion, characteristics for target use cases and details of evaluation on low power WUS were discussed, the following agreements were made [1].

Conclusion:

The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group.

Agreement

The following characteristics for target use cases are considered in the study item:

- IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics,

  - FFS: latency

  - primary for small form devices

  - power-sensitive

  - static, nomadic or limited mobility

- Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc.,

  - FFS: latency

  - primary for small form devices,

  - power-sensitive

  - low/medium speed, FFS: high speed

- eMBB cases including e.g., XR/smart glasses, smart phones and etc.,

  - FFS: latency

  - devices form is various and not restricted

  - power-sensitive

  - low/medium speed, FFS: high speed

Note: other use cases/characteristics are not precluded if any.

Agreement

For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows,

- Alt 1: (15000, 400ms)

- Alt 2: ([40000], [800ms])

Company to report which alternative they use for which use cases.

Agreement

For coverage evaluation, the following is used,

| Number of RX chains at the UE’s MR antenna elements for UE | Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS. |
| --- | --- |
| Number of RX chains antenna elements for LP-WUR | 1 Rx Note: agreed in RAN1#110bis |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Reference data rates for MR eMBB | Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Reference PDCCH configuration | SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | Urban: NloS Rural: NloS and LoS |
| Bandwidth | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| Delay spread | Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

Agreement

For link-level simulation of LP-WUS, the following table is used as starting point,

- FFS for other assumptions if any

- Note: The assumptions are not intended to limit the scope of the study or the design.

Table XX. Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK, FSK, OFDM Company to report which option for OOK /FSK /OFDM is used |
| Channel structure | - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence). |
| SCS of OFDM generator for NR signal | 30kHz/15KHz |
| Configuration for LP-WUS signal | For OOK/FSK waveform, # of segments in one OFDM symbol (M) - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report |
| WUS duration | Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS [1%] 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target is determined - Alt 1: FAR target is determined per single WUS attempt/trial, - FFS: Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - Length of reference time duration: [0.32s, 1.28s,10.24s, 20.48s, 2621.44s,10485.76s], other values not precluded - FFS: possible values for reference time durations - Companies to report details of receiver behaviour, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead - FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details. - The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group |
| Code scheme | For data part, code rate {1/2, 1/4, ….} is reported by companies, company to report coding scheme, e.g., manchester code or any other schemes Companies to report, if any, the coding scheme (e.g., manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….) |
| gNB Channel BW | 20MHz, FFS other values |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4,8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| Guard band | E.g., 6 subcarriers, {1, 2, …} RBs on each side of LP-WUS. |
| Filter | X-th Order filterButterworth (e.g. Butterworth, Chebyshev and so on) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y = 4.32MHz (12RB, 30KHz) or ({4,8} RB, 30kHz) Companies to report any other assumptions if needed, cutoff frequency? |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional |
| Sampling Rate | - Companies to report. For evaluation, same sampling rate is assumed for different waveforms/options. |
| ADC bit width | 1-bit, 4-bit, 8-bit, ideal and other options are not precluded |
| Channel Model | TDL-C 300ns FFS: other channels See link coverage assumption table (will copy and paste here) |
| Noise Figure for WUR | Among [9, 12, 15, 18, 21, 24] dB. Company to report - RF Envelop detector: - IF Envelope detector: - BB envelope detector: - FSK: - OFDM: |
| Impairment modelling | - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report |

Working assumption:

- For evaluation of LP-WUR frequency and time errors, the following is used,

| Parameter | Value |
| --- | --- |
| LO XO Oscillator max frequency error [ppm], LO XO Oscillator frequency drift [ppm/s] | option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies |
| RTC max frequency error [ppm] | 20 |

- Company to report how to use the clocks for LR on/off states

  - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state.

  - For Option 3/4,  cannot be used

    - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state,

      - e.g., option 3/4 is not applicable

        - when MR is in ULPS’ultra-deep sleep state’ with [0.015] power units and LR is in off state or,

        - when LR monitoring power less than 1 [TBD] power unit,

    - Note: Assumptions important for achieving performance by option 1/2/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc.

- If MR is in other state than ULPS’ultra-deep sleep state’(e.g., ‘deep sleep’), the clock running for MR can be used for LR.

    - assumptions important for achieving performance by using MR clock for LR should be declared

  - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option

- Company to report the initial/residual frequency error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1:

    - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1+Fr

    - When ΔF frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error

    - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach.

    - Fr is residual frequency error after the previous frequency synchronization.

      - [Fr=0.]

      - FFS: [Fr>0 and explanation of its modeling and how it is obtained at each clock synchronization/calibration]

      - If residual frequency error after the previous synchronization is justified to be zero or marginal compared with Fe should be declared.

    - FFS: Frequency displacement (Fd), before detection of a current sync signal defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous sync signal detection synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm).

    - Model 2: random frequency drifting, FFS details

- Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal,

  - The following are examples for consideration, other approaches are not precluded,

    - Model 1 [R1-2301438] [R1-2301558][R1-1714993]:

    - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (saturated liner region)

    - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = ±0.5 * F’ * T2 ΔT = Fr*T ±0.5 * F’ *T2 (transient region)

    - The transition between transient and saturated linear region (from synchronization or calibration point/time) occurs at time Ts= [Fe /( 0.5F’)] [Ts= (Fe-Fr)/( F’)]

    - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach

    - FFS: Fr ≠ 0, e.g., ΔT = Fr*T ±0.5 * F’ *T2 ,  Ts= (Fe∓Fr)/( 0.5F’)

    - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous sync signal detection synchronization/calibration (Tr); Te= ΔT+ Tr

    - Model 2: random time drifting, FFS details

FFS: Phase noise model

Agreement

- When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

  - raw information bit-size

  - [time/frequency resources (including any guard bands), if applicable]

  - [total energy of LP-WUS across the time/frequency resources]

  - FFS: false alarm probability/rate

  - FFS: misdetection probability/rate

are kept [comparable or fixed]. Study at least

  - impact of timing error

  - impact of frequency error

  - impact of phase noise and I/Q imbalance, if applicable

  - impact of ADC resolution and sampling rate

  - impact of interference

  - impact of delay spread

  - impact of doppler spread

- Companies to report

  - how they modelled SINR

  - time/frequency resources (including any guard bands) for the scheme

  - false alarm probability/rate and misdetection probability/rate

  - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms

- When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

In this contribution, we provide evaluation results on low power WUS.

Power consumption evaluation

Evaluation Assumptions

In this section, some evaluation results on power consumption and latency are provided, we consider the following assumptions and the power models.

Table 1: UE Power Consumption Model for Main Radio and LP-WUR

| Main Radio |  |  |
| --- | --- | --- |
| Power State | Characteristics | Relative Power |
| Ultra-deep sleep | Power state for main radio of UEs with LP-WUS receiver. | 0.015 |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| LP-WUR |  |  |
| Power State | Characteristics | Relative Power |
| LP-WUR ‘off’ | The LP-WUR does not perform monitoring. | 0.001 |
| LP-WUR ‘on’ | The LP-WUR performs monitoring. | 0.01/0.05/0.1/0.5/1/2/4 |

Table 2: UE Power Consumption for Main Radio during the state transition

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Ultra-deep sleep | 15000(1) | 400ms (2) |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| (1) Ramp-down and ramp-up without sync/re-sync transition energy. (2) The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the ramp-up time. Energy consumption for sync/re-sync is separately calculated. * Immediate transition is assumed for power saving study purpose from or to a non-sleep state. |  |  |

Table 3: Simulation assumptions (based TR 38.840)

| Parameter |  | Values |
| --- | --- | --- |
| Frequency |  | 4GHz |
| Subcarrier spacing |  | 15 kHz |
| System bandwidth |  | 100MHz |
| Link Adaptation (MCS select) |  | Off |
| Retransmission |  | Off |
| i-DRX cycle length |  | 1.28(s) |
| Number of POs in Paging Frame |  | 1 |
| Number of SSB before PO / PEI |  | 1, 2, 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) |
| Number of SSBs for sync/re-sync for MR |  | 2/4/6 |
| LP-WUS monitoring |  | Option1: Continuously monitoring Option2: Discontinuously monitoring, with [64] ms as the period for complete an on-and-off cycle, and [64] ms as the active time for monitoring LP-WUS every cycle. And ramp-up time is 10ms. |
| Traffic model | Y | 1.28s |
|  | Per UE paging rate RE | 1% within duration Y |
|  | The mean arrival time P | P = Y/RE = 128s |
|  | The number of UEs in the group | 10 |
|  | Per group paging probability RG | RG = 1 – (1 – RE)N=9.56% |
| LP-WUS duration |  | 1ms |
| LP-WUS monitoring duty cycle |  | 128ms |
| The miss-detection rate (MDR) of LP-WUS |  | 1% |
| The false-alarm rate (FAR) of LP-WUS |  | 0.1% |

Evaluation Results

- Processing Timeline

Rel-17 UE paging reception procedure with PEI is used as baseline, which is shown in Figure 1. The number of SSB bursts processed, PEI-O monitoring duration, PO monitoring duration, and intermediate sleep states are shown for three different SNR cases (low, medium, and high) when PEI is configured.

Figure 1: Rel-17 UE paging reception procedure with PEI (baseline)

Assume UE monitors one paging occasion in a paging cycle for I-DRX cycle length of 1.28s after waking up from deep sleep. The power consumption in a paging cycle for UE operations can be calculated as follows.

Table 4: Power consumption of Rel-17 UE operations in a paging cycle

| UE operations in a paging cycle | Duration (ms) | Relative Power (unit) | Relative Power Consumption (unit*ms) |
| --- | --- | --- | --- |
| Not Paged |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Deep Sleep | 1274 | 1 | 1*1274 + 450 Transition Energy = 1724 |
| Total | 1280 | - | 2324 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 2324 / 1280 = 1.8156 |
| Low SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Light Sleep | 14 | 20 | 20*14 + 100Transition Energy = 380 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (6) SSB Processing | 2 | 100 | 100*2 = 200 |
| (7) Light Sleep | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (8) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (9) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 4976 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 4976 / 1280 = 3.8875 |
| Medium SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Light Sleep | 14 | 20 | 20*14 + 100Transition Energy = 380 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Deep Sleep | 28 | 1 | 1*28 + 450 Transition Energy = 478 |
| (6) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (7) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 4534 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 4534 / 1280 = 3.5422 |
| High SINR case (Paged) |  |  |  |
| (1) SSB Processing | 2 | 100 | 100*2 = 200 |
| (2) PEI-O Monitoring | 4 | 100 | 100*4 = 400 |
| (3) Deep Sleep | 44 | 1 | 1*44 + 450 Transition Energy = 494 |
| (4) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (5) Deep Sleep | 1226 | 1 | 1*1226 + 450 Transition Energy = 1676 |
| Total | 1280 | - | 3970 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 3970 / 1280 = 3.1016 |

Rel-18 UE paging reception procedure with LP-WUR is shown in Figure 2. The MR is in ultra-deep sleep state before the trigger from the LP-WUR. We assume ramp-up time is 400ms, and MR need consider SSB search and synchronization after wake-up. In our assumption, the SSB search time is 20ms, while the time of (re-)synchronization is 120ms (6 SSB bursts) in Low SINR case.

Figure 2: Rel-18 UE paging reception procedure with LP-WUR

Assume UE monitors one paging occasion in a paging cycle for I-DRX cycle length of 1.28s after waking up from ultra-deep sleep. The power consumption in a paging cycle for UE operations can be calculated as follows.

Table 5: Power consumption of Rel-18 UE with LP-WUR operations in a paging cycle

| UE operations in a paging cycle | Duration (ms) | Relative Power (unit) | Relative Power Consumption (unit*ms) |
| --- | --- | --- | --- |
| Not Paged |  |  |  |
| MR Power Consumption: |  |  |  |
| (1) Ultra-deep sleep | 1280 | 0.015 | 1280*0.015 = 19.2 |
| LP-WUR Power Consumption: (continuously monitoring) |  |  |  |
| (1) On state | 1280 | 0.5 | 1280 * 0.5 = 640 |
| Total | - | - | 19.2MR + 640LP-WUR =659.2 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 659.2 / 1280 = 0.515 |
| LP-WUR Power Consumption: (discontinuously monitoring) |  |  |  |
| (1) On state | 64 (Transition time = 10ms) | 0.5 | 64 * 0.5 + 2.505 Transition Energy = 34.505 |
| (2) Off state | 64 | 0.001 | 64 * 0.001 = 0.064 |
| (3) cycle of (1)&(2) | 10 times | 34.505+0.064 = 34.569 | 34.569 * 10 = 345.69 |
| Total | - | - | 19.2MR + 345.69LP-WUR = 364.89 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 364.89 / 1280 = 0.2851 |
| Low SINR case (Paged) |  |  |  |
| (1) SSB Search | 20 | 100 | 100*20 = 2000 |
| (2) Freq. sync | 60 (3 SSB bursts) | 200 + 460 = 660 | 660*3 = 1980 |
| (3) SSB Processing | 2 | 100 | 100*2 = 200 |
| (4) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (5) SSB Processing | 2 | 100 | 100*2 = 200 |
| (6) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (7) SSB Processing | 2 | 100 | 100*2 = 200 |
| (8) Light Sleep | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (9) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (10) Ultra-deep Sleep | 1146 | 0.015 | 0.015*1146 + 15000 Transition Energy = 15017.19 |
| Total (continuously monitoring) | 1280 | - | 21977.19MR + 640LP-WUR = 22617.19 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 22617.19 / 1280 = 17.6697 |
| Total (discontinuously monitoring) | 1280 | - | 21977.19MR + 345.69LP-WUR = 22322.88 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 22322.88 / 1280 = 17.4398 |
| Medium SINR case (Paged) |  |  |  |
| (1) SSB Search | 20 | 100 | 100*20 = 2000 |
| (2) SSB Processing | 2 | 100 | 100*2 = 200 |
| (3) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (6) SSB Processing | 2 | 100 | 100*2 = 200 |
| (7) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (8) SSB Processing | 2 | 100 | 100*2 = 200 |
| (9) Light Sleep | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (10) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (11) Ultra-deep Sleep | 1186 | 0.015 | 0.015*1186 + 15000 Transition Energy = 15017.79 |
| Total (continuously monitoring) | 1280 | - | 20657.79MR + 640LP-WUR = 21297.79 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 21297.79 / 1280 = 16.6389 |
| Total (discontinuously monitoring) | 1280 | - | 20657.79MR + 345.69LP-WUR = 21003.48 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 21003.48 / 1280 = 16.4090 |
| High SINR case (Paged) |  |  |  |
| (1) SSB Search | 20 | 100 | 100*20 = 2000 |
| (2) SSB Processing | 2 | 100 | 100*2 = 200 |
| (3) Light Sleep | 18 | 20 | 20*18 + 100 Transition Energy = 460 |
| (4) SSB Processing | 2 | 100 | 100*2 = 200 |
| (5) Light Sleep | 8 | 20 | 20*8 + 100 Transition Energy = 260 |
| (6) PO Monitoring | 4 | 300 | 300*4 = 1200 |
| (7) Ultra-deep Sleep | 1226 | 0.015 | 0.015*1226 + 15000 Transition Energy = 15018.39 |
| Total (continuously monitoring) | 1280 | - | 19338.39MR + 640LP-WUR = 19978.39 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 19978.39 / 1280 = 15.6082 |
| Total (discontinuously monitoring) | 1280 | - | 19338.39MR + 345.69LP-WUR = 19684.08 |
| Average power consumption = Total Energy / Length of I-DRX cycle |  |  | 19684.08 / 1280 = 15.3782 |

Different SINR levels, e.g. low, medium and high could be assumed to evaluation. UE need different number of SSB bursts to acquire synchronization. LP-WUR can monitor LP-WUS under “continuously monitoring” manner or “discontinuously monitoring” manner.

Table 5: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Paging)

Additional transition energy from ultra-deep sleep is 15000

| DRX settings | SINR | Paging Rate (Per UE) | Scheme | Relative Power for LP-WUR on | Average power | PS gain |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 1% | Rel-17 UE with PEI | - | 1.8384 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 0.2131 | 88.40% |
|  |  |  |  | 0.05 | 0.2531 | 86.22% |
|  |  |  |  | 0.1 | 0.3031 | 83.50% |
|  |  |  |  | 0.5 | 0.7031 | 61.74% |
|  |  |  |  | 1 | 1.2031 | 34.54% |
|  |  |  |  | 2 | 2.2031 | -19.85% |
|  |  |  |  | 4 | 4.2031 | -128.64% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 0.2093 | 88.62% |
|  |  |  |  | 0.05 | 0.2308 | 87.44% |
|  |  |  |  | 0.1 | 0.2578 | 85.98% |
|  |  |  |  | 0.5 | 0.4734 | 74.25% |
|  |  |  |  | 1 | 0.7429 | 59.59% |
|  |  |  |  | 2 | 1.2820 | 30.26% |
|  |  |  |  | 4 | 2.3601 | -28.38% |
|  | Medium |  | Rel-17 UE with PEI | - | 1.8346 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 0.2020 | 88.99% |
|  |  |  |  | 0.05 | 0.2420 | 86.81% |
|  |  |  |  | 0.1 | 0.2920 | 84.08% |
|  |  |  |  | 0.5 | 0.6920 | 62.28% |
|  |  |  |  | 1 | 1.1920 | 35.03% |
|  |  |  |  | 2 | 2.1920 | -19.48% |
|  |  |  |  | 4 | 4.1920 | -128.50% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 0.1979 | 89.21% |
|  |  |  |  | 0.05 | 0.2195 | 88.04% |
|  |  |  |  | 0.1 | 0.2465 | 86.57% |
|  |  |  |  | 0.5 | 0.4621 | 74.81% |
|  |  |  |  | 1 | 0.7316 | 60.12% |
|  |  |  |  | 2 | 1.2707 | 30.74% |
|  |  |  |  | 4 | 2.3488 | -28.03% |
|  | High |  | Rel-17 UE with PEI | - | 1.8298 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 0.1907 | 89.58% |
|  |  |  |  | 0.05 | 0.2307 | 87.39% |
|  |  |  |  | 0.1 | 0.2807 | 84.66% |
|  |  |  |  | 0.5 | 0.6807 | 62.80% |
|  |  |  |  | 1 | 1.1807 | 35.47% |
|  |  |  |  | 2 | 2.1807 | -19.18% |
|  |  |  |  | 4 | 4.1807 | -128.49% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 0.1866 | 89.80% |
|  |  |  |  | 0.05 | 0.2082 | 88.62% |
|  |  |  |  | 0.1 | 0.2351 | 87.15% |
|  |  |  |  | 0.5 | 0.4508 | 75.36% |
|  |  |  |  | 1 | 0.7203 | 60.63% |
|  |  |  |  | 2 | 1.2594 | 31.17% |
|  |  |  |  | 4 | 2.3375 | -27.75% |

Proposal 1: For evaluation of power consumption, FAR target is determined per single WUS attempt/trial.

Observation 1: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5/1, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 34.54%~88.40% power saving gain under Low SINR case, while have 35.03%~88.99% power saving gain under Medium SINR case, and have 35.47%~89.58% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 2: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4, LP-WUR monitor LP-WUS under “continuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 3: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5/1/2, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner can have 30.26%~88.62% power saving gain under Low SINR case, while have 30.74%~89.21% power saving gain under Medium SINR case, and have 31.17%~89.80% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 4: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 4, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Assume the number of UEs in the group is 10, when RE is 1%, the per group paging probability RG is 9.56%. Evaluation results on power saving gain is summarized in Table 6.

Table 6: Power Saving Gain of LP-WUS compared to DRX with PEI (Per UE Group Paging)

Additional transition energy from ultra-deep sleep is 15000

| DRX settings | SINR | Paging Rate (Per Group) | Scheme | Relative Power for LP-WUR on | Average power | PS gain |
| --- | --- | --- | --- | --- | --- | --- |
| I-DRX = 1.28s (107 Cycle) (FAR = 0.1%) | Low | 9.56% | Rel-17 UE with PEI | - | 2.0247 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.7558 | 13.28% |
|  |  |  |  | 0.05 | 1.7958 | 11.30% |
|  |  |  |  | 0.1 | 1.8458 | 8.83% |
|  |  |  |  | 0.5 | 2.2458 | -10.92% |
|  |  |  |  | 1 | 2.7458 | -35.62% |
|  |  |  |  | 2 | 3.7458 | -85.01% |
|  |  |  |  | 4 | 5.7458 | -183.79% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 1.7518 | 13.48% |
|  |  |  |  | 0.05 | 1.7733 | 12.41% |
|  |  |  |  | 0.1 | 1.8003 | 11.08% |
|  |  |  |  | 0.5 | 2.0159 | 0.43% |
|  |  |  |  | 1 | 2.2854 | -12.88% |
|  |  |  |  | 2 | 2.8245 | -39.50% |
|  |  |  |  | 4 | 3.9026 | -92.75% |
|  | Medium |  | Rel-17 UE with PEI | - | 1.9898 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.6518 | 16.99% |
|  |  |  |  | 0.05 | 1.6918 | 14.98% |
|  |  |  |  | 0.1 | 1.7418 | 12.46% |
|  |  |  |  | 0.5 | 2.1418 | -7.64% |
|  |  |  |  | 1 | 2.6418 | -32.77% |
|  |  |  |  | 2 | 3.6418 | -83.02% |
|  |  |  |  | 4 | 5.6418 | -183.53% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 1.6478 | 17.19% |
|  |  |  |  | 0.05 | 1.6693 | 16.11% |
|  |  |  |  | 0.1 | 1.6963 | 14.75% |
|  |  |  |  | 0.5 | 1.9119 | 3.92% |
|  |  |  |  | 1 | 2.1814 | -9.63% |
|  |  |  |  | 2 | 2.7205 | -36.72% |
|  |  |  |  | 4 | 3.7986 | -90.90% |
|  | High |  | Rel-17 UE with PEI | - | 1.9454 | - |
|  |  |  | LP-WUR continuously monitoring | 0.01 | 1.5478 | 20.43% |
|  |  |  |  | 0.05 | 1.5878 | 18.38% |
|  |  |  |  | 0.1 | 1.6378 | 15.81% |
|  |  |  |  | 0.5 | 2.0378 | -4.75% |
|  |  |  |  | 1 | 2.5378 | -30.46% |
|  |  |  |  | 2 | 3.5378 | -81.86% |
|  |  |  |  | 4 | 5.5378 | -184.67% |
|  |  |  | LP-WUR discontinuously monitoring | 0.01 | 1.5438 | 20.64% |
|  |  |  |  | 0.05 | 1.5653 | 19.54% |
|  |  |  |  | 0.1 | 1.5923 | 18.15% |
|  |  |  |  | 0.5 | 1.8079 | 7.07% |
|  |  |  |  | 1 | 2.0774 | -6.79% |
|  |  |  |  | 2 | 2.6165 | -34.50% |
|  |  |  |  | 4 | 3.6946 | -89.92% |

Observation 5: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 8.83%~13.28% power saving gain under Low SINR case, while have 12.46%~16.99% power saving gain under Medium SINR case, and have 15.81%~20.43% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 6: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.5/1/2/4, LP-WUR monitor LP-WUS under “continuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 7: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner can have 0.43%~13.48% power saving gain under Low SINR case, while have 3.92%~17.19% power saving gain under Medium SINR case, and have 7.07%~20.64% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 8: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging,  when relative power for ‘LP-WUR on state’ is 1/2/4, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 2: For I-DRX cycle, the different value of “additional transition energy from ultra-deep sleep” will case different conclusion of whether LP-WUR has power saving gain compared to I-DRX with PEI or not. Prioritize the case ‘LP-WUR on state’ is smaller than 1.

Proposal 3: Both “continuously monitoring” and “discontinuously monitoring” manner should be further studied.

LP-WUS coverage evaluation

Evaluation Assumptions

- Evaluation methodologies

In RAN1#110-bis-e, it was agreed to study coverage of WUS based on methodology and assumptions in R17 CovEnh SI.

Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

Thus, according to the agreement, we need to study the coverage of LP-WUS in terms of MIL margin.

- General Evaluation Assumption

Table 7: General Evaluation Assumption for LP-WUS Coverage Evaluation

| Number of RX chains at the UE’s MR | 2 Rx |
| --- | --- |
| Number of RX chains for LP-WUR | 1 Rx |
| Scenario and frequency | Urban: 2.6GHz (TDD) Rural: 700MHz (FDD) |
| Reference data rates for MR | Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps |
| Reference PDCCH configuration | SCS; 15kHz/30kHz. Aggregation level; 8. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. BLER; 1% BLER,. |
| Pathloss model (select from LoS or NLoS) | NloS |
| Bandwidth | 100MHz for 2.6GHz. 20MHz for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C |
| Delay spread | 300ns |
| UE velocity | 3km/h |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 TXRUs for 700 MHz - 64TxRUs for 2.6 and 4 GHz. |

- Evaluation Assumption

Table 8: Simulation assumptions for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/700MHz |
| Waveform | OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) |
| Channel structure | Payload(32bits)+CRC(8bits) |
| SCS of OFDM generator for NR signal | 15KHz |
| Configuration for LP-WUS signal | For OOK waveform - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) |
| WUS duration | Number of OFDM symbols: 1*40*2 = 80 |
| Code scheme | Manchester code and the code rate (e.g., 1/2) |
| gNB Channel BW | 20MHz |
| LP-WUS BW | - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,24 RBs) for LP-WUS transmission for 15kHz SCS GB is symmetrically placed on each side of LP-WUS |
| Adjacent subcarrier interference | - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB |
| ADC bit width | 1-bit |

Evaluation Results

We provide the coverage comparison between LP-WUS and legacy NR signals. The results are summary as following.

Table 9: Coverage performance for LP-WUS and legacy NR signals

|  | PDCCH AL8 | PUSCH | LP-WUS |
| --- | --- | --- | --- |
| Urban (2.6GHz) | 151.33 dB | 137.38 dB | 137.40 |
| Rural 700MHz | 150.42 dB | 141.67 dB | 139.10 |

Observation 9: The coverage performance of LP-WUS of OOK-1 is worse than PDCCH and could be comparable to PUSCH.

Proposal 4: Channel structure and Waveform should be determined for final comparison.

Proposal 5: The data rate and payload of LP-WUS should be determined for final comparison.

Conclusion

In this contribution, we provide some evaluation results on power consumption and coverage performance. We have following observations and proposals:

Proposal 1: For evaluation of power consumption, FAR target is determined per single WUS attempt/trial.

Observation 1: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5/1, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 34.54%~88.40% power saving gain under Low SINR case, while have 35.03%~88.99% power saving gain under Medium SINR case, and have 35.47%~89.58% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 2: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 2/4, LP-WUR monitor LP-WUS under “continuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 3: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5/1/2, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner can have 30.26%~88.62% power saving gain under Low SINR case, while have 30.74%~89.21% power saving gain under Medium SINR case, and have 31.17%~89.80% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 4: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per UE= 1% and per UE paging, when relative power for ‘LP-WUR on state’ is 4, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 5: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1, LP-WUR monitor LP-WUS under “continuously monitoring” manner can have 8.83%~13.28% power saving gain under Low SINR case, while have 12.46%~16.99% power saving gain under Medium SINR case, and have 15.81%~20.43% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 6: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.5/1/2/4, LP-WUR monitor LP-WUS under “continuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 7: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging, when relative power for ‘LP-WUR on state’ is 0.01/0.05/0.1/0.5, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner can have 0.43%~13.48% power saving gain under Low SINR case, while have 3.92%~17.19% power saving gain under Medium SINR case, and have 7.07%~20.64% power saving gain under High SINR case compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Observation 8: For I-DRX cycle length of 1.28s, with FAR = 0.1%, Paging Rate Per Group= 9.56% and per UE Group paging,  when relative power for ‘LP-WUR on state’ is 1/2/4, LP-WUR monitor LP-WUS under “discontinuously monitoring” manner have not power saving gain compared to I-DRX with PEI (Additional transition energy from ultra-deep sleep is 15000, and  ramp-up time is 400ms).

Proposal 2: For I-DRX cycle, the different value of “additional transition energy from ultra-deep sleep” will case different conclusion of whether LP-WUR has power saving gain compared to I-DRX with PEI or not. Prioritize the case ‘LP-WUR on state’ is smaller than 1.

Proposal 3: Both “continuously monitoring” and “discontinuously monitoring” manner should be further studied.

Observation 9: The coverage performance of LP-WUS of OOK-1 is worse than PDCCH and could be comparable to PUSCH.

Proposal 4: Channel structure and Waveform should be determined for final comparison.

Proposal 5: The data rate and payload of LP-WUS should be determined for final comparison.

Reference

- Draft_Minutes_report_RAN1#112_v040