3GPP TSG RAN WG1 #112bis-e		                                             R1-2302574

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.2

Source: 	OPPO

Title:	The study of power domain enhancements

Document for:	Discussion & Decision

Introduction

In RAN1#112 meeting, the following agreement has been made on power domain enhancements [1]:

Agreement

Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input).

  - FFS: details.

Agreement

If FDSS-SE is supported in Rel-18, RAN1 to further study the following approaches for DMRS, when the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30:

- Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered:

  - A.1: The sequence is a low PAPR Type 1 DMRS sequence.

  - A.2: The sequence is a low PAPR Type 2 DMRS sequence.

FFS: how the sequence is extended.

- Approach B – the DMRS sequence is not extended: A DMRS sequence based on low PAPR type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

Note: if low PAPR type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification

Performance metrics considered for the study are PAPR, CM[, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

Agreement

If FDSS-SE is supported in Rel-18, and RB allocations resulting in DMRS sequence length smaller than 30 before extension of the sequence, if any, are supported, RAN1 to study at least the following approaches:

- Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered:

  - A.1: The sequence is obtained by DFT transformation of an existing DMRS sequence, e.g., low PAPR Type 1 DMRS sequence.

  - A.2: The sequence is a Type 1 or Type 2 DMRS sequence.

FFS: how the sequence is extended.

- Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

Note: if low PAPR type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification

Note: Other sequences are not precluded for Approach A and Approach B.

Performance metrics considered for the study are PAPR, CM [, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

Agreement

Include in the LS to RAN4 for reporting LLS results

Note: The excel file is used to collect the results.

Working Assumption

- The following set of configurations is for companies’ consideration for the comparsion of the performance of DMRS with FDSS-SE.

| No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- |
| #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 8 | 0 [only QPSK] | 6 | 8 | 1 [only QPSK] | ¼ |
| 8 | 6 | 6 | 8 | 8 | ¼ |
| 40 | 2 | 30 | 40 | 3 | ¼ |
| 40 | 6 | 30 | 40 | 8 | ¼ |
| [6 | 3 | 4 | 6 | 5 | 1/3] |
| [36 | 7 | 32 | 36 | 8 | 1/9] |

- FR1 4GHz Urban scenario is prioritized.

- The following filters are for companies’ consideration for the calibration of the performance of DMRS with FDSS-SE

  - 3-tap (0.28 1 0.28)

  - [Truncated RRC (0.5, 0.1667) or 2-tap (1 0.28)]

- Note1: Considered metrics are PAPR/CM, 10% BLER SNR of data for the considered DMRS configuration (for measuring impact of channel estimation accuracy)[, and OBO]

- Note2: companies are encouraged to consider a receiver which at least makes use of the extension for the decoding (e.g., MRC)

- Note3: The values above serve as a common basis, but any other configuration can be studied by companies.

In this contribution, we further discuss the power domain enhancements for NR coverage enhancements.

Power domain enhancements by tone reservation

The principle of Tone Reservation is to reserve a small number of subcarriers at the transmitter as Peak Reduction Tones (PRTs) to reduce the PAPR of the waveform. At the receiver, these Peak Reduction Tones (PRTs) can be easily removed. In the TR-based OFDM scheme, peak reduction tones (PRT) are reserved to generate PAPR reduction signals. These reserved tones do not carry any data information, and they are only used for reducing PAPR. To preserve the EVM, there is no overlap between the PRTs and the subcarriers used for transmitting the data. The locations of PRT are illustrated in Figure 1.

Figure 1: Tone reservation with sideband PRTs

Simulation assumptions

The performance of CM/PAPR reduction as well as LLS performance are evaluated. The simulation assumptions for evaluations are listed in Table-1.

Table 1: Simulation assumptions for Tone reservation w/ spectrum extension

| Scenario | Rural | Urban |  |
| --- | --- | --- | --- |
| Carrier frequency | 700MHz Rural) | 4GHz (Urban) | 28GHz (Urban) |
| Channel | PUSCH, 14 symbols |  |  |
| Channel BW | 20MHz | 100MHz | 100MHz |
| SCS | 15 kHz | 30 kHz | 120 kHz |
| Channel model | TDL-D 30ns | TDL-C 300ns | TDL-A 30ns |
| UE speed | 3km/h | 3km/h | 3km/h |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |
| MCS | 2 for α = 3/8 8 for α = 1/4 |  |  |
| Number of Tx antennas | 1 | 1 | 1 |
| Number of Rx antennas | 2 | 4 | 2 |
| Number of DMRS symbols | 2 | 2 | 2 |
| Number of PUSCH data symbols | 12 | 12 | 12 |
| Number of PRBs (Data) | 8/16 | 8/16 | 8/16 |
| MCS | 0 | 0 | 0 |
| Extension factor (α) | 1/4: PRB before extension is 6, PRB after extension is 8, MCS=8; 3/8: PRB before extension is 10, PRB after extension is 16, MCS=2; |  |  |
| BLER | 10% | 10% | 10% |

Simulation results

The evaluation results of PAPR and CM for Rural scenario are summarized as following:

Table 2. Comparison summary of performance comparison

|  |  |  | PAPR |  | CM |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CDF@99% | gain | CDF@99% | CM reduction |
| Rural | (α) = 1/4 | Baseline | 10.07 | 0.1350 | 5.736 | 0.1930 |
|  |  | TR | 9.935 |  | 5.543 |  |
|  | (α) = 3/8 | Baseline | 10.07 | 0.1520 | 5.769 | 0.9210 |
|  |  | TR | 9.918 |  | 4.848 |  |
| Urban (4GHz) | (α) = 1/4 | Baseline | 10.18 | 0.2420 | 5.737 | 0.2700 |
|  |  | TR | 9.938 |  | 5.467 |  |
|  | (α) = 3/8 | Baseline | 10.08 | 0.2910 | 5.761 | 0.9170 |
|  |  | TR | 9.789 |  | 4.844 |  |
| Urban (28GHz) | (α) = 1/4 | Baseline | 10 | 0.1660 | 5.735 | 0.2200 |
|  |  | TR | 9.834 |  | 5.515 |  |
|  | (α) = 3/8 | Baseline | 10.14 | 0.1730 | 5.77 | 0.9080 |
|  |  | TR | 9.967 |  | 4.862 |  |

Observation 1:  The extension factor of tone reservation will affect the performance of CM/PAPR and BLER. A larger extension factor could have more performance gain.

Observation 2:  ~0.20 dB CM reduction is obtained at CDF@99% when the extension factor α is 1/4, and ~0.90 dB CM reduction is obtained at CDF@99% when the extension factor α is 3/8.

Observation 3:  0.1~0.3 dB PAPR reduction is obtained at CDF@99% when the extension factor is 1/4 and 3/8.

Observation 4: Performance gain of tone reservation technique is not significant.

Proposal 1: Tone reservation can be considered as a candidate MPR/PAR solution to be further studied, including the signal design of PRT.

Enhancements on increasing UE power high limit for CA and DC

The following agreement was reached to realize increasing UE power high limit in RAN1#112 meeting.

Agreement Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input). - FFS: details.

Regarding information exchange needed between the UE and gNB to improve scheduling and network performance when using higher power CA/DC, RAN4 has discussed UL power associated with possible solutions and several proposed schemes [2].

R4-2303560 <Recommended WF> - RAN4 discussion will focus on the following solutions that have been proposed in this meeting: - Power class fallback ΔPPowerClass with aperiodic PHR. - Report power-class fallback ΔPPowerClass in the PHR per serving cell, any power-class change, fallback or return to declared power class, should trigger an aperiodic PHR. This also includes FDD PC2. - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, should also trigger an aperiodic PHR. - For EN-DC report power-class fallback ΔPPowerClass,EN-DC in the multi-entry PHR for the BC. - Power class being used by the UE. Because reporting ΔPPowerClass must be a huge burden for both UE and network. - For single band HPUE operation, PC being used by a UE must be able to be reported per serving cell. - For UL inter band CA HPUE operation, PC being used by a UE must be able to be reported per serving cell per band within a band combination as well as CA PC being used CA for the band combination itself. - The sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE, as well as period of applicability of the ∆PPowerClass report. - Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing. - Enhance the current power headroom reporting framework to enable P-MPR reporting (via MPE field) for FR1 carriers.

RAN4 has discussed several proposed schemes, however, there is no consensus on them yet. In our views, RAN1 can further discuss the signaling impact for the above solutions, while the final decision will be based on both conclusion of RAN4 and RAN1 latter.

Proposal 2: RAN1 further discuss the signaling impact for the proposed solutions on increasing UE power high limit for CA and DC.

Conclusion

In this contribution, we have discussed power domain enhancements for NR coverage enhancements. We have following observation and proposal.

Observation 1:  The extension factor of tone reservation will affect the performance of CM/PAPR and BLER. A larger extension factor could have more performance gain.

Observation 2:  ~0.20 dB CM reduction is obtained at CDF@99% when the extension factor α is 1/4, and ~0.90 dB CM reduction is obtained at CDF@99% when the extension factor α is 3/8.

Observation 3:  0.1~0.3 dB PAPR reduction is obtained at CDF@99% when the extension factor is 1/4 and 3/8.

Observation 4: Performance gain of tone reservation technique is not significant.

Proposal 1: Tone reservation can be considered as a candidate MPR/PAR solution to be further studied, including the signal design of PRT.

Proposal 2: RAN1 further discuss the signaling impact for the proposed solutions on increasing UE power high limit for CA and DC.

Reference

- Draft_Minutes_report_RAN1#112_v040

- R4-2303560, WF on coverage enhancement part 1, Huawei, HiSilicon