3GPP TSG RAN WG1 #112bis		                                           R1-2302575

e-Meeting, April 17th – April 26th, 2023

Item:	9.12.3

Source: 	OPPO

Title:		Considerations on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion & Decision

# Introduction

The discussion on specifying dynamic switching between DFT-S-OFDM and CP-OFDM has progressed in last meeting with:

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In this contribution, we further analyze the schemes for dynamic switching of different waveforms of PUSCH. Our views on mechanism and applicability of the indication are further discussed.

# DCI scheduled PUSCH waveform change

A dedicated bit in DCI is adopted for the dynamic scheduling. Still, there are few details to be finished to support complete functionality of dynamic waveform change. The mechanism for having this bit field and using it should be discussed. How to support other DCI formats and other PUSCHs is not concluded.

## Waveform indication by DCI 0_1&0_2

The configured “transformPrecoder” is now directly disabled or enabled by a new bit field. By receiving that, UE know if it should transmit CP-OFDM or DFT-S-OFDM in scheduled PUSCH resources. Then, the UE’s understanding of the UL DCI size of format 0_1&0_2 will be different.

Only one DCI size is to be assumed in detection. Thus, the DCI field alignment should be done to solve the size ambiguity issues of the cases of enabled/disabled transformPrecoder. Several proposed methods were discussed to address it. In general, one is padding at the end, another is padding per applicable field.

For the first method, the transformPrecoder bit field should be in the beginning of the format. The DCI size will be always be assumed with the maximum size of 2 waveforms. When UE decoding, it should find in the beginning about the wave form and determining the exact location of every DCI field. Some higher priority bits like UL/DL bits have to be in front the transformPrecoder bit field. For the padding bits in the end of DCI format, the exact location of padding should be carefully chosen. There are 2 padding procedure is already in the end of format 0_1&02. Also, the subclause 7.3.1.0 have whole set of DCI size alignments.

For the second method, the per field mapping impact several DCI fields. It would be 4 as in the following based on the current specification.

– Precoding information and number of layers

– Antenna ports

– PTRS-DMRS association

– DMRS sequence initialization

For DMRS sequence initialization, we can simply assume: 0 bit and 1 padding bit if transform precoder is enabled. And then 1 bit if transform precoder is disabled.

For other field, more changes needed as for one case of the antenna ports as example.

Subclause 7.3.1.1.2, TS38.212.

“-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

…”

As example, the Antenna ports field for “dmrs-Type=1, and maxLength=1” and DFT-S-OFDM should be appended with 1 bit, when the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. Then, the field can be always interpreted from MSB.

In general, we see the first method do not have benefit of the DCI size as most of the bits padding to DFT-s-OFDM DCI. The second method can have clean solution specific for few relevant fields.  Thus, the second method can be introduced with less complexity. Still, the second method can apply the new field in front to help parsing the other fields.

Proposal 1: When new 1-bit field for dynamic waveform indication from UL scheduling DCI is configured, bits are padded into the DCI fields depending on waveform to always match CP-OFDM case.

The new 1-bit field is located after UL/DL Identifier.

## DCI formats

It has been agreed that waveform change can be indicated by DCI format 0_1/0_2. The fallback DCI format 0_0 could be also supported. It was claimed that the DCI format 0_0 may be set in default, which did not connect to any wave form. Thus, it is difficult to reinterpret the DCI field for implying different wave forms. However, once the UE get connected to the network, the dynamic indication can be enabled without ambiguity. Thus, we think it is still feasible.

For format 0_0, the advantage is that the compact DCI size is very likely to be used in coverage edge. In that case, higher layer MIMO would not be frequently used. Then, it is more important to allow the format to quickly change the wave form to adapt the channel condition.

Since the format 0_0 is for fallback, the additional bit is not adopted. Instead, we should reuse existing field. We prefer explicit definition. A new column can be added in the default TDRA table of PUSCH to support dynamic wave form switching.

Proposal 2: Indication of dynamic switching of PUSCH wave form is also supported by DCI format 0_0.

New column can be added in the default TDRA table of PUSCH to support dynamic wave form switching.

It was discussed if the other format like DL scheduling can also support dynamic wave form change. As we shown the scenarios earlier. Scheduler decision will be made for each scheduling instant. Thus, best indication of wave form changing should be from the same DCI associated with the PUSCH transmission. For any other DCI, there is no guaranteed of timing triggering wave form switching and the real PUSCH transmission.

Proposal 3: Indication of dynamic switching of PUSCH wave form is not supported by DL scheduling DCI.

## Applicability of waveform change

As we discussed earlier, the UE would change to DFT-S-OFDM when it reached to its power limit. Then, around 3dB lower CM property of DFT-S-OFDM can allow UE to transmit higher power by the PA. Those power change could happen under condition of UE mobility and channel fluctuation. The current Dynamic waveform switching enhancement is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI. This can be understood as regardless of the initial transmission and retransmission.

For the case of DCI format 0_1 or 0_2 in PDCCH scrambled by CS-RNTI with NDI=1, it also indicates waveforms of the associated PUSCH. However, it is unclear for other cases including PDCCH scrambled by CS-RNTI with NDI=0. Since we did not introduce dynamic change for configured grant, the exact behavior should be concluded. Thus, in configured grant, all the PUSCH other than scheduled by PDCCH scrambled by CS-RNTI with NDI=1 should follow the waveform of the earlier transmission.

Proposal 4: In configured grant, the PUSCH transmissions other than scheduled by DCI format 0_1 or 0_2 in PDCCH scrambled by CS-RNTI with NDI=1 should follow the waveform of the earlier transmission.

# Msg3 PUSCH waveform

For the initial access, Msg3 PUSCH waveform is configured by RACH configuration. It cannot be dynamically determined in the RACH procedure in earlier releases.

RACH-ConfigCommon ::=               SEQUENCE {

………

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

………

}

The above configuration is per cell. UEs in a cell may have different channel conditions and geometries. One common configuration cannot well cover all UEs. When a UE is in cell edge, it may ramp up power for multiple times and getting close to its power limit. Then the UE may need to switch to DFT-S-OFDM in transmission of Msg3 PUSCH. Dynamic indication of Msg3 waveform help the cell to adapt for different UEs.

The supporting of that adaption can be implemented by similar ways of the dynamically scheduled PUSCH. One example could be redefining the resource allocation and associated with different waveforms for different entries.

Since it has the application scenario and feasible, a dynamic mechanism to indicate waveform of Msg3 should be supported.

Proposal 5: RAN1 introduces indication of dynamic switching between DFT-S-OFDM and CP-OFDM for Msg3 PUSCH.

# Assistant information for waveform switching

In earlier discussion, assistant information is generally assumed to be useful in the enhancement. Without UE power information, gNB cannot estimate when the UE should switch its waveform. We think the PHR related information is already sufficient for that prediction. With current PHR triggering schemes, it is unclear whether we should introduce further assistant information or reporting mechanism. Further evaluation results should be shown to justify the enhancement by assistant information.

# Conclusion

In this contribution, we discussed the issues how to support the dynamically changed waveforms. We discuss further on details and have our proposals:

Proposal 1: When new 1-bit field for dynamic waveform indication from UL scheduling DCI is configured, bits are padded into the DCI fields depending on waveform to always match CP-OFDM case.

The new 1-bit field is located after UL/DL Identifier.

Proposal 2: Indication of dynamic switching of PUSCH wave form is also supported by DCI format 0_0.

New column can be added in the default TDRA table of PUSCH to support dynamic wave form switching.

Proposal 3: Indication of dynamic switching of PUSCH wave form is not supported by DL scheduling DCI.

Proposal 4: In configured grant, the PUSCH transmissions other than scheduled by DCI format 0_1 or 0_2 in PDCCH scrambled by CS-RNTI with NDI=1 should follow the waveform of the earlier transmission.

Proposal 5: RAN1 introduces indication of dynamic switching between DFT-S-OFDM and CP-OFDM for Msg3 PUSCH.

# Reference

[1] RP-221858, New WI: Further NR coverage enhancements, China Telecom