3GPP TSG RAN WG1 #112bis-e                                                                          R1-2302580

e-Meeting, April 17th –April 26th, 2023

Source:	New H3C

Title:	Discussions on SRS enhancement in Rel-18

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN1 #112 meeting, SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18 in Rel.18 NR MIMO evolution was discussed. Some agreements were made as below [1].

Agreement

For SRS interference randomization, support:

- Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

- Option 1: Reuse the SRS sequence identity nIDSRS.

- Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

- At least s = 2

  - FFS: s = 4, s = 8.

- m = 2,4,8, 10,12,14, and m is a multiple of s.

- Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

- FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

- FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- Option 2: For comb 4, do not support 4 comb offsets.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

- Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

- Option 2: Within a slot, hopping based on only the symbol index l'.

- Option 3: No intra-slot hopping.

- FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number.

- FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

- FFS: At least support reinitialization at the beginning of each radio frame.

Conclusion

No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

- Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

- Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Conclusion

No consensus to support the following for TDD CJT SRS enhancement in Rel-18:

- Further enhancements to frequency hopping

- Sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

- Precoded SRS for DL CSI acquisition

- Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Configuration of v (sequence index within a group) per SRS resource

- Multiplying mask sequence to the legacy SRS sequence

This contribution focuses on further discussion of SRS enhancement for TDD CJT and UL 8Tx operation.

- Discussion

Based on the agreement in last meeting[1], we have below agreement on the time-domain hopping behavior:

Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index l'. - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame.

Compared with Option1 and Option 3, option 2, is already adopted by group/sequence hopping. The benefit of Option 1 isn’t clear and isn’t justifed.

Proposal 1: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', Option 2: Within a slot, hopping based on only the symbol index l'  is supported.

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, we have below agreement.

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Compared with option 2-1, Option 2-2 can keep the phase continuity for SRS transmissions on one port.

Proposal2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols is supported.

- Conclusion

In this contribution, we discussed about SRS enhancement in Rel-18. The proposals are as follows:

Proposal 1: For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', Option 2: Within a slot, hopping based on only the symbol index l'  is supported.

Proposal2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols is supported.

- Reference

- Chairman’s Notes, RAN1#112, final, February 27th – March 03rd, 2023