3GPP TSG RAN WG1 #112bis-e	R1-2302583

e-Meeting, April 17th – April 26th, 2023

Source:	Toyota InfoTechnology Center

Title:	Discussion on SL positioning reference signal

Agenda item:	9.5.1.1

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved [1] (and then revised at RAN#99 in [4]). The relevant WID objective is:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we provide our views as a way forward on the design aspects of the SL positioning reference signal (SL-PRS).

# Discussion

## SL-PRS frequency-domain and time-domain pattern

In previous RAN1 meetings, the follow relevant agreements on SL-PRS sequence design pattern were made:

RAN1 #109-e Agreement With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects: - N>=1 (where N=1 corresponds to full RE mapping pattern) - Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1) - The number of symbols of SL-PRS within a slot - Any relation to the comb-N option - RE offset pattern repetitions within a slot - FFS: Other frequency domain pattern(s) RAN1 #110 Agreement With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning - Note: there could be multiple values for M, N RAN1 #110 Agreement With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS. - Note: this does not preclude comb N=1 - FFS: single symbol SL-PRS, if supported RAN1 #110bis-e Agreement With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics: - With regards to the value N (comb size) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround: - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12} - FFS: the values considered as potential candidate values for M - FFS1: Whether to consider N>12 as a potential candidate value(s) - The symbols of a SL-PRS resource within a slot are consecutive symbols - FFS: consecutive and/or non-consecutive symbols for shared resource pool (if supported) - FFS: RE-Offset sequence within a SL-PRS resource, including whether to have in the end of the SL-PRS pattern a symbol with the same RE-offset as the first symbol, for phase-tracking purpose RAN1 #112 Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs. RAN1 #112 Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. RAN1 #112 Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. RAN1 #112 Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

At RAN1#112 meeting, the issue of In-Band Emission (IBE) interference was mentioned by a number of companies (see [2] and the FL summary [3]).

As a consequence, RAN1#112 agreed on the following with regards to IBE interference:

RAN1 #112 Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs.

3GPP RAN1 #112 agreed to support comb-based multiplexing of SL PRS from different UEs in a slot at least for dedicated resource pools. Figure 1 shows an example of four different SL-PRS resource sets in a slot:

Figure 1. Example of four SL-PRS resource sets in a slot.

One issue here is a near-far problem due to IBE interference among UEs transmitting SL-PRS in the same slot as shown in Figure 2. In Figure 2, Tx UE 1 and Tx UE 2 use adjacent REs for SL-PRS transmissions, where Tx UE 1 is far from Rx UE, while Tx UE 2 is close to Rx UE. In this case, the leakage of Tx UE 2’s signal (i.e. IBE) causes a near-far problem, which interferes with Tx UE 1’s signal at Rx UE and degrades the positioning accuracy of sidelink positioning. We refer the IBE interference within a slot as “short-term” IBE interference.

Figure 2. Example of near-far problem due to in-band emission interference among UEs transmitting SL-PRS in the same slot.

In addition, if multiple Tx UEs transmit SL-PRS in periodic/semi-persistent scheduling (SPS) manner based on comb-based multiplexing of SL-PRS in the same slots, it causes a “(semi-)persistent” near-far problem, in which the near-far problem occurs in consecutive SL-PRS transmissions. This issue occurs when the relative distances among Tx UEs and Rx UE are kept almost the same over consecutive SL-PRS transmissions, and each Tx UE uses the same SL-PRS resource set that are subject to IBE interference every periodic SL-PRS transmission. This would degrade the positioning accuracy for a longer period. We refer this issue as “persistent” IBE interference.

Observation 1: Comb-based multiplexing of SL-PRS from different UEs create a near-far problem due to IBE interference among UEs transmitting SL-PRS in the same slot.

Observation 2: Comb-based multiplexing of SL-PRS from different UEs in SPS or periodic scheduling create persistent IBE interference issues.

Proposal 1: RAN1 to design the comb-based SL-PRS multiplexing solution so that it mitigates the associated short-term IBE interference within the slot.

Proposal 2: RAN1 to design the comb-based SL-PRS multiplexing solution for periodic/semi-persistent SL-PRS transmissions so that it mitigates the associated persistent IBE interference in multiple consecutive SL PRS transmissions.

Regarding the possible comb sizes, one of the previous RAN1#112 agreement states:

RAN1 #112 Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12.

The comb size 1 mitigates the IBE issue because there is only one UE involved. However, the comb size 1 reduces the number of multiplexed UEs.

Observation 3: Although the comb size 1 mitigates the IBE issue as there is only one UE involved, it reduces the number of multiplexed UEs.

One solution to mitigate the short-term IBE interference is that the Tx UE applies a frequency RE separation for the SL-PRS resource sets, as shown in Figure 3. This frequency RE separation can be provided by (pre-)configuration. The more frequency RE separation, the less IBE interference, but the smaller number of multiplexing UEs possible.

Figure 3. Examples of SL-PRS resource sets with different RE separation values between two adjacent active SL-PRS resource sets.

One solution to mitigate the persistent IBE interference would be that the Tx UE changes SL-PRS resource set every SL-PRS transmission in SPS or periodic scheduling, where a frequency RE separation is applied to SL-PRS resource sets.

Collision of SL-PRS resource sets among different Tx UEs can be avoided based on resource reservation and resource (re-)selection with the granularity of the SL-PRS resource set and slot.

Proposal 3: To mitigate the short-term and persistent IBE interference issues, the Tx UE applies a frequency RE separation for the SL-PRS resource sets. This frequency RE separation can be provided by (pre-)configuration.

Proposal 4: To mitigate the persistent IBE interference issue, the Tx UE changes SL-PRS resource set every SL-PRS transmission in SPS or periodic scheduling, where a frequency RE separation is applied to SL-PRS resource sets. Collision of SL-PRS resource sets among different Tx UEs can be avoided based on resource reservation and resource (re-)selection with the granularity of the SL-PRS resource set and slot.

# Conclusions

In this contribution, we have made the following observations and proposals related to the design of SL-PRS for SL positioning:

Observation 1: Comb-based multiplexing of SL-PRS from different UEs create a near-far problem due to IBE interference among UEs transmitting SL-PRS in the same slot.

Observation 2: Comb-based multiplexing of SL-PRS from different UEs in SPS or periodic scheduling create persistent IBE interference issues.

Observation 3: Although the comb size 1 mitigates the IBE issue as there is only one UE involved, it reduces the number of multiplexed UEs.

Proposal 1: RAN1 to design the comb-based SL-PRS multiplexing solution so that it mitigates the associated short-term IBE interference within the slot.

Proposal 2: RAN1 to design the comb-based SL-PRS multiplexing solution for periodic/semi-persistent SL-PRS transmissions so that it mitigates the associated persistent IBE interference in multiple consecutive SL PRS transmissions.

Proposal 3: To mitigate the short-term and persistent IBE interference issues, the Tx UE applies a frequency RE separation for the SL-PRS resource sets. This frequency RE separation can be provided by (pre-)configuration.

Proposal 4: To mitigate the persistent IBE interference issue, the Tx UE changes SL-PRS resource set every SL-PRS transmission in SPS or periodic scheduling, where a frequency RE separation is applied to SL-PRS resource sets. Collision of SL-PRS resource sets among different Tx UEs can be avoided based on resource reservation and resource (re-)selection with the granularity of the SL-PRS resource set and slot.

# References

- RP-223549, Work Item Description on Expanded and Improved NR Positioning, RAN#98.

- RP-2300952, On SL Positioning Reference Signals, RAN1#112, Intel Corporation.

- RP-2302096, FL summary #4 on SL positioning reference signal, RAN1#112, Moderator (Intel Corporation).

- RP-230328, Work Item Description on Expanded and Improved NR Positioning, RAN#99.