3GPP TSG RAN WG1 #112bis-e		R1-2302585

e-Meeting, April 17th – April 26th, 2023

Agenda Item:     9.1.1.1

Source:	Spreadtrum Communications

Title:	Unified TCI framework extension for multi-TRP

Document for:	Discussion and decision

Introduction

In RAN #94 meeting，the following objectives on unified TCI framework for multi-TRP have been agreed [1].

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Further, during the RAN1#109-e~RAN1#112 meetings, some agreements were achieved [2]-[6]. In this contribution, we provide some discussion on the unified TCI framework extension for indication of multiple DL/UL TCI states and power control for UL MTRP transmission.

Discussion

TCI state update and activation

According to the offline pre-discussion for RAN1#112b, the following proposal was listed:

Proposal 2.1: On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, one or two joint TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, one or two DL TCI states and/or one or two UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - The indicated joint/DL/UL TCI state(s) is updated according to the joint/DL/UL TCI state(s) mapped to a TCI codepoint received by the UE - FFS: If the UE receives a TCI codepoint mapped with one single joint TCI state in a serving cell configured with joint DL/UL TCI mode, or receives a TCI codepoint mapped with one single DL and/or UL TCI state in a serving cell configured with separate DL/UL TCI mode, whether the UE shall switch to single TRP operation in the serving cell? - FFS: Whether TCI state activation command (MAC-CE) should indicate the followings? - For a serving cell configured with joint DL/UL TCI mode, each activated joint TCI state in TCI state activation command corresponds to the 1st joint TCI state or 2nd joint TCI state. - For a serving cell configured with separate DL/UL TCI mode, each activated DL/UL TCI state in TCI state activation command corresponds the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state. - Note: Detail on how to indicate above is up to RAN2 design

For the 1st FFS, the remaining question is that if the UE receives a TCI codepoint mapped with one single TCI state, whether the UE shall switch to single TRP operation in the serving cell. Since unified TCI for MTRP can provide multiple TCI states for multiple TRP transmissions via a single beam indication instance, even including both uplink and downlink channels and some reference signals, it is too restricted to determine whether to perform MTRP-to-STRP switch only by the number of indicated TCI states. It is unreasonable to switch the MTRP to STRP scheme simultaneously for all channels, which will loss the flexibility of transmission. In other word, the scheme of switching between STRP and MTRP should be discussed separately for each channel. In addition, in Rel-17, The SRS resource set indicator has been introduced to indicate the STRP/MTRP switch for PUSCH, and a new DCI field was introduced in RAN1#111 to indicate how many or which indicated TCI state(s) is(are) applied for PDSCH reception, which could be regarded as a dynamic STRP/MTRP switching indicator.

Based on the discussion above, we tend to support that the TCI field is only used for TCI state update as in Rel-17. For example, if currently applied TCI states are {2 pairs of DL and UL TCI states} while {1 UL TCI state for TRP1+1 DL TCI state for TRP2} is newly indicated, the UE should apply the corresponding UL TCI state for TRP1 and DL TCI state for TRP2, and keep the others TCI states (i.e., UL TCI state for TRP2 and DL TCI state for TRP1) as the same.

Proposal 1: For s-DCI based MTRP, if the UE receives a TCI codepoint mapped with one single joint TCI state in a serving cell configured with joint DL/UL TCI mode, or receives a TCI codepoint mapped with one single DL and/or UL TCI state in a serving cell configured with separate DL/UL TCI mode, the UE should NOT switch to single TRP operation in the serving cell.

For the 2nd FFS, the design principle of TCI state activation MAC CE is discussed. During the previous meetings, it was agreed that the existing TCI field in DCI format 1_1/1_2 is utilized to indicate the TCI state(s) for one of or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list. Thus, it is straightforward to activate one or two joint (UL/DL) TCI state(s) simultaneously for each serving cell in an activation command (MAC CE). In this way, up to 2 DL TCI states and 2 UL TCI states can be mapped to a TCI codepoint for separate TCI activation, up to 2 joint TCI states can be mapped to a TCI codepoint for joint TCI activation.

Proposal 2: For a serving cell configured with joint (or separate) DL/UL TCI mode, each activated joint (or DL/UL) TCI state in TCI state activation command corresponds to the 1st joint TCI state and/or 2nd joint TCI state (or corresponds the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state).

Mapping between indicated TCI states and channels/signals

PDSCH

For m-DCI based PDSCH MTRP scheme, it has been agreed that the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value.

For s-DCI based PDSCH reception, the following agreements were achieved in RAN1#112:

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

In terms of detailed design of the new indicator field, 2 bits-length field was introduced, e.g., “00”, “01”, “10” stand for the UE shall apply the 1st, 2nd, or both indicated TCI states, respectively, if the UE does not support two default beams. Besides, in the 1st FFS, whether or how to use “11” codepoints to indicate the association between indicated TCI state(s) and PDSCH reception in different transmission scheme should be considered. We think for PDSCH TDM/SDM/FDM the indicated TCI states can be applied with {1st, 2nd} and {2nd ,1st} order, so the codepoint “10” and “11” could be used for mapping order swapping.

Proposal 3: The codepoint “10” and “11” could be used for mapping order swapping for PDSCH SDM/TDM/FDM scheme.

If the new DCI field is not present, some default rules should be introduced to determine the association between indicated joint/DL TCI state(s) and PDSCH reception as mentioned in FFS. A simple way could be considered that the UE always applies the first indicated joint/DL TCI state for PDSCH reception.

Proposal 4: If the new DCI field is not present, the UE should always apply the first indicated joint/DL TCI state for PDSCH reception.

When PDSCH is scheduled/activated by DCI format 1_0/1_1/1_2, and the TCI field does not exist, a fixed rule should be introduced to determine the TCI state(s). If UE is configured with both sfnSchemePdcch and sfnSchemePdsch, the indicated joint/DL TCI state(s) for PDSCH reception should refer to the TCI state(s) that is (are) applied to PDCCH. Otherwise, if sfnSchemePdsch is not configured, the UE could assume that the TCI state for the PDSCH is identical to the first TCI state which is applied for the CORESET used for the PDCCH transmission which is consistent with the current specification.

Proposal 5: When the TCI field is not present, the indicated joint/DL TCI state(s) to PDSCH reception can be determined as:

- If UE is configured with both sfnSchemePdcch and sfnSchemePdsch, the TCI state(s) applied to PDSCH is identical to the TCI state(s) whichever is applied for the PDCCH.

- Otherwise, if sfnSchemePdsch is not configured, the TCI state for the PDSCH is identical to the first TCI state which is applied for the PDCCH transmission.

Besides, the previous discussion all based on the premise that the the offset between the reception of the DCI  and the corresponding PDSCH reception is equal or greater than a threshold, the case that scheduling offset is less than a threshold also needs to be considered as well. In order to reduce the specification impact, the legacy behaviour should be reused as much as possible, which can be summarized as below:

Proposal 6: For the case that the scheduling offset is less than a threshold, if all of the indicated TCI state(s) is (are) associated with the PCI of the serving cell, the indicated TCI state(s) is(are) applied to PDSCH reception.

Proposal 7: For the case that the scheduling offset is less than a threshold, if the one of indicated TCI states is associated with a PCI different from the serving cell, the TCI state for PDSCH reception follows the TCI state that applied to the CORESET with lowest ID in the latest slot.

PUCCH

For s-DCI based PUCCH transmission, it has been agreed that the mapping between indicated TCI state(s) and PUCCH resource/group is informed via RRC configuration

Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design

For m-DCI based MTRP, the association between indicated TCI states and PUCCH transmission also needs to be determined. According to the discussion in RAN1#112, four options are listed:

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

Since there are multiple types of PUCCH resource, a straightforward and simple solution is expected to indicate the mapping between the indicated TCI state and PUCCH transmission, if the mapping rule for different types of PUCCH are different, the discussion will become complicated. In fact, Opt1 or Opt2 can be applied to all the cases including the PUCCH carrying HARQ-ACK/BFR that are mentioned in Opt3 and Opt4. Besides, the Rel-18 BFR has not been discussed in detail, opt 4 should not be agreed in advance. Thus, Opt1 or Opt 2 should be considered as a unified solution. In essence, option 1 and option 2 are the same solution, but considering the agreement for S-DCI based PUCCH transmission, option 2 is preferred to achieve a unified solution for s-DCI based and m-DCI based PUCCH transmission.

Proposal 8: for M-DCI based PUCCH transmission, Opt 2 is preferred.

PUSCH

Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

For s-DCI based transmission, the “SRS resource set indicator field” is introduced in format 0_1/0_2 to indicate how to apply the indicated TCI state(s) to the PUSCH transmission. According to the agreement, the codepoints “00, 01” or “10, 11” denote one of or both of indicated TCI states is (are) applied to the PUSCH transmission, respectively. In last meeting, an FFS was left that is whether and how to use “10” and “11” for SDM and SFN PUSCH scheme. In terms of SDM and SFN based PUSCH transmission, one or two TCI state(s) is (are) applied. The PUSCH antenna port(s) corresponding to the SRS port(s) is associated with the indicated SRS resource in the first/second SRS resource set. The codepoint “10” could indicate the association between indicated TCI states and SRS resource sets in different order and the codepoint “11” could be reserved.

Proposal 9: For SDM and SFN based PUSCH scheme, the codepoint “10” denotes the first indicated TCI state is associated with the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the first SRS resource set, the second TCI state is associated with the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the second SRS resource set, and codepoint “11” could be reserved.

Besides, for the 2nd FFS in agreement, we think the beam misalignment between the indicated TCI state(s) for PUSCH transmission and the spatial filter(s) for SRS should be avoided by gNB implement.

Proposal 10: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission should be avoided by gNB implement.

For PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG), the field “SRS resource set indicator” does not exist, then, an indicator could be introduced in SRS resource set to indicate the association between indicated TCI state(s) and PUSCH transmission.

Proposal 11: For s-DCI based PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG), an indicator could be introduced in each SRS resource set to indicate which joint/UL TCI state(s) is (are) applied to PUSCH transmission.

For M-DCI based PUSCH transmission, it has been agreed that the UE shall apply the indicated joint/UL TCI state specific to a CORESETPoolIndex value to PUSCH scheduled/activated by PDCCH on a CORESET that is associated with the same CORESETPoolIndex value.

Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

In Rel-17, for m-DCI based MTRP PUSCH, one or two SRS set(s) can be considered for transmission, however, in Rel-18 unified TCI framework, to achieve a unified solution and facilitate the TRP-specific power control and refer to the existing agreement in AI 9.1.4.1, we suggest to only support the configuration of two SRS resource sets for MTRP transmission.

Proposal 12: for m-DCI based MTRP transmission in Rel-18 unified framework, only support the configuration of two SRS resource sets.

Agreement Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly. - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB - FFS: the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH Agreement - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

For type 1 CG-PUSCH, the PUSCH transmission is configured by RRC. In order to be consistent with the existing agreement in AI 9.1.4.1, in this case, if two SRS resource sets are configured, an indicator SRS_resource_set_index could be introduced in ConfiguredGrantConfig to indicate the association between the configured SRS resource set and CORESETPoolIndex. Then, the UE shall applied the indicated TCI state to the SRS resource set that associated with the same CORESETPoolIndex value for type 1 CG PUSCH transmission.

Proposal 13: For type 1 CG-PUSCH transmission, an indicator could be introduced in ConfiguredGrantConfig to inform which TCI states is applied to PUSCH transmission.

CSI-RS

With regards to the CSI-RS sharing the indicated TCI state, the mapping between the signals and indicated TCI state(s) should be determined.

For s-DCI based transmission, the following was proposed in offline pre-discussion.

Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM, down-select one from the following alternatives: - Alt1: An RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set - Alt2: For aperiodic CSI-RS for enhanced group-based beam reporting or NCJT CSI measurement, the indicated joint/DL TCI state applied to the aperiodic CSI-RS is determined based on a fixed rule. Otherwise, an RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set.

Both in Alt1 and Alt2, an RRC configuration is introduced for each CSI-RS resource set to inform which indicated TCI state is applied for. The difference between the two alternatives is that the CSI-RS for enhanced group-based beam reporting and NCJT CSI measurement are discussed separately in Alt2, additional new fixed rule should be introduced. We think Alt1 is a general and simple way to indicate the mapping between the indicated TCI state and CSI-RS resource set.

As for the special cases mentioned above, the absence of QCL-info is not expected, which brings little specification impact. If Alt1 is supported, what is the detailed design of the fixed rule? Refer to the TS 38.214:

For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCIState's for the aperiodic CSI-RS resources associated with the CSI triggering state.

Whether or not it means all of the information contained in QCL-info (i.e., a list of TCI states for the AP CSI-RS resources associated with the CSI triggering state) should be determined by default, if yes, it will bring lots of workload.

Proposal 14: For S-DCI based MTRP, an RRC configuration is introduced for each AP CSI-RS resource set to inform which indicated TCI state is applied for.

Proposal 15: For some special cases (e.g., aperiodic CSI-RS for NCJT CSI measurement or enhanced group based reporting), the absence of QCL-info is not expected.

For m-DCI based transmission, if AP CSI-RS sharing the indicated TCI state that is triggered by PDCCH, the indicated TCI state applied to the AP CSI-RS follows the TCI state of the CORESET carrying the triggering DCI.

Proposal 16: For m-DCI based transmission, the indicated TCI state applied to the AP CSI-RS follows the TCI state of the CORESET carrying the triggering DCI.

UL PC for UL MTRP

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

Based on the agreement in RAN1#112 meeting, the group arrived at the conclusion: for PUSCH/PUCCH/SRS, the corresponding UL power control parameter setting {p0, alpha, closed loop index} and a PL-RS can be determined based on the indicated joint/UL TCI state(s) which is (are) applied to the PUSCH/PUCCH/SRS transmission. However, if the power control parameter settings are not configured in the indicated TCI states, two default PC parameter settings can be configured in BWP-UplinkDedicated, where each default PC parameter setting is used for downlink path-loss estimation for a TRP.

Proposal 17: If the power control parameters are not configured with indicated TCI state(s), two default power control parameter sets can be configured in BWP-UplinkDedicated.

STxMP

Power control

For Rel-17 unified TCI framework, UL power control parameter setting (including PLRS, P0, alpha, closed loop index) can be configured in the indicated TCI state. In Rel-18 STxMP, the Rel-17 power control mechanism can be reuse for panel-specific power control. When multiple unified TCI states are indicated for multi-TRP respectively, the power control parameters corresponding to each SRS resource set can be determined by the indicated TCI state corresponding to the same TRP as the SRS resource set. Besides, closed-loop power control in Rel-17 can be reused for STxMP, e.g. two TPC fields for Rel-17 UL mTRP.

Proposal 18: For power control of STxMP, support panel-specific power control mechanism including closed-loop power control and open-loop power control determined by indicated unified TCI states.

In Rel-15/Rel-16/Rel-17, the transmission power is calculated according to the same power control formulations in clause 7 of TS 38.213, which would not exceed the maximum allowed transmission power. However, as shown in Figure 1, when STxMP PUSCHs from different UE panels are transmitted, the transmission power of each PUSCH can be achieved independently as the legacy scheme. However, there is possibility that the sum power of the two PUSCHs may exceed the maximum allowed transmission power for one UE. Thus, enhancement on power control for STxMP PUSCH should be considered.

Figure 1. One illustration of two PUSCHs for different TRPs

Proposal 19: For STxMP scheme, when the total transmission power of PUSCHs from different UE panels exceeds the maximum allowed transmission power per UE, power control enhancement should be considered.

Beam reporting and indication

To discuss about the beam reporting and beam indication for STxMP, the main issue is whether the enhancement of beam management for STxMP transmission should be supported.

For simultaneous transmission with multi-panel, the key features are listed as follow:

- Two UL beams which can be transmitted simultaneously should be reported to gNB

- Two UL/joint TCI states need to be indicated by gNB

In Rel-17, the UE capability value was introduced to support UL panel activation and selection, the UE can report one index of UE capability value for each reported CRI/SSBRI in one beam reporting. In the procedure of beam management, the gNB will transmit reference signals to UE. If the higher layer parameter groupBasedReporting is set to ‘disabled’, the UE should report nrofReportedRS different CRIs or SSBRIs, which could be received and transmitted simultaneously by the UE. For beam indication, if STxMP is configured, the gNB will indicate two joint TCI states. In this case, although gNB has no awareness of association with UL beams and panels, UE can associate the UL beams and panels by implement in the measurement stage. While two joint TCI states are indicated by gNB, UE has the capability to apply the indicated TCI states by different panels for UL STxMP transmission. In summary, if Rel-18 STxMP transmission is enabled, the beam reporting and indication scheme in Rel-17 can be reused as much as possible.

Proposal 20: For STxMP transmission, the mechanism of beam reporting in Rel-17 can be reused as much as possible.

PHR report

PHR report can be regarded as two parts: PHR triggering and PHR reporting. In terms of PHR triggering in Rel-17, if the phr-ProhibitTimer expires and (or) the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell, the PHR is triggered. The joint trigger condition mentioned above is corresponded to Rel-17 MTRP PUSCH repetition. However, in Rel-18 STxMP transmission, two panels will transmit in a serving cell simultaneously, corresponding to different TRPs. For s-DCI based STxMP, the joint triggering condition in Rel-17 could be reused to reduce the impact of specification. For m-DCI based STxMP, the triggering condition in Rel-17 could be reused as well, alternatively, since the power reduction should be performed on two panels separately, it is also reasonable to consider a separate triggering condition to achieve efficient PHR reporting.

Proposal 21: For s-DCI based transmission, the joint triggering condition in Rel-17 could be reused, in addition, for m-DCI based STxMP transmission, a separate triggering condition could be considered as well.

When the triggering condition is met, two PH values are calculated and reported. Whether the PH value is actual or virtual depends on the transmission scheme, i.e. STRP transmission or STxMP transmission. In terms of PHR report determination, the PHR reporting is determined as follows in Rel-17.

When PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, second PHR value is determined as: - If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value, is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual. - If there are multiple repetitions associated with the other TRP in slot n, the earliest one in slot n is selected. - If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), a second PHR value is reported as virtual PHR. - If the first PHR value is virtual, a second PHR value is reported as virtual PHR.

In STxMP case, for s-DCI based and m-DCI based transmission, the design of PHR reporting in Rel-17 could be reused to STxMP as much as possible. For m-DCI based STxMP, it is reasonable to consider the separate PHR reporting per panel, especially for non-ideal backhaul case, where PHR reporting for each panel can be triggered, calculated and transmitted respectively.

Proposal 22:

- For s-DCI based STxMP transmission, the design of PHR reporting in Rel-17 could be reused to STxMP as much as possible.

- For m-DCI based STxMP, support separate PHR reporting.

BFR Aspect

The BFR procedure has been introduced since in Rel-15 to recover the communication link between gNB and UE. In Rel-15, the UE can be provided a configuration for PRACH transmission to carry BFRQ information for the PCell. Then, in order to support MTRP transmission, the MTRP based BFR is introduced in Rel-17. As for Rel-18, in MTRP transmission, the TRP-specific BFR has been discussed in RAN 1#110b-e.Based on all of the above, how to perform the TRP-specific BFR and further reduce the cost of signaling is an issue worth discussing. For unified TCI framework, when CC list is configured, a common TCI state ID can be used to provide common QCL information and/or common UL TX spatial filter across the configured CCs, which can be used to reduce signaling overhead for TRP-specific BFR. For example, if CC list is configured, BFD RS set can be configured on the reference CC and the beam failure declaration is applied to all CCs in the CC list. While the new beam in BFR MAC CE can be used for the common TCI state update for the CC list.

Proposal 23: Consider the overhead reduction of TRP-specific BFR procedure in CA case.

Conclusion

In this contribution, extension of unified TCI framework for MTRP operation was discussed. Based on the discussion in the previous sections, we made the following proposals:

Proposal 1: For s-DCI based MTRP, if the UE receives a TCI codepoint mapped with one single joint TCI state in a serving cell configured with joint DL/UL TCI mode, or receives a TCI codepoint mapped with one single DL and/or UL TCI state in a serving cell configured with separate DL/UL TCI mode, the UE should NOT switch to single TRP operation in the serving cell.

Proposal 2: For a serving cell configured with joint (or separate) DL/UL TCI mode, each activated joint (or DL/UL) TCI state in TCI state activation command corresponds to the 1st joint TCI state and/or 2nd joint TCI state (or corresponds the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state).

Proposal 3: The codepoint “10” and “11” could be used for mapping order swapping for PDSCH SDM/TDM/FDM scheme.

Proposal 4: If the new DCI field is not present, the UE should always apply the first indicated joint/DL TCI state for PDSCH reception.

Proposal 5: When the TCI field is not present, the indicated joint/DL TCI state(s) to PDSCH reception can be determined as:

- If UE is configured with both sfnSchemePdcch and sfnSchemePdsch, the TCI state(s) applied to PDSCH is identical to the TCI state(s) whichever is applied for the PDCCH.

- Otherwise, if sfnSchemePdsch is not configured, the TCI state for the PDSCH is identical to the first TCI state which is applied for the PDCCH transmission.

Proposal 6: For the case that the scheduling offset is less than a threshold, if all of the indicated TCI state(s) is (are) associated with the PCI of the serving cell, the indicated TCI state(s) is(are) applied to PDSCH reception.

Proposal 7: For the case that the scheduling offset is less than a threshold, if the one of indicated TCI states is associated with a PCI different from the serving cell, the TCI state for PDSCH reception follows the TCI state that applied to the CORESET with lowest ID in the latest slot.

Proposal 8: for M-DCI based PUCCH transmission, Opt 2 is preferred.

Proposal 9: For SDM and SFN based PUSCH scheme, the codepoint “10” denotes the first indicated TCI state is associated with the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the first SRS resource set, the second TCI state is associated with the PUSCH antenna port(s) corresponding to the SRS port(s) of the indicated SRS resource in the second SRS resource set, and codepoint “11” could be reserved.

Proposal 10: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission should be avoided by gNB implement.

Proposal 11: For s-DCI based PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG), an indicator could be introduced in each SRS resource set to indicate which joint/UL TCI state(s) is (are) applied to PUSCH transmission.

Proposal 12: for m-DCI based MTRP transmission in Rel-18 unified framework, only support the configuration of two SRS resource sets.

Proposal 13: For type 1 CG-PUSCH transmission, an indicator could be introduced in ConfiguredGrantConfig to inform which TCI states is applied to PUSCH transmission.

Proposal 14: For S-DCI based MTRP, prefer Alt 1, an RRC configuration is introduced for each AP CSI-RS resource set to inform which indicated TCI state is applied for.

Proposal 15: For some special cases (e.g., aperiodic CSI-RS for NCJT CSI measurement or enhanced group based reporting), the absence of QCL-info is not expected.

Proposal 16: For m-DCI based transmission, the indicated TCI state applied to the AP CSI-RS follows the TCI state of the CORESET carrying the triggering DCI.

Proposal 17: If the power control parameters are not configured with indicated TCI state(s), two default power control parameter sets can be configured in BWP-UplinkDedicated.

Proposal 18: For power control of STxMP, support panel-specific power control mechanism including closed-loop power control and open-loop power control determined by indicated unified TCI states.

Proposal 19: For STxMP scheme, when the total transmission power of PUSCHs from different UE panels exceeds the maximum allowed transmission power per UE, power control enhancement should be considered.

Proposal 20: For STxMP transmission, the mechanism of beam reporting in Rel-17 can be reused as much as possible.

Proposal 21: For s-DCI based transmission, the joint triggering condition in Rel-17 could be reused, in addition, for m-DCI based STxMP transmission, a separate triggering condition could be considered as well.

Proposal 22:

- For s-DCI based STxMP transmission, the design of PHR reporting in Rel-17 could be reused to STxMP as much as possible.

- For m-DCI based STxMP, support separate PHR reporting.

Proposal 23: Consider the overhead reduction of TRP-specific BFR procedure in CA case.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)

- 3GPP RAN1, RAN1#109-e, Chairman Notes.

- 3GPP RAN1, RAN1#110, Chairman Notes.

- 3GPP RAN1, RAN1#110b-e, Chairman Notes.

- 3GPP RAN1, RAN1#111, Chairman Notes.

- 3GPP RAN1, RAN1#112, Chairman Notes.