3GPP TSG RAN WG1 #112bis-e		  R1-2302588

e-Meeting, April 17th – April 26th, 2023

Agenda Item:     9.1.3.1

Source:	Spreadtrum Communications

Title:	Discussion on increased number of orthogonal DMRS ports

Document for:	Discussion and decision

Introduction

In RAN1#112 meeting, RAN1 had an extensive discussion on increasing the maximum number of DMRS ports for PDSCH/PUSCH larger than Rel-15 for CP-OFDM. During the meeting, some important agreements have been achieved [1].

In this contribution, we provide our view on the detailed design issues of Rel-18 DMRS ports.

Discussion

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, the following working assumption was achieved as below,

Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11.

For two codewords, several companies proposed different port combinations during the last meeting. We think only one port combination for each layer should be defined. So it seems not possible to consider compromise or merge from multiple solutions. Since majority companies support the working assumption, we suggest to confirm it in this meeting.

Proposal 1: Confirm the working assumption of DMRS port combinations for 2 CWs in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH.

MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports by different CDM groups has been supported with no scheduling restriction. For MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group, we think it is beneficial which can achieve additional scheduling flexibility.

On the other hand, MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be performed by gNB implementation. For example, if a UE is scheduled with Rel.15 DMRS using FD-OCC [+1 +1], another UE can be co-scheduled with Rel.18 DMRS using FD-OCC [+1 -1 +1 -1]. The UE using Rel.15 DMRS can assume that the co-scheduled UE is also using Rel.15 DMRS. At the same time, the UE using Rel.18 DMRS can assume that the co-scheduled UE is also using Rel.18 DMRS.

Observation 1: MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be achieved by scheduling restriction at gNB side and does not require additional enhancement for legacy UEs and Rel.18 UEs.

Proposal 2: For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, it can be supported by gNB implementation without spec impact.

For enhancement of DMRS port allocation tables for PUSCH with rank = 5-8, there’s a working assumption to adopt separate DMRS ports tables.

Working assumption - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

We support to confirm the working assumption. In our views, separate DMRS ports tables enables unified design for rank = 1-4 and rank = 5-8. That is, the rank is indicated jointly with TPMI as legacy uplink.

Proposal 3: Confirm the working assumption of separate DMRS ports tables for PUSCH with rank = 5-8.

Regarding the maximum number of PTRS ports for up to 8 PUSCH layers, RAN1 has discussed during several meetings without consensus. In our views, 4 non-coherent antennas can be configured with 2 PTRS ports since Rel.15, we think 4 non-coherent antenna groups (each group contains 2 antennas) should also be able to share 2 PTRS ports. Another concern on supporting 4 PTRS ports is that the PTRS overhead will be doubled. Therefore, we support up to 2 PTRS port.

Proposal 4: For 8Tx PUSCH, support up to 2 ports PTRS for CP-OFDM.

Another issue is on the PTRS power boosting. In Rel.15, the factor related to PUSCH to PT-RS power ratio per layer per RE () has been defined in Table 6.2.3.1-3 of TS 38.214 [2]. According to the table, different values can be selected based on the higher layer parameter ptrs-Power, the number of PUSCH layers and the transmission scheme including coherent type for codebook based transmission.

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. All cases; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial coherent; Non-coherent and non-codebook based. 00; 0; 3; 3Qp-3; 4.77; 3Qp-3; 6; 3Qp; 3Qp-3. 01; 0; 3; 3; 4.77; 4.77; 6; 6; 6. 10; Reserved. 11; Reserved.

In Rel.18, since the maximum number of PUSCH layers has been extended from 4 to 8, it is desired to define the values offor more than 4 PUSCH layers. First of all, when the higher layer parameter ptrs-Power is ‘01’, the value of = 10log10().

Proposal 5: For PTRS power boosting for 8Tx, when the higher layer parameter ptrs-Power is ‘01’, = 10log10().

When the higher layer parameter ptrs-Power is ‘00’, it is also straightforward to define the value of for full coherent case (= 10log10()) and non-coherent/non-codebook based cases (= 3N-3, where N is the number of scheduled PTRS ports). For partial coherent case, the assumption of port coherency scheme needs to be considered to determine the value of . In AI 9.1.4.2, it has been agreed that two or four coherent groups are assumed for 8 Tx partial coherent UE, and the antenna ports within one group are assumed to be full coherent. The assumptions can be reused in this AI.

Agreement in RAN1#110b For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7} - Note: Other alternatives which are not foreseen are not precluded Agreement in RAN1#112 For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used

For 8 Tx partial coherent case, each PTRS port is shared by up to 4 DMRS ports which are transmitted by one 4 Tx coherent group or two 2 Tx coherent groups. Therefore, for each PTRS port, it is possible to reuse the current table by applying the number of associated DMRS ports to the number of PUSCH layers. Where Qp is fixed to 1.

For example, assume that the higher layer parameter ptrs-Power is ‘00’, the number of PUSCH layers is 6, and each PTRS port is shared by 3 DMRS ports. If UE is equipped with two 4 Tx coherent groups, according to the table, 4.77 will be selected, and the value of is 3N-3+4.77, where N is the number of scheduled PTRS ports. Alternatively, if UE is equipped with four 2 Tx coherent groups, 3Qp-3 = 0 will be selected, and the value of can be 3N-3.

Proposal 6: For PTRS power boosting for 8Tx, when the higher layer parameter ptrs-Power is ‘00’,

- For full coherent case, = 10log10()

- For non-coherent/non-codebook based cases, = 3N-3, where N is the number of scheduled PTRS ports

- For partial coherent case, Table 6.2.3.1-3 of TS 38.214 can be reused for the determination of.

Conclusion

In this contribution, we gave our views on the detailed design of Rel-18 DMRS ports. The following proposals and observation are achieved:

Proposal 1: Confirm the working assumption of DMRS port combinations for 2 CWs in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH.

Observation 1: MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group can be achieved by scheduling restriction at gNB side and does not require additional enhancement for legacy UEs and Rel.18 UEs.

Proposal 2: For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, it can be supported by gNB implementation without spec impact.

Proposal 3: Confirm the working assumption of separate DMRS ports tables for PUSCH with rank = 5-8.

Proposal 4: For 8Tx PUSCH, support up to 2 ports PTRS for CP-OFDM.

Proposal 5: For PTRS power boosting for 8Tx, when the higher layer parameter ptrs-Power is ‘01’, = 10log10().

Proposal 6: For PTRS power boosting for 8Tx, when the higher layer parameter ptrs-Power is ‘00’,

- For full coherent case, = 10log10()

- For non-coherent/non-codebook based cases, = 3N-3, where N is the number of scheduled PTRS ports

- For partial coherent case, Table 6.2.3.1-3 of TS 38.214 can be reused for the determination of.

References

[1] Draft_Minutes_report_RAN1#112_v040