3GPP TSG RAN WG1 #112bis-e		R1-2302594

e-Meeting, April 17th – April 26th, 2023

Agenda Item:     9.2.2.2

Source:	Spreadtrum Communications

Title:	             Discussion on other aspects on AI/ML for CSI feedback

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. Three use cases are identified as initial set of use case shown below, and representative sub use cases should be recognized by RAN#98.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

The assessment of potential specification impact for use cases are also one part of the work of the SID, including RAN1, RAN2 and RAN4.

Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In previous meetings [2, 3], for use case-CSI feedback enhancement, after extensive discussion, we have the following agreement and conclusion on sub use cases.

Agreement Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case. Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

In this paper, we would discuss sub use case, and present our initial thoughts on potential specification work.

Discussion

Potential specification impact for sub use case - Spatial-frequency domain CSI compression

For the sub use case - Spatial-frequency domain CSI compression, one illustration can be seen in Figure 1,

Figure 1 One illustration for sub use case - CSI compression and recovery

where one AI/ML module/encoder with the functionality of compressing information located at UE side, and one AI/ML module/decoder with the functionality of recovering the compressed information located at gNB side. From Figure 1, it can be observed that the functionality of AI/ML module is still embedded in the legacy CSI framework, i.e., based on CSI-RS measurement, calculating CSI, and then reporting CSI. But we also notice that there are some places to be enhanced. For example, how to align the compression behavior and recovery behavior between UE and gNB.

Proposal 1: Legacy CSI framework can be reused for the sub use case - Spatial-frequency domain CSI compression. Additional enhancement can be considered.

The lifecycle of AI/ML model usually include model training, model validation, model test, model deployment, model inference, model monitoring, and model update. In general, model training includes model training, model validation, and model test. Next, we would discuss the standard impact on legacy CSI framework possibly brought about by AI/ML operation, from the perspective of the lifecycle of AI/ML model.

Model training

Regarding AI/ML model training for two-sided model, we have the following agreement shown below [4]:

Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded.

Furthermore, in RAN1#111 meeting [3], we have the following conclusion.

Conclusion In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

To facilitate the discussion on model training, last meeting [6], we have the conclusion to provide further guidance.

Conclusion In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects: - Whether model can be kept proprietary - Requirements on privacy-sensitive dataset sharing - Flexibility to support cell/site/scenario/configuration specific model - gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware - Model update flexibility after deployment - feasibility of allowing UE side and NW side to develop/update models separately - Model performance based on evaluation in 9.2.2.1 - Whether gNB can maintain/store a single/unified model - Whether UE device can maintain/store a single/unified model - Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use - Whether training data distribution can be matched to the device that will use the model for inference - Whether device capability can be considered for model development - Other aspects are not precluded Note: training data collection and dataset/model delivery will be discussed separately

For type 1, AI/ML model can be trained at UE side or gNB side. If the model is trained at UE side, CSI reconstruction part should be transferred to NW side. If the model is trained at NW side, the CSI generation part should be transferred to UE side. Model representation format (MRF) is needed to be defined in 3GPP. Since model is generated at one side, compatibility with the hardware of another side would become the key issue, e.g., compiling capability, buffer size and so on.

For type 2, joint training of the AI/ML model at NW side and UE side respectively is carried out. No model delivery/transfer is expected, but there would be gradient/training dataset exchange between two sides. It can match well with UE and NW hardware respectively, and the model proprietary can be kept. The performance is also good, since the whole models are trained in the same FP&BP loop, and optimized loss function can be considered by NW for MU-MIMO, CJT, NCJT, etc. However, it would cause large overhead, and multiple models to be maintained at both UE side and NW side. That’s the main reason that we have concluded training type2 is deprioritized.

For type 3, separated training between UE side and NW side is executed without model transfer, where training dataset should be delivered for either UE first training or NW first training. Generation model and reconstruction model are not trained in the same FP loop and BP loop. Thus, the performance may be not good. However based on the observation from papers for AI9.2.2.1, the performance loss is marginal. In addition, the models generated by type3 can match well with UE and NW hardware respectively, and the model proprietary can be kept. By proper generalization methods, it is possible that only one model is needed for some functionality either at UE side or NW side.

Following the guidance of the conclusion achieved last meeting, further analysis on pros and cons of Training type 1, 2 and 3 are provide in Table 1.

Table 1 Analysis on Pros and Cons of Training type 1, 2 and 3

|  | Training Type 1 | Training Type 2 | Training Type 3 |
| --- | --- | --- | --- |
| Whether model can be kept proprietary | × | √ | √ |
| Requirements on privacy-sensitive dataset sharing | √ | × | × |
| Flexibility to support cell/site/scenario/configuration specific model | √ | × | √ |
| gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware | × | √ | √ |
| Model update flexibility after deployment | √ | × | √ |
| feasibility of allowing UE side and NW side to develop/update models separately | × | × | √ |
| Model performance based on evaluation in 9.2.2.1 | good | well | well |
| Whether gNB can maintain/store a single/unified model | √, if gNB generates the model; otherwise × | × | √ |
| Whether UE device can maintain/store a single/unified model | √, if UE generates the model; otherwise × | × | √ |
| Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | × | × | √ |
| Whether training data distribution can be matched to the device that will use the model for inference | √ | √ | √ |
| Whether device capability can be considered for model development | × | √ | √ |

Proposal 2: To facilitate the discussion, views on Pros and Cons of all of Training types are needed to be aligned. What shown in Table 1 can be considered.

Data collection

In RAN1#112 meeting [6], we have the following agreement on data collection.

Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

Data collection for model training stage

For training type 1, there seems to be no need to do enhancement other than assisted signaling when training at UE side since channel information can be achieved by UE naturally. Considering model generality issue, assisted signaling such as antenna layout, TXRU mapping of NW side can be also included into CSI-RS resource configuration.  For training at NW side, SRS can be utilized to achieve the channel information.

For training type 2, it is joint training at both UE side and gNB side, then the dataset exchange for one side is necessary.

For training type 3, For UE first training, the original channel information can be achieved by utilizing CSI-RS. However, dataset for gNB to train the reconstruction model is needed to be delivered by UE to gNB, and enhancement may be needed. It is similar for NW first training case.

Proposal 3: For AI/ML model training type 2 and type 3, data collection is needed.

Proposal 4: For AI/ML model training Type 1, data collection may be not needed to be specified other than assisted signalling, e.g, antenna layout for one CSI-RS resource.

For online training of AI/ML model, additional complexity is needed, e.g., a large amount of data needed to be storage, and tackled. It can not be beared at UE side, e.g, for smartphone. Thus, we suggest to focus on offline AI/ML model training at the first stage. Online training for advanced UE, e.g., laptop, can be considered in future.

Proposal 5: Offline AI/ML model training is the first priority.

For offline training, we think there is no need to have specification work. It is up to implementation.

Data collection for fine-tuning/update stage

Different from model training, relatively little of dataset is needed for model fine-tuning/update. The dataset collection procedure can be considered if model fine-tuning/update is supported.

If model transfer assumed, it seems data collection is not needed since both UE and gNB can achieve the original channel information by utilizing CSI-RS and SRS, respectively.

If model transfer not assumed, data collection may be needed. For data collection at UE side for AI/ML model training type3 with NW first training, one possible procedure is shown as Figure 2-a. Therein step 3-a is optional. If SRS can be used to achieve original channel information at gNB side, option 3-a can be dropped.

| Figure 2-a: Data collection at UE side | Figure 2-b: Data collection at NW side |
| --- | --- |

Figure 2 Data collection procedure

For data collection at NW side for AI/ML model training type3 with NW first training, SRS can be utilized, otherwise, ground-truth CSI reporting is needed.

Proposal 6: If model transfer supported, data collection procedure is not needed.

Proposal 7: If model transfer not supported, for UE side, data collection procedure may be needed.

Proposal 8: If model transfer not supported, for NW side, data collection procedure may be needed or not depending on whether SRS can be utilized.

Data collection for monitoring stage

There are some monitoring metrics that may require data, such as monitoring based on input or output. However, in our mind, the justification is not clear. Firstly, monitoring based on system performance can be utilized, which is kind of being up to implementation. After all, the ultimate goal of AI/ML enabled air interface is to improve the system/UE performance. Secondly, the benefit of monitoring based on input/output is not clear.

Observation 1: It may be not necessary to do data collection for model monitoring

Model inference

For legacy CSI framework, one UE could be configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList) in one CSI-MeasConfig.

Figure 3 Legacy CSI framework

For P/SP CSI report, one CSI reporting setting could be linked to two resource settings where the first one is for channel measurement, the second is used for interference measurement performed on CSI-IM. For AP CSI report, one CSI reporting setting could be linked to two or three resource settings, where NZP CSI-RS could be used for interference measurement except from CSI-IM. Only one CSI-RS resource set is configured for each P/SP CSI resource setting. For AP CSI resource setting, multiple CSI-RS resource sets could be configured per CSI resource setting, but only one resource set would be selected by DCI. In principle, for AI//ML enabled CSI feedback, all time domain behavior can be considered. But considering AI/ML model may be scenario specific, and the high complexity, we prefer to consider aperiodic CSI reporting enabled by AI/ML firstly.

Proposal 9: Aperiodic CSI reporting should be considered firstly.

In legacy system, based on CSI-ResourceConfig and/or CSI-ReportConfig, UE could be aware of the specific measurement behavior. For example, if reportQuantity in CSI-ReportConfig is set to 'cri-RI-PMI-CQI', it implies that the CSI report should include CRI, RI, PMI, and CQI. Thus, the specification should be enhanced to make UE aware of the CSI report enabled by AI/ML.

Proposal 10: The configuration of CSI-ResourceConfig and/or CSI-ReportConfig should be enhanced.

Regarding CSI report, we have the following agreement [6]:

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

The output of CSI generation part can be taken as the PMI. In legacy system, CQI, RI and PMI are jointly calculated. However, for AI/ML based CSI compression, it is not known to UE about how to calculate PMI by NW. In current specification, in some degree, CQI calculation could reflect UE’s receiver capability. This is also important and should be considered in AI/ML based case. If AI/ML model is trained per rank, RI information also should be included to reflect AI/ML model selection. Thus, in our mind, CQI/RI still should be included in the CSI report.

Proposal 11: CQI/RI still should be included in the CSI report.

Regarding CQI calculation, for option 2a, CSI reconstruction part is assumed to be available at UE side. In our understanding, it seems not to be realistic. The SID has clearly stated that the privacy should be kept. However, option 2a would bring about privacy exposure. In addition, it also requires UE to store/maintain recovery model, and more storage size is needed. For option 2b, it is essentially equal to option 1a. CSI-RS can be precoded or not depending on gNB’s implementation and configuration. In final, for option 2b, CQI still should be calculated based on target CSI with realistic channel measurement. For option 1c, it would largely increase the UE complexity for the sake of that the legacy PMI calculation is also needed other than AI/ML operation. Assuming ideal CSI compression and recovery by AI/ML operation, option 1a can work well. Even if it is not ideal, option 1a/option 1b still can be considered with some compensation at UE side or NW side.

Proposal 12: Regarding CQI calculation, option 1a and/or option 1b can be considered.

Regarding the co-existence with legacy non-AI/ML-based CSI, after discussion, we have the following agreement [5, 6].

Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission Agreement - Codebook subset restriction - CSI processing Unit

In legacy system, one CSI report is associated with a priority value. It is introduced to solve the collision when the time occupancy of the physical channels scheduled to carry CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. Since the high cost and potential high performance to be achieved for AI/ML enabled CSI compression and recovery, the priority for AI/ML based CSI feedback may be needed to be considered.

Proposal 13: The priority for AI/ML based CSI feedback needs to be considered.

In current specification [7], priority value for one CSI report is defined as PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where

- y=0 for aperiodic CSI reports to be carried on PUSCH, y=1 for semi-persistent CSI reports to be carried on PUSCH, y=2 for semi-persistent CSI reports to be carried on PUCCH and y=3 for periodic CSI reports to be carried on PUCCH;

- k=0 for CSI reports carrying L1-RSRP or L1-SINR and  k=1 for CSI reports not carrying L1-RSRP or L1-SINR;

- c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-s is the reportConfigID and Msis the value of the higher layer parameter maxNrofCSI-ReportConfigurations.

A first CSI report is said to have priority over second CSI report if the associated PriiCSIy,k,c,s value is lower for the first report than for the second report.

Considering AI/ML enabled CSI compression and recovery could provide high accuracy CSI feedback and low overhead, we prefer to set the priority of AI/ML enabled CSI compression and recovery higher than legacy CSI carrying RI/CQI/PMI. It can be achieve by only setting k is larger than 1, e.g., 2 while the priority calculation still is reused. It is one simple solution. We are also open to other solutions.

Proposal 14: Introducing k>1  for CSI reports carrying CSI compression information enabled by AI/ML operation in the priority rule for CSI reports.

Codebook subset restriction is applied to provide codebook subset restriction when UE calculating PMI, to reduce UE’s complexity. However, since there is no codebook for AI/ML enabled CSI compression and recovery, the codebook subset restriction can not be considered in CSI compression using two-sided model use case.

Observation 2: Codebook subset restriction can be not considered in CSI compression and recovery using two-sided model use case.

In legacy system, for demonstrating the complexity of CSI measurement and reporting, CSI processing unit (CPU) and CSI processing time requirement are introduced. For AI/ML enabled operation, the computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm, which may be totally different from legacy method. How to define and reflect the complexity of CSI feedback enabled by AI/ML operation in the specification should be considered. For example, considering the high complexity, the occupied CPUs may be not only denoted by the number of CMR resources associated with the CSI report, and additional enhancement is needed.

Proposal 15: How to define/reflect the complexity of the AI/ML operation in the specification should be considered.

Model monitoring and update

Regarding model monitoring, we have achieved great progress in previous meeting, and have the following agreements [5, 6].

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

Based on the above, there are multiple performance monitoring metrics/methods for both NW-side and UE-side performance monitoring to be considered. Next, we would provide our analysis on each metric for NW-side and UE-side performance monitoring separately.

For NW-side performance monitoring,

  - Intermediate KPIs: Since gNB can not directly achieve raw channel information, UE would be needed to report the information over the air interface. It would cost a large of overhead.

  - Eventual KPIs: The performance can be based on PDSCH demodulation performance. For example, based on the probability of NACK in one duration, gNB could evaluate the accuracy of the CSI report including the compressed information generated by AI/ML module. For this metric, like legacy behavior, it is up to gNB’s implementation, and no spec impact is expected.

  - Legacy CSI based monitoring:  Since the high cost/complexity of AI/ML module, we think that higher performance requirement should be set for AI/ML enabled CSI feedback. Otherwise, the performance gain compared to legacy CSI feedback, e.g., Type-II codebook, can not be reflected, and legacy CSI feedback may be enough. Thus, legacy CSI based monitoring can be considered.

  - Input or Output data based monitoring: The validity of the AI/ML output, e.g., out-of-distribution detection, drift detection of output data, or something simple like checking SNR, delay spread and so on, can be considered, since the data distribution of AI/ML output can reflect the applied scenario/usage of AI/ML model.

For UE-side performance monitoring,

  - Intermediate KPIs: The final inference result is obtained by gNB. gNB could deliver the result to UE side for SGCS evaluation over the air interface. A large of overhead would be consumed. Another way is to simulate gNB’s AI/ML operation (e.g., assuming generation model is available at UE side) to achieve the final inference for UE. Obviously, it would bring huge burden on UE. In addition, model transfer/exposure is needed.

  - Eventual KPIs: The performance can be based on PDSCH demodulation performance. The performance metric can be BLER, the probability of NACK and others. On the other hand, in our mind, the UE is more sensitive to complexity. If there is no enough computation resource reserved, UE also can request to deactivate AI/ML operation for CSI feedback. However, the monitoring behavior can be totally up to UE’s implementation.

  - Legacy CSI based monitoring: Obviously, it is not applicable for UE-side performance monitoring.

  - Input or Output data based monitoring: The validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread and so on, can be considered, since the data distribution of AI/ML input can reflect the applied scenario/usage of AI/ML model.

Observation 3: For UE-side performance monitoring, eventual KPIs and input data based monitoring metric can be considered.

Observation 4: For NW-side performance monitoring, eventual KPIs, legacy CSI based monitoring and output data based monitoring metric can be considered.

Potential specification impact for CSI prediction

For CSI prediction with UE-sided model, in our mind, data collection, model training, and model deployment can be up to UE’s implementation. The model identification stage is needed, and it can be functionality based identification. For inference stage, other than model activation / deactivation/ update/ switching mechanism, the assisted signaling such measured resource and predicted CSI window may be needed. For performance monitoring, UE could monitor it by itself, and report the decision to NW. The potential specification impact may include the following:

  - Model identification

  - Model activation/deactivation/update/switching/monitoring by UE or NW

  - Assisted signaling, such as measurement resources and predicted CSI window

Proposal 16: For CSI predication, the potential specification impact includes model identification, model activation/deactivation/update/switching/monitoring by UE or NW, and assisted signaling such as measurement resources and predicted CSI window.

Conclusion

In this contribution, we provide our opinions on standard impacts of sub use case – CSI compression and recovery and CSI prediction:

Observation 1: It may be not necessary to do data collection for model monitoring

Observation 2: Codebook subset restriction can be not considered in CSI compression and recovery using two-sided model use case.

Observation 3: For UE-side performance monitoring, eventual KPIs and input data based monitoring metric can be considered.

Observation 4: For NW-side performance monitoring, eventual KPIs, legacy CSI based monitoring and output data based monitoring metric can be considered.

Proposal 1: Legacy CSI framework can be reused for the sub use case - Spatial-frequency domain CSI compression. Additional enhancement can be considered.

Proposal 2: To facilitate the discussion, views on Pros and Cons of all of Training types are needed to be aligned. What shown in Table 1 can be considered.

Proposal 3: For AI/ML model training type 2 and type 3, data collection is needed.

Proposal 4: For AI/ML model training Type 1, data collection may be not needed to be specified other than assisted signalling, e.g, antenna layout for one CSI-RS resource.

Proposal 5: Offline AI/ML model training is the first priority.

Proposal 6: If model transfer supported, data collection procedure is not needed.

Proposal 7: If model transfer not supported, for UE side, data collection procedure may be needed.

Proposal 8: If model transfer not supported, for NW side, data collection procedure may be needed or not depending on whether SRS can be utilized.

Proposal 9: Aperiodic CSI reporting should be considered firstly.

Proposal 10: The configuration of CSI-ResourceConfig and/or CSI-ReportConfig should be enhanced.

Proposal 11: CQI/RI still should be included in the CSI report.

Proposal 12: Regarding CQI calculation, option 1a and/or option 1b can be considered.

Proposal 13: The priority for AI/ML based CSI feedback needs to be considered.

Proposal 14: Introducing k>1  for CSI reports carrying CSI compression information enabled by AI/ML operation in the priority rule for CSI reports.

Proposal 15: How to define/reflect the complexity of the AI/ML operation in the specification should be considered.

Proposal 16: For CSI predication, the potential specification impact includes model identification, model activation/deactivation/update/switching/monitoring by UE or NW, and assisted signaling such as measurement resources and predicted CSI window.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Draft Report of 3GPP TSG RAN WG1 #109-e

- Draft Report of 3GPP TSG RAN WG1 #111

- Draft Report of 3GPP TSG RAN WG1 #110

- Draft Report of 3GPP TSG RAN WG1 #110b-e

- Draft Report of 3GPP TSG RAN WG1 #112

- 3GPP TS 38.214: "NR; Physical layer procedures for data"