3GPP TSG RAN WG1 #112bis-e		R1-2302597

e-Meeting, April 17th – April 26th, 2023

Agenda Item:     9.2.4.2

Source:	Spreadtrum Communications

Title:	             Discussion on other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. Three use cases are identified as initial set of use case shown below, and representative sub use cases should be recognized by RAN#98.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

The assessment of potential specification impact for use cases are also one part of the work of the SID, including RAN1, RAN2, and RAN4.

Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In RAN1#110 [2], for use case - Positioning accuracy enhancements, after extensive discussion, we have the following agreement.

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results

In RAN1#110b-e [3], based on AI/ML based positioning method and AI/ML model located node, we have the following agreement on the cases of AI/ML based positioning.

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In this paper, we would present our initial thoughts on potential specification work.

Discussion

In this paper, we would discuss the standard impact brought about by AI/ML operation, from the perspective of the lifecycle of AI/ML model. The lifecycle of AI/ML model usually include model training, model validation, model test, model deployment, model inference, model monitoring, and model update. In general, model generation includes model training, model validation, and model test.

- Model deployment

For both direct AI/ML positioning and AI/ML assisted positioning, in general AI/ML model can be located either at UE side or NW side, both of which are one-sided model. For one-sided model, normally, the AI/ML model can be offline trained by the side as inference node, or can be offline trained by one side and transferred to inference node. For the latter, more specification work is needed, e.g., how to transfer AI/ML model, the parameters of AI/ML model, and so on, which is one general issue to be discussed in AI9.2.1.

Observation 1: For both of direct AI/ML positioning and AI/ML assisted positioning, AI/ML model can be delivered or not. It can wait for the progress of AI9.2.1.

Considering how to deliver AI/ML model and how to define an AI/ML model representation format (MRF) across platforms are not clear at present, which is also being discussed in RAN2/SA, we suggest that at present we can focus on AI/ML model training and inference located at the same side firstly.

Proposal 1: For both of direct AI/ML positioning and AI/ML assisted positioning, suggest to focus on the training and inference located at the same side at present.

- Model inference

Last meeting [4, 5], we have the following agreement about AI/ML model inference.

Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

From the perspective of input/output of AI/ML model, the possible specification impact can include the following:

- New measurement metric and reporting, depending on the input/output of AI/ML model

  - For case 2a/3a, the output of AI/ML model can be new metric, e.g., LOS/NLOS hard/soft judgement, TOA, path phase. Whether to support either or both can depend on the progress of AI9.2.4.1.

  - For UE/NG-RAN node assisted positioning and AI/ML model located at LMF (i.e., case 2b/case 3b), CIR/PDP can be as the input of AI/ML model, and they should be reported by UE/ NG-RAN to LMF. Whether to support either or both of CIR and PDP can depend on the progress of AI9.2.4.1.

  - For case 1, it can be up to implementation, and no enhancement is needed.

- Procedure to enable direct AI/ML positioning and AI/ML assisted positioning

  - From the perspective of configuration and reporting procedure, it seems to be enough to reuse the current procedure where AI/ML related information should be included in legacy message.

  - AI/ML model activation/deactivation/switching/update related procedure may be needed to be considered. But it can be waited until the progress of 9.2.1.

Proposal 2: For case 2a and case 3a, the output of AI/ML model can be considered as new metric, e.g., LOS/NLOS hard/soft judgement, Path phase. Whether to support it depends on the evaluation of AI9.2.4.1.

Proposal 3: For case 2b and case 3b, the input of AI/ML model can be considered as new metric, e.g., CIR/PDP. Whether to support it depends on the evaluation of AI9.2.4.1.

In legacy system, there is no explicit complexity characterization for positioning procedure. However, for AI/ML enabled operation, computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm. It is newly introduced by AI/ML algorithm, and may be much higher than legacy method. Whether/how to define and reflect the complexity of positioning enabled by AI/ML operation in the specification can be studied.

Proposal 4: Whether/How to define/reflect the complexity of the AI/ML operation in the specification can be studied.

- Model monitoring and update

Regarding model monitoring, we have the following agreements [3, 4, 5]:

- Agreement - Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition - Note: Model monitoring metric calculation may be done at NW or UE Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

Since positioning is one relatively independent process, in some degree, the system performance is not related to positioning performance. System performance is not preferred as one monitoring metric.

Input data based monitoring, including the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread and so on, can be considered, since the data distribution of AI/ML input can reflect the applied scenario/usage of AI/ML model. In addition, there seem no overhead issue and no specification impact, and it can be up to implementation of the entity which the model located at.

Observation 2: Input data based monitoring can be considered for the evaluation of positioning monitoring, and it seems no spec enhancement is needed.

Regarding output data based monitoring metric, for direct positioning, e.g., case 1/case 2b/case 3b, both PRU and UE with GNSS capability can provide position information as ground-truth label to evaluate the accuracy of the output.  For AI/ML assisted positioning, the ground truth label may be LOS/NLOS identification, AOD, TDOA, etc., rather than coordinate. Output data based monitoring is not preferred since it is difficult to directly achieve the ground-truth label of the output. Although the immediate result, e.g., LOS/NLOS identification, AOD, TOA, can be inferred based on position information provided by PRU or UE with GNSS capability, it seems to be more natural to directly evaluate the accuracy of positioning.

Observation 3: For AI/ML assisted positioning, monitoring based on model output is not necessary.

Proposal 5: For direct AI/ML positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label for output based monitoring metric.

Proposal 6: For AI/ML assisted positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label to evaluate the positioning accuracy.

Given data privacy issue, for case 1/case 2a/case 2b/case 3a/case 3b, ground-truth label can be reported to LMF, and it is up to LMF to monitor and evaluate the performance of AI/ML. Then the decision can be delivered by LMF to UE/gNB for case 1/case 2a/ case 3a. For case 1 with direct positioning, other mechanisms also can be considered, e.g., ground-truth label can be delivered to UE via LMF, and the model can be monitored by the UE itself.

Proposal 7: For all cases, ground-truth labels can be reported to LMF, and it is up to LMF to monitor/evaluate the performance of AI/ML model. For case 1 with direct positioning, other mechanisms also can be considered.

In addition, for monitoring based on inference accuracy, integrity can be considered to be utilized. In Rel-17, integrity is introduced for GNSS, which is a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts. In Rel-18, it has been extended to study solutions for integrity for RAT dependent positioning techniques. In our mind, the integrity mechanism can be leveraged as one tool to evaluate the performance the AI/ML model. However, considering the framework/methodology of AI/ML based positioning has not been established, it is fine to consider the integrity metric in later stage.

Observation 4: The integrity mechanism can be considered as one tool to evaluate/monitor the performance of AI/ML model.

Conclusion

In this contribution, we provide our opinions on standard impacts of positioning enabled by AI/ML:

Observation 1: For both of direct AI/ML positioning and AI/ML assisted positioning, AI/ML model can be delivered or not. It can wait for the progress of AI9.2.1.

Observation 2: Input data based monitoring can be considered for the evaluation of positioning monitoring, and it seems no spec enhancement is needed.

Observation 3: For AI/ML assisted positioning, monitoring based on model output is not necessary.

Observation 4: The integrity mechanism can be considered as one tool to evaluate/monitor the performance of AI/ML model.

Proposal 1: For both of direct AI/ML positioning and AI/ML assisted positioning, suggest to focus on the training and inference located at the same side at present.

Proposal 2: For case 2a and case 3a, the output of AI/ML model can be considered as new metric, e.g., LOS/NLOS hard/soft judgement, Path phase. Whether to support it depends on the evaluation of AI9.2.4.1.

Proposal 3: For case 2b and case 3b, the input of AI/ML model can be considered as new metric, e.g., CIR/PDP. Whether to support it depends on the evaluation of AI9.2.4.1.

Proposal 4: Whether/How to define/reflect the complexity of the AI/ML operation in the specification can be studied.

Proposal 5: For direct AI/ML positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label for output based monitoring metric.

Proposal 6: For AI/ML assisted positioning, both PRU and UE with GNSS capability can be as the entity to provide data/ground-truth label to evaluate the positioning accuracy.

Proposal 7: For all cases, ground-truth labels can be reported to LMF, and it is up to LMF to monitor/evaluate the performance of AI/ML model. For case 1 with direct positioning, other mechanisms also can be considered.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Draft Report of 3GPP TSG RAN WG1 #110

- Draft Report of 3GPP TSG RAN WG1 #110b-e

- Draft Report of 3GPP TSG RAN WG1 #111

- Draft Report of 3GPP TSG RAN WG1 #112