3GPP TSG RAN WG1 #112bis-e	                                              R1-2302604

e-Meeting, April 17th – April 26th, 2023

Agenda Item:     9.4.3

Source:	Spreadtrum Communications

Title:	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Document for:	Discussion and decision

Introduction

As agreed in Rel-18 WID on sidelink evolution, the main topics focus on initial beam-pairing, beam maintenance, and beam failure recovery, etc. In the latest RAN-1 meeting, the following agreements have been achieved:

Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

Initial beam pairing procedure

In Uu beam management, initial beam pairing is done during RACH procedure where SSB is used as the measured reference signal. However, there is no PRACH-like channel in sidelink. For legacy S-SSB, it is transmitted in a SFN-like way where no UE-specific or beam-specific information is carried. Besides, the S-SSB’s bandwidth is not broadband limiting its ability on beam measuring. Thus, legacy S-SSB is not appropriate for beam management.

Initial beam pairing means a coarse beam(s) determination, which is not necessary confined before unicast link establishment. Even after link establishment, such coarse procedure is still beneficial especially considering the overhead brought by fined beam management, e.g., via SL CSI-RS. For low mobility and latency non-sensitive scenario, rough beam pairing could be sufficient. It is favorable to find a universal initial beam pairing procedure applied for both before or after unicast link establishment.

Figure.1 Illustration of S-SSB for beam management

Proposal-1: Initial beam pairing procedure can be executed before or after unicast link establishment.

Proposal-2: Beam management oriented S-SSB could be introduced in sidelink FR2.

Beam maintenance

In R15/16, there exist two limitations for sidelink transmission:

Constraint-1: Except for S-SSB, there is no periodic/semi-persistent reference signal transmission

Constraint-2: Except for S-SSB, the transmission of standalone RS is not allowed.

However, in order to monitor the quality of directional channel at any time like Uu’s, it is necessary to introduce periodic or semi-persistent SL CSI-RS in sidelink FR2. Meanwhile, this introduction naturally means breaking both of the two constraints, since standalone SL CSI-RS is probably unavoidable for beam management.

Figure.2 Illustration of slot structure for SL-CSI-RS

Proposal-3: Periodic/Semi-persistent CSI-RS should be supported for sidelink beam maintenance.

Proposal-4: Standalone CSI-RS for beam maintenance should be supported for sidelink FR2.

Under the beam management architecture of Rel-15, each type or resource of uplink and downlink transmission such as CORESET/PUSCH/PUCCH/CSI-RS/SRS can be configured with its corresponding beam direction. It can be said that flexibility reached its peak in the R15. However, it is discovered that this flexibility is actually redundant, and beams of different types or different resources can actually use the same beam in most cases. Therefore, a large proportion of spec-effort on the beam management enhancements of R16 and R17 are to simplify the complexity of beam management architecture. We believe that the Rel-17 unified TCI framework should be a baseline for sidelink beam management, which can configure the beam directions for sidelink's bidirectional transmission at once, while also configuring the same beam for many different types of channels. Meanwhile, the four types of QCL (A/B/C/D) could all be considered for sidelink FR2.

Proposal-5: Rel-17 unified TCI framework should be a baseline for sidelink beam management.

Proposal-6: QCL type A/B/C/D should all be considered for sidelink FR2.

Beam correspondence is signaled as a UE’s capability in Uu link. Similar assumption can also be used for sidelink FR2. It is favorable to find a universal beam maintenance procedure that can be applied for both beam-correspondence and non-beam-correspondence cases.

Proposal-7: Both beam-correspondence and non-beam-correspondence cases should be considered for sidelink FR2.

Beam failure recovery

Figure.3 Illustration of SL BFR procedure

In Uu BFR procedure, periodic BFD-RS (q0) and candidate-RS (q1) are supported for beam quality monitoring and new beam identification, respectively. Similar to Uu’s case, we can also introduce the concept of q0 and q1 for sidelink beam failure recovery. Beam failure instance can inherit the definition of Uu’s, i.g., based on the monitoring result of q0. However, besides q0 there are more options for determining the beam quality. For instance, HACK-ACK information, CSI report and CBR values can also be considered, which also directly or indirectly reflect beam quality.

Proposal-8: The definition of beam failure instance can be determined via a down-selection among the following parameters:

- CSI-RS measurement

- HACK-ACK

- CSI report

- CBR value

In Uu link, UE always follows the instruction of gNB’s signaling and transits BF information to gNB if BF event occurs. Unlike Uu’s case, the subject importance of Tx-UE and Rx-UE is more equivalent. Thus, beam failure monitoring/declaration can be done at Tx-UE or Rx-UE. The relevant details could be further investigated.

Proposal-9: The declaration of beam failure event could be Tx-UE or Rx-UE, and further investigations are expected.

Miscellaneous

Directional transmission is regarded as a fundamental feature in SL FR2. Its impact is not only for the aspects list above. Resource (re-)selection counting the directional feature is a potential enhancement. Besides, directional CBR measurement could be considered in sidelink FR2.

Proposal-10: Resource (re-)selection should be re-evaluated counting the directional feature.

Proposal-11: Directional CBR measurement could be considered in sidelink FR2.

Conclusion

In this contribution, we provide our view on SL FR2 in Rel-18. The followings are our proposals:

Proposal-1: Initial beam pairing procedure can be executed before or after unicast link establishment.

Proposal-2: Beam management oriented S-SSB could be introduced in sidelink FR2.

Proposal-3: Periodic/Semi-persistent CSI-RS should be supported for sidelink beam maintenance.

Proposal-4: Standalone CSI-RS for beam maintenance should be supported for sidelink FR2.

Proposal-5: Rel-17 unified TCI framework should be a baseline for sidelink beam management.

Proposal-6: QCL type A/B/C/D should all be considered for sidelink FR2.

Proposal-7: Both beam-correspondence and non-beam-correspondence cases should be considered for sidelink FR2.

Proposal-8: The definition of beam failure instance can be determined via a down-selection among the following parameters:

- CSI-RS measurement

- HACK-ACK

- CSI report

- CBR value

Proposal-9: The declaration of beam failure event could be Tx-UE or Rx-UE, and further investigations are expected.

Proposal-10: Resource (re-)selection should be re-evaluated counting the directional feature.

Proposal-11: Directional CBR measurement could be considered in sidelink FR2.

References

[1] Report of 3GPP TSG RAN WG1 #112