3GPP TSG RAN WG1 #112bis		                R1-2302608

Online, April 17th – April 26th, 2023

Source:               Spreadtrum Communications

Title:                   Discussion on NR DL and UL carrier phase positioning

Agenda Item:     9.5.2

Document for:   Discussion and Decision

Introduction

The work item for NR DL and UL carrier phase positioning was approved in RAN#98e [1] to improve positioning accuracy, and some objectives are as follows:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In the RAN1#112 meeting, some agreements on assumptions to evaluate the carrier phase positioning and observations were achieved [2]. In this contribution, we provide our views on the carrier phase positioning from RAN1 perspective.

Impact of NR CPP on specification

In this section, the impact of NR CPP on specification will be discussed.

NR carrier phase measurements

In RAN1#112 meeting, the following agreement was made on the NR carrier phase measurements.

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP

For NR carrier phase positioning, DL and UL carrier phase measurements will subsequently be considered in accordance with the conclusions agreed at the latest RAN1#111 meeting. For DL carrier phase positioning, there are currently two candidates will be considered. First one is the difference between the carrier phase measured from the DL PRS signal(s) of the target TRP and the carrier phase measured from the DL PRS signal(s) of the reference TRP, and the second one is the carrier phase measured from the DL PRS signal(s) of a TRP.

For the first option, although UE only needs to report a difference value, the information available to the network side will be correspondingly less. For example, when UE measures PRS signal(s) from the target TRP and the reference TRP, the two links may have different error values. When only the difference between the two carrier phase measurements is reported, the network side can only make compensation according to the difference. Once an error occurs when UE receives, it is difficult for the network side to find the problem only from the difference value. For the second option, although the overhead is higher than that of the first option, more information can be obtained by gNB, which will help the gNB to find the error generated by the UE side. Therefore, two methods of reporting carrier phase should be supported.

Proposal 1: For DL carrier phase positioning, two methods of reporting carrier phase should be supported.

Different from GNSS carrier phase positioning, existing positioning technologies such as RSTD and TDOA can be used for NR carrier phase positioning to obtain integer parts. Therefore, we can stick with the existing framework. As for multi RTT method, because it has both uplink and downlink measurement, if combined with carrier phase positioning, it needs to measure the uplink and downlink carrier phase respectively. Relatively speaking, it will be more complicated than one-way positioning method. Moreover, WID does not include the combining UL and DL carrier phase positioning method at present, so it can be postponed in Rel-18. The carrier phase can be reported together with the RSTD and TDOA.

Proposal 2: The carrier phase can be reported together with the current positioning measurements, such as RSTD and TDOA.

- For integer ambiguity resolution, some agreements were reached in last meeting.

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

According to the principle of carrier phase positioning, the final distance between TRP and UE will be composed of integer and fractional parts. The value of the fractional part can only reach one wavelength at most, so the accuracy of the final positioning result greatly depends on the determination of the integer part. In order to get the final integer part, UE or LMF needs to search integer value N to get the optimal solution. In order to obtain accurate results, large search range of integer values N will be considered. However, in this case, huge complexity is brought to UE or LMF. If we can obtain a limited search range with a correct N value in based on previous measurements, it will greatly reduce complexity while maintaining accuracy.

Based on the above analysis, to reduce UE complexity, LMF can provide a search range or some alternative values for UE-based/assisted positioning. Similarly, for UE-assisted positioning, the UE can report a recommended search range or some alternative values to TRP or LMF. Hence, we have the following proposal.

Proposal 3: The range or some alternative values of integer N should be provided/reported for carrier phase positioning.

Physical Layer Procedures

In RAN1#111 meeting, the reference signals used for NR carrier phase measurements have been discussed. The agreement was reached as follows.

Agreement Capture the following TP into TR 38.859 as a conclusion (Section 6.3.3) Regarding the reference signals for NR carrier phase positioning: - Existing DL PRS and UL SRS for positioning purpose are recommended as the reference signals to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning if NR CPP is introduced. - Note: The use of SRS MIMO for NR carrier phase positioning is transparent for UE

Based on the agreement of reusing existing Rel-16 PRS and SRS for NR carrier phase positioning, and based on the analysis of reported measurements in Section 2.1, NR carrier phase measurements can be requested, measured and reported together with the existing positioning measurements. Thus, existing physical layer procedures can be re-used and enhanced for supporting the NR carrier phase positioning at least for basic carrier phase positioning method (e.g., differential carrier phase positioning), including the measurement configuration, measurement request and measurement report with potential enhancements of the message content (e.g., the configuration related to the NR carrier phase positioning).

Another point to consider if there is an impact on the physical layer procedures is the introduction of PRU in dual differential carrier phase positioning. For illustrative purposes, a figure of the dual differential carrier phase positioning with PRU is shown below. PRS will be transmitted to target UE and PRU by 2 TRPs.

Figure 1 Dual differential carrier phase positioning with PRU

For dual differential carrier phase positioning with PRU, the initial random phase error at both TRP side and UE side can be removed by dual differential. From a specification impact perspective, a PRU is a specific UE with known location, so the existing physical layer procedures can equally applicable to the PRU without additional enhancements.

As a result, existing physical layer procedures can be reused to support NR carrier phase positioning, including measurement configurations, measurement requests, and measurement reports.

Observation 1：Existing physical layer procedures can be reused to support NR carrier phase positioning.

Multipath mitigation methods for the carrier phase positioning

In RAN1#111 and RAN#1 112 meeting, the following agreements were made on NR carrier phase positioning (CPP).

Agreement Capture the following TP into TR 38.859 as a conclusion: - Multipath mitigation methods for the carrier phase positioning are recommended to be introduced during normative work, if NR CPP is introduced. The candidate solutions may include, but are not limited to, the following: - Reporting of the carrier phase of the first path - At least reporting of the carrier phase of the first path, and optionally, the additional paths. - The use of LOS/NLOS indication for the carrier phase measurements. - Note: Rel-17 LOS/NLOS indicator can be considered as a starting point. - Reporting of other channel information together with carrier phase measurements, such as existing RSRP/RSRPP. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

The calculation of positioning system mainly depends on the first path signals measurement, carrier phase positioning is no exception. Therefore, it is important to identify and separate the first path from the others.

Optionally reporting the carrier phase of additional paths will also help improve positioning performance. Since the main purpose of this chapter is to further improve positioning accuracy, additional path reports should also be considered.

Proposal 4: For carrier phase positioning, both carrier phases of the first path and the additional paths can be reported.

For reporting other channel information, such as RSRP/RSRPP are also used to enhance positioning techniques in Rel-17. Thus, they can be further introduced into carrier phase positioning.

Proposal 5: For multipath mitigation, support the reporting of other existing channel information together with carrier phase positioning.

Conclusion

In this contribution, we provided our opinions on representative sub use cases and potential specification impact on AI/ML for beam management. The following proposals have been achieved:

Proposal 1: For DL carrier phase positioning, two methods of reporting carrier phase should be supported.

Proposal 2: The carrier phase can be reported together with the current positioning measurements, such as RSTD and TDOA.

Proposal 3: The range or some alternative values of integer N should be provided/reported for carrier phase positioning.

Proposal 4: For carrier phase positioning, both carrier phases of the first path and the additional paths can be reported.

Proposal 5: For multipath mitigation, support the reporting of other existing channel information together with carrier phase positioning.

Observation 1：Existing physical layer procedures can be reused to support NR carrier phase positioning.

Reference

- RP-223549, “New WID on Expanded and Improved NR Positioning”, 3GPP TSG RAN Meeting #98e.

- Chair’s Notes, 3GPP TSG RAN WG1 #112 v20