3GPP TSG RAN WG1 #112bis-e                                                  R1-2302609

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.3

Source:	Spreadtrum Communications

Title:	Discussion on LPHAP (Low Power High Accuracy Positioning)

Document for:	Discussion and decision

Introduction

The WID on LPHAP is approved in [1].

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this paper, we share our views of positioning in RRC_INACTIVE.

SRS for positioning configurations in multiple cells

In R17, the SRS configuration is considered as valid under the following criteria:

- The inactivePosSRS-TimeAlignmentTimer is running;

- RSRP increased/decreased within inactivePosSRS-RSRP-ChangeThreshold;

- No cell re-selection happened after receiving the configuration;

- If the UE in RRC_INACTIVE mode determines that the UE is not able to accurately measure the configured DL RS in SRS-SpatialRelationInfoPos for a SRS resource for positioning, the SRS transmission is suspended.

- If UE determines that it cannot accurately measure the pathloss RS, the SRS transmission is suspended.

For a configured SRS in multiple cells, when cell reselection happens within the SRS positioning validity area, the UE can keep the SRS configuration and continue the SRS transmission if the UE is under a positioning procedure. However, due to the mobility of the UE, potential cell reselection will occur within the multiple cells, which may be cause the SRS configuration to no longer be valid.

Timing alignment and UL Interference

In RAN1#112, the following agreement on UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area had been achieved [2].

Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

In Rel-17, in order to avoid interference between UEs in the serving RAN node, UE needs to maintain the validity of the TA of the serving cell for SRS transmission, including the following criteria.

- The inactivePosSRS-TimeAlignmentTimer is running;

- RSRP increased/decreased within inactivePosSRS-RSRP-ChangeThreshold.

However, due to the mobility of the UE, potential cell reselection will occur within the multiple cells. When the UE camped-on a new cell, which belongs to the multiple cells, how does the UE determine the TA value of sending SRS. The following three options had been agreed to further study in last meeting.

- Option 1: UE maintains the TA obtained from the last serving cell within the validity area

- Option 2: UE autonomously adjusts the TA

- Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

For Option 1, to maintain the TA from the old serving cell, the geometric coverage of cells within the validity area should be similar, and the size of the validity area taking into the account of synchronization conditions between cells should be covered by CP.

For Option 2, UE autonomously adjusts the TA based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell.

For Option 3, when the UE camped-on a new cell, UE can update the TA for the new cell by RACH. In this case, UE needs to initiate random access frequently to obtain TA, which causes high UE power consumption.

In our view, Option 1 and/or Option 2 can be considered for timing alignment within multiple cells.

Proposal 1: The following TA determination schemes can be considered for timing alignment within multiple cells.

- Option 1: UE maintains the TA obtained from the last serving cell within the validity area

- Option 2: UE autonomously adjusts the TA

Spatial relation

The following agreement on the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state had been achieved [2].

Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration.

In Rel-17, the spatial relation validity criteria for SRS transmission in RRC_inactive mode is as follows:

- If the UE in RRC_INACTIVE mode determines that the UE is not able to accurately measure the configured DL RS in SRS-SpatialRelationInfoPos for a SRS resource for positioning, the SRS transmission is suspended.

For the spatial relation information is absent in the configuration, considering UE power consumption, we prefer UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter. The criterion on UE determination of the fixed spatial domain transmission filter is up to UE implementation.

Proposal 2: For the spatial relation information is absent in the configuration, UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter.

For the spatial relation information is provided in the configuration, we prefer the information can be configured subset of cells.

Proposal 3: For the spatial relation information is provided in the configuration, the information can be configured subset of cells.

Pathloss

The following agreement on the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state had been achieved [2].

Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

In Rel-17, the pathloss RS validity criteria for SRS transmission in RRC_inactive mode is as follows:

- If the UE is in the RRC_INACTIVE state and determines that the UE is not able to accurately measure PLb,f,cqd, the UE does not transmit SRS for the SRS resource set

For Option 1, the criterion on UE determination of pathloss RS is up to UE implementation. UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area.

Proposal 4: For Option 1, the criterion on UE determination of pathloss RS is up to UE implementation.

For Option 2, pathloss RS can be configured as a cell-agnostic DL RS and pathloss RS can be configured as a fixed SSB within the validity area. We prefer the information can be configured subset of cells.

Proposal 5: For Option 2, pathloss RS can be configured as a cell-agnostic DL RS and pathloss RS can be configured as a fixed SSB within the validity area.

Common SRS parameters across multiple cells

In R17, the SRS parameter configuration framework for positioning in RRC_INACTEVE mode is shown in the figure below.

Figure 1: The SRS parameter configuration framework for positioning in RRC¬_INACTEVE mode

In RAN1#112, the following agreements on common SRS parameters across multiple cells had been achieved [2].

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold

For the SRS sequenceId, whether sequenceId is configured commonly across cells or per cell had been discussed in last meeting. In our view, in order to avoid the NW to blindly monitor multiple SRS resources, the SRS sequenceId should be configured commonly across cells.

Proposal 6: The SRS sequenceId should be configured commonly across cells.

Conclusion

In this contribution, we provided initial views on LPHAP (Low Power High Accuracy Positioning). In summary, we have following observations and proposals:

Proposal 1: The following TA determination schemes can be considered for timing alignment within multiple cells.

-	Option 1: UE maintains the TA obtained from the last serving cell within the validity area

-	Option 2: UE autonomously adjusts the TA

Proposal 2: For the spatial relation information is absent in the configuration, UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter.

Proposal 3: For the spatial relation information is provided in the configuration, the information can be configured subset of cells.

Proposal 4: For Option 1, the criterion on UE determination of pathloss RS is up to UE implementation.

Proposal 5: For Option 2, pathloss RS can be configured as a cell-agnostic DL RS and pathloss RS can be configured as a fixed SSB within the validity area.

Proposal 6: The SRS sequenceId should be configured commonly across cells.

Reference

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation.

- 3GPP RAN1#112 Chairman’s Notes, February 27th – March 3rd, 2023.