3GPP TSG RAN WG1 #112bis-e                                                   R1-2302611

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.5

Source:	Spreadtrum Communications

Title:	Discussion on positioning for RedCap UEs

Document for:	Discussion and decision

Introduction

The WID on positioning for RedCap UEs is approved in [1].

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

In this paper, we share our views on frequency hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning.

PRS resource configuration for hopping

Configuration level of DL PRS frequency Hopping

In NR Rel-16/17, the UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer. Each positioning frequency layer will be configured with one or more PRS resource set(s) of one or more TRP(s). In other word, a DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets. Each DL PRS resource set consists of one or more DL PRS resource(s) where each has an associated spatial transmission filter.

Figure 2.1-1: The DL PRS resource configuration framework.

In RAN1#112, the following agreement on specification of Rx and Tx hopping for DL PRS had been achieved. Only Rx frequency hopping of the DL PRS is supported [2].

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported.

For Rx PRS frequency hopping, there are following two options:

- Option 1: UE performs frequency hopping reception of PRS within one PRS resource.

- Option 2: UE performs frequency hopping reception of PRS between PRS resources.

For Option 1, there are two PRS frequency hopping reception schemes

- Scheme 1: UE perform frequency hopping reception between different PRS transmission repetitions within the same SRS resource cycle, as shown in the Figure 2.1-2.

Figure 2.1-2: UE perform frequency hopping reception between different PRS transmission instances within the same SRS resource cycle

- Scheme 2: UE performs PRS frequency hopping reception during different transmission periodicity of the same PRS resource, as shown in the Figure 2.1-3.

Figure 2.1-3: UE performs PRS frequency hopping reception during different transmission periodicity of the same PRS resource

For Scheme 1, in existing specifications, a PRS resource can be configured with a maximum of 32 repetitions, and the minimum time interval between adjacent PRS repeated transmissions can be 1 time slot. It can meet the timing requirements of PRS frequency hopping reception. For Scheme 2, in existing specifications, the minimum PRS transmission periodicity is 4 time slots. In this case, UE performs PRS frequency hopping reception during different transmission periodicity of the same PRS resource is difficult to meet the timing requirements of PRS frequency hopping reception.

Therefore, for UE performs frequency hopping reception of PRS within one PRS resource, we prefer UE perform frequency hopping reception between different PRS transmission repetitions within one period of one PRS resource.

Proposal 1: UE perform frequency hopping reception between different PRS transmission repetitions within one period of one PRS resource can be considered.

For Option 2, UE utilizes different PRS resources to perform frequency hopping reception of PRS, as shown in Figure 2.1-4. The network can achieve UE to utilize different PRS resources for PRS frequency hopping reception through slot offset and periodicity configuration corresponding to different PRS resources.

Figure 2.1-4: DL PRS hopping between PRS resources

Proposal 2: UE performs frequency hopping reception of PRS between PRS resources can be considered.

MG based and gapless measurements for DL PRS with frequency hopping

In RAN1#112, the following agreement on MG based and gapless measurements for DL PRS with frequency hopping had been achieved [2].

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW

In order to reduce positioning delay, R17 introduces positioning measurement based on PPW. The network uses high-level signaling to configure PPW and measure gap for UE. When the PRS resource is in the BWP and the SCS is consistent with the currently active BWP. UE can apply PPW to measure the PRS resource. When PRS resources are outside the currently activated BWP or PRS SCS are inconsistent with the currently activated BWP, UE needs to apply the measurement Gap for PRS measurement. For PRS frequency hopping transmission, a PRS frequency hopping transmission cannot only be within the currently active BWP (as shown in the Figure 2.2-1), so UE cannot use PPW for PRS frequency hopping reception. In our opinion, PRS measurement based on PPW cannot be applied to PRS frequency-hopping receiver of Redcap positioning. Only measurement gap based measurement for DL PRS frequency hopping should be considered.

Figure 2.2-1:  A PRS frequency hopping transmission cannot only be within the currently active BWP.

Proposal 3: Only measurement gap based measurement for DL PRS frequency hopping should be considered.

In the existing specification, UE can be configured with per-UE measurement gap pattern and/or per-FR measurement gap pattern. The measurement gap length and measurement gap repetition period are shown in the table below [TS 38.133]. From the table, we can see that the minimum time interval between two adjacent measuring gap timing is 20ms. Considering the limitation of PRS frequency hopping transmission timing, it is impossible to apply multiple measurement gaps to complete one PRS frequency hopping reception. In our view, UE can only apply a single instance of a measurement gap to complete one PRS frequency hopping reception. In the current measurement gap configuration, the maximum length of measurement gap can be configured to 5.5ms, which may meet one PRS frequency hopping reception.

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Proposal 4: UE can only apply a single instance of a measurement gap to complete one PRS frequency hopping reception.

SRS resource configuration for hopping

In NR Rel-16/17, the UE can be configured with one or more SRS resource set(s) for positioning. Each SRS resource set consists of one or more SRS resource(s). A maximum of 16 SRS resource sets can be configured on each BWP. The configuration framework of SRS for positioning is shown in Figure 3.

Figure 3: The configuration framework of SRS for positioning.

In RAN1#112, the following agreement on SRS hopping configuration had been achieved [2].

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

As can be seen from the above agreement, SRS frequency hopping transmission is no longer limited to the currently active BWP, that is to say, the resource allocation of SRS can be outside the currently active BWP.

SRS hopping within one resource

In R15, SRS support frequency-hopping transmission within one SRS resource. The following two frequency-hopping transmission schemes can be implemented using the existing SRS parameter configuration.

- Scheme 1: When the repetition factor (R) is less than the number of SRS symbols (nrofSymbols), SRS frequency hopping transmission between different symbols can be realized, as shown in the Figure 3.1-1.

Figure 3.1-1: SRS frequency hopping transmission between different symbols within one instance of SRS resource

- Scheme 2: When the SRS repetition factor is equal to the number of SRS symbols, frequency hopping transmission between different transmission instance of the same SRS resource can be realized, as shown in the Figure 3.1-2.

Figure 3.1-2: Frequency hopping transmission between different transmission instance of the same SRS resource

For Scheme 1, there is no time interval between two adjacent hops, which cannot meet the requirements of redcap positioning for SRS frequency hopping transmission.

For Scheme 1, in order to meet the requirements of redcap positioning for SRS transmission timing, the network needs to be configured with dense periodic SRS resources, which can cause serious resource overhead.

Therefore, the SRS frequency hopping transmission within one SRS resource in existing specification cannot meet the requirements of Redcap positioning.

Observation 1: The SRS frequency hopping transmission within one SRS resource in existing specification cannot meet the requirements of Redcap positioning.

Another way to implement SRS frequency hopping within one SRS resource for redcap positioning is to enhance the existing SRS resource configuration. A SRS resource in the frequency domain is divided into multiple frequency domain resource segments, and each frequency domain resource segment corresponds to a SRS frequency hopping frequency domain resource. In the time domain, each SRS frequency hopping occupies multiple OFDM symbols. Figure 3.1 shows an example of SRS frequency hopping transmission in one PRS resource. The network needs to configure the frequency domain location of each SRS frequency hop, or UE determine the frequency domain location of each SRS frequency hop according to the SRS frequency domain resource determination in the existing specification. In addition, the network also needs to configure the time domain location of each SRS frequency hopping in one SRS resource, such as configuring the time interval between two adjacent SRS frequency hopping.

Compared with the legacy SRS resource, the number of symbols occupied by a SRS resource in the time domain is significantly increased for frequency hopping transmission in one SRS resource, and the increased number is related to the number of SRS frequency hopping.

In our view, the specification impact of SRS frequency hopping transmission within an SRS resource is significant.

Observation 2: The specification impact of SRS frequency hopping transmission within an SRS resource is significant.

SRS hopping between resources

In the SRS resource configuration framework of R16/R17, the configuration of the time/frequency domain location of SRS resources is per SRS resource. Therefore, for SRS frequency-hopping transmission between SRS resources, a PRS frequency-hopping pattern contains multiple SRS resources, and the time/frequency domain positions corresponding to different SRS resources are different. Figure 3.2-1 shows an example of SRS frequency hopping transmission between SRS resources.

Figure 3.2-1: SRS hopping between resources

For periodic SRS resource configuration, the network realizes multiple SRS resources to form SRS frequency hopping transmission through time domain parameters (e.g., start offset, period) and frequency domain location configurations of different SRS resources, as shown in Figure 3.2-2.

Figure 3.2-2: Multiple SRS resources to form SRS frequency hopping transmission

For aperiodic SRS transmissions, the network can trigger an SRS resource set through DCI. An SRS resource set can contain multiple SRS resources. In this case, the network implements one or more SRS frequency hopping transmission patterns by configuring the time-frequency resource parameters of each SRS resource in the resource set, as shown in Figure 3.2-3.

Figure 3.2-3: Network trigger an SRS resource set through DCI.

In addition, for frequency hopping transmission between SRS resources, the network needs to indicate which SRS resources can form frequency hopping transmission, or the specification define relevant rules, and the UE determines which SRS resources can form frequency hopping transmission based on the rules. In our view, SRS hopping between resources brings relatively small specification impact.

Observation 3: SRS hopping between resources brings relatively small specification impact.

Proposal 5: SRS hopping between SRS resources should be supported.

The time for RF retuning

In RAN1#112, the following agreement on switching time to allow RF retuning between adjacent hops had been achieved [2].

Agreement For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same.

For UL SRS Tx frequency hopping, there are two scheme can be considered to reserve the switching time between hops.

- Scheme 1: Define scheduling restriction rules for SRS frequency hopping transmission.

- Scheme 2: Support an UL time window where the UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning.

In scheme 1, for SRS hopping within one resource, there are two scheduling restriction rules can be considered.

- Restriction rules 1: No receive/send operations are performed on the first X time units (e.g. X symbols) of each SRS resource start position, and no receive/send operations are performed on the last X time units (e.g. X symbols) of each SRS resource end position, as shown in Figure 3.3-1.

Figure 3.3-1: Scheduling Restriction rules 1 for SRS hopping within one resource

- Restriction rules 2: No receive/send operations are performed on X time units (e.g. X symbols) before the start position of each hop in each SRS resource, and no receive/send operations are performed on X time units (e.g. X symbols) after the end position of each hop in each SRS resource, as shown in Figure 3.3-2.

Figure 3.3-2: Scheduling Restriction rules 2 for SRS hopping within one resource

On Restriction rules 1, during SRS hoping transmission, UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning. On Restriction rules 2, during SRS hoping transmission, UE may expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning. In order to meet the requirements of redcap positioning for SRS transmission timing, the time gap between two SRS hop is very small (e.g., <1ms). It's impossible for UE to receive/transmit other signals/channels during SRS hoping transmission. Therefore, scheduling restriction rules 1 can be considered, if SRS hopping within one resource is supported.

For SRS hopping between resources, there are also two scheduling restriction rules can be considered.

- Restriction rules 1: No receive/send operations are performed on the first X time units (such as X symbols) before the first SRS resource in the time domain in each SRS hopping pattern, and no receive/send operations are performed on the X time units (such as X symbols) after the end position of the last SRS resource in the time domain in each SRS hopping pattern, as shown in Figure 3.3-3.

Figure 3.3-3: Scheduling Restriction rules 1 for SRS hopping between resources

- Restriction rules 2: No receive/send operations are performed on the first X time units (e.g. X symbols) of each SRS resource start position, and no receive/send operations are performed on the last X time units (e.g. X symbols) of each SRS resource end position, as shown in Figure 3.3-4.

Figure 3.3-4: Scheduling Restriction rules 2 for SRS hopping between resources

On Restriction rules 1, during SRS hoping transmission, UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning. On Restriction rules 2, during SRS hoping transmission, UE may expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning. In order to meet the requirements of redcap positioning for SRS transmission timing, the time gap between two SRS hop is very small (e.g., <1ms). It's impossible for UE to receive/transmit other signals/channels during SRS hoping transmission. Therefore, scheduling restriction rules 1 can be considered, if SRS hopping between resources is supported.

For scheme 2, similar to DL PRS measurement, the network configures periodic SRS transmission windows to the UE, and the UE can complete a SRS frequency hopping transmission using one instance of SRS transmission window, as shown in the Figure 3.3-5. During the SRS transmission window, UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning.

Figure 3.3-5: SRS transmission window

In our view, introducing SRS window will bring significant standardization work and also cause serious data interruption. We prefer define scheduling restriction rules for SRS frequency hopping transmission.

Proposal 6: Define scheduling restriction rules for SRS frequency hopping transmission should be supported.

Frequency overlapping between hops and number of hops

In the SI phase, some companies conducted simulation evaluation for the following two situations.

- Frequency hopping without phase compensation

- Overlapped FH with phase compensation

The simulation results show that the receiver needs to perform phase measurement and compensation to achieve high-precision positioning through SRS/PRS hopping. In our view, overlapping RB is required between two adjacent SRS/PRS hopping in the frequency domain to facilitate phase measurement and compensation at the receiver. The number of overlapping frequency resources in adjacent hops  required for the UE to perform phase offset compensation the overlap between hop for DL PRS Rx frequency hopping is a UE capability. For positioning for RedCap UEs, the number of hops for DL PRS Rx frequency hopping is also a UE capability.

Proposal 7: The number of overlapping frequency resources in adjacent hops  required for the UE to perform phase offset compensation the overlap between hop for DL PRS Rx frequency hopping is a UE capability.

Proposal 8: For positioning for RedCap UEs, the number of hops for DL PRS Rx frequency hopping is a UE capability.

Conclusion

In this contribution, we provided views on Redcap positioning. In summary, we have following observations and proposals:

Observation 1: The SRS frequency hopping transmission within one SRS resource in existing specification cannot meet the requirements of Redcap positioning.

Observation 2: The specification impact of SRS frequency hopping transmission within an SRS resource is significant.

Observation 3: SRS hopping between resources brings relatively small specification impact.

Proposal 1: UE perform frequency hopping reception between different PRS transmission repetitions within one period of one PRS resource can be considered.

Proposal 2: UE performs frequency hopping reception of PRS between PRS resources can be considered.

Proposal 3: Only measurement gap based measurement for DL PRS frequency hopping should be considered.

Proposal 4: UE can only apply a single instance of a measurement gap to complete one PRS frequency hopping reception.

Proposal 5: SRS hopping between SRS resources should be supported.

Proposal 6: Define scheduling restriction rules for SRS frequency hopping transmission should be supported.

Proposal 7: The number of overlapping frequency resources in adjacent hops  required for the UE to perform phase offset compensation the overlap between hop for DL PRS Rx frequency hopping is a UE capability.

Proposal 8: For positioning for RedCap UEs, the number of hops for DL PRS Rx frequency hopping is a UE capability.

Reference

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation.

- 3GPP RAN1#112 Chairman’s Notes, February 27th – March 3rd, 2023.