3GPP TSG RAN WG1 #112bis-e                                                         R1-2302612

e-meeting, April 17th – April 26th, 2023

Agenda Item:	9.6.1

Source:	Spreadtrum Communications

Title:	Discussion on enhanced support of RedCap devices

Document for:	Discussion and decision

Introduction

To further expand the market for RedCap use cases with relatively low cost, low energy consumption and low data rate requirements, e.g., industrial wireless sensor network use cases, some further enhancements should be considered in Rel-18.

According to the latest Rel-18 RedCap WID [1], at least the following objectives should be specified.

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

For UE peak data rate reduction, it was agreed in RAN#99e that standalone PR1 is also supported [2][3].

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

In this contribution, we share our views for the remaining issues based on the progress of RAN1#112[4] and RAN#99e [2][3].

Complexity/cost reduction solutions

UE BB bandwidth reduction

Initial separate BWP

In the last RAN1 meeting, it was concluded that there is no consensus to continue discussion on separate initial separate BWP [4].

Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap Ues is allowed to be configured by the SIB in the cell”.

Based on the conclusion, RAN1 should not further discuss it, unless RAN2 asks RAN1’s input.

Random access timeline

RAN1 has made the following agreement regarding the RAR bandwidth and Msg3 timeline in the last meeting [4].

Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs

The X is used to ensure the RAR PDSCH processing time and Msg3 preparation time for Rel-18 RedCap. A larger number of PDSCH PRBs (>5MHz) corresponding to larger PDSCH processing time (i.e., N1). In the worst case (20MHz RAR PDSCH), X may be equivalent to 3*N1, i.e. 3/1.5 ms for 15/30kHz SCS.

However, as indicated by several companies that N1 also includes processing time of DMRS, the value of X for PDSCH processing is not lineally increased along with PRB number. Then, the X should be smaller than 3/1.5 ms. On the other hand, the extremely small value of X should be avoided, as such kind of values will lead to higher requirements on Rel-18 eRedCap processing capabilities.

Based on the above, we prefer “2/1 ms for 15/30kHz SCS”.

Proposal 1: For the “FFS: value(s) of X”, 2/1 ms for 15/30kHz SCS is selected.

Separate early indication

Separate early indication(s) in Msg1 only, or in Msg3 only, or in both Msg1 and Msg3 were discussed in last RAN1 meeting. After several rounds heated discussion, RAN1 still could not achieve any conclusion.

There seems to be no disagreement on separate early indication in Msg3 in RAN1, and RAN2 already agreed to introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap [5].

Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not). We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap.

The controversial part in RAN1 is Msg1, and based on above, it seems RAN2 has no idea for this part.

The proponent companies in RAN1 think that Msg1 indication provides flexibility for the NW to allocate different BW for RAR PDSCH. Without separate Msg1 indication, the gNB may have to always delay RAR or limit RAR PDSCH bandwidth to ensure eRedCap UEs can access. The opposite companies think that the RACH resource is already fragmented by several features, further fragmentation should be avoided.

During the online discussion, many companies further clarified that the motivation to differentiate Rel-17 and Rel-18 RedCap UE is the timeline relaxation for MSG3. If the X value is very small, the difference between Rel-17 and Rel-18 RedCap UE during random access is negligible, and then Msg1 indication is not needed. As we discussed in 2.1.2, larger X values are required, then Msg1 based early indication is considerable. In addition, the Msg1 early indication is anyway configurable at NW side, RACH fragmentation issue is not always exist.

Proposal 2: Msg1 based early indication for Rel-18 RedCap is supported.

Simultaneous reception

For two broadcast transmissions, it was agreed that there is no need to relax the requirements.

Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

For simultaneous reception of one unicast PDSCH and one broadcast PDSCH, no consensus was reached, and the lasted FL proposal is as follows:

High Priority Proposal 2.5-2c: For UE BB complexity reduction, it is FFS whether some specification changes (e.g., regarding prioritization for processing and decoding) are needed for simultaneous reception of unicast and broadcast PDSCH transmissions.

In our understanding, the UE only has the capability to decode up to 25 or 12 PRBs per slot, if the total PRB numbers of two PDSCHs are more than 25 or 12, the UE cannot decode both PDSCHs within a slot. However, the post-FFS buffer is assumed 20MHz, both PDSCHs can be buffered and the decoding of broadcast PDSCH can be generally delayed, as the broadcast PDSCH does not require feedback. In this regards, if simultaneous reception of unicast and broadcast PDSCH is allowed for Rel-18 RedCap, the processing priority can be up to UE implementations according to the feedback requirements, specification changes are not needed.

If simultaneous reception of unicast and broadcast PDSCH is not allowed, we think the current spec for FR2 can be reused.

“On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.”

Proposal 3: If simultaneous reception of unicast and broadcast PDSCH is allowed for Rel-18 RedCap, it is up to UE to decide the processing periodicity, specification changes are not needed.

Proposal 4: If simultaneous reception of unicast and broadcast PDSCH is not allowed, the current spec for FR2 can be a reference.

Msg4 PDSCH bandwidth

For Msg4 PDSCH, it is a common understanding that the span of Msg4 PDSCH can be larger than ~5 MHz per slot. But whether a larger number of PRBs for Msg4 and UE behavior is controversial. In this regard, only a working assumption was reached.

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

For the sub-bullet, some companies suppose that the wording should be “not expected” rather than “not required”. In our understanding, the sub-bullet is more about “processing”.

- “not required to process” means the case (i.e., Msg4 PDSCH with a larger number of PRBs) is anyway allowed. For example, even the Msg4 DCI is intended for a Rel-18 RedCap UE (BB BW reduction), the Msg4 PDSCH can still be larger than 25/12PRBs. The example case may not be happened at gNB side, but from spec perspective, this case is not precluded.

- “not expected to process” means that a Msg4 PDSCH with a larger number of PRBs and intended for the BB BW reduction UE is not expected. In our understanding, this is reasonable and does not mean any restriction on gNB, since if the scheduled Msg4 is not intended for the BB BW reduction UE, larger BW for Msg4 is still allowed. When the Rel-18 RedCap UE receives a larger Msg4 PDSCH, the UE can know that the PDSCH is not for it. In this case, the UE (BB BW reduction) behavior is to drop the PDSCH and restart another RACH procedure.

Based on the above, we prefer to modify the working assumption as follows:

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required expected to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 5: Confirm the working assumption with following modification.

Working assumption:

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required expected to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

MsgB PDSCH bandwidth

For MsgA PUSCH bandwidth, the following agreements were reached. The agreement means, if MsgA PUSCH resource is shared between Rel-18 RedCap and non-Rel-18 RedCap, at least some of the MsgA PUSCH resources are limited within 5MHz while other MsgA PUSCH resources may be more than 5MHz.

Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

The limitation on MsgB PDSCH bandwidth was also discussed, the latest FL proposal is as follows.

Medium Priority Proposal 2.9-1b: The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

We understand that the gNB should already know of RedCap UE type from MsgA. Therefore, the same principle as Msg4 should be applied for MsgB transmission. We support the latest FL proposal.

Proposal 6：The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Further UE peak rate reduction

Constraint for add-on PR1

For add-on PR1, the following agreement was made in the last RAN1 meeting online session:

Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

For BW3/PR3 +PR1, the reference RedCap is 5MHz (12 PRBs) + 1Rx+1MIMO layer+64QAM SCS+FD_FDD. For comparison purposes, the relationship between the supported peak data rate, the reported capabilities and the constraint can be found in table 1 and 2 (take 30KHz as an example).

Table 1: The supported peak data rate and the corresponding capability combination (30KHz SCS)

| Data rare(DL/UL) | SF=1 | SF=0.8 | SF=0.75 | SF=0.4 |
| --- | --- | --- | --- | --- |
| 64QAM =6 | 19.3Mbps/20.6Mbps (5MHz BB BW) | 15.4Mbps/16.5Mbps | 14.5Mbps/15.5Mbps | 7.7Mbps/8.2Mbps |
| 16QAM = 4 | 12.8Mbps/13.7Mbps | 10.3Mbps/10.9Mbps | 9.7Mbps/10.3Mbps | 5.1Mbps/5.5Mbps |
| QPSK = 2 | 6.4Mbps/6.9Mbps | 5.1Mbps/5.5Mbps | 4.8Mbps/5.2Mbps | 2.6Mbps/2.7Mbps |
| BPSK = 1 | 3.2Mbps/3.4Mbps | 2.6Mbps/2.7Mbps | 2.4Mbps/2.6Mbps | 1.3Mbps/1.4Mbps |

Table 2: The corresponding constraint (30KHz SCS)

| Q×F | SF=1 | SF=0.8 | SF=0.75 | SF=0.4 |
| --- | --- | --- | --- | --- |
| 64QAM =6 | 6 | 4.8 | 4.5 | 2.4 |
| 16QAM = 4 | 4 | 3.2 | 3 | 1.6 |
| QPSK = 2 | 2 | 1.6 | 1.5 | 0.8 |
| BPSK = 1 | 1 | 0.8 | 0.75 | 0.4 |

As the calculated peak data rate for X = 3 and X = 3.2 are quite similar. We may slightly prefer 3.2.

Proposal 7：For add-on PR1, the constraint is relaxed to 3.2.

Constraint for standalone PR1

For 20MHz +PR1, the reference RedCap is 20MHz (51 PRBs)+1Rx+1MIMO layer+64QAM+FD_FDD. For comparison purposes, the relationship between the supported peak data rate, the reported capabilities and the constraint can be found in table 3 and 4 (take 30KHz as an example).

Table 3: The supported peak data rate and the corresponding capability combination (30KHz SCS)

| Data rare(DL/UL) | SF=1 | SF=0.8 | SF=0.75 | SF=0.4 |
| --- | --- | --- | --- | --- |
| 64QAM =6 | 81.9Mbps/87.6Mbps (20MHz BW) | 65.6Mbps/70.1Mbps | 61.5Mbps/65.7Mbps | 32.8Mbps/35.04Mbps |
| 16QAM = 4 | 54.6Mbps/58.4Mbps | 43.7Mbps/46.7Mbps | 41Mbps/43.8Mbps | 21.9Mbps/23.36Mbps |
| QPSK = 2 | 27.3Mbps/29.2Mbps | 21.9Mbps/23.36Mbps | 20.5Mbps/21.9Mbps | 10.9Mbps/11.68Mbps |
| BPSK = 1 | 13.65Mbps/14.6Mbps | 10.9Mbps/11.68Mbps | 10.25Mbps/10.95Mbps | 5.46Mbps/5.84Mbps |

Table 4: The corresponding constraint

| Q×F | SF=1 | SF=0.8 | SF=0.75 | SF=0.4 |
| --- | --- | --- | --- | --- |
| 64QAM =6 | 6 | 4.8 | 4.5 | 2.4 |
| 16QAM = 4 | 4 (the current one) | 3.2 | 3 | 1.6 |
| QPSK = 2 | 2 | 1.6 | 1.5 | 0.8 |
| BPSK = 1 | 1 | 0.8 | 0.75 | 0.4 |

Based on the above, the constraint is relaxed to 0.75 for standalone PR1.

Proposal 8：For standalone PR1, the constraint is relaxed to 0.75.

Clarification on constraint relaxation

For non-Rel-18 RedCap UE, the current constraint is a minimum value (ie., vLayers·Qm·f ≥ 4). This means any combination of scaling factor and modulation order is allowed, as long as the product is greater than 4. Based on this, Rel-17 RedCap can actually achieve different peak data rates.

Table 5: Possible peak data rate for Rel-17 RedCap

| Reported modulation order and scaling factor | Product of vLayers·Qm·f | Peak data rate achieved by R17 RedCap (DL/UL) |
| --- | --- | --- |
| 64QAM, SF=1 | 6 | 81.9Mbps/87.6Mbps |
| 64QAM, SF=0.8 | 4.8 | 65.6Mbps/70.1Mbps |
| 64QAM, SF=0.75 | 4.5 | 61.5Mbps/65.7Mbps |
| 16QAM, SF=1 | 4 | 54.6Mbps/58.4Mbps |

For Rel-18 RedCap, two relaxed constraint values for add-on PR1 and standalone PR1 will be introduced. For these two constraints, there are two interpretations:

  - The relaxed constraint value is the minimum value of the product of vLayers·Qm·f.  This is similar to the interpretation as the current constraint. Based on this interpretation, different and higher peak data rates (>10Mbps) by reporting different scaling factors and modulation orders are allowed.

  - The relaxed constraint value is the only value of the product of vLayers·Qm·f.  This means only one peak data rate (~10Mbps) is allowed, and the reported scaling factor and modulation order are fixed.

As the interpretation are related to the UE implementation and value reporting, therefore, we think it is necessary to clarify it in RAN1.

Proposal 9: Clarify the meaning of the relaxed constraint in RAN1.

Proposal 10: For the clarification on the relaxed constraint, one of the following can be selected:

- The relaxed constraint value is the minimum value of the product of vLayers·Qm·f.

- The relaxed constraint value is the only value of the product of vLayers·Qm·f.

Conclusion

Based on the analyses, we have the following proposals:

Proposal 1: For the “FFS: value(s) of X”, 2/1 ms for 15/30kHz SCS is selected.

Proposal 2: Msg1 based early indication for Rel-18 RedCap is supported.

Proposal 3: If simultaneous reception of unicast and broadcast PDSCH is allowed for Rel-18 RedCap, it is up to UE to decide the processing periodicity, specification changes are not needed.

Proposal 4: If simultaneous reception of unicast and broadcast PDSCH is not allowed, the current spec for FR2 can be a reference.

Proposal 5: Confirm the working assumption with following modification.

Working assumption:

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required expected to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 6：The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Proposal 7：For add-on PR1, the constraint is relaxed to 3.2.

Proposal 8：For standalone PR1, the constraint is relaxed to 0.75.

Proposal 9: Clarify the meaning of the relaxed constraint in RAN1.

Proposal 10: For the clarification on the relaxed constraint, one of the following can be selected:

- The relaxed constraint value is the minimum value of the product of vLayers·Qm·f.

- The relaxed constraint value is the only value of the product of vLayers·Qm·f.

Reference

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices. RAN#98e, December 12-16, 2022.

- RAN#99 meeting report, Rotterdam, Netherlands, March 20-23, 2023

- RP-230778, Proposal for PR1 in eRedCap. RAN#99, Rotterdam, Netherlands, March 20-23, 2023.

- Chair's notes RAN1#112, Athens, Greece, 27th February – 3rd March 2023.

- Chair's notes RAN2#121, Athens, Greece, 27th February – 3rd March 2023.