3GPP TSG RAN WG1 #112bis-e                                                   R1-2302613

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.1

Source:	Spreadtrum Communications

Title: 	Discussion on NES techniques in spatial and power domains

Document for:	Discussion and decision

# Introduction

It was agreed in [1] to specify the NES techniques in spatial and power domain.

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

The contribution focuses on the NES techniques in spatial and power domain.

In the contribution, the design principle is to minimize of spec impact and achieve better back compatibility. It is better that the legacy UEs can well coexist with R18 NES UEs. The early deployment is strived to enable the current network applying the spatial domain NES techniques as soon as possible.

# Spatial domain

## Logical antenna ports vs. antenna elements

The spatial domain NES usually means adaptation of spatial elements, e.g.

- Logical antenna ports (each corresponds to a transceiver chain/branch), or

- Antenna elements.

In RAN1#112 [2], for ease of discussion, two types of mapping between multiple antenna elements and a logical antenna port are agreed.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

In current spec, mapping between multiple antenna elements and a logical antenna port does not exist. In our understanding, Type-1 means flexibility across multiple logical antenna ports, and Type-2 means flexibility within a logical antenna port. In general, a logical antenna port means a CSI-RS port used for CSI acquisition instead of a DMRS port. Usually, a DMRS port (MIMO layer) is generated by a CSI-RS port or precoding of multiple CSI-RS ports used for CSI acquisition.

Observation 1: A logical antenna port in the Type 1/2 definition means a CSI-RS port used for CSI acquisition.

On the other hand, beam is not mentioned here. In our view, beam usually means analog beamforming and have effect on large scale properties, i.e. QCL parameters. gNB can configure SSB or beamformed CSI-RS for beam management. This topic is more related to CSI-RS for CSI acquisition. If CSI-RS for beam management is involved, the scope is even wider.

Observation 2: A logical antenna port in the Type 1/2 definition does not mean a CSI-RS port used for beam management.

In this paper, we will assume the analog beam is not impacted, but the digital “beam” (indeed eigen vectors of channel matrix) is impacted due to PMI/RI/CQI adaptation. By the way, we propose to clarify that the adaptation of analog beam is discussed separately, and the corresponding spec impact could be beam reporting.

Proposal 1: The adaptation of PMI/RI/CQI calculation and reporting is prioritized, and the adaptation of CRI/RSRP/SINR is down-prioritized or discussed separately.

For Type 2, when a part of/subset of antenna elements associated to a logical antenna port is disabled/enabled, the analog beam shape for the logical antenna port corresponding to CSI-RS for CSI acquisition is changed. In other words, QCL properties of CSI-RS for CSI acquisition are changed. In this case, CRI/RSRP/SINR may need to be updated. CRI/RSRP/SINR is down-prioritized in our view. Therefore, we propose to down-prioritize Type 2 also.

Proposal 2: Type 2 is down-prioritized.

## Multiple CSIs

Adaptation of spatial elements will be defined as multiple CSIs in the spec. Multiple CSIs may mean multiple CSI-RS resource configuration and multiple CSI report configuration. Definitions of CSI-RS resource configuration and CSI report configuration is shown in Appendix A.1 and Appendix A.2 respectively.

In RAN1#112 [2], for CSI-RS resource configuration, there were two alternatives agreed.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns”

In RAN1#112 [2], for CSI report configuration, there were two alternatives agreed.

Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration

Definition of spatial adaptation pattern is not given. In our view, for Type 1, spatial adaptation pattern means the number of ports in a CSI-RS resource; for Type 2, spatial adaptation pattern means the number of antenna elements in a CSI-RS port.

Observation 3: For Type 1, spatial adaptation pattern means the number of ports in a CSI-RS resource; for Type 2, spatial adaptation pattern means the number of antenna elements in a CSI-RS port.

## Dynamic switching vs. non-dynamic switching

In our view, dynamic switching and non-dynamic switching are two different design principle no matter for CSI-RS resource or CSI report.

    - CSI-RS resource configuration

      - A1-1)

        - Dynamic switching: Only one CSI-RS resource is valid in a given time interval.

        - Non-dynamic switching: Multiple CSI-RS resources are valid once configured.

      - A1-2)

        - Dynamic switching: Only one spatial adaptation pattern is valid in a given time interval.

        - Non-dynamic switching: Multiple spatial adaptation patterns are valid once configured.

    - CSI report configuration

      - A2-1)

        - Dynamic switching: Only one CSI report configuration is valid in a given time interval.

        - Non-dynamic switching: Multiple CSI report configurations are valid once configured.

      - A2-2)

        - Dynamic switching: Only one spatial adaptation pattern (i.e. CSI report sub-configuration) is valid in a given time interval.

        - Non-Dynamic switching: Multiple spatial adaptation patterns (i.e. multiple CSI report sub-configurations) are valid once configured.

We compare dynamic switching and non-dynamic switching in the following table.

Table 1: Comparison among dynamic switching and non-dynamic switching

|  | Dynamic switching | Non-dynamic switching |
| --- | --- | --- |
| UE calculation complexity | Lower: Only one CSI-RS resource or CSI report configuration is valid in a given time interval | Higher: Multiple CSI-RS resources or CSI report configurations |
| Spec impact | UE capability of the number of CSI-RS resources or CSI report configuration is interpreted as for the number CSI-RS resources or CSI report configurations for a given time interval | Revise UE capability |

We prefer dynamic switching because UE calculation complexity can be lower.

Proposal 3: Dynamic switching between CSI-RS resources or spatial adaptation patterns for a CSI-RS resource can be considered.

Proposal 4: Dynamic switching between CSI report configurations or spatial adaptation patterns for a CSI report configuration can be considered.

## Whether to define spatial adaptation pattern

In our view, the main different between A1-1) and A1-2) is whether to define spatial adaptation pattern, as well as the main different between A2-1) and A2-2). For A1-1) and A2-1) there is no need to define spatial adaptation pattern.

If spatial adaptation pattern is not defined, for current CSI report framework, there are three methods to implement multiple CSIs.

    - Method-1: From bottom to top. Multiple CSI-RS port numbers (i.e. nrofPorts in CSI-RS-ResourceMapping) can be adapted.

    - Method-2: From top to bottom. Multiple CSI report configurations can be adapted.

    - Method-3: In the middle. Multiple CSI-RS resource can be adapted.

For Method-1, a CSI-RS port number is defined in CSI-RS-ResourceMapping (see Appendix A.3), which is in about lowest level of CSI report configuration. CSI-RS port number can be regarded as a legacy form of spatial adaptation pattern.

For Method-2, a CSI report configuration is defined in CSI-ReportConfig (see Appendix A.2), and corresponding CSI-RS resource are defined embedded.

For Method-3, a CSI-RS resource is defined in CSI-ResourceConfig (see Appendix A.1).

For Method-1, Method-2 and Method-3, multiple CSIs can be supported. For minimize spec impact, we do not support to define spatial adaptation pattern.

However, it may be argued that for Method-2 and Method-3, the legacy maximum number of CSI report configurations and CSI-RS resources may need to be updated.

The maximum number of CSI report configurations is limited in current spec.

CSI-ReportFramework ::= SEQUENCE { maxNumberPeriodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4), maxNumberPeriodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-triggeringStatePerCC ENUMERATED {n3, n7, n15, n31, n63, n128}, maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4), simultaneousCSI-ReportsPerCC INTEGER (1..8) } CSI-ReportFrameworkExt-r16 ::= SEQUENCE { maxNumberAperiodicCSI-PerBWP-ForCSI-ReportExt-r16 INTEGER (5..8) }

The maximum number of CSI-RS resources is limited in current spec.

SupportedCSI-RS-Resource ::= SEQUENCE { maxNumberTxPortsPerResource ENUMERATED {p2, p4, p8, p12, p16, p24, p32}, maxNumberResourcesPerBand INTEGER (1..64), totalNumberTxPortsPerBand INTEGER (2..256) }

Nevertheless, in our view, if dynamic switching is supported, increasing the maximum number of CSI report configurations and CSI-RS resources does not mean higher UE calculation complexity. Hence, we are open for increasing the maximum number of CSI report configurations and CSI-RS resources.

Proposal 5: Spatial adaptation pattern is not defined.

If spatial adaptation pattern is not defined, comparing A1-1) and A1-2), A1-2) is not necessary at all. Comparing A2-1) and A2-2), we are not sure whether A2-2) can be supported without defining spatial adaptation pattern.

Proposal 6: A1-2) is not supported.

## Multiple CSIs compression

In RAN1#112 [2], multiple CSIs compression schemes were proposed as FFS.

Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account.

If we only consider the digital beam which is represented by PMI/RI/CQI, different spatial adaptation patterns may mean different digital beam set. We are not sure the correlation between different digital beam sets, i.e. different spatial adaptation patterns, can be utilized.

Observation 4: Whether the correlation between different digital beam sets, i.e. different spatial adaptation patterns, can be utilized needs carefully study.

If the correlation between different digital beam sets can be utilized, we prefer use it to reduce UE complexity by exploiting the correlation at first. Anyway, Multiple CSIs without compression can be supported as baseline, and multiple CSIs with compression needs to consider UE complexity reduction.

Proposal 7: Multiple CSIs without compression can be supported as baseline, and multiple CSIs with compression needs to consider UE complexity reduction.

# Power domain

In RAN1#112 [2], it was agreed to study where/how to configure multiple power offset values between PDSCH and CSI-RS, and whether/how to indicated to UE.

Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

In our view, it is general that a power offset value can be RRC configured in NZP-CSI-RS-Resource. Multiple power offsets can be configured and dynamic switching among multiple power offsets can be considered.

Proposal 8: Dynamic switching among multiple power offsets between PDSCH and CSI-RS can be considered.

# Conclusion

We have the following observations.

Spatial domain:

Observation 1: A logical antenna port in the Type 1/2 definition means a CSI-RS port used for CSI acquisition.

Observation 2: A logical antenna port in the Type 1/2 definition does not mean a CSI-RS port used for beam management.

Observation 3: For Type 1, spatial adaptation pattern means the number of ports in a CSI-RS resource; for Type 2, spatial adaptation pattern means the number of antenna elements in a CSI-RS port.

Observation 4: Whether the correlation between different digital beam sets, i.e. different spatial adaptation patterns, can be utilized needs carefully study.

We have the following proposals.

Spatial domain:

Proposal 1: The adaptation of PMI/RI/CQI calculation and reporting is prioritized, and the adaptation of CRI/RSRP/SINR is down-prioritized or discussed separately.

Proposal 2: Type 2 is down-prioritized.

Proposal 3: Dynamic switching between CSI-RS resources or spatial adaptation patterns for a CSI-RS resource can be considered.

Proposal 4: Dynamic switching between CSI report configurations or spatial adaptation patterns for a CSI report configuration can be considered.

Proposal 5: Spatial adaptation pattern is not defined.

Proposal 6: A1-2) is not supported.

Proposal 7: Multiple CSIs without compression can be supported as baseline, and multiple CSIs with compression needs to consider UE complexity reduction.

Power domain:

Proposal 8: Dynamic switching among multiple power offsets between PDSCH and CSI-RS can be considered.

# Reference

- RP-223540, “New WID: Network energy savings for NR”, Huawei, RAN#98e, Dec.  12th - 16th, 2022.

- 3GPP RAN1#112 Chairman’s Notes, Feb. 27th – Mar. 3rd, 2022.

# Appendix

## A.1  CSI-RS resource configuration

CSI-ResourceConfig ::= SEQUENCE { csi-ResourceConfigId CSI-ResourceConfigId, csi-RS-ResourceSetList CHOICE { nzp-CSI-RS-SSB SEQUENCE { nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId OPTIONAL -- Need R }, csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId }, bwp-Id BWP-Id, resourceType ENUMERATED { aperiodic, semiPersistent, periodic }, ..., [[ csi-SSB-ResourceSetListExt-r17 CSI-SSB-ResourceSetId OPTIONAL -- Need R ]] }

## A.2  CSI report configuration

CSI report configuration is defined as follows 38.331-h40.

CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, carrier ServCellIndex OPTIONAL, -- Need S resourcesForChannelMeasurement CSI-ResourceConfigId, csi-IM-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R nzp-CSI-RS-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R reportConfigType CHOICE { periodic SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH SEQUENCE { reportSlotConfig ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320}, reportSlotOffsetList SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32), p0alpha P0-PUSCH-AlphaSetId }, aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) } }, reportQuantity CHOICE { none NULL, cri-RI-PMI-CQI NULL, cri-RI-i1 NULL, cri-RI-i1-CQI SEQUENCE { pdsch-BundleSizeForCSI ENUMERATED {n2, n4} OPTIONAL -- Need S }, cri-RI-CQI NULL, cri-RSRP NULL, ssb-Index-RSRP NULL, cri-RI-LI-PMI-CQI NULL },

## A.3  CSI-RS resource RE mapping

CSI-RS resource RE mapping is defined as follows 38.331-h40.

CSI-RS-ResourceMapping ::= SEQUENCE { frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32}, firstOFDMSymbolInTimeDomain INTEGER (0..13), firstOFDMSymbolInTimeDomain2 INTEGER (2..12) OPTIONAL, -- Need R cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4}, density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL }, freqBand CSI-FrequencyOccupation, ... }