3GPP TSG RAN WG1 #112bis-e        	                                               R1-2302616

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.9.1

Source:	Spreadtrum Communications

Title:	Discussion on coverage enhancement for NR NTN

Document for:	Discussion and decision

Introduction

In RAN#99, the revised WID on NR NTN (Non-Terrestrial Networks) enhancements is approved in [1].

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarization loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g., repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g., time-frequency pre-compensation) [RAN1]

In this contribution, we share our initial views on coverage enhancements to NR over NTN.

PUCCH for Msg4 HARQ-ACK

In RAN1#112, the following working assumption on PUCCH repetition for Msg4 HARQ-ACK had been achieved [2].

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g., DMRS ports

In our view, the above Working assumption should be confirmed

Proposal 1: Confirm the above Working assumption.

How to transmit information from UE

As mentioned above in working assumption, for Option A, in Rel-17, a separate PRACH resource is configured for Msg3 PUSCH repetition request, such as separate RO or preambles.

If configure a group of preambles for repetition request of PUCCH for Msg4 HARQ-ACK, the total preambles will be divided into four groups as shown in Figure 1. It is no doubt that leads to further fragmentation of PRACH resources.

Figure 1: Preambles for different use case

As shown in Figure 2, there is a joint indication mechanism of repetition request for these two channels. The preambles configured for Msg3 repetition could also request repetition of PUCCH for Msg4 HARQ-ACK. But there is a situation that when Msg3 doesn’t need repetition, PUCCH for Msg4 HARQ-ACK may still need repetition. Hence, we think it is necessary that PRACH resource partitioning for PUCCH repetition request/report.

Figure 2: Joint indication mechanism of Msg3 and PUCCH

Proposal 2: For PRACH (Preamble and/or occasion) based signaling for request or capability report, it is necessary for PRACH resource partitioning for PUCCH and how to avoid further fragmentation of PRACH resources needs to be considered.

For Option B, Msg3 carries the RRC signaling RRCSetupRequest, which has one spare bit that can be used for repetition request of PUCCH for Msg4 HARQ-ACK. Besides, as shown in table 2.1.1 which from TS 38.321 [3], there are some reserved LCID/eLCID can be indicated for repetition request or repetition capability.

Table 2.1.1: Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

For Option C, UE uses different scrambling and/or DMRS ports to send Msg3 to indicate the request of repetition and/or capability to the network. It would have big impact on gNB. gNB need to blind decoding by assuming different scrambling and/or DMRS ports for Msg3 receiving.

Proposal 3: UE may directly report the request of repetition and/or capability through higher layer signaling in Msg3, such as RRC signaling and reserved LCID/eLCID.

Determination of whether to request

In RAN1#112, the following working assumption on PUCCH repetition for Msg4 HARQ-ACK had been achieved [2].

Proposed Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report.

We could conclude that a RSRP threshold will be used to determine whether to repetition for PUCCH for Msg4 HARQ-ACK. It could be understood easily the situation that the threshold is configured, just like Msg3 repetition, if measured RSRP is lower than a RSRP threshold, UE capable of Msg3 repetition sends request to gNB.

As shown above in proposed working assumption, if the RSRP threshold is not configured, or if the configured RSRP threshold is X, UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK. It seems that this situation is not reasonable. Even UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK, it doesn’t mean the UE need repetition. If UE need repetition, does it send repetition request again? Otherwise, the gNB will determine whether repetition or not.

Since the RSRP threshold mechanism is introduced, we think it is enough to determine repetition or not depend on the relationship between UE measured RSRP and the RSRP threshold. In our view, the RSRP threshold should always be configured.

Proposal 4: The situation that “If the RSRP threshold is not configured, or if the configured RSRP threshold is X” is unreasonable, the RSRP threshold should always be configured by gNB and smaller than X.

Dynamic indication by gNB

RAN1#112 had achieved agreements as follows:

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design. - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

For Alt 1-1, if using the existing field in DCI scheduling the Msg4 PDSCH for dynamic indication, it would influence the original indication of that field to some extent and reduce the scheduling flexibility. In addition, considering that introducing a new field would increase the DCI size, so we don’t prefer Alt 1.

For Alt 3, we think changing CRC scrambling of DCI scheduling the Msg4 PDSCH has high impact on transmitter/receiver complexity.

For Alt 2 and Alt 4, it is reasonable assumption that the path loss experienced by UE for MSG3 transmission would not change significantly with MSG4 HARQ ACK transmission. As mentioned in section 2.1, both Msg3 and PUCCH may need repetition, it seems feasible that PUCCH repetition factor is indicated jointly with Msg3 repetition factor or implicit indication with mapping between MSG3 repetitions and PUCCH repetition for MSG4 HARQ ACK. Importantly, these two alternatives may have less impaction on exited specification.

Proposal 5: Alt 2 and Alt 4 should be considered as a solution of dynamic indication.

Considering that the coverage area of an NTN cell is larger than TN, the coverage conditions of different UEs in different cells are expected to be quite diverse. In NTN scenario, one cell can contain one or more beams. The channel condition of these beams might be different. For example, as shown in Figure 3, a represents the minimum elevation angle and b represents central elevation angle of the farthest beam. It is clearly that elevation angles of the farthest beam are very different from that of the central beam which means UEs in different beams may need to perform different repetition values.

Figure 3: Angles of different beams in one cell

To satisfy the repetition values of these UEs, in our view, a possible solution is that gNB configures different repetition values of PUCCH for Msg4 HARQ-ACK for different beams.

Proposal 6: Beam-level repetition value configuration of PUCCH for Msg4 HARQ-ACK can be supported.

DMRS bundling for PUSCH

During the RAN1#112, one proposal 2-3_v3 listed the question of NTN-specific PUSCH DMRS bundling in [4] as shown below.

Proposal 2-3_v3 For NTN-specific PUSCH DMRS bundling, further discuss whether nominal/actual TDW determination defined in R17 can be reused for NTN or not, e.g., the following options - Option 1: Actual TDW determination based on timing of TA pre-compensation update causing phase discontinuity and power inconsistency. UE determines the timing. - Nominal TDW is configured by gNB as in the existing spec. - UE reports information on the timing. FFS details - Actual TDW is determined by the reported information and events defined in R17. - New event is defined based on TA pre-compensation update. FFS details - Option 2: Actual TDW determination based on gNB configuration and/or indication - Option 2-1: Determined as in the existing spec. No spec change is assumed. - Nominal TDW is configured by gNB as in the existing spec. - Actual TDW is determined by events defined in R17. - Note: no new event is defined for the TA pre-compensation update. - Option 2-2: Determined by new gNB configuration/indication. - FFS details - e.g., TA pre-compensation timing is indicated by gNB - e.g., further segmented configuration is provided by gNB - New event is defined based on new gNB configuration/indication. FFS details FFS: whether to define another new event to decide actual TDW (e.g., antenna switching, update of epoch time, etc.)

We will share our opinion on TDW determination.

Without pre-compensation, timing error accumulate in long time transmission with large repetition number, which would cause the phase rotation. In other words, larger PUSCH repetition number may cause phase discontinuity.

As mentioned in section 1, the target scenario of uplink coverage enhancements for NTN is parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS). Because the channel condition is LOS, the distance between UE and satellite can represent the elevation angle. Long distance represents small elevation angle and shorter distance represents bigger elevation angle. When the distance between UE and satellite is long, e.g., small elevation angel, UE may need more repetitions. When elevation angel is 90 deg., the distance is shortest, so the repetition number is least.

Observation 1: UE of different angel need different repetition number.

Different elevation angles correspond to different changing rate of Doppler and timing drift. We think that different elevation angles could be associated with different TDW size. Considering that different elevation angel represent different repetition number, we can conclude that there might be a relationship between PUSCH repetition number and TDW size.

Proposal 7: TDW size should be associated with PUSCH repetition number through configuration by gNB.

Conclusion

As summary, we have the following observations and proposals.

Observation 1: UE of different angel need different repetition number.

Proposal 1: Confirm the above Working assumption.

Proposal 2: For PRACH (Preamble and/or occasion) based signaling for request or capability report, it is necessary for PRACH resource partitioning for PUCCH and how to avoid further fragmentation of PRACH resources needs to be considered.

Proposal 3: UE may directly report the request of repetition and/or capability through higher layer signaling in Msg3, such as RRC signaling and reserved LCID/eLCID.

Proposal 4: The situation that “If the RSRP threshold is not configured, or if the configured RSRP threshold is X” may need be reconsidered.

Proposal 5: Alt 2 and Alt 4 should be considered as a solution of dynamic indication.

Proposal 6: Beam-level repetition value configuration of PUCCH for Msg4 HARQ-ACK can be supported.

Proposal 7: TDW size should be associated with PUSCH repetition number through configuration by gNB.

Reference

- RP-230809, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, RAN Vice-Chair (AT&T), Thales.

- 3GPP RAN1#112 Chairman’s Notes, February 27th – March 3rd, 2023.

- TS 38.321, Medium Access Control (MAC) protocol specification (Release 17).

- R1-2301838, “Session notes for 9.11 NTN (Non-Terrestrial Networks) enhancements”, Ad-Hoc Chair, Huawei.