3GPP TSG RAN WG1 #112bis-e			R1-2302620

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.10.2

Source: 	Spreadtrum Communications

Title: 	Discussion on timing advance management to reduce latency

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new Work Item [1] was agreed to support timing advance management.  Some agreements were achieved during RAN1#112 meeting.

This contribution provides further considerations on timing advance management.

TA acquisition

PDCCH ordered RACH for TA acquisition

During the last meeting, PDCCH ordered RACH triggered by source cell was agreed to support.

Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement On whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

Configuration of RACH resource for candidate cell(s)

It was agreed that configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order. The remaining issue is how to configure RACH resource for candidate cell(s), the framework of which should be given by RAN1, and then designed by RAN2.

Firstly, configuration of RACH resource for candidate cell(s) can be provided per candidate cell if multiple candidate cells are applied. UE can get TA of the candidate cell via RACH procedure if the RACH is based on SSB from the candidate cell. To trigger RACH procedure for a candidate cell, RACH resource should be configured for the candidate cell. Since each configured candidate cell is likely to be provided by reference configuration and delta configuration, it is natural to configure RACH resource for each configured candidate cell. Sharing RACH resource among candidate cells may cause some issues like collision of preamble/RO.

Second, according to the issue of the configuration of RACH resource is configured under the serving cell or candidate cell, we prefer the RACH resource is configured under the candidate cell.

- Configuration of RACH resource for candidate cell(s) can be provided per candidate cell

Content of PDCCH order

For the indication of candidate cell in DCI, it was agreed that the PDCCH order from the source cell contains the indication of candidate cell using the reserved bit(s) for indication of cell identity.

The bits length of the indication of candidate cell in PDCCH order is fixed, can depend on the maximum number of candidate cells for a cell group. The bits length does not related with how many candidate cells are actually configured or how many candidate cells are configured with RACH resource. Same principle of other fields such as SSB index in the PDCCH order.

The position of the indication of candidate cell in PDCCH order is right after PRACH Mask index, which is the most straight forward way.

- The bits length of the indication of candidate cell in PDCCH order is fixed, depending on the maximum number of candidate cells for a cell group.

- The location of the indication of candidate cell field in PDCCH order is right after PRACH Mask index.

With RAR transmission

When with RAR transmission, the RAR is received from serving cell or candidate cell is still under discussion.

If RAR is received from candidate cell, there would be more challenges, including:

- More related parameters needs to be configured, such as Type1-PDCCH CSS of the candidate cell, RAR window length

- More interruptions on the serving cell. Since UE have to switch from the serving cell to candidate cell to monitoring PDCCH and receive RAR. The duration is intolerable, since it can up to 10ms in licensed band and 40ms in unlicensed band. During this long period, new UE behaviour needs to study, i.e if UE monitoring the PDCCH on candidate cell, what is UL or DL happens on the serving cell, because the serving cell has no idea when or whether UE has returned back.

- More complexity to implement. Transmit PRACH only is easier than UE has to receive PDCCH and RAR on the candidate cell. There should have more discussion on the requirements for this case.

So for with RAR transmission, we suggest RAR is received from the serving cell, the content of RAR is same as the current.

- For with RAR transmission, RAR is received from the serving cell.

Without RAR transmission

For RAR transmission, no RAR is the default UE behaviour, since TA indication can be carried in the LTM command. RAR is useless. However, considering the current RACH procedure, it is considered to be complete until RAR is received. So in order to align the understanding between gNB and UE, it needs additional rule to skip RAR for PDCCH ordered RACH for TA acquisition. One efficient method is RACH procedure on the candidate cell is considered as complete when the RACH transmission times to a specific value e.g. PreambleTransMax.

Whether or not UE can autonomous re-transmission of PRACH, it is not preferred to have autonomous re-transmission.  The reasons include:

- First, the autonomous re-transmission has some benefit when the UL coverage is limited. However, the autonomous re-transmission value is fixed and difficult to choose a reasonable number for a UE, since there is lack of channel information before head and unable to change according to measurement. Thus, the re-transmission number is an inconsequent configuration which easy to decrease the efficiency of RACH resource.

- Second, the transmit power of subsequent PRACH is not an easy issue to decide. Because power ramping is not applicable, which can easy lead to great interference unconditionally. If the same power applied to all re-transmissions of PRACH, it is not clear how much gain it could get.

- If reception of RAR is not configured/indicated, UE autonomous re-transmission of PRACH is not allowed.

- RACH procedure on the candidate cell is considered as complete when the RACH transmission triggered by PDCCH order is transmitted.

UE-based TA measurement

During the last RAN1 meeting, there was a working assumption of UE-based TA measurement that UE derives TA based on Rx timing difference between serving cell and candidate cell was made. However, there are other issues need to discuss and then confirm the WA.

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

On the timing of the UE-based TA measurement through Rx timing difference of the target cell, there are some essential issues need to be studied:

- Before the cell switch command given to the UE, the timing that UE has to do the TA acquirement needs to discuss. One possible time is after DL synchronization.

- Whether or not this type of TA acquisition has further trigger or just by UE decision, or is based on certain conditions need more study.

- The third issue is when serving cell and candidate cell have different DL timing and/or UL timing, whether UE-based TA measurement through Rx timing difference can be supported. One possible method is gNB provides according information for UE to modify the TA.

- for UE-based TA measurement through Rx timing difference, study the following two issues:

- Issue1: When UE has to do the TA acquirement

- Issue2: Whether or not this type of TA acquisition has further trigger or just by UE decision, or is based on certain conditions

- Issue3: When serving cell and candidate cell have different DL timing and/or UL timing, whether UE-based TA measurement through Rx timing difference can be supported

Other mechanism to obtain TA of candidate target cell

For mechanism to obtain TA of candidate target cell, both of RACH-based and RACH-less were agreed to support during RAN1#109b-e meeting. Besides PDCCH ordered RACH, there are still other solutions for TA acquisition.

Agreement On mechanism to acquire TA of the candidate cells, the following solutions can be further studied: • RACH-based solutions e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd • RACH-less solutions e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

RACH-based solutions

For UE-triggered RACH, it suffers more challenges comparing with PDCCH-ordered RACH.

- First, UE has to decide when to trigger a RACH procedure. However, as we known, the additional event triggering should be defined first.

- Second, UE-triggered RACH have more steps, including RAR reception, MSG3 transmission, contention resolution. All four steps need to be improved to support before HO command in L1/L2 mobility. Not only the significant standards work but also it may impact the data transmission on the source cell.

- UE-triggered RACH suffers more challenges, including transition/reception interruption to the source cell, and more specification work.

RACH-less solutions

For RACH-less solutions, RACH-less mechanism as in LTE can be supported. The UE can skip the transmission of PRACH preamble and the reception of RACH Response to the target cell only Timing Advance (TA) of the target cell is neither 0 nor the same as the source cell, otherwise RACH-less cannot be performed.

- For RACH-less solutions, RACH-less mechanism as in LTE can be supported

For SRS based TA acquisition, we do not support it due to its complexity design and huge specification impact. SRS based TA acquisition requires to resolve the following two issues. First, SRS transmission towards the candidate cell, including the SRS configuration and the timing of transmission. Second, SRS detection of candidate cell, including resource reservation and big efforts for proceed. Thus, SRS based TA acquisition is not preferred.

- SRS based TA acquisition is not supported in Rel-18 mobility.

Number of TA for candidate cells needs to be acquired

For number of TA needs to be acquired, we support the proposal 1.3 from the FL. For the second FFS, two TAs for a candidate cell cannot be configured. It is not a typical case for L1/L2 mobility.

- Support the proposal 1.3 for number of TA needs to be acquired

Proposal 1.3: For TA management in L1/L2 based mobility, support at least one of each of the candidate cells can may be associated with 1 acquired TA/TAG other than the TA/TAG used for the serving cell • FFS: the maximal number of TA/TAG associated with candidate cell(s) can be handled by UE • FFS: whether two TA(s)/TAG(s) for a candidate cell can be configured • Note1: for STRP operation, more than 1 TA/TAG per candidate cell is not needed • Note2: it can be separately discussed regarding whether the TA/TAG of candidate cell(s) is indicated before or by the received cell switch command if network indication is supported • Note3: it is also possible that no additional TA is required for candidate cells when candidate cells use the same TA as the serving cell. • Note4: the main bullet doesn’t imply that the candidate cells either have to be in the same TAG as the serving cell or have to belong to a second TAG. For example, if we agree to have N candidate target cells for TA acquisition, each of them can be associated with at least one TA/TAG according to this proposal.

TA indication

Association between TA and candidate target cell

According to the association between TA and candidate target cell, the following two alts can both be studied.

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

For Alt1, associate TA/TAG and candidate cell implicitly, such as TA/TAG and TCI state are associated. When TCI state is indicated, the associated TA value will be applied to the candidate cell. Alt 2 associate TA/TAG and candidate cell by explicit configuration.

Alt 2 is more generic for every methods to obtain TA value for a candidate cell. RACH-based or RACH less. For example, all the RACH less methods may not have a chance to indicate TCI state, but it always aware of the candidate cell.

- Support Alt2: Associate TA/TAG and candidate cell explicitly

Conclusion

In this contribution, we made the following proposals.

- Configuration of RACH resource for candidate cell(s) can be provided per candidate cell

- The bits length of the indication of candidate cell in PDCCH order is fixed, depending on the maximum number of candidate cells for a cell group.

- The location of the indication of candidate cell field in PDCCH order is right after PRACH Mask index.

- For with RAR transmission, RAR is received from the serving cell.

- If reception of RAR is not configured/indicated, UE autonomous re-transmission of PRACH is not allowed.

- RACH procedure on the candidate cell is considered as complete when the RACH transmission triggered by PDCCH order is transmitted.

- for UE-based TA measurement through Rx timing difference, study the following two issues:

- Issue1: When UE has to do the TA acquirement

- Issue2: Whether or not this type of TA acquisition has further trigger or just by UE decision, or is based on certain conditions

- Issue3: When serving cell and candidate cell have different DL timing and/or UL timing, whether UE-based TA measurement through Rx timing difference can be supported

- UE-triggered RACH suffers more challenges, including transition/reception interruption to the source cell, and more specification work.

- For RACH-less solutions, RACH-less mechanism as in LTE can be supported

- SRS based TA acquisition is not supported in Rel-18 mobility.

- Support the proposal 1.3 for number of TA needs to be acquired

Proposal 1.3: For TA management in L1/L2 based mobility, support at least one of each of the candidate cells can may be associated with 1 acquired TA/TAG other than the TA/TAG used for the serving cell • FFS: the maximal number of TA/TAG associated with candidate cell(s) can be handled by UE • FFS: whether two TA(s)/TAG(s) for a candidate cell can be configured • Note1: for STRP operation, more than 1 TA/TAG per candidate cell is not needed • Note2: it can be separately discussed regarding whether the TA/TAG of candidate cell(s) is indicated before or by the received cell switch command if network indication is supported • Note3: it is also possible that no additional TA is required for candidate cells when candidate cells use the same TA as the serving cell. • Note4: the main bullet doesn’t imply that the candidate cells either have to be in the same TAG as the serving cell or have to belong to a second TAG. For example, if we agree to have N candidate target cells for TA acquisition, each of them can be associated with at least one TA/TAG according to this proposal.

- Support Alt2: Associate TA/TAG and candidate cell explicitly

References

RP-222332 Revised WID on Further NR mobility enhancements

R1-2302165	Moderator summary on Timing advance management for LTM: Round 4	Moderator(CATT)