3GPP TSG RAN WG1 #112bis-e                                                  R1-2302621

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.11.1

Source:	Spreadtrum Communications

Title:	Discussion on evaluation on low power WUS

Document for:	Discussion and decision

# Introduction

The SID of the low power WUS (LP-WUS) can be found in [1].

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

The contribution focuses on the evaluation methodology and some initial evaluation results.

# Use cases

In RAN#112 [2], it was agreed to define three use cases as follows.

Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any.

# Evaluation methodology

## Power model for the main radio

In RAN1#110bis-e [3], the power model for the main radio was widely discussed and had the good progress.

Agreement Take the following power model for main radio for evaluation in LP-WUS/WUR SI, - For IoT and wearable cases, reuse TR38.875 power model as baseline. - For eMBB and other cases, reuse TR38.840 power model as baseline. - Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver and reusing power model option 1 value of ‘Ultra-deep sleep’for LPHAP evaluation, i.e., - FFS: The details of ‘Ultra-deep sleep’ power state Agreement - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.] - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.] - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio. - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption. Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS.

### The total transition time/energy

It seems that the total transition energy includes the rump up/down energy and sync/re-sync energy, and the total transition time includes the rump up/down time and sync/re-sync time. For ease of understanding, we consider cell search as a part of sync/re-sync.

### Ramp-up/down

In RAN1#112 [2], it was agreed there are two alternatives for transition energy and ramp-up time.

Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

### Sync/re-sync

Cell frequency search

In general, sync/re-sync includes cell frequency search (SS-raster search), timing acquisition and frequency error correction. However, it is not clear whether cell frequency search belongs to sync/re-sync during the main radio wake-up. In our view, companies did not consider cell frequency search for discussion of sync/re-sync. We try to discuss cell frequency search initially.

Specifically, if the LP-WUR supports mobility (continuous coverage), i.e. the LP-WUR can re-select a cell in time/frequency domain during UE movement, and if the LP-WUS is deployed in the same frequency as SSB in the cell, the main radio can know the frequency of the cell to be selected via the LP-WUR. In this case, the cell frequency search is not necessary. The restriction is that the LP-WUR should support mobility and the LP-WUS should be deployed in the same frequency as SSB in the cell.

Observation 1: If the LP-WUR supports mobility (continuous coverage) and if the LP-WUS is deployed in the same frequency as SSB in the cell, the cell frequency search is not necessary at the LP-WUR after wake-up.

As counterpart, if the LP-WUR does not support mobility, i.e. the LP-WUR cannot re-select a cell during UE movement and the main radio should be switched on to re-select a cell, the main radio can know the frequency of cell to be selected. In this case, the cell frequency search is not necessary.

Observation 2: If the LP-WUR does not support mobility, the cell frequency search is not necessary at the LP-WUR after wake-up, which has been done in cell re-selection at the LP-WUR.

For simplicity, we can assume cell frequency search is not included in sync/re-sync.

Timing acquisition and frequency error correction

The main radio can only rely on periodic reference signal for timing acquisition and frequency error correction. For simple case, the periodic reference signal can be SSB. Nevertheless, the periodic reference signal can include both SSB and additional TRS (e.g. R17 defined additional TRS) for optimization purpose. In RAN1#111 [4], the number of SSBs for timing acquisition and frequency error correction is widely discussed, and only the upper bound was agreed.

Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

Since we aim to converge to one or two set of values for the total transition time/energy in RAN1#112, we try to provide the candidate values of the number of SSBs.

In [4], additional 3 SSBs w.r.t. deep sleep is assumed for both timing acquisition and frequency error correction. In our view, the additional x SSBs can be used to acquire timing by decoding PBCH content and detecting time-index in PBCH-DMRS, and another additional y SSBs can be used for frequency error correction (from 10 ppm to 2 ppm). The value of x and y depends on different channel condition. It should be noted that 1/2/3 SSBs for wake-up from deep sleep (frequency error is from 2 ppm to 0.5ppm) are assumed in R17 PEI discussion for different channel condition respectively. Therefore, we have the following number of SSBs for different channel condition respectively and calculate sync/re-sync time/energy.

Table 1: Sync/re-sync time/energy

|  | The number of SSBs | Sync/re-sync time | Sync/re-sync energy |
| --- | --- | --- | --- |
| Deep sleep of the main radio | {1, 2, 3} | {2, 22, 42}ms Assume 2ms for duration of a SSB burst and 20ms for periodicity of SSB bursts | {60*2, 60*2 + 20*18 + 100 + 60*2, (60*2 + 20*18 + 100)*2 + 60*2} = {120, 700, 1280} |
| Ultra deep sleep of the main radio in worse channel condition | 9 3 SSBs for timing acquisition, 3 SSBs for frequency error correction, 3 SSBs for fine sync for PO reception | 162ms | (60*2 + 20*18 + 100)*8 + 60*2 = 4760 |
| Ultra deep sleep of the main radio in general channel condition | 6 2 SSBs for timing acquisition, 2 SSBs for frequency error correction, 2 SSBs for fine sync for PO reception | 102 | (60*2 + 20*18 + 100)*5 + 60*2 = 3020 |
| Ultra deep sleep of the main radio in good channel condition | 3 1 SSB for timing acquisition, 1 SSB for frequency error correction, 1 SSB for fine sync for PO reception | 42ms | (60*2 + 20*18 + 100)*2 + 60*2 = 1280 |

Proposal 1: 3/6/9 SSBs for sync/re-sync for ultra-deep sleep can be assumed for different channel condition respectively.

## Power model for the LP-WUR

In RAN1#110bis-e [3], the power model for the LP-WUR was also widely discussed and had the good progress.

Agreement The following power model for LP-WUR/WUS evaluation is considered, - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: - [0.001] - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] - Other values are not precluded to be evaluated - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed. Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) FFS: LP-WUR power consumption values for FR2.

In RAN1#111 [4], the power model for the LP-WUR was further discussed and the following agreement was achieved.

Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

For the power value of the LP-WUR ‘on’, for simplicity, we select three “typical” values, i.e. 0.01, 0.1 and 1, and calculate the transition energy using the equation TLR, ramp-up *(PON+POFF)/2.

Table 2: The relative power values for the LP-WUR

|  | Relative power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) | Note |
| --- | --- | --- | --- | --- |
| Off | 0.001 | - | - | - |
| On | 0.01 | 1*(0.01+0.001)/2≈0.006 | 1 | Example: Without mixer, without FLL |
|  | 0.1 | 5*(0.1+0.001)/2≈0.25 | 5 | Example: With mixer, with FLL |
|  | 1 | 10*(1+0.001)/2≈5 | 10 | Example: With mixer, with PLL |

As shown the next sections, it can be observed that with low-complexity LP-WUR architecture and LP-WUS design, there could be large performance loss compared to R17 PEI, and thus it leads to very large resource overhead, e.g. exceeding AL 16 PDCCH resource. In our view, it is a critical issue for RRC CONNECTED state. Therefore, we think for RRC CONNECTED state high-complexity LP-WUR architecture (e.g. architecture with high-rate ADC and PLL,) can be used which is also power consumed.

Observation 3: For RRC IDLE or INACTIVE state, the power value of LP-WUR ‘on’ can be small, e.g. 0.01, for low-complexity LP-WUR architecture. For RRC CONNECTED state, the power value of LP-WUR ‘on’ can be large, e.g. 1, for high-complexity LP-WUR architecture.

## Performance metrics

In RAN1#110bis-e [3], the potential performance metrics were listed as follows.

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. FFS: Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS]. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD. FFS: UPT; FFS Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any)

In RAN1#111 [4], the potential performance metrics were updated as follows.

Update the agreement in RAN1#110bis-E as follows, For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS] Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included. FFS: UPT; FFS The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any)

We will focus on RRC IDLE/INACTIVE state and discuss the system overhead, power consumption and latency.

## Traffic model

In RAN1#111 [4], the traffic model option 1 was updated as follows.

Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with i-DRX cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - L=4 (as agreed in RAN1#110bis)

For RRC IDLE/INACTIVE state, if the LP-WUS contains the whole UE ID, the LP-WUS can wake up a UE, and per group paging probability RG is equal to per UE paging probability RE. If the LP-WUS contains a part of UE ID, the LP-WUS can wake up a group of UEs, and per group paging probability RG is much larger than per UE paging probability RE, since value of N is much larger than 1.

However, how to relate the bits number of UE ID in the LP-WUS to value of N is not simple, and value of N is also related to UE distribution in a cell. We may only use a typical value of N. For example, we can choose 10 as value of N like discussion of R17 PEI. We should keep in mind that a small bits number of UE ID in the LP-WUS may imply a large value of N in realistic.

Observation 4: The bits number of UE ID in the LP-WUS is related to value of N, and a small bits number of UE ID in the LP-WUS may imply a large value of N in realistic.

# System overhead

## Comparing different schemes of the LP-WUS

### Comparing system overhead for a given BLER performance

In R17 PEI, link level simulation can help to compare different schemes of PEI in terms of system overhead. Companies provided the system overhead for a required MDR at a required SINR. It is assumed that FAR can be always achieved by long CRC. The required SINR is the SINR to achieve the required MDR of paging PDCCH/PDSCH in a certain setting.

Similarly, for the LP-WUS, link level simulation can help to compare different schemes of the LP-WUS in terms of system overhead, and companies can also provide the system overhead for a required MDR at a required SINR. However, the required SINR cannot be related to paging PDCCH/PDSCH. Actually, the required SINR should be related to the target coverage, e.g. target of the maximum coupling loss in link budget leads to the required SINR. The target coverage for the LP-WUS can be decided as some candidates.

Proposal 2: The target coverage for the LP-WUS can be decided as some candidates.

It should be noted that LP-WUS contains preamble and data payload. For simplicity, we assume preamble is sequence based and it is long enough to achieve the required MDR and FAR. Here, we mainly focus on data payload of the LP-WUS.

In RAN1#110bis-e [3], the required MDR/FAR for the LP-WUS was discussed and provided as follows.

Agreement For the performance evaluations of LP-WUS candidate designs, it is assumed that - The miss-detection rate (MDR) of LP-WUS [1%], - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying reported by companies - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead - FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details.

In RAN1#112 [2], the required MDR is updated as 1%, and the required FAR was updated as [0.1%, 1%].

### Comparing BLER performance for a give system overhead

Alternatively, we can compare BLER performance for a given system overhead (e.g. fixed time/frequency resource), which is shown in RAN1#112 agreement [2].

Agreement - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - raw information bit-size - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - FFS: false alarm probability/rate - FFS: misdetection probability/rate are kept [comparable or fixed]. Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

For total energy of LP-WUS across the time/frequency resource, in our view it is absorbed in SINR. In RAN4 spec (see Appendix A.4), SNR is already defined by the average signal power across frequency resource. It can be easily extended to the average signal power across time/frequency resource. In the simulation, raw information bit 0 and 1 are equally distributed, so there is no worry about the same SINR corresponds to different total energy.

Observation 5: SINR can be defined by the average signal power across time/frequency resource.

## Comparing R17 PEI and the LP-WUS

According to that of R17 PEI (Appendix A.5), parameters for R17 PEI and the LP-WUS are listed as follows.

Table 3: Parameters for R17 PEI and the LP-WUS

|  | R17 PEI | The LP-WUS |
| --- | --- | --- |
| General parameters | Behavior A + channel based |  |
| The required MDR | 0.1% To meet the joint MDR of PEI and paging PDCCH, which is 1%. In this case, MDR of paging PDCCH approaches 1% | 1% |
| The required FAR | 1% It is mainly used for sequence-based PEI. For channel-based PEI, the actual FAR is much lower due to long CRC | 0.1% or 1% For channel-based preamble and data payload, it can be satisfied by using more than10 bits CRC |
| The required SINR | SINR when MDR of paging PDCCH is 1% (assuming the same coverage of R17 PEI and the LP-WUS) |  |
| Paging PDCCH reference | AL 8, 41-bit payload |  |
| Paging PDSCH reference | MCS0, TB scaling 1.0 It may imply the required CFO for PO reception | N/A |

It can be observed that the LP-WUS can use R17 PEI as reference in terms of the required MDR/FAR and SINR.  Therefore, we can also estimate the system overhead of the LP-WUS roughly by comparing to R17 PEI.

We provide our estimated values for the loss and gain of the LP-WUS compared to R17 PEI as follows like those reported by [5].

Table 4: The loss and gain of the LP-WUS (MC-ASK/FSK based) compared to R17 PEI

|  | Value of loss or gain | Note |
| --- | --- | --- |
| Rx number | ~3dB loss | Assume 2 Rx for R17 PEI and 1 Rx for the LP-WUS |
| Noise figure | ~8dB loss | Due to low-complexity of the LP-WUR architecture [5] |
| Sampling rate | ~6dB gain | Assuming 4 samples per OOK symbol. ~4x sampling rate compared to baseband bandwidth |
| Bit-width of ADC | ~6dB loss | Assuming 2 bits width of ADC for the LP-WUR, compared to 8 bits width of ADC for R17 PEI |
| Channel coding scheme | ~3dB loss | Polar code has gain compared to Manchester code |
| Waveform | ~3dB gain | ASK/FSK is more robust than QPSK+OFDM, since QPSK+OFDM is sensitive to frequency error and phase noise, in which ICI will cause degradation |
| Coverage shrinkage | ~x dB gain | For discontinuous coverage, link budget of the LP-WUS is x dB shrinkage than that of R17 PEI. In this case, the main radio is always switched on at the cell edge |

It can be observed that the LP-WUS may have about (11-x) dB performance loss compared to R17 PEI, where x is value of coverage shrinkage compared to R17 PEI.

Observation 6: The LP-WUS may have about (11-x) dB performance loss compared to R17 PEI, where x is value of coverage shrinkage compared to R17 PEI.

From above two tables, the resource overhead for the LP-WUS can be estimated according to that of R17 PEI (Appendix A.5).

Table 5: Resource overhead for R17 PEI and the LP-WUS

|  | R17 PEI |  | The LP-WUS |  |
| --- | --- | --- | --- | --- |
| Information bits | 12 bits | 41 bits | 12 bits: a small part of 48-bit for ng-5G-S-TMSI. The main radio should monitor PO after wake-up | 41 bits: the main part of 48-bit for ng-5G-S-TMSI). The main radio may not monitor PO after wake-up, if the remaining bits for ng-5G-S-TMSI is carried by location of the LP-WUS occasion, like PO location which carries some bits of UE ID |
| Occupying REs | 288 (576 may be also feasible since R17 PEI has lower MDR than paging PDCCH) | 576 | 288*y (11-x) dB => y times of REs compared to R17 PEI | 576*y (11-x) dB => y times of REs compared to R17 PEI |

It can be observed that the resource overhead of the LP-WUS is much larger than that of R17 PEI. For example, when y=5 (i.e. 5dB coverage shrinkage compared to R17 PEI), the LP-WUS may need 288*4 or 576*4 REs for 12 or 41 bits respectively.

Observation 7: System overhead of the LP-WUS is much larger than that of R17 PEI.

# Power saving gain

According to power models and evaluation assumptions, Power Saving Gain (PSG) can be achieved by numerical analysis or system evaluation.

## The preferred evaluation assumptions

For evaluation assumptions with multiple options, we select the preferred options.

- For the LP-WUS

  - Power model

    - LP-WUS off: power value 0.001

    - LP-WUS on: power value 0.1, transition energy 0.25, transition time 5ms

  - Duration of the LP-WUS burst is 2ms:  The LP-WUS is like SSB supporting beam sweeping

  - The LP-WUS is periodically detected by the LP-WUR, and the periodicity is paging cycle for I-DRX

- For the main radio

  - Power model is for eMBB and other cases, i.e. reusing TR38.840 power model as baseline

  - The number of SSBs to process is under assumption of the worse channel condition is worse

- For traffic model

  - Per group paging probability RG is assumed as 10%

## Baseline scheme

R17 PEI can be baseline scheme, which is more power efficient than R15 PO monitoring.

Figure 1: Illustration of baseline scheme, i.e. R17 PEI

### RRM measurement

To be fair, the neighboring-cell measurement relaxation is assumed for R17 PEI, i.e. there is no intra-frequency or inter-frequency measurement during the evaluation time.

It seems fair to consider the serving-cell measurement relaxation for R17 PEI, but the serving-cell measurement is included in SSB processing for sync before PEI in each paging cycle, and thus there seems no additional power saving for the serving-cell measurement relaxation. On the contrary, we think the serving-cell measurement can be supported by the LP-WUS, so there could be fair comparison.

Observation 8: For R17 PEI as baseline scheme, the serving-cell measurement relaxation is not considered.

### Power consumption

For DRX cycle 1.28s, UE processing timeline can be found in Appendix A.6. When PEI indicates not to monitor PO, the power consumption is about 2350. When PEI indicates to monitor PO, the power consumption is about 4011. Therefore, when per group paging probability is 10%, the total power consumption is about 2350*0.9+4011*0.1≈2516.

For DRX cycle 5.12s, UE processing timeline can be found in Appendix A.8. When PEI indicates not to monitor PO, the power consumption is about 6190. When PEI indicates to monitor PO, the power consumption is about 7556. Therefore, when per group paging probability is 10%, the total power consumption is about 6190*0.9+7556*0.1≈6327.

## LP-WUS scheme

There could be two schemes for processing timeline for the LP-WUS.

For Scheme-1, the main radio does not need to monitor PO after wake-up. The latency can be small, but the resource overhead may be large since the LP-WUS may contain one or more entire UE ID, which could be 48 bits for ng-5G-S-TMSI.

For Scheme-2, the main radio still needs to monitors PO after wake-up, it is a little like R17 PEI, and balance between the latency and the resource overhead can be balanced.

In fact, Scheme-1 and Scheme-2 can be combined if we can set PO as 0 (i.e. no paging being monitored) for Scheme-2. Moreover, power consumption after being paged (i.e. PRACH and the following procedures) is not counted in R17 PEI evaluation. Hence, we can start from Scheme-2 for evaluation.

Figure 2: Illustration of Scheme-2

### RRM measurement

It should be noted that baseline scheme still includes serving-cell measurement. Fortunately, whether the LP-WUR supports the serving cell measurement is under discussion. With function of serving-cell measurement, the power consumption can be compared between baseline scheme and LP-WUS scheme. In this sense, both R17 PEI with (neighboring-cell measurement relaxation) and the LP-WUS can be used in the low-mobility or stationary scenarios at least.

### Power consumption

For DRX cycle 1.28s, UE processing timeline can be found in Appendix A.7. When the LP-WUS indicates not to monitor PO, the power consumption is about 1.7.

- For Alt 1, i.e. transition energy 15000, when the LP-WUS indicates to monitor PO, the power consumption is about 21670. Therefore, when per group paging probability is 10%, the total power consumption is about 1.7*0.9+21670*0.1 ≈ 2169. Therefore, the power saving gain is (2516-2169)/2516 ≈ 14%.

- For Alt 2, i.e. transition energy 40000, when the LP-WUS indicates to monitor PO, the power consumption is about 46270. Therefore, when per group paging probability is 10%, the total power consumption is about 1.7*0.9+46270*0.1 ≈ 4629. Therefore, the power saving gain is (2516-4629)/2516 ≈ -84%.

It can be observed that when transition energy is not huge, there is power saving gain for normal DRX length (e.g. 1.28s) for the LP-WUS, but when transition energy is huge, there is negative power saving gain for normal DRX length.

Observation 9: When transition energy is not huge, there is positive power saving gain for normal DRX length (e.g. 1.28s) for the LP-WUS.

For DRX cycle 5.12s, UE processing timeline can be found in Appendix A.9. When the LP-WUS indicates not to monitor PO, the power consumption is about 5.6.

- For Alt 2, i.e. transition energy 40000, when the LP-WUS indicates to monitor PO, the power consumption is about 50110. Therefore, when per group paging probability is 10%, the total power consumption is about 5.6*0.9+50110*0.1 ≈ 5016. Therefore, the power saving gain is (6327-5016)/ 6327≈ 21%.

It can be observed that if DRX cycle is extended, even when transition energy is huge, there is still positive power saving gain.

Observation 10: When transition energy is huge, there is positive power saving gain for long DRX length (e.g. 5.12s) for the LP-WUS.

According to above two observations, different DRX cycle length can be considered for different case of transition energy of main radio.

Proposal 3: The LP-WUS can be applied in different DRX cycle length for different case of transition energy of main radio.

# Latency

For discontinuously (periodically) monitoring the LP-WUS, the latency depends on the periodicity of monitoring. For continuous monitoring the LP-WUS, the latency could be reduced with cost of power consumption. Discontinuously monitoring and continuous monitoring can be used in different use cases for different latency requirement. In fact, we do not expect both power consumption and latency can be improved obviously in a single use case for WAN. For LAN, especially small house, it can be true, which is proved by commercial use, e.g. Bluetooth or power-saving WiFi. Coverage and mobility are always difficulties for WAN.

Proposal 4: The LP-WUS can be applied for different case of latency to handle tradeoff between power saving gain and latency.

# Conclusion

We have the following observations.

Evaluation methodology:

Observation 1: If the LP-WUR supports mobility (continuous coverage) and if the LP-WUS is deployed in the same frequency as SSB in the cell, the cell frequency search is not necessary at the LP-WUR after wake-up.

Observation 2: If the LP-WUR does not support mobility, the cell frequency search is not necessary at the LP-WUR after wake-up, which has been done in cell re-selection at the LP-WUR.

Observation 3: For RRC IDLE or INACTIVE state, the power value of LP-WUR ‘on’ can be small, e.g. 0.01, for low-complexity LP-WUR architecture. For RRC CONNECTED state, the power value of LP-WUR ‘on’ can be large, e.g. 1, for high-complexity LP-WUR architecture.

Observation 4: The bits number of UE ID in the LP-WUS is related to value of N, and a small bits number of UE ID in the LP-WUS may imply a large value of N in realistic.

System overhead:

Observation 5: SINR can be defined by the average signal power across time/frequency resource.

Observation 6: The LP-WUS may have about (11-x) dB performance loss compared to R17 PEI, where x is value of coverage shrinkage compared to R17 PEI.

Observation 7: System overhead of the LP-WUS is much larger than that of R17 PEI.

Power saving gain:

Observation 8: For R17 PEI as baseline scheme, the serving-cell measurement relaxation is not considered.

Observation 9: When transition energy is not huge, there is positive power saving gain for normal DRX length (e.g. 1.28s) for the LP-WUS.

Observation 10: When transition energy is huge, there is positive power saving gain for long DRX length (e.g. 5.12s) for the LP-WUS.

We have the following proposals.

Evaluation methodology:

Proposal 1: 3/6/9 SSBs for sync/re-sync for ultra-deep sleep can be assumed for different channel condition respectively.

System overhead:

Proposal 2: The target coverage for the LP-WUS can be decided as some candidates.

Power saving gain:

Proposal 3: The LP-WUS can be applied in different DRX cycle length for different case of transition energy of main radio.

Latency:

Proposal 4: The LP-WUS can be applied for different case of latency to handle tradeoff between power saving gain and latency.

# Reference

- RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97e, Sep.  12th - 16th, 2022.

- 3GPP RAN1#112 Chairman’s Notes, Feb. 27th – Mar. 3rd, 2022.

- 3GPP RAN1#110bis-e Chairman’s Notes, Oct. 10th - 19th, 2022.

- 3GPP RAN1#111 Chairman’s Notes, Nov. 14th - 18th, 2022.

- R1-2209503, “Low power WUS receiver architectures”, MediaTek, RAN1#110bis-e, Oct. 10th - 19th, 2022.

# Appendix

## A.1  Agreement for baseline assumptions of system level simulation

In RAN1#110bis-e [3], baseline assumptions of system level simulation was agreed as follows.

Agreement The following is assumed for RRC IDLE/INACTIVE evaluation, Parameters; Value. i-DRX cycle length; 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI. e-DRX cycle length; 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE. Number of POs in Paging Frame; 1. Number of DRXs per PTW; 4. Number of SSB before PO / PEI; 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’. Sync/re-sync after ultra-deep sleep; companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync. RRM Measurement; Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not.. LP-WUS monitoring; Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle.. Traffic; Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies.. Others; Reported by companies.

## A.2  Agreement for baseline assumptions of link level simulation

In RAN1#112 [2], baseline assumptions of link level simulation was agreed as follows.

Agreement For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS Attributes; Assumptions. Carrier Frequency; 2.6GHz/4GHz/700MHz. Waveform; OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used. Channel structure; - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence).. SCS of OFDM generator for NR signal; 30kHz/15KHz. Configuration for LP-WUS signal; For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report. WUS duration; Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols. MDR/FAR assumption; - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials. Code scheme; Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….). gNB Channel BW; 20MHz, FFS other values. LP-WUS BW; Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS. Filter; X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed. Adjacent subcarrier interference; - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional. Sampling Rate; - Companies to report.. ADC bit width; 1-bit, 4-bit, 8-bit, ideal and other options are not precluded. Channel Model; See link coverage assumption table (will copy and paste here). Impairment modelling; - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report.

## A.3  Agreement for baseline assumptions of coverage evaluation

In RAN1#110bis-e [3], coverage in terms of link budget was discussed and some assumptions were achieved.

Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

In RAN1#112 [2], coverage evaluation assumptions were agreed.

Agreement For coverage evaluation, the following is used, Number of RX chains at the UE’s MR antenna elements for UE; Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS.. Number of RX chains antenna elements for LP-WUR; 1 Rx Note: agreed in RAN1#110bis. Scenario and frequency; Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD). Reference data rates for MR eMBB; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional). Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Pathloss model (select from LoS or NLoS); Urban: NloS Rural: NloS and LoS. Bandwidth; 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD). Channel model for link-level simulation; TDL-C for NLOS, TDL-D for LOS.. Delay spread; Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns. UE velocity; Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor. Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled.. Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

## A.4  SNR definition in 38.101-4

The SNR definition in 38.101-4-h70 is shown as follows.

The reference point SNR is defined as: SNR=j=1NRXEs(j)j=1NRXNoc(j) - NRX denotes the number of receiver reference points, and the super script receiver reference point j. - The above SNR definition assumes that the REs are not precoded, and does not account for any gain which can be associated to the precoding operation. - Unless otherwise stated, the SNR refers to the SSS wanted signal. - The downlink SSS transmit power is defined as the linear average over the power contributions in [W] of all resource elements that carry the SSS within the operating system bandwidth. - The power ratio of other wanted signals to the SSS is defined in Clause C.3.1.

## A.5  Agreement for PEI overhead in RAN1#104b-e (only Behavior A listed)

Agreement:

Observation 1a:

For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified:

- For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs.

- For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1

  - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability

  - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs

  - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability

  - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource

Observation 2a:

For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e:

- If Behv-A is assumed,

| Paging Setting | PEI candidate design | Physical-layer configuration and resource | UE (sub)group indication capcity | Number of companies providing performance results |
| --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |
|  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |
|  | SSS-based PEI | 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol) | 3 bits | 1 (IDCC) |
|  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |
|  |  |  | 3 bits | 1 (IDCC) |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |
|  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |
|  |  |  | 6 bits | 1 (CATT) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |
|  |  |  | 4 bits | 1 (MTK) |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 4 (OPPO, ZTE, MTK, Intel) |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | 3 bits | 1 (Intel) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 8 bits | 1 (Intel) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) |
|  |  |  | 4 bits | 1 (MTK) |

Observation 3a:

For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation.

- The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO.

- Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI.

- If Behv-A is assumed:

| Paging Setting | PEI candidate design |  | Physical-layer configuration and resource | UE (sub)group indication capacity | Number of companies providing performance results |  | Average resource overhead per PO (REs) |  |  |  | PO and PEI related assumptions | Resource sharing assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI |  | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |  | 17.2 |  | OPPO |  | 1 PEI for up to 12 PO's | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 17.2 |  | ZTE |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 17.6 |  | HW/HiSi |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 21.8 |  | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |  | 49.5 |  | vivo |  | 1 PEI for 4 PO | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 57.6 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam |  |
|  |  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |  | 57.6 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 576.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  | SSS-based PEI |  | 1-symbol SSS, occupying 132 REs (11 RB x 2 symbols) | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |  | 25.4 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 25.4 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 26.4 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 254.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  |  |  |  | 264.0 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 288.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |  | 34.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI |  | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |  | 14.4 |  | Intel |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |  | 123.4 |  | HW/HiSi |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 168.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |  | 21.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |  | 28.8 |  | vivo |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 28.8 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 6 bits | 1 (CATT) |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |  | 30.0 |  | OPPO |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 30.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 300.0 |  | QC |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  | 4 bits | 1 (MTK) |  | 26.0 |  | MTK |  | 1 PEI for up to 4 PO's | Dynamic rate-matching in PDSCH |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs |  | 12 bits |  | 4 (OPPO, ZTE, MTK, Intel) | 34.4 | OPPO |  | 1 PEI for up to 12 PO's |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 43.6 | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |  |
|  |  |  |  |  |  |  | 57.6 | Intel |  | 1 PEI for 1 PO |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) |  | 4 bits |  | 1 (MTK) | 34.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms |  | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) |  | 3 bits |  | 1 (Intel) | 14.4 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) |  | 8 bits |  | 1 (Intel) | 21.6 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) |  | 1 bit |  | 1 (OPPO) | 30.0 | OPPO |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  |  |  | 4 bits |  | 1 (MTK) | 26.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |

## A.6  UE processing timeline for baseline scheme (DRX cycle 1.28s)

The following table shows UE processing timeline and power consumption, when PEI indicates not to monitor PO.

Table 6: UE processing timeline and power consumption, when PEI indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 = 120 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 = 420 |
| PEI | Early indicate not to monitor PO | 2 | 50 * 2 = 100 |
| Deep sleep | Power saving | 1260 | 1 * 1260 + 450 = 1710 |
| Total |  | 1280 | 2350 |

The following table shows UE processing timeline and power consumption, when PEI indicates paging message. We assume time gap between PEI and the associated PO is well configured which can accommodate two SSB bursts.

Table 7: UE processing timeline and power consumption, when PEI indicates to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 = 120 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 = 420 |
| PEI | Early indicate to monitor PO | 2 | 50 * 2 = 100 |
| SSB proc. | Coarse/fine synchronization, serving-cell RRM measurement | 2 | 50 * 2 = 100 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 = 460 |
| SSB proc. | Fine synchronization, serving-cell RRM measurement | 2 | 50 * 2 = 100 |
| Light sleep | Power saving | 8 | 20 * 8 + 100 = 260 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Deep sleep | Power saving | 1226 | 1 * 1226 + 450 = 1676 |
| Total |  | 1280 | 4011 |

## A.7  UE processing timeline for LP-WUS scheme (DRX cycle 1.28s)

We assume the location of the LP-WUS is the same as that of PEI.

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is not paged”).

Table 8: UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 13 + 5 | 0.001 * 13 + 0.25 = 0.263 |
| LP-WUR on | Indicate MR not to monitor PO | 2 | 0.1 * 2 = 0.2 |
| LP-WUR off | Power saving | 1260 | 0.001 * 1260 = 1.26 |
| Total |  | 1280 | ~1.7 |

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is paged”).

Table 9: UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO, for Alt-1 (i.e. transition energy is 15000)

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 13 + 5 | 0.001 * 13 + 0.25 = 0.263 |
| LP-WUR on | Indicate NR to monitor PO | 2 | 0.1 * 2 = 0.2 |
| Main radio ramp-up/down | The main radio is ramp-up/down | 400 | 15000 |
| SSB proc. within 8 SSB periodicities | The main radio is sync/re-sync | 2 * 8 | 50 * 2 * 8 = 800 |
| Light sleep within 8 SSB periodicities | Power saving | 18 * 8 | (20 * 18 + 100) * 8 = 3680 |
| SSB proc. within 1 SSB periodicity | The main radio is sync/re-sync | 2 | 50 * 2 = 100 |
| Light sleep within 1 SSB periodicity | Power saving | 8 | 20 * 8 + 100 = 260 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 = 220 |
| Deep sleep | Power saving | 680 | 1 * 680 + 450 = 1130 |
| Total |  | 1280 | 21670 |

Table 10: UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO, for Alt-2 (i.e. transition energy is 40000)

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 13 + 5 | 0.001 * 13 + 0.25 = 0.263 |
| LP-WUR on | Indicate NR to monitor PO | 2 | 0.1 * 2 = 0.2 |
| Main radio ramp-up/down | The main radio is ramp-up/down | 800 | 40000 |
| SSB proc. within 8 SSB periodicities | The main radio is sync/re-sync | 2 * 8 | 50 * 2 * 8 = 800 |
| Light sleep within 8 SSB periodicities | Power saving | 18 * 8 | (20 * 18 + 100) * 8 = 3680 |
| SSB proc. within 1 SSB periodicity | The main radio is sync/re-sync | 2 | 50 * 2 = 100 |
| Light sleep within 1 SSB periodicity | Power saving | 8 | 20 * 8 + 100 = 260 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 = 220 |
| Deep sleep | Power saving | 280 | 1 * 280 + 450 = 730 |
| Total |  | 1280 | 46270 |

## A.8 UE processing timeline for baseline scheme (DRX cycle 5.12s)

The following table shows UE processing timeline and power consumption, when PEI indicates not to monitor PO.

Table 11: UE processing timeline and power consumption, when PEI indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 = 120 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 = 420 |
| PEI | Early indicate not to monitor PO | 2 | 50 * 2 = 100 |
| Deep sleep | Power saving | 5100 | 1 * 5100 + 450 = 5550 |
| Total |  | 5120 | 6190 |

The following table shows UE processing timeline and power consumption, when PEI indicates paging message. We assume time gap between PEI and the associated PO is well configured which can accommodate two SSB bursts.

Table 12: UE processing timeline and power consumption, when PEI indicates to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 = 120 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 = 420 |
| PEI | Early indicate to monitor PO | 2 | 50 * 2 = 100 |
| SSB proc. | Coarse/fine synchronization, serving-cell RRM measurement | 2 | 50 * 2 = 100 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 = 460 |
| SSB proc. | Fine synchronization, serving-cell RRM measurement | 2 | 50 * 2 = 100 |
| Light sleep | Power saving | 8 | 20 * 8 + 100 = 260 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Deep sleep | Power saving | 5066 | 1 * 5066 + 450 = 5516 |
| Total |  | 5120 | 7556 |

## A.8  UE processing timeline for LP-WUS scheme (DRX cycle 5.12s)

We assume the location of the LP-WUS is the same as that of PEI.

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is not paged”).

Table 13: UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 13 + 5 | 0.001 * 13 + 0.25 = 0.263 |
| LP-WUR on | Indicate MR not to monitor PO | 2 | 0.1 * 2 = 0.2 |
| LP-WUR off | Power saving | 5100 | 0.001 * 5100 = 5.1 |
| Total |  | 5120 | ~5.6 |

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is paged”).

Table 14: UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO, for Alt-2 (i.e. transition energy is 40000)

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 13 + 5 | 0.001 * 13 + 0.25 = 0.263 |
| LP-WUR on | Indicate NR to monitor PO | 2 | 0.1 * 2 = 0.2 |
| Main radio ramp-up/down | The main radio is ramp-up/down | 800 | 40000 |
| SSB proc. within 8 SSB periodicities | The main radio is sync/re-sync | 2 * 8 | 50 * 2 * 8 = 800 |
| Light sleep within 8 SSB periodicities | Power saving | 18 * 8 | (20 * 18 + 100) * 8 = 3680 |
| SSB proc. within 1 SSB periodicity | The main radio is sync/re-sync | 2 | 50 * 2 = 100 |
| Light sleep within 1 SSB periodicity | Power saving | 8 | 20 * 8 + 100 = 260 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 = 220 |
| Deep sleep | Power saving | 4120 | 1 * 4120 + 450 = 4570 |
| Total |  | 1280 | 50110 |