3GPP TSG RAN WG1 #112bis-e	R1-2302627

Electronic meeting, 17th – 26th April, 2023

Agenda item:		9.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on the general aspects of ML for Air-interface

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], and objectives of the SI are as follows:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we further discuss the general aspects of AI/ML with respect to air interface based on the agreements from RAN WG#1.

# Discussion

## Prioritization within the scope of the study

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface establishes a foundation for further evolution of ML at air interface and paves a road to 6G. Rel.18 is the first step in the long-lasting journey toward the expansion of ML functionalities in the air interface. Considering time constraints in Rel.18 and current progress, we believe that RAN1 needs to narrow down the scope of the study and deprioritize the most challenging topics (such as open format models or online training methods), leaving those for study in future releases.

At the same time, it is important to ensure that the AI/ML framework produced in Rel.18 is future-proof, i.e., it can be extended with more disruptive features in the future without reverting to agreed solutions.

We suggest during RAN1#112bis-e to discuss and agree on the topics that can be deprioritized for the remainder of the study.

Proposal 1: RAN1 to discuss prioritization of topics to focus on in Rel.18 considering the following principles:

- Narrow down the scope and ensure timely completion of the study;

- Ensure that AI/ML framework produced by Rel.18 is future-proof.

## Functionality identification

Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately.

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

The previous working assumption and the agreements in RAN1#111 [3] and RAN1#112 [2] have introduced two different ML-related identification types: Functionality identification and Model identification. These agreements define to a very large extent the 3GPP framework of AI/ML for Rel-18. Figure 1 illustrates the usage of Functionality and Model ID with applicable conditions information (also discussed before in [4]-[6]).

Figure 1: ML-enabled Feature: Usage of Functionality and Model ID with applicable conditions information.

### Functionality-based LCM

#### Applicable conditions

RAN1#112 [2] has discussed the following FL proposal on applicable conditions.

[FL4] Proposal 5-8i: At least for UE-side models and UE-part of two-sided models, RAN1 to study - How to define and study a (set of) applicable conditions for functionalities/[models]. - Note: Applicable conditions may be used to enable development of scenario/configuration/[site]-specific models [and, if needed, report the models’ applicability to the Network]. - Whether and how to define performance targets (possibly as a part of applicable conditions) for functionality/[models] - Whether and how UE reports a (set of) applicable conditions for supported functionalities (and if needed, for supported models) and/or supported set of functionalities.

As the FL proposal mentions, RAN1 shall first identify the applicable conditions for supported functionality/functionalities of a given sub-use case (ML-enabled feature). In Functionality identification and functionality-based LCM, knowing the UE conditions (including parameters/configurations) is required at the network as the first step prior to any other step, as this shall reveal the background conditions when using ML models for supporting a given ML-enabled feature. These applicable conditions may be depended on different sub-use cases, but we expect that at least a common set of applicable conditions (i.e., the definitions of the parameters are common, but the parameter values might be different depending on the Functionality or Feature) can be derived across all sub-use cases that are under discussion in Rel-18.

In Figure 1 we show the potential use of the applicable conditions. Each Functionality can be identified by a subset of applicable conditions, which does not prevent individual applicable conditions to be re-used in different sets and/or different Functionalities.

The examples are provided in Tables 1-3, for selected sub-use cases of beam prediction, CSI compression, and positioning. Only the most essential applicable conditions are mentioned in these tables and further conditions can be identified depending on the additional features considered in different sub-use cases.

Table 1: Applicable conditions for BM-Case1

| Condition | Description |
| --- | --- |
| 1. Support Top-K DL Tx beam prediction (K = 1, 2, 4, [8]) | This defines the support of predicting best-K NZP CSI-RS resources based on SSB and/or CSI-RS-based RSRP measurements. |
| 2. Set B conditions | - Measured DL RS (SSB, CSI-RS, SSB and CSI-RS) Defines support of using SSB and/or CSI-RS-based RSRP measurements. - Measured DL RS set dimension (4, 8, 12, [16]) Indicates the minimum number of NZP-CSI-RS resources that shall be measured and used by the UE for predicting best-K NZP CSI-RS resources - Measured DL RS set pattern (e.g., fixed, pre-configured list, random) Indicates the limitations on the set pattern for Set B conditions |
| 3. Set A conditions | - Predicted DL RS (CSI-RS) Defines support of predicting CSI-RS resources.. - Predicted DL RS set dimension (16, 32, 64) Indicates the maximum number of NZP-CSI-RS resources that shall be configured as the prediction NZP-CSI-RS resource set |
| 4. NW-side performance monitoring conditions | - Support measurements of Predicted DL RS set (full Set A, partial Set A) Defines the support of measuring the NZP-CSI-RS resources that correspond to Set A. - Measurement periodicity (100 ms, 200 ms) Indicates the minimum periodicity when supporting NZP-CSI-RS resources that correspond to Set A |
| 5. Conditions on supporting ML functionalities | - Max number of supported functionalities (1, 2, 4, 8,) Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE - Delay in activating a functionality (2 ms, 4 ms, .) Indicates the delay required when activating or switching a functionality. - Generalization condition of functionalities (true, false) Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation of whether the functionality is applicable or not. |

Table 2: Applicable conditions for CSI-Compression

| Condition | Description |
| --- | --- |
| 1. CSI-RS measurement conditions | - Maximum number of simultaneously active CSI-RS ports/resources - Maximum number of simultaneously active CSI-RS ports/resources for each AI/ML-based CSI type Defines the maximum number of CSI-RS ports/resources that can be simultaneously active per band or per band combination. “Simultaneous” for CSI-RS means, in any slot, the number of active CSI-RS resources/ports |
| 2. CSI-RS and CSI reports configuration conditions | - Maximum number of configured CSI-RS/IM ports/resources (e.g., in CSI-RS-IM-ReceptionForFeedback) - Maximum number of configured CSI Report Settings (e.g., in csi-ReportFramework) Defines the maximum number of CSI-RS/IM ports/resources and CSI Report Settings that can be configured per BWP (regardless of whether they are active) |
| 3. CSI calculation conditions (i.e., number of occupied CPUs) | Defines the maximum number of CPUs that can be simultaneously occupied by all CSI or beam reports in any given symbol |
| 4. Output CSI conditions | Defines the supported definitions/conditions on the output CSI, including: - Compression ratio conditions (e.g., CR4, CR8,..) Defines the supported compression ratios of the compressed CSI codebook supported by the UE. - Quantizer conditions (e.g., SQ1, VQ1,..) Defines the supported quantization modes for compressed CSI codebook supported by the UE. |
| 5. Pairing ID(s) (e.g., model ID(s), dataset ID(s)) | Indicates pairing ID(s) (interpretable by the NW) to match the UE side and NW side models. One pairing ID can be reported by a bit field defined in the spec (e.g., 3 or 4 bits) which allows NW to consider selecting a matching model on the NW side. |
| 6. Conditions on supporting ML functionalities | - Max number of supported functionalities (1, 2, 4, 8, ...) Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE - Delay in activating a functionality (2 ms, 4 ms, ...) Indicates the delay required when activating or switching a functionality - Generalization condition of functionalities (true, false) Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. |

Table 3: Applicable conditions for Positioning Case 1 – Direct AI/ML (AIML_direct_DL_CIR_ UEside).

| Condition | Description |
| --- | --- |
| 1. Supported N’t (N’t = 64, 128, 256, 512) | Indicates the N’t values that the UE is capable to consider in AIML_direct_DL_CIR_UEside positioning. Note: N’t is the number of the first time domain samples for the CIR input parameter. |
| 2. Supported N_port (N_port = 1,2,4) | Indicates the N_port is the number of transmit/receive antenna port pairs to consider in AIML_direct_DL_CIR_UEside positioning. Note: the antenna port is a logical entity that can be connected to different physical antennas. |
| 3. Supported N_TRP (N_TRP = 1, 2, …, 72) | Indicates the number of N_TRP, which is the number of TRPs to consider for AIML_direct_DL_CIR_UEside positioning. |
| 4. Supported set conditions for measured DL PRS | Defines support of using DL PRS based CIR measurements for AIML_direct_DL_CIR_UEside. |
| 5. Supported performance monitoring conditions | Defines the minimum periodicity to report performance monitoring for AIML_direct_DL_CIR_UEside. |
| 6. Supported estimated CIR quantization reporting | Defines the discrete characteristics of the CIR for AIML_direct_DL_CIR_UEside. |
| 7. Conditions on supporting ML functionalities | - Max number of supported functionalities (1, 2, 4, 8, ...) Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE - Delay in activating a functionality (2 ms, 4 ms, ...) Indicates the delay required when activating or switching a functionality - Generalization condition of functionalities (true, false) Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. |

Proposal 2: At least for UE-side models and UE-part of two-sided models, define the sub-use case specific applicable conditions for functionalities and identify the common applicable conditions for functionalities across different sub-use cases.

Next, in Table 4, we provide a framework for identifying a common set of applicable conditions which can be derived across all sub-use cases that are under discussion in Rel-18.

Table 4: Framework for applicable conditions for all ML-enabled use cases/Features

| Condition | Description |
| --- | --- |
| 1. Conditions on inference (use case specific) | Indicates the capabilities associated with configurations/parameters (use-case specific) for functionalities of ML-enabled feature. |
| 2. Conditions on performance monitoring (use case specific) | Indicates UE support for NW-sided functionality monitoring, and conditions on related configuration options for functionality performance monitoring. If applicable, indicates UE support for UE-sided functionality performance monitoring, and conditions on feedback/reporting (triggers, events, proxy KPIs, reporting mechanisms, etc.) |
| 3. Conditions on functionality configurations (generic) | Indicates the max number of configured/activated functionalities, delays in activating/switching of functionalities, and Generalization condition of functionalities |
| 4. Conditions on functionality validation procedure (use case specific) | Indicates UE support for Functionality validation procedure (NW-initiated, UE-triggered). Indicates conditions for validation procedure (delays, measurement configurations, reporting configurations) |
| 5. Conditions on supporting fallback (use case specific) | Indicates UE support for one or more fallback features (triggers/events, delays, etc.) |
| 6. Context information (use case specific) - OPTIONAL | If applicable, indicates UE support for monitoring and reporting conditions for UE-side inference context e.g., radio KPIs not explicitly linked to the ML Functionality, and/or non-radio metrics (position, movement, temperature, etc.) |

Proposal 3: At least for UE-side models and UE-part of two-sided models, the applicable conditions for functionalities shall contain the following (see Table 4),

- Conditions on inference (use case specific)

- Conditions on Performance monitoring (use case specific)

- Conditions on functionality configurations (generic)

- Conditions on functionality validation procedure (use case specific)

- Conditions on supporting fallback (use case specific)

- If applicable context information (use case specific)

#### Reporting of applicable conditions

RAN1#112 [2] agreed that the UE capability reporting is a starting point for functionality identification. As shown in Tables 1 and 2, the applicable conditions are more similar to UE feature group (FG) components in the legacy UE capability reporting framework. In general, we see a few options to discuss further,

- Option 1: Reporting of applicable conditions only via UE capability reporting.

  - Considering BM-Case1 as an example, the listed components in Table 1 are reported by the UE capability signaling by a UE supports BM-Case1 with the candidate values that are defined by the specification for FG components. Some components may be defined as basic components and others may define as optional components.

  - This option can be used only for static capabilities/applicable conditions (not expected to change over time)

  - This option should still be enough to make things work in Rel-18/19.

- Option 2: Reporting of static applicable conditions via UE capability reporting and additional applicable conditions (semi-static) via a separate reporting method.

  - Considering BM-Case1 as an example, some applicable conditions in Table 1 are reported by the UE capability signaling by a UE supports BM-Case1 with the candidate values that are defined by the specification for FG components. Other applicable conditions in Table 1 can be reported with some other reporting framework (separated from the UE FG specification).

  - It may be possible to report an update some of the applicable conditions by the UE. However, RAN1 shall carefully consider the signaling impacts of this as it may increase the signaling design.

Proposal 4: UE capability reporting to be used only for reporting static applicable conditions.

Proposal 5: RAN1 to study whether an additional reporting method is needed for reporting applicable conditions in addition to UE capability reporting framework.

#### Create/configure functionalities

Even though the above sub-sections discuss applicable conditions and reporting methods, the main purpose of defining applicable conditions and reporting applicable conditions to the network is to configure the UE with one or more functionality to operate/control AI/ML-enabled features. We expect the following alternatives to consider in creating/configuring the functionalities.

- Alt 1: NW-configured functionalities

  - Functionalities (one or more) are created as the network prefers (similar to many other RRC/LPP configurations in NR) based combination of appliable conditions (at least the parameter combinations). Each functionality may refer to an RRC/LPP configuration that refers to the selected/configured applicable condition using the ML-enabled feature.

  - Applicable conditions are reported via UE capability reporting.

  - If there is more than one functionality, those are identified by an RRC (or LPP) list ID or mode (similar to legacy).

- Alt 2: UE-reported functionalities

  - The UE may directly report one or multiple functionalities that it supports where each functionality may correspond to a combination of appliable conditions (at least the parameter combinations).

  - This is more like UE-created functionality reporting.

  - To support this via UE-capability reporting, we need to consider applicable parameter combination set reporting (e.g., a combination may contain parameters for input/output/and many other conditions) but legacy UE capability reporting has to adjust to fit such framework.

- Alt 3: NW-configured, UE confirmed functionalities

  - This may be a combination of Alt 1 and Alt 2, where Alt 1 is considered first. Later the UE can indicate preferred functionalities (dynamic or semi-static) without using UE-capability reporting to allow functionality switching/activation/etc.

Proposal 6: At least for UE-side models and UE-part of two-sided models, the NW creates/configures functionalities to the UE with each functionality referring to a configuration message (e.g., RRC or LPP) that contains NW-selected applicable conditions (according to the UE capability).

- Further study whether an additional step of reporting UE-preferred functionalities is needed after NW configures functionalities towards the UE (e.g., prior to any selection/activation of a functionality).

#### Functionality performance monitoring

Functionalities are different for each (sub)use case, UE or NW or both can activate a limited number of Functionality at a time (see Figure 1). The reasons for this could be limitations in available resources or other reasons. Therefore, the procedure for selecting a Functionality is clearly needed. Relying on functionality-based ML-feature management, allows NW to keep control over ML feature, without engaging in managing ML models used in each functionality. A UE may have multiple models for different scenarios within the same ML functionality. It would be sufficient to inform NW about the ML feature and related functionalities. This would alleviate the issue of registering and validating each UE side model separately to the NW, while ensuring a reasonable level of NW control over the ML features.

According to RAN1's previous discussion, for UE-sided and two-sided models, Functionality performance monitoring can happen either in NW or UE. Even though there can be monitoring on the UE side, functionality performance monitoring shall always be an NW-controlled monitoring process. With NW-controlled monitoring, it is up to the NW to configure the frequency of the monitoring (for example, periodically or event-triggered) as well as the types of monitoring (for example, data distribution, inference accuracy, proxy KPI distribution, system KPI).

The performance of Functionality (at gNBs or UEs) may degrade i.e., the performance drops in a certain context or negatively impacts the performance of other UEs, gNBs. If the Network detects performance issues due to a UE-side Functionality operating at Level Y or Z, it should be able to switch to another UE Functionality for the same ML-enabled Feature, with better performance, or deactivate the ML-enabled feature (i.e., switch to non-ML fallback).

The NW is likely to have additional awareness about radio environmental conditions in its service area that are not available to the UEs. For example, NW is aware about coverage enhancement solutions, such as NW-controlled repeaters, that are transparent to UEs but may significantly impact the performance ML-enabled features. Also, NW may detect that some UEs experienced issues with active Functionality (supported by a certain ML Model). Then the NW may trigger a recommendation to the UE to switch the Functionality proactively to Functionalities used at other UEs in similar conditions​ (same location, type of UE, UE mobility, UE antenna configuration, etc.). In some cases, NW may even reduce the overheads of performance monitoring or measurements by implementing per group reporting, e.g., users following the same trajectory (in a bus) can minimize their measurement reporting overhead if they experience similar measurements according to the most recent history.

Observation 1: The network is better aware of the radio environmental conditions in its service area/cells/sector/beam compared to the localized awareness of individual UEs.

Proposal 7: At least for UE-side models and UE-part of two-sided models, functionality or functionalities associated with the ML-enabled feature are always monitored based on an NW-controlled manner.

An important aspect discussed in RAN1 #112 is highlighted in yellow below, where ML models are not identified by the NW and UE is allowed to do the model-based LCM.

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

When the UE’s handles model-based LCM as UE implementation-specific matter (e.g., a UE may support more than one ML model for a given Functionality and decide switching across these ML models without impacting the functionality), there may be performance variations for the functionality even though it is not always visible at the UE side. From the NW perspective, the performance monitoring may be carried out only for the functionality level and any changes due to background ML model changes at the UE may not be visible at the NW or controllable by the NW. If the overall performance of the functionality degrades or varies significantly over time (i.e., not reliable ML model inference), due to the autonomous model selection, activation, switching, and updates at the UE side, the network may initiate the deactivation of the whole functionality (e.g., deactivation of the use of CSI prediction). Therefore, a high-level tracking of UE’s model-LCM changes is needed (may not be the explicit information on the exact LCM step) such that NW can indicate the UE on certain time instances where model-LCM impacted the functionality performance monitored by the NW side.

Proposal 8: At least for UE-side models and UE-part of two-sided models, to ensure performance is not impacted due to UE’s autonomous model-LCM changes (e.g., model update, switch) for a used/activated functionality, RAN1 to support NW to track the performance variations due to the UE’s model-LCM changes and partially control UE’s model-LCM changes.

- Study the exact performance monitoring framework and control mechanism to enable such a controllability

#### Functionality selection, (de)activation, switching

A UE-side Functionality can be activated, from a set of configured functionalities, for inference operation and it shall be done by the NW. If UE wishes to indicate a preference on activating a Functionality for inference, the network should first allow this operation, based on the UE ML-related support information it has available (provided by the UE) and other system information in the cell. The Functionality deactivation operation can be triggered either by gNB or UE. Activation, deactivation (fallback to a non-ML function), and switching of a Functionality can be executed immediately or after a certain time, and within a certain allowed time window.

Observation 2: Functionality selection/activation is up to the NW and is based on the configured functionalities to the UE.

Proposal 9: RAN1 to prioritize Network-controlled Functionality selection, activation, deactivation, switching, and fallback, and focus only on the following variants:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE – mainly for deactivation of an activated functionality

  - Event-triggered as configured by the network, UE’s decision is reported to network

It is also important to consider the Functionalities which are supported by two-sided ML Models, i.e., require both gNB and UE ML Model inference operations, like studied in the CSI compression use case. The above proposed network-controlled Functionality mechanism is still applicable provided the network is made aware of the pairing ID which is providing implicit information on dataset/model considered at the UE.

# Conclusion

In this contribution, we discuss further aspects related to AI/ML for air-Interface, with the following observations and proposals.

Observations

Observation 1: The network is better aware of the radio environmental conditions in its service area/cells/sector/beam compared to the localized awareness of individual UEs.

Observation 2: Functionality selection/activation is up to the NW and is based on the configured functionalities to the UE.

Proposals

Proposal 1: RAN1 to discuss prioritization of topics to focus on in Rel.18 considering the following principles:

- Narrow down the scope and ensure timely completion of the study;

- Ensure that AI/ML framework produced by Rel.18 is future-proof.

Proposal 2: At least for UE-side models and UE-part of two-sided models, define the sub-use case specific applicable conditions for functionalities and identify the common applicable conditions for functionalities across different sub-use cases.

Proposal 3: At least for UE-side models and UE-part of two-sided models, the applicable conditions for functionalities shall contain the following (see Table 4),

- Conditions on inference (use case specific)

- Conditions on Performance monitoring (use case specific)

- Conditions on functionality configurations (generic)

- Conditions on functionality validation procedure (use case specific)

- Conditions on supporting fallback (use case specific)

- If applicable context information (use case specific)

Proposal 4: UE capability reporting to be used only for reporting static applicable conditions.

Proposal 5: RAN1 to study whether an additional reporting method is needed for reporting applicable conditions in addition to UE capability reporting framework.

Proposal 6: At least for UE-side models and UE-part of two-sided models, the NW creates/configures functionalities to the UE with each functionality referring to a configuration message (e.g., RRC or LPP) that contains NW-selected applicable conditions (according to the UE capability).

- Further study whether an additional step of reporting UE-preferred functionalities is needed after NW configures functionalities towards the UE (e.g., prior to any selection/activation of a functionality).

Proposal 7: At least for UE-side models and UE-part of two-sided models, functionality or functionalities associated with the ML-enabled feature are always monitored based on an NW-controlled manner.

Proposal 8: At least for UE-side models and UE-part of two-sided models, to ensure performance is not impacted due to UE’s autonomous model-LCM changes (e.g., model update, switch) for a used/activated functionality, RAN1 to support NW to track the performance variations due to the UE’s model-LCM changes and partially control UE’s model-LCM changes.

- Study the exact performance monitoring framework and control mechanism to enable such a controllability

Proposal 9: RAN1 to prioritize Network-controlled Functionality selection, activation, deactivation, switching, and fallback, and focus only on the following variants:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE – mainly for deactivation of an activated functionality

  - Event-triggered as configured by the network, UE’s decision is reported to network

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN #94-e.

- R1-2301868, “Final Summary of General Aspects of AIML Framework”, 3GPP RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2213003, “Summary#1 of General Aspects of AI/ML Framework”, 3GPP RAN WG1 #111, November 14 – 18, 2022.

- R1-2300603. “Further discussion on the general aspects of ML for Air-interface”, 3GPP RAN WG1 #112, Athens, Greece, February 27th – March 3rd, 2023

- R1-2212326, “Further discussion on the general aspects of ML for Air-interface”, TSG RAN WG1 #111, November 14 – 18, 2022.

- R1-2209366, “Further discussion on the general aspects of ML for Air-interface” TSG RAN WG1 #110bis-e, October 10 – 19, 2022.