3GPP TSG RAN WG1 #112bis-e	R1-2302633

Electronic Meeting, 17 – 26 April, 2023

Agenda item:		9.2.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Other aspects on ML for positioning accuracy enhancement

Document for:		Discussion and Decision

# Introduction

A new study item (SI) on artificial intelligence (AI) / machine learning (ML) for new radio (NR) air interface was agreed in 3GPP TSG RAN Meeting #94e [1]. One use case studied in the SI includes position accuracy enhancement for different scenarios (e.g., those with heavy non-line-of-sight (NLOS) links). One of the targets under consideration is to assess potential specification impact on PHY layer aspects as RAN1.

In this contribution, we mainly present our views on cases and their respective specification impact for AI/ML based positioning accuracy enhancement. The study is done on agreements made in all meetings until 3GPP RAN1#112 meetings, which are shared in Appendix A. We also include in the discussion proposals not agreed upon topics for further study in 3GPP RAN1#112 meeting and previous meetings.

The rest of this paper is organized as follows, in Section 2 we show our own mapping and illustrative concepts of each AI/ML positioning representative cases, in Section 3 we discuss the generic framework on functionality identification already agreed in the agenda item 9.2.1 considering AI/ML positioning particularities and potential applicable conditions. In Section 4 we discuss aspects and specification impact related to data collection for training and monitoring. In Section 5 and Section 6 we discuss specification impact on performance monitoring and model inference respectively. In Section 7 we discuss other aspects not considered in the previous sections. Finally, in Section 8 we provide a summary of observations and proposals.

# Cases of AI/ML based positioning

At the RAN1#110-bis-e meeting, the following agreement defined some representative cases:

Agreement - Study and provide inputs on benefit(s) and potential specification impact, at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML-assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML-assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

## Our understanding of each case is described and illustrated in the following subsections.

## Case 1

In this case the model deployment is done in the UE-side. Here, the direct AI/ML and AI/ML assisted positioning sub-use cases can be deployed to get as output the horizontal position or intermediate feature, respectively. For both cases, the parameter measurement used as input is done in the UE (downlink positioning) and the final position estimation is also in the UE-side. An illustration indicating the reference signal used to generate measurements, and the entities involved in each case are presented in Figure 1.

- Direct AI/ML Positioning; - Assisted AI/ML Positioning. Figure 1: Case 1 – UE-based positioning with UE-side model, (a) direct AI/ML (b) or AI/ML assisted positioning

## Case 2a

In this case the position estimation is done in the LMF-side. However, the model is deployed in the UE-side to perform the AI/ML assisted positioning method, which estimates the intermediate feature. Here, the measurement is done in the UE (downlink positioning). This intermediate feature is reported to the LMF to estimate the position. An illustration of this case is represented in Figure 2.

Figure 2: Case 2a – UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

## Case 2b

The difference of this case with Case 2a is that the model is deployed in the LMF-side to enable a direct AI/ML positioning method. As the previous case, the measurement is done in the UE (downlink positioning). An illustration of this case is represented in Figure 3.

Figure 3: Case 2b – UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

## Case 3a

The difference of this case with Case 2a is that the model is deployed in the gNB-side to enable an AI/ML assisted positioning method. Other difference is that the measurement is done in the gNB (uplink positioning). Here, the output is an intermediate feature that should be reported to the LMF. Finally, the LMF perform the final position estimation. An illustration of this case is represented in Figure 4.

Figure 4: Case 3a – NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

## Case 3b

In this case, the model output is the horizontal position, which is estimated in the LMF-side using measurements already obtained in the gNB (uplink positioning) and using a model deployed in the LMF-side using a direct AI/ML positioning. An illustration of this case is represented in Figure 5.

Figure 5: Case 3b – NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Each of these cases are discussed accordingly to individual perspectives, including data collection, performance monitoring, and model inference in the following sections.

Proposal 1: For understanding various AI/ML positioning cases, it would be beneficial to include illustrative figures in the technical report (TR 38.843).

# Framework for AI/ML based Positioning

Based on the agreement in general aspects Agenda item 9.2.1 in RAN1#112 meeting related to AI/ML functionality identification:

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

Our understanding is that the LTE positioning protocol (LPP) represents the 3GPP legacy framework for positioning. Thus, the current procedures already specified in the 3GPP 37.355 can be reused to enable the transfer of capabilities from the UE to the NW(LMF). For instance, in Figure 6 and Figure 7 are shown already standardized procedures to transfer capabilities and indication capabilities from the target UE to the LMF.

Figure 6 - LPP capabilitiy transfer procedure already standardized in TS 3GPP 37.355.

Figure 7 - LPP capability indication procedure already standardized in TS 3GPP 37.355.

Besides that, in 3GPP RAN1#112 meeting the following note was agreed between companies:

Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

To address this note, some examples of functionalities or positioning methods are shared in Table 1, specifically, the examples are focusing on Cases 1 and 2a. In the same table, we also considered two extra fields, they are the UE’s capability conditions (UE capability report), and the mandatory/optional field. This is a preliminary mapping that should be discussed and iterated between companies to enhance our mutual understanding following the general framework defined in the agenda item 9.2.1.

Proposal 2: Regarding functionality identification framework, to study and discuss the potential specification impact for functionalities and reporting of UE’s applicable conditions.

Table 1 - Potential new methods and capabilities conditions for target UEs based on the functionality framework.

| Functionality/feature/ positioning method | UE’s applicable condition (UE capability report) | Mandatory / optional |
| --- | --- | --- |
| AIML_direct_DL_CIR_ UEside Description: the fingerprint position is estimated in the UE-side when the input is CIR. (Case 1) – Direct AI/ML | Supported N’t: To indicate the N’t values that the UE is capable to consider in AIML_direct_DL_CIR_UEside positioning. N’t = 64, 128, 256, 512. | Mandatory |
|  | Supported N_port: To indicate the N_port is the number of transmit/receive antenna port pairs to consider in AIML_direct_DL_CIR_UEside positioning. N_port = 1,2,4. | Mandatory |
|  | Supported N_TRP: To indicate the number of N_TRP, which is the number of TRPs to consider for AIML_direct_DL_CIR_UEside positioning. N_TRP = 1, 2, …, 72. (32x3) | Optional |
|  | Supported on collecting and labeling dataset for training, updating, and monitoring: Defines the support of the UE to receive data collection assistance from the LMF. See Note 1. | Optional |
|  | Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements for AIML_direct_DL_CIR_UEside. | Optional |
|  | Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift for AIML_direct_DL_CIR_UEside. | Optional |
|  | Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring for AIML_direct_DL_CIR_UEside. | Mandatory |
|  | Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR for AIML_direct_DL_CIR_UEside. | Optional |
|  | Conditions on supporting ML functionalities - Max number of supported functionalities (1, 2, 4, 8, ...): Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE. | Mandatory |
|  | Conditions on supporting ML functionalities - Delay in activating a functionality (2 ms, 4 ms, ...): Indicates the delay required when activating or switching a functionality. | Mandatory |
|  | Conditions on supporting ML functionalities - Generalization condition of functionalities (true, false): Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. | Mandatory |
| AIML_assisted_DL_CIR_ UEside Description: the intermediate feature is estimated in the UE-side when the input is CIR for AI/ML assisted. (Case 1) – AI/ML Assisted | Supported intermediate_feature: To indicate the intermediate feature supported by the AIML_assisted_DL_CIR_ UEside method. - ToA - LOS/NLOS indication - AoA - Selected positioning anchors - ... See Note 2. | Mandatory |
|  | Supported N’t: To indicate the N’t values that the UE is capable to consider in AIML_assisted_DL_CIR_ UEside positioning. N’t = 64, 128, 256, 512. | Mandatory |
|  | Supported N_port: To indicate N_port, which is the number of transmit/receive antenna port pairs to consider in AIML_assisted_DL_CIR_ UEside positioning. N_port = 1,2,4 | Mandatory |
|  | Supported N_TRP: To indicate the number of N_TRP, which is the number of TRPs to consider for AIML_assisted_DL_CIR_ UEside positioning N_TRP = 1, 2, …, 72. (32x3) | Optional |
|  | Supported on collecting and labeling dataset for training, updating, and monitoring: Defines the support of the UE to receive data collection assistance from the LMF for AIML_assisted_DL_CIR_ UEside. See Note 1. | Optional |
|  | Set Conditions -measured DL PRS: Defines support of using PRS based CIR measurements for AIML_assisted_DL_CIR_ UEside. | Optional |
|  | Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements for AIML_assisted_DL_CIR_ UEside. | Optional |
|  | Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift for AIML_assisted_DL_CIR_ UEside. | Optional |
|  | Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring for AIML_assisted_DL_CIR_ UEside. | Mandatory |
|  | Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR for AIML_assisted_DL_CIR_ UEside. | Optional |
|  | Conditions on supporting ML functionalities - Max number of supported functionalities (1, 2, 4, 8, ...): Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE. | Mandatory |
|  | Conditions on supporting ML functionalities - Delay in activating a functionality (2 ms, 4 ms, ...): Indicates the delay required when activating or switching a functionality. | Mandatory |
|  | Conditions on supporting ML functionalities - Generalization condition of functionalities (true, false): Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. | Mandatory |
| AIML_assisted_DL_CIR_ LMFside Description: the fingerprint position is calculated in the LMF-side when the input is CIR for the AI/ML assisted positioning. (Case 2a) – AI/ML Assisted | Supported intermediate_feature: To indicate the intermediate feature supported by the AIML_assisted_DL_CIR_ LMFside. - ToA - LOS/NLOS indication - AoA - Selected positioning anchors - ... | Mandatory |
|  | Supported N’t: To indicate the N’t values that the UE is capable to consider in AIML_assisted_DL_CIR_ LMFside positioning. N’t = 64, 128, 256, 512. | Mandatory |
|  | Supported N_port: To indicate N_port, which is the number of transmit/receive antenna port pairs to consider AIML_assisted_DL_CIR_ LMFside. N_port = 1,2,4 | Optional |
|  | Supported N_TRP: To indicate N_TRP, which is the number of TRPs to consider for AIML_assisted_DL_CIR_ LMFside positioning N_TRP = 1, 2, …, 72. | Optional |
|  | Supported on collecting and labeling dataset for training, updating, and monitoring: Defines the support of the UE to receive data collection assistance from the LMF for AIML_assisted_DL_CIR_ LMFside positioning. See Note 1. | Optional |
|  | Supported anchor indication from the LMF: Indicates whether anchors selection by the LMF is supported for AIML_assisted_DL_CIR_ LMFside positioning. | Optional |
|  | Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements for AIML_assisted_DL_CIR_ LMFside positioning. | Optional |
|  | Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift for AIML_assisted_DL_CIR_ LMFside positioning. | Optional |
|  | Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring for AIML_assisted_DL_CIR_ LMFside positioning. | Mandatory |
|  | Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR for AIML_assisted_DL_CIR_ LMFside. | Optional |
|  | For LOS/NLOS classification - supported ue_channel features: Indicates which UE channel features can be used for determining LOS/NLOS indication for AIML_assisted_DL_CIR_ LMFside. UE_channel_features = {CIR_energy, CIR_maximun_amplitude, CIR RMS delay spread, CIR skewness, CIR skewness, CIR_kurtosis.} | Optional |
|  | Conditions on supporting ML functionalities - Max number of supported functionalities (1, 2, 4, 8, ...): Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE. | Mandatory |
|  | Conditions on supporting ML functionalities - Delay in activating a functionality (2 ms, 4 ms, ...): Indicates the delay required when activating or switching a functionality. | Mandatory |
|  | Conditions on supporting ML functionalities - Generalization condition of functionalities (true, false): Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not. | Mandatory |

Note 1: AIML functionality in Case 1 and Case 2a positioning may need disclosure by the UE if the UE itself requires some assistance from the NW to manage its own AIML models that support said functionality. For example, the UE may need help with: - Collecting and labeling training data: to be used for model tuning. In this case, the UE may indicate to the NW that it wants to trigger training data collection. The NW may assist with configuring DL signals which the UE uses to obtain training data inputs. The NW may also assist with labeling said data inputs. - When to trigger model updates: For example, the UE may indicate to the NW that it wants to update its own AIML assisted positioning and that its training dataset is imbalanced (e.g. UE has 70% of training data labeled as LOS and 30% as NLOS). Thus, whenever the NW detects that the UE may be in NLOS conditions, it may inform the UE that it can start collecting training data, to balance its dataset. In summary, a positioning method and/or capability report (as defined in TS 37.355), may be enhanced with IEs indicating the usage of AIML if such usage is conditioned by some NW assistance information transfer.

The selection of suitable anchors (TRP) is a key step in achieving higher positioning accuracy. In the scope of AI/ML assisted positioning the UE may have capability to perform anchor selection function using an ML model on UE-side. In this case, the intermediate feature could be the indication of selected positioning anchors. This intermediate feature may need to be indicated to the network if it requires some assistance from the NW in managing the corresponding AI/ML assisted positioning method.

Proposal 3: RAN1 to study and discuss the specification impact of selected positioning anchors indication as intermediate feature on AI/ML assisted positioning.

Observation 1: In the scope of functionality identification framework, the new set of methods and their respective UE’s capability conditions can be mapped using as a reference the current LPP framework defined in TS 3GPP 37.355 for AI/ML positioning cases.

Proposal 4: RAN1 to consider at least the following mandatory UE’s applicable conditions on supporting ML functionalities for all AI/ML positioning enhancement cases (Case 1, Case 2, and Case 3):

  - Max number of supported functionalities (1, 2, 4, 8, ...)

Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE

  - Delay in activating a functionality (2 ms, 4 ms, ...)

Indicates the delay required when activating or switching a functionality

  - Generalization condition of functionalities (true, false)

Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not.

Proposal 5: RAN1 to consider at least the following mandatory UE’s applicable conditions for Cases 1 and Case 2a:

  - Supported N’t: To indicate the N’t values that the UE is capable of considering (N’t = 64, 128, 256, 512).

  - Supported N_port: To indicate N_port, which is the number of transmit/receive antenna port pairs to consider (N_port = 1,2,4).

  - Supported N_TRP: To indicate N_TRP, which is the number of TRPs to (N_TRP = 1, 2, …, 72)

Proposal 6: RAN1 to consider at least the following optional UE’s applicable conditions for Cases 1 and Case 2a:

  - Supported on collecting and labeling dataset for training, updating, and monitoring: It defines the support of the UE to receive data collection assistance from the LMF.

  - Supported anchor indication from the LMF: It indicates whether anchors selection by the LMF is supported.

  - Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements.

  - Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift.

  - Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring.

  - Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR.

Proposal 7: RAN1 to consider at least the following mandatory UE’s applicable condition for AI/ML assisted positioning in Case 1 and Case 2a:

  - Supported intermediate_feature: To indicate the intermediate feature (ToA, LOS/NLOS indication, AoA, Selected positioning anchors).

Proposal 8: RAN1 to consider at least the following optional UE’s applicable condition for Case 2a:

  - For LOS/NLOS classification - supported ue_channel features: Indicates which UE channel features can be used for determining LOS/NLOS indication (UE_channel_features = {CIR_energy, CIR_maximun_amplitude, CIR RMS delay spread, CIR skewness, CIR_kurtosis}).

Specification impact: In the agreement on functionality identification of the agenda item 9.2.1 related to AI/ML general aspects, it is indicated a legacy 3GPP framework should be considered as starting point of discussion. Based on this assumption the specification impact is minimized. In the case of positioning, the LPP is the protocol to be reused. However, it is expected to include new positioning methods based on AI/ML and new reporting IEs.

Observation 2: Considering as starting point, the LPP protocol as the 3GPP legacy framework of the functionality identification aspects on AI/ML enhanced positioning cases is beneficial for the current study item. Besides that, it is expected to include new positioning methods based on AI/ML and new reporting IEs.

# Data Collection

During the 3GPP meeting RAN1#112, there has been agreements made in the context of training data generation and collection. Furthermore, data collection related to model monitoring has been also discuss. In this section, we will continue discuss data collection and highlight the common and use-case specific challenges.

Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection E.g., requested quality of training data

## Common Challenges for Data Collection (Applicable to all use cases)

Data collection is a lengthy and computationally expensive process. The process includes generation of data samples, entity generating data samples, mechanisms, and transfer of generated data samples. In the current functionality identification framework, we prioritize data collection for model updating (re-tunning or fine tuning) and monitoring can. However, the aspects related to model training may not be precluded.

Proposal 9: RAN1 to deprioritize the study and discussion of the specification impact of data collection for model training for the current functionality identification framework.

Proposal 10: RAN1 to prioritize the study and discussion of the specification impact of data collection for model update/re-tunning and performance monitoring considering legacy 3GPP framework for positioning.

For model updating and monitoring, each sample along with ground truth label is required. Based on the RAN1#112 meeting and before agreements, there are multiple aspects related to data collection which are not limited to the following list:

  - Entity and mechanism to generate ground truth labels

  - Applicable conditions to generate the ground truth label

  - Other training data (measurement corresponding to the model inputs)

  - Associate information of training data such as quality indicator for ground truth, quality of training data

  - Transfer of training data between different network entities (UE, gNB and LMF)

  - Assistance signaling and procedures to facilitate generation/collection of training data

The previous listed aspects suffer issues related to data availability, noisy ground truth labels, data availability in the presence of abnormal propagation conditions, and RF imperfection. These issues are discussed in the following subsections highlighting potential specification impacts in each of them.

Proposal 11: For data collection, RAN1 to study and discuss the potential specification impact of at least the following challenges: data availability, noise ground truth, presence of abnormal propagation conditions, and RF imperfection. Reusing as much as possible the legacy 3GPP framework on positioning.

### Data availability

Limited dataset: The availability of required data density for model training and testing/validation is one of the key challenges in machine learning, especially in the context of positioning, where obtaining the ground truth labels in terms of UE location, LOS / NLOS condition, etc., is challenging. To overcome the limited dataset availability in real world conditions, additional dataset can be generated using simulation framework to reach a combined training dataset with targeted density (assuming grid based or uniform distribution).

As discussed in Sec. 3.1.3 of [3], there are various techniques, such as data augmentation, that could be applied in such scenarios to ensure sufficient model performance.

Proposal 12: For ground truth labels, RAN1 to study and discuss the potential specification impact of at least the UE distribution used for model updating and performance monitoring for positioning cases. Reusing as much as possible the legacy 3GPP framework on positioning.

### Noisy labels

For machine learning, especially in the context of positioning, it is challenging to derive ground truth or the correct/ complete labeling information without manual intervention. For instance, consider that a data sample is made from measurements of NR signals and their corresponding label, consisting of the position of the target device at the time of measurement. For each of these samples, it may be possible to obtain noisy labels (inaccurate position) from several sources like GNSS, RAT, etc. When this is the case, samples with one or several sources of noisy labels should be pre-evaluated before transmission and collection of new data samples either for model updating and model (re)-training.  The pre-evaluation metric may depend on the estimation variance of the UE position, so the samples with very poor quality (highly inaccurate position) are discarded.

Observation 3: Regarding data collection, the noisy label evaluation can be assisted by the LMF. Samples with one or several sources of noisy labels should be pre-evaluated as part of data collection for model updating and model retraining.

Furthermore, the consistency of the obtained labels changes the decision to accept or reject the sample where there is opportunity to label a sample with multiple positioning sources. To enable the sample evaluation process, a label consistency score (LCS) may be defined by the LMF. The selected LCS metric may have the following properties:

- Providing higher LCS value when positioning estimation(s) has/have higher accuracy.

- In the case of consistency between estimated positions from various sources, LCS values increase proportionally to the number of positioning sources.

Additionally, a threshold for LCS may be signaled by the LMF to enable the sample rejection process. Such threshold may be defined based on the network requirement, e.g., target accuracy, or ML-related parameters, e.g., current collected training dataset size.

One important aspect of successful model updating is the processing of noisy ground truth label(s). For samples with only one labeling source, we may use the variance of position estimation weight in the effects of each sample on the training results.

Observation 4: In case of the opportunity to label a sample with multiple positioning sources, one may exploit all the information from different positioning sources and incorporate them in the labels for training the ML model.

Proposal 13: For UE-sided model (Case 1 and Case 2a), RAN1 to study solutions and their respective specification impact, as part of data collection, using network driven evaluation to verify the consistency of labels.

### Abnormal propagation conditions

Lifecycle management of a positioning ML model requires periodic model updates, where the periodicity depends on the following:

- The inference frequency, i.e., how often the model is used after deployment.

- The type of UE that uses the model because different UEs have different capabilities to collect data and/or update the model accordingly.

- The positioning task that the model is solving, e.g., direct positioning AI/ML or AI/ML assisted positioning (LOS detection, location estimation, etc.).

- The availability of data, i.e., how much, how diverse, and how often data can be collected.

- The reliability of the data, i.e., if the data is characterizing well the conditions for which the ML model is being updated.

While items 1-3 are intrinsic to the ML function and/or UE type and thus relatively controllable, items 4-5 are extrinsic and typically depend on the propagation and interference conditions in which the UE finds itself.  Thus, if only items 1-3 are accounted for when managing the ML model update, while items 4 and 5 are disregarded, the ML model may be updated to learn a set of peculiar/rare channel conditions that do not represent well the average daily conditions. In other words, the model update is done based on a situation that does not represent typical wireless conditions, but an anomaly, leading to an overall unrealistic model that does not generalize well. As a consequence, the ML model lost generalization capabilities on how to deal with average conditions and exhibits poor performance in most cases. This phenomenon is called catastrophic forgetting, and it is one of the caveats of continual learning/updating of ML functions.

Exemplary rare conditions that may lead to a catastrophic forgetting of a positioning ML function are linked to the cases in which a positioning link towards a set of TRPs is temporarily blocked or severely attenuated due to an anomaly in the wireless channel. Such anomalies often occur due to very high speeds and/or the presence of structures that create a Faraday cage/shield around either the positioning receiver or transmitter, like buildings covered in metallic scaffolding, large structures associated with construction sites, outdoor concerts, road work, etc.

To manage rare events occurrences during data collection, the following aspects may be necessary:

- Defining conditions to identify different types of positioning rare events relevant for an ML-positioning function, where such function is either LOS-detection or location estimation.

- Defining actions to manage each type of rare event by the UE, e.g., whether to discard the measurement, whether to use the measurement to update the model or roll-back a previous model, etc.

Observation 5: Any given positioning measurement that does not represent well the expected distribution for a set of TRPs relative to the UE location may be seen as abnormal/rare events. Nevertheless, some rare events may prove useful to the ML positioning function, while others may be entirely detrimental.

Proposal 14: RAN1 to define means on identification and management of abnormal propagation conditions during data collection and the potential specification impact (e.g., define conditions to identify abnormal propagation conditions, define actions to manage such as whether to discard associated measurement, etc.).

### RF imperfections

It is well known that RF imperfections introduce various distortions (e.g., carrier frequency offset, sampling time offset, TX/RX beam offsets, clock offsets and drifts, phase noise, etc.) to the received signal. In data communications, these errors are estimated with the network support (e.g., by frequency, and time offsets are compensated for during synchronization). However, that support is not available in positioning, and as a result, the distortions are absorbed in the total positioning error. These distortions typically depend on the hardware limitations of the different antenna configurations and form factors such as ADC resolutions and crystal oscillators.

Observation 6: Because of RF limitations, some impairments are generated, such as phase rotation and delays of the positioning signal by the RF chain at the baseband receiver. As a result, a positioning entity (UE, TRP, etc.) hosting the ML positioning function could experience certain RF-based signal distortions which are not considered explicitly or characterized and compensated for updating the model.

Observation 7: RF-based signal imperfections are different between host-type devices. For example, a PRU or gNB hosting the ML model would require adapting the model to its RF-specific characteristics.

Therefore, there is a need for a framework through which the generic ML positioning model is customized to the specific NR elements host types. Specifically, before the ML model is deployed on a large scale, it is refined (updated) based on at least intrinsic characteristics (e.g., RF limitations) of the NR element types, including target UE, PRU, or gNBs.

One way of enabling the above scenario would be that LMF selects a head NR unit as representative for a given NR element type and thus for a given expected intrinsic distortion range. Next, the model is customized and refined with the help of these head units to compensate the element-specific distortion.

Proposal 15: RAN1 to study the specification impact on using the existing positioning framework, through which a generic ML positioning model can be customized to the specific NR elements host types - including target UE, PRU, or gNBs and their RF chain imperfections.

## Use-case specific specification aspects for data collection

### Case 1

For data collection, the collected data samples contain the channel observations such as CIR, PDP with corresponding ground truth labels. The ground truth is collected using the PRU and UE with known locations. It is worth to note that the data samples with respective ground truth (GT) may be noisy therefore associated information such as quality indicator and related information such as scenario, cluster density, UE spatial distributions, PRS configurations, validity conditions etc. may be included as well. The metric to estimated inter-point distance (IPD discussed in evaluation contribution R1-2302632) may be utilized as quality indicator to keep the quality of training samples and ground truth. The mechanism to generate ground truth may utilize PRU with known location, for UE-based and UE-assisted positioning RAT-depended positioning methods can be used for the generation of ground truth or UE utilized Non-NR methods such as GPS and GNSS. In case of specs impact network may assist the UE with label consistency and provide configurations related to IPD configuration to perform data collection. Furthermore, to avoid catastrophic forgetting problem, abnormal radio propagation conditions need to be identified and managed. To this end, conditions to identify different positioning rare events that impacts ML-positioning are to be defined. Also, actions (discard the measurement, roll-back a previous model, etc.) to manage such events are to be defined.

### Case 2a

Case 2a is similar to Case 1 but the only difference is that it focuses on UE-assisted based positioning (UE-side model), in this case the model output new measurement e.g., ToA, path phase and existing measurement report e.g., RSTD, LOS/NLOS indicator, RSRPP. The label can be timing measurement ToA, angle measurement and other features such as LOS/NLOS, RSTD. The label calculation may follow the same approach as Case 1.

### Case 2b

In case 2b, the model resides at the LMF side. The model input may utilize the new measurements such CIR/PDP and existing measurement e.g., RSRP/RSRPP/RSTD and output the location estimation. For data collection phase multiple possibilities can be considered for the samples and corresponding ground truth label extraction. LMF may generate location (i.e., GT) based on the NR-RAT dependent positioning method. In other option UE with support of non-NR positioning method may generate location (GT) and transmit them to LMF.

### Case 3a

In case 3a, the model resides at the gNB-side and AIML assisted positioning is followed. The model input the uplink channel observations (e.g., CIR, PDP) and output the intermediate features (ToA, LOS/NLOS, etc). LMF or TRP calculate the ground truth (intermediate features) by known location of TRP and the PRU. The gNB utilize the GT provided by the LMF. The label can be timing measurement ToA, angle measurement and other features such as LOS/NLOS, RSTD.

### Case 3b

In case 3b, the model resides on the LMF side and AIML direct positioning is followed. The model input the uplink channel observations (e.g., CIR, PDP), timing and angle measurements or existing measurements such as RSRP/RSRPP/RSTD obtained from TRPs. The output of the model is the direct location estimations. LMF may utilize the NR-RAT dependent UL positioning methods to obtain the ground truth (i.e., location) or UE generate location based on the non-NR positioning methods and transfer the label data to LMF.

A summary of cases and data collection conditions, considering quality indicators, mechanisms to generate ground truth, and potential specification impact are summarized in Table 2.

Table 2 - List of positioning use cases for data collection and its specific specification impact.

| Cases | Model input/output | Entity providing Ground Truth (GT) | Data collection conditions | Quality Indicator | Mechanism to generate GT | Specification impact |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 UE-based positioning with UE-side model | (Direct AI/ML positioning) Input: CIR/PDP or existing measurements Output: UE position | PRU/UE | * Scenario (e.g. clutter density, frequency) * UE spatial distribution (IPD condition) * PRS configuration * Validity conditions (e.g., Area) * Abnormal propagation condition (rare event condition) | * Estimated IPD * Abnormal propagation (rare event) identifier | Non-NR positioning methods such as GPS or GNSS & NR-RAT dependent positioning method (e.g., DL-TDOA). | - NW to indicate to UE requirements & parameters e.g. IPD threshold, data augmentation configuration - NW assistance to verify label inconsistency - NW to assist UE in identifying/managing abnormal propagation conditions |
|  | (AI/ML Assisted Positioning) Input: CIR/PDP or existing measurements Output: Intermediate feature |  |  | * Estimated IPD * Abnormal propagation (rare event) identifier | Intermediate features estimated for a set of inputs (selected DL PRS measurements) |  |
| Case 2a UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning | Input: DL PRS measurements and output: Intermediate feature | PRU/UE | * Scenario (e.g. clutter density, frequency) * UE spatial distribution (IPD condition * PRS configuration *Quality indicator Validity conditions (e.g., Area) *Abnormal propagation condition (rare event condition) | * Abnormal propagation (rare event) identifier | Intermediate features estimated for a set of inputs (selected DL PRS measurements) | - NW to select PRS configuration - NW to indicate to UE requirements & parameters e.g. IPD threshold, data augmentation configuration - NW to assist UE in identifying/managing abnormal propagation conditions |
| Case 2b UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning | Input: Intermediate features and output: UE position | LMF or UE/PRU generate the GT | * Scenario (e.g. clutter density, frequency) * UE spatial distribution (IPD condition *Quality indicator * Validity conditions (e.g., Area) *Abnormal propagation condition (rare event condition) | * Abnormal propagation (rare event) identifier | Intermediate features estimated for specific positions (Non-NR positioning methods such as GPS or GNSS & NR-RAT dependent positioning method (e.g., DL-TDOA)) | - NW to select PRS configuration - NW to indicate to UE requirements & parameters e.g. IPD threshold, data augmentation configuration - NW to assist UE in identifying/managing abnormal propagation conditions |
| Case 3a NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning | Input: UL SRS measurements and output: Intermediate features | LMF/TRP generate the GT by known location of TRP and PRU | * Scenario (e.g. clutter density, frequency) * UE spatial distribution (IPD condition * UL SRS configuration *Quality indicator *Validity conditions (e.g., Area) | * Abnormal propagation (rare event) identifier | Intermediate features estimated for a set of inputs (selected UL SRS measurements | - gNB to select UL SRS configuration - LMF to indicate to gNB requirements & parameters e.g. IPD threshold, data augmentation configuration |
| Case 3b NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning | Input: Intermediate features & Output : UE position | PRU/UE & gNB | * Scenario (e.g. clutter density, frequency) * UE spatial distribution (IPD condition *Quality indicator *Validity conditions (e.g., Area) | * Estimated IPD * Abnormal propagation (rare event) identifier | Intermediate features estimated for specific positions (Non-NR positioning methods such as GPS or GNSS & NR-RAT dependent positioning method (e.g., UL-TDOA)) | - gNB to select UL SRS configuration - LMF to indicate to gNB requirements & parameters e.g. IPD threshold, data augmentation configuration |

Proposal 16: For Case 1, RAN1 to study the specification impact of the NW indication to UE requirements and parameters related to data collection (e.g., IPD threshold, data augmentation configuration, abnormal propagation condition). Reusing as much as possible the legacy 3GPP framework on positioning.

Proposal 17: For Case 2a and Case 2b, RAN1 to study the specification impact of NW selecting a set of PRS configurations related to data collection.

Proposal 18: For Case 3a and Case 3b, RAN1 to study the specification impact of gNB selecting a set of SRS configurations related to data collection.

# Performance Monitoring

To be aligned with the current agreements in the agenda item 9.2.1 related to AI/ML general aspects, the term model monitoring is generalized with the term performance monitoring based on the 3GPP legacy framework, which is represented by the LPP in AI/ML positioning enhancement. The performance monitoring indicates the fault detection or fault diagnosis performance of the deployed model. As a result of such indication action, other LCM steps are triggered, such as model activation, deactivation, updating, switching and fallback. In this section, performance monitoring includes aspects related to model input and output, calculation of monitoring metric and decision making depending on each of the use-case.

Two approaches have been agreed in RAN1-112 meeting for model monitoring with presence of ground truth and non-existence of ground truth.

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded

To overcome the challenges on functionalities aided AI/ML, the network may collect monitoring data to evaluate the performance of the AI/ML model in each functionality or method. In turn, depending on the outcome, the network may update, re-select or deactivate the AI/ML model. For this purpose, the required data or labeled data could be made available at the entity using the AI/ML model via network assistance.

The performance monitoring would also depend on whether the model is used for direct AI/ML positioning or assisted AI/ML positioning. For example, in assisted AI/ML positioning, to improve the training, the intermediate feature extraction needs to be tested/validated. Such validation may be done using specific "labeled data". The "labels" represent the ground truth feature, such as true LOS/NLOS classification or true CIR. Since data labeling is not for free, it typically requires external gears/devices that support such function.

The main candidate device to support the above could be the positioning reference unit (PRU) introduced and discussed in TS 38.305. The PRU is intrinsically suitable to accommodate real-world measurement and can provide labels for AI/ML models. For example, network assistance can be used to provide the PRU positioning measurement report and their associated true labels to monitor the given AI/ML model without knowing the details of the model at the UE. Moreover, the PRU positioning measurement and their corresponding labeled ground truth can also be used to fine-tune the AI/ML model at the UE-side in cases 1 and 2a.

Proposal 19: Regarding performance monitoring, RAN1 to study the specification impact of using PRU through LPP protocol.

## Generic monitoring mechanism

A generic mechanism for model performance monitoring is shown in Figure 8. Here, we consider the scenario where a first NR entity (Entity 1) is the node responsible for model training and inference (e.g., UE in for case 1 and case 2a). Whenever Entity detects changes in the environment, it may request to other Entity 2 (e.g., LMF) to share relevant data for model monitoring. The Entity 2 could then collect the data from various "trusted" entities as Entity 3 (e.g., PRU) and share it with the Entity 1. The NR entity could compute and report model monitoring metrics to Entity 2, where the details of the metric could depend on whether a direct or AI/ML-assisted positioning method is being implemented. Based on the reported model monitoring performance metric, the Entity 2 could trigger actions such as fine-tuning or updating the model, model deactivation, or switching to classical positioning methods, etc.

Figure 8 - Mechanism to monitor model performance for cases 1 and 2A. Similar mechanism may be considered for cases 2B and 3.

Proposal 20: For case 1 and case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contains monitoring data for AI/ML model performance (including samples with corresponding labels).

Figure 9- Monitoring metric estimation

In Figure 9, for both direct and AI/ML-assisted positioning method, the model monitoring could be realized using monitoring metric which can be estimated simultaneously with the UE position or intermediate feature for direct AI/ML and AI/ML assisted case respectively. To this end, the AI/ML model should be designed and trained to provide this additional output.

The performance monitoring on AI/ML models may be triggered by the following events:

- Insufficient positioning accuracy

- Positioning measurements statistically different than those used for model training

- Environment changes e.g., change in maximum Doppler shift, etc.

Observation 8: Performance monitoring may be done proactively (e.g., on a periodic basis), or reactively (over demand basis). Any entity (the UE or the LMF) that detects an environment change may trigger a reactive performance monitoring.

Observations 9: A new measurement report such as monitoring metric can be used to switch the positioning functionalities (direct AIML, AI/ML assisted). The monitoring metric can be considered as an associated information to the given inference output.

Proposal 21: RAN1 to further study and discuss the possible specification impact considering additional model outputs, which can deliver new monitoring metric to trigger model updating/switching.

## Case specific monitoring mechanism

In the following, we provide specific discussion on each AI/ML positioning case.

### Case 1

Case 1 – direct AIML positioning:

In case of direct AI/ML positioning, the UE may monitor the performance of the model using network assistance data, where such assistance data may come as:

- Labeled test data: This data set may be collected from PRUs and may be labeled with the PRU location.

- Unlabeled data: This type of data refers to a statistical characterization of the model input and/or model output which may be provided by the NW. The UE may assess whether the input/output of the model under test obeys the statistics reported by the NW.

Case 1 – assisted AI/ML positioning

In the case of assisted AIML positioning, the UE may request assistance about assessing the validity/accuracy of the intermediate features which the UE extracts from the DL PRS e.g., LOS indication, CIR estimate, etc.

Proposal 22: For Case 1, RAN1 to consider the NW assistance for performance monitoring using both labeled and unlabeled data. Such data may be collected from PRUs and/or other UEs (in case of unlabeled data) by the NW.

### Case 2a

In this AI/ML assisted case, there is specification impact when the UE monitors the performance of the intermediate feature using network assistance data.

Observation 10: In Case 2a, depending on the type of intermediate feature reported to the LMF, the UE may need specific network assistance. The assistance depends on the type of intermediate feature (e.g., range/distribution).

Proposal 23: For Case 2a, RAN1 to study and discuss at least potential specification impact when labelled data is used for monitoring. The labels are the intermediate features, which are extracted by the PRU. PRU data is collected by the NW, potentially cleaned, and then transferred to the target UE.

Proposal 24: For case 1, case 2a (UE-side model) considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contain data for monitoring the AI/ML model performance (including samples with corresponding labels).

### Case 2b

In case 2B, the UE is sending positioning measurements to the LMF, and the LMF applies direct AIML positioning to obtain the UE location. To monitor its own model, the LMF may use, similarly to the other cases, data collected from the PRUs.

### Case 3a

In the case 3a, the gNB may request assistance about assessing the validity/accuracy of the intermediate features which the gNB extracts from the UL SRS using AIML e.g., LOS indication, CIR estimate, etc.

Observation 11: NW may assist performance monitoring using both labelled and unlabelled test data for case 3a using NRPPa interface. The NW may clean the test data prior to sending it to the gNB via NRPPa.

### Case 3b

In case 3B, the gNB is sending positioning measurements to the LMF, and the LMF applies direct AIML positioning to obtain the UE location.

Observation 12: For case 3b, when the performance monitoring is triggered, the LMF may collect monitoring data from selected gNBs using the existing positioning legacy 3GPP protocols.

## Further Specification Impact

### Output-based model monitoring

Output-based monitoring may be realized based on different monitoring levels. The first one is the monitoring level 0 in which the UE/gNB performs the following actions:

- Performs continuous self-evaluation of the AI/ML model output parameter.

- In case the self-evaluation achieves a specific condition, the UE/gNB requests network guidance. For example, in LOS/NLOS classification, the AI/ML model output monitoring can be complemented with PRU information provided by network to the UE.

- Execute the life cycle management (LCM) procedure (update/fine-tuning/switching) on the AI/ML model used in the UE/gNB as a consequence of the AI/ML model output monitoring. The LCM action is defined by the LCM management system, which resides in the NW/gNB.

If monitoring level 0 achieves certain conditions, it triggers other levels, such as monitoring level 1 and monitoring level 2, which represent two different conditions to monitor the AI/ML model output.

In addition to the monitoring levels described above, assistance of the PRU in monitoring level 2 to supply complimentary information to the output model validation may be an option. If a PRU is in the range coverage of the serving cell, the monitored model is delivered to the PRU to compare the intermediate metric output with the ground true value of the same metric on the PRU.

Proposal 25: For Case 1 AI/ML direct and assisted AI/ML positioning, RAN1 to study the potential specification impact when UE triggers a request to the NW based on its self-evaluation of the model performance (e.g., model output related performance). This request can be also used to trigger any LCM step (enable model updating within a functionality, functionality switching) and may be set over a legacy 3GPP framework.

Observation 13: Output-based performance monitoring for cases 2b, 3b can be implemented autonomously by the LMF and does not require standardization.

Observation 14: Output-based performance monitoring for case 3a may follow proposal 25 the difference being that the request is sent over a different legacy 3GPP framework (NRPPa).

A generic illustration of the self-evaluation for both cases (direct AI/ML positioning and AI/ML assisted positioning) is shared in Figure 10. Here are highlighted the Memory buffer block, the statistics block, the latest sample indication, and the IF conditional block.

Figure 10 - Illustration of the self-evaluation of the output model for both scenarios (Direct and Assisted positioning with AI/ML models). Here, the buffer memory considers the N samples, which are used to obtain a certain statistic metric. This statistic metric is compared with the latest sample to trigger or not based on a specific condition (IF condition).

# Model Inference

## Case specific model inference mechanisms

### Case 1

The input for model inference could be an already standardized measurement, as RSRP. However, in this study item new measurements are candidates to be used such as CIR, PDP. For direct AI/ML and AI/ML assisted positioning there is no relevant specification impact. However, a new IE and methods should be included in the LPP to support at least the estimated positioning report.

### Case 2a

For inference purposes, the UE report the intermediate feature already estimated with an AI/ML model in the UE-side. The LMF receives this report to use it as input of a legacy methodology to estimate the position. In terms of specification impact, it is expected new reporting IEs of the intermediate feature using LPP.

### Case 2b

The position inference is done in the LMF using the Direct AI/ML positioning and the measurement entity is the UE. Therefore, it is expected a specification impact when the UE reports the measurement to the LMF by LPP. At least new IEs should be defined.

### Case 3a

The model inference that obtains the intermediate feature for AI/ML assisted positioning is estimated in the gNB and reported to the LMF by NRPPa protocol. As previous cases, it is expected at least new IEs supporting the inference output.

### Case 3b

The model inference that obtains the horizontal positioning for AI/ML assisted positioning is estimated in LMF. The inference is done using the measurements reported by the gNB to the LMF by the NRPPa protocol. For this inference scenario, it is expected to use at least the LMF to report the measurements (e.g., CIR, PDP, RSRP, etc.).

Observation 15: The existing legacy 3GPP framework may be extended with possible new methods and IEs to support and report the inference output of AI/ML positioning enhancement.

## Further Specification Impact

### Model Input/Output for AI/ML-assisted positioning (Cases 1, 2a, and 3a)

#### Model input type and size aspects

For one-sided AIML positioning models, RAN1 discusses using CIR and PDP measurements as input parameters. Both model inputs require signaling measurements with high dimensionality (which scale linearly with the number of receive antennas, sampling resolution, and a certain number of PRBs), with potentially high reporting overhead. This would imply that with these model inputs, the network would need to collect measurements over significant periods for different PRS bandwidths/carriers and for UEs with a variable number of RX antennas to create a sufficiently diverse training dataset, as well as a systematic collection of CIR/PDP measurements from the UE for model inference – in case of LMF/network-based model inference. Such considerations make it necessary to investigate models that could enable vendor-agnostic training data collection with minimal overhead.

Observation 16: The solution approaches presented so far as part of this study have considered only one-sided models, with the AI/ML model output directly indicating the UE location or providing intermediate features that are used by classical positioning approaches to estimate the UE location. However, for some cases using CIR and PDP with high dimensionality as model input could cause significant overhead for data collection for model training and inference. It is also unclear how an AI/ML model for positioning can cope with variable CIR/PDP size and shape.

Proposal 26: RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples.

Furthermore, positioning reference signals may be aggregated over multiple carriers, where each aggregated carrier signal may occupy a different bandwidth, implement a different comb, have a different duration, etc. Hence, a positioning signal can occupy various resource elements from one session to the other, and a machine learning positioning module should be able to cope with this variable signal size. To ensure the above, the NW may request positioning measurements (for training, inference, testing, etc.) from each UE/TRP in a selected fixed format (size and type) which is agnostic to the size and shape of the positioning reference signals.

In one example, this may be realized by projecting the received signal onto a space indicated by, e.g., the LMF. Specifically, the LMF transfers to the UE a projector/basis and requires the UE to return the results of projecting each received signal onto the selected basis.

In another example, a similar approach to that used in the CSI feedback use case may be applied where the UE/TRP is instructed to collect P/SRS samples and encode them in a code H to be used on the NW side. Generating code H may be seen as compressing the received signal, while using code H by the LMF may be equivalent to expand the code H.

Furthermore, when the AIML positioning model resides in the positioning receiver (e.g., UE or TRP), the latter should be able to run the AIML model no matter the size and shape of the received signal.

Observation 17: A positioning reference signal receiver (e.g., UE or TRP) that uses AI/ML positioning (direct or assisted) must ensure that the input to the AI/ML block has fixed format characteristics (e.g., size and shape), regardless of the size and shape of the received positioning reference signal.

Proposal 27: RAN1 to study solutions that enable the UE to report positioning measurements in a fixed format characteristics (e.g., size and shape) independent of the PRS configuration, including what type of assistance the LMF may provide for enabling the fixed format.

#### Channel Classification for Positioning

As introduced in Section 2, AI/ML-assisted positioning can be used to determine various intermediate features to be used for calculating the positioning estimate. One of such intermediate features can indicate the type of the channel such as LOS/NLOS, on which the measurements are conducted. Based on this feature, the associated measurement can be then evaluated to consider whether/how to be incorporated in the positioning calculation.

However, the optimal selection of input, i.e., channel features, to be used for the channel classification task by the ML model is highly dependent on the environment setting and the available bandwidth of the channel for the measurements. The radio environment can be very diverse among different scenarios, such as in terms of the objects' size, distribution, and material or blockers leading to differences in penetration, reflection, and diffraction of radio signals. Furthermore, available bandwidth, e.g., depending on the capability of the UE, significantly impacts the channel measurements by determining their time resolution and further affecting the features extracted from these measurements.

To better illustrate the idea, let's refer to Figure 8 and Figure 9. Here, the RMS delay spread is a distinguishing channel feature for LOS/NLOS classification when 500 MHz of BW is available in Scenario 1 (Figure 8). At the same time, its distinguishing ability is degraded for 50 MHz or lower BW in Scenario 1. On the other hand, in Scenario 2 (Figure 9), the RMS delay spread is not a good feature for classification, even for 500 MHz of available bandwidth.

Figure 8 - Distribution of RMS delay spread for LOS vs NLOS measurements with different available bandwidths in Scenario 1.

Figure 9 - Distribution of RMS delay spread for LOS vs. NLOS measurements with different available bandwidths in Scenario 2.

Observation 18: The performance of LOS/NLOS classification using AI/ML has a strong dependency on the environmental setting as well as the bandwidth capabilities of the UE.

To mitigate the impact of diverse environmental settings and bandwidth capabilities on the effectiveness of AI/ML-based NLOS classification, the network may adapt the channel features used for the classification based on the identified conditions and capabilities. That is, the network can determine a set of features associated with a given setting (e.g., a specific set of cells) and the bandwidth reported by the UE that is best suitable for LOS/NLOS classification purposes using AI/ML models. For example, the network could signal to the UE that a specific set of features (e.g., Kurtosis, RMS delay spread, or mean excess delay) should be used for the classification.

Nevertheless, environmental conditions may change during a positioning session, meaning that the proper set of channel features should also be updated. In such case, to avoid additional signaling concerning the channel feature updates, the network may provide to the UE not just a single set of channel features but rather a superset (i.e., a list) of channel feature sets. This list of feature sets should apply to multiple environmental scenarios (e.g., based on the cell in which the UE is residing) and the UE bandwidth capabilities and thus may be ranked as such. By obtaining such a ranked feature set list, the UE may be configured in advance with the set of channel features to be used for AI/ML LOS indication.

Specification impact: For the UE-sided models, the outcome of the LOS/NLOS classification at the UE side can be either utilized by the UE itself in the case of UE-based positioning (i.e., Case 1), or it could be reported to the LMF in the case of UE-assisted/LMF-based positioning (i.e., Case 2a). Similarly, if the model is at the gNB side (i.e., Case 3a), the classification of UL channels as LOS/NLOS, are to be indicated to LMF for AI/ML-assisted positioning.

In Case 1 and Case 2a, the LMF can provide the ranked list of channel features to UE for channel classification purposes. For this, LPP protocol can be extended to introduce new content of assistance data.

Similarly in Case 3a, the LMF can provide the assistance data providing the ranked list of channel features to the Gnb, which can be enabled by extending the NRPPa protocol.

Proposal 28: For Case1, Case 2a, and Case 3a, the network should be able to assist UE and/or gNB for LOS/NLOS classification by providing a ranked list of channel features such as containing CIR characteristics, RMS delay spread, etc. The ranked list should be based on the UE and/or gNB capabilities in terms of bandwidth, computation, and environmental setting.

### Anchor Selection for Positioning

Yet another challenge in positioning is the selection of positioning anchors (i.e., the nodes that will transmit and/or receive reference signals for positioning) to do positioning measurements. Various factors related to anchors have a direct impact on positioning accuracy. These include the channel quality between the target UE and the anchors (e.g., LOS/NLOS, SINR), geometric arrangement of the anchors with respect to each other and the target UE, the confidence of the anchor locations, relative distance and/or speed between the anchors and the target UE. The anchor selection problem is further complicated when different types of anchors such as gNB/TRP, GNSS, RSU, PRU, or another UE (in case of SL positioning) with different mobility and channel characteristics appear and disappear from the UE surroundings over time.

Common approaches to solve the anchor selection problem rely on various channel metrics between the UEs and anchors, such as LOS/NLOS classification, ToA, etc. Based on such metrics, anchors are then (not) selected if they (do not) satisfy specific criteria, e.g., a channel metric value below/above a threshold. However, such approaches might become inefficient under mobile conditions, where the thresholds must be dynamically adjusted, e.g., according to the varying channel conditions.

To help make efficient decisions on anchor selection under dynamically mobile conditions with a multitude of candidate anchors available, ML methods could be utilized instead.

Specification impact: Considering different positioning cases, the ML model selecting the anchors may reside either at the i) UE, ii) gNB, or iii) LMF side. Among these options, a gNB-sided model (as in Case 3a) would be restrictive considering that the UE may be mobile. An LMF-sided model would have access to more knowledge in general, e.g., the location of the surrounding anchors, whereas a UE-sided model would be able to work also outside the network coverage and may enable a faster operation. In any case, the model input requires data relating to the properties of potential anchors (e.g., their location, link conditions, etc.). Further, to train the model, a signal indicative of the positioning QoS resulting from the selected anchor, such as in terms of positioning accuracy, is necessary.

If the model selecting the anchor(s) resides at the UE, the output of the model can be used for UE-based positioning (i.e., Case 1) and LMF-based positioning (i.e., Case 2a). If LMF is involved, it would request UE to determine and propose candidate anchors for positioning by providing necessary assistance data (including any potential list of anchors). To train the model, the UE would request data from the network, such as indicative of the positioning accuracy resulting from the anchor selection.

Proposal 29: In Case 1 and Case 2a, a UE-sided model may conduct anchor selection for positioning. For this, LMF may request UE to do anchor selection, and provide any necessary assistance data (e.g., containing a list of candidate anchors with their locations). To train the ML model, UE may request information from LMF on the positioning QoS resulting from the selected anchor(s).

# Other aspects

## Intermediate feature reporting aspects

### CIR/PDP reporting

CIR/PDP reports assume that multipath component indexed k has a corresponding delay d[k] = kT, where T is the sampling time of the system. However, it is rarely the case that the first Nt CIR components are all non-negligible. In other words, it is rarely the case that the reported (non-negligible) CIR component “k” is associated with delay kT. Therefore, in addition to the NTRP * Nport * Nt  CIR/PDP report, an associated delay vector may need to be reported, so only under these conditions a CIR component is fully characterized by a gain and a delay value.

Thus, it is likely that the CIR/PDP positioning reports become unprecedently large, which is especially problematic when the UE sends the reports i.e., in cases 1 – assisted, cases 2. Because of these reasons, such reports may require quantization as illustrated in Figure 11. As of today, there is no agreement in RAN on how to quantize these entries and how to exploit the propagation channel properties in quantizing these entries so that the quantization loss is minimized.

Observation: Assuming, the example from the figure below, the LMF may assist with defining a quantization strategy tailored to both:

- the UE capabilities (e.g., sampling resolution, number of RF chains)

- the predicted propagation channel characteristics (e.g., based on past PRS measurements and/or localization accuracies).

Figure 11: CIR/PDP extraction and quantization in the UE.

The LMF may configure a type of quantization e.g., scalar/vector and define the quantization codebook which the UE may use to quantize the CIR/PDP in cases 1 and 2. Such configuration should be dependent on the UE capabilities, where such capabilities may include explicit quantization-related UE limitations.

For example, Channel response vectors that UE extracts across its RX antenna ports are known to exhibit

- spatial correlation (inversely proportional to the antenna separation) and

- delay correlation as a consequence of imperfect bandpass, non-linearity added by the gNB’s amplifier transmitter chain, filters and limited sampling resolution i.e., clusters of estimated taps in delay domain are highly correlated as they, in fact, correspond to a single propagation path.

See Figure 12 for an exemplary depiction of the above the true propagation channel which consists of 4 dominant paths, is resolved by the UE into 4 clusters of taps at each of its 2 RX antenna ports. The taps within the cluster are heavily corelated. Similarly, the clusters are also correlated across RX antenna ports.

Figure 12: Exemplary CIR samples accross RX ports.

In sum, the estimated CIR samples exhibit both intra- and inter- cluster correlation. It becomes thus apparent that a quantization strategy that exploits:

- Property 1: the intra-inter-cluster correlation, and

- Property 2: the cluster size in delay domain (in essence the expected number of reflections and their separation in delay domain) is beneficial to minimize both the quantization loss and the associated CIR reporting overhead.

Property 1 is intrinsic to the UE (e.g., antenna separation, sampling rate, CIR estimator capability) and Property 2 is extrinsic to the UE (e.g., how far the clusters are separated in delay domain, the number of clusters). Therefore, prior to selecting the best quantization strategy, the LMF would benefit from learning:

- What are the above UE capabilities?. This request may be resolved with existing LPP signalling, by adding new IE, and;

- What is the expected channel profile?, for instance, urban, sub-urban, indoor, etc. This information may be learned from the serving gNB of the target UE, or based on the most recent LPP session outcome, etc. (as part of the current LPP initialization session).

Observation 19: Using the above information, the LMF may select a quantization strategy including e.g.: scalar quantization (SQ) and vector quantization (VQ).

Proposal 30: RAN1 to study intermediate feature quantization impact on localization accuracy.

Proposal 31: RAN1 to assess what type of assistance information LMF can provide to the UE regarding the quantization of the intermediate feature.

### Soft RSTD reporting

Unlike the hard selection wherein only a single estimate is reported back, soft reporting involves with the reporting of certain intermediate metrics that characterizes the distribution of the estimation to improve the detection accuracy at the LMF. One such approach is to model the estimation as Gaussian process and provide the LMF with the mean and variance associated with the probable LOS path. In certain cases, the stronger path cannot be identified with confidence, an additional NLOS path can also be signalled to the LMF.

Figure 13 provides a visual illustration of multi-tap PDP observed for a TRP by the UE using PRS. Upon inputting the estimated CIR/PDP of each TRP to the AIML model, we can obtain a soft estimate as shown in Fig. 1. Each prominent tap can be characterized as a Gaussian distribution with certain uncertainty and mean. Once the mean and variance are obtained for each tap, a single tap or with an additional NLOS tap can be fed back to the LMF for position estimation.

Figure 13 - Visual representation of ToA (evaluated from PDP/CIR) distribution (Gaussian) for each TRP .

Since the Gaussian distribution is characterized completely by mean and variance, LMF can the use this observation to perform likelihood combining to estimate the UE location. It is evident that by knowing the distribution of the observation outperforms compared to single feedback information considered in hard RSTD approach. However, the trade-off between the signalling overhead (using quantized feedback) vs the positioning accuracy must be studied. Additionally, the computational complexity involved in the generation of soft ToA/RSTD metric at the UE together with the LMF complexity involved with the Likelihood fusion must also be considered.

Observation 20: For soft RSTD, model complexity cannot be ignored while studying the trade-off between the performance gain and the reporting overhead.

Observation 21: The uncertainty involved with the ToA/RSTD estimation also depends on the operating SNR conditions and freshness of collected samples distribution.

# Conclusion

In this contribution, we make the following observations and proposals:

## Observations

Observation 1: In the scope of functionality identification framework, the new set of methods and their respective UE’s capability conditions can be mapped using as a reference the current LPP framework defined in TS 3GPP 37.355 for AI/ML positioning cases.

Observation 2: Considering as starting point, the LPP protocol as the 3GPP legacy framework of the functionality identification aspects on AI/ML enhanced positioning cases is beneficial for the current study item. Besides that, it is expected to include new positioning methods based on AI/ML and new reporting IEs.

Observation 3: Regarding data collection, the noisy label evaluation can be assisted by the LMF. Samples with one or several sources of noisy labels should be pre-evaluated as part of data collection for model updating and model retraining.

Observation 4: In case of the opportunity to label a sample with multiple positioning sources, one may exploit all the information from different positioning sources and incorporate them in the labels for training the ML model.

Observation 5: Any given positioning measurement that does not represent well the expected distribution for a set of TRPs relative to the UE location may be seen as abnormal/rare events. Nevertheless, some rare events may prove useful to the ML positioning function, while others may be entirely detrimental.

Observation 6: Because of RF limitations, some impairments are generated, such as phase rotation and delays of the positioning signal by the RF chain at the baseband receiver. As a result, a positioning entity (UE, TRP, etc.) hosting the ML positioning function could experience certain RF-based signal distortions which are not considered explicitly or characterized and compensated for updating the model.

Observation 7: RF-based signal imperfections are different between host-type devices. For example, a PRU or gNB hosting the ML model would require adapting the model to its RF-specific characteristics.

Observation 8: Performance monitoring may be done proactively (e.g. on a periodic basis), or reactively (over demand basis). Any entity (the UE or the LMF) that detects an environment change may trigger a reactive performance monitoring.

Observations 9: A new measurement report such as monitoring metric can be used to switch the positioning functionalities (direct AIML, AI/ML assisted). The monitoring metric can be considered as an associated information to the given inference output.

Observation 10: In Case 2a, depending on the type of intermediate feature reported to the LMF, the UE may need specific network assistance. The assistance depends on the type of intermediate feature (e.g. range/distribution).

Observation 11: NW may assist performance monitoring using both labelled and unlabelled test data for case 3a using NRPPa interface. The NW may clean the test data prior to sending it to the gNB via NRPPa.

Observation 12: For case 3b, when the performance monitoring is triggered, the LMF may collect monitoring data from selected gNBs using the existing positioning legacy 3GPP protocols.

Observation 13: Output-based performance monitoring for cases 2b, 3b can be implemented autonomously by the LMF and does not require standardization.

Observation 14: Output-based performance monitoring for case 3a may follow proposal 25 the difference being that the request is sent over a different legacy 3GPP framework (NRPPa).

Observation 15: The existing legacy 3GPP framework may be extended with possible new methods and IEs to support and report the inference output of AI/ML positioning enhancement.

Observation 16: The solution approaches presented so far as part of this study have considered only one-sided models, with the AI/ML model output directly indicating the UE location or providing intermediate features that are used by classical positioning approaches to estimate the UE location. However, for some cases using CIR and PDP with high dimensionality as model input could cause significant overhead for data collection for model training and inference. It is also unclear how an AI/ML model for positioning can cope with variable CIR/PDP size and shape.

Observation 17: A positioning reference signal receiver (e.g. UE or TRP) that uses AI/ML positioning (direct or assisted) must ensure that the input to the AI/ML block is of fixed characteristics (e.g. size and shape), regardless of the size and shape of the received positioning reference signal.

Observation 18: The performance of LOS/NLOS classification using AI/ML has a strong dependency on the environmental setting as well as the bandwidth capabilities of the UE.

Observation 19: Using the above information, the LMF may select a quantization strategy including e.g.: scalar quantization (SQ) and vector quantization (VQ).

Observation 20: For soft RSTD, model complexity cannot be ignored while studying the trade-off between the performance gain and the reporting overhead.

Observation 21: The uncertainty involved with the ToA/RSTD estimation also depends on the operating SNR conditions and freshness of collected samples distribution.

## Proposals:

Proposal 1: For understanding various AI/ML positioning cases, it would be beneficial to include illustrative figures in the technical report (TR 38.843).

Proposal 2: Regarding functionality identification framework, to study and discuss the potential specification impact for functionalities and reporting of UE’s applicable conditions.

Proposal 3: RAN1 to study and discuss the specification impact of selected positioning anchors indication as intermediate feature on AI/ML assisted positioning.

Proposal 4: RAN1 to consider at least the following mandatory UE’s applicable conditions on supporting ML functionalities for all AI/ML positioning enhancement cases (Case 1, Case 2, and Case 3):

  - Max number of supported functionalities (1, 2, 4, 8, ...)

Indicates the maximum number of functionalities (e.g., number of parameter combinations that enable ML-enabled feature) that can be configured toward the UE

  - Delay in activating a functionality (2 ms, 4 ms, ...)

Indicates the delay required when activating or switching a functionality

  - Generalization condition of functionalities (true, false)

Indicates that the UE supports any functionality configured considering the parameter combinations of 1-4 and can be used towards the UE without any validation whether functionality is applicable or not.

Proposal 5: RAN1 to consider at least the following mandatory UE’s applicable conditions for Cases 1 and Case 2a:

  - Supported N’t: To indicate the N’t values that the UE is capable to consider (N’t = 64, 128, 256, 512).

  - Supported N_port: To indicate N_port, which is the number of transmit/receive antenna port pairs to consider (N_port = 1,2,4).

  - Supported N_TRP: To indicate N_TRP, which is the number of TRPs to (N_TRP = 1, 2, …, 72)

Proposal 6: RAN1 to consider at least the following optional UE’s applicable conditions for Cases 1 and Case 2a:

  - Supported on collecting and labeling dataset for training, updating, and monitoring: It defines the support of the UE to receive data collection assistance from the LMF.

  - Supported anchor indication from the LMF: It indicates whether anchors selection by the LMF is supported.

  - Supported set conditions for measured DL PRS: Defines support of using DL PRS based CIR measurements.

  - Supported performance monitoring conditions – Supports model drift identification: Defines support of data distribution measurement to identify label drift and features drift.

  - Supported performance monitoring conditions – minimum measurement report periodicity: Defines the minimum periodicity to report performance monitoring.

  - Supported estimated CIR quantization reporting: defines the discrete characteristics of the CIR.

Proposal 7: RAN1 to consider at least the following mandatory UE’s applicable condition for AI/ML assisted positioning in Case 1 and Case 2a:

  - Supported intermediate_feature: To indicate the intermediate feature (ToA, LOS/NLOS indication, AoA, Selected positioning anchors).

Proposal 8: RAN1 to consider at least the following optional UE’s applicable condition for Case 2a:

  - For LOS/NLOS classification - supported ue_channel features: Indicates which UE channel features can be used for determining LOS/NLOS indication (UE_channel_features = {CIR_energy, CIR_maximun_amplitude, CIR RMS delay spread, CIR skewness, CIR_kurtosis}).

Proposal 9: RAN1 to deprioritize the study and discussion of the specification impact of data collection for model training for the current functionality identification framework.

Proposal 10: RAN1 to prioritize the study and discussion of the specification impact of data collection for model update/re-tunning and performance monitoring considering legacy 3GPP framework for positioning.

Proposal 11: For data collection, RAN1 to study and discuss the potential specification impact of at least the following challenges: data availability, noise ground truth, presence of abnormal propagation conditions, and RF imperfection. Reusing as much as possible the legacy 3GPP framework on positioning.

Proposal 12: For ground truth labels, RAN1 to study and discuss the potential specification impact of at least the UE distribution used for model updating and performance monitoring for positioning cases. Reusing as much as possible the legacy 3GPP framework on positioning.

Proposal 13: For UE-sided model (Case 1 and Case 2a), RAN1 to study solutions and their respective specification impact, as part of data collection, using network driven evaluation to verify the consistency of labels.

Proposal 14: RAN1 to define means on identification and management of abnormal propagation conditions during data collection and the potential specification impact (e.g., define conditions to identify abnormal propagation conditions, define actions to manage such as whether to discard associated measurement, etc.).

Proposal 15: RAN1 to study the specification impact on using the existing positioning framework, through which a generic ML positioning model can be customized to the specific NR elements host types - including target UE, PRU, or gNBs and their RF chain imperfections.

Proposal 16: For Case 1, RAN1 to study the specification impact of the NW indication to UE requirements and parameters related to data collection (e.g., IPD threshold, data augmentation configuration, abnormal propagation condition). Reusing as much as possible the legacy 3GPP framework on positioning.

Proposal 17: For Case 2a and Case 2b, RAN1 to study the specification impact of NW selecting a set of PRS configurations related to data collection.

Proposal 18: For Case 3a and Case 3b, RAN1 to study the specification impact of gNB selecting a set of SRS configurations related to data collection.

Proposal 19: Regarding performance monitoring, RAN1 to study the specification impact of using PRU through LPP protocol.

Proposal 20: For Case 1 and Case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contains monitoring data for AI/ML model performance (including samples with corresponding labels).

Proposal 21: RAN1 to further study and discuss the possible specification impact considering additional model outputs, which can deliver new monitoring metric to trigger model updating/switching.

Proposal 22: For Case 1, RAN1 to consider the NW assistance for performance monitoring using both labeled and unlabeled data. Such data may be collected from PRUs and/or other UEs (in case of unlabeled data) by the NW.

Proposal 23: For Case 2a, RAN1 to study and discuss at least potential specification impact when labelled data is used for monitoring. The labels are the intermediate features, which are extracted by the PRU. PRU data is collected by the NW, potentially cleaned, and then transferred to the target UE.

Proposal 24: For Case 1, Case 2a (UE-side model) considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contain data for monitoring the AI/ML model performance (including samples with corresponding labels).

Proposal 25: For Case 1 AI/ML direct and assisted AI/ML positioning, RAN1 to study the potential specification impact when UE triggers a request to the NW based on its self-evaluation of the model performance (e.g., model output related performance). This request can be also used to trigger any LCM step (enable model updating within a functionality, functionality switching) and may be set over a legacy 3GPP framework.

Proposal 26: RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples.

Proposal 27: RAN1 to study solutions that enable the UE to report positioning measurements in a fixed format characteristics (e.g., size and shape) independent of the PRS configuration, including what type of assistance the LMF may provide for enabling the fixed format.

Proposal 28: For Case 1, Case 2a, and Case 3a, the network should be able to assist UE and/or gNB for LOS/NLOS classification by providing a ranked list of channel features such as containing CIR characteristics, RMS delay spread, etc. The ranked list should be based on the UE and/or gNB capabilities in terms of bandwidth, computation, and environmental setting.

Proposal 29: In Case 1 and Case 2a, a UE-sided model may conduct anchor selection for positioning. For this, LMF may request UE to do anchor selection, and provide any necessary assistance data (e.g., containing a list of candidate anchors with their locations). To train the ML model, UE may request information from LMF on the positioning QoS resulting from the selected anchor(s).

Proposal 30: RAN1 to study intermediate feature quantization impact on localization accuracy.

Proposal 31: RAN1 to assess what type of assistance information LMF can provide to the UE regarding the quantization of the intermediate feature.

# References

- 3GPP RP-213599, "New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface" 3GPP RAN-94-e.

- 3GPP R1-2212331, "Evaluation on AI/ML for positioning accuracy enhancement," Nokia, Nokia Shanghai Bell, 3GPP RAN1-111.

- 3GPP R1-2209371, "Evaluation on AI/ML for positioning accuracy enhancement," Nokia, Nokia Shanghai Bell, 3GPP RAN1-110bis-e.

- 3GPP R1-2205633, "Final Summary of [109-e-R18-AI/ML-07] Email discussion on evaluation of AI/ML for positioning accuracy enhancement," RAN1-109-e, May 2022.

- 3GPP TS 38.305, "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17)".

# Appendix A

The agreements reached on the agenda item 9.2.4.2 related to use cases and specification impact for AI/ML based positioning accuracy enhancement are indicated in each 3GPP RAN1 meeting.

Agreements reached in 3GPP RAN1#109e meeting

Agreement Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels. - Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2. - Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement. - Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case Agreement For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

Agreements reached in 3GPP RAN1#110 meeting

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity Conclusion To use the following terminology defined in TS 38.305 when describe their proposed positioning methods - UE-based - UE-assisted/LMF-based - NG-RAN node assisted Note: companies are required to clarify their positioning method(s) when their approaches do not fall in one of the above

Agreements reached in 3GPP RAN1#110-bis-e meeting

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

Agreements reached in 3GPP RAN1#111 meeting

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI. Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305 Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network Agreement For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as representative sub-use cases.

Agreements reached in 3GPP RAN1#112 meeting

Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection - E.g., requested quality of training data Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).