3GPP TSG RAN WG1 #112bis-e                      		R1-2302640

e-Meeting, April 17th – 26th, 2023

Source:	CATT

Title:	Discussion on RAN2 LS on RAN dependency for Ranging & Sidelink Positioning

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on the issues on RAN dependency for Ranging/Sidelink Positioning in RAN2 reply LS R1-2302281(R2-2302255) [1].

Discussion on SL positioning QoS parameters

In the RAN2 reply LS R1-2302281(R2-2302255) [1], RAN2 discussed the issues in the reply LS on RAN dependency from SA2 [2], and would like to ask SA2 to take the following RAN2 feedback into consideration:

Regarding issue 1, RAN2 has concluded to use PC5-U as the transport layer for SLPP. Regarding issue 2, from RAN2 perspective, SL positioning QoS parameters may include: - for absolute & relative positioning: absolute/relative horizontal accuracy, verticalCoordinateRequest, absolute/relative vertical accuracy, response time, and velocityRequest. - for ranging: distance accuracy, direction accuracy, response time, and velocityRequest. Whether additional QoS parameters are required may need to be addressed during the work item. Regarding issue 3, RAN2 has concluded that RAN2 will not discuss assistant UE in Rel-18.

Regarding the RAN2 feedback on issue 2 listed in the reply LS, from RAN2 perspective, SL positioning QoS parameters may include:

- for absolute & relative positioning: absolute/relative horizontal accuracy, verticalCoordinateRequest, absolute/relative vertical accuracy, response time, and velocityRequest. - for ranging: distance accuracy, direction accuracy, response time, and velocityRequest.

Then, RAN2 would like RAN1 to evaluate whether RAN2’s understanding on SL positioning QoS parameters is correct and whether additional parameters are needed.

In our view, SL positioning QoS parameters may include SL positioning accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

For absolute & relative SL positioning, SL positioning QoS parameters may include absolute/relative horizontal accuracy, absolute/relative vertical accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

For ranging, SL positioning QoS parameters may include distance accuracy, direction accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

Proposal 1: Regarding the issue of SL positioning QoS parameters in the RAN2 reply LS R1-2302281(R2-2302255), suggest providing the following response:

- From RAN1 perspective, SL positioning QoS parameters may include SL positioning accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

  - For absolute & relative SL positioning, SL positioning QoS parameters may include absolute/relative horizontal accuracy, absolute/relative vertical accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

  - For ranging, SL positioning QoS parameters may include distance accuracy, direction accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

Conclusion

In this contribution, we provide our views on the issue of SL positioning QoS parameters in RAN2 reply LS R1-2302281(R2-2302255). Our proposal is given as follows:

Proposal 1: Regarding the issue of SL positioning QoS parameters in the RAN2 reply LS R1-2302281(R2-2302255), suggest providing the following response:

- From RAN1 perspective, SL positioning QoS parameters may include SL positioning accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

  - For absolute & relative SL positioning, SL positioning QoS parameters may include absolute/relative horizontal accuracy, absolute/relative vertical accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

  - For ranging, SL positioning QoS parameters may include distance accuracy, direction accuracy, SL positioning service latency, SL positioning service availability and effective SL positioning distance.

References

  - R2-2302255, “Reply LS on RAN dependency for Ranging & Sidelink Positioning”, RAN2, 3GPP TSG RAN2#121.

  - S2-2301464, “Reply LS on Reply LS on RAN dependency for Ranging/Sidelink Positioning”, SA2, 3GPP TSG SA2 Meeting #S2-154AHE.