3GPP TSG RAN WG1 #112bis-e 	     		  R1-2302643

e-Meeting, April 17th – 26th, 2023

Title:	[Draft] Reply LS on error source distributions

Response to:	R1-2302282(R2-2302271)

Release:	Rel-18

Work Items:	NR_pos_enh2-Core

Source:	CATT [to be RAN1]

To:	RAN2, RAN3

CC:

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.cn

Attachment:

1. Overall Description:

RAN1 thanks RAN2 LS (R2-2302271), which indicates that RAN2 has reached the following agreement:

Agreements: RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

In the LS, RAN2 requests RAN1 to confirm the above agreement and provide the parameters (e.g. mean and standard deviation) for the overbound Gaussian distribution.

RAN1 has discussed above issue in RAN1#112bis-e, and would provide the following response to RAN2:

Response to RAN2:

- Regarding RAN2’s agreement, RAN1 confirms that each error source can be over-bounded by a Gaussian distribution.

- Regarding the mean and standard deviation for the overbound Gaussian distribution, RAN1 provides the following response:

- For Rel-18, RAN1 recommends all identified error sources can be treated as independent, Gaussian distribution variables. The time correlation of the error sources is not considered.

- For Rel-18, each of the error sources can be considered as a variable following a zero-mean Gaussian distribution.

- The standard deviation of the Gaussian distribution of the error sources can be presented by the quality parameters defined in TS 37.355 and TS 38.455 as shown in the following table.

- Note: Whether to use existing parameters or introduce new parameters to represent the standard deviation of the Gaussian distribution of the error sources can be determined by RAN2 or RAN3.

| Error source | Standard deviation | Description |
| --- | --- | --- |
| RSTD error | nr-TimingQuality (TS 37.355) nr-PathQuality (TS 37.355) | nr-TimingQuality specifies the target device′s best estimate of the quality of the RSTD measurement. nr-PathQuality specifies the target device′s best estimate of the quality of the detected timing of the additional path. |
| UE Rx-Tx time difference measurement error | nr-TimingQuality (TS 37.355) nr-PathQuality (TS 37.355) | nr-TimingQuality specifies the target device′s best estimate of the quality of the UE Rx-Tx time measurement. nr-PathQuality specifies the target device′s best estimate of the quality of the detected timing of the additional path. |
| Inter-TRP synchronization | rtd-RefQuality (TS 37.355) rtd-Quality (TS 37.355) | rtd-RefQuality specifies the quality of the timing of reference TRP. rtd-Quality specifies the quality of the RTD. |
| TRP/ARP location error | locationUNC (TS 37.355) | locationUNC specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. |
| RTOA measurement error | Timing Measurement Quality (TS 38.455) | specifies the quality of the RTOA measurement. |
| AOA measurement error | Angle Measurement Quality (TS 38.455) | specifies the quality of the AoA measurement |

2. Actions:

To RAN2, RAN3,

ACTION: RAN1 respectfully requests RAN2 and RAN3 to take the above responses into account in future work.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #113				May 22 to May 26, 2023			Incheon, KR

TSG-RAN WG1 Meeting #114				Aug. 21 to Aug. 25, 2023		Toulouse, FR