3GPP TSG-RAN WG1 #112bis-e	R1-2302651

# e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on collision handling between valid PRACH occasion and NCD-SSB in Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, |  |  |  |  |  | Date: |  |  | 2023-04-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TDD, current PRACH occasion validation depends on CD-SSB, which reflects the fact that CD-SSB is cell-common and always-on. And in a sense, PRACH occasion can be regared as cell-common. Hence, collision handling between CD-SSB and valid PRACH occasion is not needed. In Rel-17, RedCap UE is introduced. Due to the narrow UE bandwidth, RedCap UE may operate in a BWP without CD-SSB but with NCD-SSB. Considering the reasons above, validation of PRACH occasion shall still depends on CD-SSB. But collision handling rule between NCD-SSB and valid PRACH occasion is missing in current specification. In this case, RedCap UE shall not receive NCD-SSB, similar to other DL signals/channels, when collided with valid PRACH occasion. Otherwise, RedCap UE may be seriously interfered by CLI from legacy UE nearby. Also a TDD gNB is unable to transmit NCD-SSB when receiving/detecting PRACH for legacy UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 11.1, add the missing description of ‘not receiving NCD-SSB’ when NCD-SSB is collided with valid PRACH occasion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour is not clear when collision happens between NCD-SSB and valid PRACH occasion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

11.1	Slot configuration

<Unchanged parts are omitted>

For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in clause 8.1, the UE does not receive PDCCH, PDSCH, CSI-RS, or SS/PBCH blocks provided by NonCellDefiningSSB in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

<Unchanged parts are omitted>