3GPP TSG RAN WG1 #112bis-e		                                                          R1-2302666

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type-1/2/3 HARQ-ACK codebook generation with both of maxNrofCodeWordsScheduledByDCI for unicast and multicast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, the maximum numbers of code words that a unicast DCI and a multicast DCI may schedule are separately configured by maxNrofCodeWordsScheduledByDCI in PDSCH-config and maxNrofCodeWordsScheduledByDCI in PDSCH-configMulticast respectively. When both maxNrofCodeWordsScheduledByDCI in PDSCH-config and maxNrofCodeWordsScheduledByDCI in PDSCH-configMulticast are configured, it is not clear whether maxNrofCodeWordsScheduledByDCI in PDSCH-config or in PDSCH-configMulticast is used for Type-1/2/3 HARQ-ACK codebook generation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The clarification on maxNrofCodeWordsScheduledByDCI in relevant section as: - For Type-1 HARQ-ACK codebook generation, if a UE is provided fdmed-ReceptionMulticast, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config is applied to Type-1 HARQ-ACK codebook for the set SU of serving cells, and the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast is applied to Type-1 HARQ-ACK codebook for the set SM of serving cells. - For Type-1 HARQ-ACK codebook generation, if a UE is provided type1-Codebook-Generation-Mode = 'mode1', the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast is applied to Type-1 HARQ-ACK codebook for the first K1,UM set, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config is applied to Type-1 HARQ-ACK codebook for the second K1,U\M set, and the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast is applied to Type-1 HARQ-ACK codebook for the third K1,M\U set - For Type-1 HARQ-ACK codebook generation, if a UE is not provided type1-Codebook-Generation-Mode = 'mode1', the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast is applied to Type-1 HARQ-ACK codebook. - For Type-2 HARQ-ACK codebook generation, maxNrofCodeWordsScheduledByDCI in PDSCH-config or in PDSCH-configMulticast are applied to Type-1 HARQ-ACK codebook for unicast and multicast, respectively. - For Type-3 HARQ-ACK codebook generation, the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast is set as the value of NTB,cDL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE doesn’t know how to use maxNrofCodeWordsScheduledByDCI to generate Type-1/2/3 HARQ-ACK codebook when both of maxNrofCodeWordsScheduledByDCI for unicast and multicast are configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4, 9.1.3.1, 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

<Unchanged Text Omitted>

If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats

-	a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U  with applying the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and by setting NcellsDL=NcellsDL,M with applying the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = 'mode1', the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R, and for each of the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-config, the maxNrofCodeWordsScheduledByDCI configured in PDSCH-configMulticast as the value of maxNrofCodeWordsScheduledByDCI to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.

If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI and is not provided type1-Codebook-Generation-Mode = 'mode1' and is not provided fdmed-ReceptionMulticast, the UE generates a Type-1 HARQ-ACK codebook by applying the maximum value of maxNrofCodeWordsScheduledByDCI configured in PDSCH-config and in PDSCH-configMulticast in the following pseudo-code.

If timeDomainHARQ-BundlingType1 is provided

-	set RT'=R

-	set RT to the set of row indexes that include the last SLIV of each row of set RT'

<Unchanged Text Omitted>

<Unchanged Text Omitted>

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs for multicast or G-CS-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause per G-RNTI for multicast or per G-CS-RNTI  with applying maxNrofCodeWordsScheduledByDCI configured in pdsch-ConfigMulticast except the procedures for SPS PDSCHs and applies the procedures in this clause with applying maxNrofCodeWordsScheduledByDCI configured in pdsch-Config for unicast DCI formats, and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats excluding the unicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-CS-RNTI values excluding the multicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for unicast and multicast SPS PDSCH receptions.

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling PDSCH reception, on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values, or a dl-DataToUL-ACK, dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17 value if the PDSCH-to-HARQ_feedback timing indicator field is not present in a DCI format, for PUCCH transmission with HARQ-ACK information in slot n, as described in clause 9.2.3, in response to PDSCH receptions, or in response to a DCI format having associated HARQ-ACK information without scheduling PDSCH reception

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided.

<Unchanged Text Omitted>

<Unchanged Text Omitted>

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated numbers of HARQ processes NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is zero.

Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI which is configured in PDSCH-config or  maximum parameter value configured in PDSCH-config and in PDSCH-configMulticast if configured for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1

<Unchanged Text Omitted>