3GPP TSG RAN WG1 #112bis-e		                                                         R1-2302671

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on alignment of the condition on RTgeneration and candidate PDSCH reception determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For multi-PDSCHs scheduling, pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is defined in TS 38.331, there is no explicit description to indicate that at least one row contains multiple SLIVs when pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is configured. Note such restriction is used in TS 38.214 as following yellow color text. 5.1.2.1 Resource allocation in time domain (in TS 38.214) …… If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17.. In TS 38.213, parameter pdsch-TimeDomainAllocationListForMultiPDSCH is used to determine candidate PDSCH reception in section 9.1.2.1, but there is no limitation to ensure pdsch-TimeDomainAllocationListForMultiPDSCH has at least one row containning multiple SLIVs If gNB configures pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH, RT will be NULL set, there will be error case from for UE implementation during candidate PDSCH reception determination. |
| Summary of change: |  | Align the restriciton of pdsch-TimeDomainAllocationListForMultiPDSCH as in TS 38.214 for candidate PDSCH reception determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error case may be caused during candidate PDSCH reception determination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged Text Omitted> while k<CK1 if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk if pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; elseif pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while <Unchanged Text Omitted> while k<CK1 if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk if pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; elseif pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows end if Set CR to the cardinality of R Set r=0 – index of row in set R if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, nD=nD+1; else while r<CR if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if pdsch-TimeDomainAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU R=R\r; elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1. R=R\r; R'=R'\r; else r=r+1; end if end while <Unchanged Text Omitted>

-