3GPP TSG RAN WG1 #112bis-e		                                                         R1-2302672

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Discussion on RT generation and candidate PDSCH reception determination for the features extending NR operation to 71 GHz

Agenda Item:	7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, the condition of RT generation and candidate PDSCH reception determination is discussed. It is shown that the error case will occur if UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 when there is no row contains multiple SLIVs for PDSCH. Two possible schemes are proposed to resolve the above issue.

- Discussion

For supporting multi-PDSCHs scheduled by DCI format 1_1, pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is defined in TS 38.331, and there is no explicit description to tell that at least one row contains multiple SLIVs in pdsch-TimeDomainAllocationListForMultiPDSCH-r17. Moreover, in TS 38.214 [2], the limitation of pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is also specified as following yellow color text.

5.1.2.1 Resource allocation in time domain …… If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17. If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell. If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, when any two DL DCIs end in the same symbol and at least one of the DCIs schedules multiple PDSCHs, the UE does not expect that the scheduled PDSCH(s) by the two DCIs have overlapping spans, where the span associated with a DCI is defined from the beginning of the first scheduled PDSCH or up to the end of the last scheduled PDSCH.. ……

As the text described in section 5.1.2.1 of [2], it can be understood that the current specification allows gNB to configure pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH.

Observation 1: gNB may configure pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH.

In section 9.1.21 of [1], two steps are described about how to generate RT and determine candidate PDSCH reception when UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17. The RT can be seen as a TDRA that each row only contain one SLIV for UE to determine candidate PDSCH reception. There are two conditions that UE will generateRT; one condition is that timeDomainHARQ-BundlingType1 is provided as following yellow colour words. The other condition is that the set of rows R includes a row with more than one SLIV entry as described in [6, TS 38.214] and timeDomainHARQ-BundlingType1 is not provided as following pink colour words

9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel If timeDomainHARQ-BundlingType1 is provided - set RT'=R - set RT to the set of row indexes that include the last SLIV of each row of set RT' If the set of rows R includes a row with more than one SLIV entry as described in [6, TS 38.214] and timeDomainHARQ-BundlingType1 is not provided, the set of rows R and the set of slot timing values K1 are updated in this clause according to the following pseudo-code. set R to the set of rows set CR to the cardinality of R set r=0 – index of row in set R set RT=∅ set K1,T=∅ while r<CR set Pr to the set of entries for row r set K0,r to the set of K0 values of entries for row r set ∆K0,r=maxK0K0,r-K0,r set CPr to the cardinality of Pr set C∆K0,r to the cardinality of ∆K0,r set p=0 – index of element in Prset d=0 – index of element in ∆K0,r while p<CPr RT=RT∪Prp; p=p+1; end while while d<C∆K0,r K1,T=K1,T∪K1+∆K0,r(d)⋅2μUL-μDL∪K1+∆K0,r(d)⋅2μUL-μDL; d=d+1; end while r=r+1; end while K1=K1,T; ----------------------- End of text to TS 38.213 v17.5.0 -------------------------

During the procedure of candidate PDSCH reception determination, the  RT will be applied in two conditions. One condition is that if pdsch-TimeDomainAllocationListForMultiPDSCH and timeDomainHARQ-BundlingType1 are provided as following yellow colour words, and this condition can work well. The other condition is that elseif pdsch-TimeDomainAllocationListForMultiPDSCH is provided and timeDomainHARQ-BundlingType1 is not provided as following pink colour words. For the second condition of applying   RT, if UE is configured  the pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which none row contain multiple SLIVs for PDSCH, RT will be NULL set as described in RT generation section, it is error case from UE view since it will result R is NULL set.

While k<CK1 if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk if pdsch-TimeDomainAllocationListForMultiPDSCH and timeDomainHARQ-BundlingType1 are provided for serving cell c R=RT; R'=RT'; elseif pdsch-TimeDomainAllocationListForMultiPDSCH is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c R=RT; else Set R to the set of rows

Observation 2: if gNB configures pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH, RT will be NULL set, there will be error case from for UE implementation during candidate PDSCH reception determination.

To resolve the error case during candidate PDSCH reception determination, there are two possible schemes. One approach is that clarify that there is at least one row contain multiple SLIVs when the UE is configured pdsch-TimeDomainAllocationListForMultiPDSCH-r17. Otherwise, if gNB will configure TDRA in which each row include only one SLIV, parameter TimeDomainAllocationList shall be used. The other approach is to align the condition of RT generation and candidate PDSCH reception determination as is done in TS 38.214.

Proposal 1：To resolve the error case during candidate PDSCH reception determination, there are two possible schemes can be downtown selected by RAN1.

- Alt-1: (No CR) Clarify in RAN1 that if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 if configured , there is at least one row contains multiple SLIVs; otherwise, if gNB configures TDMA in with each row only include one SLIV,   parameter TimeDomainAllocationList shall be used  .

- Alt-2: Modify the spec to align the limitation condition RTgeneration and candidate PDSCH reception determination as is used in TS 38.214.

The draft CR [3] is attached if Alt-2 is adopted by RAN1.

- Conclusion

In this contribution, the condition RT generation and candidate PDSCH reception determination is discussed. Two observations and one proposal are given as follows.

Observation 1: gNB may configure pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH

Observation 2: if gNB configures pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which no row contains multiple SLIVs for PDSCH, RT will be NULL set, there will be an error case from for UE implementation during candidate PDSCH reception determination.

Proposal 1：To resolve the error case during candidate PDSCH reception determination, there are two possible schemes can be downtown selected by RAN1.

- Alt-1: (No CR) Clarify in RAN1 that if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 if configured , there is at least one row contains multiple SLIVs; otherwise, if gNB configures TDMA in with each row only include one SLIV,   parameter TimeDomainAllocationList shall be used  .

- Alt-2: Modify the spec to align the limitation condition RTgeneration and candidate PDSCH reception determination as is used in TS 38.214.

The draft CR [3] is attached if Alt-2 is adopted by RAN1.

- References

3GPP TS 38.213 V17.5.0:” NR; Physical layer procedures for control”

3GPP TS 38.214 V17.5.0:” NR; Physical layer procedures for data”

R1-2302671, “Draft CR on Alignment the condition R_T generation and candidate PDSCH reception determination for the features extending NR operation to 71 GHz”, CATT, RAN1#112bis-e, April 17th – April 26th, 2023