3GPP TSG RAN WG1 #112bis-e                                                                                   R1-2302676

e-Meeting, April 17th – 26th, 2023

Source:                 CATT

Title:	PDCCH skipping interaction with HARQ

Agenda Item:	9.8

Document for:	Discussion and Decision

Introduction

The PDCCH skipping interaction with HARQ was discussed in Rel-17 UE power saving enhancement, Rel-18 XR, and Rel-18 TEI in RAN1. The proposal of XR enhancement with PDCCH skipping with resuming PDCCH monitoring when NACK sent was proposed in RAN1#98e and RAN1#99. However, the PDCCH skipping with HARQ interaction does not provide the justification of power saving gain in all scenarios. In this paper, we provide evaluation results for the PDCCH skipping interaction with HARQ scheme [1] and enhanced PDCCH skipping schemes [2]. The detailed simulation assumptions are listed in Appendix.

Discussion

In RAN#112, PDCCH skipping scheme enhanced with HARQ retransmission [1] to achieve a good trade-off between UE power consumption and capacity performance was proposed in Rel-18 TEI. The same proposal was brought to RAN#99 to be included in the revised XR WID. In this scheme, UE would resume PDCCH monitoring behaviour when the initial or retransmission of last packet fails. The UE still consumes the power for unnecessary PDCCH monitoring while waiting for the gNB processing of retransmission packet. The enhanced PDCCH skipping scheme along with MAC go-to-sleep signaling in [2] can provide appropriate solution of UE power saving. UE could continuously skip the PDCCH MOs until the DCI is successfully decoded at the time of packet arrival. When the XR packet transmission is completed, the MAC control can terminate the drx-onDuartion Timer and drx-Inactivity Timer as the go-to-sleep signaling to allow UE fast transition to the sleep state after correct decoding the last XR packet within the cycle.

The above two enhanced PDCCH skipping schemes are summarized in Table 1.

Table 1:  PDCCH skipping enhancement schemes

| Schemes | Procedure |
| --- | --- |
| Baseline 1：Always-on | PDCCH monitoring is based on the configured PDCCH monitoring cycle of the search space. |
| PDCCH skipping enhancement scheme 1 (Rel-17 PDCCH skipping with resuming PDCCH monitoring when NACK) | When the XR packet transmission is completed, UE would be indicated by scheduling DCI to skip an interval of PDCCH skipping and return to normal PDCCH monitoring afterward. UE would resume PDCCH monitoring if UE transmits NACK after PDCCH skipping starts. |
| PDCCH skipping enhancement scheme 2 (Continuous PDCCH skipping and dynamic go-to-sleep indication) | UE can be indicated by non-scheduling and scheduling DCI to skip one or more MOs continuously. When the XR packet transmission is completed, the MAC control can terminate the drx-onDuartion Timer and drx-Inactivity Timer as the go-to-sleep signaling to allow UE fast transition to the sleep state after correct decoding the last XR packet within the cycle. |

Table 2 shows the evaluation results of the enhanced PDCCH skipping schemes compared to always-on. We can observe that continuous PDCCH skipping and go-to-sleep indication can achieve 29.4% power saving gain with negligible capacity degradation compared with always-on. UE can be indicated by non-scheduling and scheduling DCI to skip duration of PDCCH monitoring continuously. When the XR packet transmission is completed, the MAC control can terminate the drx-onDuartion Timer and drx-Inactivity Timer as the go-to-sleep signaling to allow UE fast transition to the sleep state after correct decoding the last XR packet within the cycle. It can save obvious power consumption before XR packet arrival and after XR packet completed transmission, but hardly introduce additional latency for the packet delivery.

PDCCH skipping interaction with HARQ scheme with short skipping duration can hardly provide any power saving with 7.1% power saving gain over that of always-on. In this case, the power consumption for PDCCH monitoring before XR packet arrival cannot be saved. Although the longer skipping duration of PDCCH skipping can provide similar or slightly higher UE power saving with 31.5% power saving gain comparing to that of PDCCH skipping enhancement with MAC-CE based go-to-sleep signaling,  the XR capacity performance suffer significant loss. The long PDCCH skipping duration will have a large latency for the subsequent XR packet delivery unless it is the last packet. From SA2 reply LS to RAN1 on XR and Media Services in R1-2210826 in [3], the XR packets contain one or more PDU sets in a period of time. The gNB might need to wait for some PDU sets to arrive with PDCCH skipping. The PDCCH skipping with resuming PDCCH monitoring after NACK transmission will have the results of power saving performance with short skipping duration or degrading the capacity performance severely with long skipping duration.

Table 2: Evaluation results of PDCCH skipping enhancement schemes compared to always-on

| Schemes | Mean PSG compared to always-on | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline: Always-on | - | 11.5/12 |
| PDCCH skipping enhancement scheme 1Note1 | 7.1% | 10.8/12 |
| PDCCH skipping enhancement scheme 1Note2 | 31.5% | 5.4/12 |
| PDCCH skipping enhancement scheme 2Note1 | 29.4% | 10.7/12 |
| Note1: Skipping duration = 3 slots (1.5ms); Note2: Skipping duration = 16 slots (8ms); |  |  |

Observation 1: The PDCCH skipping scheme enhanced with continuous PDCCH skipping and go-to-sleep indication can achieve 29.4% power saving gain with negligible capacity degradation compared with that of always-on.

Observation 2: PDCCH skipping interaction with HARQ scheme with short skipping duration can hardly provide any power saving with 7.1% power saving gain over that of always-on.

Observation 3: Although the longer skipping duration of PDCCH skipping can provide similar or slightly higher UE power saving with 31.5% power saving gain comparing to that of PDCCH skipping enhancement with MAC-CE based go-to-sleep signaling, the XR capacity performance would suffer significant loss.

In RAN#112, some companies proposed PDCCH monitoring adaptation schemes [2] with the evaluation results of enhanced PDCCH skipping with HARQ interaction scheme showing relatively good performance in UE power saving and system capacity. However, the proposed scheme does not consider the SSSG switching delay and the addition power consumption for detecting SSSG switching indication. The MAC-CE based go-to-sleep signaling scheme considers the 3ms processing delay after HARQ-ACK transmission and provide 19.3%~20.8% power saving gain by terminating the drx-onDuartion Timer and drx-Inactivity Timer for UE to go to sleep mode directly.

Proposal 1: PDCCH skipping scheme with resuming PDCCH monitoring after NACK provides only negligible UE power saving gain with short skipping duration and have severe degradation in system capacity for longer skipping duration. PDCCH skipping scheme with resuming PDCCH monitoring after NACK should not be supported.

- 3 Conclusion

Based on above discussion, we have observations and proposals, as follows:

Observation 1: The PDCCH skipping scheme enhanced with continuous PDCCH skipping and go-to-sleep indication can achieve 29.4% power saving gain with negligible capacity degradation compared with that of always-on.

Observation 2: PDCCH skipping interaction with HARQ scheme with short skipping duration can hardly provide any power saving with 7.1% power saving gain over that of always-on.

Observation 3: Although the longer skipping duration of PDCCH skipping can provide similar or slightly higher UE power saving with 31.5% power saving gain comparing to that of PDCCH skipping enhancement with MAC-CE based go-to-sleep signaling, the XR capacity performance would suffer significant loss.

Proposal 1: Rel-17 PDCCH skipping scheme with resuming PDCCH monitoring after NACK provides only negligible UE power saving gain with short skipping duration and have severe degradation in system capacity for longer skipping duration. PDCCH skipping scheme with resuming PDCCH monitoring after NACK should not be supported.

References

R1-2300484, Rel-18 TEI proposals, vivo, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

R1-2208952, UE power saving techniques for XR, CATT, RAN1#110bis e-meeting, October 10th – 19th, 2022.

R1-2210826, LS on XR and Media Services, vivo, RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- 4 Appendix

The simulation assumptions of Indoor Hotspot are shown in Table 3.

Table 3: Simulation parameters of Indoor Hotspot

| Scenario | Indoor Hotspot |
| --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 |
| Carrier frequency | FR1:4GHz |
| Bandwidth | FR1:100MHz |
| Subcarrier spacing | FR1: 30 kHz |
| BS height | 3m |
| UE height | hUT=1.5 m |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| Receiver | MMSE-IRC |
| UE speed | 3 km/h |
| Channel estimation | Realistic |
| MCS | Up to 64QAM |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, |
| TX power | gNB: FR1: 24dBm/20MHz |
| gNB antenna configuration | gNB: - FR1:32TxRU, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| TDD configuration | DDDSU for DL |
| MIMO transmission | MU-MIMO for DL |
| Scheduling | PF, Dynamic scheduling |