3GPP TSG RAN WG1 #112bis-e                      	                       R1-2302680

e-Meeting, April 17th – 26th, 2023

Source:	CATT

Title:		Further discussion on unified TCI framework extension for multi-TRP operation

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In this contribution, our views on extending Rel-17 unified TCI to multi-TRP operation as well as multi-panel UL transmission are analyzed, including power control related issues.

Extension for indication of multiple DL/UL TCI states

In RAN1 #112 meeting, the following agreements on unified TCI for multi-TRP have been achieved [1]:

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

In this section, CJT-based PDSCH transmission, TCI state association, TCI state update, beam indication signaling and TRP-specific BFR enhancement are discussed.

CJT-based PDSCH transmission

Based on previous agreement, up to two joint TCI states are supported for CJT-based PDSCH reception. Whether one or two TCI states are applied depends on UE capability. If UE supports two indicated TCI states, three alternatives on QCL assumptions are proposed for down-selection in RAN1#111 meeting. From the views of some companies, gNB pre-compensation has to be used for CJT, otherwise the delay or frequency offset across TRPs could not be handled by UE. Then some parameters of the QCL assumption for the second TCI state could be removed. Taking Alt2 (i.e. PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state) as an example, frequency pre-compensation is used. TRS in the first indicated joint TCI state is used for Doppler estimation, and TRS in each joint TCI state is used for delay estimation, respectively. This is similar as PDSCH-SFN schemeB. However, for PDSCH-SFN schemeA in Rel-17, pre-compensation is not needed for PDSCH, and two TRP specific TRS are transmitted. With Doppler and delay estimated for each TRP, UE is feasible to compensate the offset across TRPs. In our opinion, CJT-based PDSCH is similar as that of PDSCH-SFN, and it would work based on merely the mechanism of PDSCH-SFN schemeA. Therefore, gNB pre-compensation (similar as PDSCH-SFN schemeB) may not be mandatory for CJT-based PDSCH transmission. Consequently, QCL-Type A should be configured in each TCI state.

Proposal 1: For CJT-based PDSCH transmission, when two joint TCI states are indicated, PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA (Alt1).

TCI state association

Within the unified TCI framework, when single or multiple TCI states are indicated, UE has to correctly associate each TCI state to corresponding channel resource(s) or transmission repetition(s). Therefore, Rel-18 has to specify the TCI state association with resource and/or time domain repetition. TCI state association for some channels with certain scheduling conditions or configurations has been agreed in the previous meetings. The association schemes for the other conditions or configurations are discussed in the following.

TCI state update for S-DCI based multi-TRP

In Rel-17 multi-TRP, TCI state ID(s) is directly configured for PDCCH/PUCCH. In addition, in Rel-15/16, TCI state ID and SpatialRelationInfo are configured for CSI-RS and SRS, respectively. However, such configuration is not applicable to Rel-18 unified TCI, where association with the indicated TCI states would be configured instead for these channels or reference signals. To define this association, TCI state indicators (the order index of the indicated TCI states, e.g. TCI_ind_0 denotes the first TCI state, TCI_ind_1 denotes the second TCI state,…), which refer to the indicated TCI states, should be introduced in Rel-18.

Proposal 2: To define the association between the indicated TCI state and signals/channels, TCI state indicators referring to the indicated TCI states should be introduced in Rel-18.

If PDSCH is scheduled by DCI format 1_0, since there is no TCI field, default TCI state(s) should be defined for PDSCH reception. When the UE is not configured with PDSCH-SFN or PDSCH-CJT, the first indicated TCI state could be defined as the default TCI state as a simple solution. In Rel-17, if UE is configured with PDSCH-SFN, one or two default TCI states are specified depending on UE capability (i.e. whether or not to support dynamic switching between sTRP and PDSCH-SFN). However, as unified TCI for mTRP is Rel-18 feature and PDSCH-CJT is also introduced in Rel-18, from our point of view, a new default TCI state rule is preferred for Rel-18 for less spec complexity. Therefore, the simple solution, i.e. using the first indicated TCI state as the default TCI state, should also be applied when the UE is configured with PDSCH-SFN or PDSCH-CJT. When the indicator field is not present in DCI format 1_1/1_2, according to the agreement, default indicated TCI state(s) are also needed for PDSCH reception. To avoid spec complexity, a common default TCI state, i.e. the first indicated TCI state, is preferred for both cases.

Proposal 3: For PDSCH reception scheduled by DCI format 1_1/1_2 without the new indicator field or by DCI format 1_0 with or without PDSCH-SFN/PDSCH-CJT configured, the first indicated TCI state is used as the default TCI state for PDSCH reception (Alt2).

For S-DCI based MTRP PUSCH transmission, one open issue is how to apply the joint/UL TCI state(s) to SDM/SFN based PUSCH transmission occasions when the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the SRS resource set indicator. Since the association between an SRS resource set for CB/NCB and SDM/SFN based PUSCH transmission occasions(s) would be defined in AI 9.1.4.1, whether codepoint “11” of the SRS resource set indicator is reserved is still FFS. It is our view that for SDM and SFN based PUSCH Tx schemes, when the SRS resource set indicator field indicates codepoint "10", same scheme as that for TDM based PUSCH Tx scheme can be used, i.e., the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB. If codepoint “11” of the SRS resource set indicator is not reserved, the UE applies the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB is also preferred.

Proposal 4: For SDM and SFN based PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG), when two SRS resource sets for CB/NCB are configured, support the followings:

- If the DCI format 0_1/0_2 indicates codepoint "10" for the SRS resource set indicator:

  - The UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB;

- If codepoint "11" for the SRS resource set indicator is not reserved, when the DCI format 0_1/0_2 indicates codepoint "11" for the SRS resource set indicator:

  - The UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB.

The above association is applicable to DG-PUSCH and Type 2 CG PUSCH. For Type 1 CG PUSCH, similar as PDCCH, TCI state association is defined for each Type 1 CG PUSCH configuration.

Proposal 5: For Type 1 CG PUSCH, TCI state association is configured for each Type 1 CG PUSCH configuration.

TCI state update for M-DCI based multi-TRP

It has been agreed that the indicated TCI state specific to a CORESETPoolIndex value is applied to PDCCH and its scheduled/activated PDSCH on a CORESET that is associated with the same CORESETPoolIndex value. For PUCCH, there is a case that PUCCH transmission is scheduled by a DCI with DL assignment. For the other cases, there is no explicit or implicit association with CORESETPoolIndex. For the former case, PUCCH transmission could follow the TCI state of the scheduling DCI. For the other cases, referring to the TCI association mechanism mentioned above for S-DCI based PDCCH/PUCCH, CORESETPoolIndex could be configured for each resource. In our opinion, it seems not necessary to introduce two different association rules for PUCCH transmission. Therefore, CORESETPoolIndex configured for each PUCCH resource is preferred.

Proposal 6: For M-DCI based multi-TRP, for PUCCH, CORESETPoolIndex could be configured for each resource (Alt1).

For Type 1 CG PUSCH, according to the agreement of the last meeting, when two SRS resource sets are configured，SRS_resource_set_index would be configured in the Type 1 CG configuration. SRS_resource_set_index indicates one SRS resource set, which is associated with a CORESETPoolIndex value. Therefore, in this case, Type 1 CG PUSCH is actually associated with CORESETPoolIndex. The indicated TCI state associated with the same CORESETPoolIndex should be applied to PUSCH transmission. When only one SRS resource set is configured, there is no explicit or implicit association with CORESETPoolIndex. In this case, CORESETPoolIndex could be configured in the SRS resource set.

Proposal 7: For M-DCI based multi-TRP, for Type 1 CG PUSCH, when two SRS resource sets are configured, the indicated TCI state specific to a CORESETPoolIndex value that is indirectly associated with the Type 1 CG PUSCH would be applied to PUSCH transmission; when only one SRS resource set is configured, CORESETPoolIndex could be configured in the SRS resource set.

TCI state update for reference signals

In Rel-17 unified TCI framework, reference signals (e.g. CSI-RS, SRS) could follow the indicated TCI state(s) or not depending on the configuration. If not, Rel-15/16 TCI state update signaling/configuration design(s) are reused. The same rule should be extended to Rel-18. If RS is configured to share the indicated TCI state(s), the association of the TCI state(s) to the RS has to be designed.

Same as Rel-17, P/SP CSI-RS can never share the indicated TCI states in Rel-18, and the legacy RRC/MAC-CE signalling mechanism is always used for TCI state update. For aperiodic CSI-RS, we prefer to define a single association rule, where RRC configuration is used in the CSI report setting to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set. There are concerns for the scenarios of enhanced group-based beam reporting and NCJT CSI measurement, where CSI-RS resources within the CSI-RS resource set may use different TCI states. In our opinion, for these scenarios, the CSI-RS resource set could be configured not to follow the indicated TCI states, and Rel-17 mechanism would be used. Therefore, special rule is not necessary.

Proposal 8: For aperiodic CSI-RS, if configured to follow the indicated TCI state, RRC configuration (e.g. a TCI indicator) is used in the CSI report setting to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set.

Besides, for M-DCI based multi-TRP, when two SRS resource sets are configured for CB/NCB, a SRS resource set is associated with a CORESETPoolIndex value. In this case, the indicated TCI state specific to the CORESETPoolIndex value should be applied to SRS transmission. When only one SRS resource set is configured for CB/NCB, the association could be achieved by its triggering DCI. Namely, aperiodic SRS set could follow the TCI state specific to the CORESETPoolIndex value of the CORESET carrying the triggering DCI.

Proposal 9: For M-DCI based multi-TRP, when only one SRS resource set is configured for CB/NCB, the aperiodic SRS set could follow the TCI state specific to the CORESETPoolIndex value of the CORESET carrying the triggering DCI.

TCI state update and activation

Based on the agreement of the last meeting, for S-DCI based multi-TRP, the existing TCI field in DCI format 1_1/1_2 can indicate TCI state(s) for one or both of the two TRPs. The switching between single TRP and multi-TRP does not depend on the number of TCI states indicated in DCI format 1_1/1_2. As discussed above, when two joint/DL TCI states are indicated, PDSCH transmission could either be single TRP or multi-TRP, depending on the value of the new TCI indicator field in the DCI. Similarly, for PUSCH, dynamic switching is also supported according to the “SRS resource set” field in DCI format 0_1/0_2. Therefore, when TCI states for both of the two TRPs are indicated, each channel could determine its single TRP or multi-TRP transmission separately. However, there is an issue when TCI state(s) for only one TRP is indicated. UE needs to discriminate whether sTRP/mTRP switching is indicated or TCI state update for only one TRP is assumed. For the latter case, UE further needs to know the TCI state(s) of which TRP should be updated. As a candidate scheme, an additional field could be introduced in the activation MAC-CE or DCI format 1_1/1_2. For example, additional flag is introduced for each codepoint in the activation MAC-CE to indicate sTRP/mTRP switching or TCI state update. If Rel-17 MAC-CE is used as shown in Figure 1, a 2-bits flag is added to Pi.

Figure 1：Rel-17 TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

Proposal 10: For S-DCI based multi-TRP, when UE receives a TCI codepoint mapped with TCI state(s) of one TRP, whether sTRP/mTRP switching or TCI state update for only one TRP is assumed should be indicated. An additional field could be introduced in the activation MAC-CE for the indication.

Enhancement to TRP-specific BFR

TRP-specific BFR has been supported in Rel-17, which should be extended to unified TCI. In Rel-17, both explicit BFD RS and implicit BFD RS have been specified. For explicit BFD RS, two BFD RS sets (each corresponding to one TRP) could be configured for either S-DCI or M-DCI. However, for implicit BFD RS, two BFD RS sets are only applicable to M-DCI. Since for M-DCI, each CORESETPoolIndex configured by RRC corresponds to one TRP. Then, the RS within the TCI states for those CORESETs associated with the same CORESETPoolIndex could be used as BFD RS for one TRP. But there is no such TRP-ID like parameter for S-DCI. In Rel-18, as discussed in the last several meetings, if all the CORESETs follow the indicated TCI state(s), a fixed rule could be used to determine the BFD-RS set, i.e. the UE determines the BFD-RS for the first and second BFD-RS sets from the first and second indicated joint/DL TCI states, respectively. For those CORESET(s) not following the indicated TCI state(s), Rel-15/16 TCI state update signaling/configuration design(s) are reused. Namely, MAC-CE is used to activate one or two TCI states from a set of RRC configured TCI states for a CORESET. In our opinion, a new field could be introduced in this MAC-CE to indicate the activated TCI states belong to which BFD-RS set.

Proposal 11: For the CORESET(s) not following the indicated TCI state(s), a new field could be introduced in Rel-15/16 activation MAC-CE to indicate its activated TCI state(s) belong to which BFD-RS set.

In Rel-17 unified TCI framework, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for DL receptions. And the UE uses the same UL spatial filter as the one associated with the index q new or the last PRACH transmission for UL transmissions. Considering Rel-17 multi-TRP, TRP-specific NBI RS set is configured and q new is reported using BFR MAC-CE associated with a BFD-RS set (i.e. a TRP). For Rel-18 UTCI, the beam resetting mechanism of Rel-17 could be extended to TRP-specific beam resetting. Namely, if UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding q new , if reported, in the BFR MAC-CE.

Proposal 12: If UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding

q new, if reported, in the BFR MAC-CE.

Association of panel(s) and TCI state(s) for simultaneous multi-panel UL transmission

In Rel-17, UE-initiated UL panel activation and selection is supported for fast uplink panel selection, via UE reporting a list of UE capability values along with the pairs of SSBRI/CRI and L1 RSRP/L1 SINR in a beam reporting instant. Each UE capability value comprises the maximum supported number of SRS antenna ports, and the maximum supported numbers of SRS antenna ports of any two UE capability values are different.

For STxMP UL transmission in Rel-18, up to 2 TRPs and up to 2 panels is considered. When two TCI states are selected, the two TCI states may correspond to same or different panels. In terms of RF functionality, in most cases, a UE panel is not able to generate two analog beams simultaneously. Therefore indicating two beams corresponding to the same panel for simultaneous UL transmission should be avoided. That means that for the configuration of unified TCI for simultaneous UL transmission, gNB has to know the correspondence between panels and TCI states.

Proposal 13: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

There are various approaches to identify the correspondence between panels and TCI states.

One approach is to use UE capability value to indicate the association between SSB resources/CSI-RS resources and panels as that for Rel-17 fast panel selection. Note that in Rel-17, two different UE capability values must correspond to two different maximum supported numbers of SRS antenna ports. That is, if a UE receives two CSI-RS resources with two panels, respectively, and the two panels correspond to same maximum supported number of SRS antenna ports, same UE capability value would be reported. Then gNB does not know whether the two CSI-RS resources correspond to same panel or different panels. The two UL panels deployed by UE may correspond to same or different maximum number of SRS antenna ports, and same or different maximum number of MIMO layers. Therefore allowing UE capability values for two panels correspond to same or different maximum number of SRS antenna ports, and same or different maximum supported number of MIMO layers should be supported.

Proposal 14: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Another approach is using group-based beam reporting to faciliate gNB to know which beams can be used for UL transmission simultaneously. UE can report two groups of beams, with each group corresponds to a panel. Then any two SSB resources/CSI-RS resources in different beam groups can be used for UL transmission simultaneously.

Proposal 15: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

Power control enhancement for Multi-TRP transmission

In RAN1 #112 meeting, the following agreement on power control enhancement for multi-TRP transmission was achieved [1]:

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

This agreement shows that mechanism on the determination of UL PC parameter setting and PL-RS on unified TCI framework can be extended to the Rel-18 multi-TRP uplink transmission. An issue needs to be further discussed is how to determine the maximum Tx power for STxMP when UE calculates UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s).

For NR Rel-18 STxMP transmission, upon the LS from RAN4 [2], each panel has its own maximum transmission power, and the total transmission power of a UE is also limited. Power control mechanisms need to be considered for these limitations. Suppose UE has two panels, the maximum transmission power for the ith panel is Pcmaxi. The maximum total transmission power of the UE is Pcmax which is configured per CC. Basically, power control mechanism of the following cases should be studied:

Case1: P1 + P2 <= Pcmax with Pi < =Pcmax,i

Case2: P1 + P2  > Pcmax  with Pi < =Pcmax,i

where P1 and P2 represents the transmission power of panel 1 and panel 2 determined from equation (1) provided by [3]  respectively, with Pcmax replaced by Pcmax1 and Pcmax2 in the calculation.

(1)

For Case1, only power control of per-panel power limitation need to be considered, in the legacy mechanism, the definition of Pcmax in equation (1) is a cell-specific parameter to determine the transmission power of PUSCH transmission. It is determined from SIB parameter and UE power class limitation. If per-panel power limitation is considered, Pcmax,i for panel i shall be used. Pcmax,i  can be obtained by UE capability reporting and RRC configuration.

For Case 2, how to determine actual transmission power of panel 1 and panel 2 on the basis of P1 and P2 to meet the requirement of total transmission power limitation needs to be further studied.

Figure 3: Power restriction cases with Pi <= Pcmax,i  in NR Rel-18 STxMP transmission

For Case 2, power control schemes considering both per-panel power limitation and total transmission power limitation need to be considered. Suppose UE has two panels, the maximum transmission power for panel 1 and panel is Pcmax1 = 23dBm, Pcmax2 = 23dBm. The maximum total transmission power of the UE Pcmax = 20dBm. For once transmission, UE determines its real PUSCH transmission power P1 = 21dBm and P2 = 22dBm individually according to equation (1) with Pcmax replaced by Pcmax1 and Pcmax2 respectively. In addition, considering Pcmax is limited by 20dBm and P1 + P2  > Pcmax, power control mechanism, e.g. power back-off mechanism can be used to reduce the transmission power of both TRPs or one of the TRPs, detailed power back-off mechanisms can be further studied. E.g. UE will reduce the power of UE-to-TRP1 according to its pathloss etc.

Proposal 16: For STxMP transmission in Rel-18, to determine PUSCH transmission power per panel m, the following formula is used, where Pcmax,m can be obtained by UE capability reporting and RRC configuration. i.e.

PPUSCH,b,f,c,mi,j,qd,l=minPCMAX,f,c,m(i)POPUSCH,b,f,cj+10log102u∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,c(i,l)

Proposal 17: For STxMP transmission in Rel-18, if the transmit power across multiple panels exceeds total power limitation, some power back-off mechanisms can be considered. E.g. UE will perform power backoff of one UE-to-TRP link according to its pathloss etc.

For the default UL PC parameter determination, In RAN1 #105 e-meeting, it was agreed that for Rel-17 unified TCI framework, if UL PC parameter is not associated with the indicated TCI state, the setting of UL PC parameters(P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or joint TCI states for each of the PUSCH and PUCCH. Detailedly, the default UL PC parameter settings are configured in BWP-UplinkDedicated. This conclusion can be straightly extended to Rel-18 unified TCI multi-TRP transmission.

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }

pusch-Config                        SetupRelease { PUSCH-Config }

......

ul-powerControl-r17                Uplink-powerControlId-r17                                         OPTIONAL,  -- Cond NoTCI-PC

}

Per TRP default power control parameter has been supported in Rel-17 M-TRP PUSCH/PUCCH repetition due to the different pathloss between two UE-to-TRP links. It should also be supported for M-TRP with Rel-18 unified TCI.

Proposal 18： Support two default UL PC parameter settings configured in BWP-UplinkDedicated if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting for PUCCH/PUSCH M-TRP transmission.

Conclusions

In this contribution, we discuss extending Rel-17 unified TCI to multi-TRP use cases and multi-panel UL transmission. The power control related issues are also analyzed. We have the following proposals:

Proposal 1: For CJT-based PDSCH transmission, when two joint TCI states are indicated, PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA (Alt1).

Proposal 2: To define the association between the indicated TCI state and signals/channels, TCI state indicators referring to the indicated TCI states should be introduced in Rel-18.

Proposal 3: For PDSCH reception scheduled by DCI format 1_1/1_2 without the new indicator field or by DCI format 1_0 with or without PDSCH-SFN/PDSCH-CJT configured, the first indicated TCI state is used as the default TCI state for PDSCH reception (Alt2).

Proposal 4: For SDM and SFN based PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG), when two SRS resource sets for CB/NCB are configured, support the followings:

- If the DCI format 0_1/0_2 indicates codepoint "10" for the SRS resource set indicator:

  - The UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB;

- If codepoint "11" for the SRS resource set indicator is not reserved, when the DCI format 0_1/0_2 indicates codepoint "11" for the SRS resource set indicator:

  - The UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB.

Proposal 5: For Type 1 CG PUSCH, TCI state association is configured for each Type 1 CG PUSCH configuration.

Proposal 6: For M-DCI based multi-TRP, for PUCCH, CORESETPoolIndex could be configured for each resource (Alt1).

Proposal 7: For M-DCI based multi-TRP, for Type 1 CG PUSCH, when two SRS resource sets are configured, the indicated TCI state specific to a CORESETPoolIndex value that is indirectly associated with the Type 1 CG PUSCH would be applied to PUSCH transmission; when only one SRS resource set is configured, CORESETPoolIndex could be configured in the SRS resource set.

Proposal 8: For aperiodic CSI-RS, if configured to follow the indicated TCI state, RRC configuration (e.g. a TCI indicator) is used in the CSI report setting to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set.

Proposal 9: For M-DCI based multi-TRP, when only one SRS resource set is configured for CB/NCB, the aperiodic SRS set could follow the TCI state specific to the CORESETPoolIndex value of the CORESET carrying the triggering DCI.

Proposal 10: For S-DCI based multi-TRP, when UE receives a TCI codepoint mapped with TCI state(s) of one TRP, whether sTRP/mTRP switching or TCI state update for only one TRP is assumed should be indicated. An additional field could be introduced in the activation MAC-CE for the indication.

Proposal 11: For the CORESET(s) not following the indicated TCI state(s), a new field could be introduced in Rel-15/16 activation MAC-CE to indicate its activated TCI state(s) belong to which BFD-RS set.

Proposal 12: If UE receives the BFRR corresponding to a BFR MAC-CE associated with a BFD-RS set, the DL and UL channels/RSs of the TRP sharing the indicated TCI state(s) are all reset to the corresponding

q new, if reported, in the BFR MAC-CE.

Proposal 13: For STxMP UL transmission, how to facilitate gNB to know the correspondence between panels and TCI states should be studied.

Proposal 14: For STxMP UL transmission, consider enhancing UE capability values to represent panels. The following two enhancements can be considered:

- Two different UE capability values correspond to same or different maximum supported number of SRS antenna ports are supported;

- Using UE capability value to indicate maximum supported number of MIMO layers.

Proposal 15: For STxMP UL transmission, group-based beam reporting is considered, with each group corresponds to a panel.

Proposal 16: For STxMP transmission in Rel-18, to determine PUSCH transmission power per panel m,  the following formula is used, where Pcmax,m can be obtained by UE capability reporting and RRC configuration. i.e.

PPUSCH,b,f,c,mi,j,qd,l=minPCMAX,f,c,m(i)POPUSCH,b,f,cj+10log102u∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,c(i,l)

Proposal 17: For STxMP transmission in Rel-18, if the transmit power across multiple panels exceeds total power limitation, some power back-off mechanisms can be considered. E.g. UE will perform power backoff of one UE-to-TRP link according to its pathloss etc.

Proposal 18： Support two default UL PC parameter settings configured in BWP-UplinkDedicated if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting for PUCCH/PUSCH M-TRP transmission.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112.

- R1-2302265, 3GPP RAN WG1 Meeting #112bis-e

- 3GPP RAN1, TS 38.213 V17.0.