3GPP TSG RAN WG1 #112bis                                                                                  R1-2302683

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	DMRS enhancements in Rel.18

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

Introduction

As approved in RAN #94-e meeting, DMRS enhancements include the following objectives [1]:

Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DMRS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DMRS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, possible enhancements on increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO are discussed.

Enhancements on increasing number of orthogonal DMRS ports

In RAN1 #110b-e, RAN1 #111 and RAN1#112 meeting, the following agreements related to DMRS ports indication and orphan RE handling were achieved.

Agreement - For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated: - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11) - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23) - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case, - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE. - FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not. - FFS: Whether the new antenna port(s) table is specified or not. - FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc. - FFS: Cat.3 for M-TRP case. - Note: DMRS port index for PDSCH is determined by p +1000 Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11. Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows: - For 1 CW, - 1) Rows 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1) Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows: - For 1 CW, - 2) Row 9-11 - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s): - For one CW, support at least row 30 in the following table. - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). - FFS: other rows are not precluded Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 30; 2; 0,2,3. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved. Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - 1) The number of consecutively scheduled PRBs for PDSCH is even. - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded.

In this section, our views on antenna port(s) tables for PDSCH and PUSCH with up to 4 layers, scheduling restrictions, orphan RE handling and sequence mapping are provided.

  - Antenna port(s) tables for PDSCH

In this sub-section, port combinations for M-TRP case and for eType 1 and eType 2 DMRS are discussed, and detailed tables are elaborated.

- Port combinations for M-TRP case

According to the agreement, {0, 2, 3} is supported for S-DCI based M-TRP, and other rows are not precluded. According to the definition of Cat.3 port combinations, only one CDM group is used per UE for up to 4 ranks. Therefore, Cat.3 port combination is not fit for S-DCI based M-TRP transmission. Since {0, 2, 3} is Cat.1 port combination, similar Cat.2 port combination which supports layer combination 1+2 can also be considered if the size of antenna port(s) field is not increased. Therefore, port combinations {8, 10, 11} and {12, 14, 15} can also be supported by eType 1 DMRS and eType 2 DMRS, respectively.

Proposal 1: For Rel.18 eType 1 and eType 2 DMRS, port combinations  {8, 10, 11} and {12, 14, 15} can also be supported for S-DCI based M-TRP.

- eType1 DMRS with maxLength=1

For eType1 DMRS with maxLength=1, rows 21-23 need further study. Row 21 and row 22 are Cat.2 port combinations and they are designed for SU-MIMO. Considering that the size of antenna port(s) field is not affected by these two rows, row 21 and row 22 can be kept to increase the scheduling flexibility of the gNB. In row 23, there are two possible port combinations. With port combination {8, 10}, Cat.1 and Cat.2 port combinations have a fixed port offset. With port combination {9, 11}, UEs with 3+3+2 layers can be co-scheduled by row 23, 26 ({0, 1, 8}) and 28 ({2, 3, 10}), respectively. Since rank 3 transmission is not a typical scenario in MU-MIMO, and Cat.1 port combinations were fully discussed in Rel.15, the corresponding Cat.2 port combination (i.e., port combination {8, 10}) is desirable.

Table 1: Antenna port(s) (1000 + DMRS port) for 1CW, dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | 2 | 8-10 |
| 22 | 2 | 8-11 |
| 23 | 2 | 8, 10 |

Proposal 2: For Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for 1 CW, rows 21-23 in Table 1 are supported.

- eType1 DMRS with maxLength=2

For eType 1 DMRS with maxLength=2, the number of Cat.1 port combinations is 32. If Cat.2 port combinations have the same quantity as Cat.1 port combinations, the size of antenna port(s) field will be larger than 6 bits (considering that Cat.3 port combinations are also needed). In order to minimize the signaling overhead, some Cat.2 port combinations which are used for high-rank SU-MIMO transmission can be dropped. Table 2 shows the detailed antenna port(s) which includes both Cat.2 and Cat.3 port combinations.

Table 2: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Notes |
| 0 | 1 | 0 | 1 | Cat. 1 | 0 | 2 | 0-4 | 2 | [Rank 5-8 with one or two DMRS symbols] |
| 1 | 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,4,6 | 2 |  |
| 2 | 1 | 0,1 | 1 |  | 2 | 2 | 0,1,2,3,4,5,6 | 2 |  |
| 3 | 2 | 0 | 1 |  | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |  |
| 4 | 2 | 1 | 1 |  | 4 | 2 | 0,1,2,3,8 | 1 |  |
| 5 | 2 | 2 | 1 |  | 5 | 2 | 0,1,2,3,8,10 | 1 |  |
| 6 | 2 | 3 | 1 |  | 6 | 2 | 0,1,2,3,8,9,10 | 1 |  |
| 7 | 2 | 0,1 | 1 |  | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |  |
| 8 | 2 | 2,3 | 1 |  | 8-63 | Reserved | Reserved | Reserved |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |  |  |
| 31 | 2 | 0,2,3 | 1 |  |  |  |  |  |  |
| 32 | 1 | 8 | 1 | Cat.2 |  |  |  |  |  |
| 33 | 1 | 9 | 1 |  |  |  |  |  |  |
| 34 | 1 | 8,9 | 1 |  |  |  |  |  |  |
| 35 | 2 | 8 | 1 |  |  |  |  |  |  |
| 36 | 2 | 9 | 1 |  |  |  |  |  |  |
| 37 | 2 | 10 | 1 |  |  |  |  |  |  |
| 38 | 2 | 11 | 1 |  |  |  |  |  |  |
| 39 | 2 | 8,9 | 1 |  |  |  |  |  |  |
| 40 | 2 | 10,11 | 1 |  |  |  |  |  |  |
| 41 | 2 | 8-10 | 1 |  |  |  |  |  |  |
| 42 | 2 | 8-11 | 1 |  |  |  |  |  |  |
| 43 | 2 | 8,10 | 1 |  |  |  |  |  |  |
| 44 | 2 | 8 | 2 |  |  |  |  |  |  |
| 45 | 2 | 9 | 2 |  |  |  |  |  |  |
| 46 | 2 | 10 | 2 |  |  |  |  |  |  |
| 47 | 2 | 11 | 2 |  |  |  |  |  |  |
| 48 | 2 | 12 | 2 |  |  |  |  |  |  |
| 49 | 2 | 13 | 2 |  |  |  |  |  |  |
| 50 | 2 | 14 | 2 |  |  |  |  |  |  |
| 51 | 2 | 15 | 2 |  |  |  |  |  |  |
| 52 | 2 | 8,9 | 2 |  |  |  |  |  |  |
| 53 | 2 | 10,11 | 2 |  |  |  |  |  |  |
| 54 | 2 | 12,13 | 2 |  |  |  |  |  |  |
| 55 | 2 | 14,15 | 2 |  |  |  |  |  |  |
| 56 | 2 | 8,12 | 2 |  |  |  |  |  |  |
| 57 | 2 | 10,14 | 2 |  |  |  |  |  |  |
| 58 | 2 | 8,10,11 | 2 |  |  |  |  |  |  |
| 59 | 1 | 0,1,8 | 1 | Cat.3 |  |  |  |  |  |
| 60 | 1 | 0,1,8,9 | 1 |  |  |  |  |  |  |
| 61 | 2 | 0,1,8 | 1 |  |  |  |  |  |  |
| 62 | 2 | 2,3,10 | 1 |  |  |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 1 |  |  |  |  |  |  |

Proposal 3: For Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, Table 2 is adopted to indicate antenna port(s).

- eType2 DMRS with maxLength=1

With similar design principle, antenna port(s) for eType 2 DMRS with maxLength=1 is shown in Table 3.

Table 3: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 3 | 0-4 | [Rank 5-8 with one DMRS symbol] |
| 1 | 1 | 1 |  | 1 | 3 | 0-5 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,12 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,12,13, |  |
| 4 | 2 | 1 |  | 4 | 2 | 0,1,2,3,12,13,14 |  |
| 5 | 2 | 2 |  | 5 | 2 | 0,1,2,3,12,13,14,15 |  |
| 6 | 2 | 3 |  | 6-63 | Reserved | Reserved |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 3 | 0 |  |  |  |  |  |
| 12 | 3 | 1 |  |  |  |  |  |
| 13 | 3 | 2 |  |  |  |  |  |
| 14 | 3 | 3 |  |  |  |  |  |
| 15 | 3 | 4 |  |  |  |  |  |
| 16 | 3 | 5 |  |  |  |  |  |
| 17 | 3 | 0,1 |  |  |  |  |  |
| 18 | 3 | 2,3 |  |  |  |  |  |
| 19 | 3 | 4,5 |  |  |  |  |  |
| 20 | 3 | 0-2 |  |  |  |  |  |
| 21 | 3 | 3-5 |  |  |  |  |  |
| 22 | 3 | 0-3 |  |  |  |  |  |
| 23 | 2 | 0,2 |  |  |  |  |  |
| 24 | 2 | 0,2,3 |  |  |  |  |  |
| 25 | 1 | 12 | Cat.2 |  |  |  |  |
| 26 | 1 | 13 |  |  |  |  |  |
| 27 | 1 | 12,13 |  |  |  |  |  |
| 28 | 2 | 12 |  |  |  |  |  |
| 29 | 2 | 13 |  |  |  |  |  |
| 30 | 2 | 14 |  |  |  |  |  |
| 31 | 2 | 15 |  |  |  |  |  |
| 32 | 2 | 12,13 |  |  |  |  |  |
| 33 | 2 | 14,15 |  |  |  |  |  |
| 34 | 2 | 12-14 |  |  |  |  |  |
| 35 | 2 | 12-15 |  |  |  |  |  |
| 36 | 3 | 12 |  |  |  |  |  |
| 37 | 3 | 13 |  |  |  |  |  |
| 38 | 3 | 14 |  |  |  |  |  |
| 39 | 3 | 15 |  |  |  |  |  |
| 40 | 3 | 16 |  |  |  |  |  |
| 41 | 3 | 17 |  |  |  |  |  |
| 42 | 3 | 12,13 |  |  |  |  |  |
| 43 | 3 | 14,15 |  |  |  |  |  |
| 44 | 3 | 16,17 |  |  |  |  |  |
| 45 | 3 | 12-14 |  |  |  |  |  |
| 46 | 3 | 15-17 |  |  |  |  |  |
| 47 | 3 | 12-15 |  |  |  |  |  |
| 48 | 2 | 12,14 |  |  |  |  |  |
| 49 | 2 | 12,14,15 |  |  |  |  |  |
| 50 | 1 | 0,1,12 | Cat.3 |  |  |  |  |
| 51 | 1 | 0,1,12,13 |  |  |  |  |  |
| 52 | 2 | 0,1,12 |  |  |  |  |  |
| 53 | 2 | 0,1,12,13 |  |  |  |  |  |
| 54 | 2 | 2,3,14 |  |  |  |  |  |
| 55 | 2 | 2,3,14,15 |  |  |  |  |  |
| 56 | 2 | 4,5,16 |  |  |  |  |  |
| 57 | 2 | 4,5,16,17 |  |  |  |  |  |
| 58 | 3 | 0,1,12 |  |  |  |  |  |
| 59 | 3 | 0,1,12,13 |  |  |  |  |  |
| 60 | 3 | 2,3,14 |  |  |  |  |  |
| 61 | 3 | 2,3,14,15 |  |  |  |  |  |
| 62 | 3 | 4,5,16 |  |  |  |  |  |
| 63 | 3 | 4,5,16,17 |  |  |  |  |  |

Proposal 4: For Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, Table 3 is adopted to indicate antenna port(s).

- eType2 DMRS with maxLength=2

With similar design principle, antenna port(s) for eType 2 DMRS with maxLength=2 is shown in Table 4.

Table 4: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Notes |
| 0 | 1 | 0 | 1 | Cat. 1 | 0 | 3 | 0-4 | 1 | [Rank 5-8 with one or two DMRS symbols] |
| 1 | 1 | 1 | 1 |  | 1 | 3 | 0-5 | 1 |  |
| 2 | 1 | 0,1 | 1 |  | 2 | 2 | 0,1,2,3,6 | 2 |  |
| 3 | 2 | 0 | 1 |  | 3 | 2 | 0,1,2,3,6,8 | 2 |  |
| 4 | 2 | 1 | 1 |  | 4 | 2 | 0,1,2,3,6,7,8 | 2 |  |
| 5 | 2 | 2 | 1 |  | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |  |
| 6 | 2 | 3 | 1 |  | 6 | 2 | 0,1,2,3,12 | 1 |  |
| 7 | 2 | 0,1 | 1 |  | 7 | 2 | 0,1,2,3,12,13, | 1 |  |
| 8 | 2 | 2,3 | 1 |  | 8 | 2 | 0,1,2,3,12,13,14 | 1 |  |
| 9 | 2 | 0-2 | 1 |  | 9 | 2 | 0,1,2,3,12,13,14,15 | 1 |  |
| 10 | 2 | 0-3 | 1 |  | 10-63 | Reserved | Reserved | Reserved |  |
| 11 | 3 | 0 | 1 |  |  |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |  |  |
| 58 | 2 | 0,2,3 | 1 |  |  |  |  |  |  |
| 59 | 1 | 12 | 1 | Cat.2 |  |  |  |  |  |
| 60 | 1 | 13 | 1 |  |  |  |  |  |  |
| 61 | 1 | 12,13 | 1 |  |  |  |  |  |  |
| 62 | 2 | 12 | 1 |  |  |  |  |  |  |
| 63 | 2 | 13 | 1 |  |  |  |  |  |  |
| 64 | 2 | 14 | 1 |  |  |  |  |  |  |
| 65 | 2 | 15 | 1 |  |  |  |  |  |  |
| 66 | 2 | 12,13 | 1 |  |  |  |  |  |  |
| 67 | 2 | 14,15 | 1 |  |  |  |  |  |  |
| 68 | 2 | 12-14 | 1 |  |  |  |  |  |  |
| 69 | 2 | 12-15 | 1 |  |  |  |  |  |  |
| 70 | 3 | 12 | 1 |  |  |  |  |  |  |
| 71 | 3 | 13 | 1 |  |  |  |  |  |  |
| 72 | 3 | 14 | 1 |  |  |  |  |  |  |
| 73 | 3 | 15 | 1 |  |  |  |  |  |  |
| 74 | 3 | 16 | 1 |  |  |  |  |  |  |
| 75 | 3 | 17 | 1 |  |  |  |  |  |  |
| 76 | 3 | 12,13 | 1 |  |  |  |  |  |  |
| 77 | 3 | 14,15 | 1 |  |  |  |  |  |  |
| 78 | 3 | 16,17 | 1 |  |  |  |  |  |  |
| 79 | 3 | 12-14 | 1 |  |  |  |  |  |  |
| 80 | 3 | 15-17 | 1 |  |  |  |  |  |  |
| 81 | 3 | 12-15 | 1 |  |  |  |  |  |  |
| 82 | 2 | 12,14 | 1 |  |  |  |  |  |  |
| 83 | 3 | 12 | 2 |  |  |  |  |  |  |
| 84 | 3 | 13 | 2 |  |  |  |  |  |  |
| 85 | 3 | 14 | 2 |  |  |  |  |  |  |
| 86 | 3 | 15 | 2 |  |  |  |  |  |  |
| 87 | 3 | 16 | 2 |  |  |  |  |  |  |
| 88 | 3 | 17 | 2 |  |  |  |  |  |  |
| 89 | 3 | 18 | 2 |  |  |  |  |  |  |
| 90 | 3 | 19 | 2 |  |  |  |  |  |  |
| 91 | 3 | 20 | 2 |  |  |  |  |  |  |
| 92 | 3 | 21 | 2 |  |  |  |  |  |  |
| 93 | 3 | 22 | 2 |  |  |  |  |  |  |
| 94 | 3 | 23 | 2 |  |  |  |  |  |  |
| 95 | 3 | 12,13 | 2 |  |  |  |  |  |  |
| 96 | 3 | 14,15 | 2 |  |  |  |  |  |  |
| 97 | 3 | 16,17 | 2 |  |  |  |  |  |  |
| 98 | 3 | 18,19 | 2 |  |  |  |  |  |  |
| 99 | 3 | 20,21 | 2 |  |  |  |  |  |  |
| 100 | 3 | 22,23 | 2 |  |  |  |  |  |  |
| 101 | 3 | 12,13,18 | 2 |  |  |  |  |  |  |
| 102 | 3 | 14,15,20 | 2 |  |  |  |  |  |  |
| 103 | 3 | 16,17,22 | 2 |  |  |  |  |  |  |
| 104 | 3 | 12,13,18,19 | 2 |  |  |  |  |  |  |
| 105 | 3 | 14,15,20,21 | 2 |  |  |  |  |  |  |
| 106 | 3 | 16,17,22,23 | 2 |  |  |  |  |  |  |
| 107 | 1 | 12 | 2 |  |  |  |  |  |  |
| 108 | 1 | 13 | 2 |  |  |  |  |  |  |
| 109 | 1 | 18 | 2 |  |  |  |  |  |  |
| 110 | 1 | 19 | 2 |  |  |  |  |  |  |
| 111 | 1 | 12,13 | 2 |  |  |  |  |  |  |
| 112 | 1 | 18,19 | 2 |  |  |  |  |  |  |
| 113 | 2 | 12,13 | 2 |  |  |  |  |  |  |
| 114 | 2 | 14,15 | 2 |  |  |  |  |  |  |
| 115 | 2 | 18,19 | 2 |  |  |  |  |  |  |
| 116 | 2 | 20,21 | 2 |  |  |  |  |  |  |
| 117 | 2 | 12,14,15 | 1 |  |  |  |  |  |  |
| 118 | 1 | 0,1,12 | 1 | Cat.3 |  |  |  |  |  |
| 119 | 1 | 0,1,12,13 | 1 |  |  |  |  |  |  |
| 120 | 2 | 0,1,12 | 1 |  |  |  |  |  |  |
| 121 | 2 | 0,1,12,13 | 1 |  |  |  |  |  |  |
| 122 | 2 | 2,3,14 | 1 |  |  |  |  |  |  |
| 123 | 2 | 2,3,14,15 | 1 |  |  |  |  |  |  |
| 124 | 3 | 0,1,12 | 1 |  |  |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 1 |  |  |  |  |  |  |
| 126 | 3 | 2,3,14 | 1 |  |  |  |  |  |  |
| 127 | 3 | 2,3,14,15 | 1 |  |  |  |  |  |  |

Proposal 5: For Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, Table 4 is adopted to indicate antenna port(s).

  - Antenna port(s) tables for PUSCH with up to 4 layers

According to WID, a common design between DL and UL DMRS is preferable. Therefore, the entries defined for PDSCH DMRS can be reused directly for antenna port(s) tables for PUSCH. Similar to M-TRP case, STxMP is under discussion, and entry {0, 2, 3} has already been supported. Similarly, entries {8, 10, 11} (or {12, 14, 15} for Rel.18 eType 2 DMRS) can also be supported by antenna port(s) tables for PUSCH.

Proposal 6: For Rel.18 eType 1 and eType 2 PUSCH DMRS, layer combination 1+2 is supported.

- {0, 2, 3} and {8, 10, 11} are supported for Rel.18 eType 1 PUSCH DMRS.

- {0, 2, 3} and {12, 14, 15} are supported for Rel.18 eType 2 PUSCH DMRS.

- eType1 DMRS with maxLength=1

In RAN1 #112 meeting, DMRS tables for eType1 DMRS with maxLength=1for rank 1-4 were agreed,  and some port combinations need further check. For rank = 2, row 7 is Cat.2 port combination and is derived from row 3. Considering that the size of antenna port(s) field is not increased with this row, row 7 can be supported. With similar principle, row 1 for rank = 3 and rank = 4 can also be supported.

Proposal 7: Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the following rows are supported.

- Row 7 for rank = 2.

- Row 1 for rank = 3.

- Row 1 for rank = 4.

- eType1 DMRS with maxLength=2

For eType1 DMRS with maxLength=2, port combinations can be the same as PDSCH DMRS tables with the same configuration. The detailed entries are shown in Table 5 to Table 8.

Table 5: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 6: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,10 | 1 |
| 14 | 2 | 8,9 | 2 |
| 15 | 2 | 10,11 | 2 |
| 16 | 2 | 12,13 | 2 |
| 17 | 2 | 14,15 | 2 |
| 18 | 2 | 8,12 | 2 |
| 19 | 2 | 10,14 | 2 |
| 20-31 | Reserved | Reserved | Reserved |

Table 7: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 0,2,3 | 1 |
| 4 | 2 | 8,10,11 | 2 |
| 5 | 1 | 0,1,8 | 1 |
| 6 | 2 | 0,1,8 | 1 |
| 7 | 2 | 2,3,10 | 1 |
| 8-31 | Reserved | Reserved | Reserved |

Table 8: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 1 | 0,1,8,9 | 1 |
| 5 | 2 | 2,3,10,11 | 1 |
| 6-31 | Reserved | Reserved | Reserved |

Proposal 8: Adopt Table 5 to Table 8 to indicate antenna port(s) for Rel.18 eType 1 PUSCH DMRS with maxLength=2.

- eType2 DMRS with maxLength=1

For eType2 DMRS with maxLength=1, port combinations can be the same as PDSCH DMRS tables with the same configuration. The detailed entries are shown in Table 9 to Table 12.

Table 9: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 10: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| 13 | 2 | 12,14 |
| 14-31 | Reserved | Reserved |

Table 11: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 2 | 0,2,3 |
| 4 | 2 | 12-14 |
| 5 | 3 | 12-14 |
| 6 | 3 | 15-17 |
| 7 | 2 | 12,14,15 |
| 8 | 1 | 0,1,12 |
| 9 | 2 | 0,1,12 |
| 10 | 2 | 2,3,14 |
| 11 | 2 | 4,5,16 |
| 12 | 3 | 0,1,12 |
| 13 | 3 | 2,3,14 |
| 14 | 3 | 4,5,16 |
| 15-31 | Reserved | Reserved |

Table 12: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank =4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2 | 2 | 12-15 |
| 3 | 3 | 12-15 |
| 4 | 1 | 0,1,12,13 |
| 5 | 2 | 0,1,12,13 |
| 6 | 2 | 2,3,14,15 |
| 7 | 2 | 4,5,16,17 |
| 8 | 3 | 0,1,12,13 |
| 9 | 3 | 2,3,14,15 |
| 10 | 3 | 4,5,16,17 |
| 11-31 | Reserved | Reserved |

Proposal 9: Adopt Table 9 to Table 12 to indicate antenna port(s) for Rel.18 eType 2 PUSCH DMRS with maxLength=1.

- eType2 DMRS with maxLength=2

For eType2 DMRS with maxLength=2, port combinations can be the same as PDSCH DMRS tables with the same configuration. The detailed entries are shown in Table 13 to Table 16.

Table 13: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 23 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 14: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| 25 | 2 | 12,14 | 1 |
| 26 | 3 | 12,13 | 2 |
| 27 | 3 | 14,15 | 2 |
| 28 | 3 | 16,17 | 2 |
| 29 | 3 | 18,19 | 2 |
| 30 | 3 | 20,21 | 2 |
| 31 | 3 | 22,23 | 2 |
| 32 | 1 | 12,13 | 2 |
| 33 | 1 | 18,19 | 2 |
| 34 | 2 | 12,13 | 2 |
| 35 | 2 | 14,15 | 2 |
| 36 | 2 | 18,19 | 2 |
| 37 | 2 | 20,21 | 2 |
| 38-63 | Reserved | Reserved | Reserved |

Table 15: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 2 | 0,2,3 | 1 |
| 7 | 2 | 12-14 | 1 |
| 8 | 3 | 12-14 | 1 |
| 9 | 3 | 15-17 | 1 |
| 10 | 3 | 12,13,18 | 2 |
| 11 | 3 | 14,15,20 | 2 |
| 12 | 3 | 16,17,22 | 2 |
| 13 | 2 | 12,14,15 | 1 |
| 14 | 1 | 0,1,12 | 1 |
| 15 | 2 | 0,1,12 | 1 |
| 16 | 2 | 2,3,14 | 1 |
| 17 | 3 | 0,1,12 | 1 |
| 18 | 3 | 2,3,14 | 1 |
| 19-63 | Reserved | Reserved | Reserved |

Table 16: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank=4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5 | 2 | 12-15 | 1 |
| 6 | 3 | 12-15 | 1 |
| 7 | 3 | 12,13,18,19 | 2 |
| 8 | 3 | 14,15,20,21 | 2 |
| 9 | 3 | 16,17,22,23 | 2 |
| 10 | 1 | 0,1,12,13 | 1 |
| 11 | 2 | 0,1,12,13 | 1 |
| 12 | 2 | 2,3,14,15 | 1 |
| 13 | 3 | 0,1,12,13 | 1 |
| 14 | 3 | 2,3,14,15 | 1 |
| 15-63 | Reserved | Reserved | Reserved |

Proposal 10: Adopt Table 13 to Table 16 to indicate antenna port(s) for Rel.18 eType 2 PUSCH DMRS with maxLength=2.

  - Scheduling Restrictions

In RAN1 #112 meeting, the following proposal was discussed but no agreement was achieved.

FL Proposal 2.5A - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 1) For PUSCH, there is no restriction. - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec. - Note: MU-MIMO restriction in Rel.17 is applied. - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group, - UE does not expect such MU-MIMO in a CDM group. - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following. - Alt.1: UE does not expect such MU-MIMO in a CDM group. - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports. - Dedicated UE capability is introduced. - The signaling is at least by RRC (FFS: whether to support DCI based signaling). - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

For PUSCH, Rel.15 DMRS ports and Rel.18 DMRS ports can be scheduled in one CDM group if the interference between them can be removed by the gNB. Otherwise, Rel.15 DMRS ports and Rel.18 DMRS ports are not scheduled in one CDM group. Therefore, it is up to gNB implementation and there is no restriction.

For Rel.18 UE1 indicated with Rel.18 legacy ports and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group, the interference between different DMRS ports can be removed by both UEs, since the receiver of the two UEs can remove this kind of interference. Therefore, no additional restriction is needed.

For Rel.18 UE1 indicated with Rel.18 new ports and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group, considering that Rel.15 UE may not have the capability to remove the interference from Rel.18 new ports, UE does not expect such MU-MIMO in a CDM group.

For Rel.18 UE1 indicated with Rel.18 new ports and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, considering that Rel.18 UE have the capability to remove the interference from Rel.18 new ports, no additional restriction is needed.

Proposal 11: For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- 1) For PUSCH, there is no restriction.

- 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel.18 legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

  - Note: MU-MIMO restriction in Rel.17 is applied.

- 3) For PDSCH, between Rel.18 UE1 indicated with Rel.18 new ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

  - UE does not expect such MU-MIMO in a CDM group.

- 4) For PDSCH, between Rel.18 UE1 indicated with Rel.18 new ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, Alt.3 is supported.

  - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

  - Orphan RE handling

With regard to orphan RE handling, there are two remaining issues. The first issue is whether restriction on scheduling of different UEs in case of MU-MIMO is needed or not. According to current specification, the UE does not expect the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be misaligned in the PRG-level grid to this UE with PRG=2 or 4. With this rule and the already agreed scheduling restriction for single user, orphan RE issue can be avoided in MU-MIMO case.

The second issue is scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. Since different TRPs (TCI states) are used for different frequency parts, joint channel estimation can only be done within each frequency part. Accordingly, restriction 1 and 2 agreed in RAN1#110b-e meeting are needed for each frequency part.

Proposal 12: For orphan RE issue, restriction on scheduling of different UEs in case of MU-MIMO is not needed.

Proposal 13: For M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme, the following scheduling restrictions are needed for each frequency part:

- The number of consecutively scheduled PRBs for PDSCH is even.

- The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.

  - Sequence mapping

With length 4 FD-OCC, patterns of Rel.18 eType 1 DMRS and eType 2 DMRS can be designed as same as that in Rel.15 type 1 DMRS and type 2 DMRS.

In Rel.15 DMRS, sequence rm is mapped to resource elements k,lp,μ according to equation (1):

αk,lp,μ=βPDSCHDMRSwfk'wtl'r2n+k'k=4n+2k'+∆ Configuration type 16n+k'+∆ Configuration type 2k'=0,1l=l+l'n=0,1,…                                                               (1)

Parameter k' is included in wfk' , k and r2n+k'. In wfk', k' is the index of OCC weighting. In parameter k, k' determines the frequency resources (subcarriers) used for DMRS transmission.

In Rel.18 DMRS with length 4 FD-OCC, four values of k'(e.g., k0, k1, k2 and k3) are needed, and they are corresponding to four resource elements in frequency domain, respectively.

In Rel.18 eType 2 DMRS, four values of k' can be 0, 1, 6 and 7 to facilitate resource mapping in frequency domain. Take CDM group 0 as an example, these values can ensure that DMRS occupies the 1st, 2nd, 7th, and 8th REs in one RB. However, these values of  k'  will complicate sequence generation of rm. For example, UE may need to generate longer PN sequence than needed if r8n+k' is used in resource mapping equation, since not all values in the generated sequence are used for DMRS transmission. On the other hand, sequence orthogonality between multiple ports may be an issue due to the same reason (discontinuous values in the generated sequence are used for DMRS transmission).

Four values of k' can also be 0, 1, 2 and 3 to simplify sequence generation, and r4n+k' is used in resource mapping equation. In order to ensure that DMRS occupies the 1st, 2nd, 7th, and 8th REs in one RB for CDM group 0, parameter k can be modified, and equation (2) is used in sequence mapping.

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=12n+k'+∆,k'=0,1 12n+k'+4+∆,k'=2,3k'=0,1,2,3l=l+l'n=0,1,…                                                               (2)

In Rel.18 eType 1 DMRS, four values of k' can also be 0, 1, 2 and 3, and equation (3) can be used in sequence mapping.

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=8n+2k'+∆k'=0,1,2,3l=l+l'n=0,1,…                                                             (3)

Proposal 14: The following sequence mapping equations are adopted for Rel.18 eType 1 DMRS and Rel.18 eType 2 DMRS, respectively:

- Rel.18 eType 1 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=8n+2k'+∆k'=0,1,2,3l=l+l'n=0,1,…

- Rel.18 eType 2 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=12n+k'+∆,k'=0,1 12n+k'+4+∆,k'=2,3k'=0,1,2,3l=l+l'n=0,1,…

Enhancements for UL 8Tx

In RAN1 #112 meeting, the following agreements were achieved on DMRS and PTRS for UL 8Tx.

Agreement For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping. - Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4 Table 4 Different RE offsets set for different Rel.18 DMRS port indexes DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH). DM-RS Configuration type 1; DM-RS Configuration type 2. resourceElementOffset; resourceElementOffset. offset00; offset01; offset10; offset11; offset00; offset01; offset10; offset11. 0; 0; 2; 6; 8; 0; 1; 6; 7. 1; 2; 4; 8; 10; 1; 6; 7; 0. 2; 1; 3; 7; 9; 2; 3; 8; 9. 3; 3; 5; 9; 11; 3; 8; 9; 2. 4; -; -; -; -; 4; 5; 10; 11. 5; -; -; -; -; 5; 10; 11; 4. 8; 4; 6; 10; 0; -; -; -; -. 9; 6; 8; 0; 2; -; -; -; -. 10; 5; 7; 11; 1; -; -; -; -. 11; 7; 9; 1; 3; -; -; -; -. 12; -; -; -; -; 6; 7; 0; 1. 13; -; -; -; -; 7; 0; 1; 6. 14; -; -; -; -; 8; 9; 2; 3. 15; -; -; -; -; 9; 2; 3; 8. 16; -; -; -; -; 10; 11; 4; 5. 17; -; -; -; -; 11; 4; 5; 10. Working assumption - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. Agreement - For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS.

In this section, DMRS indication and design of PTRS for UL 8Tx are discussed.

  - DMRS indication for UL 8Tx

In Rel.17, same designs of DMRS are used for PDSCH and PUSCH in CP-OFDM, except for the following indication mechanisms:

- For PUSCH, DMRS is indicated from ports combinations with total ports number equals to the number of layers indicated by TPMI (for codebook based PUSCH)/SRI(for non-codebook based PUSCH);

- For PDSCH, DMRS is indicated from all ports combinations.

In RAN1#111 meeting, it was agreed that for > 4 layers PUSCH, new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS are supported, and for the new antenna ports tables,

- For Type 1/Type 2 Rel.15 DMRS ports, the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

- For Rel.18 eType1/eType2 DMRS ports, new antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8

For up to 4 layers 4Tx PUSCH transmission with partial coherent UL codebook in Rel.15, layers associated to the same antenna port group can be mapped to different DMRS CDM groups. For example, if the DMRS for PUSCH is configured with dmrs-Type=1, maxLength=2 and 1 front-load symbol, DMRS ports 0-2 would be used for PUSCH transmission with rank = 3. When TPMI = 0 for rank 3 is indicated, layer 1 and layer 3 are associated to the same antenna port group, and layer 1 is mapped to DMRS port 0 in DMRS CDM group 0 and layer 3 is mapped to DMRS port 2 in DMRS CDM group 1. When TPMI = 1/2 for rank 3 is indicated, layer 2 and layer 3 are associated to the same antenna port group, and layer 2 is mapped to DMRS port 1 in DMRS CDM group 0 and layer 3 is mapped to DMRS port 2 in DMRS CDM group 1. That is, no matter which TPMI for rank 3 in partial coherent UL codebook is indicated, the two layers associated to the same antenna port group would be mapped to two CDM groups respectively. For > 4 layers PUSCH with partial-coherent UL codebook, it is also not make sense to restrict the layers associated to the same antenna port group to be always multiplexed into the same DMRS CDM group.

Proposal 15: For > 4 layers PUSCH with Type 1/Type 2 Rel.15 DMRS ports and partial coherent UL codebook, the layers associated to the same antenna port group can be multiplexed into the same DMRS CDM group or be mapped to different DMRS CDM groups.

In Rel.15, PUSCH and PDSCH use the same DMRS port combinations for rank = 1, 2, 3, 4 for all codebook subsets and all DMRS types. For PUSCH transmission with rank = 5, 6, 7, 8, it is natural to use the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH for all codebook subsets and all DMRS types.

Proposal 16: For > 4 layers PUSCH with Type 1/Type 2 Rel.15 DMRS ports, the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH is reused for partial coherent UL codebook.

Proposal 17: For > 4 layers PUSCH with Rel.18 eType1/eType2 DMRS ports, the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH is used.

  - PTRS-DMRS association for UL 8Tx

For PUSCH transmission with rank = 5, 6, 7, 8 for a partial-coherent UE or a non-coherent UE, 2 PTRS ports can be configured. Each of the 2 PTRS ports can be associated with up to 4 layers. Therefore 2 bits for each PTRS port are needed. One possible solution is to extend the size of PTRS-DMRS association field to be 4 bits, with 2 bits for UL PTRS port 0 and 2 bits for UL PTRS port 1, respectively. For example, the following table can be considered for PTRS-DMRS association field:

| Value of the most significant 2 bits | DMRS port | Value of the least significant 2 bits | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

This solution requires 2 more bits in DCI for PTRS indication.

For PUSCH transmission with rank = 5, 6, 7, 8, if the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH are used, there is only one or two DMRS port combinations for each rank. Since the bitwidth of Antenna port filed in DCI format 0_1/0_2 is 3 to 5 bits, the Antenna port filed can be used to indicate the mapping of PTRS ports and DMRS ports. One solution is to use the 2-bit PTRS-DMRS association field for one UL PTRS port and 2 bits in Antenna port filed for another UL PTRS port when the layers of PUSCH are associated with 2 UL PTRS ports. Then the size of DCI would not be increased, and the 2-bit PTRS-DMRS association filed can be used to indicate the DMRS port for PTRS transmission when the layer(s) of PUSCH is associated with one UL PTRS port as that for Rel.15. One example is shown as follows:

- The PTRS-DMRS association field in DCI format 0_1/0_2 is associated with UL PTRS port 0 as follows:

| Value | DMRS port |
| --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |
| 1 | 2nd DMRS port which shares PTRS port 0 |
| 2 | 3rd DMRS port which shares PTRS port 0 |
| 3 | 4th DMRS port which shares PTRS port 0 |

- 2bits of Antenna port field in DCI format 0_1/0_2 are associated with UL PTRS port 1 as follows:

| Value | DMRS port |
| --- | --- |
| 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 1 |

Proposal 18: For > 4 layers PUSCH configured with maximum 2 PTRS port, the size of PTRS-DMRS association field is 2 bits in DCI format 0_1/0_2,

- For PUSCH transmission with rank > 4,  the PTRS-DMRS association field and 2 bits in Antenna port filed are used for the indication of UL PTRS port 0 and UL PTRS port 1 respectively;

- For PUSCH transmission with rank = 1, 2, 3, 4,  the PTRS-DMRS association field is used for DMRS indication for UL PTRS port(s), with legacy indication schemes reused.

Since PTRS is a single layer transmission associated with PUSCH layer “x”/“y”, when multiple PUSCH layers share the same PTRS port, the transmission power of the PTRS port might be boosted by borrowing power from other PUSCH layers (layers other than “x”/“y”) that sharing the same PTRS port. When the actual number of PTRS ports for a PUSCH transmission is 2, the two PTRS ports would be mapped to different REs in an FDMed manner. Then the transmission power of a PTRS port also can be boosted by borrow power from the muted REs of the PTRS port due to the transmission of the other PTRS port.

In Rel.15, the transmission power of PTRS is scaled with a factor  , where is the PUSCH to PT-RS power ratio per layer per RE and  is given by , with shown in Table 6.2.3.1-3 in TS 38.214 as follows:

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

In the above table, Qp is the number of PTRS ports configured to the UE.

The UE shall assume ptrs-Power is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH.

For the design of PTRS power boosting in Rel.15, row "00" assumes the maximum transmit power of each PA of the UE equals to Pmax/N and row “01” assumes each PA of the UE is a full power PA, where Pmax is the maximum transmit power corresponding to UE’s power class, and N is the maximum number of SRS ports supported by the UE in one SRS resource.

For PTRS power boosting for UL 8Tx, the two UE capabilities also should be considered. Then, for row “01”, the transmission power of a PTRS port can be borrowed from all other layers, i.e., the value of αlayerPUSCHfor PUSCH transmission with RI= nlayerPUSCH is 10log10nlayerPUSCH dB.

For row “00” with UL 8Tx, by assuming the maximum transmit power of each PA of the UE equals to Pmax/N, the following principles can be considered:

- For non-codebook based PUSCH transmission: the value of is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered). That is because for non-codebook based PUSCH transmission, whether the two PUSCH layers sharing the same PTRS port are transmitted with the same PA(s) is up to UE’s implementation, which is unknown to gNB. Therefore the gNB shall not assume a PTRS port can borrow power from other PUSCH layers sharing the same PTRS port, but only can borrow power from muted REs due to the transmission of the other PTRS port(s).

- For full coherent codebook based PUSCH transmission: the value of αlayerPUSCHfor a PTRS port for a PUSCH transmission with RI= nlayerPUSCH is 10log10nlayerPUSCH dB. That is because for full coherent codebook based PUSCH transmission, all PUSCH layers share the same PAs and the same PTRS port. UE is capable to borrow transmit power from all other PUSCH layers.

- For non-coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered). That is because for non-coherent codebook based PUSCH transmission, different  PUSCH layers would be transmitted with different PAs,  UE can’t borrow power from PUSCH layers sharing the same PTRS port, but only can borrow power from muted REs due to the transmission of the other PTRS port(s).

- For partial-coherent codebook based PUSCH transmission: the value of αlayerPUSCHfor a PTRS port for a PUSCH transmission with RI= nlayerPUSCH is 10log10minLxQp,nlayerPUSCH dB, where Lx is the number of PUSCH layers sharing the same non-zero power antenna ports as the PUSCH layer the PTRS port is associated to (i.e., the PUSCH layer indicated by “PTRS-DMRS association” field). Note that according to the agreements on the principle for partial-coherent codebook design for UL 8Tx, any two PUSCH layers would either share the same non-zero power antenna port(s) or not share any non-zero power antenna port, and it is possible that 4 antenna ports that can transmit coherently would be divided into two groups, with each group associated with a PUSCH layer respectively. That means that a PTRS port can borrow power from the PUSCH layers sharing the same non-zero power antenna port(s) as the PUSCH layer the PTRS port is associated to, as well as the power from muted REs due to the transmission of the other PTRS port(s). However, a PTRS port cannot borrow power from a PUSCH layer with non-zero antenna ports  different to the non-zero antenna ports of the PUSCH layer the PTRS port is associated to, no matter whether the non-zero antenna ports of the two PUSCH layers are within the same or different coherent groups. That is because different PAs used by different antenna ports is assumed. The minimum operation in the formula is necessary since it is possible that the number of PUSCH layers sharing the same non-zero power antenna port(s) for a PTRS port is larger than the number of PUSCH layers sharing the other PTRS port(s).

Proposal 19: For 8Tx PUSCH, when higher layer parameter ptrs-Power is set to state "00", the PUSCH to PTRS power ratio per layer per RE (αlayerPUSCH) for PUSCH transmission with RI= nlayerPUSCH is 10log10nlayerPUSCH dB.

Proposal 20: For 8Tx PUSCH, when higher layer parameter ptrs-Power is set to state "01", specify PUSCH to PTRS power ratio per layer per RE (αlayerPUSCH) based on the following principles:

- For non-codebook based PUSCH transmission: the value of  is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered), where Qp is the number of PTRS ports configured to the UE;

- For full coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10nlayerPUSCH dB, where nlayerPUSCH is the number of PUSCH layers;

- For non-coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered), where Qp is the number of PTRS ports configured to the UE;

- For partial-coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10minLxQp,nlayerPUSCH dB, where Lx is the number of PUSCH layers sharing the same non-zero power antenna ports as the PUSCH layer the PTRS port is associated to, Qp is the number of PTRS ports configured to the UE and nlayerPUSCH is the number of PUSCH layers.

Conclusions

In this contribution, enhancements on increasing number of orthogonal DMRS ports and enhancements for UL 8Tx are discussed. Based on the above discussion, we have the following proposals:

Proposal 1: For Rel.18 eType 1 and eType 2 DMRS, port combinations  {8, 10, 11} and {12, 14, 15} can also be supported for S-DCI based M-TRP.

Proposal 2: For Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for 1 CW, rows 21-23 in Table 1 are supported.

Proposal 3: For Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, Table 2 is adopted to indicate antenna port(s).

Proposal 4: For Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, Table 3 is adopted to indicate antenna port(s).

Proposal 5: For Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, Table 4 is adopted to indicate antenna port(s).

Proposal 6: For Rel.18 eType 1 and eType 2 PUSCH DMRS, layer combination 1+2 is supported.

- {0, 2, 3} and {8, 10, 11} are supported for Rel.18 eType 1 PUSCH DMRS.

- {0, 2, 3} and {12, 14, 15} are supported for Rel.18 eType 2 PUSCH DMRS.

Proposal 7: Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the following rows are supported.

- Row 7 for rank = 2.

- Row 1 for rank = 3.

- Row 1 for rank = 4.

Proposal 8: Adopt Table 5 to Table 8 to indicate antenna port(s) for Rel.18 eType 1 PUSCH DMRS with maxLength=2.

Proposal 9: Adopt Table 9 to Table 12 to indicate antenna port(s) for Rel.18 eType 2 PUSCH DMRS with maxLength=1.

Proposal 10: Adopt Table 13 to Table 16 to indicate antenna port(s) for Rel.18 eType 2 PUSCH DMRS with maxLength=2.

Proposal 11: For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- 1) For PUSCH, there is no restriction.

- 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel.18 legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

  - Note: MU-MIMO restriction in Rel.17 is applied.

- 3) For PDSCH, between Rel.18 UE1 indicated with Rel.18 new ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

  - UE does not expect such MU-MIMO in a CDM group.

- 4) For PDSCH, between Rel.18 UE1 indicated with Rel.18 new ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, Alt.3 is supported.

  - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

Proposal 12: For orphan RE issue, restriction on scheduling of different UEs in case of MU-MIMO is not needed.

Proposal 13: For M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme, the following scheduling restrictions are needed for each frequency part:

- The number of consecutively scheduled PRBs for PDSCH is even.

- The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.

Proposal 14: The following sequence mapping equations are adopted for Rel.18 eType 1 DMRS and Rel.18 eType 2 DMRS, respectively:

- Rel.18 eType 1 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=8n+2k'+∆k'=0,1,2,3l=l+l'n=0,1,…

- Rel.18 eType 2 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=12n+k'+∆,k'=0,1 12n+k'+4+∆,k'=2,3k'=0,1,2,3l=l+l'n=0,1,…

Proposal 15: For > 4 layers PUSCH with Type 1/Type 2 Rel.15 DMRS ports and partial coherent UL codebook, the layers associated to the same antenna port group can be multiplexed into the same DMRS CDM group or be mapped to different DMRS CDM groups.

Proposal 16: For > 4 layers PUSCH with Type 1/Type 2 Rel.15 DMRS ports, the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH is reused for partial coherent UL codebook.

Proposal 17: For > 4 layers PUSCH with Rel.18 eType1/eType2 DMRS ports, the same DMRS port combination(s) as that for rank = 5, 6, 7, 8 for PDSCH is used.

Proposal 18: For > 4 layers PUSCH configured with maximum 2 PTRS port, the size of PTRS-DMRS association field is 2 bits in DCI format 0_1/0_2,

- For PUSCH transmission with rank > 4,  the PTRS-DMRS association field and 2 bits in Antenna port filed are used for the indication of UL PTRS port 0 and UL PTRS port 1 respectively;

- For PUSCH transmission with rank = 1, 2, 3, 4,  the PTRS-DMRS association field is used for DMRS indication for UL PTRS port(s), with legacy indication schemes reused.

Proposal 19: For 8Tx PUSCH, when higher layer parameter ptrs-Power is set to state "00", the PUSCH to PTRS power ratio per layer per RE (αlayerPUSCH) for PUSCH transmission with RI= nlayerPUSCH is 10log10nlayerPUSCH dB.

Proposal 20: For 8Tx PUSCH, when higher layer parameter ptrs-Power is set to state "01", specify PUSCH to PTRS power ratio per layer per RE (αlayerPUSCH) based on the following principles:

- For non-codebook based PUSCH transmission: the value of  is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered), where Qp is the number of PTRS ports configured to the UE;

- For full coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10nlayerPUSCH dB, where nlayerPUSCH is the number of PUSCH layers;

- For non-coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10Qp dB (or 3Qp-3 dB if up to 2 PTRS port is considered), where Qp is the number of PTRS ports configured to the UE;

- For partial-coherent codebook based PUSCH transmission: the value of αlayerPUSCH is 10log10minLxQp,nlayerPUSCH dB, where Lx is the number of PUSCH layers sharing the same non-zero power antenna ports as the PUSCH layer the PTRS port is associated to, Qp is the number of PTRS ports configured to the UE and nlayerPUSCH is the number of PUSCH layers.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94-e, December 6th – 17th, 2021.