3GPP TSG RAN WG1 #112bis-e			R1-2302684

e-Meeting, April 17th – April 26th, 2023

Source: 	CATT

Title:	Discussion on Rel-18 SRS enhancement

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

- Introduction

In previous meetings, it was agreed to specify SRS enhancements for TDD CJT and UL 8Tx, and several agreements have been reached. Specifically, for SRS enhancement for TDD CJT, both cyclic shift hopping and comb offset hopping are supported. For SRS enhancement for UL 8Tx, both 8-port SRS resource with the 8 ports mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof) and 8-port SRS resource with the resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot are supported.

In this contribution, we present our views on remaining issues on the design of SRS for TDD CJT and UL 8Tx operation.

- Discussion

  - SRS enhancement targeting TDD CJT

In previous meetings, it was agreed to specify SRS enhancements targeting TDD CJT, and several agreements were reached.

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity nIDSRS. - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index l'. - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame.

In this contribution, we provide our views on the design of SRS for TDD CJT.

Combined cyclic shift hopping and comb offset hopping

In the previous meetings, both comb offset hopping and cyclic shift hopping were supported for randomization of SRS interference. We support the combination of cyclic shift hopping and comb offset hopping for a UE, as the orthogonal resources that can be hopped by SRS ports increase. By using these orthogonal resources, the probability of interference decreases and the degree of interference randomization increases. This is because as the number of resources that can be hopped by SRS ports increases, the interference becomes more randomized.

When using the combination of cyclic shift hopping and comb offset hopping, cyclic shift hopping and comb offset hopping are configured independently. That is, they are configured with two different initialization IDs, and they follow their own pseudo-random sequence. Cyclic shift hopping and comb offset hopping can be turned on at the same time, but they are operated independently of each other. The orthogonal resources occupied by the SRS port are determined by both of them. For example, there are totally 4 comb offsets, the hopping pattern for comb offset hopping is based on the pseudo-random sequence c(i). Comb offset hops to the second comb offset on the current OFDM symbol. There are a total of 12 cyclic shifts, and the hopping pattern for cyclic shift hopping is based on the pseudo-random sequence c(i). It is independent of comb offset hopping. Cyclic shift hops to the 10th cyclic shift on the current OFDM symbol. When comb offset hopping and cyclic shift hopping are turned on at the same time, the SRS port occupies the second comb offset in the frequency domain and the 10th cyclic shift in the code domain on the current symbol. This is a simple procedure without increasing UE implementation complexity.

Proposal 1: Support the combination of comb offset hopping and cyclic shift hopping.

The initialization of the pseudo-random sequence

For the initialization of pseudo-random sequence c(i), we support option 2. That is, the initialization of the network-configured ID should be a new ID. Because comb offset hopping and/or cyclic shift hopping and group hopping and/or sequence hopping may be configured at the same time, reusing nIDSRS makes the configuration of SRS resource ID complex as all hopping features shall be considered jointly.

Comb offset hopping and cyclic shift hopping should use different IDs, because comb offset hopping and cyclic shift hopping are configured independently, and using different IDs can make configuration more flexible. If the two hoppings use different initialization IDs, comb offset hopping and cyclic shift hopping can flexibly adjust the initialization IDs of their respective pseudo-random sequences.

Proposal 2: The initialization of the network-configured ID should be a new ID. Comb offset hopping and cyclic shift hopping should use two separate new IDs.

The time-domain hopping behavior

For the time-domain hopping behavior of SRS comb offset hopping and/or cyclic shift hopping, we support option 2. That is, within a slot, hopping is based on only the symbol index l'. For option 2, the value of comb offset hopping and cyclic shift hopping of SRS port at different OFDM symbols are different, and the probability of strong interference at all OFDM symbols is low. For option 1, the SRS transmission of several consecutive OFDM symbols is exactly the same. If strong interference occurs, the SRS on these OFDM symbols will all be interfered. Option 2 is better in terms of degree of interference randomization than option 1.

According to current specification, all existing signal sequences have re-initialization. Re-initialization at the beginning of each radio frame ensures that the c(i) of the newly added SRS can be aligned with the existing SRS so that they remain orthogonal over time. Thus re-initialization should be supported. We support initialization at the beginning of each radio frame, and it can also be flexibly extended to the beginning of every N radio frames for initialization as needed.

Proposal 3: Support hopping based on only the symbol index l' within a slot.

Proposal 4: At least support reinitialization at the beginning of each radio frame.

  - SRS enhancement for UL 8Tx

In previous RAN1 meetings, the following agreements were reached regarding 8Tx SRS for codebook based PUSCH transmission and antenna switching. In this section, we discuss resource mapping, configuration and power control for SRS enhancement for UL 8Tx.

Agreement For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support: - For comb 2, support 1 and 2 comb offsets - For comb 4, support 2 and [4] comb offset - For comb 8, support 4 comb offsets Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

    - 8-port SRS resource based on legacy non-TDMed schemes

In Rel-15, when a 4-port SRS resource is mapped to 2 comb offsets, SRS ports 1000 and 1002 are mapped to one comb offset, and SRS ports 1001 and 1003 are mapped to the other comb offset, i.e., SRS ports with consecutive indexes are mapped to different comb offsets. Even indexes are mapped to one comb offset, and odd indexes are mapped to the other comb offset. Such design results in a unified cyclic shift determination mechanism for 4-port SRS resources for a given comb value. The same principle can be extended to an 8-port SRS resource configured with 2 comb offsets, i.e., the two comb offset are mapped with SRS ports {1000, 1002, 1004, 1006} and {1001, 1003, 1005, 1007}, respectively. Similarly, for an 8-port SRS resource configured with 4 comb offsets, the 4 comb offsets can be mapped with SRS ports {1000, 1004}, {1001, 1005}, {1002, 1006} and {1003, 1007}, respectively.

Proposal 5: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- For 8 ports mapped to 2 comb offsets, the 2 comb offsets are mapped with SRS ports {1000, 1002, 1004, 1006} and {1001, 1003, 1005, 1007}, respectively.

- For 8 ports mapped to 4 comb offsets, the 4 comb offsets are mapped with SRS ports {1000, 1004}, {1001, 1005}, {1002, 1006} and {1003, 1007}, respectively.

It is known that for two SRS ports mapped to the same comb offset, the greater the difference between the cyclic shifts of the two SRS ports is, the better the performance of receiving the two SRS ports is. Therefore in Rel-15, the cyclic shift offsets of the SRS ports mapped to the same comb offset are evenly spaced and the largest possible offset values between the SRS ports are adopted. The same principle can be extended to 8-port SRS resources.

For an 8-port SRS resource based on legacy non-TDMed schemes configured with comb 2, both 1 and 2 comb offsets are supported. On 1 comb offset, the cyclic shift for each SRS port should be different. Since the maximum number of cyclic shifts nSRScs,max for comb 2 is 8, the 8 cyclic shifts shall be allocated to the 8 ports respectively. The following formula can be considered:

nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max

For an 8-port SRS resources based on legacy non-TDMed schemes configured with comb 4, the SRS ports are mapped to 2 comb offsets, with 4 SRS ports for each comb offset. Since the maximum number of cyclic shifts nSRScs,max is 12, the offset value of cyclic shifts between any two of the 4 SRS ports mapped to the same comb offset can be 3X, where X is integers depend on the port indexes of the SRS ports. If the 2 comb offsets are mapped with SRS ports of {1000, 1002, 1004, 1006} and {1001, 1003, 1005, 1007}, respectively, the following formula can be considered:

nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,max

For an 8-port SRS resources based on legacy non-TDMed schemes configured with comb 8, the SRS ports are mapped to 4 comb offsets, with 2 SRS ports for each comb offset. Similar as that for a 2-port SRS resource configured with comb 8, the cyclic shift offset between the two SRS ports mapped to the same comb offset can be 3. If the 4 comb offsets are mapped with SRS ports of {1000, 1004}, {1001, 1005}, {1002, 1006} and {1003, 1007} respectively, the following formula can be considered:

nSRScs,i=nSRScs+nSRScs,maxpi-10004NapSRS4 mod nSRScs,max

Proposal 6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), the following formulas for nSRScs,i are supported:

- For comb 2, nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max;

- For comb 4, nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,max;

- For comb 8, nSRScs,i=nSRScs+nSRScs,maxpi-10004NapSRS4 mod nSRScs,max.

For a 4-port SRS resource, nSRScs is used to indicate whether the 4 ports are mapped to 1 comb offset or 2 comb offsets when comb 2 or comb 4 is configured. When nSRScs∈nSRScs,max2, …, nSRScs,max-1, the 4 ports are mapped to 2 comb offsets, otherwise, the 4 ports are mapped to 1 comb offset. For an 8-port SRS resource configured with comb 2, similar mechanism can be considered, i.e., the number of comb offsets is indicated by nSRScs.

Proposal 7: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), when comb 2 is configured, the number of the comb offsets is indicated by nSRScs.

    - 8-port SRS resource based on TDMed schemes

According to the agreement in RAN1 #112 meeting, for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the 8 ports are equally partitioned into s subsets with each subset having 8/s different ports. Each of the m OFDM symbols has only one subset. And the existing resource mapping designed for 8/s ports on each OFDM symbol is reused. For an SRS resource used for codebook based PUSCH transmission, SRS ports for the same coherent group mapped to the same OFDM symbol(s) (if possible) or consecutive OFDM symbols (if mapped to the same OFDM symbols is not allowed) are desirable. Therefore for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s = 2, each subset can be the set of SRS ports for a coherent group of a partial-coherent codebook designed for an 8Tx UEs with Ng=2. If TDM factor s = 4 is adopted, each subset can be the set of SRS ports for a coherent group of a partial-coherent codebook designed for 8Tx UEs with Ng=4.

Proposal 8: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s,

- For s = 2, the SRS ports for the same coherent group of a partial-coherent codebook designed for 8Tx UEs with Ng=2 are mapped to the same OFDM symbol(s);

- For s = 4 (if supported), the SRS ports for the same coherent group of a partial-coherent codebook designed for 8Tx UEs with Ng=4 are mapped to the same OFDM symbol(s).

For a TDM based 8-port SRS resource, if the existing resource mapping designed for 4-port SRS resource is being reused, the ports of the 8-port SRS resource on an OFDM symbol shall be one-to-one mapped to the 4 ports of the 4-port SRS resource in an incremental order. That is, if the SRS ports in an OFDM symbol are arranged in ascending order as {a, b, c, d}, then ports a, b, c, d are mapped to the same frequency-domain resources and the same cyclic shifts as ports 1000, 1001, 1002, and 1003 of a 4-port SRS resource configured with the same nSRScs and comb value, respectively. For example, if SRS ports 1000, 1002, 1004, and 1006 of a TDM based 8-port SRS resource are transmitted on the same OFDM symbol, port 1000 would mapped to the same frequency-domain resources and with the same cyclic shift as port 1000 of a 4-port SRS resource configured with the same nSRScs and comb value, and port 1002 would mapped to the same frequency-domain resources and with the same cyclic shift as port 1001 of a 4-port SRS resource configured with the same nSRScs and comb value. Likewise, port 1004 would mapped to the same frequency-domain resources and with the same cyclic shift as port 1003 of a 4-port SRS resource configured with the same nSRScs and comb value, and port 1006 would mapped to the same frequency-domain resources and with the same cyclic shift as port 1003 of a 4-port SRS resource configured with the same nSRScs and comb value.

Proposal 9: For reusing the existing resource mapping designed for 4-port SRS resource to an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols, the ports of the 8-port SRS resource on an OFDM symbol are one-to-one mapped to the ports of an 4-port SRS resource in an incremental order.

In RAN1 #112 meeting, the following two options on TDM pattern for a TDM based 8-port SRS resource with TDM factor s ≥ 2 were provided:

- Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

- Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

For the two options, option 2-1 is preferred due to the following reasons:

- Compared to option 2-2, option 2-1 can complete the transmission of one repetition of all 8 ports earlier;

- When there is a large interference lasting for several OFDM symbols during the transmission of the SRS resource, it is possible that only a part of ports can be received with option 2-2, while one or more repetitions of all SRS ports can be received with option 2-1.

Proposal 10: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, regarding the TDM pattern, option 2-1 is adopted:

- Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

In Rel-17, the transmit power for an SRS resource is split equally across the antenna ports of the SRS resource. For a TDM based 8-port SRS resource, each OFDM symbol is mapped with part of SRS ports. At least for UEs with UE Capability 1 under Rel-16 UE capability definitions, they are capable of transmitting SRS resources in each OFDM symbols with full power, which can improve SRS coverage and reliability. Therefore, power control enhancement for SRS should be considered, e.g., splitting the transmit power for an SRS resource equally across the antenna ports mapped onto one OFDM symbol, instead of splitting the transmit power for an SRS resource equally across all antenna ports of the SRS resource.

Proposal 11: Power control enhancement should be considered for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols.

    - Configurations for an 8-port SRS resource

For an 8-port SRS resource, whether it is TDM based or is based on legacy non-TDMed schemes can be configured with an explicit RRC parameter indicating the TDM factor. If the SRS resource is based on legacy non-TDMed schemes, 1 is indicated by the RRC parameter, and if the SRS resource is based on TDMed schemes with TDM factor s, s is indicated by the RRC parameter.

Proposal 12: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, an explicit RRC parameter indicating the TDM factor is supported, with value “1” of the RRC parameter indicating the SRS resource is based on legacy non-TDMed schemes.

- Conclusions

In this contribution, we provided our views on the enhancement of SRS for Rel-18 TDD CJT and UL 8Tx operation. We have the following proposals:

Proposal 1: Support the combination of comb offset hopping and cyclic shift hopping.

Proposal 2: The initialization of the network-configured ID should be a new ID. Comb offset hopping and cyclic shift hopping should use two separate new IDs.

Proposal 3: Support hopping based on only the symbol index l' within a slot.

Proposal 4: At least support reinitialization at the beginning of each radio frame.

Proposal 5: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- For 8 ports mapped to 2 comb offsets, the 2 comb offsets are mapped with SRS ports {1000, 1002, 1004, 1006} and {1001, 1003, 1005, 1007}, respectively.

- For 8 ports mapped to 4 comb offsets, the 4 comb offsets are mapped with SRS ports {1000, 1004}, {1001, 1005}, {1002, 1006} and {1003, 1007}, respectively.

Proposal 6: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), the following formulas for nSRScs,i are supported:

- For comb 2, nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max;

- For comb 4, nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,max;

- For comb 8, nSRScs,i=nSRScs+nSRScs,maxpi-10004NapSRS4 mod nSRScs,max.

Proposal 7: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), when comb 2 is configured, the number of the comb offsets is indicated by nSRScs.

Proposal 8: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols and with TDM factor s,

- For s = 2, the SRS ports for the same coherent group of a partial-coherent codebook designed for 8Tx UEs with Ng=2 are mapped to the same OFDM symbol(s);

- For s = 4 (if supported), the SRS ports for the same coherent group of a partial-coherent codebook designed for 8Tx UEs with Ng=4 are mapped to the same OFDM symbol(s).

Proposal 9: For reusing the existing resource mapping designed for 4-port SRS resource to an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols, the ports of the 8-port SRS resource on an OFDM symbol are one-to-one mapped to the ports of an 4-port SRS resource in an incremental order.

Proposal 10: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, regarding the TDM pattern, option 2-1 is adopted:

- Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Proposal 11: Power control enhancement should be considered for an 8-port SRS resource with resource mapping based on TDM onto m ≥ 2 OFDM symbols.

Proposal 12: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, an explicit RRC parameter indicating the TDM factor is supported, with value “1” of the RRC parameter indicating the SRS resource is based on legacy non-TDMed schemes.

- References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94-e, December 6th – 17th, 2021.