3GPP TSG RAN WG1 #112bis-e                                                                           R1-2302685

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Further discussion on enhancements on UL precoding indication for multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

- Introduction

As approved in RAN#94-e meeting, multi-panel UL transmission related enhancements include the following objectives [1]:

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, enhancements on STxMP PUSCH + PUSCH transmission with single DCI and multi-DCI based and STxMP PUCCH + PUCCH transmission are discussed.

- Enhancements on STxMP PUSCH transmission

  - SDM based PUSCH transmission

In RAN1#112 meeting [2], maximal number of layers for sTRP vs SDM scheme and DCI field/codepoint design, the number of CWs, SRS resource set configuration, DMRS indication, PTRS configuration and dynamic switching for SDM scheme were discussed. In the following sections, PTRS-DMRS association scheme, dynamic switching issues and UL precoding schemes will be discussed.

    - PTRS-DMRS association

In RAN1#112 meeting [2], following agreement was achieved:

Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission

According to the agreement, the remaining issue is whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission. For Rel-18, there is no parameter to configure the actual max PTRS port number. From the view of UE capability, as up to 2 PTRS ports are supported for SDM transmission, which can be the case for the multi-panel transmission scenario, it is necessary to configure additional parameter to indicate the actual max PTRS port number.

Proposal 1: Configure additional RRC parameter to indicate the actual max PTRS port number (1 or 2).

2.1.2 Dynamic switching between SDM and sTRP transmission

In RAN1#112 meeting [2] and RAN1#111 meeting [3], following agreements and working assumptions for the configuration of maximal number of layers for dynamic switch between sTRP vs SDM and DCI design have been made:

Agreement For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. - FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding. Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

According to the agreements, remaining issues include the determination on the maximal number of layers for sTRP transmission, how to interpret each codepoint of “SRS resource set indicator” and determining the size of each DCI field and aligning the DCI size for sTRP transmission and SDM transmission.

Regarding the maximal number of layers for sTRP transmission, the working assumption for the maximal number of layers for SDM and sTRP should be supported. From capability point of view, up to four layers can be supported in sTRP transmission. In order to support four layers in non-codebook based PUSCH transmission, four SRS resources are needed in one SRS resource set. For codebook based PUSCH transmission, if ul-FullPowerTransmission = fullpowerMode1 and transform precoder is disabled, 1 more bit is needed by TPMI field if four layers are supported in sTRP transmission (as {1+3} and {3+1} combinations are not supported by SDM in Rel-18). However, these issues can be resolved by specific design. Considering that four-layer transmission has already been supported in sTRP transmission when TDM based PUSCH repetition is performed, and it can bring performance gain, the max number of layers can be four when UE switches to sTRP transmission.

Proposal 2: The max number of layers can be four when UE switches to sTRP transmission.

For the maximal number of layers for sTRP transmission, one straightforward way is that the maximal number of layers of sTRP transmission can be configured by the maxRank (or Lmax). In this case, the maximal number of layers is aligned with the max rank to save potential extra UE cost. Similarly, for SDM transmission, configure one maximal number of layers can save the configuration overhead.

Proposal 3: Regarding the dynamic switching between STxMP SDM scheme and sTRP transmission, working assumption is confirmed.

Regarding the FFS issue for whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same, it is our view that the total number of antenna ports for the SDM and sTRP can be different. However, firstly, we should clarify that the two panels can transmit different number of ports. In this case, it is straightforward that the total number of used PUSCH antenna ports for SDM or sTRP can be different, thus it is not necessary to restrict the number of both to be the same.

Proposal 4: The total number of used PUSCH antenna ports for the SDM and sTRP can be different.

Regarding how to interpret each codepoint of “SRS resource set indicator” which are 00, 01, 10 and 11, and for each codepoint, how to interpret the TPMI field for CB PUSCH or SRI field for NCB PUSCH, the indication table from Rel-17 can be reused accordingly. In detail, following interpretations are proposed: The SRS resource set indicator codepoint 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively. The first TPMI indicates the precoding/rank for CB PUSCH or first SRI field indicates precoding/rank NCB PUSCH. (The second field can be reserved/unusedt). SRS resource set indicator codepoint 10 indicates SDM with first SRS resource set and second SRS resource set. The first TPMI or SRI field and the second TPMI or SRI field indicate the precoding/rank/SRI for the first SRS resource set and the second SRS resource set for CB or NCB respectively. The codepoint 11 of SRS resource set indicator is reserved or unused.

Proposal 5: For the codepoint of the “SRS resource set indicator”, use the following interpretations:

- SRS resource set indicator codepoint 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively.

- Alt1: 1st TPMI indicates the precoding/rank for CB PUSCH or 1st SRI field indicates precoding/rank NCB PUSCH. (2nd field can be reserved/unused or absent)

- SRS resource set indicator codepoint 10 indicates SDM with 1st SRS resource set and 2nd SRS resource set. The 1st TPMI or SRI field and 2nd TPMI or SRI field indicates the precoding/rank/SRI for the 1st SRS resource set and 2nd SRS resource set for CB or NCB respectively

- The codepoint 11 of SRS resource set indicator:

- Alt1: reserved/unused

Regarding the design for TPMI/SRI field of DCI, to clarify the statement, the first TPMI/SRI field for sTRP transmission aligns with the first TPMI/SRI field of SDM transmission. One possible method is that the bitwidth of the first TPMI/SRI field is determined based on the maximal number of layers of sTRP when the DCI indicates sTRP transmission, while the bitwidth of the first TPMI/SRI is determined based on the maximal number of layers of the first SRS resource set in SDM when the DCI indicates SDM transmission. Compared with another potential solution that the bitwidth of the first TPMI/SRI field is determined based on maximal value for either the maximal number of layers of sTRP or the SDM transmission, this can reduce the overhead of the DCI signaling. When UE is configured with maxRank=3 or 4 for sTRP transmission, and codebookSubset = partialAndNonCoherent, using the above solution, it can confirm that the first TMPI field contains 6 bits while the second TMPI field contains 4 bits (or reserved). In this way, 1 bit is saved compared with the above alternative solution that leads to a 6+5 bits combination.

Proposal 6: The size of the first TPMI/SRI is determined based on the maximal number of layers of sTRP when the DCI indicates sTRP transmission, while the size of the first TPMI/SRI is determined based on the maximal number of layers of the first SRS resource set in SDM when the DCI indicates SDM transmission.

    - UL precoding indication

In RAN1#110b-e meeting [4], following agreement was achieved:

Agreement For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB . - FFS : These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based.

One remaining issue is whether these two SRS resource sets can have different number of SRS resources for CB or NCB. In Rel-17 TDM based PUSCH repetition, two SRS resource sets have the same number of SRS resource sets for both CB and NCB, and two SRS resource sets are corresponding to different panels. In SDM transmission, two SRS resource sets are also related to two panels. Therefore, these two schemes can have similar SRS resource sets to simplify design. Accordingly, two SRS resource sets have same number of SRS resources.For Rel-18 multi-panel transmission, it is up to capability of UE to choose the number of the transmitted number of ports, thus it is reasonable for two SRS resources to have different number of SRS ports. Another issue is whether two SRS resources indicated by the two SRI fields can have different number of SRS ports in CB based PUSCH. Usually, layers transmitted by different panels are sent to different TRPs and decoded by different TRPs separately. Therefore, there is no need to restrict the number of SRS ports indicated by SRI fields. For example, if SRS resources in one SRS resource set have different numbers of SRS ports (e.g., when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2'), the indicated SRS resources by two SRI fields can have different number of SRS ports. If all SRS resources in two SRS resource sets have the same number of SRS ports, the indicated SRS resources by two SRI fields can have the same number of SRS ports.

Proposal 7: For CB-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports.

  - SFN based PUSCH transmission

In RAN1#111 meeting, it was agreed to support SFN based PUSCH transmission. In the following sections, we will discuss the DMRS indication scheme, PTRS-DMRS association scheme, dynamic switching issues and UL precoding schemes for SFN based PUSCH transmission.

2.2.1 DMRS indication

As SFN scheme is agreed to be used in STxMP transmission, the associated DMRS indication should be discussed. the transmission schemes between the SFN and the SDM are different. In SDM, if the antenna ports indicate port 0 and port 1, then the two panels will use port 0 and port 1 separately. By contrast, for the SFN scheme, if the antenna ports indicate port 0 and port 1, then the two panels will use both port 0 and port 1. As the DMRS ports for the two panels are the same, the DMRS ports can be determined by the antenna port indication for both panels. In our view, the DMRS indication for the SFN should be the same or similar to the indication method in TDM scheme.

Proposal 8: For SFN based PUSCH transmission, use the same DMRS indication as for the TDM scheme.

2.2.2 PTRS-DMRS association

In RAN1#112 meeting [2], the following agreement was achieved:

Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner

According to the agreement, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB. In order to simplify design, it can be assumed that the coherent types of the two panels are the same for CB, and the actual number of PTRS port(s) can be determined by coherent type of the precoder indicated by the 1st TPMI field. In other words, PTRS-DMRS association method defined in Rel.15 can be reused, and UE doesn’t expect that coherent types indicated by the two TPMI fields are different.

Similarly, Rel.15 mechanism can also be reused for NCB. Parameter ptrs-PortIndex is configured for each SRS resource, and the actual number of PTRS port(s) depends on the number of PTRS port indexes indicated by the 1st SRI field. UE doesn’t expect that the number of PTRS port indexes indicated by two SRI fields are different.

Proposal 9: Rel-15 PTRS-DMRS association method is supported for SFN scheme assuming that:

  - For CB, same coherent type is indicated by the two TPMI fields.

  - For NCB, same number of PTRS port indexes are indicated by the two SRI fields.

2.2.3 Dynamic switching between SFN and sTRP transmission

In RAN1#111 meeting[3], the following agreement was achieved:

Agreement - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission.

The remaining issue is the indication detail for the SRS recourse set indicator. In our view, same configuration for SDM scheme can be used for SFN scheme. Specifically, for the SRS resource set indicator, codepoint 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively. The first TPMI indicates the precoding/rank for CB PUSCH and first SRI field indicates precoding/rank NCB PUSCH. (The second field can be reserved/unused). For SRS resource set indicator codepoint 10, it indicates SFN with first SRS resource set and second SRS resource set. The first TPMI or SRI field and the second TPMI or SRI field indicates the precoding/rank/SRI for the first SRS resource set and the second SRS resource set for CB or NCB respectively. The codepoint 11 of SRS resource set indicator is reserved or unused.

Proposal 10: In SFN scheme, use same configuration and codepoint interpretations for SRS resource set indicator field as SDM scheme.

In RAN1#112 meeting [2], the following agreement was achieved:

Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN

Regarding the separate parameter of the maximal number of layers for STxMP SFN scheme, one straightforward way is reuse the same configuration for SDM scheme. In this case, the parameter can be configured for both the first SRS resource set and the second SRS resource set.

Proposal 11: In SFN scheme, configure separate parameter of the maximal number of layers for STxMP SFN scheme.

  - M-DCI based PUSCH transmission

In RAN1#110b-e meeting, it was agreed to support STxMP PUSCH+PUSCH transmission in M-DCI based system in Rel-18. In this section, detailed designs in M-DCI based PUSCH transmission are provided.

    - Scheduling restrictions

In RAN1#110b-e meeting [4], the following agreements related to scheduling restrictions were achieved. According to the agreements, the number of layers scheduled by two PUSCHs and the power adjustments of two PUSCHs need further discussion.

Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2. Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occasion in the case of fully/partially overlapping in time domain

Similar to Rel-16 M-DCI based PDSCH scheduling, UE is not expected to assume different DMRS configurations with respect to the actual number of front-loaded DMRS symbol(s), the actual number of additional DMRS symbol(s), the actual DMRS symbol location, and DMRS configuration type.

Proposal 12: The two scheduled PUSCHs have the same DMRS configuration with respect to

  - the actual number of front-loaded DMRS symbol(s).

  - the actual number of additional DMRS symbol(s).

  - the actual DMRS symbol location.

  - DMRS configuration type.

Figure 1 shows two examples of power allocation methods for two panels in M-DCI based STxMP PUSCH transmission. In Figure 1.a, the maximum Tx power of each panel is either pre-defined or pre-configured (e.g., P2 for each panel, and P is the maximum transmit power of the UE or per-UE power limit), and the sum of these two maximum Tx powers is not larger than P. Note that the maximum Tx power of each panel is set to ensure that the total transmit power of two PUSCHs is small than P (i.e., scheduling restriction, similar to maximum number of layers), and it can be the same as the per-panel power limit (if supported) or different from the per-panel power limit. In one example, both per-panel power limit and per-UE power limit are P, and the maximum Tx power of each panel is P2 which is different from the per-panel power limit. In this power allocation method, although per-UE power limit can always be satisfied, the transmit power of the UE is not utilized efficiently.

In Figure 1.b, the maximum Tx power of each panel is either not defined or is equal to per-panel power limit. In this method, the total scheduled transmit power of two PUSCHs may be larger than per-UE power limit, thus, power adjustment between two PUSCHs is necessary. Different from maximum number of layers, transmit power can be adjusted or determined (e.g., power backoff) by UE. Therefore, there is no need to define the maximum Tx power for each panel, and power adjustment can be performed by UE to sufficiently utilize the transmit power.

Figure 1: Two examples of power allocation methods for two panels.

Proposal 13: Except for per-panel power limit (if supported), the maximum Tx power of each panel (scheduling restriction) is not defined or introduced for M-DCI based STxMP PUSCH transmission.

    - UL precoding indication

In RAN1#110b-e meeting [4], the following agreement related to precoding indication for M-DCI based STxMP PUSCH transmission was achieved. According to the agreements, the configuration of SRS resources, SRS ports, codebooks and maxRanks need further discussion.

Agreement Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly. - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB - FFS: the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH

In M-DCI based PUSCH transmission, two PUSCHs are independently scheduled by two TRPs. Each PUSCH is transmitted with Rel-15/16 NCB/CB transmission scheme. Therefore, each SRS resource set can be configured according to the rule defined in Rel-15/16.

For CB, when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', a maximum of 2 or 4 SRS resources can be configured in an SRS resource set. Besides, the maximum number of configured SRS resources is 2. Naturally, the maximal number of indicated SRS resources in each set for CB is one. In Rel-15, the maximal number of SRS ports in the configured SRS resource set is 4, which can also be reused by M-DCI based PUSCH transmission.

For NCB, the maximum number of SRS resources per SRS resource set that can be configured is 4. As we discussed in section 2.3.1, the maximum number of layers for each TRP can be configured by RRC signaling. Accordingly, the maximal number of indicated SRS resources in each set for NCB depends on the maximal supported layers for each TRP.

Proposal 14: The maximal number of configured SRS resources in each set for NCB/CB is 4.

- When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number is 2 or 4. Otherwise, the maximum number is 2.

Proposal 15: The maximal number of indicated SRS resources in each set for NCB depends on the maximal supported layers for each TRP.

Proposal 16: The maximal number of SRS ports in each set for CB is 4.

In Rel-16 M-DCI based PUSCH transmission, two scheduled PUSCHs that are non-overlapped in time domain are transmitted by one panel in different time instants or by two different panels in different time instants. These two PUSCHs are corresponding to the same SRS resource set and share the same precoding related parameters (e.g., maxRank, codebookSubset, and txConfig which are configured in parameter PUSCH-Config). In Rel-18 M-DCI based STxMP transmission, two PUSCHs are corresponding to two different panels, hence panel related parameters can be configured separately. If two panels have different coherent transmission capabilities, codebookSubset can be configured separately for two PUSCHs. As we discussed in section 2.3.1, the maximum number of layers for each TRP can be configured by RRC signaling, therefore, maxRank can also be configured separately for two PUSCHs. According to the agreement, each SRS resource set is corresponding to one TRP, and precoding method can be applied no matter same or different transmission schemes are adopted by two TRPs. Therefore, txConfig can also be configured separately for two SRS resource sets to not limit the scheduling flexibility of the gNB.

Proposal 17: Separate codebookSubsets, maxRanks and txConfigs are configured for different SRS resource sets.

    - Simultaneous transmission of PUSCH and PUSCH

For the UL STxMP transmission using M-DCI scheme, when two PUSCHs are overlapped in the time domain, if the total number of transmission layers of two PUSCHs is larger than the maximal number of layers for the mTRP, the two PUSCHs will cause collision. To solve such potential issue, UE should be able to avoid such collision. Two alternatives can be considered:

Alt 1: UE drops two collided PUSCHs.

Alt 2: UE transmits one of the PUSCHs within the maximal number of layers that has high priority signal/transmission time/layer(s).

Regarding alt 2, there are several potential ways for UE to decide which PUSCH has higher priority. Firstly, UE can choose different types of signals. For example, if the first PUSCH contains both data and UCI (ACK/NACK, SR or CSI) while the second PUSCH only contains data, UE will transmit the first PUSCH. Secondly, UE can choose different transmission times. E.g., if the start time of the first PUSCH is earlier than the second PUSCH, UE will transmit the first PUSCH. Another way is that UE can decide on the number of layers of the PUSCH. E.g., if the first PUSCH has more layers (within the maximal number of permitted layers) to transmit than the second PUSCH, UE will transmit the first PUSCH. Or if the first PUSCH is configured a transmission time with higher priority by gNB, UE will transmit the first PUSCH.

Proposal 18: If two overlapped PUSCHs collide due to the actual total number of layers of them is larger than the maximal number of layers for mTRP, UE chooses one of two following alternatives to solve the issue:

Alt 1: UE drops two collided PUSCHs.

Alt 2: UE transmits one of the PUSCHs within the maximal number of layers that has high priority signal/transmission time/layer(s).

In current specification, predefined rules are specified for UE to resolve overlapping of DG/CG PUSCHs with same or different priority indexes. In STxMP PUSCH transmission, enhancements on predefined rules are necessary for UE to determine more than one PUSCH to transmit.

For DG PUSCH, priority index is provided by priority indicator field in a DCI format. In M-DCI based STxMP PUSCH transmission, all DCI formats are supported when two different values of coresetPoolIndex are configured. Thus priority indexes can be indicated by two DCI formats associated with two TRPs. Similarly, parameter phy-PriorityIndex can be configured for a CG PUSCH. Therefore, priority indexes can be indicated for PUSCHs when M-DCI based STxMP PUSCH transmission is performed. Since CG PUSCH is designed for URLLC traffic and M-DCI based transmission is mainly for throughput enhancements, different priority indexes or different priorities (e.g., when parameter prioLowDG-HighCG or prioHighDG-LowCG is configured) can be supported for different PUSCHs.

Proposal 19: Different priority indexes or different priorities can be indicated for PUSCHs in M-DCI based STxMP PUSCH transmission.

If a UE would transmit multiple PUSCHs with same priority index, at most two PUSCHs can be scheduled or transmitted simultaneously. When a DG PUSCH is scheduled or a type 2 CG PUSCH is activated, UE can also transmit a type 1 CG PUSCH simultaneously. When the number of scheduled or activated DG PUSCHs and/or type 2 CG PUSCHs is 2, another CG PUSCH will not be transmitted simultaneously. The time interval between the third CG PUSCH and the transmitted PUSCHs (e.g., the first and the second PUSCH) should be defined to ensure that processing time of PUSCH is enough. The following two options can be considered:

- Option 1: The gap between the starting symbol of the third CG PUSCH and the ending symbol of the later PDCCH is N2 (shown in Figure 2), where the later PDCCH is the PDCCH which schedules or activates the first or the second PUSCH and ends later in time.

- Option 2: The gap between the starting symbol of the third CG PUSCH and the ending symbol of the associated PDCCH is N2 (shown in Figure 3), where the associated PDCCH and the third CG PUSCH are associated with the same value of parameter coresetPoolIndex.

Figure 2: The time interval between the third CG PUSCH and the simultaneous transmitted PUSCHs in option 1.

Figure 3: The time interval between the third CG PUSCH and the simultaneous transmitted PUSCHs in option 2.

Both options can ensure that processing time of PUSCH is enough. Since the scheduling is performed by each TRP independently, the processing time can also be defined within the transmission of one TRP.

Proposal 20: If multiple PUSCHs have same priority index or priority, the time interval between the other CG PUSCH and the simultaneously transmitted PUSCHs should be defined.

If a UE would transmit multiple PUSCHs with different priority indexes, more than two PUSCHs can be scheduled and only two PUSCHs can be transmitted. The transmission of low priority PUSCHs will be cancelled. The following conditions can be considered to determine which PUSCHs can be transmitted.

- The transmitted PUSCHs are associated with different panels (e.g., coresetPoolIndex or SRS resource set).

- The total number of layers of two transmitted PUSCHs is not larger than 4.

- The transmitted PUSCHs have larger priority indexes or larger priorities (corresponding to parameter prioLowDG-HighCG or prioHighDG-LowCG).

Proposal 21: If multiple PUSCHs have different priority indexes or priorities, coresetPoolIndex, SRS resource set, the number of layers or priority indexes can be used to determine which PUSCHs can be transmitted.

    - Simultaneous transmission of PUCCH and PUSCH

In RAN1#112 meeting [2], the following agreement related to UCI multiplexing was achieved, and UCI multiplexing rule needs further study.

Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH:

According to section 2.3.3, two PUSCHs can be determined to transmit simultaneously. UCI multiplexing can be performed after UE determines up to two transmitted PUSCHs to ensure that UCI can always be transmitted.

Figure 4 shows an example of simultaneous scheduling of PUCCH and PUSCH. One PUCCH (PUCCH1) and two PUSCHs (PUSCH1 and PUSCH2) would transmit in one slot and they are overlapping at some symbols. Since PUCCH1 and PUSCH1 are both transmitted for TRP1, PUCCH1 can be multiplexed on PUSCH1. In other words, UCI multiplexing can be performed within each TRP/panel. In STxMP PUSCH transmission, each PUSCH is associated with one TRP (e.g., through parameter coresetPoolIndex). However, in periodic CSI report, there is no association between PUCCH resource and TRP. How to determine the associated PUSCH for each PUCCH needs further study. Considering that TCI state and parameter coresetPoolIndex can be used to differentiate TRP in M-DCI based scheduling, the following association methods can be considered:

- Option 1: PUCCH and PUSCH are associated with each other if they have the same TCI state.

- Option 2: PUCCH and PUSCH are associated with each other if they are associated with the same coresetPoolIndex value.

Figure 4: An example of simultaneous scheduling of PUCCH and PUSCH

With unified TCI framework, the TCI states of each PUSCH and PUCCH are either the first indicated TCI state or the second indicated TCI state. Therefore, the UE can multiplex UCI in PUSCH which has the same TCI state as the PUCCH. However, if unified TCI framework is not used, new mechanism is still needed for UCI multiplexing. In option 2, the association depends on the parameter coresetPoolIndex, and the association between PUCCH and coresetPoolIndex is established. According to [2], the association between PUCCH and coresetPoolIndex is also studied to determine the corresponding TCI state for PUCCH. Actually, both association methods can work. In order to reduce standardization effort, option 2 is preferred and the association between PUCCH and coresetPoolIndex can be referred to the discussion in unified TCI session.

Proposal 22: UCI multiplexing can be performed after UE determines up to two simultaneously transmitted PUSCHs to ensure that UCI can always be transmitted.

Proposal 23: In M-DCI based STxMP PUSCH transmission, a UE multiplexes UCI/PUCCH on PUSCH which is associated with the same coresetPoolIndex value as the PUCCH.

- The association between PUCCH and coresetPoolIndex can be referred to the discussion/conclusion in unified TCI state session.

When there are multiple PUSCHs in multiple serving cells for UCI multiplexing and two PUSCHs are transmitted simultaneously in one serving cell, whether there is a need to enhance PUSCH selection procedure for UCI multiplexing should be discussed. In current specification, selection procedure (order) is specified, and PUSCH is selected according to the following order: aperiodic CSI > DG PUSCH > PUSCH with the smallest ServCellIndex > the earliest PUSCH that the UE transmits in the slot. In STxMP PUSCH transmission, both transmitted PUSCHs can be seen as candidate PUSCHs. Different selection rule will lead to different selection results. Take Figure 5 as an example, DG PUSCH 1 (corresponding to value 1 of coresetPoolIndex) and CG PUSCH 2 (corresponding to value 0 of coresetPoolIndex) are transmitted in CC 3, and CG PUSCH 1 is transmitted in CC 1. If two simultaneous PUSCHs are selected jointly, e.g., CG PUSCH 2 and DG PUSCH 1 can both be selected due to the high priority of DG PUSCH 1. Then UCI is multiplexed in CG PUSCH 2 since both UCI and CG PUSCH 2 are associated with value 0 of coresetPoolIndex. If two simultaneous PUSCHs are selected separately, UCI will be multiplexed in DG PUSCH 1 according to current PUSCH selection rule. All candidate PUSCHs can also be divided to two groups, and PUSCH is selected from the group which associates with the same value of coresetPoolIndex with the UCI.  From multiple-panel transmission point of view, it is preferred that UCI is multiplexed in CG PUSCH 2, which ensures that TCI state of UCI is not changed after UCI multiplexing. Therefore, we propose to enhance PUSCH selection rule in STxMP PUSCH transmission.

Figure 5: An example of PUSCH selection

Proposal 24: In UCI multiplexing with multiple serving cells, PUSCH selection rule shall be enhanced to ensure that the TCI state of UCI is not changed after UCI multiplexing.

- The detailed selection rule is for further study.

In Figure 4, both PUCCH and PUSCHs are scheduled simultaneously. After UCI multiplexing, PUCCH is not transmitted and two PUSCHs are transmitted. Figure 6 shows another example of simultaneous scheduling of PUCCH and PUSCH. Since PUCCH1 and PUSCH2 are associated with different TRPs, UE won’t multiplex UCI carried by PUCCH1 in the PUSCH2. However, according to WID, only PUSCH+PUSCH or PUCCH+PUCCH are supported. Therefore, mechanisms (e.g., multiplexing or dropping) are needed to avoid simultaneous transmission of PUCCH and PUSCH.

In the example of Figure 6, two channels may carry different types of information, have different priority indices, and be corresponding to different coresetPoolIndex values, CC index. Then the priority orders of the above-mentioned factors should be studied. On the other hand, UCI multiplexing between different TRPs can also be considered to ensure that information carried in both PUSCH and PUCCH can be transmitted.

Figure 6: Another example of simultaneous scheduling of PUCCH and PUSCH

Proposal 25: Mechanisms (e.g., multiplexing or dropping) are needed to avoid simultaneous transmission of PUCCH and PUSCH.

  - UCI multiplexing within one TRP or between two TRPs can be considered.

  - Dropping rules which are related to information type, priority index, coresetPoolIndex value, and CC index can be further considered.

  - SFN based PUCCH transmission

In SFN based PUSCH transmission, RRC signaling is used to switch among STxMP SFN scheme and Rel-17 TDM based mTRP PUSCH scheme. Similarly, in SFN based PUCCH transmission, RRC signaling can also be used to switch among STxMP SFN scheme and Rel-17 TDM based mTRP PUCCH scheme. The RRC signaling can be configured in each PUCCH resource.

Proposal 26: Per PUCCH RRC configuration is used to switch among STxMP SFN scheme and Rel-17 TDM based mTRP PUCCH scheme.

- Conclusions

In this contribution, we provide our views on enhancements on STxMP PUSCH transmission and enhancements on STxMP PUCCH transmission. Based on the above discussion, we have the following proposals:

Proposal 1: Configure additional RRC parameter to indicate the actual max PTRS port number(1 or 2).

Proposal 2: The max number of layers can be four when UE switches to sTRP transmission.

Proposal 3: Regarding the dynamic switching between STxMP SDM scheme and sTRP transmission, working assumption is confirmed.

Proposal  4: the total number of used PUSCH antenna ports for the SDM and sTRP can be different.

Proposal 5: For the codepoint of the “SRS resource set indicator”, use the following interpretations:

- SRS resource set indicator codepoint 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively.

- Alt1: 1st TPMI indicates the precoding/rank for CB PUSCH or 1st SRI field indicates precoding/rank NCB PUSCH. (2nd field can be reserved/unused or absent)

- SRS resource set indicator codepoint 10 indicates SDM with 1st SRS resource set and 2nd SRS resource set. The 1st TPMI or SRI field and 2nd TPMI or SRI field indicates the precoding/rank/SRI for the 1st SRS resource set and 2nd SRS resource set for CB or NCB respectively

- The codepoint 11 of SRS resource set indicator:

- Alt1: reserved/unused

Proposal 6: The size of the first TPMI/SRI is determined based on the maximal number of layers of sTRP when the DCI indicates sTRP transmission, while the size of the first TPMI/SRI is determined based on the maximal number of layers of the first SRS resource set in SDM when the DCI indicates SDM transmission.

Proposal 7: For CB-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports.

Proposal 8: For SFN based PUSCH transmission, use the same DMRS indication as for the TDM scheme.

Proposal 9: Rel-15 PTRS-DMRS association method is supported for SFN scheme assuming that:

  - For CB, same coherent type is indicated by the two TPMI fields.

  - For NCB, same number of PTRS port indexes are indicated by the two SRI fields.

Proposal 10: In SFN scheme, use same configuration and codepoint interpretations for SRS resource set indicator as SDM scheme.

Proposal 11: In SFN scheme, configure separate parameter of the maximal number of layers for STxMP SFN scheme.

Proposal 12: The two scheduled PUSCHs have the same DMRS configuration with respect to

  - the actual number of front-loaded DMRS symbol(s).

  - the actual number of additional DMRS symbol(s).

  - the actual DMRS symbol location.

  - DMRS configuration type.

Proposal 13: Except for per-panel power limit (if supported), the maximum Tx power of each panel (scheduling restriction) is not defined or introduced for M-DCI based STxMP PUSCH transmission.

Proposal 14: The maximal number of configured SRS resources in each set for NCB/CB is 4.

- When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number is 2 or 4. Otherwise, the maximum number is 2.

Proposal 15: The maximal number of indicated SRS resources in each set for NCB depends on the maximal supported layers for each TRP.

Proposal 16: The maximal number of SRS ports in each set for CB is 4.

Proposal 17: Separate codebookSubsets, maxRanks and txConfigs are configured for different SRS resource sets.

Proposal 18: If two overlapped PUSCHs collide due to the actual total number of layers of them is larger than the maximal number of layers for mTRP, UE chooses one of two following alternatives to solve the issue:

Alt 1: UE drops two collided PUSCHs.

Alt 2: UE transmits one of the PUSCHs within the maximal number of layers that has high priority signal/transmission time/layer(s).

Proposal 19: Different priority indexes or different priorities can be indicated for PUSCHs in M-DCI based STxMP PUSCH transmission.

Proposal 20: If multiple PUSCHs have same priority index or priority, the time interval between the other CG PUSCH and the simultaneously transmitted PUSCHs should be defined.

Proposal 21: If multiple PUSCHs have different priority indexes or priorities, coresetPoolIndex, SRS resource set, the number of layers or priority indexes can be used to determine which PUSCHs can be transmitted.

Proposal 22: UCI multiplexing can be performed after UE determines up to two simultaneously transmitted PUSCHs to ensure that UCI can always be transmitted.

Proposal 23: In M-DCI based STxMP PUSCH transmission, a UE multiplexes UCI/PUCCH on PUSCH which is associated with the same coresetPoolIndex value as the PUCCH.

- The association between PUCCH and coresetPoolIndex can be referred to the discussion/conclusion in unified TCI state session.

Proposal 24: In UCI multiplexing with multiple serving cells, PUSCH selection rule shall be enhanced to ensure that the TCI state of UCI is not changed after UCI multiplexing.

- The detailed selection rule is for further study.

Proposal 25: Mechanisms (e.g., multiplexing or dropping) are needed to avoid simultaneous transmission of PUCCH and PUSCH.

  - UCI multiplexing within one TRP or between two TRPs can be considered.

  - Dropping rules which are related to information type, priority index, coresetPoolIndex value, and CC index can be further considered.

Proposal 26: Per PUCCH RRC configuration is used to switch among STxMP SFN scheme and Rel-17 TDM based mTRP PUCCH scheme.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN#94-e, December 6th – 17th, 2021.

- 3GPP TSG RAN WG1 Meeting RAN1 #112 Chairman's Notes, February 27 th – March 3 rd, 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #111 Chairman's Notes, November 14th – 18th, 2022.

- 3GPP TSG RAN WG1 Meeting RAN1 #110bis-e Chairman's Notes, October 10th – 19th, 2022.