3GPP TSG RAN WG1 #112bis-e		                                                      R1-2302686

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Discussion on SRI/TPMI enhancement for 8TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, enhancements for UL 8TX operations were agreed and the following objectives were approved in the WID for Rel-18 NR MIMO [1]:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8TX UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, codebook design, SRI/TPMI enhancement, 2 CWs transmission and full power transmission for UL 8TX operation are discussed, and evaluation results are provided.

- UL 8TX codebook design

  - Non-coherent codebook

A non-coherent UE can only be configured with non-coherent codebook. Since a non-coherent UE is not capable of transmitting a single layer with multiple antennas coherently, each precoding matrix in a non-coherent codebook shall be a port selection precoding matrix, with one port selected for each layer.

In RAN1 #112 meeting, the following agreement was achieved on codebook design for non-coherent UEs [2]:

Agreement For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission. 10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001 With the scaling factor of 122.

For the non-coherent codebook of 4TX for CP-OFDM waveform in Rel-15, the total number of precoding matrices in non-coherent codebook is 12, with all of port selection precoding matrices for rank=1/2/4 included, and one precoding matrix for rank =3 included. For UL 8TX in Rel-18, if all of port selection precoding matrices are included in the non-coherent codebook, the number of precoding matrices for each rank would be as follows:

- Rank 1: C81=8;

- Rank 2: C82=28;

- Rank 3: C83=56;

- Rank 4: C84=70;

- Rank 5: C85=56;

- Rank 6: C86=28;

- Rank 7: C87=8;

- Rank 8: C88=1.

If all of the precoding matrices are included, the number of precoding matrices and the bit widths for TPMI field when maxRank=1, 2, ⋯, 8 are shown in Table 1. It can be seen that 8 bits are needed when maxRank >= 4 for 8TX. Compared with 4TX, the bit widths for TPMI field are doubled.

Table 1: Number of precoding matrices and bit widths of TPMI field for non-coherent codebook for UL 8TX

| maxRank | Number of TPMIs | Bitwidth for TPMI field (bits) |
| --- | --- | --- |
| 1 | 8 | 3 |
| 2 | 36 | 6 |
| 3 | 92 | 7 |
| 4 | 162 | 8 |
| 5 | 218 | 8 |
| 6 | 246 | 8 |
| 7 | 254 | 8 |
| 8 | 255 | 8 |

In Rel-15, for UL 4TX, all of the precoding matrices in non-coherent codebook are included in partial-coherent codebook and 8TX full-coherent codebook. Whether the principle is extended to UL 8TX operation should be studied. If the principle is extended to UL 8TX operation (i.e., precoding matrices in 8TX non-coherent codebook are included in 8TX partial-coherent codebook and 8TX full-coherent codebook), whether all or a subset of precoding matrices in non-coherent codebook are included in partial-coherent codebook and full-coherent codebook depends on the expected overhead of TPMI indication for the partial-coherent/full-coherent codebook.

In MIMO system, the gain of increasing the number of precoding matrixes for high ranks would be much lower than that for low ranks, and the probability of scheduling UEs with high ranks would be lower than that for low ranks. Therefore, if a subset of port selection precoding matrices are included in the non-coherent codebook, keeping all port selection precoding matrices for low ranks (e.g. rank=1,2) and down selecting port selection precoding matrices for high ranks (e.g. rank>2) shall be considered.

Proposal 1: For UL 8TX operation, whether a subset or all of precoding matrices in non-coherent codebook are included in partial-coherent codebook and full-coherent codebook shall be studied.

Proposal 2: For UL 8TX operation, if a subset of port selection precoding matrices is supported for non-coherent codebook, where all port selection precoding matrices for low ranks (e.g. for rank=1,2) are kept and precoding matrices for high ranks (e.g. for rank>2) are down selected.

  - Partial-coherent codebook

In RAN1 #112 meeting, the following agreements have been reached on codebook design for partial-coherent UEs [2]:

Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS: whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt 1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt 2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

For partial-coherent 8TX UEs with Ng = 2, precoding matrixes can be generated based on NR Rel-15 UL 4TX codebook according to the following structures:

  - For rank = 1, 12AO, and O12A;

  - For rank = 2/3/4, 12AO, O12A, and 12B1OO12B2;

  - For rank > 4, 12B1OO12B2;

where r is the rank of the PUSCH transmission, A, B1 and B2 are 4TX precoders selected from Rel-15 UL 4TX codebook,

A=a1,1⋯a1,r⋮⋱⋮a4,1⋯a4,r, B1=b1,1,1⋯b1,1,l⋮⋱⋮b1,4,1⋯b1,4,l, B2=b2,1,1⋯b2,1,r-l⋮⋱⋮b2,4,1⋯b2,4,r-l,

and l∈max1,r-4,⋯,minr,4.

For partially coherent uplink precoding by an 8TX UE codebook, there are many rank and layer splitting cases about how to split the layers to antenna groups. Supporting all splitting cases can achieve optimal performance, but leads to largest overhead for precoding indication. Therefore, a balance between performance and overhead of the codebook is needed.

In RAN1 #112 meeting, it was agreed to select rank and layer splitting cases from the following cases for partially coherent uplink precoding by an 8TX UE codebook with Ng=2 for rank > 1:

Table 2: Rank and layer splitting cases for 8TX partial-coherent UEs with Ng = 2

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (2,1), (1,2) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2), (3,1), (1,3) |
| 5 |  | (4,1), (1,4), (2,3), (3,2) |
| 6 |  | (4,2), (2,4), (3,3) |
| 7 |  | (4,3), (3,4) |

To start with, three codebooks generated with rank and layer splitting cases as shown in Table 3 are considered. For each codebook, all of the precoders are generated based on 4TX full-coherent precoder(s). The number of precoders for each codebook is also provided in Table 3.

Table 3: Codebooks for 8TX partial-coherent UEs with Ng = 2

|  | Alt 1 |  | Alt 2 |  | Alt 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | Layer splitting cases | Number of TPMIs | Layer splitting cases | Number of TPMIs | Layer splitting cases | Number of TPMIs |
| 1 | (0,1) (1,0) | 32 | (0,1) (1,0) | 32 | (0,1) (1,0) | 32 |
| 2 | (1,1) | 256 | (1,1) | 256 | (1,1) (0,2) (2,0) | 272 |
| 3 | (1,2) | 128 | (1,2) (2,1) | 256 | (1,2) (2,1) (0,3) (3,0) | 264 |
| 4 | (2,2) | 64 | (2,2) | 64 | (2,2) (1,3) (3,1) (0,4) (4,0) | 196 |
| 5 | (2,3) | 32 | (2,3) (3,2) | 64 | (2,3) (3,2) (1,4) (4,1) | 128 |
| 6 | (3,3) | 16 | (3,3) | 16 | (3,3) (2,4) (4,2) | 48 |
| 7 | (3,4) | 8 | (3,4) (4,3) | 16 | (3,4) (4,3) | 16 |
| 8 | (4,4) | 4 | (4,4) | 4 | (4,4) | 4 |
| maxRank 4 |  | 480 |  | 608 |  | 764 |
| maxRank 8 |  | 540 |  | 708 |  | 960 |

In Figure 1, the evaluation results of the three codebooks provided in Table 3 are provided for a UL 8TX UE with (N1, N2) = (4, 1), with the simulation assumptions provided in Table 10 in Annex. It can be seen from Figure 1 that comparable performance can be achieved by Alt 1 and Alt 2. Alt 3 slightly outperforms Alt 1 and Alt 2, with the performance gain no more than 3.5%. Table 4 shows the distribution of rank and layer splitting cases for Alt 3. It can be seen that the probabilities of cases with all layers come from one antenna group are quite small, and the probabilities of cases where the difference between the number of layers split to the two antenna groups is no more than 1 are higher than the other cases.

Figure 1: Performance comparison of codebooks for 8TX partial-coherent UEs with Ng = 2

Table 4: Distribution of rank and layer splitting cases for Alt 3 in Table 3

|  | (-,0) | (-,1) | (-,2) | (-,3) | (-,4) |
| --- | --- | --- | --- | --- | --- |
| (0,-) |  | 0.70% | 0.00% | 0.00% | 0.00% |
| (1,-) | 0.21% | 9.68% | 6.83% | 0.09% | 0.02% |
| (2,-) | 0.00% | 6.35% | 35.93% | 8.03% | 2.99% |
| (3,-) | 0.00% | 0.10% | 7.44% | 8.37% | 3.55% |
| (4,-) | 0.00% | 0.02% | 3.64% | 4.13% | 1.92% |

Observation 1: Regarding the performance of codebooks for partial-coherent 8TX UEs with Ng=2,

- Comparable performance can be achieved by UL 8TX codebook based on Alt 1 and Alt 2;

- Alt 3 slightly outperforms Alt 1 in terms of average throughput, 95% performance and 5% throughput (about 3.7%, 3.4% and 3.3% respectively).

Observation 2: Regarding the distribution of precoders in Alt 3 in Table 2,

- Precoders with all layers come from one coherent antenna group are rarely used;

- The probabilities of cases where the difference between the number of layers split to the two antenna groups is no more than 1 (i.e., cases with form of (x, x)/ (x, x+1)/ (x+1, x)) are higher than the other cases.

Proposal 3: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, the following rank and layer splitting cases are supported:

- For rank 1, support (1,0) and (0,1);

- For rank 2, support (1,1);

- For rank 3, support (1,2);

- For rank 4, support (2,2);

- For rank 5, support (2,3);

- For rank 6, support (3,3);

- For rank 7, support (3,4);

- For rank 8, support (4,4).

For a 8TX precoding matrix generated with two Rel-15 UL 4TX full-coherent precoding matrixes for rank 3/5/7, the numbers of layers that split to two antenna groups are not equal. Then the precoding matrix cannot keep constant modulus property as Rel-15 UL 4TX precoders.

In order to keep constant modulus property for a UL 8TX partially coherent transmission, the design of precoders for Rel-15 UL 4TX partial-coherent codebook for rank 3 can be referred to. In Rel-15, a partial-coherent precoding matrix for rank 3 is generated with a UL 2TX full-coherent precoder for rank 1 and a UL 2TX non-coherent precoder for rank 2. Similarly, an UL 8TX partial-coherent precoding matrix for rank 3 can be constructed from a UL 4TX full-coherent precoder for rank 1 and a UL 4TX partial-coherent precoder for rank 2, as shown as Alt 2 in Table 5. Then the constant modulus property of UL 8TX partial-coherent codebook can be kept.

Table 5: Structures of partial-coherent precoders for rank 3

|  | Alt 1 | Alt 2 |
| --- | --- | --- |
| Rank 3 | 12B1OO12B2, 12B2OO12B1 Where B1=12b1,1,1b1,2,1b1,3,1b1,4,1 and B2=18b2,1,1b2,1,2b2,2,1b2,2,2b2,3,1b2,3,2b2,4,1b2,4,2 | 12B1OO12B2, 12B2OO12B1 Where B1=12b1,1,1b1,2,1b1,3,1b1,4,1 and B2=12b2,1,100b2,2,2b2,3,100b2,4,2 |
| Number of TPMIs | 16*8*2=256 | 16*8*2=256 |

Figure 2: Performance comparison of partial-coherent codebooks for rank 3

In Figure 2, performance comparison of two codebooks designed for rank-3 are provided, with the two codebooks shown in Table 5. As shown in Figure 2, comparable performance can be achieved by Alt 1 and Alt 2. Therefore for partial-coherent precoding for an UL 8TX UE with rank 3, the precoders  can be generated by a UL 4TX rank-1 full-coherent precoder and a UL 4TX rank-2 partial-coherent precoder. Besides, according to the evaluation of rank and layer splitting cases, it can be found that there is only a slight performance gain where (1,2) and (2,1) are both supported compared with only supporting (1,2). Therefore, the structure of partial-coherent precoders for rank 3 can be improved, as show in Table 6.

Table 6: Structures of partial-coherent precoders for rank 3 with constant modulus property

|  | Rank 3 |
| --- | --- |
| Structure of precoders | 12B1OO12B2, Where B1=12b1,1,1b1,2,1b1,3,1b1,4,1 and B2=12b2,1,100b2,2,2b2,3,100b2,4,2 |
| Number of TPMIs | 16*8=128 |

The principle can be extended to rank 5 and rank 7, with the structures of precoders shown in Table 7 and Table 8 respectively. As shown in Table 7, for rank 5, constant modulus property is kept by generating the precoder with 2 UL 4TX partial-coherent precoders. As shown in Table 8, for rank 7, constant modulus property is kept by generating the precoder with a UL 4TX partial-coherent precoder and a UL 4TX non-coherent precoder.

Table 7: Structures of partial-coherent precoders for rank 5 with constant modulus property

|  | Rank 5 |
| --- | --- |
| Structure of precoders | 12B1OO12B2, Where B1=12b1,1,100b1,2,2b1,3,100b1,4,2 and B2=12b2,1,1000b2,2,20b2,3,10000b2,4,3 |
| Number of TPMIs | 8*2=16 |

Table 8: Structures of partial-coherent precoders for rank 7 with constant modulus property

|  | Rank 7 |
| --- | --- |
| Structure of precoders | 12B1OO12B2, Where B1=12b1,1,1000b1,2,20b1,3,10000b1,4,3 and B2=12b2,1,10000b2,2,20000b2,3,30000b2,4,4 |
| Number of TPMIs | 2*1=2 |

Proposal 4: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2,

- For rank 3, precoding design is based on Rel-15 UL 4TX codebook with a full-coherent precoder and a partial-coherent precoder used;

- For rank 5, precoding design is based on Rel-15 UL 4TX codebook with two partial-coherent precoders used;

- For rank 7, precoding design is based on Rel-15 UL 4TX codebook with a partial-coherent precoder and a non-coherent precoder used.

In RAN1 #112, it was agreed that precoders for partial-coherent UEs with Ng = 4 should be down-selected from the following two structures:

- Alt 1: Precoding design is based on Rel-15 UL 2TX codebook,

  - Full-coherent precoders are used

- Alt 2: Precoding design is based on Rel-15 UL 4TX codebook,

  - Partial-coherent precoders are used

In Table 9, two codebooks generated based on the two alternatives are provided. For the codebook provided in the table, Aa,b,c represents a UL 2TX/ 4TX precoder, where ‘a’ represents the type of the precoder (i.e., UL 2TX precoder, or UL 4TX precoder), ‘c’ represents the rank of the precoder, and ‘b’ is an index. For the codebook of Alt 2 in Table 9, the following rank and layer splitting cases are included:

- For rank 1, support (1,0) and (0,1);

- For rank 2, support (1,1), (2,0), (0, 2);

- For rank 3, support (1,2), (2,1);

- For rank 4, support (2,2).

For the alternatives in Table 9, compared to Alt 2, Alt 1 has a higher degree of freedom on rank and layer splitting, but leads to a large indication overhead.

Table 9: Codebooks for 8TX partial-coherent UEs with Ng = 4

|  | Alt 1 |  | Alt 2 |  |
| --- | --- | --- | --- | --- |
| Rank | Layer splitting cases | Number of TPMIs | Layer splitting cases | Number of TPMIs |
| 1 | 12A2Tx,1,1000 , 012A2Tx,1,100 , 0012A2Tx,1,10 , 00012A2Tx,1,1 | 16 | 12A4Tx,1,10 , 012A4Tx,1,1 | 16 |
| 2 | 12A2Tx,1,10012A2Tx,2,10000 , 12A2Tx,1,1000012A2Tx,2,100 , 12A2Tx,1,100000012A2Tx,2,1 , 0012A2Tx,1,10012A2Tx,2,100 , 0012A2Tx,1,1000012A2Tx,2,1 , 000012A2Tx,1,10012A2Tx,2,1 | 96 | 12A4Tx,1,10012A4Tx,2,1 , 12A4Tx,1,20 , 012A4Tx,1,2 | 80 |
| 3 | 12A2Tx,1,100012A2Tx,2,100012A2Tx,3,1000 , 12A2Tx,1,100012A2Tx,2,100000012A2Tx,3,1 , 12A2Tx,1,100000012A2Tx,2,100012A2Tx,3,1 , 00012A2Tx,1,100012A2Tx,2,100012A2Tx,3,1 | 256 | 12A4Tx,1,10012A4Tx,2,2 , 12A4Tx,1,20012A4Tx,2,1 | 128 |
| 4 | 12A2Tx,1,1000012A2Tx,2,1000012A2Tx,3,1000012A2Tx,4,1 | 256 | 12A4Tx,1,20012A4Tx,2,2 | 64 |
| maxRank 4 |  | 624 |  | 288 |

In Figure 3, simulation results for the two alternatives are provided. It can be seen from the results that for maxRank = 2, comparable performance can be achieved by Alt and Alt 2. However, for maxRank = 4, significantly performance gain can be achieved by Alt 1 compared to Alt 2. That is because for UL transmission of a partial-coherent 8TX UE with Ng=4, when maxRank ≤ 4, the transmission power is proportional to the number of layers, and full power only can be achieved by rank 4. Therefore more precoding matrixes for rank 3 and 4 would result in better performance.

For partially coherent uplink precoding by an 8TX UE codebook for Ng=2, if Alt 1 (i.e., precoding design is based on Rel-15 UL 2TX codebook) is adopted, the overhead of the partial-coherent codebook can be quite large, down selection on precoders for several ranks is preferred to restrict the codebook size be the same level (i.e., the overhead is the same) as that for Ng=4. Another alternative is adopt Alt 2 (i.e., precoding design is based on Rel-15 UL 2TX codebook), with more precoding matrixes than the solution in Table 9 included for rank = 3 and rank =4.

Figure 3: Performance comparison of codebooks for 8TX partial-coherent UEs with Ng = 4

Observation 3: Regarding the performance of codebooks for partial-coherent 8TX UEs with Ng=4,

- For maxRank = 2, comparable performance can be achieved by UL 8TX codebook based on Alt 1 and Alt 2;

- For maxRank = 4, the codebook based on Alt 1 outperforms the codebook based on Alt 2 in terms of average throughput and 5% throughput (about 5.5% and 10.8% respectively).

Proposal 5: For partially coherent uplink precoding (maxRank = 4) by an 8TX UE codebook, Ng=4,

- Precoding design is based on Rel-15 UL 2TX codebook,

  - If Alt 1 (i.e., precoding design is based on Rel-15 UL 2TX codebook) is adopted, down selection on precoders for several ranks is considered to restrict the codebook size be the same level as that for Ng=4.

  - If Alt 2 (i.e., precoding design is based on Rel-15 UL 2TX codebook) is adopted, the number of precoding matrixes for rank 3 and rank 4 shall be larger than that for rank 1 and rank 2.

  - Full-coherent codebook

In RAN1 #111 meeting, it was agreed to study NR Rel-15 single panel DL Type I codebook as the starting point for codebook design for full-coherent UEs. In RAN1 #112 meeting, the antenna layout for 8TX full-coherent UEs and the influence of oversampling factor was further discussed. The following agreements have been achieved in previous meetings [2][5]:

Agreement (RAN1 #111) For a fully coherent uplink precoding by an 8TX UE, - Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a. No LS to RAN4 will be needed Agreement (RAN1 #112) For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2) A pair of (N1, N2) can be configured with subject to UE capability. Agreement (RAN1 #112) Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement (RAN1 #112) For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results

For NR Rel-15 DL single panel Type I codebook, for a given (N1, N2), only one (O1, O2) combination is supported. For full-coherent codebook generated based  NR Rel-15 single panel DL Type I codebook, there is no strong motivation to support multiple combinations of (O1, O2) for a given (N1, N2).

Proposal 6: For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), combinations of (O1, O2) other than (1, 1) are not supported.

- SRS configuration and SRI/TPMI indication

  - Codebook based PUSCH

For a codebook based 8TX PUSCH transmission configured with maxRank no more than 8, if TPMI and TRI are separately indicated, the bitwidth of the TPMI filed would be determined by the maximum number of precoders of layers from 1 to maxRank, and the bitwidth of TRI would be determined by maxRank. Since the differences of the number of precoders for ranks of UL 8TX can be quite large for the codebook designed based on the agreed principles, the total overhead of TPMI and TRI for indicating TPMI and TRI separately can be larger than that for indicating TPMI and TRI jointly as that used in Rel-15 in several cases. Therefore, on TPMI indication for codebook based 8TX PUSCH transmission, the legacy framework in Rel-15 is reused, i.e., TPMI and TRI are jointly indicated by a single field.

Proposal 7: For TPMI indication for codebook based 8TX PUSCH, the legacy framework in Rel-15 is reused, i.e., TPMI and TRI are jointly indicated by a single field.

  - Non-codebook based PUSCH

In RAN1 #110bis-e meeting, the following agreement has been achieved on SRS configuration for non-codebook based 8TX transmission [4]:

Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt 1 is supported, that is - Alt 1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

One open issue is whether multiple SRS resource sets are supported for non-codebook-based UL transmission by an 8TX UE. For Rel-17 M-TRP PUSCH transmission, two SRS resource sets with usage set to “nonCodebook” can be configured for two TRPs respectively. If 8 single-port SRS resources are configured in two SRS resource sets for UL 8TX, a mechanism of identifying whether the two SRS resource sets are for UL 8TX or for M-TRP PUSCH transmission is needed. It causes unnecessary spec efforts. Therefore for SRS configuration for non-codebook based PUSCH transmission, except for M-TRP transmission schemes, configuring multiple SRS resource sets is not preferred.

Proposal 8: For SRS configuration for non-codebook based 8TX PUSCH, except for M-TRP transmission schemes, configuring multiple SRS resource sets is not supported.

In RAN1 #112 meeting, the following agreement has been achieved on SRI indication for non-codebook based 8TX PUSCH transmission [2]:

Agreement For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an NSRS bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For NSRS≤4, Rel-15 SRI indication is reused

On SRI indication for non-codebook based 8TX PUSCH transmission with NSRS>4, option 2 is preferred since compared to option 1, it can save large SRI indication overheads in several cases. For example, when Lmax=1 is configured, only 3 bits is needed for option 2, but NSRS bits are always needed for option 1. Then, compared to option 1, 2, 3, 4, and 5 bits can be saved by option 2 for NSRS = 5, 6, 7, 8, respectively.

In Rel-15, the candidate values for Lmax includes all values no larger than 4. For Rel-18 UL 8TX operation, it is natural to extend the candidate values to include all values no larger than 8.

Proposal 9: For NCB-based 8TX PUSCH transmission with NSRS>4, a legacy-based SRI indication solution is supported.

Proposal 10: For NCB-based 8TX PUSCH transmission, candidate values of Lmax is extended to include values of up to 8.

- 2 CWs transmission

In RAN1 #110bis-e meeting, the following working assumption on dual CW transmission has been achieved [4]:

Working Assumption For uplink transmission with rank>4, support dual CW transmission.

Figure 4 provides performance comparison of UL 8TX transmission with 1 CW and 2 CWs for rank= 5/6/7/8. It can be seen from the evaluation results that compared to 1 CW, obvious performance gain can be achieved by 2 CWs.

Figure 4: Performance comparison between 1 CW and 2 CWs

Observation 4: For 8TX PUSCH with rank 5~8, compared to PUSCH transmission with 1 CW,

- When the precoder is SVD-based, the average performance gain of PUSCH transmission with 2 CWs is 0.76%~24.13%.

Considering the benefits of 2 CWs for rank > 4, the working assumption on supporting 2 CWs for UL transmission with rank > 4 should be confirmed. Besides, for PUSCH transmission with more than 4 layers, we prefer to only support 2 CWs transmission.

Proposal 11: Confirm the following working assumption:

- For uplink transmission with rank>4, support dual CW transmission.

Proposal 12: For PUSCH transmission with rank>4, UE always applies 2 CWs transmission.

For PDSCH, RRC parameter maxNrofCodewordsScheduledByDCI is used to indicate whether two codeword transmission is enabled (i.e., whether 2 MCS/RV/NDI fields are included in the DCI scheduling PDSCH). For codebook based PUSCH transmission, RRC parameters maxRank and maxRankDCI-0-2 in pusch-Config are used to indicate the maximum number of layers for PUSCH transmission for DCI format 0_1 and 0_2 respectively. Since 2 CWs transmission is only supported for rank>4, maxRank and maxRankDCI-0-2 can be used to indicate whether two CWs transmission is enabled for DCI format 0_1 and 0_2 respectively.

Proposal 13: For codebook based 8TX PUSCH transmission, maxRank and maxRankDCI-0-2 are used to indicate whether two CWs transmission is enabled for DCI format 0_1 and 0_2 respectively.

For non-codebook based PUSCH transmission, RRC parameter maxMIMO-Layers, if is configured, in PUSCH-ServingCellConfig is used to indicate the maximum number of layers for PUSCH transmission, otherwise, the maximum number of layers for non-codebook based PUSCH transmission is given by the  maximum number of layers for PUSCH supported by the UE. Since 2 CWs transmission is only supported for rank>4, maxMIMO-Layers and UE’s capability on the maximum number of layers for PUSCH supported by the UE can be used to indicate whether two CWs transmission is enabled for non-codebook based PUSCH transmission.

Proposal 14: For non-codebook based 8TX PUSCH transmission, maxMIMO-Layers, if is configured, is used to indicate whether 2 CWs transmission is enabled; if maxMIMO-Layers is not configured, if the maximum number of layers for PUSCH supported by the UE is larger than 4, 2 CWs transmission is enabled, and if the maximum number of layers for PUSCH supported by the UE is no more than 4, 2 CWs transmission is disabled.

For PDSCH, when 2 CWs transmission is enabled for a DCI, a transport blocks is disabled if it is indicated with IMCS=26 and rvid=1, and if only one transport block is enabled, then the enabled transport block is always mapped to the first CW. For PUSCH, reusing the same scheme is not necessary. Since only one CW is enabled when rank≤4, and 2 CWs are enabled when rank>4, the number of transmission layers can be used to determine whether the second transport block is disabled.

Proposal 15: For 8TX PUSCH transmission, if 2 CWs for rank>4 is supported, the number of transmission layers for the PUSCH is used to determine whether the second transport block is disabled.

In RAN1 #112 meeting, the following agreement on UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE was achieved [2]:

Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt 1: First CW - Alt 2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

In LTE, when the CQI/PMI information is multiplexed on a PUSCH transmission with 2 CWs, the CQI/PMI information is always multiplexed only the CW with the highest MCS, or the first CW for the case where MCSs are the same. That is, Alt 2 is supported for CQI/PMI information multiplexing on PUSCH transmission with 2 CWs in LTE. It is natural to reuse the mechanism for UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE.

Proposal 16: To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on the CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW).

In RAN1 #112 meeting, the following agreement has been achieved on NDI and RV indication for dual CW 8TX PUSCH transmission [2]:

Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal

There was also a working assumption on DMRS indication for 8TX PUSCH transmission with rank = 5-8 [2]:

Working Assumption To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI 9.1.4.2 [or AI 9.1.3.1].

It is our view that the second set of NDI (1 bit) and RV (2 bits) should be indicated by two new filed for the second CW respectively.

Proposal 17: To support dual CW PUSCH transmission for rank>4 by an 8TX UE, two new fields are used to indicate the second set of NDI (1 bit) and RV (2 bits) fields respectively.

As we analyzed in the accompanied document in [8], the reserved field in antenna ports field can be reused for PTRS-DMRS association indication for 8TX PUSCH transmission with rank > 4 and configured with maximum 2 PTRS ports. For a PUSCH transmission with rank >4 with maximum 2 PTRS ports configured, each of the 2 PTRS ports can be associated with up to 4 layers. Therefore 2 bits for each PTRS port are needed. One possible solution is to extend the size of PTRS-DMRS association field to be 4 bits, with 2 bits for UL PTRS port 0 and 2 bits for UL PTRS port 1, respectively. Such solution requires 2 more bits in DCI for PTRS indication. For PUSCH transmission with rank > 4, if the same DMRS port combination(s) as that for rank > 4 for PDSCH are used, there is only one or two DMRS port combination(s) for each rank. Since the bitwidth of antenna port filed in DCI format 0_1/0_2 is 3 to 5 bits, the antenna port filed can be used to indicate the mapping of PTRS ports and DMRS ports. One solution is to use the 2-bit PTRS-DMRS association field for one UL PTRS port and 2 bits in antenna port filed for another UL PTRS port. Then the size of DCI would not be increased. The details of the solution for the PTRS-DMRS association indication for 8TX PUSCH transmission with rank > 4 and configured with maximum 2 PTRS ports are shown as follows:

- The PTRS-DMRS association field in DCI format 0_1/0_2 is associated with UL PTRS port 0 as follows:

| Value | DMRS port |
| --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |
| 1 | 2nd DMRS port which shares PTRS port 0 |
| 2 | 3rd DMRS port which shares PTRS port 0 |
| 3 | 4th DMRS port which shares PTRS port 0 |

- 2 bits of antenna port field in DCI format 0_1/0_2 are associated with UL PTRS port 1 as follows:

| Value | DMRS port |
| --- | --- |
| 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 1 |

Proposal 18:  For 8TX PUSCH transmission with rank > 4 and configured with maximum 2 PTRS ports, the reserved field in antenna ports field is used to indicate the associated DMRS for UL PTRS port 1.

- Full power transmission

In RAN1 #111 meeting, the following agreements on UL 8TX with full power operation have been achieved [5]:

Agreement Study full TX power uplink codebook-based transmission by a partial/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN1 #112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2=⋯=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2=⋯=P8=23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2=⋯=P7=14 dBm P8=23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2=⋯=P6=14 dBm P7=P8≥20 dBm Example 2b: P1=P2=⋯=P8=20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2=⋯=P4=14 dBm P5=P6=⋯=P8≥17 dBm Example 3b: P1=P2=⋯=P8=17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2=14 dBm P3=P4=⋯=P8≥15.3 dBm Example 4b: P1=P2=⋯=P8=15.3 dBm. Agreement For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design. Note: This does not mandate support of all Rel-16 modes.

Since the cost of CPE/FWA/vehicle/Industrial devices is not as limited as that of traditional handheld devices, it is more likely that CPE/FWA/vehicle/Industrial devices with 8TX are equipped with full power PAs for partial or all of antennas. It is our view that at least the full power transmission mode for UE Capability 1 should be supported, i.e., Rel-16 UL FTxP mode 0 (i.e. scaling factor s=1 for all precoders) is supported for UL 8TX.

Proposal 19: For an 8TX UE with UE Capability 1, scaling factor s=1 for all precoders is supported.

For full TX power transmission by UE Capability 2/3, one open issue is which full TX power transmission scheme is supported. It is our view that Rel-16 UL FTxP mode 2 is supported.

Proposal 20: Support Rel-16 UL FTxP mode 2 for an 8TX UE with UE Capability 2/3.

- Conclusions

In this contribution, we provided our views on the enhancements for enabling UL 8TX. We have the following observations and proposals:

Observation 1: Regarding the performance of codebooks for partial-coherent 8TX UEs with Ng=2,

- Comparable performance can be achieved by UL 8TX codebook based on Alt 1 and Alt 2;

- Alt 3 slightly outperforms Alt 1 in terms of average throughput, 95% performance and 5% throughput (about 3.7%, 3.4% and 3.3% respectively).

Observation 2: Regarding the distribution of precoders in Alt 3 in Table 2,

- Precoders with all layers come from one coherent antenna group are rarely used;

- The probabilities of cases where the difference between the number of layers split to the two antenna groups is no more than 1 (i.e., cases with form of (x, x)/ (x, x+1)/ (x+1, x)) are higher than the other cases.

Observation 3: Regarding the performance of codebooks for partial-coherent 8TX UEs with Ng=4,

- For maxRank = 2, comparable performance can be achieved by UL 8TX codebook based on Alt 1 and Alt 2;

- For maxRank = 4, the codebook based on Alt 1 outperforms the codebook based on Alt 2 in terms of average throughput and 5% throughput (about 4.5% and 10.8% respectively).

Observation 4: For 8TX PUSCH with rank 5~8, compared to PUSCH transmission with 1 CW,

- When the precoder is SVD-based, the average performance gain of PUSCH transmission with 2 CWs is 0.76%~24.13%.

Proposal 1: For UL 8TX operation, whether a subset or all of precoding matrices in non-coherent codebook are included in partial-coherent codebook and full-coherent codebook shall be studied.

Proposal 2: For UL 8TX operation, if a subset of port selection precoding matrices is supported for non-coherent codebook, where all port selection precoding matrices for low ranks (e.g. for rank=1,2) are kept and precoding matrices for high ranks (e.g. for rank>2) are down selected.

Proposal 3: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, the following rank and layer splitting cases are supported:

- For rank 1, support (1,0) and (0,1);

- For rank 2, support (1,1);

- For rank 3, support (1,2);

- For rank 4, support (2,2);

- For rank 5, support (2,3);

- For rank 6, support (3,3);

- For rank 7, support (3,4);

- For rank 8, support (4,4).

Proposal 4: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2,

- For rank 3, precoding design is based on Rel-15 UL 4TX codebook with a full-coherent precoder and a partial-coherent precoder used;

- For rank 5, precoding design is based on Rel-15 UL 4TX codebook with two partial-coherent precoders used;

- For rank 7, precoding design is based on Rel-15 UL 4TX codebook with a partial-coherent precoder and a non-coherent precoder used.

Proposal 5: For partially coherent uplink precoding (maxRank = 4) by an 8TX UE codebook, Ng=4,

- Precoding design is based on Rel-15 UL 2TX codebook,

  - If Alt 1 (i.e., precoding design is based on Rel-15 UL 2TX codebook) is adopted, down selection on precoders for several ranks is considered to restrict the codebook size be the same level as that for Ng=4.

  - If Alt 2 (i.e., precoding design is based on Rel-15 UL 2TX codebook) is adopted, the number of precoding matrixes for rank 3 and rank 4 shall be larger than that for rank 1 and rank 2.

Proposal 6: For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), combinations of (O1, O2) other than (1, 1) are not supported.

Proposal 7: For TPMI indication for codebook based 8TX PUSCH, the legacy framework in Rel-15 is reused, i.e., TPMI and TRI are jointly indicated by a single field.

Proposal 8: For SRS configuration for non-codebook based 8TX PUSCH, except for M-TRP transmission schemes, configuring multiple SRS resource sets is not supported.

Proposal 9: For NCB-based 8TX PUSCH transmission with NSRS>4, a legacy-based SRI indication solution is supported.

Proposal 10: For NCB-based 8TX PUSCH transmission, candidate values of Lmax is extended to include values of up to 8.

Proposal 11: Confirm the following working assumption:

- For uplink transmission with rank>4, support dual CW transmission.

Proposal 12: For PUSCH transmission with rank>4, UE always applies 2 CWs transmission.

Proposal 13: For codebook based 8TX PUSCH transmission, maxRank and maxRankDCI-0-2 are used to indicate whether two CWs transmission is enabled for DCI format 0_1 and 0_2 respectively.

Proposal 14: For non-codebook based 8TX PUSCH transmission, maxMIMO-Layers, if is configured, is used to indicate whether 2 CWs transmission is enabled; if maxMIMO-Layers is not configured, if the maximum number of layers for PUSCH supported by the UE is larger than 4, 2 CWs transmission is enabled, and if the maximum number of layers for PUSCH supported by the UE is no more than 4, 2 CWs transmission is disabled.

Proposal 15: For 8TX PUSCH transmission, if 2 CWs for rank>4 is supported, the number of transmission layers for the PUSCH is used to determine whether the second transport block is disabled.

Proposal 16: To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on the CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW).

Proposal 17: To support dual CW PUSCH transmission for rank>4 by an 8TX UE, two new fields are used to indicate the second set of NDI (1 bit) and RV (2 bits) fields respectively.

Proposal 18:  For 8TX PUSCH transmission with rank > 4 and configured with maximum 2 PTRS ports, the reserved field in antenna ports field is used to indicate the associated DMRS for UL PTRS port 1.

Proposal 19: For an 8TX UE with UE Capability 1, scaling factor s=1 for all precoders is supported.

Proposal 20: Support Rel-16 UL FTxP mode 2 for an 8TX UE with UE Capability 2/3.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN1#94-e, December 6th – 17th, 2021.

- 3GPP TSG RAN WG1 Meeting RAN1 #112 Chairman's Notes, Mar. 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #110 Chairman's Notes, Aug. 2022.

- 3GPP TSG RAN WG1 Meeting RAN1 #110bis-e Chairman's Notes, Oct. 2022.

- 3GPP TSG RAN WG1 Meeting RAN1 #111 Chairman's Notes, Nov. 2022.

- R1-2300654, Discussion on SRS enhancement in Rel-18, CATT, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2210378, FL Summary SRI/TPMI Enhancements Third Round, Moderator (InterDigital, Inc.), RAN1 #110bis-e, e-Meeting, October 10th – 19th, 2022.

- R1-2302683, DMRS enhancements in Rel-18, CATT, RAN1 #112bis-e, e-Meeting, April 17th – April 26th, 2023.

- Annex

Table 10: SLS simulation parameters for 8Tx UL operation

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 3.5 GHz |  |
| Bandwidth | 20 MHz |  |
| SCS | 30 kHz |  |
| Scenario | Indoor FWA: Umi(ISD=200m),100% Indoor, 3km/h |  |
| Antennas at UE (M,N,P,Mg,Ng) | Ng=1 | 4Tx: (1,2,2,1,1), (dH,dV)=(0.5,0.5)λ 8Tx: (2,2,2,1,1), (dH,dV)=(0.5,0.5)λ (1,4,2,1,1), (dH,dV)=(0.5,0.5)λ |
|  | Ng=2 | 8Tx: (1,2,2,1,2), (dH,dV)=(0.5,0.5)λ (1,2,2,2,1), (dH,dV)=(0.5,0.5)λ |
| Antennas at gNB (M,N,P,Mg,Ng,Mp,Np) | (8,8,2,1,1,4,8), (dH,dV)=(0.5,0.8)λ |  |
| MIMO scheme | SU-MIMO |  |
| Modulation | Up to 64 QAM |  |
| Traffic model | FTP model 1: Packet size 500KB, RU~= 50% |  |
| UE power rating | 23 dBm |  |
| Power control | P0 = -50, Alpha = 0.8 |  |