3GPP TSG RAN WG1 #112bis-e		R1-2302690

e-Meeting, April 17th –26th, 2023

Source:	CATT

Title:	PRACH coverage enhancements

Agenda Item:	9.12.1

Document for:	Decision

Introduction

In RAN#94-e meeting, the following objective was approved to specify coverage enhancement for PRACH [1].

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

In this contribution, we provide our views on different aspects of multiple PRACH transmissions.

Discussion

Multiple PRACH transmissions with same beam

According to the WID, multiple PRACH transmissions with same beam for 4-step RACH are to be specified. It is expected that PRACH performance can be improved if multiple transmissions are combined at receiver side. To that end, same preamble should be used for the multiple PRACH transmissions in one RACH attempt which has been agreed in RAN1#110bis-e meeting [2]. Whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt is FFS. In addition, only one RAR window supported for RAR monitoring was agreed in RAN1#112 meeting [3].

Agreement - For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt. Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

For different preambles for multiple PRACH transmissions in one RACH attempt, considering only one RAR window is supported for RAR monitoring, gNB requires to be aware of which preambles belong to the same UE so that the RAR information can be feedback, which complicates the gNB behavior. From UE perspective, UE has to transmit preambles according to the predefined or preconfigured pattern for multiple PRACH transmission which also complicates UE behavior compared with same preamble. In addition, the PRACH performance of different preambles is expected to be worse than same preamble due to the loss of combination gain at gNB side. Therefore, the motivation and benefit to use different preambles for multiple PRACH transmissions in this case is unclear.

Based on the above analysis, we propose that different preambles utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt is not supported.

Proposal 1: For multiple PRACH transmissions with same beam, it is not supported to utilize different preambles in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

The start position of the RAR window is to be decided. The following two alternatives are illustrated in Figure 1.

- Alt 1: RAR window starting after the first RO

- Alt 2: RAR window starting after the last RO

Figure 1: Examples of different RAR window determination.

For Alt 1, the advantage of earlier start of the RAR window is to achieve potential early termination. However, the probability of early termination is low if the number of multiple PRACH transmissions is determined appropriately. In order to achieve potential early termination, gNB is required to perform detection and combination upon reception of each PRACH repetition which complicates the gNB implementation and increases gNB power consumption. From UE perspective, UE may need to monitor RAR for longer time compared with Alt 2. Therefore, RAR window starting after the last RO for multiple PRACH transmissions is proposed.

Proposal 2: For multiple PRACH transmissions with same beam, UE monitors RAR after the PRACH occasion corresponding to the last PRACH repetition.

Another remaining issue is the determination of the RA-RNTI. In Rel-17, the RA-RNTI is determined based on time and frequency location of PRACH as follows [4].

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

Considering the RAR message need to be in response to multiple PRACH transmissions, the existing RA-RNTI calculation cannot be applied. Similar as the RAR window for a BL/CE UE in LTE, a reference RO can be used to calculate the RA-RNTI for multiple PRACH transmissions, e.g. the RO corresponding to the first PRACH transmission or the last PRACH transmission.

Proposal 3: RA-RNTI is calculated based on the PRACH occasion corresponding to the first PRACH repetition or the last repetition.

UE can perform multiple attempts during a RACH procedure. For multiple PRACH transmissions, in addition to the determination of SSB/CSI-RS and the power for each RACH attempt, it needs to be discussed how to determine the number of PRACH repetitions for each RACH attempt.

Firstly, we analyze the above UE behaviors in the first RACH attempt in a RACH procedure.

In existing NR specifications, UE selects an SSB/CSI-RS by comparing the SS-RSRP/CSI-RSRP with a threshold provided by gNB. To be specific, if at least one of the SSBs/CSI-RSs with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS provided by gNB is available, UE selects an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. Otherwise, UE selects any SSB/CSI-RS. The same approach can be applied for the first RACH attempt for multiple PRACH transmissions. Considering that the SS-RSRPs/CSI-RSRPs are relatively small in case of limited coverage, there may be no SSB/CSI-RS with SS-RSRP/CSI-RSRP above the threshold if legacy rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS are used for SSB/CSI-RS selection. Then, any SSB/CSI-RS can be selected by UE including SSBs/CSI-RSs with worse performance among all of the SSBs/CSI-RSs. Hence, it is proposed to study whether to introduce a separate lower RSRP threshold for SSB/CSI-RS determination for PRACH repetitions.

Proposal 4: For multiple PRACH transmissions with same beam, at least for the first RACH attempt in a RACH procedure, SSB/CSI-RS is selected based on SSB/CSI-RS RSRP thresholds as in existing specifications.

- FFS: whether to introduce separate RSRP thresholds for SSB/CSI-RS selection for PRACH repetitions from legacy PRACH transmission without repetition.

It has been agreed that at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions for the first RACH attempt in RAN1#111 meeting [6]. In our opinion, there is no need to use other measured/computed metrics or conditions to determine the number of PRACH transmissions.

Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

Furthermore, the SSB-RSRP threshold(s) used to determine the number of PRACH transmissions should be separate from the SSB-RSRP threshold(s) used for SSB selection, which is similar as that introduced in Msg3 PUSCH enhancement to determine whether to trigger Msg3 PUSCH repetition.

In addition, considering more than one values for the number of multiple PRACH transmissions are supported in a serving cell [3], separate RSRP thresholds should be provided by gNB for UE to determine the number of PRACH repetitions.

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details

For CFRA triggered by PDCCH order, the number of PRACH transmissions can be potentially indicated in the PDCCH order using the reserved bits.

Proposal 5: For multiple PRACH transmissions with same beam, at least for the first RACH attempt, only SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions for CBRA.

- SSB-RSRP threshold(s) for determination of number of PRACH transmissions are separately configured from the SSB-RSRP threshold(s) for SSB selection.

Proposal 6: For CFRA triggered by PDCCH order, the number of PRACH transmissions is indicated by the PDCCH order at least for the first RACH attempt.

Next, we analyze the UE behaviors in the subsequent RACH attempt(s) except the first attempt in a RACH procedure. A straightforward way is to perform the selection of SSB/CSI-RS and determination of number of PRACH repetitions independently in each RACH attempt as for the first RACH attempt. However, power ramping is applied only when UE does not change the spatial domain transmission filter and SSB compared with the last RACH attempt [4][5]. Then, the application of power ramping would be limited since power ramping cannot be applied as long as the SSB and/or the number of repetitions changes compared with that in the last RACH attempt. To avoid the limitation, an alternative approach is to keep the number of PRACH repetitions and SSB/CSI-RS unchanged and apply power ramping first if a RACH attempt fails until a certain condition is met, e.g. the PRACH transmission power exceeds the maximum transmission power, or the number of RACH attempts reaches a threshold etc. After that, UE can determine SSB/CSI-RS and the number of PRACH repetitions based on one of the two options below.

- Option 1: SSB/CSI-RS is unchanged. The number of PRACH repetitions increases to a next larger number of PRACH repetitions configured in the cell.

- Option 2: The selection of SSB/CSI-RS and the determination of number of PRACH repetitions are performed in the same way as in the first RACH attempt.

For Option 1, it is assumed that the channel condition is relatively stable. Thus, the selected SSB/CSI-RS in the first RACH attempt can still be suitable for the subsequent RACH attempts. UE improves the success possibility of random access by increasing the number of PRACH repetitions to a next larger number of repetitions when a certain condition is met.

For Option 2, considering the failure of random access may be caused by the improper SSB/CSI-RS or the number of PRACH repetitions, the re-selection of SSB/CSI-RS and the re-determination of number of PRACH repetitions are performed in the same way as in the first RACH attempt, respectively.

For both options, the PRACH transmission power needs to be re-calculated.

Proposal 7: For multiple PRACH transmissions with same Tx beam, for subsequent RACH attempt(s), power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met.

Msg3 PUSCH repetition has been supported in Rel-17 to improve the coverage of Msg3 PUSCH. According to the simulation results in [7], the coverage performance of PRACH is better than that of Msg3 PUSCH. In other words, if coverage of PRACH is an issue, coverage of Msg3 PUSCH would be an issue as well. Therefore, from coverage perspective, if PRACH repetitions are triggered for PRACH coverage improvement, Msg3 PUSCH repetitions should also be enabled. This could also simplify the interaction between PRACH repetitions and Msg3 PUSCH repetitions, since there is no need to distinguish between the two cases of Msg3 PUSCH with repetitions and Msg3 PUSCH without repetition in case PRACH repetitions are triggered, which avoids further PRACH partitioning. In addition, even if the Msg3 PUSCH repetitions are always enabled if PRACH repetitions are triggered, four candidate numbers of Msg3 repetitions including the value ‘1’ representing no Msg3 PUSCH repetition can be dynamically indicated by gNB. Hence, it is still up to gNB to decide whether to perform Msg3 PUSCH repetitions.

Proposal 8: If PRACH repetitions are triggered, Msg3 PUSCH repetition is enabled.

On the other hand, when coverage of Msg3 PUSCH is an issue, coverage of PRACH may not be an issue. Hence, the RSRP threshold(s) for PRACH repetition request should be separate from that for Msg3 repetition request.

Proposal 9: The RSRP threshold(s) for PRACH repetition request should be separate from that for Msg3 repetition request.

In order to differentiate PRACH transmissions with repetitions and PRACH transmissions without repetition at gNB side, time/frequency/preamble resource partitioning is required. The following working assumptions were achieved in the last meeting. In addition, the concept of ‘RO group’ is introduced to facilitate discussion.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. FFS: the time span of RO group. FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. FFS: other details

In the Rel-17 specifications, a list of additional RACH configurations can be configured by AdditionalRACH-Config-r17 in addition to the legacy RACH configuration configured by rach-ConfigCommon in the IE of BWP-UplinkCommon. In each legacy or additional RACH configuration, a set of feature combinations can be configured by featureCombinationPreamblesList-r17. For each feature combination, a set of preambles for this partition are configured by FeatureCombinationPreambles-r17. The preambles not associated with any feature can be used for basic PRACH transmission.

For Rel-18 PRACH resource partitioning, Rel-17 framework of feature combination can be reused and multiple PRACH transmission with same Tx beam can be introduced as a new feature in addition to the existing features including redcap, small data, NSAG and Msg3 repetition.

Rel-17 framework of additional RACH configuration can be reused to support both shared and separate ROs with single PRACH transmission. For shared ROs, a legacy or an additional RACH configuration can be configured with feature combinations for multiple PRACH transmissions and other single PRACH transmissions. For separate RO, an additional RACH configuration can be configured with feature combination for multiple PRACH transmissions only. The detailed signaling design should be up to RAN2.

Based on the above analysis, the two working assumptions can be confirmed with updates.

Proposal 10: Confirm the following two Working Assumptions with updates:

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Proposal 11: Multiple PRACH transmissions with same Tx beam is introduced as a new feature.

In addition, it has been supported that PRACH resources need to be differentiated between multiple PRACH transmissions with different number of multiple PRACH transmissions if multiple values of PRACH repetition factor are configured. In order to differentiate the PRACH resources, multiple values for the number of multiple PRACH transmissions can be regarded as different features when configuring the framework of feature combination and additional RACH configuration.

Proposal 12: Multiple values for the number of multiple PRACH transmissions can be regarded as different features when configuring the framework of feature combination and additional RACH configuration.

Further, we discuss the remaining issues on ‘RO group’.

Considering the collision between valid ROs for legacy single PRACH transmission and other features supported in Rel-17 has existed due to the additional RACH configuration, it is not necessary to specially address the collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features.

Proposal 13: It is not necessary to address the collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features.

To ensure the alignment between UE and gNB, the starting position and the time span of the RO group need to be specified. At least the following options can be considered.

- Option 1: The time span of a RO group can go across any system frame boundary.

- Option 2: The time span of a RO group cannot go across the system frame boundary between every 160 msec.

For multiple PRACH transmissions on separate ROs, new SSB-to-RO mapping pattern should be considered to reduce the transmission delay of PRACH repetitions. Concretely, for Option 1, the current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from SFN #0, where N is the number of PRACH transmissions. The same SSB-to-RO mapping is applied to the remaining valid ROs in every N consecutive ROs in time domain. An example is shown in Figure 2(a) with three SSBs and four repetitions. It is assumed that only one RO is configured in a system frame and the RO in SFN 4 is invalid. The current SSB-to-RO mapping is applied to the ROs in SFN0, SFN5, SFN9, SFN13, SFN17 and so on. Within each four consecutive valid ROs in time domain, the same SSB-to-RO mapping of the first ROs in time domain is applied to the remaining ROs.

For Option 2, the current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from the first system frame every 160 msec. The same SSB-to-RO mapping is applied to the remaining valid ROs in every N consecutive ROs in time domain. PRACH occasions not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions. As shown in Figure 2(b), the current SSB-to-RO mapping is applied to the ROs in SFN0, SFN5, and SFN9 in the first 160 msec. ROs in SFN13, SFN14 and SFN15 are not mapped to any SS/PBCH block indexes. The same mapping pattern repeats in the every 160ms.

Figure 2: The time span of the RO groups for separate ROs

Proposal 14: For multiple PRACH transmissions with same Tx beam on separate ROs, new SSB-to-RO mapping pattern is defined from one of the following options.

- Option 1: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from SFN #0. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain.

- Option 2: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from the first system frame every 160ms. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions.

For multiple PRACH transmissions with same Tx beam on shared RO, legacy SSB-to-RO mapping pattern should be reused. For RO group, for Option 1, every N consecutive valid ROs in time domain mapped to the same SS/PBCH block index are bundled as one RO group starting from SFN #0. An example is shown in Figure 3(b) with four SSBs and two repetitions. It is assumed that only one RO is configured in a system frame and the RO in SFN 4 is invalid. ROs in SFN0 and SFN5 associated with SSB1 are bundled as a RO group for SSB1. Similarly, ROs in SFN12 and SFN16 are bundled as another RO group for SSB1, and so on.

For Option 2, every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from the first system frame every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions. As shown in Figure 3(c), the ROs in SFN12~14 are not used for the determination of the RO groups, which is different from that in Figure 3(b).

Figure 3: The time span of the RO groups for shared RO

Proposal 15:  For multiple PRACH transmissions with same Tx beam on shared RO, to determine the starting position and the time span of a RO group, down select from the following two options:

- Option 1: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from SFN #0.

- Option 2: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from the first system frame every 160ms. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions.

In summary, for multiple PRACH transmissions on both separate ROs and shared ROs, Option 1 can utilize all the valid ROs in time domain to avoid PRACH resource waste at the cost of complexity on the determination of RO group compared with Option 2. There is a tradeoff between efficiency and complexity to do the down selection.

In addition, it needs to be discussed whether association period and association pattern period need to be defined for multiple PRACH transmissions with both shared RO and separate ROs. If defined, there may be some valid ROs that cannot be mapped to the SSB index, which results in the waste PRACH resource.

Proposal 16: Further discuss on whether the association period and association pattern period are needed for multiple PRACH transmissions.

The following agreement was achieved in RAN1#110bis-e meeting for RO locations for multiple PRACH transmissions [2]:

Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Frequency diversity gain can be achieved by frequency hopping in eMTC and can be considered for multiple PRACH transmissions in NR as well. For eMTC, the starting RB changes every system frame or every two system frames based on whether a PRACH resource occurs in every radio frame as follows [8].

For BL/CE UEs and for each PRACH coverage enhancement level, if frequency hopping is enabled for a PRACH configuration by the higher-layer parameter prach-HoppingConfig, the value of the parameter depends on the SFN and the PRACH configuration index and is given by - In case the PRACH configuration index is such that a PRACH resource occurs in every radio frame when calculated as below from Table 5.7.1-2 or Table 5.7.1-4, - otherwise where is the system frame number corresponding to the first subframe for each PRACH repetition, corresponds to a cell-specific higher-layer parameter prach-HoppingOffset. If frequency hopping is not enabled for the PRACH configuration then .

However, there may be much more ROs in a radio frame in NR and the maximum number of multiple PRACH transmissions is 8 in NR, if supported. Hence, the starting RB needs to change more frequently to obtain the diversity gain if frequency hopping is supported for PRACH transmissions. At least the following two options can be considered to determine the starting RB for multiple PRACH transmissions.

- Option 1: The starting RB of a RO is determined based on the absolute position of the RO in time domain, e.g. the slot index within a radio frame.

- Option 2: The starting RB of a RO is determined based on the relative position of the RO in time domain, e.g. the transmission index within multiple PRACH transmissions.

For both options, a PRACH frequency offset should be introduced to determine the frequency position of two hops along with the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 when PRACH hopping is enabled.

Proposal 17: Frequency hopping can be considered for multiple PRACH transmissions with same beam.

- FFS: how to determine the starting RB of different PRACH transmissions.

There is another issue on the execution order between the application of validation rules for ROs and the determination of ROs for multiple PRACH transmissions. According to the current specification, validation rule for ROs has been specified and a UE only transmits PRACH in valid PRACH slots. If shared RO with separate preambles is supported to differentiate the multiple PRACH transmissions with single PRACH transmission, validation rules applied after determining the ROs for multiple PRACH occasions would result in different UE behaviours of single PRACH transmission and multiple PRACH transmissions in invalid ROs. Hence, we propose that the validation rule should be performed before determining the ROs used for multiple PRACH transmissions.

Proposal 18: Validation rule should be performed before determining the ROs used for multiple PRACH transmissions.

Multiple PRACH transmissions with different beams

Potential use cases for multiple PRACH transmissions with different beams have been discussed in the past few meeting and the latest conclusion was achieved in the last meeting [3].

Conclusion For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS. Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

Indeed, two different implementations can be considered for different beams associated with the same SSB/CSI-RS. One is to assume that different beams are narrower beams corresponding to a same SSB beam. Another is to assume that the direction of UE transmitting beam is slightly different from that of gNB receiving beam. In this condition, the actual beam direction of different beams is based on UE implementation, i.e. fine tune the whole beam direction corresponding to a same SSB beam. Regardless, the target use case and potential benefit for PRACH coverage enhancement should be provided.

In general, the benefit and target scenario of multiple PRACH transmissions with different beams is unclear for us, which should be provided to justify the support of multiple PRACH transmissions with different beams.

Proposal 19: The benefit and target scenario of multiple PRACH transmissions with different beams should be provided to justify the support of multiple PRACH transmissions with different beams.

Conclusion

In this contribution, we share our views on PRACH coverage enhancement. The proposals are summarized as follows:

Proposal 1: For multiple PRACH transmissions with same beam, it is not supported to utilize different preambles in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Proposal 2: For multiple PRACH transmissions with same beam, UE monitors RAR after the PRACH occasion corresponding to the last PRACH repetition.

Proposal 3: RA-RNTI is calculated based on the PRACH occasion corresponding to the first PRACH repetition or the last repetition.

Proposal 4: For multiple PRACH transmissions with same beam, at least for the first RACH attempt in a RACH procedure, SSB/CSI-RS is selected based on SSB/CSI-RS RSRP thresholds as in existing specifications.

- FFS: whether to introduce separate RSRP thresholds for SSB/CSI-RS selection for PRACH repetitions from legacy PRACH transmission without repetition.

Proposal 5: For multiple PRACH transmissions with same beam, at least for the first RACH attempt, only SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions for CBRA.

- SSB-RSRP threshold(s) for determination of number of PRACH transmissions are separately configured from the SSB-RSRP threshold(s) for SSB selection.

Proposal 6: For CFRA triggered by PDCCH order, the number of PRACH transmissions is indicated by the PDCCH order at least for the first RACH attempt.

Proposal 7: For multiple PRACH transmissions with same Tx beam, for subsequent RACH attempt(s), power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met.

Proposal 8: If PRACH repetitions are triggered, Msg3 PUSCH repetition is enabled.

Proposal 9: The RSRP threshold(s) for PRACH repetition request should be separate from that for Msg3 repetition request.

Proposal 10: Confirm the following two Working Assumptions with updates:

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Proposal 11: Multiple PRACH transmissions with same Tx beam is introduced as a new feature.

Proposal 12: Multiple values for the number of multiple PRACH transmissions can be regarded as different features when configuring the framework of feature combination and additional RACH configuration.

Proposal 13: It is not necessary to address the collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features.

Proposal 14: For multiple PRACH transmissions with same Tx beam on separate ROs, new SSB-to-RO mapping pattern is defined from one of the following options.

- Option 1: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from SFN #0. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain.

- Option 2: Current SSB-to-RO mapping is applied to the first of every N consecutive valid ROs in time domain starting from the first system frame every 160ms. The same SSB-to-RO mapping of the first of every N consecutive valid ROs is applied to the remaining valid ROs in every N consecutive ROs in time domain. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions.

Proposal 15:  For multiple PRACH transmissions with same Tx beam on shared RO, to determine the starting position and the time span of a RO group, down select from the following two options:

- Option 1: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from SFN #0.

- Option 2: every N consecutive valid ROs mapped to the same SS/PBCH block index are bundled as one RO group starting from the first system frame every 160ms. ROs not associated with SS/PBCH block indexes after an integer number of RO groups for all SS/PBCH block indexes, if any, are not used for PRACH transmissions.

Proposal 16: Further discuss on whether the association period and association pattern period are needed for multiple PRACH transmissions.

Proposal 17: Frequency hopping can be considered for multiple PRACH transmissions with same beam.

- FFS: how to determine the starting RB of different PRACH transmissions.

Proposal 18: Validation rule should be performed before determining the ROs used for multiple PRACH transmissions.

Proposal 19: The benefit and target scenario of multiple PRACH transmissions with different beams should be provided to justify the support of multiple PRACH transmissions with different beams.

Reference

- RP-213579, New WI: Further NR coverage enhancements, China Telecom, RAN#94-e, December 6th – 17th, 2021.

- Chairman's Notes RAN1#110bis-e, 3GPP TSG RAN WG1 Meeting #110bis-e, e-Meeting, October 10th – 19th, 2022.

- Chairman's Notes RAN1#112, 3GPP TSG RAN WG1 Meeting #112 Meeting, February 27th –March 3rd, 2023.

- TR38.321, Medium Access Control (MAC) protocol specification, v17.3.0.

- TR38.213, Physical layer procedures for control, v17.3.0.

- Chairman's Notes RAN1#111, 3GPP TSG RAN WG1 Meeting #111 Meeting, November 14th – 18th, 2022.

- TR38.830, Study on NR coverage enhancements, v17.0.0.

- TR36.211, Physical channels and modulation, v17.2.0.