3GPP TSG RAN WG1 #112bis-e		                                                      R1-2302691

e-Meeting, April 17th –26th, 2023

Source:	CATT

Title:	Discussion on enhancements to reduce MPR/PAR

Agenda Item:	9.12.2

Document for:	Decision

- Introduction

One of the objectives in Rel-18 further NR coverage enhancements WID [1] is power domain enhancements as follows:

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution, we provide our views on enhancements to reduce MPR/PAR.

- Discussion

According to the following agreement, it was agreed that at least pi/2-BPSK and QPSK modulation are considered for power-domain enhancements targeting MPR/PAR reduction and FFS higher modulation orders.

Agreement For power-domain enhancements targeting MPR/PAR reduction, study the following configurations for DFT-S-OFDM: - At least pi/2-BPSK and QPSK modulation are considered - FFS: other modulations, e.g., 16-QAM - Any number of RB can be considered - The starting RB of the allocation can be any RB in the BWP - FFS: - Whether restrictions on the number of allocated RB or on the starting RB of the allocation are considered.

It is more reasonable for coverage limited UEs to use a relatively low modulation order to achieve better transmission performance. Hence, modulation orders higher than QPSK, e.g. 16 QAM, should not be considered for MPR/PAR reduction. In addition, as mentioned in [2][3], FDSS without spectrum extension with pi/2-BPSK has been extensively studied and provides more significant PAR reduction compared with FDSS with spectrum extension. Hence, we do not think any further enhancements are needed for pi/2-BPSK.

Proposal 1: For power-domain enhancements targeting MPR/PAR reduction, only QPSK modulation is considered for DFT-s-OFDM.

The discussions and simulations in this contribution focus on the DMRS sequence length before extension larger than or equal to 30. It was agreed to further study the following approaches for DMRS in this case in RAN1#112 meeting [4].

Agreement If FDSS-SE is supported in Rel-18, RAN1 to further study the following approaches for DMRS, when the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30: - Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered: - A.1: The sequence is a Type 1 DMRS sequence. - A.2: The sequence is a Type 2 DMRS sequence. FFS: how the sequence is extended. - Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. Note: if type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification Performance metrics considered for the study are PAPR, CM[, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

For Approach A, DMRS sequence length depends on the number of PRBs in the inband and then cyclically extends to the PRBs in the extension band. The sequence can be a Type 1 or Type 2 DMRS sequence. Considering that Type 1 DMRS sequence is Zadoff-Chu sequence, there are two alternative extension methods to achieve the sequence extension. One is to reuse the same logic of cyclically extending Zadoff-Chu sequence to generate DMRS in the inband, i.e. per RE extension, which can preserve the characteristic of low PAPR. Another is to reuse the same extension method as data, e.g. symmetric extension. In this case, DMRS sequence is per RB extended. Considering the cyclic extension has already been performed based on Zadoff-Chu sequence to generate a DMRS sequence in the inband, directly copying the PRBs on both edges in the inband to the extended PRBs will destroy the continuity of the Zadoff-Chu sequence and increase the PAPR as we provided in [5]. Type 2 DMRS sequence is pseudo-random sequence, which can directly reuse the same extension method as data.

For Approach B, DMRS sequence length depends on the number of PRBs in the inband and extension band without extension. Also, the sequence can be a Type 1 or Type 2 DMRS sequence.

We first evaluate the PAPR and CM performance for DMRS and data considering different DMRS sequence generation approaches according to the following configurations in Table 1. 3-tap filter with factors (0.28, 1, 0.28) is assumed and other simulation assumptions can be found in Appendix.

Table 1: Simulation cases of FDSS

| Case | No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| Case 1 | 8 | 6 | 6 | 8 | 8 | 1/4 |
| Case 2 | 40 | 2 | 30 | 40 | 3 | 1/4 |

The PAPR and CM of data and DMRS without FDSS with FDSS-SE for Case 1 and Case 2 in Table 1 are shown in Figure 1 and Figure 2, respectively.

Figure 1: PAPR and CM for no FDSS and FDSS with SE (QPSK, 6+2 PRBs, SE factor = 1/4).

Figure 2: PAPR and CM for no FDSS and FDSS with SE (QPSK, 30+10 PRBs, SE factor = 1/4).

The PAPR and CM@10% CCDF of data and DMRS are summarized in Table 2.

Table 2: PAPR and CM performance under case 1 and case 2

|  |  |  | Case 1 |  | Case 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No FDSS | FDSS-SE | No FDSS | FDSS-SE |
| PAPR (dB) | Data |  | 6.36 | 3.3 | 6.87 | 3.38 |
|  | DMRS | Approach A.2_per RE | 5.02 | 2.3 | 6.11 | 3.05 |
|  |  | Approach A.1_per RB |  | 4.91 |  | 6.17 |
|  |  | Approach A.2 |  | 2.04 |  | 2.18 |
|  |  | Approach B.1 |  | 3.62 |  | 3.84 |
|  |  | Approach B.2 |  | 1.8 |  | 2.03 |
| CM (dB) | Data |  | 1.57 | 0.47 | 1.39 | 0.35 |
|  | DMRS | Approach A.1_per RE | 1.78 | 0.08 | 2.29 | 0.3 |
|  |  | Approach A.1_per RB |  | 1.27 |  | 1.78 |
|  |  | Approach A.2 |  | -0.67 |  | -0.72 |
|  |  | Approach B.1 |  | 1.49 |  | 1.55 |
|  |  | Approach B.2 |  | -0.72 |  | -0.77 |

It can be observed that all the approaches reduce the PAPR and CM of DMRS compared with PAPR and CM of DMRS without FDSS.

For Type 1 DMRS sequence, it is observed that Approach A.1 with per RB extension and Approach B.1 lead to higher PAPR and CM of DMRS compared with that of data for FDSS-SE. Therefore, for Type 1 DMRS sequence, if supported, Approach A with RE extension should be adopted.

Proposal 1: For Type 1 DMRS sequence, if supported, Approach A with RE extension should be adopted.

For Type 2 DMRS sequence, both Approach A and Approach B achieve lower PAPR and CM of DMRS compared with that of data for FDSS-SE.

Observation 1: For FDSS with SE, Type 2 DMRS sequence with both Approach A and Approach B achieve lower PAPR/CM of DMRS compared with PAPR/CM of data.

The BLER performances using Type 1 DMRS sequence with RE extension and Type 2 DMRS sequence with both Approach A (Approach A.2) and Approach B (Approach B.2) for Case 1 and Case 2 in Table 1 are shown in Figure 3 and Figure 4, respectively. The excess bands are dropped at the receiver side.

Figure 3: BLER for Type 1 DMRS sequence and Type 2 DMRS sequence (Case 1).

Figure 4: BLER for Type 1 DMRS sequence and Type 2 DMRS sequence (Case 2).

1.07dB and 0.13dB degradation @10% BLER are observed for Type 2 DMRS sequence with both Approach A and Approach B compared with that for Type 1 DMRS sequence with RE extension in Case 1 and Case 2, respectively. In addition, if Type 2 DMRS sequence is used, there is additional limitation that the number of PRBs in the inband and extension band must be valid DFT sizes.

Considering the BLER performance degradation and the additional scheduling restriction of Type 2 DMRS sequence, Type 1 DMRS sequence is preferred.

Observation 2: Type 2 DMRS sequence leads to BLER performance degradation and additional limitation that the number of PRBs in the inband and extension band must be valid DFT sizes compared with Type 1 DMRS sequence.

Proposal 2: For DMRS sequence length before extension larger than or equal to 30 for FDSS-SE if supported, DMRS sequence is generated according to Approach A.1 with RE extension.

- Conclusions

In this contribution, we discussed power domain enhancements with the following observations and proposals.

Observation 1: For FDSS with SE, Type 2 DMRS sequence with both Approach A and Approach B achieve lower PAPR/CM of DMRS compared with PAPR/CM of data.

Observation 2: Type 2 DMRS sequence leads to BLER performance degradation and additional limitation that the number of PRBs in the inband and extension band must be valid DFT sizes compared with Type 1 DMRS sequence.

Proposal 1: For Type 1 DMRS sequence, if supported, Approach A with RE extension should be adopted.

Proposal 2: For DMRS sequence length before extension larger than or equal to 30 for FDSS-SE if supported, DMRS sequence is generated according to Approach A.1 with RE extension.

- Reference

- RP-221858, Revised WID on Further NR coverage enhancements, China Telecom, RAN#96.

- R4-2217745, WF on enhancements to reduce MPR/PAR, Nokia, Nokia Shanghai Bell, RAN4#104bis-e.

- R4-2215514, “Scope of the work for MPR/PAR objective”, Nokia, Nokia Shanghai Bell, RAN4#104bis-e.

- Chairman's Notes RAN1#112, 3GPP TSG RAN WG1 Meeting #112 Meeting, February 27th –March 3rd, 2023.

- R1-2300668, Discussion on enhancements to reduce MPR/PAR, CATT, RAN1#112.

# Appendix

Table 4: simulation assumptions

| Channel | PUSCH, 14 symbols |
| --- | --- |
| Carrier frequency and scenario | 4GHz (Urban), |
| Channel BW | 100MHz for Urban |
| SCS | 30 kHz (4GHz), |
| Channel model | TDL-C 300ns for FR1 Urban (4GHz), |
| UE speed | 3km/h |
| Waveform | According to agreements |
| Modulation | According to agreements |
| Number of Tx antennas | 1, |
| Number of Rx antennas | 4 for FR1 Urban, |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| HARQ configuration | No retransmissions |
| Frequency hopping | Disabled |
| Number of PRBs | Reported by companies |
| MCS | Chosen as a function of the number of PRBs to guarantee same spectral efficiency between MPR/PAR reduction solutions and baseline/benchmarks as per agreements |
| Extension factor [FDSS-SE] / sideband size [TR] (α) | [1/4] |
| BLER | 10% |