3GPP TSG RAN WG1 #112bis-e		                                                      R1-2302692

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Dynamic switching between DFT-S-OFDM and CP-OFDM

Agenda Item:	9.12.3

Document for:	Decision

- Introduction

The following agreements and conclusions were made in RAN1#112 on dynamic switching between DFT-S-OFDM and CP-OFDM.

Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In this contribution, we provide our views on remaining issues on dynamic switching between DFT-S-OFDM and CP-OFDM based on above agreements/working assumption.

- Discussion

Applicability of dynamic waveform switching

It was agreed that dynamic waveform switching is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1 and is not applicable to Type 1 CG PUSCH. In addition, it was concluded that there is no consensus to support dynamic waveform switching for Type 2 CG PUSCH. We discuss the applicability to PUSCH scheduled by RAR UL grant and DCI format 0_0 below.

PUSCH scheduled by RAR UL grant

For PUSCH transmissions during random access, including PUSCH scheduled by RAR UL grant, PUSCH scheduled by fallbackRAR UL grant, PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI and MsgA PUSCH, the motivation and benefit of dynamic switching between DFT-S-OFDM and CP-OFDM are not clear. In order to differentiate UEs supporting dynamic UL waveform switching from legacy UEs, additional PRACH/preamble resource partitioning would be required which is not desirable. In addition, there would be significant specification impact on RAR UL grant, fallbackRAR UL grant and DCI format 0_0 design. It is therefore proposed that dynamic switching between DFT-S-OFDM and CP-OFDM is not applicable to PUSCH transmissions during random access.

PUSCH scheduled by DCI format 0_0

Dynamic switching of UL waveforms for PUSCH transmissions scheduled by non-fallback DCI formats has already been supported. For PUSCH transmissions scheduled by fallback DCI formats, the DMRS configuration and DMRS port are fixed in the specification and only RA type 1 is supported. The potential benefit to support dynamic switching of UL waveforms for PUSCH transmissions scheduled by fallback DCI formats, if any, would be very limited without significant standardization efforts. Given that dynamic switching between DFT-S-OFDM and CP-OFDM can already be supported in existing specifications by switching fallback DCI and non-fallback DCI and by non-fallback DCI in Rel-18, it is not necessary to support dynamic switching of UL waveforms for PUSCH transmissions scheduled by fallback DCI formats.

Proposal 1: Dynamic switching between DFT-S-OFDM and CP-OFDM is not applicable to:

- PUSCH scheduled by RAR UL grant

- PUSCH scheduled by fallback RAR UL grant

- MsgA PUSCH

- PUSCH scheduled by DCI format 0_0

DMRS type for different waveforms

DMRS type 1 is applicable to both CP-OFDM and DFT-S-OFDM while DMRS type 2 is applicable to CP-OFDM only but not applicable to DFT-S-OFDM.

According to current specifications, DMRS type 1 applies to PUSCH transmissions during random access and PUSCH scheduled by DCI format 0_0. For CG PUSCH, DMRS type is configured per CG configuration. Given that it has been concluded that dynamic waveform switching is not applicable to CG PUSCH, semi-static configuration of DMRS type for CG PUSCH is fine, which is not expected to be conflict with the configured waveform for a certain CG configuration.

For PUSCH scheduled by non-fallback DCI, for each DMRS mapping type (i.e. DMRS mapping Type A and mapping Type B), one DMRS type is configured by dmrs-Type in DMRS-UplinkConfig. If DMRS type 2 is configured and DFT-S-OFDM is dynamically indicated, it is problematic.

All the four potential combinations of DMRS types for CP-OFDM and DFT-S-OFDM are summarized in Table 1. Combination #1-2 assumes a single configuration of DMRS type to be applied to both waveforms and it is problematic as discussed above. Combination #1-4 assumes separate configurations of DMRS types for different waveforms but DMRS type 2, which is not applicable to DFT-S-OFDM, is configured for DFT-S-OFDM. It is clearly not a reasonable combination and thus does not need to be considered.

Table 1: DMRS types for CP-OFDM and DFT-S-OFDM

|  | CP-OFDM | DFT-S-OFDM | Comments |
| --- | --- | --- | --- |
| Combination #1-1 | DMRS type 1 | DMRS type 1 | OK |
| Combination #1-2 | DMRS type 2 | DMRS type 2 | Problematic for DFT-S-OFDM For single configuration for both waveforms only |
| Combination #1-3 | DMRS type 2 | DMRS type 1 | OK |
| Combination #1-4 | DMRS type 1 | DMRS type 2 | Not reasonable |

To resolve the problem of combination #1-2 in Table 1, the following options can be considered:

- Option 1: For dynamic waveform switching, keep one DMRS type configuration for each DMRS mapping type in PUSCH-Config as in existing specification and the configuration applies to both waveforms.

  - Only DMRS type 1 can be configured, i.e. DMRS type 2 cannot be configured.

- Option 2: For dynamic waveform switching, keep one DMRS type configuration for each DMRS mapping type in PUSCH-Config as in existing specification.

  - If DMRS type 1 is configured, it is applied to both waveforms;

  - If DMRS type 2 is configured, for PUSCHs scheduled by non-fallback DCI, DMRS type 2 is applied if CP-OFDM is indicated and DMRS type 1 is applied if DFT-S-OFDM is indicated.

- Option 3: For dynamic waveform switching, introduce separate DMRS type configurations for CP-OFDM and DFT-S-OFDM respectively for each DMRS mapping type in PUSCH-Config.

  - It is not expected that DMRS type 2 is configured for DFT-S-OFDM.

Option 1 supports combination #1-1 only while Option 2 and Option 3 additionally support combination #1-3. Compared with Option 2, Option 3 would have RRC impact and the signaling overhead is higher. So there is no advantage of Option 3 compared with Option 2. It is proposed to consider Option 1 and Option 2 only.

Proposal 2: For dynamic waveform switching, keep one DMRS type configuration for each DMRS mapping type in PUSCH-Config as in existing specification. Down-select from the following two options:

- Option 1: only DMRS type 1 can be configured and the configuration applied to both waveforms

- Option 2: both DMRS type 1 and DMRS type 2 can be configured

  - If DMRS type 1 is configured, it is applied to both waveforms.

  - If DMRS type 2 is configured, for PUSCHs scheduled by non-fallback DCI, DMRS type 2 is applied if CP-OFDM is indicated and DMRS type 1 is applied if DFT-S-OFDM is indicated.

Configuration of RA type for different waveforms

RA type 1 is applicable to both CP-OFDM and DFT-S-OFDM while RA type 0 is applicable to CP-OFDM only but not applicable to DFT-S-OFDM.

According to current specifications, RA type 1 is applied for PUSCH scheduled by fallback DCI. For CG PUSCH, RA type is configured per CG configuration. Given that it has been concluded that dynamic waveform switching is not applicable to CG PUSCH, semi-static configuration of RA type for CG PUSCH is fine, which is not expected to be conflict with the configured waveform for a certain CG configuration.

For PUSCH scheduled by DCI format 0_1 and 0_2, RA types are configured by resourceAllocation and resourceAllocationDCI-0-2 in PUSCH-Config respectively, which can be resourceAllocationType0, resourceAllocationType1 or dynamicSwitch. If resourceAllocationType0 is configured and DFT-S-OFDM is indicated, it is problematic.

All the potential combinations for RA types for CP-OFDM and DFT-S-OFDM are summarized in Table 2. Combinations #2-2, #2-3, #2-7 and #2-8 assume separate configurations of RA types for different waveforms. But RA type 0 only, which is not applicable to DFT-S-OFDM, is configured for DFT-S-OFDM for combinations #2-2 and #2-3. Dynamic switching is configured for DFT-S-OFDM but not for CP-OFDM for combinations #2-7 and #2-8 even though only RA type 1 is applicable for DFT-S-OFDM while both RA types are applicable for CP-OFDM. Combinations #2-2, #2-3, #2-7 and #2-8 are clearly not reasonable combinations and thus do not need to be considered.

Table 2: RA types for CP-OFDM and DFT-S-OFDM

|  | CP-OFDM | DFT-S-OFDM | Comments |
| --- | --- | --- | --- |
| Combination #2-1 | RA type 0 only | RA type 0 only | Problematic for DFT-S-OFDM For single configuration for both waveforms only |
| Combination #2-2 | RA type 1 only | RA type 0 only | Not reasonable |
| Combination #2-3 | Dynamic switch | RA type 0 only | Not reasonable |
| Combination #2-4 | RA type 0 only | RA type 1 only | OK |
| Combination #2-5 | RA type 1 only | RA type 1 only | OK |
| Combination #2-6 | Dynamic switch | RA type 1 only | OK |
| Combination #2-7 | RA type 0 only | Dynamic switch | Not reasonable |
| Combination #2-8 | RA type 1 only | Dynamic switch | Not reasonable |
| Combination #2-9 | Dynamic switch | Dynamic switch | OK |

To resolve the problem of combination #2-1 in Table 2, the following options can be considered:

- Option 1: For dynamic waveform switching, keep one RA type configuration in PUSCH-Config as in existing specification and the configuration applies to both waveforms.

  - Only resourceAllocationType1 and dynamicSwitch can be configured, i.e. resourceAllocationType0 cannot be configured.

- Option 2: For dynamic waveform switching, keep one RA type configuration in PUSCH-Config as in existing specification.

  - Option 2-1: only resourceAllocationType1 and dynamicSwitch can be configured, i.e. resourceAllocationType0 cannot be configured.

    - If dynamicSwitch is configured, RA type 1 is applied for PUSCHs scheduled by non-fallback DCI and indicated to be DFT-S-OFDM

  - Option 2-2: resourceAllocationType0, resourceAllocationType1 and dynamicSwitch can be configured.

    - If resourceAllocationType0 is configured, for PUSCHs scheduled by non-fallback DCI, RA type 0 is applied if CP-OFDM is indicated and RA type 1 is applied if DFT-S-OFDM is indicated.

    - If dynamicSwitch is configured,

      - Option 2-2-1: The configuration applies to both waveforms.

      - Option 2-2-2: For PUSCHs scheduled by non-fallback DCI, RA type 1 is applied if DFT-S-OFDM is indicated.

- Option 3: For dynamic waveform switching, introduce separate RA type configurations for CP-OFDM and DFT-S-OFDM respectively in PUSCH-Config.

  - If dynamic waveform switching is enabled, it is not expected that RA type 0 only is configured for DFT-S-OFDM.

  - If dynamic waveform switching is enabled, it is not expected that dynamic switch is configured for DFT-S-OFDM and RA type 0 only or RA type 1 only is configured for CP-OFDM.

Whether each option supports or not support combinations #2-4/#2-5/#2-6/#2-9 are summarized in the following table.

|  | CP-OFDM | DFT-S-OFDM | Option 1 | Option 2-1 | Option 2-2-1 | Option 2-2-2 | Option 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Combination #2-4 | RA type 0 only | RA type 1 only | Not support | Not support | Support | Support | Support |
| Combination #2-5 | RA type 1 only | RA type 1 only | Support | Support | Support | Support | Support |
| Combination #2-6 | Dynamic switch | RA type 1 only | Not support | Support | Not support | Support | Support |
| Combination #2-9 | Dynamic switch | Dynamic switch | Support | Not support | Support | Not support | Support |

Considering that dynamic switch can support both RA type 0 and RA type 1 dynamically and only RA type 1 is applicable for DFT-S-OFDM, it is not necessary to support both combination #2-6 and combination #2-9. It is noted that all the options at least support one of combinations #2-6 and #2-9.

For combination #2-4, from DCI overhead perspective, if the size of FDRA field is aligned between CP-OFDM and DFT-S-OFDM, combination #2-4 can only save 1 bit compared with combinations #2-6 and #2-9. Even if the size of FDRA field is not aligned between CP-OFDM and DFT-S-OFDM, considering that the overhead of RA type 0 is typically higher than RA type 1, there is still only 1 bit saving compared with combination #2-6. But combination#2-4 is less flexible since RA type 1 is not supported for CP-OFDM compared with combination #2-6/#2-9. Therefore, there seems to be no strong motivation to support combination #2-4.

Based on the above analysis, Option 3 which would have RRC impact and higher signaling overhead is not considered. Option 1-1, which simply follows the RRC configuration without adaptation for DFT-S-OFDM is our first preference. Option 2-2-1 can also be considered as it follows the same principle as for Option 2 in Proposal 2 for DMRS type configuration, where a configuration that is not applicable for DFT-S-OFDM is configured for both waveforms and it is interpreted as another configuration for DFT-S-OFDM instead.

Proposal 3: For dynamic waveform switching, keep one RA type configuration in PUSCH-Config as in existing specification. Down-select from the following two options:

- Option 1 (1st preference): only resourceAllocationType1 and dynamicSwitch can be configured and the configuration applied to both waveforms

- Option 2( 2nd preference): resourceAllocationType0 can be configured

  - If resourceAllocationType0 is configured, for PUSCHs scheduled by non-fallback DCI, RA type 0 is applied if CP-OFDM is indicated and RA type 1 is applied if DFT-S-OFDM is indicated.

  - If resourceAllocationType1 or dynamicSwitch is configured, the configuration applied to both waveforms.

DCI size alignment between CP-OFDM and DFT-S-OFDM

For DCI size alignment between CP-OFDM and DFT-S-OFDM, two approaches were discussed:

- Align DCI size per format

- Align DCI size per field

During the discussions in RAN1#112, the issue of DCI size alignment between C-RNTI and CS-RNTI was brought up. To be more specific, NDI in DCI addressed to CS-RNTI is used to indicate whether the DCI is for CG Type 2 activation or CG retransmission. However, the sizes of Frequency domain resource assignment and Frequency hopping flag preceding NDI field can be different for DCI for CG Type 2 activation and DCI for CG retransmission, which are based on separate RRC parameters. In order to avoid the ambiguity of NDI field position in the DCI, it was agreed to adopt per-field alignment between C-RNTI and CS-RNTI [2].

Considering the differentiation between CG Type 2 activation DCI and CG retransmission scheduling DCI, four options were identified for DCI size alignment when DWS indication is configured in DCI format 0_1/0_2 as captured in FL summary in [3].

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

Among the four options, Option 1 performs per-format alignment. Two alternatives are to be down-selected depending on whether any DCI field preceding NDI has size that depends on waveform. Among the DCI fields preceding NDI, FDRA can potentially have waveform dependent field size. Based on Proposal 3, if the configured RA type applies to both waveforms, the field size FDRA is not dependent of waveform so Alt. B is adopted. Otherwise if the RA type is different if resourceAllocationType0 is configured, both Alt. A and Alt. B are possible. For simplicity, Alt. B is preferred.

Option 2 performs per-field alignment which is quite clear and simple. Given that Alt. B is assumed for Option 1, Option 3 which is a mix of Option 1 and Option 2 is not relevant since the fields preceding NDI are not waveform dependent. Option 4 applies per-format alignment between different waveforms for DCI addressed to C-RNTI and per-field alignment for DCI address to CS-RNTI. But fields introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM; otherwise cannot be used for CS-RNTI and NDI=1. In addition, the advantage of Option 4 is unclear.

Among the four options, Option 2 is still our preference.

Proposal 4: For DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2, Option 2 (per-field alignment) is adopted.

- Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM).

- Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI.

- Conclusions

In this contribution, we discussed dynamic switching between DFT-S-OFDM and CP-OFDM with the following proposals.

Proposal 1: Dynamic switching between DFT-S-OFDM and CP-OFDM is not applicable to:

- PUSCH scheduled by RAR UL grant

- PUSCH scheduled by fallback RAR UL grant

- MsgA PUSCH

- PUSCH scheduled by DCI format 0_0

Proposal 2: For dynamic waveform switching, keep one DMRS type configuration for each DMRS mapping type in PUSCH-Config as in existing specification. Down-select from the following two options:

- Option 1: only DMRS type 1 can be configured and the configuration applied to both waveforms

- Option 2: both DMRS type 1 and DMRS type 2 can be configured

  - If DMRS type 1 is configured, it is applied to both waveforms.

  - If DMRS type 2 is configured, for PUSCHs scheduled by non-fallback DCI, DMRS type 2 is applied if CP-OFDM is indicated and DMRS type 1 is applied if DFT-S-OFDM is indicated.

Proposal 3: For dynamic waveform switching, keep one RA type configuration in PUSCH-Config as in existing specification. Down-select from the following two options:

- Option 1 (1st preference): only resourceAllocationType1 and dynamicSwitch can be configured and the configuration applied to both waveforms

- Option 2( 2nd preference): resourceAllocationType0 can be configured

  - If resourceAllocationType0 is configured, for PUSCHs scheduled by non-fallback DCI, RA type 0 is applied if CP-OFDM is indicated and RA type 1 is applied if DFT-S-OFDM is indicated.

  - If resourceAllocationType1 or dynamicSwitch is configured, the configuration applied to both waveforms.

Proposal 4: For DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2, Option 2 (per-field alignment) is adopted.

- Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM).

- Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI.

- Reference

- RP-221858, Revised WID on Further NR coverage enhancements, China Telecom, RAN#96

- R1-1813935, Offline discussion on remaining issues for UL transmission with configured grant, NTT DOCOMO, RAN1#95

- R1-2302222, Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM, Moderator (InterDigital), RAN1#112