3GPP TSG RAN WG1 #112bis-e                                         R1-2302697

e-Meeting, April 17th – April 26th, 2023

Source: 	CATT

Title:	Evaluation on AI/ML-based beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting, the following agreements were made for evaluation on AI/ML for spatial-domain beam prediction and time-domain prediction [1]:

Agreement - Further study the impact of quantization error of inputed L1-RSRP (for training and inference) for AI/ML model for beam management. - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model. Agreement - Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy. Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options are not precluded and can be reported by companies. - Note: This is only for evaluation discussion Agreement - For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as: - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam. Agreement - For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector. - Detailed assumption can be reported by companies. - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly - Note: Specification impact can be discussed separately. Agreement - Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable) - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C - Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

In this contribution, evaluation methodology and KPIs for AI/ML based beam management enhancement are discussed. Simulation results for beam pair and Tx beam prediction are also provided.

- Discussion

  - Simulation assumptions

Based on the agreements in the previous meeting [2], the Dense Urban (macro-layer only, TR 38.913) scenario is the basic one for dataset generation and performance evaluation for AI/ML based beam management. Our simulation assumptions are shown in Table 33 in Annex.

In our simulation of BM-Case1 beam pair prediction, Set A comprises 128 beam pairs, consisting of 32 gNB DL Tx beams and 4 UE DL Rx beams, while Set B comprises 32 beam pairs, consisting of 8 selected gNB DL Tx beams and 4 UE DL Rx beams.

For BM-Case1 DL Tx beam prediction, Set A has 32 Tx beams and Set B has 8 Tx beams. According to the following agreements achieved in RAN1#111 meeting, we respectively evaluate the performance of DL Tx beam prediction with Rx beam option1, 2a, 2b and 3.

Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample. - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - Option 3: Measurements of random Rx beam(s) per model input sample

To predict DL beam pairs and DL Tx beam, we utilize both fixed and variable patterns in our simulations. Figure 1 displays the fixed pattern, with 8 Tx beams chosen from 32 Tx beams. For the variable pattern, we simulate Opt B and Opt C in accordance with the following RAN1#110b-e meeting agreement. For Opt B, two pre-configured pattern sets which both include four pre-configured fixed patterns are illustrated in Figure 2 and Figure 3. In Opt C, the 8 gNB DL Tx beams are randomly selected from 32 gNB DL Tx beams. To predict beam pairs, all 4 RX beams are employed to receive each DL Tx beam.

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.

Figure 1: Fixed pattern 1 for DL Tx beam

Figure 2: Pre-configured pattern Set-1 for DL Tx beam

Figure 3: Pre-configured pattern Set-2 for DL Tx beam

For BM-Case2 beam pair prediction, Set A consists of 256 beam pairs, comprising of 32 gNB DL Tx beams and 8 UE DL Rx beams, whereas Set B consists of 8 beam pairs selected randomly in Set A.

  - Model description

In our simulation, we investigated a ResNet-based AI/ML model, as illustrated in Figure 4. The model comprises a three-layer fully connected structure and two Resblocks. Each Resblock includes a three-layer convolution structure, with residual calculation performed between the first and last convolution layer. This structure is repeated between the two Resblocks. The input data is the L1-RSRP vector (and beam ID), and the output of the ID model includes the Top-N Tx beam ID for Tx beam prediction and Tx-Rx beam pair ID for beam pair prediction. The output of the RSRP model includes the RSRP values of all Tx beam for Tx beam prediction or the RSRP values of all beam pairs for beam pair prediction.

Figure 4: ResNet based Model

In our simulation of temporal beam prediction, we used a MLP-Mixer-based AI/ML model. As shown in Figure 5, The MLP-Mixer consists of per-patch linear embeddings, mixer layers, and a classifier head. Mixer layers contain one token-mixing MLP and one channel-mixing MLP, each consisting of two fully-connected layers and GELU nonlinearity. Other components include: skip-connections, dropout, and layer norm on the channels. The model splits the input into multiple patches (each patch does not overlap), convert each patch into a feature embedding through the operation of the per-patch fully-connected layer, and then send it to N mixer Layers. Finally, mixer combines the standard classification header with the global average pooling layer, and then uses fully connected for classification. The input to the model is the RSRP value of Set B with random pattern which is selected from Set A in observation window T1. The model outputs all RSRP value of Set A in predict window T2. To achieve this, the input RSRP values in the observation window T1 are split into multiple patches, and pass through a per-patch fully-connected layer to obtain feature embedding. These embeddings are then sent it to N mixer Layers, where the horizontal and vertical dimensions of the patches are mixed by transposing and dividing them into multiple patches again. Finally, the output of all RSRP value of Set A in predict window T2 are obtained through the global average pooling layer and fully connected layer.

Figure 5: MLP-Mixer-based model

  - KPI

During the previous meetings, the KPIs for evaluating the performance of AI/ML-based beam management were discussed. Based on the agreements made in the previous meetings, the KPIs can be divided into two categories: beam prediction accuracy-related KPIs and system level performance-related KPIs.

In our simulation, we use the following KPIs:

- Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

- Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”. The value of K is 3.

- Average L1-RSRP difference of Top-1 predicted beam.

For the AI/ML model taking L1-RSRP as model output, we also take predicted L1-RSRP as KPI. The definition of predicted L1-RSRP is as follows.

- The difference between the predicted L1-RSRP of Top-1/K predicted beam and the ideal L1-RSRP of the same beam.

For both DL beam pair prediction and DL Tx beam prediction, Option A in the following agreement is used for the definition of Top-1 genie-aided Tx beam/Tx-Rx beam pair.

Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options are not precluded and can be reported by companies. - Note: This is only for evaluation discussion

Simultaneously, we adopt the following assumptions to calculate the RS overhead reduction for BM-Case1 and BM-Case2.

RS overhead reduction for BM-Case1:

- Option 1: RS OH reduction%=1-NM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

  - where M is the total number of beams (pairs) to be predicted

RS overhead reduction for BM-Case2:

- Option 2: RS OH reduction%=1-NM

  - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

  - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

  - Beam management procedures

During the discussion in previous meeting, FL summarized the following options for beam management procedure for both BM-Case1 and BM-Case2.

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - P2 is used to select Top-1 DL Tx beam for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams can be recommended by UE for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

- Other options are not precluded.

We generally agree with the above options, except for the following two comments:

For Option 2 and Option 4, we think P2 is not mandatory for selecting the Top-1 DL Tx beam. The gNB can always select one DL Tx beam among the Top-K Tx beams, e.g., based on resource allocation, which is similar to the legacy mechanism. In the legacy mechanism, UE can report up to 4 beams and gNB select one of them for DL data transmission.

For Tx-Rx beam pair prediction for gNB side AI/ML, the Rx beam of the selected Tx-Rx beam pair can be recommended by gNB for DL data reception, if some assistance information can be provided by UE. The details of assistance information for beam indication can be studied in AI 9.2.3.2.

Therefore, we propose the following changes to the above options for beam management procedure for both BM-Case1 and BM-Case2.

Proposal 1: For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure:

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 Tx-Rx beam pair can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

    - the Rx beam of the Top-1 Tx-Rx beam pair can be recommended by gNB for DL data reception

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams of Top-K Tx-Rx beam pairs can be recommended by UE for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Top-K Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - the Rx beam corresponding with the selected DL Tx beam can be recommended by gNB

    - P3 procedure is not needed

- Other options are not precluded.

- Simulation results

We conducted an assessment of spatial beam prediction for both beam pair prediction and DL Tx beam prediction, utilizing fixed, random, and pre-configured patterns. We also evaluated the accuracy of combined time domain and spatial domain beam prediction. Furthermore, we investigated the generalization performance of the models. The details of dataset, model inputs/outputs and evaluation results can be found in the excel sheet we submitted together.

  - Beam pair prediction

    - Effect of fixed Set B pattern on prediction performance

In the evaluation of beam pair prediction, we use the fixed Set B pattern in Figure 1 as shown in Section 2.1. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference or predicted Top-1 RSRP difference are presented in Table 1.

Table 1: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with fixed pattern

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 90.47 | 80.05 |
| Top-3/1 | 99.99 | 99.88 |
| Average L1-RSRP diff | 0.0386 | 0.0411 |
| Predicted L1-RSRP diff | N/A | 0.4018 |

Observation 1：In Tx-Rx beam pair prediction with fixed Set B pattern, the prediction accuracy of top-3/1 can be greater than 99%.

    - Effect of random Set B pattern on prediction performance

We evaluate the variable Set B pattern of Opt C. The input is the RSRPs of a random Set B pattern changed among Set A beam pairs and implicitly with beam pair ID. The simulation results are shown in Table 2.

Table 2: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with variable Set B pattern of Opt C with beam ID as model input

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 70.84 | 70.06 |
| Top-3/1 | 87.00 | 86.79 |
| Average L1-RSRP diff | 1.0647 | 0.8029 |
| Predicted L1-RSRP diff | N/A | 0.9652 |

Observation 2：In Tx-Rx beam pair prediction with variable Set B pattern of Opt C, the prediction accuracy of top-3/1 can be greater than 85%.

Observation 3：In Tx-Rx beam pair prediction with variable Set B pattern of Opt C, the prediction accuracy has decreased compared to fixed Set B pattern.

Observation 4：For models taking RSRP as output, the prediction L1-RSRP difference can reflect the accuracy of Top-N beam prediction.

    - Effect of pre-configured pattern on prediction performance

We evaluated the performance of pre-configured patterns shown in Figure 2. The input is the L1-RSRPs of a Set B pattern randomly selected among the pre-configured patterns. The simulation results are shown in Table 3.

Table 3: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with pre-configured pattern

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 89.76 | 78.59 |
| Top-3/1 | 99.13 | 98.74 |
| Average L1-RSRP diff | 0.0402 | 0.1862 |
| Predicted L1-RSRP diff | N/A | 0.4769 |

Observation 5: For Beam pair prediction with variable Set B pattern of Opt B, the Top-3 accuracy can be greater than 95%.

Observation 6: For BM-Case1, variable pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns has better performance than Opt C.

  - DL Tx beam prediction

    - Effect of random pattern on prediction performance

The model input is the L1-RSRPs of a variable Set B pattern of Opt C, where Set B was randomly changed among Set A, implicitly with beam ID. The simulation results are shown in Table 4. We adopt option 2b for Rx beam selection, i.e., one specific Rx beam is used for all model input samples.

Table 4: Evaluation results for BM-Case1 without model generalization for Tx beam prediction with variable pattern of Opt C with Rx beam option 2b

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 58.33 | 43.25 |
| Top-3/1 | 70.43 | 66.87 |
| Average L1-RSRP diff | 1.4047 | 1.2817 |
| Predicted L1-RSRP diff | N/A | 2.3687 |

When Option 1 is chosen for Rx beam selection, which involves exhaustive beam sweeping to measure the "best" Rx beam for each model input sample, the simulation results are presented in Table 5.

Table 5: Evaluation results for BM-Case1 without model generalization for Tx beam prediction with variable pattern of Opt C with Rx beam option 1

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 75.12 | 67.69 |
| Top-3/1 | 90.25 | 90.07 |
| Average L1-RSRP diff | 0.6214 | 0.1745 |
| Predicted L1-RSRP diff | N/A | 0.5982 |

Observation 7：For DL Tx beam prediction with variable Set B pattern of Opt C, the prediction performance of Rx beam option 1, i.e., the “best” Rx beam, is better than that of option 2b, i.e., one specific Rx beam.

    - Effect of pre-configured pattern on prediction performance

In the evaluation of pre-configured patterns, four different patterns are designed for a pattern set. For the selection of Set B pattern, we evaluated pre-configured patterns shown in Figure 2. During the training and inference phase, Set B is randomly changed among the four pre-configured patterns. When Rx beam option 1 is selected, the simulation results are shown in Table 6.

Table 6: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with pre-configured patterns Set-1 and Rx beam option1

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 90.07 | 77.91 |
| Top-3/1 | 99.86 | 97.84 |
| Average L1-RSRP diff | 0.2955 | 0.1031 |
| Predicted L1-RSRP diff | N/A | 0.4013 |

We also evaluate the DL Tx beam prediction performance when training and inference using different pre-configured patterns. The pre-configured pattern used for inference is shown in Figure 3, and the pre-configured pattern used for training is shown in Figure 2. The simulation results are provided in Table 7.

Table 7: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction when training and inference use different pre-configured patterns with Rx beam option1

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 8.26 | 3.84 |
| Top-3/1 | 28.78 | 26.99 |
| Average L1-RSRP diff | 2.5731 | 1.1231 |
| Predicted L1-RSRP diff | N/A | 1.9523 |

Observation 8：For DL Tx beam prediction, the prediction accuracy is greatly degraded when training and inference using different pre-configured patterns.

    - Effect of Rx beam selection on prediction

We also evaluated different Rx beam selection for fixed Set B pattern. We used fixed pattern 1 in Figure 1, and the simulation assumptions are described in Section 2.1.

The simulation results of Rx beam option 1 are provided in Table 8.

Table 8: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 1

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 91.06 | 79.27 |
| Top-3/1 | 99.97 | 99.11 |
| Average L1-RSRP diff | 0.0412 | 0.0614 |
| Predicted L1-RSRP diff | N/A | 0.3012 |

For Rx beam option 2a, we select a specific Rx beam as the receiving beam for all Tx beams of a sample. We select the specific Rx beam for each sample in order of 1, 2, 3, 4, 1, 2, 3, 4,… The simulation results are provided in Table 9.

Table 9: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 2a

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 91.22 | 76.59 |
| Top-3/1 | 99.91 | 98.88 |
| Average L1-RSRP diff | 0.0508 | 0.0914 |
| Predicted L1-RSRP diff | N/A | 0.4327 |

For Option2b, we select the second Rx beam as the Rx beam of all samples. The simulation results are provided in Table 10.

Table 10: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 2b

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 82.07 | 75.21 |
| Top-3/1 | 98.68 | 98.20 |
| Average L1-RSRP diff | 0.0515 | 0.1068 |
| Predicted L1-RSRP diff | N/A | 0.4833 |

For Option3, a randomly selected Rx beam is used for each sample. The simulation results are provided in Table 11.

Table 11: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 3

| KPI | Model with beam ID as output | Model with RSRP as output |
| --- | --- | --- |
| Top-1 | 83.11 | 75.55 |
| Top-3/1 | 99.03 | 98.60 |
| Average L1-RSRP diff | 0.0516 | 0.1104 |
| Predicted L1-RSRP diff | N/A | 0.4492 |

Observation 9: Selecting the best Rx beam can achieve the best beam prediction accuracy, while requires Rx beam sweeping, which will increase the complexity of UE implementation.

  - Generalization

To investigate the generalization capability of AI model, we conducted research on generalization under various Set B conditions, scenario configurations, frequencies, ISD, and UE distributions.

    - Different Set B

During the training phase, the Set B includes 32 beam pairs. In the inference phase, we test our model with three different SetBs, which include 16, 32, and 64 beam pairs, respectively. Both fixed pattern and random pattern are considered in our simulation.

For the Set B including 16 beam pairs, it comprises of 4 selected DL Tx beams and 4 DL Rx beams. For selecting the 4 DL Tx beams, we use the fixed pattern as illustrated in Figure 6. For Set B including 32 beam pairs, it comprises of 8 selected DL Tx beams and 4 DL Rx beams. The fixed pattern we used is shown in Figure 1.

For the Set B including 64 beam pairs, it comprises of 16 selected DL Tx beams and 4 DL Rx beams. We use a fixed pattern as illustrated in Figure 7.

Figure 6: Fixed pattern 2 selection of DL TX beam

set

Figure 7: Fixed pattern 3 selection of DL TX beam

Scenario #1: During the training phase, the input Set B consists of 32 beam pairs shown in Figure 1. In the inference phase, we test our model with three different SetBs, which includes 16, 32, and 64 beam pairs, respectively. The accuracy-related KPIs are evaluated using a fixed pattern, where the training is conducted using fixed pattern 1 and the inference is conducted using fixed pattern 1, 2, and 3, respectively. The results are presented in Table 12. Additionally, we evaluate the accuracy-related KPIs using a random pattern of Opt C, where the input set B is randomly changed among the set A, and beam ID is implicitly used as the model input. The results are presented in Table 13.

Table 12: Evaluation results for BM-Case1 with different Set B of inference for Tx-Rx beam pair prediction with fixed pattern

| Training with Set B pattern | fixed pattern 1 |  |  |
| --- | --- | --- | --- |
| Inference with Set B pattern | fixed pattern2 | fixed pattern 1 | fixed pattern 3 |
| Top-1 | 27.61 | 95.63 | 84.14 |
| Top-3/1 | 45.50 | 99.93 | 97.10 |
| Average L1-RSRP diff | 12.4924 | 0.0323 | 4.3980 |

Table 13: Evaluation results for BM-Case1 with different Set B of inference for Tx-Rx beam pair prediction random pattern and beam ID

| Training with number of beam pairs in Set B | 32 |  |  |
| --- | --- | --- | --- |
| Inference with number of beam pairs in Set B | 16 | 32 | 64 |
| Top-1 | 45.90 | 70.04 | 77.09 |
| Top-3/1 | 76.95 | 89.22 | 93.59 |
| Average L1-RSRP diff | 4.2358 | 1.4057 | 0.1321 |

Scenario#2: We mixed the dataset from fixed pattern 1, 2, 3 together. For each pattern, we select 50000 samples for training and 10000 samples for inference. In the inference phase, we simulate the prediction performance of fixed pattern 1, 2, 3, respectively. The simulation results are shown in Table 14. We also investigate the generalization performance of mixed data with random pattern (Set B is randomly changed among Set A beams pairs), the simulation results are shown in Table 15.

Table 14: Evaluation results for BM-Case1 with mixed Set B for Tx-Rx beam pair prediction with fixed pattern

| Training with Set B pattern | Mixed pattern of fixed pattern1, fixed pattern 2, fixed pattern 3 |  |  |
| --- | --- | --- | --- |
| Inference with Set B pattern | fixed pattern2 | fixed pattern1 | fixed pattern3 |
| Top-1 | 85.60 | 94.19 | 94.69 |
| Top-3/1 | 97.71 | 99.77 | 99.90 |
| Average L1-RSRP diff | 1.1749 | 0.0835 | 0.0108 |

Table 15: Evaluation results for BM-Case1 with mixed Set B for Tx-Rx beam pair prediction with random pattern and beam ID

| Training with number of beam pairs in Set B | 16,32,64 |  |  |
| --- | --- | --- | --- |
| Inference with number of beam pairs in Set B | 16 | 32 | 64 |
| Top-1 | 78.98 | 79.02 | 79.14 |
| Top-3/1 | 90.25 | 92.58 | 93.01 |
| Average L1-RSRP diff | 1.0579 | 1.0683 | 1.0652 |

Observation 10: Significantly performance degradation is observed, when AI/ML model is trained and inferenced with different Set B, e.g., the number of Set B of training is 32 and inference is 16.

Observation 11: Training with mixed Set B of different sizes can improve generalization performance.

    - Different deployment scenarios

For our research on generalization, we have conducted additional studies on different scenario configurations. Specifically, we utilized various scenario configurations during the training and inference phases to examine the generalization of our model.

Scenario #3: For beam pair prediction, we use beam pairs in the Uma scenario to generate the training dataset, and we tested the model's performance using the dataset in the Uma and Umi scenario, respectively. Additionally, we trained the model using a mixed dataset from Umi and Uma scenario, where the training data consisted of 75K samples for each scenario. The simulation results using fixed pattern 1 and random pattern can be found in Table 16 and Table 17, respectively.

Scenario #4: For Tx beam prediction, the deployment scenarios used for training and inference are the same as Scenario #3. The simulation results using fixed pattern 1 and random pattern can be found in Table 18 and Table 19, respectively.

Table 16: Evaluation results for BM-Case1 with different scenario for Tx-Rx beam pair prediction with fixed pattern

| Training | Uma |  | Umi |  | Umi&Uma |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inference | Uma | Umi | Uma | Umi | Uma | Umi |
| Top-1 | 95.60 | 94.91 | 96.57 | 96.98 | 96.62 | 96.49 |
| Top-3/1 | 98.77 | 96.38 | 95.99 | 98.39 | 97.98 | 98.54 |
| Average L1-RSRP diff | 0.0369 | 0.0202 | 0.0358 | 0.0204 | 0.0624 | 0.0694 |

Table 17: Evaluation results for BM-Case1 with different scenario for Tx-Rx beam pair prediction with random pattern with beam ID

| Training | Uma |  | Umi |  | Umi&Uma |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inference | Uma | Umi | Uma | Umi | Uma | Umi |
| Top-1 | 79.60 | 74.19 | 72.11 | 78.99 | 78.81 | 78.62 |
| Top-3/1 | 96.22 | 95.10 | 96.11 | 97.45 | 98.77 | 97.96 |
| Average L1-RSRP diff | 0.2568 | 1.0021 | 1.0611 | 0.5041 | 0.0695 | 0.0599 |

Table 18: Evaluation results for BM-Case1 with different scenario for Tx beam prediction with fixed pattern

| Training | Uma |  | Umi |  | Umi&Uma ） |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inference | Uma | Umi | Uma | Umi | Uma | Umi |
| Top-1 | 95.96 | 94.28 | 93.36 | 95.22 | 94.95 | 94.68 |
| Top-3/1 | 99.89 | 95.99 | 99.64 | 98.67 | 98.51 | 99.63 |
| Average L1-RSRP diff | 0.0225 | 0.0442 | 0.0859 | 0.0358 | 0.0562 | 0.0419 |

Table 19: Evaluation results for BM-Case1 with different scenario for Tx beam prediction with random pattern with beam ID

| Training | Uma |  | Umi |  | Umi&Uma ） |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inference | Uma | Umi | Uma | Umi | Uma | Umi |
| Top-1 | 54.21 | 51.93 | 54.29 | 53.54 | 55.14 | 53.29 |
| Top-3/1 | 72.21 | 72.00 | 73.36 | 76.02 | 75.01 | 74.21 |
| Average L1-RSRP diff | 4.0623 | 3.8288 | 3.5145 | 3.3595 | 3.5646 | 3.1464 |

Observation 12: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different scenario configurations, e.g., the model trained by data samples in Uma and tested by data samples in Umi.

    - Different UE distributions

We evaluated the generalization of AI/ML models across scenarios with varying distributions of indoor and outdoor UE, including 0%, 50%, and 80% indoor UEs.

In Scenario #5: For beam pair prediction, we trained three models with 0%, 50%, 80% indoor UE proportion, respectively, and we tested three configurations (0%, 50%, and 80% indoor UEs) for each model. The simulation results of using fixed pattern 1 and random pattern are shown in Table 20 and Table 21, respectively.

In Scenario #6: For Tx beam prediction, the UE distributions used for training and inference are the same as Scenario#5. The simulation results can be found in Table 22 and Table 23.

Table 20 Evaluation results for BM-Case1 with different UE distributions for Tx-Rx beam pair prediction with fixed pattern

| Training | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Top-1 | 94.45 | 94.35 | 93.74 | 94.32 | 94.38 | 93.75 | 93.70 | 93.53 | 93.87 |
| Top-3/1 | 99.89 | 99.86 | 99.80 | 99.89 | 99.84 | 99.79 | 99.82 | 99.81 | 99.81 |
| Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 |

Table 21: Evaluation results for BM-Case1 with different UE distributions for Tx-Rx beam pair prediction with random pattern with beam ID

| Training | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Top-1 | 67.12 | 66.80 | 66.77 | 65.01 | 65.15 | 65.01 | 57.68 | 58.04 | 57.91 |
| Top-3/1 | 94.46 | 94.64 | 94.62 | 93.82 | 93.76 | 93.77 | 87.94 | 88.11 | 87.96 |
| Average L1-RSRP diff | 1.0876 | 1.0588 | 1.0742 | 1.2980 | 1.3050 | 1.3132 | 2.7632 | 2.7284 | 2.7445 |

Table 22: Evaluation results for BM-Case1 with different UE distributions for Tx beam predictions with fixed pattern

| Training | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Top-1 | 98.01 | 98.00 | 98.07 | 97.95 | 98.00 | 98.17 | 97.96 | 97.89 | 98.19 |
| Top-3/1 | 99.99 | 99.99 | 99.99 | 99.99 | 99.99 | 99.99 | 99.99 | 99.99 | 99.99 |
| Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 |

Table 23: Evaluation results for BM-Case1 with different UE distributions for Tx beam predictions with random pattern with beam ID

| Training | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inference | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Top-1 | 71.66 | 71.79 | 71.95 | 72.25 | 71.87 | 71.81 | 58.28 | 57.90 | 57.95 |
| Top-3/1 | 92.83 | 92.74 | 92.87 | 92.84 | 92.71 | 92.88 | 86.06 | 85.93 | 85.92 |
| Average L1-RSRP diff | 0.0558 | 0.0569 | 0.0558 | 0.0602 | 0.0589 | 0.0604 | 3.4826 | 3.5442 | 3.5321 |

Observation 13: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different UE distributions, e.g., the model trained by data samples with 0% indoor UE distribution and tested by data samples with 50% indoor UE distribution.

    - Different carrier frequency

We evaluated the generalization under different carrier frequency models, including 20G and 30G.

Scenario#7: For beam pair prediction, we trained two models from the data samples at 20GHz and 30GHz carrier frequency, respectively. We tested each model with data samples at 20GHz and 30GHz. The simulation results using fixed pattern 1 and random pattern are shown in Table 24 and Table 25, respectively.

Scenario#8: For Tx beam prediction, the carrier frequency used for training and inference are the same as Scenario#7. The simulation results are shown in Table 26 and Table 27, respectively.

Table 24: Evaluation results for BM-Case1 with different carrier frequency for Tx-Rx beam pair predictions with fixed pattern

| Training | 20G |  | 30G |  |
| --- | --- | --- | --- | --- |
| Inference | 20G | 30G | 20G | 30G |
| Top-1 | 92.20 | 91.59 | 90.26 | 95.62 |
| Top-3/1 | 99.62 | 98.99 | 97.82 | 98.63 |
| Average L1-RSRP diff | 0.4590 | 0.0358 | 0.0182 | 0.0201 |

Table 25: Evaluation results for BM-Case1 with different carrier frequency for Tx-Rx beam pair predictions with random pattern with beam ID

| Training | 20G |  | 30G |  |
| --- | --- | --- | --- | --- |
| Inference | 20G | 30G | 20G | 30G |
| Top-1 | 64.25 | 64.30 | 58.34 | 59.81 |
| Top-3/1 | 93.35 | 93.21 | 87.56 | 89.16 |
| Average L1-RSRP diff | 0.0239 | 0.0231 | 0.0870 | 0.0752 |

Table 26: Evaluation results for BM-Case1 with different carrier frequency for Tx beam predictions with fixed pattern

| Training | 20G |  | 30G |  |
| --- | --- | --- | --- | --- |
| Inference | 20G | 30G | 20G | 30G |
| Top-1 | 93.35 | 93.01 | 90.63 | 94.06 |
| Top-3/1 | 99.97 | 97.10 | 97.95 | 99.93 |
| Average L1-RSRP diff | 0.0529 | 0.0326 | 0.1801 | 0.0579 |

Table 27：Evaluation results for BM-Case1 with different carrier frequency for Tx beam predictions with random pattern with beam ID

| Training | 20G |  | 30G |  |
| --- | --- | --- | --- | --- |
| Inference | 20G | 30G | 20G | 30G |
| Top-1 | 56.58 | 53.75 | 51.76 | 58.57 |
| Top-3/1 | 87.26 | 85.29 | 87.52 | 87.60 |
| Average L1-RSRP diff | 0.0460 | 0.1240 | 0.0237 | 0.0586 |

Observation 14: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different carrier frequency, e.g., the model trained by data samples of carrier frequency 20G and tested by data samples of carrier frequency 30G.

    - Different ISD

In the previous simulations, the ISD was set as 200 meters. To investigate the impact of different ISD values on model generalization, we conducted simulations using ISD values of 200 meters and 500 meters.

Scenario #9: For beam pair prediction, we trained two models with data samples in ISD of 200 meters and 500 meters, respectively. We tested each model with data samples in ISD of 200 meters and 500 meters. The simulation results using fixed pattern 1 and random pattern are shown in Table 28 and Table 29.

Scenario #10: For Tx beam prediction, the ISD used for training and inference are the same as Scenario#9. The simulation results are shown in Table 30 and Table 31, respectively.

Table 28: Evaluation results for BM-Case1 with different ISD for Tx-Rx beam pair predictions with fixed pattern

| Training | 200 |  | 500 |  |
| --- | --- | --- | --- | --- |
| Inference | 200 | 500 | 200 | 500 |
| Top-1 | 91.47 | 90.22 | 90.35 | 91.98 |
| Top-3/1 | 99.69 | 99.46 | 99.39 | 99.73 |
| Average L1-RSRP diff | 0.0003 | 0.0005 | 0.0005 | 0.0003 |

Table 29: Evaluation results for BM-Case1 with different ISD for Tx-Rx beam pair predictions with random pattern with beam ID

| Training | 200 |  | 500 |  |
| --- | --- | --- | --- | --- |
| Inference | 200 | 500 | 200 | 500 |
| Top-1 | 61.35 | 62.19 | 60.29 | 61.94 |
| Top-3/1 | 87.91 | 90.01 | 89.54 | 91.65 |
| Average L1-RSRP diff | 0.0468 | 0.0502 | 0.0479 | 0.0391 |

Table 30: Evaluation results for BM-Case1 with different ISD for Tx beam predictions with fixed pattern

| Training | 200 |  | 500 |  |
| --- | --- | --- | --- | --- |
| Inference | 200 | 500 | 200 | 500 |
| Top-1 | 98.14 | 98.10 | 98.06 | 98.02 |
| Top-3/1 | 99.99 | 99.98 | 99.98 | 99.98 |
| Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 |

Table 31: Evaluation results for BM-Case1 with different ISD for Tx beam predictions with random pattern with beam ID

| Training | 200 |  | 500 |  |
| --- | --- | --- | --- | --- |
| Inference | 200 | 500 | 200 | 500 |
| Top-1 | 71.09 | 70.86 | 70.63 | 70.95 |
| Top-3/1 | 93.02 | 92.01 | 92.11 | 93.00 |
| Average L1-RSRP diff | 0.0439 | 0.0573 | 0.1287 | 0.0582 |

Observation 15: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different ISDs, e.g., the model trained by data samples in ISD 200 and tested by data samples in ISD 500.

  - Temporal beam prediction of BM-Case2

In this section, we will assess the performance of the combined time domain and spatial domain beam prediction method. For the Set B pattern, we use the variable Set B pattern of Opt C in our simulations. We use the recent 8 RSRP measurements with a periodicity of 40ms in observation window T1, the total observation window T1 is 320ms. Based on these 8 RSRP measurements, we predict window T2 with one future time instants, four future time instants, and eight future time instants, respectively. Namely, predicted the beam pairs for future 40ms, 160ms, and 320ms. In addition, for each time instance of T1, we select 8 beam pairs out of 256 beam pairs, while predicting the beam pairs from all 256 beam pairs in T2, i.e., Set B is a subset of Set A. We present the simulation results in Table 32.

Table 32 performance of beam pair prediction in temporal domain (T2=1, 4, 8)

| Scenario | Top-1 accuracy | Top-3 Accuracy | Ave. RSRP diff. |
| --- | --- | --- | --- |
| 8 predict 1 | 71.99 | 87.58 | 3.05 |
| 8 predict 4 | 69.71 | 86.13 | 3.50 |
| 8 predict 8 | 65.60 | 82.33 | 4.12 |

Observation 16: For BM-Case2, the top-1 prediction accuracy is greater than 65%, and the top3/1 prediction accuracy is greater than 80% when predicting the beam pairs for future 1, 4, and 8 time instants.

- Conclusions

In this contribution, the simulation results for Alt.2 of BM-Case1 and BM-Case2 are given and discussed. The observations and proposals are summarized as follows:

Observation 1：In Tx-Rx beam pair prediction with fixed Set B pattern, the prediction accuracy of top-3/1 can be greater than 99%.

Observation 2：In Tx-Rx beam pair prediction with variable Set B pattern of Opt C, the prediction accuracy of top-3/1 can be greater than 85%.

Observation 3：In Tx-Rx beam pair prediction with variable Set B pattern of Opt C, the prediction accuracy has decreased compared to fixed Set B pattern.

Observation 4：For models taking RSRP as output, the prediction L1-RSRP difference can reflect the accuracy of Top-N beam prediction.

Observation 5: For Beam pair prediction with variable Set B pattern of Opt B, the Top-3 accuracy can be greater than 95%.

Observation 6: For BM-Case1, variable pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns has better performance than Opt C.

Observation 7：For DL Tx beam prediction with variable Set B pattern of Opt C, the prediction performance of Rx beam option 1, i.e., the “best” Rx beam, is better than that of option 2b, i.e., one specific Rx beam.

Observation 8：For DL Tx beam prediction, the prediction accuracy is greatly degraded when training and inference using different pre-configured patterns.

Observation 9: Selecting the best Rx beam can achieve the best beam prediction accuracy, while requires Rx beam sweeping, which will increase the complexity of UE implementation.

Observation 10: Significantly performance degradation is observed, when AI/ML model is trained and inferenced with different Set B, e.g., the number of Set B of training is 32 and inference is 16.

Observation 11: Training with mixed Set B of different sizes can improve generalization performance.

Observation 12: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different scenario configurations, e.g., the model trained by data samples in Uma and tested by data samples in Umi.

Observation 13: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different UE distributions, e.g., the model trained by data samples with 0% indoor UE distribution and tested by data samples with 50% indoor UE distribution.

Observation 14: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different carrier frequency, e.g., the model trained by data samples of carrier frequency 20G and tested by data samples of carrier frequency 30G.

Observation 15: Generalization performance is not sensitive, when AI/ML model is trained and inferenced with different ISDs, e.g., the model trained by data samples in ISD 200 and tested by data samples in ISD 500.

Observation 16: For BM-Case2, the top-1 prediction accuracy is greater than 65%, and the top3/1 prediction accuracy is greater than 80% when predicting the beam pairs for future 1, 4, and 8 time instants.

Proposal 1: For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure:

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 Tx-Rx beam pair can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

    - the Rx beam of the Top-1 Tx-Rx beam pair can be recommended by gNB for DL data reception

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams of Top-K Tx-Rx beam pairs can be recommended by UE for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Top-K Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - the Rx beam corresponding with the selected DL Tx beam can be recommended by gNB

    - P3 procedure is not needed

- Other options are not precluded.

- References

- Report of RAN1#112meeting, Chair, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2208969, Report of RAN1#110bis-e , Online, October 10th – 19th, 2022.

- Annex

Table 33: Simulation assumptions for Dense Urban scenario for Alt.2 of BM-Case1

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (19 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 10 UEs per sector/cell for full buffer traffic 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| BF scheme | DFT codebook |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |