3GPP TSG RAN WG1 #112bis-e                                         R1-2302698

e-Meeting, April 17th – April 26th, 2023

Source: 	CATT

Title:	Discussion on AI/ML-based beam management

Agenda Item:	9.2.3.2

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting, the following agreements were made for other aspects on AI/ML for beam management [1]:

Conclusion For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized. Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered Conclusion Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused). Conclusion Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indictation/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

In RAN#98 meeting, the sub use cases for AI/ML based beam management were confirmed as following:

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

In this contribution, we will focus on the potential spec impacts for above BM-Case1 and BM-Case2.

- Discussion

  - Scope clarification

For the type of beam prediction, one debating point of previous meetings is whether the DL beam pair prediction is feasible since it may require the gNB and UE to disclose the beam information.

For NW-side model, it is not practical to use a model for each UE due to large model training and storage overhead, so that the AI/ML model would be used to predict the best beam pairs for different UEs. Since the Rx beam patterns are implementation dependent, different UEs can have different Rx beam patterns. The model generalization capability of different Rx beam patterns should be considered when training the model. Different Rx beam patterns will result in different beam pair patterns. Hence, it can be viewed as the Set B pattern is variable. According to the evaluation results in our contribution [2], if Tx beam ID and Rx beam ID are implicitly used as additional model input, the AI/ML model can provide good performance for the case on random pattern of Set B. The key issue is how to map correct Tx beam ID and Rx beam ID as model input when considering various Rx beam patterns of different UEs. For Tx beam ID, the NW can generate a mapping between SSB/CSI-RS and Tx beam IDs, when UE reports the L1-RSRP and the RS indicator, the corresponding Tx beam ID can be obtained by the NW. For Rx beam ID, if the UE reports Rx beam related information (e.g., relative Rx beam info), the NW can map the reported Rx beams to correct Rx beam ID. After model inference, the NW can map the predicted best Rx beam ID to predicted relative Rx beam info and send the predicted relative Rx beam info to UE for providing the spatial Rx parameters of DL reception. The procedure for beam pair prediction of NW-side model is shown in Figure 1.

Figure 1: Procedure for beam pair prediction of NW-side model

In RAN1#112 meeting, there is no consensus on directly reporting UE Rx beam shape/direction. To facilitate the NW mapping with correct Rx beam IDs as model input, the contents of relative Rx beam information with preserving sensitive proprietary information need to be studied.

For UE-side model, the AI/ML model can be trained by NW and transferred/delivered to UE, or trained by UE side. If the model is trained by NW, the model can be a cell-specific model. During model training, UEs can report the L1-RSRP and relative Rx beam info to NW. NW could generate a mapping between SSB/CSI-RS and Tx beam ID, and a mapping between relative Rx beam info and Rx beam ID Then NW can use L1-RSRP with corresponding Tx and Rx beam ID to train the AI/ML model. For model inference at UE side, both the mapping between SSB/CSI-RS and Tx beam ID, and the mapping between relative Rx beam info and Rx beam ID used by the NW for model training should be known by the UEs For Tx beam ID, the gNB and UE can define a mapping rule between RS ID and Tx beam ID to align the understanding between the NW and UE, and the exact Tx beam corresponding to the Tx beam ID is only known by the NW. For Rx beam ID mapping, how to align the Rx mapping rules between gNB and UEs should be further studied. The procedure for beam pair prediction of UE-side model trained by NW is shown in Figure 2.

Figure 2: Procedure for beam pair prediction of UE-side model trained by NW

For the model trained by UE side, the corresponding Tx beams of the training data set may be from various gNBs, and the number and patterns of Tx beams from various gNBs may be different. To facilitate the UE map the correct Tx beam ID as model input, the gNB can send relative Tx beam information to UE. The contents of the relative Tx beam information should be further studied. The procedure for beam pair prediction of UE-side model trained by UE side is shown in Figure 3.

Figure 3: Procedure for beam pair prediction of UE-side model trained by UE-side

In the discussion of last meeting, some companies thought that for UE-side model, there is no difference between Tx beam prediction and beam pair prediction from the specification perspective. In our opinion, at least the CSI-RS/SSB configuration may be different for Tx beam prediction and beam pair prediction. For beam pair prediction, the CSI-RS/SSB will be configured to support the Rx beam sweeping. For Tx beam prediction, Rx beam sweeping is not needed if the Tx beam prediction is for a specific Rx beam.

Based on the above analysis, we propose the following proposals.

Proposal 1: For DL beam pair prediction with a NW-side model, study how to report/send relative Rx beam information when preserving sensitive proprietary information.

Proposal 2: For DL beam pair prediction with a UE-side model, study the following aspects:

- For model training at NW side, study how to report relative Rx beam information when preserving sensitive proprietary information;

- For model training at UE side, study how to send/report relative Tx beam information when preserving sensitive proprietary information.

Proposal 3: For DL beam pair prediction with model training at NW side and inference at UE side, study how to align the mapping rule for Tx beam ID and the mapping rule for Rx beam ID between the NW and UE.

  - Spec impacts

In this section, we will discuss the details of spec impacts on AI/ML-based beam management.

    - Model inputs

For the sub use case BM-Case1 and BM-Case2, the AI/ML inputs were discussed during RAN1#110 meeting but no consensus was concluded. In RAN1#109-e meeting, the following conclusion for the AI/ML inputs was achieved [3].

Conclusion: Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose. Conclusion Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance): - Alt.1: Only L1-RSRP measurement based on Set B - Alt 2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

In our simulation, we simulate several alternatives and compare their performance [2]. Based on the simulation results, the input of L1-RSRP measurement based on Set B with fixed pattern has the best performance. For Set B with variable pattern, if beam (pair) ID is implicitly used as model input, the model performance is also satisfactory. The key and difficulty is how to take the correct beam (pair) ID as model input. Based on our discussion in Section 2.1, some assistance information (e.g., relative Tx/Rx beam info) can be used to map the Tx/Rx beam ID. Whether it is beneficial to use the assistance information (e.g., relative Tx/Rx beam info) as model input can be discussed and evaluated in AI 9.2.3.1.

Regarding using CIR based on Set B as AI/ML inputs, if AI/ML training or inference is at NW side, it will occupy more UL feedback resources than using L1-RSRP measurement without significant performance improvement.

Proposal 4: For the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B;

- Alt.2: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID.

Proposal 5: Whether it is beneficial to take the assistance information as model input can be evaluated in AI 9.2.3.1.

    - Data collection

Data collection is related with model training/inference/monitoring. In the following discussion, we will focus on the data collection for model training. The data collection for model inference will be discussed in Section 2.2.4 and the data collection for model monitoring will be discussed in Section 2.2.5.

In RAN1#109-e meeting, the following conclusion for the relation between Set B and Set A was achieved for BM-Case1 [3].

- Alt.1: Set B is a subset of Set A

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

For Alt.1, gNB only needs to transmit RS in Set A and UE can get the measurement results of Set A and Set B. The gNB can indicate the beam pattern of Set B in Set A to UE, or the UE can suggest the beam pattern of Set B according to the measurement results. For Alt.2, gNB needs to transmit RS in both Set A and Set B.

In RAN1#110bis [4], the following agreement was reached for the selection of Set B for Alt.1 and Alt.2.

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.

For Option 1, gNB sends the RS corresponding to the fixed Set B pattern. If the model is trained at NW side, UE needs to report the L1-RSRPs of Set B optionally with the corresponding RS/beam indicators. For Option 2, the Set B pattern is variable. For Opt A, gNB sends the RS corresponding to the pre-configured patterns in order. For Opt B, the Set B pattern is randomly changed among pre-configured patterns and Set A, respectively. For Opt B, the Set B pattern can be selected by gNB and UE. For example, for UE selection, gNB can configure the measurement RS of several pre-configured Set B patterns to UE. UE will measure the L1-RSRPs of each Set B pattern, and select the L1-RSRPs of the pattern with highest quality (e.g., highest average L1-RSRP) to report. In that case, the Set B pattern information should also be reported.

Regarding the label of model outputs, it may depend on the AI algorithm and model functionality. For example, if supervised learning is used, the following two options can be used as the labels for model training.

- Opt.1: Top-K beam (pair) IDs

- Opt.2: L1-RSRPs of Set A

For different combinations of relations of Set B and Set A with model functionality, the reporting contents are summarized in Table I.

Table I: Reporting contents for different combinations of NW-side model training

|  | Opt.1 | Opt.2 |
| --- | --- | --- |
| Alt.1 | L1-RSRPs of Set B [+RS/beam (pair) indicators of Set B]] +Top-K beam (pair) IDs + [pattern info] | L1-RSRPs of Set A [+RS/beam (pair) indicators of Set A] + [pattern info] |
| Alt.2 |  | L1-RSRPs of Set B [+RS/beam (pair) indicators of Set B] + L1-RSRPs of Set A [+RS/beam (pair) indicators of Set A] + [pattern info] |

Based on the reporting contents in Table I, we propose the following proposal.

Proposal 6: Regarding the training data collection for AI/ML model training at NW side, study the following options as a starting point for the contents of collected data:

- Opt.1: UE sends M1 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- Opt.2: UE sends M2 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, sends M3 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to another beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- Opt.3: UE sends M4 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, sends M5 RS/beam (pair) indicators based on the measurement corresponding to another beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- FFS: the details of beam pattern information.

For model training at UE side, the UE can use the measurement results as model inputs and labels for model training. The details of model inputs and labels for model training at UE sided can be left into UE implementation. In RAN1#111 [5], the following agreement was achieved for model training at UE side.

Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded

To initiate the data collection for model training, the UE would send the indication/request signaling to the network. The Tx beam pattern of Set A can be determined by the gNB since it is related to the antenna/beam configuration of the gNB. For the Tx beam pattern of Set B, it can be determined by gNB and signaled to UE, or UE can determine Set B from Set A. For beam pair prediction, the Rx beam pattern of Set B and Set A can be determined by UE. The configuration of RS should be aligned with Rx beam sweeping assumption of Set B and Set A to support the beam measurement of Set B and Set A. Therefore, indication/request from UE to gNB for RS transmission should be studied.

Besides, the preferred size of training dataset and minimum periodicity of the RS can be indicated from the UE to network. The UE can indicate a preferred dataset size when considering both training complexity and model performance. The minimum periodicity of RS is related to the UE speed. For example, if the minimum RS periodicity for low-speed UEs is too small, the UE may get the same L1-RSRP results from several subsequent measurements.

Proposal 7: Regarding the training data collection for AI/ML model training at UE side, at least study the specification impact on the following aspects:

- Indication/request from UE to gNB for RS transmission to be aligned with Rx beam sweeping assumption;

- Indication of the preferred size of training dataset ;

- Indication of the minimum periodicity of the RS transmission.

    - Model/functionality identification

Model identification or functionality identification is a process/method of identifying an AI/ML model/functionality for the common understanding between the NW and the UE. In our companion contribution [6], we discuss the information can be provided by t model/functionality identification. At least for collaboration Level y, gNB doesn’t know the exact model in UE side, if the model deployed at UE is proprietary. Thus, sufficient information of the AI/ML model shall also be provided to enable LCM in 3GPP network.

Regarding to the model identification, the identification information can be the information of model functionality, model inputs/nominal inputs and outputs/nominal outputs, assistance information for inference, applicable conditions, and other information. The information of model functionality can be BM-Case1 and BM-Case2, or DL beam pair prediction and DL Tx beam predication. The information of model inputs can be the number and pattern(s) of DL Tx beams or beam pairs in Set B. gNB can use the model functionality and information of model inputs to decide how to transmit the RS in DL Tx beam of Set B. The information of model outputs can be the number of predicted beam, e.g., N predicted DL Tx beams or beam pairs and/or L1-RSRP. Based on the information of model outputs, gNB can decide the feedback UL resources for the number of N best beam ID and/or L1-RSRP. The assistance information for inference can be the relative beam information while preserving sensitive proprietary information. The information of applicable conditions can enable model monitoring based on applicable conditions, and can be used for model switching/selection/fallback. The information on model performance, which may be hypothetic or predicted, can help the network to decide whether to active the model. For other information, UE may also need to report to gNB when considering whether the AI/ML model can provide co-existence or concurrent use with other AI/ML models and/or non-AI/ML features [6].

Proposal 8: Regarding the model identification of BM-Case1 and BM-Case2, study the following aspects as a starting point for identification information which UE should provide to gNB:

- Information on model functionality, e.g., BM-Case1/BM-Case2 or DL beam pair/Tx beam prediction;

- Information of model inputs/nominal inputs, e.g., the number and pattern(s) of DL Tx beams or beam pairs in Set B;

- Information of model outputs/nominal outputs, e.g., the number of predicted beam and/or L1-RSRP;

- Information on assistance information for inference, e.g., the relative beam information;

- Information on model performance;

- Information on concurrent use with other AI/ML models and/or non-AI/ML features;

- Information on applicable conditions.

Regarding to the functionality identification, the UE also needs to provide the identification information to gNB, and it was agreed to reuse 3GPP UE capability report as the starting point. For a given sub-use-case, one or more functionalities may be provided by the UE, and UE can use more than one model to realize a functionality. How to distinguish different functionalities is not clear so far. One possible way is to use large granularity characteristics such as input type and/or output type, allowing network have correct understanding on ‘what is required’ and ‘what can be provided’ [6]. For example, we have two models, i.e., model 1 and model 2. Model 1 is for beam pair prediction and model 2 is for DL Tx beam prediction on a specific Rx beam. For model 1 inference, UE needs to sweep the Rx beams to measure the L1-RSRP of Set B. But for model 2 inference, the Rx beam sweeping is not needed for Set B measurement. Obviously, it requires different RS configurations at the gNB side, hence model 1 and model 2 cannot be within a functionality. For another example, we have model 3 and model 4, which are both used for Tx beam prediction. The output of model 3 is Top-K beam ID while the output of model 4 is the predicted L1-RSRP. For model 3, UE could provide the Top-K beam ID to gNB, and for model 4, UE could provide the Top-K beam ID and corresponding predicted L1-RSRPs to gNB, which requires different report quantities and resources configuration. Hence, it is not proper to group model 3 and model 4 into one functionality.

Proposal 9: Regarding the functionality identification of BM-Case1 and BM-Case2, study distinguishing different functionalities by large granularity characteristics, e.g. input type and/or output type.

    - Model inference

For inputs of model inference, gNB only needs to transmit RS with beams in Set B. UE can measure the received RS in Set B and get L1-RSRP as model inputs. If the inference is performed at UE side, the UE gets the measurement results (e.g., L1-RSRP) of Set B optionally with the corresponding RS indicators as model inputs, which can be left to UE implementation. If the inference is performed at gNB side, the UE needs to report measurement results (e.g., L1-RSRP) of Set B optionally with the corresponding RS indicators to gNB as model inputs. Thus UE may need to report measurement results of more than 4 beams optionally with the corresponding RS indicators, which can use L1 signaling or MAC CE.

Proposal 10: Regarding the data collection for AI/ML model inference at NW side, UE needs to report the measurement results of more than 4 beams optionally with the corresponding RS indicators to gNB, which can use L1 signaling or MAC CE.

For NW-sided model, to determine and indicate the best beam (pair) from the predicted Top-K beam (pairs), the following two options can be considered.

Opt.1: the gNB sweeps over the predicted Top-K beam (pairs) and UE measures the L1-RSRP of the Top-K beam (pairs). Then, the legacy beam reporting and TCI indication mechanism can be used.

Opt.2: the gNB directly determines the best beam (pair) from the predicted Top-K beam (pairs) and indicate the corresponding spatial Rx parameter to UE.

For Opt.1, sweeping over the predicted Top-K beam (pairs) can cause additional latency and RS transmission overhead. For Opt.2, the spatial Rx parameter is like QCL-Type D of legacy TCI state. How to indicate the spatial Rx parameter to UE should be studied. Besides, in legacy TCI indication, the QCL-Type D info cannot be individually indicated. For example, for the DM-RS of PDSCH/PDCCH, the TCI state provides only QCL-type A or QCL-type A with QCL-type D. Since UE may not measure the best beam (pair) before, the UE may not obtain the information of QCL-type A. Whether it is feasible to only provide spatial Rx parameter needs to be further discussed. For BM-Case2, since the predicted beams are for the multiple future time instances, how to indicate the beam for multiple future time instances can also be studied.

Proposal 11: For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following options of beam indication mechanism with potential down-selection:

- Opt1: reusing legacy TCI indication mechanism (e.g., Rel-15/16 TCI framework and Rel-17 unified TCI framework);

- Opt2: only indicate spatial Rx parameter to UE.

Proposal 12: For BM-Case2 with a network-side AI/ML model, study how to indicate the beam for multiple future time instances.

If the model inference is performed at UE side, the UE measures the received RS in Set B and predicts Top-K beam (pairs) and/or the L1-RSRP of Set A. It was agreed in RAN1#112 [2] to further study the reporting of predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s). One open issue is whether/how to differentiate predicted L1-RSRP and measured L1-RSRP. In our opinion, it is not needed to differentiate the predicted L1-RSRP and measured L1-RSRP. Both the predicted L1-RSRP and measured L1-RSRP are used for best beam (pair) determination, if there is large error of the predicted L1-RSRP, the performance degradation will be realized by model monitoring procedure.

Proposal 13: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when reporting the predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s), there is no need to differentiate predicted L1-RSRP and measured L1-RSRP.

    - Model monitoring

The propagation environment in the system may change due to various factors, e.g. moving of UE and new obstacles. Due to the large change of propagation environment, the performance of AI/ML based beam management may deteriorate dramatically. In order to avoid long time performance degradation, AI/ML model performance monitoring is needed, and some actions should be taken when the AI/ML model becomes invalid.

For AI/ML based beam management with a UE-side model, it was agreed in RAN1#110bis to study three alternatives for model monitoring, i.e., UE-side model monitoring, NW-side model monitoring, and hybrid model monitoring. For UE-side model monitoring, UE needs to calculate the KPIs and make decision of model selection/activation/ deactivation/switching/fallback operation. The RS configuration/beam reporting/ beam indication for to be activated AI/ML models and for legacy beam management may be different from that of to be deactivated models, hence UE should report the decision of model activation/ deactivation/switching/fallback to the network, and the decision shall be applied after UE receiving the acknowledgement from the network.

Proposal 14: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Alt.1 UE-side model monitoring, study the potential specification impacts on the following aspects:

- Reporting the decision of model activation/ deactivation/switching/fallback to the network;

- Acknowledgement mechanism of model activation/ deactivation/switching/fallback from the network.

Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1and BM-Case2, the following four alternatives have been agreed in RAN1#112 to study with potential down-selection.

•	Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

•	Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

•	Alt.3: Performance metric based on input/output data distribution of AI/ML

•	Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

Alt.1 and Alt.4 are the intermediate KPIs of the AI/ML model. The intermediate KPIs is more straightforward than Alt.2 link quality related KPIs, since link quality KPIs may be influenced by other factors. For Alt.3, the specific metrics and the model monitoring performance should be studied in AI 9.2.3.1 to evaluate whether it is valid to take Alt.3 for model monitoring.

For Alt.1, how to calculate the KPIs can be further studied. For example, the beam prediction accuracy KPIs can be calculated between the model outputs and best beam(s) of Set A. To get the best beam(s) of Set A, gNB still needs to transmit RS with beams in Set A. Thus, the power consumption and RS resources can NOT be saved compared with R16/R17 legacy beam management procedure. To save the power consumption and RS resources, it’s better to study how to calculate the intermediate KPIs only based on Set B, so that gNB doesn’t need to transmit RS in Set A at all, for example, UE measures multiple Set Bs to constitute Set A or nearly constitute Set A, so that UE can get the best beam(s) of Set A based on the measurement of multiple Set Bs.

Proposal 15: Regarding the model monitoring for BM-Case1 and BM-Case2, study the following aspects for the intermediate KPI calculation:

- How to calculate the intermediate KPIs based on both of Set A and Set B;

- How to calculate the intermediate KPIs only based on Set B.

If the performance of AI/ML based beam management is degraded, the following procedures should be studied, e.g., updating the AI/ML model, switching the AI/ML model to another one, and fall back to legacy beam management. Some signaling exchange between the gNB side and UE side may be needed. Besides, the model switch/fallback events should be defined to ensure the robustness. For example, the beam failure event is defined as the number of beam failure instance within a timer is greater than a threshold. A similar event can be defined for model update/switching/fallback to consider both robustness and latency.

Proposal 16: Regarding the model monitoring for BM-Case1 and BM-Case2, study the specification impacts on the following aspects:

- Model update/switching/fallback procedures based on model monitoring results, including the signaling exchange between the gNB and UE;

- Trigger condition for model update/switching/fallback.

- Conclusion

In this contribution, the related spec impacts for AI/ML based beam management are discussed. The proposals are summarized as follows:

Proposal 1: For DL beam pair prediction with a NW-side model, study how to report/send relative Rx beam information when preserving sensitive proprietary information.

Proposal 2: For DL beam pair prediction with a UE-side model, study the following aspects:

- For model training at NW side, study how to report relative Rx beam information when preserving sensitive proprietary information;

- For model training at UE side, study how to send/report relative Tx beam information when preserving sensitive proprietary information.

Proposal 3: For DL beam pair prediction with model training at NW side and inference at UE side, study how to align the mapping rule for Tx beam ID and the mapping rule for Rx beam ID between the NW and UE.

Proposal 4: For the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B;

- Alt.2: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID.

Proposal 5: Whether it is beneficial to take the assistance information as model input can be evaluated in AI 9.2.3.1.

Proposal 6: Regarding the training data collection for AI/ML model training at NW side, study the following options as a starting point for the contents of collected data:

- Opt.1: UE sends M1 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- Opt.2: UE sends M2 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, sends M3 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to another beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- Opt.3: UE sends M4 L1-RSRPs optionally with the RS/beam (pair) indicators based on the measurement corresponding to a beam set, sends M5 RS/beam (pair) indicators based on the measurement corresponding to another beam set, and optionally sends the beam pattern information (e.g., the Set B beam pattern);

- FFS: the details of beam pattern information.

Proposal 7: Regarding the training data collection for AI/ML model training at UE side, at least study the specification impact on the following aspects:

- Indication/request from UE to gNB for RS transmission to be aligned with Rx beam sweeping assumption;

- Indication of the preferred size of training dataset ;

- Indication of the minimum periodicity of the RS transmission.

Proposal 8: Regarding the model identification of BM-Case1 and BM-Case2, study the following aspects as a starting point for identification information which UE should provide to gNB:

- Information on model functionality, e.g., BM-Case1/BM-Case2 or DL beam pair/Tx beam prediction;

- Information of model inputs/nominal inputs, e.g., the number and pattern(s) of DL Tx beams or beam pairs in Set B;

- Information of model outputs/nominal outputs, e.g., the number of predicted beam and/or L1-RSRP;

- Information on assistance information for inference, e.g., the relative beam information;

- Information on model performance;

- Information on concurrent use with other AI/ML models and/or non-AI/ML features;

- Information on applicable conditions.

Proposal 9: Regarding the functionality identification of BM-Case1 and BM-Case2,study distinguishing different functionalities by large granularity characteristics, e.g. input type and/or output type.

Proposal 10: Regarding the data collection for AI/ML model inference at NW side, UE needs to report the measurement results of more than 4 beams optionally with the corresponding RS indicators to gNB, which can use L1 signaling or MAC CE.

Proposal 11: For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following options of beam indication mechanism with potential down-selection:

- Opt1: reusing legacy TCI indication mechanism (e.g., Rel-15/16 TCI framework and Rel-17 unified TCI framework);

- Opt2: only indicate spatial Rx parameter to UE.

Proposal 12: For BM-Case2 with a network-side AI/ML model, study how to indicate the beam for multiple future time instances.

Proposal 13: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, when reporting the predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s), there is no need to differentiate predicted L1-RSRP and measured L1-RSRP.

Proposal 14: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Alt.1 UE-side model monitoring, study the potential specification impacts on the following aspects:

- Reporting the decision of model activation/ deactivation/switching/fallback to the network;

- Acknowledgement mechanism of model activation/ deactivation/switching/fallback from the network.

Proposal 15: Regarding the model monitoring for BM-Case1 and BM-Case2, study the following aspects for the intermediate KPI calculation:

- How to calculate the intermediate KPIs based on both of Set A and Set B;

- How to calculate the intermediate KPIs only based on Set B.

Proposal 16: Regarding the model monitoring for BM-Case1 and BM-Case2, study the specification impacts on the following aspects:

- Model update/switching/fallback procedures based on model monitoring results, including the signaling exchange between the gNB and UE;

- Trigger condition for model update/switching/fallback.

- References

- Chair's notes RAN1_112 eom4, Chair, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2302697, Evaluation on AI/ML for beam management, CATT, RAN1#112bis e-meeting, April 17th – April 26th, 2023.

- Final Rport of RAN1#109-e meeting, Chair, RAN1#110, e-Meeting, May 9th – 20th, 2022.

- Chairman’s Notes RAN1#110bis-e, RAN1#110bis-e, e-Meeting, October 10th - 19th, 2022.

- Final Rport of RAN1#111 meeting, Chair, RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- R1-2302694, Discussion on AI/ML framework for NR air interface, CATT, RAN1#112bis e-meeting, April 17th-April 26th, 2023.