3GPP TSG RAN WG1 #112bis-e                                         R1-2302699

e-Meeting, April 17th – April 26th, 2023

Source: 	CATT

Title:	Evaluation on AI/ML-based positioning enhancement

Agenda Item:     9.2.4.1

Document for:    Discussion and Decision

- Introduction

In RAN1 #112 meeting, the following agreements were reached [1].

Agreement Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation. Agreement For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to the first Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Agreement For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset. Note: - Baseline evaluation area for InF-DH = 120x60 m. - if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computation complexity; AI/ML. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. Agreement The agreement made in RAN1#110 AI 9.2.4.1 is updated by adding additional note: Note: if complex value is used in modelling process, the number of the model parameters is doubled, which is also applicable for other AIs of AI/ML Agreement For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference. Agreement For direct AI/ML positioning, study the performance of model monitoring methods, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation). Agreement For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring. Conclusion - No dedicated evaluation is needed for the positioning accuracy performance of model switching - It does not preclude future discussion on model switching related performance Agreement For direct AI/ML positioning, study the impact of labelling error to positioning accuracy - The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models are not precluded - [Whether/how to study the impact of labelling error to label-based model monitoring methods] - [Whether/how to study the impact of labelling error for AI/ML assisted positioning.] Observation Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

In this document, we provide our evaluation results on AI/ML-based positioning.

- Simulation assumptions

In RAN1#109-e meeting, the common scenario parameters defined in Table 6-1 of TR 38.857 and some parameters of InF-DH scenario were agreed for evaluation of AI/ML-based positioning. In our simulations, the simulation assumptions are shown in Annex, and the dataset is generated by a system level simulator based on 3GPP simulation methodology. Both cases of perfect network synchronization and network synchronization with a truncated Gaussian distribution of 50 ns are investigated in our simulations. If network synchronization error is assumed, it is defined as a TRP-specific error value between the TRP and a timing reference source which is assumed to have perfect timing. For the generalization evaluation, we also simulate the generalization performance between InF-DH and InF-SH scenarios.

In RAN1#110bis e-meeting, the evaluation methodology of the AI/ML complexity (model complexity, computation complexity) for N TRPs was determined. In our simulations, complexity of Multi-TRP (i.e., one model for N TRPs) and Single-TRP (i.e., same model for N TRPs) are the assessed. The sampling interval of all simulations is 4 ns. In our model, the parameters are in real value.

In our evaluations, we use the same area of 120m*60m for training dataset and testing dataset.

- Simulation results

For the simulation results, most of them are captured in the excel sheet which are zipped together with this contribution. Those captured in the excel sheets are not duplicated here. Additionally, the LOS/NLOS identification results and model monitoring results are still provided in this contribution.

  - Direct AI/ML positioning

For direct AI/ML positioning, AI/ML model is used to directly estimate UE’s position without intermedium ToA estimation. We provide some simulation results of direct positioning based on AI/ML model in this section, including generalization, fine-tuning, mix-train, label error and different sizes of model input. The AI/ML model for estimating UE’s position is a classical ResNet18 model. The input of this AI/ML model is CIR, and the size of CIR is 18*1*256*2. The output of this AI/ML model is UE’s horizontal positon (x, y). We chose MSE loss as the loss function. The training epoch and learn rate is set to 100 and 1*10-4.

    - AI/ML model performance of baseline and generalization

      - AI/ML model performance with perfect network synchronization

For the simulations in this section, perfect network synchronization is assumed. This AI/ML model inference may be performed at UE side or LMF side. When inference is performed at UE side, the UE may utilize the CIR estimated from DL PRS and an AI/ML model to estimate the UE’s position directly. When this model is deployed at LMF side, the UE needs to transfer the CIR to LMF for AI/ML model inference.

The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together. For perfect network synchronization, the horizontal accuracy is 0.58m@90% of CDF percentile.

Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal positioning accuracy is 0.58m@90% of CDF percentile.

      - AI/ML model performance with network synchronization error

In Rel-16, various positioning technologies were specified to support regulatory as well as commercial use cases. In Rel-17, some positioning enhancements had been introduced. However, in current IIoT scenario, it is still worth of thinking about overcoming non-ideal factors, e.g. synchronization errors between multiple TRPs. Due to the non-ideal factors, positioning accuracy traditional positioning algorithms will be degraded. In this section, we investigate whether AI/ML model can overcome the influence of network synchronization error. The network synchronization error with a truncated Gaussian distribution of 50 ns is assumed, and the network synchronization error is defined as a TRP-specific error.

The AI/ML model for estimating UE’s position is a classical ResNet model. The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together. For network synchronization with a truncated Gaussian distribution of 50 ns, the horizontal positioning accuracy is 0.84m@90% of CDF percentile. Compared to the simulation results in 3.1.1.1, the positioning accuracy with network synchronization error is similar to the positioning accuracy of the case with perfect network synchronization. When AI/ML-based approach is applied in positioning, the network synchronization error is included in the corresponding training data set and test data set. If the network synchronization error is captured in the training set, AI/ML model would be able to learn how to overcome network synchronization error to improve positioning accuracy, since AI/ML-based positioning has data-driven characteristics. Based on our simulation results, AI/ML model is able to overcome the impact of network synchronization error through learning the internal relationship between the input training data with network synchronization error and the corresponding label.

Observation 2: For directly estimating UE’s positioning with network synchronization error (truncated Gaussian distribution of 50 ns), the horizontal positioning accuracy is 0.84m@90% of CDF percentile.

    - AI/ML model generalization performance with different assumptions

We design three cases to investigate the generalization capability of AI/ML model. The configurations of 3 cases are shown in Table 1.

Table 1: Configurations of three cases for investigating generalization capability of AI/ML model

|  | Scenario | Clutter parameters | Synchronization error |
| --- | --- | --- | --- |
| Case1 | InF-DH | {40%, 2m, 2m} | w/o |
| Case2 | InF-DH | {60%, 6m, 2m} | w |
| Case3 | InF-SH | {20%, 2m, 10m} | w/o |

The AI/ML model described in Section 3.1.1.1 is trained on a dataset with parameters of {60%, 6m, 2m} from InF-DH scenario, with perfect network synchronization. The model is tested on dataset corresponding to case 1 and case 3. For case 2, the time sampling windows are shifted forward by 50ns, resulting in exclusion of last few sampling points due to the excessive time delay. However, the discarded CIR has minimal impact on the previous results. For more details on the dataset, AI/ML model and evaluation results, please refer to the accompanying excel sheet.

Specifically:

- For case 1, the test dataset used is InF-DH {40%, 2m, 2m}, and the horizontal positioning accuracy is 2.64m@90% of CDF percentile.

- For case 2, the test dataset used is InF-DH {60%, 6m, 2m} with network synchronization error, and the horizontal positioning accuracy is 12.6m@90% of CDF percentile.

- For case 3, the test dataset used is InF-SH {20%, 2m, 10m}, and the horizontal positioning accuracy is 6.48m@90% of CDF percentile.

Observation 3: When the direct AI/ML positioning is applied to different scenarios or conditions, the generalization performance can be significantly affected, leading to a significant degradation in positioning accuracy.

    - AI/ML model generalization performance with mix-train

Section 3.1.2 presents our simulation results of generalization performance of the AI/ML model. In Section 3.1.3, we investigate the mix-train method as a means to enhance the model’s generalization capability. This involves mixing the datasets from different positioning scenarios/assumptions and training the model on the mixed training dataset. The AI/ML model is then tested on separate test datasets. Our simulations feature three different cases, which are detailed in the accompanying excel sheet.

      - Mix-train with different clutter parameters

In Section 3.1.3.1, we employ the mix-train method by combining 19440 data samples with a clutter parameter of {60%, 6m, 2m} and 1080 data samples with a clutter parameter of {40%, 2m, 2m} for model training. For testing, we used 1080 data samples with a clutter parameter of {40%, 2m, 2m} and another 1080 data samples with a clutter parameter of {60%, 6m, 2m} separately. The details of dataset and the simulation results for are provided in the accompanying excel sheet submitted together. The horizontal position accuracy is 1.77m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improves positioning accuracy from 2.64m to 1.77m. However, when the AI/ML model is tested on a small dataset with the assumption of a clutter parameter of {60%, 6m, 2m}, the positioning accuracy degrades from 0.58m to 1.31m compared to the case when using only the InF-DH {60%, 6m, 2m} dataset for AI/ML model training and testing.

Observation 4: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.77m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 2.64m.

      - Mix-train with different network synchronization assumptions

In this section, we employ the mix-train method by combining 19440 data samples with perfect synchronization and 1080 data samples with 50ns synchronization error for model training. For testing, we used 1080 data samples with perfect synchronization and another 1080 data samples with 50ns synchronization error separately. The accompanying excel sheet provides details on the dataset and simulation results for the AI/ML model trained using this method, which achieved a horizontal positioning accuracy of 3.04m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improved the accuracy from 12.6m to 3.04m. However, when the AI/ML model is tested on a small dataset with the assumption of perfect network synchronization, the positioning accuracy degrades from 0.58m to 1.37m when compared to using only the dataset with perfect network synchronization for AI/ML model training and testing.

Observation 5: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 3.04m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.6m.

      - Mix-train with different scenarios

In this section, we employ the mix-train method by combining 19440 data samples with InF-DH {60%, 6m, 2m} and 1080 data with InF-SH {20%, 2m, 10m} for model training. For testing, we used 1080 data samples with InF-SH {20%, 2m, 10m} and another 1080 data samples with InF-DH {60%, 6m, 2m} separately. The accompanying excel sheet provides details on the dataset and simulation results for the AI/ML model trained using this method, which achieved a horizontal positioning accuracy of 1.81m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.1.2, the use of mixed dataset during training improved the accuracy from 6.48m to 1.81m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of InF-DH{60%, 6m, 2m}, the position accuracy is degraded from 0.58m to 1.37m compared to using only the dataset with InF-DH{60%, 6m, 2m} for AI/ML model training and testing.

Observation 6: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.48m.

    - AI/ML model generalization performance with fine-tuning

In Section 3.1.3, we investigate the generalization capability of the AI/ML model and observe that the positioning accuracy will degrade when the training and testing datasets have different clutter parameters or network synchronization assumptions. This aspect is important, as it can be computationally and storage-intensive to train an AI/ML model for every scenarios or assumptions. To enhance the accuracy of the AI/ML model when training and testing datasets have different assumptions, we explore the potential of fine-tuning the model's performance.

We also investigate the impact of the quantity of fine-tuning data on the positioning accuracy. In our simulations, we vary the size of the fine-tuning dataset, which includes: 540, 1080, 1620 and 3600 labels. The simulation results are presented in the subsequent sections.

      - Fine-tuning with different clutter parameters

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of clutter parameter {60%, 6m, 2m}. This AI/ML model serves as the basic AI/ML model, which is then fine-tuned using a small dataset that assumes a clutter parameter {40%, 2m, 2m}. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620 with the aforementioned clutter parameter of {40%, 2m, 2m} is provided in the accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.1.2, the performance is significantly improved by the fine-tuning of the AI/ML model using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}.

Observation 7: For direct AI/ML positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

      - Fine-tuning with different network synchronization assumptions

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of perfect network synchronization. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620, 3600 with network synchronization error is provided accompanying excel sheet. Compared to the positioning accuracy in Section 3.1.2, the performance is significantly improved from 12.6m to 1.43m by the fine-tuning of the AI/ML model with a small dataset with size of 3600 with the assumption of network synchronization error.

Observation 8: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

      - Fine-tuning with different scenarios

The AI/ML model described in Section 3.1.1 is trained using dataset with the assumption of InF-DH. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 540, 1080, 1620 and 3600 with InF-SH are provided in accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.1.2, the performance is significantly improved from 6.48m to 1.90m by the fine-tuning of the AI/ML model with a small dataset with size of 3600 with the assumption of different scenarios.

Observation 9: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

    - AI/ML model performance of labeling error

It was concluded in RAN1#112 meeting to study the impact of labelling error on positioning accuracy. To achieve this, we introduce errors in the x-axis and y-axis of the labels, resulting in inaccurate labels. The error in each dimension of x-axis and y-axis is modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. We then set the value of L as 1m, 3m and 5m. The simulation result is provided in the accompanying excel sheet.

Specifically:

- When the value of L is 1m, the horizontal position accuracy is 1.47m@90% of CDF percentile.

- When the value of L is 3m, the horizontal position accuracy is 2.95m@90% of CDF percentile.

- When the value of L is 5m, the horizontal position accuracy is 4.41m@90% of CDF percentile.

Observation 10: When AI/ML model is trained with different value of label error, as value of L increases, direct AI/ML positioning performance deteriorates.

    - Performance of the different model input

For AI/ML-based direct positioning, the current model’s input dimensions may cause an increase in the model complexity and computational complexity. As a result, it is attractive to reduce the input dimensions of the model.

      - Different number of TRP as model input

We have provided simulation results in the excel sheet for different numbers of TRPs used as inputs for the AI/ML model. When only part of TRPs are used as input, the FLOPs is reduced compared to the case when all TRPs are used. Additionally, the model overhead is reduced while maintaining similar horizontal positioning accuracy.

In our simulation, we select 9 TRPs out of 18 TRPs for model input. The selected TRPs are with the ID of {0, 2, 4, 6 ... 14, 16}. The horizontal position accuracy is slightly degraded from 0.58m@90% to 0.71m@90% of CDF percentile. The computational complexity is reduced from 2.78G FLOPs to 1.62G FLOPs.

Observation 11: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 12: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

  - AI/ML-assisted positioning

For AI/ML assisted positioning, AI/ML model is used to estimate timing and/or angle of measurement or LOS/NLOS. We provide some simulation results of AI/ML assisted positioning in this section. The AI/ML model is used to estimate ToA, and then the estimated ToA is used to calculate UE’s position using the traditional positioning calculation algorithms. In Section 3.2.4, The AI/ML model is used to identify LOS/NLOS. The LOS/NLOS identification can assist the non-AI/ML positioning algorithm.

    - AI/ML model for estimating ToA

      - AI/ML model performance with perfect network synchronization

For the simulations in this section, perfect network synchronization is assumed. The AI/ML model for estimating ToA is a classical ResNet18 model. The input of this AI/ML model is CIR, and the size of CIR is 18*1*256*2. The output of this AI/ML model is 18 ToAs from 18 TRPs to target UE, respectively, i.e. the size of output is 1*18. This AI/ML model inference may be performed at UE side or LMF side. When inference is performed at UE side, the UE may utilize the DL-PRS CIR and an AI/ML model to estimate ToA. When this model is deployed at LMF side, the UE needs to transfer the DL-PRS CIR to LMF for ToA estimating based on AI/ML model.

The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together. For perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

Observation 13: For AI/ML-assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

      - AI/ML model performance with network synchronization error

In this section, we conduct a simulation with network synchronization error assumption. The network synchronization error with a truncated Gaussian distribution of 50 ns is assumed, and the network synchronization error is defined as a TRP-specific error.

The AI/ML model for estimating UE’s position is a classical ResNet model. The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together. The intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy. Compared to the simulation result in the excel sheet submitted together, the positioning accuracy of AI/ML assisted positioning with network synchronization error is similar to the positioning accuracy of AI/ML assisted positioning with perfect network synchronization. Based on our simulation results, the positioning accuracy of AI/ML model with network synchronization error is not deteriorated, which means the AI/ML model can overcome the influence of network synchronization error on the test data set through learning the internal relationship between the training data set with network synchronization error and the corresponding labels.

Observation 14: For AI/ML-assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

      - AI/ML model generalization performance with different assumptions

For AI/ML assisted positioning, to investigate the AI/ML model generalization capability, we further evaluate the AI/ML model generalization performance with different assumptions between training dataset and test dataset, e.g. different clutter parameters assumptions and different network synchronization assumptions.

The AI/ML model is trained with the assumption of clutter parameter InF-DH {60%, 6m, 2m} with perfect network synchronization, tested dataset with three cases. The cases are presented in Table 1.The details of dataset, AI/ML model and evaluation results are given in the excel sheet submitted together.

Specifically:

- For case 1, the test dataset is InF-DH {40%, 2m, 2m}, and the horizontal position accuracy is 3.11m@90% of CDF percentile.

- For case 2, the test dataset is InF-DH {60%, 6m, 2m} with network synchronization, and the horizontal position accuracy is 12.8m@90% of CDF percentile.

- For case 3, the test dataset is InF-SH {20%, 2m, 10m}, and the horizontal position accuracy is 6.894m@90% of CDF percentile.

Observation 15: For generalization performance with different assumptions on AI/ML assisted positioning, the positioning accuracy is seriously degraded.

      - AI/ML model generalization performance with mix-train

In Section 3.2.1.3, the simulation results of AI/ML assisted positioning model generalization is seriously degraded. We investigate the mix-train method to solve this problem, which is to mix the dataset from different assumptions for model training. In our simulations, the AI/ML model is trained with the mix dataset and tested separately. We keep the size of mix-training dataset is same as the size of fine-tuning dataset to comparing the advantages of two schemes.

        - Mix-train with different clutter parameters

In this section, we employ the mix-train method by combining 20000 data samples with a clutter parameter of {60%, 6m, 2m} and 1600 data samples with a clutter parameter of {40%, 2m, 2m} to for model training. For testing, we used 1600 data samples with a clutter parameter of {40%, 2m, 2m} and 1600 data samples with a clutter parameter of {60%, 6m, 2m}. The details of dataset and the simulation results for an AI/ML model mix-train are provided in the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.51m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 3.11m to 1.51m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of clutter parameter {60%, 6m, 2m}, the positioning accuracy is degraded from 0.655m to 1.35m compared with AI/ML model training and testing only with InF-DH {60%, 6m, 2m}.

Observation 16: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.51m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 3.11m.

        - Mix-train with different network synchronization assumptions

In this section, we employ the mix-train method by combining 20000 data samples with perfect synchronization error and 1600 data samples with 50ns synchronization error for model training. For testing, we used 1600 data samples with perfect synchronization error and 1600 data samples with 50ns synchronization error. The details of dataset and the simulation results for an AI/ML model mix-train are provided in the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.81m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 12.8m to 1.81m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of perfect network synchronization, the positioning accuracy is degraded from 0.655m to 1.32m compared with AI/ML model training and testing only with perfect network synchronization.

Observation 17: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.8m.

        - Mix-train with different scenarios

In this section we employ the mix-train method by combining 20000 data samples with InF-DH {60%, 6m, 2m} and 1600 data samples with InF-SH {20%, 2m, 10m} for model training. For testing, we used 1600 data samples with InF-SH {20%, 2m, 10m} and 1600 data samples with InF-DH {60%, 6m, 2m}. The details of dataset and the simulation results for an AI/ML model mix-train are provided in the accompanying excel sheet submitted together. The horizontal positioning accuracy is 1.467m@90% of CDF percentile. Compared to the positioning accuracy achieved in Section 3.2.1.3, the positioning accuracy is improved from 6.894m to 1.467m by training the AI/ML model with mixed dataset. When the AI/ML model is tested by a small dataset with the assumption of InF-DH {60%, 6m, 2m}, the position accuracy is degraded from 0.655m to 1.342m compared with AI/ML model training and testing only with InF-DH {60%, 6m, 2m}.

Observation 18: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.467m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.894m.

      - AI/ML model generalization performance with fine-tuning

In Section 3.2.1.3, we investigate the generalization capability of the AI/ML model and observe that the positioning accuracy will degrade when the training and testing datasets have different clutter parameters or network synchronization assumptions. This aspect is important, as it can be computationally and storage-intensive to train an AI/ML model for every scenarios or assumptions. To enhance the accuracy of the AI/ML model when training and testing datasets have different assumptions, we explore the potential of fine-tuning the model's performance

We also investigate the impact of the quantity of fine-tuning data on the positioning accuracy. In our simulations, we vary the size of the fine-tuning dataset size, which includes: 800, 1600, 2400 and 3200 labels. The simulation results are presented in the subsequent sections.

        - Fine-tuning with different clutter parameters

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of clutter parameter {60%, 6m, 2m}. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset that assumes clutter parameter {40%, 2m, 2m}. The details of the dataset and the simulation results for an AI/ML model fine-tuned using dataset sizes of 800, 1600, 2400, 3200 with the aforementioned clutter parameter of {40%, 2m, 2m} is provided in the accompanying excel sheet. Compared to the positioning accuracy achieved in Section 3.2.1.3, the performance is significantly improved by the fine-tuning of the AI/ML model using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}.

Observation 19: For AI/ML-assisted positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

        - Fine-tuning with different network synchronization assumptions

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of perfect network synchronization. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with network synchronization error. The details of the dataset, AI/ML model and evaluation results are given in the accompanying excel sheet. The simulation results for an AI/ML model fine-tuned by 800, 1600, 2400, 3200 dataset size with network synchronization error is provided in the excel sheet submitted together. Compared to the positioning accuracy in Section 3.2.1.3, the positioning accuracy is significantly improved from 12.8m to 1.87m by the fine-tuning of the AI/ML model with a small dataset with size of 3200 with network synchronization error.

Observation 20: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

        - Fine-tuning with different scenarios

The AI/ML model described in Section 3.2.1.1 is trained using dataset with the assumption of InF-DH. This AI/ML serves as the basic AI/ML model, which is then fine-tuned using a small dataset with InF-SH. The details of the dataset, AI/ML model simulation results for an AI/ML model fine-tuned by 800, 1600, 2400, 3200 dataset size with InF-SH is provided in the accompanying excel sheet. Compared to the positioning accuracy in Section 3.2.1.3, the positioning accuracy is significantly improved from 6.894m to 2.41m by the fine-tuning of the AI/ML model with a small dataset with size of 3200 with the assumption of different scenarios.

Observation 21: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

    - AI/ML model performance of labeling error

It was concluded in RAN1#112 meeting to study the impact of labelling error on positioning accuracy. To achieve this, we introduce errors in the x-axis and y-axis of the labels, resulting in inaccurate labels. We then set the value of L as 1m, 3m and 5m. The error-added UE coordinate POS' is used to generate TOA' with the base station coordinates (no error). TOA' was then used as a label for AI-assisted positioning to train the model. The simulation result is provided in the accompanying excel sheet.

Specifically:

- When the value of L is 1m, the horizontal positioning accuracy is 1.901 m@90% of CDF percentile.

- When the value of L is 3m, the horizontal positioning accuracy is 4.392 m@90% of CDF percentile.

- When the value of L is 5m, the horizontal positioning accuracy is 6.753 m@90% of CDF percentile.

Observation 22: When AI/ML model is trained with different value of label error, as value of L increases, AI/ML–assisted positioning performance deteriorated.

    - Performance of the different model input dimensions

According to the conclusion of the RAN1#112 meeting, study the model input, considering the tradeoff among model performance, model complexity and computational complexity.

      - Different number of TRP as model input

We have provided simulation results in the excel sheet for different numbers of TRPs used as inputs for the AI/ML model. When only part of TRPs are used as input, the FLOPs is reduced compared to the case when all TRPs are used. Additionally, the model overhead is reduced while maintaining similar horizontal positioning accuracy.

In our simulation, we select 9 TRPs out of 18 TRPs for model input. The selected TRPs are with the ID of {0, 2, 4, 6 ... 14, 16}. The horizontal position accuracy is slightly degraded from 0.655m@90% to 0.97m@90% of CDF percentile. The computational complexity is reduced from 60.8M FLOPs to 38.46M FLOPs.

Observation 23: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 24: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

    - AI/ML model for LOS/NLOS identification

LOS/NLOS identification is to assist the non-AI/ML positioning algorithm. The following is our simulation content and results for different input and model generalization performance. In our simulation, we select InF-DH {40%, 2m, 2m} as baseline. Because scenario in InF-DH {40%, 2m, 2m} probability of LOS/NLOS is approximately 50%, which is suitable for the AI/ML model training.

      - AI/ML model for LOS/NLOS identification performance with different model inputs

The CIR data format is 1*256*2. The AI/ML model is trained with the CIR of InF-DH {40%, 2m, 2m}. The details of dataset and the simulation results for an AI/ML model test by 6480 dataset size with clutter parameter of InF-DH {40%, 2m, 2m} is provided in Table 2.

The AI/ML model is trained with the PDP of InF-DH {40%, 2m, 2m}. The details of dataset and the simulation results for an AI/ML model test by 6480 dataset size with clutter parameter of InF-DH {40%, 2m, 2m} is provided in Table 2.

Table 2: Evaluation results for AI/ML-based LOS/NLOS identification model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | LOS or NLOS identification accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type: CIR; Size: 1*256*2 | LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | {40%, 2m, 2m} | {40%, 2m, 2m} | 58320 | 6480 | 0.7M | 18*8M FLOPs | 94.2% |
| Type:PDP; Size: 1*256*2 | LOS or NLOS Size: 1*2 | LOS or NLOS with 100% ground truth label | InF-DH{40%, 2m, 2m} | InF-DH{40%, 2m, 2m} | 58320 | 6480 | 0.7M | 18*8M FLOPs | 94.89% |

Observation 25: For AI/ML-assisted positioning with LOS/NLOS identification, the performance is similar with model inputs between PDP and CIR.

      - AI/ML model for LOS/NLOS identification generalization performance

In this section, we investigate the generalization performance of the AI/ML-based LOS/NLOS identification. Specifically, the AI/ML model is trained with PDP as input and dataset from InF-DH {40%, 2m, 2m}, and tested on a dataset of size 6480 with a clutter parameter of InF-DH {60%, 6m, 2m}. The results show that the LOS or NLOS identification accuracy reaches 96.23%, which is even slightly better than the original result. The reason is that the InF-DH {60%, 6m, 2m} has a much higher probability of NLOS which leads to imbalanced study. Additionally, the AI/ML model is trained with PDP as input from dataset of InF-DH {40%, 2m, 2m}, and tested on a dataset of size 6480 with clutter parameter of InF-SH {20%, 10m, 2m}, The LOS/NLOS identification accuracy can be 91%. The details of dataset and the simulation results are provided in Table 3. Given the good generalization performance, mix-training and fine-tuning methods are not necessary.

Table 3: Evaluation generation performance results for AI/ML-based LOS/NLOS identification model deployed on UE/LMF-side, ResNet18

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | LOS or NLOS identification accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| Type:PDP; Size: [1,256] | Probability of los or nlos Size: [1, 2] | Los or nlos with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-DH {40%, 2m, 2m} | 58320 | 6480 | 0.7M | 8M | 94.89% |
| Type:PDP; Size: [1,256] | Probability of los or nlos Size: [1, 2] | Los or nlos with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-DH {60%,6m,2m} | 58320 | 6480 | 0.7M | 8M | 96.23% |
| Type:PDP; Size: [1,256] | Probability of los or nlos Size: [1, 2] | Los or nlos with 100% ground truth label | InF-DH {40%, 2m, 2m} | InF-SH {20%,10m,2m} | 58320 | 6480 | 0.7M | 8M | 91% |

Observation 26: For the AI/ML-assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

  - Evaluation of model monitoring

For AI/ML-based positioning, a PRU with known location can generate a ground truth label for monitoring the AI/ML model. However, collecting other ground truth labels generated by a normal UE may be challenging. Therefore, AI/ML model monitoring without ground truth label should be studied.

The label-based method allows for direct evaluation of AI/ML model performance through the use of coordinates. The statistical prediction error of the CDF is already sufficient to evaluate the AI/ML model performance..

For the label-free method, we provide the method of relative displacement between time T1 and time T2 by the motion sensor to monitor the AI/ML model performance. UE using one or more motion sensors obtains the movement information, and the movement information comprises displacement results estimated as an ordered series of points. The L1 vector was obtained by comparing the movement information at time T1 and time T2 of the kinematic apparatus. The L2 vector was obtained by comparing the predicted values of the AI/ML model at time T1 and time T2. The correlation of Vector L1 and vector L2 are evaluated. More details on this monitoring method can be found in our companion paper [2].

In this section, we provide the results of model monitoring method. We used the simulation assumptions with a poor generalization performance to evaluate our monitoring scheme.

    - Direct AI/ML positioning

For direct AI/ML positioning, we used the AI/ML model in Section 3.1.1.1 trained with the assumption of InF-DH with clutter parameter {60%, 6m, 2m}, and the AI/ML model are monitored with InF-DH {60%, 6m, 2m}, InF-DH {40%, 2m, 2m} and InF-SH {20%, 2m, 10m} respectively. The CDF plots of the |L1-L2| vectors in different test scenarios are shown below.

Figure 1: CDF of relative displacement method for direct AI/ML positioning.

As can be observed from the above figure, the error of |L1-L2| is small when the training dataset and testing dataset are from the same scenario. The error of |L1-L2| significantly increases when the training dataset and testing dataset are from different scenarios, especially in InF-SH with clutter parameter of {20%, 2m, 10m}. Such result validates the possibility of using relative displacement method to monitor the model for direct AI/ML positioning.

Observation 27: For direct AI/ML positioning, the relative displacement method can monitor the model performance.

    - AI/ML-assisted positioning

For AI/ML-assisted positioning, we used the AI/ML model in Section 3.2.1.1 trained with the assumption of InF-DH with clutter parameter {60%, 6m, 2m}, and the AI/ML model are monitored with InF-DH {60%, 6m, 2m}, InF-DH {40%, 2m, 2m} and InF-SH {20%, 2m, 10m} respectively. The CDF plots of the |L1-L2| vectors in different test scenarios are shown below.

Figure 2: CDF of relative displacement method for AI/ML-assisted positioning.

As can be observed from the above figure, the error of |L1-L2| is small when the training dataset and testing dataset are from the same scenario. The error of |L1-L2| significantly increases when the training dataset and testing dataset are from different scenarios, especially in InF-SH with clutter parameter of {20%, 2m, 10m}. Such result validates the possibility of using relative displacement method to monitor the model for AI/ML-assisted positioning.

Observation 28: For AI/ML-assisted positioning, the relative displacement method can monitor the model performance.

- Conclusions

In this contribution, we provide our views on AI/ML for positioning. The observations and proposals are summarized as follows:

Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal positioning accuracy is 0.58m@90% of CDF percentile.

Observation 2: For directly estimating UE’s positioning with network synchronization error (truncated Gaussian distribution of 50 ns), the horizontal positioning accuracy is 0.84m@90% of CDF percentile.

Observation 3: When the direct AI/ML positioning is applied to different scenarios or conditions, the generalization performance can be significantly affected, leading to a significant degradation in positioning accuracy.

Observation 4: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.77m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 2.64m.

Observation 5: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 3.04m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.6m.

Observation 6: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.48m.

Observation 7: For direct AI/ML positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 8: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 9: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 10: When AI/ML model is trained with different value of label error, as value of L increases, direct AI/ML positioning performance deteriorates.

Observation 11: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 12: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

Observation 13: For AI/ML-assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile.

Observation 14: For AI/ML-assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

Observation 15: For generalization performance with different assumptions on AI/ML assisted positioning, the positioning accuracy is seriously degraded.

Observation 16: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.51m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 3.11m.

Observation 17: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.8m.

Observation 18: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.467m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.894m.

Observation 19: For AI/ML-assisted positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 20: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 21: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

Observation 22: When AI/ML model is trained with different value of label error, as value of L increases, AI/ML–assisted positioning performance deteriorated.

Observation 23: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade.

Observation 24: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced.

Observation 25: For AI/ML-assisted positioning with LOS/NLOS identification, the performance is similar with model inputs between PDP and CIR.

Observation 26: For the AI/ML-assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

Observation 27: For direct AI/ML positioning, the relative displacement method can monitor the model performance.

Observation 28: For AI/ML-assisted positioning, the relative displacement method can monitor the model performance.

- References

- Chairman’s Notes RAN1#112, Meeting, Athens, Greece, February 27th – March 3rd, 2023.

- R1-2302700, Discussion on AI/ML-based positioning enhancement CATT, RAN1#112-bis, e-meeting, April 17th – April 26th, 2023.

- Annex

Table 4: Simulation assumptions in our simulation

| Parameters | Values |
| --- | --- |
| Scenario | 1nF-DH |
| Hall size | 120x60 m |
| Total gNB TX power, dBm | 24 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ |
| Penetration loss | 0 dB |
| Number of floors | 1 |
| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area is selected from the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. |
| UE antenna height | Baseline: 1.5 m |
| Min gNB-UE distance(2D), m | 0 m |
| gNB antenna height | Baseline: 8 m |
| Clutter parameters: {density , height ,size } | High clutter density: - {60%, 6m, 2m} - {40%, 2m, 2m} |
| Carrier frequency, GHz | 3.5 GHz |
| Bandwidth, MHz | 100 MHz |
| Subcarrier spacing, kHz | 30 kHz for 100 MHz |
| Distribution of UE location | Uniform distribution |
| Network synchronization | (1) Perfect network synchronization (2) Network synchronization with a truncated Gaussian distribution of 50 ns |