3GPP TSG RAN WG1 #112bis-e		                                                      R1-2302701

e-meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Decision

- Introduction

During RAN1#112, evaluation scenarios, methodologies and assumptions on NR duplex evolution were further discussed and many consensuses were reached on this topic [1].

In this contribution, views on remaining issues for evaluation methodologies and evaluation assumptions are further provided. And initial SLS results are updated based on the agreed simulation assumptions in RAN1#112 meeting.

- Discussion

  - LLS evaluation methodology

    - LLS purpose

The following agreement was made in RAN1#111 with a couple of FFS points.

Agreement RAN1 agrees link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2, interested companies may perform LLS at least for the following purpose: - To evaluate coverage performance - Option 1: Take link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as starting point to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD considering inter-gNB/sector interference and self interference. - Other options (e.g. SLS as a tool to obtain the coverage metric) are not precluded - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF - FFS: - To evaluate advanced receivers and realistic demodulation performance - To evaluate UE-UE CLI mitigation performance - To evaluate gNB-gNB CLI mitigation performance - To evaluate feasibility and performance of self-IC accounting for realistic non-linearities in the gNB transmit and receive chains - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

For the CLI mitigation performance evaluation, it depends more on CLI handling scheme itself. Some CLI mitigation solutions should be evaluated via LLS, e.g. advanced receiver based solution, while some CLI mitigation solutions should be evaluated via SLS, e.g. scheduling coordination based solution. From our point of view, self-interference mitigation performance mainly depends on RAN4’s input other than RAN1’s LLS evaluation. Therefore, whether LLS is used for other purposes in addition to coverage performance evaluation can be up to companies’ interests.

Proposal 1: LLS for other purpose besides coverage performance evaluation is up to companies’ interests.

    - Channel estimation for SBFD with PUSCH repetition type A and TBoMS PUSCH

Four techniques of coverage enhancement were agreed in last meeting as listed in the following agreement. Joint channel estimation across SBFD and non-SBFD slots for case 4 and case

5 are for further study.

Agreement Regarding the schemes for link level evaluation of PUSCH coverage performance, - For baseline legacy TDD, consider - Single slot PUSCH transmission - For SBFD, consider the following techniques of coverage enhancement: - Case 1: SBFD with single slot PUSCH - Case 2: SBFD with PUSCH repetition type A - Case 3: SBFD with TBoMS PUSCH - Case 4: SBFD with PUSCH repetition type A and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - Case 5: SBFD with TBoMS PUSCH and joint channel estimation - FFS: Joint channel estimation across SBFD and non-SBFD slots - UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate. Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

As UL slots could be non-continuous in legacy TDD system, e.g. {DDDSU} pattern, joint channel estimation cannot be utilized. In contrary to legacy TDD system, UL transmission can be scheduled on continues SBFD slots within UL subband in SBFD system. Therefore, performance gain from joint channel estimation may be expected in SBFD system.

Same time and frequency resource is used for all the repetitions for PUSCH repetition type A and same frequency resource is used for TBoMS in different slots. For SBFD aware UEs, separate configurations (e.g. frequency resources, UL power control parameters or beam/spatial relations) for SBFD slots and non-SBFD slots are to be studied.

If different transmission configurations are configured for SBFD and non-SBFD slots respectively, the channel could be different in different slots. From this point of view, separate channel estimation is preferred.

Proposal 2: If different transmission configurations (e.g. frequency resources, UL power control parameters or beam/spatial relations) are applied for SBFD and non-SBFD slots, separate channel estimation is preferred.

If same transmission configuration is used for SBFD and non-SBFD slots, self-interference and or CLI impact which will introduce difference between SBFD slots and non-SBFD slots should be taken into account. The gNB can decide whether self-interference exists based on DL and UL scheduling information, e.g. no SI impact exists if no DL transmission at some slot especially at low DL traffic load. The gNB can also decide whether to use joint channel estimation or not, based on SI/ CLI strength and the SI/CLI cancellation capability if SI/CLI indeed exists. If the SI/CLI impact is minor after SI/CLI cancellation, joint channel estimation may also be considered.  Therefore, based on its implementation, gNB can decide whether to use joint channel estimation or not.

Based on the discussion above, whether adopting joint channel estimation across SBFD and non-SBFD slots or not is up to scheduling information, SI/CLI impact and gNB’s implementation. To facilitate aligning evaluation results among companies, separate channel estimation for SBFD and non-SBFD slots is proposed to be baseline while JCE can be as optionally used and reported.

Proposal 3: If same transmission configuration is used for SBFD and non-SBFD slots, separate channel estimation for SBFD and non-SBFD slots is the baseline while JCE can be optionally used and reported.

- Initial SLS results

- Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

In RAN1#109-e meeting, Alt 2 and Alt 4 were agreed as two of the four SBFD configurations alternatives. The initial simulation results of Alt 2 and Alt 4 for deployment Case 1 are given respectively in the following sections.

  - Simulation results of SBFD configuration Alt 2 & Alt 4 with large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

    - Indoor office

Table 1: Evaluation results of SBFD Alt 2 for indoor in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 336.84 | 279.85 | -16.92 | 225.83 | 179.82 | -20.37 | 89.79 | 71.03 | -20.89 |
|  | 5% | 245.65 | 192.71 | -21.55 | 162.63 | 121.44 | -25.33 | 73.87 | 52.97 | -28.29 |
|  | 50% | 327.14 | 291.86 | -10.78 | 224.90 | 185.76 | -17.40 | 89.54 | 71.63 | -20.00 |
|  | 95% | 428.15 | 341.75 | -20.18 | 272.74 | 220.00 | -19.34 | 105.83 | 83.12 | -21.46 |
| DL Tail-UPT CDF (Mbps) | Mean | 218.61 | 196.07 | -10.31 | 144.31 | 129.03 | -10.59 | 65.33 | 53.90 | -17.50 |
|  | 5% | 136.24 | 134.16 | -1.53 | 99.20 | 83.71 | -15.61 | 50.27 | 42.62 | -15.22 |
|  | 50% | 221.85 | 202.31 | -8.81 | 148.74 | 134.50 | -9.57 | 65.71 | 54.80 | -16.60 |
|  | 95% | 277.20 | 237.18 | -14.44 | 180.03 | 154.15 | -14.38 | 75.16 | 61.81 | -17.76 |
| DL Median-UPT CDF (Mbps) | Mean | 347.60 | 274.83 | -20.93 | 238.67 | 185.09 | -22.45 | 88.00 | 68.68 | -21.95 |
|  | 5% | 249.78 | 186.22 | -25.45 | 164.46 | 121.13 | -26.35 | 72.22 | 52.06 | -27.91 |
|  | 50% | 332.67 | 280.50 | -15.68 | 239.85 | 191.76 | -20.05 | 89.04 | 68.55 | -23.01 |
|  | 95% | 442.41 | 336.60 | -23.92 | 304.28 | 230.41 | -24.28 | 103.03 | 78.64 | -23.67 |
| UL Average-UPT CDF (Mbps) | Mean | 131.50 | 193.87 | 47.43 | 79.45 | 109.25 | 37.51 | 27.26 | 36.62 | 34.34 |
|  | 5% | 74.10 | 135.73 | 83.17 | 53.01 | 82.72 | 56.05 | 16.80 | 25.53 | 51.96 |
|  | 50% | 130.48 | 191.90 | 47.07 | 82.37 | 107.48 | 30.48 | 26.90 | 36.19 | 34.54 |
|  | 95% | 203.08 | 265.54 | 30.76 | 106.58 | 133.57 | 25.32 | 37.29 | 47.97 | 28.64 |
| UL Tail-UPT CDF (Mbps) | Mean | 84.96 | 143.68 | 69.11 | 47.05 | 70.51 | 49.86 | 17.80 | 26.34 | 47.98 |
|  | 5% | 50.68 | 96.25 | 89.92 | 29.44 | 45.11 | 53.23 | 11.23 | 18.58 | 65.45 |
|  | 50% | 82.39 | 140.28 | 70.26 | 46.97 | 72.17 | 53.65 | 18.62 | 26.69 | 43.34 |
|  | 95% | 103.99 | 188.06 | 80.84 | 61.45 | 84.10 | 36.86 | 20.62 | 31.59 | 53.20 |
| UL Median-UPT CDF (Mbps) | Mean | 122.09 | 188.71 | 54.57 | 77.77 | 114.63 | 47.40 | 24.82 | 34.14 | 37.55 |
|  | 5% | 75.30 | 121.23 | 61.00 | 46.26 | 79.68 | 72.24 | 16.46 | 26.38 | 60.27 |
|  | 50% | 125.88 | 184.53 | 46.59 | 83.94 | 112.18 | 33.64 | 24.13 | 33.93 | 40.61 |
|  | 95% | 159.89 | 250.99 | 56.98 | 107.24 | 149.75 | 39.64 | 32.83 | 41.43 | 26.20 |
| DL Packet-Latency CDF (ms) | Mean | 11.80 | 14.15 | -19.92 | 17.48 | 21.01 | -20.19 | 46.23 | 56.09 | -21.33 |
|  | 5% | 6.97 | 10.55 | -51.36 | 12.04 | 15.72 | -30.56 | 33.91 | 41.79 | -23.24 |
|  | 50% | 11.40 | 13.58 | -19.12 | 16.47 | 19.90 | -20.83 | 44.94 | 54.54 | -21.36 |
|  | 95% | 17.32 | 19.94 | -15.13 | 26.78 | 30.26 | -12.99 | 60.82 | 75.95 | -24.88 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 7.21 | 4.91 | 31.90 | 12.47 | 8.72 | 30.07 | 37.37 | 26.89 | 28.04 |
|  | 5% | 4.79 | 2.89 | 39.67 | 8.09 | 5.47 | 32.39 | 20.76 | 18.52 | 10.79 |
|  | 50% | 6.21 | 4.63 | 25.44 | 11.33 | 8.44 | 25.51 | 35.27 | 26.22 | 25.66 |
|  | 95% | 12.00 | 7.76 | 35.33 | 21.45 | 13.10 | 38.93 | 58.53 | 36.46 | 37.71 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.02 | 5.16 |  | 19.89 | 16.52 |  | 40.94 | 32.98 |  |
|  | UL | 1.53 | 2.45 |  | 5.74 | 8.05 |  | 10.52 | 18.48 |  |
| Type-2 RU (%) | DL | 7.52 | 8.06 |  | 24.86 | 25.82 |  | 51.18 | 51.54 |  |
|  | UL | 7.66 | 6.81 |  | 28.69 | 22.36 |  | 52.61 | 51.34 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.20 |  | 1.10 | 1.00 |  | 4.90 | 5.10 |  |
|  | UL | 0.30 | 0.30 |  | 1.40 | 1.3 |  | 5.7 | 5.6 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 1, SBFD achieves better UL user throughput at all load conditions for indoor office than that of the legacy TDD at the cost of decreased DL user throughput due to more resources are allocated for UL in SBFD.

Observation 1: For indoor office, compared to legacy TDD, SBFD with Alt 2 achieves better UL user throughput at all three load conditions at the cost of decreased DL user throughput.

As can be seen from Table 1, UL latency of SBFD is reduced at all three load conditions for indoor office scenario at the cost of slightly increased DL latency.

Observation 2: For indoor office, compared to legacy TDD, SBFD with Alt 2 can significantly reduce the UL latency at the cost of slightly increased DL latency.

Table 2: Evaluation results of SBFD Alt 4 for indoor in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 336.84 | 392.24 | 16.45 | 225.83 | 242.57 | 7.41 | 89.79 | 93.95 | 4.63 |
|  | 5% | 245.65 | 264.30 | 7.59 | 162.63 | 192.06 | 18.10 | 73.87 | 76.11 | 3.03 |
|  | 50% | 327.14 | 407.96 | 24.71 | 224.90 | 243.91 | 8.45 | 89.54 | 93.20 | 4.09 |
|  | 95% | 428.15 | 487.53 | 13.87 | 272.74 | 299.90 | 9.96 | 105.83 | 110.96 | 4.85 |
| DL Tail-UPT CDF (Mbps) | Mean | 218.61 | 248.67 | 13.75 | 144.31 | 158.71 | 9.98 | 65.33 | 69.89 | 6.98 |
|  | 5% | 136.24 | 165.47 | 21.45 | 99.20 | 106.23 | 7.09 | 50.27 | 51.17 | 1.79 |
|  | 50% | 221.85 | 259.21 | 16.84 | 148.74 | 166.54 | 11.97 | 65.71 | 70.43 | 7.18 |
|  | 95% | 277.20 | 306.23 | 10.47 | 180.03 | 199.21 | 10.65 | 75.16 | 82.16 | 9.31 |
| DL Median-UPT CDF (Mbps) | Mean | 347.60 | 392.37 | 12.88 | 238.67 | 265.63 | 11.30 | 88.00 | 92.02 | 4.57 |
|  | 5% | 249.78 | 275.09 | 10.13 | 164.46 | 196.69 | 19.60 | 72.22 | 75.06 | 3.93 |
|  | 50% | 332.67 | 389.67 | 17.13 | 239.85 | 266.55 | 11.13 | 89.04 | 90.71 | 1.88 |
|  | 95% | 442.41 | 493.50 | 11.55 | 304.28 | 334.80 | 10.03 | 103.03 | 109.93 | 6.70 |
| UL Average-UPT CDF (Mbps) | Mean | 130.32 | 161.12 | 23.63 | 79.45 | 93.66 | 17.89 | 27.26 | 26.50 | -2.79 |
|  | 5% | 73.17 | 112.24 | 53.40 | 53.01 | 65.12 | 22.84 | 16.80 | 23.33 | 38.87 |
|  | 50% | 130.48 | 157.09 | 20.39 | 82.37 | 95.77 | 16.27 | 26.90 | 26.49 | -1.52 |
|  | 95% | 203.08 | 227.10 | 11.83 | 106.58 | 110.21 | 3.41 | 37.29 | 29.81 | -20.06 |
| UL Tail-UPT CDF (Mbps) | Mean | 84.96 | 106.29 | 25.11 | 47.05 | 57.32 | 21.83 | 17.80 | 21.10 | 18.54 |
|  | 5% | 50.68 | 80.22 | 58.29 | 29.44 | 45.23 | 53.63 | 11.23 | 17.54 | 56.19 |
|  | 50% | 82.39 | 104.91 | 27.33 | 46.97 | 57.62 | 22.67 | 18.62 | 21.45 | 15.20 |
|  | 95% | 103.99 | 125.90 | 21.07 | 61.45 | 67.14 | 9.26 | 20.62 | 23.04 | 11.74 |
| UL Median-UPT CDF (Mbps) | Mean | 122.09 | 169.24 | 38.62 | 77.77 | 94.94 | 22.08 | 24.82 | 25.23 | 1.65 |
|  | 5% | 75.30 | 125.19 | 66.25 | 46.26 | 72.49 | 56.70 | 16.46 | 22.32 | 35.60 |
|  | 50% | 125.88 | 171.71 | 36.41 | 83.94 | 95.62 | 13.91 | 24.13 | 25.37 | 5.14 |
|  | 95% | 159.89 | 392.24 | 145.32 | 107.24 | 115.48 | 7.68 | 32.83 | 28.16 | -14.22 |
| DL Packet-Latency CDF (ms) | Mean | 11.80 | 10.50 | 11.02 | 17.48 | 16.41 | 6.12 | 46.23 | 43.71 | 5.45 |
|  | 5% | 6.97 | 6.56 | 5.88 | 12.04 | 11.96 | 0.66 | 33.91 | 33.96 | -0.15 |
|  | 50% | 11.40 | 9.66 | 15.26 | 16.47 | 15.39 | 6.56 | 44.94 | 42.74 | 4.90 |
|  | 95% | 17.32 | 16.97 | 2.02 | 26.78 | 25.38 | 5.23 | 60.82 | 56.45 | 7.19 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 7.21 | 5.79 | 19.69 | 12.47 | 11.02 | 11.63 | 37.37 | 36.37 | 2.68 |
|  | 5% | 4.79 | 3.42 | 28.60 | 8.09 | 6.55 | 19.04 | 20.76 | 26.57 | -27.99 |
|  | 50% | 6.21 | 5.58 | 10.14 | 11.33 | 10.84 | 4.32 | 35.27 | 36.52 | -3.54 |
|  | 95% | 12.00 | 8.86 | 26.17 | 21.45 | 15.60 | 27.29 | 58.53 | 44.38 | 24.18 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.02 | 5.53 |  | 19.89 | 19.16 |  | 40.94 | 41.26 |  |
|  | UL | 1.53 | 1.65 |  | 5.74 | 5.53 |  | 10.52 | 10.17 |  |
| Type-2 RU (%) | DL | 7.52 | 6.91 |  | 24.86 | 23.95 |  | 51.18 | 51.57 |  |
|  | UL | 7.66 | 8.22 |  | 28.69 | 27.65 |  | 52.61 | 50.84 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.20 |  | 1.10 | 1.20 |  | 4.90 | 4.90 |  |
|  | UL | 0.3 | 0.3 |  | 1.4 | 1.3 |  | 5.70 | 5.8 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 2, compared to legacy TDD, SBFD with Alt 4 can significantly improve the UL UPT at low/medium load conditions and DL UPT at all load conditions for indoor office.

Observation 3: For indoor office, compared to legacy TDD, compared to legacy TDD, SBFD with Alt 4 can improve the UL UPT at low/medium load conditions and DL UPT at all load conditions.

As can be seen from Table 2, it can be observed that DL latency at all three load conditions for indoor office are reduced slightly compared to legacy TDD (due to increased HARQ-ACK transmission occasions). UL latency at low and median load conditions are reduced compared to that of legacy TDD (due to increased UL transmission occasions). And at high load condition, UL latency is increased at 5% and 50% CDF compared to that of legacy TDD (impacted by inter-subband CLI and limited UL resource which leads to packet transmitted via multiple slots) while it is reduced at 95% CDF compared to that of legacy TDD (due to increased HARQ-ACK transmission occasions).

Observation 4: For indoor office, compared to legacy TDD, SBFD with Alt 4 can reduce DL latency slightly at all three low loads and reduce UL latency at low and median loads.

    - Urban macro

Table 3: Evaluation results of SBFD Alt 2 for Urban macro in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 233.86 | 191.35 | -18.18 | 138.14 | 110.95 | -19.68 | 58.75 | 47.54 | -19.08 |
|  | 5% | 163.64 | 145.04 | -11.37 | 98.97 | 77.75 | -21.44 | 42.08 | 38.43 | -8.67 |
|  | 50% | 233.55 | 195.23 | -16.41 | 139.87 | 114.71 | -17.99 | 60.09 | 48.16 | -19.85 |
|  | 95% | 307.13 | 220.77 | -28.12 | 173.25 | 130.14 | -24.88 | 69.62 | 54.37 | -21.90 |
| DL Tail-UPT CDF (Mbps) | Mean | 161.45 | 139.63 | -13.52 | 103.25 | 88.35 | -14.43 | 43.39 | 37.76 | -12.98 |
|  | 5% | 118.31 | 106.35 | -10.11 | 73.75 | 68.83 | -6.67 | 32.93 | 29.80 | -9.51 |
|  | 50% | 161.09 | 142.89 | -11.30 | 104.62 | 90.56 | -13.44 | 44.15 | 38.80 | -12.12 |
|  | 95% | 201.31 | 158.41 | -21.31 | 130.16 | 100.92 | -22.46 | 51.70 | 42.29 | -18.20 |
| DL Median-UPT CDF (Mbps) | Mean | 232.67 | 177.63 | -23.66 | 137.67 | 116.06 | -15.70 | 58.40 | 44.68 | -23.49 |
|  | 5% | 156.56 | 132.65 | -15.27 | 96.64 | 80.71 | -16.48 | 42.45 | 35.95 | -15.31 |
|  | 50% | 233.31 | 182.85 | -21.63 | 138.53 | 118.37 | -14.55 | 60.17 | 46.24 | -23.15 |
|  | 95% | 304.00 | 201.11 | -33.85 | 173.55 | 136.31 | -21.46 | 69.74 | 51.98 | -25.47 |
| UL Average-UPT CDF (Mbps) | Mean | 92.01 | 127.72 | 38.81 | 52.71 | 72.40 | 37.36 | 22.30 | 30.17 | 35.29 |
|  | 5% | 44.15 | 84.68 | 91.80 | 28.89 | 53.40 | 84.84 | 13.52 | 24.02 | 77.66 |
|  | 50% | 94.81 | 128.12 | 35.13 | 54.04 | 72.29 | 33.77 | 22.30 | 30.01 | 34.57 |
|  | 95% | 128.74 | 160.63 | 24.77 | 72.68 | 90.94 | 25.12 | 30.41 | 35.24 | 15.88 |
| UL Tail-UPT CDF (Mbps) | Mean | 50.61 | 79.53 | 57.14 | 36.16 | 52.68 | 45.69 | 14.46 | 21.19 | 46.54 |
|  | 5% | 27.49 | 53.41 | 94.29 | 23.03 | 36.63 | 59.05 | 10.18 | 13.04 | 28.09 |
|  | 50% | 53.33 | 81.71 | 53.22 | 36.16 | 53.71 | 48.53 | 14.45 | 21.86 | 51.28 |
|  | 95% | 64.36 | 94.32 | 46.55 | 48.05 | 65.91 | 37.17 | 17.83 | 24.44 | 37.07 |
| UL Median-UPT CDF (Mbps) | Mean | 85.35 | 123.62 | 44.84 | 50.69 | 73.04 | 44.09 | 22.20 | 30.64 | 38.02 |
|  | 5% | 42.52 | 80.22 | 88.66 | 29.87 | 49.51 | 65.75 | 14.17 | 21.88 | 54.41 |
|  | 50% | 88.01 | 123.23 | 40.02 | 54.03 | 72.34 | 33.89 | 22.51 | 30.85 | 37.05 |
|  | 95% | 122.92 | 151.42 | 23.19 | 65.58 | 91.16 | 39.01 | 30.23 | 37.34 | 23.52 |
| DL Packet-Latency CDF (ms) | Mean | 17.08 | 19.76 | -15.69 | 28.89 | 34.42 | -19.14 | 67.04 | 81.92 | -22.20 |
|  | 5% | 11.06 | 13.77 | -24.50 | 21.65 | 28.42 | -31.27 | 51.67 | 66.95 | -29.57 |
|  | 50% | 16.52 | 19.28 | -16.71 | 27.93 | 33.20 | -18.87 | 65.05 | 81.03 | -24.57 |
|  | 95% | 24.59 | 26.57 | -8.05 | 39.34 | 46.71 | -18.73 | 89.28 | 99.74 | -11.72 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 11.37 | 7.82 | 31.22 | 18.06 | 13.08 | 27.57 | 47.58 | 34.49 | 27.51 |
|  | 5% | 5.84 | 4.56 | 21.92 | 10.81 | 9.19 | 14.99 | 34.36 | 28.36 | 17.46 |
|  | 50% | 10.75 | 7.55 | 29.77 | 18.14 | 12.70 | 29.99 | 44.86 | 33.59 | 25.12 |
|  | 95% | 18.40 | 10.65 | 42.12 | 26.76 | 17.54 | 34.45 | 70.62 | 43.33 | 38.64 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 5.32 | 5.17 |  | 19.62 | 16.44 |  | 41.93 | 32.61 |  |
|  | UL | 1.75 | 2.83 |  | 5.95 | 10.42 |  | 10.39 | 18.27 |  |
| Type-2 RU (%) | DL | 7.02 | 8.09 |  | 24.52 | 25.69 |  | 52.41 | 50.96 |  |
|  | UL | 8.76 | 7.85 |  | 29.75 | 28.95 |  | 51.94 | 50.76 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.2 | 0.2 |  | 0.8 | 0.8 |  | 4 | 4 |  |
|  | UL | 0.4 | 0.4 |  | 1.2 | 1.1 |  | 4.8 | 4.9 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 3, SBFD achieves better UL user throughput at all three load conditions for urban macro than that of the legacy TDD at the cost of slightly decreased DL user throughput due to more resources are allocated for UL in SBFD. It is observed that improvement of UL performance is much more than the degradation of DL performance.

Observation 5: For urban macro, compared to legacy TDD, SBFD with Alt 2 achieves better UL user throughput at all three load conditions at the cost of slightly decreased DL user throughput.

As can be seen from Table 3, UL latency is significantly reduced in SBFD at all three load conditions for urban macro at the cost of slightly increased DL latency.

Observation 6: For urban macro, compared to legacy TDD, SBFD with Alt 2 can significantly reduce the UL latency at all three load conditions with the cost of slightly increased DL latency.

Table 4: Evaluation results of SBFD Alt 4 for Urban macro in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 233.86 | 250.37 | 7.06 | 138.14 | 145.29 | 5.18 | 58.75 | 60.59 | 3.13 |
|  | 5% | 163.64 | 170.40 | 4.13 | 98.97 | 108.53 | 9.66 | 42.08 | 43.36 | 3.04 |
|  | 50% | 233.55 | 258.36 | 10.62 | 139.87 | 149.01 | 6.53 | 60.09 | 61.98 | 3.15 |
|  | 95% | 307.13 | 322.05 | 4.86 | 173.25 | 182.87 | 5.55 | 69.62 | 70.29 | 0.96 |
| DL Tail-UPT CDF (Mbps) | Mean | 161.45 | 175.91 | 8.96 | 103.25 | 107.62 | 4.23 | 43.39 | 44.51 | 2.58 |
|  | 5% | 118.31 | 133.78 | 13.08 | 73.75 | 81.43 | 10.41 | 32.93 | 35.17 | 6.80 |
|  | 50% | 161.09 | 181.21 | 12.49 | 104.62 | 109.78 | 4.93 | 44.15 | 45.50 | 3.06 |
|  | 95% | 201.31 | 204.27 | 1.47 | 130.16 | 134.12 | 3.04 | 51.70 | 53.10 | 2.71 |
| DL Median-UPT CDF (Mbps) | Mean | 232.67 | 250.25 | 7.56 | 137.67 | 146.83 | 6.65 | 58.40 | 59.88 | 2.53 |
|  | 5% | 156.56 | 169.44 | 8.23 | 96.64 | 101.37 | 4.89 | 42.45 | 44.28 | 4.31 |
|  | 50% | 233.31 | 257.34 | 10.30 | 138.53 | 152.55 | 10.12 | 60.17 | 62.37 | 3.66 |
|  | 95% | 304.00 | 301.57 | -0.80 | 173.55 | 176.27 | 1.57 | 69.74 | 70.89 | 1.65 |
| UL Average-UPT CDF (Mbps) | Mean | 92.01 | 113.31 | 23.15 | 52.71 | 58.09 | 10.21 | 22.30 | 20.81 | -6.68 |
|  | 5% | 44.15 | 81.74 | 85.14 | 28.89 | 45.48 | 57.42 | 13.52 | 18.02 | 33.28 |
|  | 50% | 94.81 | 114.55 | 20.82 | 54.04 | 58.47 | 8.20 | 22.30 | 20.58 | -7.71 |
|  | 95% | 128.74 | 129.90 | 0.90 | 72.68 | 69.75 | -4.03 | 30.41 | 23.11 | -24.01 |
| UL Tail-UPT CDF (Mbps) | Mean | 50.61 | 66.90 | 32.19 | 36.16 | 45.70 | 26.38 | 14.46 | 18.50 | 27.94 |
|  | 5% | 27.49 | 52.36 | 90.47 | 23.03 | 35.45 | 53.93 | 10.18 | 12.84 | 26.13 |
|  | 50% | 53.33 | 68.70 | 28.82 | 36.16 | 46.57 | 28.79 | 14.45 | 18.47 | 27.82 |
|  | 95% | 64.36 | 74.87 | 16.33 | 48.05 | 49.43 | 2.87 | 17.83 | 20.04 | 12.39 |
| UL Median-UPT CDF (Mbps) | Mean | 85.35 | 106.58 | 24.87 | 50.69 | 58.39 | 15.19 | 22.20 | 22.29 | 0.41 |
|  | 5% | 42.52 | 80.01 | 88.17 | 29.87 | 49.05 | 64.21 | 14.17 | 19.96 | 40.86 |
|  | 50% | 88.01 | 104.65 | 18.91 | 54.03 | 57.58 | 6.57 | 22.51 | 21.69 | -3.64 |
|  | 95% | 122.92 | 127.72 | 3.90 | 65.58 | 67.71 | 3.25 | 30.23 | 26.51 | -12.31 |
| DL Packet-Latency CDF (ms) | Mean | 17.08 | 15.80 | 7.49 | 28.89 | 27.55 | 4.64 | 67.04 | 65.02 | 3.01 |
|  | 5% | 11.06 | 10.76 | 2.71 | 21.65 | 21.70 | -0.23 | 51.67 | 52.07 | -0.77 |
|  | 50% | 16.52 | 15.42 | 6.66 | 27.93 | 26.34 | 5.69 | 65.05 | 63.08 | 3.03 |
|  | 95% | 24.59 | 22.23 | 9.60 | 39.34 | 37.24 | 5.34 | 89.28 | 90.35 | -1.20 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 11.37 | 9.61 | 15.48 | 18.06 | 16.26 | 9.97 | 47.58 | 45.86 | 3.61 |
|  | 5% | 5.84 | 5.77 | 1.20 | 10.81 | 11.09 | -2.59 | 34.36 | 30.78 | 10.42 |
|  | 50% | 10.75 | 9.26 | 13.86 | 18.14 | 17.00 | 6.28 | 44.86 | 47.50 | -5.88 |
|  | 95% | 18.40 | 14.85 | 19.29 | 26.76 | 19.63 | 26.64 | 70.62 | 52.65 | 25.45 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 5.32 | 5.74 |  | 19.62 | 24.7 |  | 41.93 | 41.35 |  |
|  | UL | 1.75 | 1.63 |  | 5.95 | 29.03 |  | 10.39 | 10.12 |  |
| Type-2 RU (%) | DL | 7.02 | 7.17 |  | 24.52 | 19.76 |  | 52.41 | 51.69 |  |
|  | UL | 8.76 | 8.17 |  | 29.75 | 5.81 |  | 51.94 | 50.62 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.2 | 0.3 |  | 0.8 | 0.9 |  | 4 | 4.1 |  |
|  | UL | 0.4 | 0.4 |  | 1.2 | 1.2 |  | 4.8 | 4.9 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 4, when comparing UL latency with the legacy TDD, it can be observed that SBFD shows slightly reduced DL latency at all three load conditions for urban macro and significant reduced UL latency at low/medium load condition.

Observation 7: For urban macro, compared to legacy TDD, SBFD with Alt 4 can reduce the DL latency at all three load conditions for urban macro and UL latency at low/medium load condition.

As can be seen from Table 4, compared to legacy TDD, SBFD with Alt 4 can slightly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro. There is a decrease for the 95%-CDF of UL UPT (in case of low HARQ retransmission number) at high load condition.

Observation 8: For urban macro, compared to legacy TDD, SBFD with Alt 4 can significantly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro while there is a decrease for the 95%-CDF of UL UPT at high load condition.

  - Simulation results of SBFD configuration Alt 2 & Alt 4 with small packet (4Kbytes for DL and 1Kbyte for UL)

    - Indoor office

Table 5: Evaluation results of SBFD Alt 2 for indoor in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.70 | 37.50 | -0.53 | 37.62 | 37.49 | -0.35 | 37.26 | 35.30 | -5.26 |
|  | 5% | 35.30 | 35.67 | 1.05 | 35.26 | 35.67 | 1.16 | 34.61 | 29.03 | -16.12 |
|  | 50% | 37.76 | 37.51 | -0.66 | 37.72 | 37.51 | -0.56 | 37.62 | 35.78 | -4.89 |
|  | 95% | 40.05 | 39.49 | -1.40 | 39.84 | 39.49 | -0.88 | 40.52 | 39.10 | -3.50 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 23.35 | -0.60 | 23.49 | 23.36 | -0.55 | 23.09 | 21.57 | -6.58 |
|  | 5% | 22.12 | 22.00 | -0.54 | 21.93 | 21.93 | 0.00 | 21.59 | 15.48 | -28.30 |
|  | 50% | 23.43 | 23.25 | -0.77 | 23.38 | 23.28 | -0.43 | 23.19 | 22.25 | -4.05 |
|  | 95% | 25.40 | 24.66 | -2.91 | 25.19 | 24.81 | -1.51 | 25.62 | 25.09 | -2.07 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 39.44 | -0.40 | 39.53 | 39.42 | -0.28 | 39.23 | 37.25 | -5.05 |
|  | 5% | 36.88 | 36.75 | -0.35 | 36.52 | 36.82 | 0.82 | 36.01 | 32.86 | -8.75 |
|  | 50% | 39.32 | 39.39 | 0.18 | 39.32 | 39.38 | 0.15 | 39.26 | 37.92 | -3.41 |
|  | 95% | 42.88 | 42.32 | -1.31 | 43.03 | 42.26 | -1.79 | 43.01 | 41.67 | -3.12 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.64 | 133.85 | 4.55 | 10.64 | 133.85 | 4.54 | 10.65 | 134.58 |
|  | 5% | 3.97 | 10.35 | 160.71 | 3.99 | 10.35 | 159.40 | 3.99 | 10.38 | 160.15 |
|  | 50% | 4.49 | 10.63 | 136.75 | 4.54 | 10.63 | 134.14 | 4.50 | 10.62 | 136.00 |
|  | 95% | 5.31 | 11.00 | 107.16 | 5.29 | 11.00 | 107.94 | 5.35 | 11.05 | 106.54 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.18 | 189.05 | 2.84 | 8.18 | 188.03 | 2.83 | 8.17 | 188.69 |
|  | 5% | 2.68 | 8.11 | 202.61 | 2.68 | 8.00 | 198.51 | 2.67 | 8.10 | 203.37 |
|  | 50% | 2.79 | 8.21 | 194.27 | 2.78 | 8.23 | 196.04 | 2.79 | 8.21 | 194.27 |
|  | 95% | 3.13 | 8.36 | 167.09 | 3.16 | 8.44 | 167.09 | 3.13 | 8.38 | 167.73 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.64 | 132.82 | 4.57 | 10.63 | 132.60 | 4.57 | 10.64 | 132.82 |
|  | 5% | 3.62 | 10.19 | 181.49 | 3.60 | 10.17 | 182.50 | 3.60 | 10.18 | 182.78 |
|  | 50% | 4.47 | 10.63 | 137.81 S | 4.49 | 10.60 | 136.08 | 4.51 | 10.60 | 135.03 |
|  | 95% | 5.87 | 11.23 | 91.31 | 5.81 | 11.20 | 92.77 | 5.84 | 11.23 | 92.29 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.86 | 1.15 | 0.87 | 0.86 | 1.15 | 0.90 | 0.93 | -3.33 |
|  | 5% | 0.52 | 0.52 | 0.00 | 0.52 | 0.52 | 0.00 | 0.52 | 0.53 | -1.92 |
|  | 50% | 0.81 | 0.81 | 0.00 | 0.80 | 0.81 | -1.25 | 0.82 | 0.86 | -4.88 |
|  | 95% | 1.35 | 1.36 | -0.74 | 1.35 | 1.35 | 0.00 | 1.38 | 1.46 | -5.80 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | 57.30 | 1.76 | 0.76 | 56.82 | 1.78 | 0.76 | 57.30 |
|  | 5% | 0.60 | 0.52 | 13.33 | 0.59 | 0.52 | 11.86 | 0.60 | 0.52 | 13.33 |
|  | 50% | 1.77 | 0.75 | 57.63 | 1.74 | 0.75 | 56.90 | 1.77 | 0.75 | 57.63 |
|  | 95% | 2.86 | 0.97 | 66.08 | 2.84 | 0.97 | 65.85 | 2.86 | 0.97 | 66.08 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.12 | 5.21 |  | 19.87 | 16.37 |  | 41.34 | 33.22 |  |
|  | UL | 1.51 | 2.81 |  | 5.68 | 7.96 |  | 10.35 | 18.44 |  |
| Type-2 RU (%) | DL | 7.65 | 8.15 |  | 24.84 | 25.58 |  | 51.67 | 51.91 |  |
|  | UL | 7.57 | 2.81 |  | 28.4 | 22.12 |  | 51.75 | 51.24 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.2 | 0.3 |  | 1.2 | 1.1 |  | 5 | 5 |  |
|  | UL | 0.3 | 2.81 |  | 1.3 | 1.4 |  | 5.6 | 5.5 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 5, compared to legacy TDD, SBFD shows comparable DL UPT and significant increased UL UPT performance.

Observation 9: For indoor office, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL UPT and significant increased UL UPT performance.

As can be seen from Table 5, SBFD has comparable DL latency performance with legacy TDD for indoor office. And SBFD shows significant UL latency performance gain compared with legacy TDD due to more UL transmission occasions and resource.

Observation 10: For indoor office, compared to legacy TDD, SBFD Alt 2 with small packet has comparable DL latency performance and shows significant UL latency performance gain compared with legacy TDD.

Table 6: Evaluation results of SBFD Alt 4 for indoor in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.70 | 42.76 | 13.42 | 37.62 | 42.64 | 13.34 | 37.26 | 41.62 | 11.71 |
|  | 5% | 35.30 | 41.58 | 17.79 | 35.26 | 41.59 | 17.95 | 34.61 | 36.61 | 5.77 |
|  | 50% | 37.76 | 42.81 | 13.37 | 37.72 | 42.69 | 13.18 | 37.62 | 42.47 | 12.89 |
|  | 95% | 40.05 | 43.92 | 9.66 | 39.84 | 43.64 | 9.54 | 40.52 | 43.60 | 7.61 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 32.68 | 39.12 | 23.49 | 32.67 | 39.08 | 23.09 | 32.10 | 39.03 |
|  | 5% | 22.12 | 32.42 | 46.56 | 21.93 | 32.36 | 47.56 | 21.59 | 29.23 | 35.39 |
|  | 50% | 23.43 | 32.85 | 40.20 | 23.38 | 32.88 | 40.63 | 23.19 | 31.79 | 37.07 |
|  | 95% | 25.40 | 33.55 | 32.09 | 25.19 | 33.51 | 33.03 | 25.62 | 33.29 | 29.94 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 42.82 | 8.13 | 39.53 | 42.91 | 8.55 | 39.23 | 41.86 | 6.71 |
|  | 5% | 36.88 | 40.66 | 10.25 | 36.52 | 41.06 | 12.43 | 36.01 | 36.68 | 1.86 |
|  | 50% | 39.32 | 42.79 | 8.83 | 39.32 | 43.00 | 9.36 | 39.26 | 42.49 | 8.23 |
|  | 95% | 42.88 | 44.74 | 4.34 | 43.03 | 44.71 | 3.90 | 43.01 | 44.42 | 3.27 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.71 | 135.38 | 4.55 | 10.69 | 134.95 | 4.54 | 10.66 | 134.89 |
|  | 5% | 3.97 | 10.02 | 152.39 | 3.99 | 10.06 | 152.13 | 3.99 | 9.92 | 148.53 |
|  | 50% | 4.49 | 10.65 | 137.19 | 4.54 | 10.65 | 134.58 | 4.50 | 10.62 | 135.93 |
|  | 95% | 5.31 | 11.47 | 116.01 | 5.29 | 11.35 | 114.56 | 5.35 | 11.37 | 112.44 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.23 | 190.81 | 2.84 | 8.22 | 189.44 | 2.83 | 8.24 | 191.26 |
|  | 5% | 2.68 | 7.98 | 197.76 | 2.68 | 8.02 | 199.25 | 2.67 | 8.02 | 200.30 |
|  | 50% | 2.79 | 8.24 | 195.34 | 2.78 | 8.22 | 195.68 | 2.79 | 8.24 | 195.49 |
|  | 95% | 3.13 | 8.65 | 176.36 | 3.16 | 8.65 | 173.73 | 3.13 | 8.61 | 175.14 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.72 | 134.57 | 4.57 | 10.72 | 134.57 | 4.57 | 10.72 | 134.65 |
|  | 5% | 3.62 | 10.03 | 177.07 | 3.60 | 9.79 | 171.94 | 3.60 | 9.81 | 172.51 |
|  | 50% | 4.47 | 10.71 | 139.60 | 4.49 | 10.63 | 136.75 | 4.51 | 10.67 | 136.51 |
|  | 95% | 5.87 | 11.43 | 94.72 | 5.81 | 11.77 | 102.58 | 5.84 | 12.06 | 106.51 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.76 | 12.64 | 0.87 | 0.76 | 12.64 | 0.90 | 0.79 | 12.22 |
|  | 5% | 0.52 | 0.52 | 0.00 | 0.52 | 0.52 | 0.00 | 0.52 | 0.52 | 0.00 |
|  | 50% | 0.81 | 0.74 | 8.64 | 0.80 | 0.75 | 6.25 | 0.82 | 0.75 | 8.54 |
|  | 95% | 1.35 | 0.97 | 28.15 | 1.35 | 0.97 | 28.15 | 1.38 | 0.98 | 28.99 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | 57.30 | 1.76 | 0.76 | 56.82 | 1.78 | 0.76 | 57.30 |
|  | 5% | 0.60 | 0.52 | 13.33 | 0.59 | 0.52 | 11.86 | 0.60 | 0.50 | 16.67 |
|  | 50% | 1.77 | 0.74 | 58.19 | 1.74 | 0.75 | 56.90 | 1.77 | 0.75 | 57.63 |
|  | 95% | 2.86 | 0.97 | 66.08 | 2.84 | 0.97 | 65.85 | 2.86 | 0.97 | 66.08 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.12 | 5.92 |  | 19.87 | 19.02 |  | 41.34 | 41.11 |  |
|  | UL | 1.51 | 1.57 |  | 5.68 | 5.75 |  | 10.35 | 50.65 |  |
| Type-2 RU (%) | DL | 7.65 | 7.40 |  | 24.84 | 23.77 |  | 51.67 | 51.39 |  |
|  | UL | 7.57 | 7.87 |  | 28.40 | 28.74 |  | 51.75 | 10.13 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.30 |  | 1.20 | 1.30 |  | 5.00 | 5.00 |  |
|  | UL | 0.30 | 0.30 |  | 1.30 | 1.20 |  | 5.60 | 5.70 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 6, compared to legacy TDD, SBFD has moderate DL UPT performance gain and shows significant UL UPT performance gain at all the three load conditions.

Observation 11: For indoor office, compared to legacy TDD, SBFD with Alt 4 shows moderate DL UPT performance gain and shows significant UL UPT performance gain at all the three load conditions.

As can be seen from Table 6, it can be observed that SBFD has significant DL latency performance gain mainly at 95% CDF and SBFD has significant UL latency performance gain compared to legacy TDD.

Observation 12: For indoor office, compared to legacy TDD, SBFD with Alt 4 shows significant DL latency performance gain mainly at 95% CDF and shows significant UL latency performance gain at most of the cases compared to legacy TDD.

    - Urban macro

Table 7: Evaluation results of SBFD Alt 2 for Urban macro in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.71 | 37.50 | -0.56 | 37.70 | 35.96 | -4.62 | 30.65 | 29.22 | -4.67 |
|  | 5% | 35.30 | 35.67 | 1.05 | 35.44 | 31.37 | -11.48 | 7.38 | 15.40 | 108.67 |
|  | 50% | 37.76 | 37.51 | -0.66 | 37.78 | 37.21 | -1.51 | 35.06 | 31.84 | -9.18 |
|  | 95% | 40.05 | 39.49 | -1.40 | 39.98 | 39.51 | -1.18 | 37.60 | 35.08 | -6.70 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 23.35 | -0.60 | 23.49 | 22.38 | -4.73 | 19.23 | 16.55 | -13.94 |
|  | 5% | 22.14 | 22.02 | -0.54 | 22.12 | 17.30 | -21.79 | 4.15 | 7.84 | 88.92 |
|  | 50% | 23.49 | 23.32 | -0.72 | 23.43 | 23.02 | -1.75 | 23.24 | 23.85 | 2.62 |
|  | 95% | 25.33 | 24.69 | -2.53 | 25.40 | 25.12 | -1.10 | 25.28 | 25.07 | -0.83 |
| DL Median-UPT CDF (Mbps) | Mean | 39.61 | 39.48 | -0.33 | 39.58 | 38.55 | -2.60 | 33.52 | 31.11 | -7.19 |
|  | 5% | 36.92 | 36.69 | -0.62 | 36.89 | 35.16 | -4.69 | 6.03 | 22.69 | 276.29 |
|  | 50% | 39.29 | 39.42 | 0.33 | 39.40 | 38.55 | -2.16 | 39.29 | 36.11 | -8.09 |
|  | 95% | 42.84 | 42.24 | -1.40 | 42.76 | 42.67 | -0.21 | 42.39 | 38.83 | -8.40 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.65 | 134.07 | 4.55 | 10.64 | 133.85 | 4.18 | 9.45 | 126.08 |
|  | 5% | 3.97 | 10.35 | 160.71 | 3.97 | 10.35 | 160.71 | 3.71 | 6.82 | 83.83 |
|  | 50% | 4.49 | 10.63 | 136.75 | 4.49 | 10.64 | 136.97 | 4.14 | 9.97 | 140.82 |
|  | 95% | 5.31 | 11.00 | 107.16 | 5.32 | 10.97 | 106.20 | 4.80 | 11.08 | 130.83 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.18 | 189.05 | 2.83 | 8.18 | 189.05 | 1.95 | 5.97 | 206.15 |
|  | 5% | 2.68 | 8.11 | 202.61 | 2.68 | 8.11 | 202.61 | 0.63 | 3.37 | 434.92 |
|  | 50% | 2.79 | 8.22 | 194.62 | 2.79 | 8.21 | 194.27 | 1.79 | 5.30 | 196.09 |
|  | 95% | 3.13 | 8.36 | 167.09 | 3.13 | 8.36 | 167.09 | 2.70 | 8.27 | 206.30 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.64 | 132.82 | 4.57 | 10.64 | 132.82 | 4.14 | 9.89 | 138.89 |
|  | 5% | 3.62 | 10.15 | 180.39 | 3.62 | 10.18 | 181.22 | 1.22 | 7.77 | 536.89 |
|  | 50% | 4.47 | 10.60 | 137.14 | 4.48 | 10.61 | 136.83 | 4.18 | 10.06 | 140.67 |
|  | 95% | 5.86 | 11.24 | 91.81 | 5.86 | 11.22 | 91.47 | 5.71 | 11.26 | 97.20 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.86 | 1.15 | 0.88 | 0.89 | -1.14 | 1.28 | 1.12 | 12.50 |
|  | 5% | 0.53 | 0.52 | 1.89 | 0.52 | 0.52 | 0.00 | 0.52 | 0.53 | -1.92 |
|  | 50% | 0.81 | 0.81 | 0.00 | 0.80 | 0.81 | -1.25 | 0.84 | 0.92 | -9.52 |
|  | 95% | 1.35 | 1.36 | -0.74 | 1.35 | 1.39 | -2.96 | 5.69 | 3.18 | 44.11 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | 57.30 | 1.76 | 0.76 | 56.82 | 2.10 | 0.88 | 58.10 |
|  | 5% | 0.60 | 0.52 | 13.33 | 0.59 | 0.52 | 11.86 | 0.60 | 0.52 | 13.33 |
|  | 50% | 1.77 | 0.74 | 58.19 | 1.74 | 0.74 | 57.47 | 1.89 | 0.79 | 58.20 |
|  | 95% | 2.86 | 0.97 | 66.08 | 2.85 | 0.97 | 65.96 | 4.07 | 1.64 | 59.71 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.15 | 5.41 |  | 19.27 | 16.94 |  | 41.48 | 32.12 |  |
|  | UL | 1.72 | 2.86 |  | 5.77 | 10.37 |  | 0.26 | 18.15 |  |
| Type-2 RU (%) | DL | 7.96 | 8.46 |  | 24.09 | 26.48 |  | 51.48 | 50.19 |  |
|  | UL | 8.62 | 7.65 |  | 28.83 | 28.80 |  | 51.31 | 50.42 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.20 |  | 0.80 | 0.90 |  | 4.10 | 3.90 |  |
|  | UL | 0.40 | 0.40 |  | 1.20 | 1.30 |  | 4.90 | 4.70 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 7, SBFD shows comparable DL UPT performance with legacy TDD except that it shows significant performance gain at 5%-CDF of DL UPT performance (multiple HARQ retransmissions are needed where SBFD can achieve significant gain) at high load condition. And compared to legacy TDD, SBFD shows significant UL UPT performance gain.

Observation 13: For urban macro, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL UPT except that it shows significant performance gain at 5%-CDF of DL UPT at high load condition and shows significant UL UPT performance gain at all three load conditions.

As can be seen from Table 7, SBFD shows comparable DL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL (multiple HARQ retransmissions are needed where SBFD can achieve significant gain) at high load condition. And compared to legacy TDD, SBFD shows significant UL latency performance gain at all three load conditions.

Observation 14: For urban macro, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL latency at high load condition and shows significant UL latency performance gain at all three load conditions.

Table 8: Evaluation results of SBFD Alt 4 for Urban macro in FR1 in SBFD Deployment Case 1

| Reported Parameters |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.71 | 42.11 | 11.67 | 37.70 | 42.01 | 11.43 | 30.65 | 36.13 | 17.88 |
|  | 5% | 35.30 | 37.78 | 7.03 | 35.44 | 37.92 | 7.00 | 7.38 | 16.59 | 124.80 |
|  | 50% | 37.76 | 42.39 | 12.26 | 37.78 | 42.39 | 12.20 | 35.06 | 41.18 | 17.46 |
|  | 95% | 40.05 | 43.80 | 9.36 | 39.98 | 43.83 | 9.63 | 37.60 | 43.55 | 15.82 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 32.63 | 38.91 | 23.49 | 32.63 | 38.91 | 19.23 | 24.54 | 27.61 |
|  | 5% | 22.14 | 32.27 | 45.75 | 22.12 | 32.16 | 45.39 | 4.15 | 9.00 | 116.87 |
|  | 50% | 23.49 | 32.83 | 39.76 | 23.43 | 32.83 | 40.12 | 23.24 | 32.41 | 39.46 |
|  | 95% | 25.33 | 33.57 | 32.53 | 25.40 | 33.63 | 32.40 | 25.28 | 33.23 | 31.45 |
| DL Median-UPT CDF (Mbps) | Mean | 39.61 | 42.62 | 7.60 | 39.58 | 42.63 | 7.71 | 33.52 | 39.44 | 17.66 |
|  | 5% | 36.92 | 40.62 | 10.02 | 36.89 | 40.55 | 9.92 | 6.03 | 26.34 | 336.82 |
|  | 50% | 39.29 | 42.54 | 8.27 | 39.40 | 42.59 | 8.10 | 39.29 | 41.39 | 5.34 |
|  | 95% | 42.84 | 44.64 | 4.20 | 42.76 | 44.76 | 4.68 | 42.39 | 44.40 | 4.74 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.64 | 133.85 | 4.55 | 10.64 | 133.85 | 4.18 | 9.10 | 117.70 |
|  | 5% | 3.97 | 9.96 | 150.88 | 3.97 | 10.00 | 151.89 | 3.71 | 6.36 | 71.43 |
|  | 50% | 4.49 | 10.60 | 136.08 | 4.49 | 10.62 | 136.53 | 4.14 | 9.31 | 124.88 |
|  | 95% | 5.31 | 11.33 | 113.37 | 5.32 | 11.26 | 111.65 | 4.80 | 10.89 | 126.88 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.24 | 191.17 | 2.83 | 8.23 | 190.81 | 1.95 | 5.97 | 206.15 |
|  | 5% | 2.68 | 8.05 | 200.37 | 2.68 | 8.04 | 200.00 | 0.63 | 3.23 | 412.70 |
|  | 50% | 2.79 | 8.23 | 194.98 | 2.79 | 8.24 | 195.34 | 1.79 | 5.59 | 212.29 |
|  | 95% | 3.13 | 8.62 | 175.40 | 3.13 | 8.58 | 174.12 | 2.70 | 8.32 | 208.15 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.72 | 134.57 | 4.57 | 10.72 | 134.57 | 4.14 | 9.66 | 133.33 |
|  | 5% | 3.62 | 9.70 | 167.96 | 3.62 | 9.70 | 167.96 | 1.22 | 6.70 | 449.18 |
|  | 50% | 4.47 | 10.65 | 138.26 | 4.48 | 10.64 | 137.50 | 4.18 | 9.93 | 137.56 |
|  | 95% | 5.86 | 11.89 | 102.90 | 5.86 | 11.83 | 101.88 | 5.71 | 11.08 | 94.05 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.77 | 11.49 | 0.88 | 0.77 | 12.33 | 1.28 | 0.93 | 27.34 |
|  | 5% | 0.53 | 0.52 | 1.89 | 0.52 | 0.52 | 0.57 | 0.52 | 0.53 | -1.92 |
|  | 50% | 0.81 | 0.75 | 7.41 | 0.80 | 0.74 | 7.77 | 0.84 | 0.78 | 7.14 |
|  | 95% | 1.35 | 0.97 | 28.15 | 1.35 | 0.97 | 28.09 | 5.69 | 3 | 47.28 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | 57.30 | 1.76 | 0.77 | 56.58 | 2.1 | 0.89 | 57.62 |
|  | 5% | 0.60 | 0.52 | 13.33 | 0.59 | 0.52 | 12.14 | 0.6 | 0.52 | 13.33 |
|  | 50% | 1.77 | 0.74 | 58.19 | 1.74 | 0.75 | 57.07 | 1.89 | 0.79 | 58.20 |
|  | 95% | 2.86 | 0.97 | 66.08 | 2.85 | 0.98 | 65.49 | 4.07 | 1.49 | 63.39 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| Type-1 RU (%) | DL | 6.15 | 5.67 |  | 19.27 | 23.71 |  | 41.48 | 41.35 |  |
|  | UL | 1.72 | 1.87 |  | 5.77 | 5.65 |  | 0.26 | 10.14 |  |
| Type-2 RU (%) | DL | 7.96 | 7.09 |  | 24.09 | 18.97 |  | 51.48 | 51.69 |  |
|  | UL | 8.62 | 9.35 |  | 28.83 | 28.25 |  | 51.31 | 50.71 |  |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.30 |  | 0.80 | 0.80 |  | 4.10 | 4.00 |  |
|  | UL | 0.40 | 0.40 |  | 1.20 | 1.30 |  | 4.90 | 4.80 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. Definition of gain for latency: X%=1-SBFD latency/TDD latency. |  |  |  |  |  |  |  |  |  |

As can be seen from Table 8, SBFD achieves significant UL UPT gain at all three load conditions for urban macro than that of the legacy TDD and shows moderate DL UPT performance gain compared to legacy TDD.

Observation 15: For urban macro, compared to legacy TDD, SBFD with Alt 4 with small packet achieves significant UPT gain at all three load conditions and moderate DL UPT performance gain.

As can be seen from Table 8, when comparing with the legacy TDD, it can be observed that SBFD shows significant UL latency gain and shows moderate DL latency performance gain.

Observation 16: For urban macro, compared to legacy TDD, SBFD with Alt 4 with small packet shows significant UL latency gain and moderate DL latency performance gain.

- Conclusions

In this contribution, further analyses and views on deployment scenarios, evaluation methodologies and evaluation assumptions for Rel-18 duplex enhancement are provided. Our proposals and observations are:

Proposal 1: LLS for other purpose besides coverage performance evaluation is up to companies’ interests.

Proposal 2: If different transmission configurations (e.g. frequency resources, UL power control parameters or beam/spatial relations) are configured for SBFD and non-SBFD slots, separate channel estimation is preferred.

Proposal 3: If same transmission configuration is used for SBFD and non-SBFD slots, separate channel estimation for SBFD and non-SBFD slots is the baseline while JCE can be optionally used and reported.

Observation 1: For indoor office, compared to legacy TDD, SBFD with Alt 2 achieves better UL user throughput at all three load conditions at the cost of decreased DL user throughput.

Observation 2: For indoor office, compared to legacy TDD, SBFD with Alt 2 can significantly reduce the UL latency at the cost of slightly increased DL latency.

Observation 3: For indoor office, compared to legacy TDD, compared to legacy TDD, SBFD with Alt 4 can improve the UL UPT at low/medium load conditions and DL UPT at all load conditions.

Observation 4: For indoor office, compared to legacy TDD, SBFD with Alt 4 can reduce DL latency slightly at all three low loads and reduce UL latency at low and median loads.

Observation 5: For urban macro, compared to legacy TDD, SBFD with Alt 2 achieves better UL user throughput at all three load conditions at the cost of slightly decreased DL user throughput.

Observation 6: For urban macro, compared to legacy TDD, SBFD with Alt 2 can significantly reduce the UL latency at all three load conditions with the cost of slightly increased DL latency.

Observation 7: For urban macro, compared to legacy TDD, SBFD with Alt 4 can reduce the DL latency at all three load conditions for urban macro and UL latency at low/medium load condition.

Observation 8: For urban macro, compared to legacy TDD, SBFD with Alt 4 can significantly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro while there is a decrease for the 95%-CDF of UL UPT at high load condition.

Observation 9: For indoor office, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL UPT and significant increased UL UPT performance.

Observation 10: For indoor office, compared to legacy TDD, SBFD Alt 2 with small packet has comparable DL latency performance and shows significant UL latency performance gain compared with legacy TDD.

Observation 11: For indoor office, compared to legacy TDD, SBFD with Alt 4 shows moderate DL UPT performance gain and shows significant UL UPT performance gain at all the three load conditions.

Observation 12: For indoor office, compared to legacy TDD, SBFD with Alt 4 shows significant DL latency performance gain mainly at 95% CDF and shows significant UL latency performance gain at most of the cases compared to legacy TDD.

Observation 13: For urban macro, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL UPT except that it shows significant performance gain at 5%-CDF of DL UPT at high load condition and shows significant UL UPT performance gain at all three load conditions.

Observation 14: For urban macro, compared to legacy TDD, SBFD with Alt 2 with small packet shows comparable DL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL latency at high load condition and shows significant UL latency performance gain at all three load conditions.

Observation 15: For urban macro, compared to legacy TDD, SBFD with Alt 4 with small packet achieves significant UPT gain at all three load conditions and moderate DL UPT performance gain.

Observation 16: For urban macro, compared to legacy TDD, SBFD with Alt 4 with small packet shows significant UL latency gain and moderate DL latency performance gain.

- Reference

- Chairman's Notes RAN1#112, 3GPP TSG RAN WG1 Meeting #112 v0.4.0, Athens, Greece, 27th February – 3rd March 2023

- Annex

Table Annex-1. System-level simulation assumption for NR Full Duplex

| Parameters | Indoor office | Urban macro |  |
| --- | --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Single layer Macro layer: Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Carrier frequency | 4G |  |  |
| Deployment case | Case 1 |  |  |
| BS antenna configuration | SBFD antenna configuration option-2 : = (4,4,2,1,1; 4,4) = (0.5, 0.5)λ, +45°/-45° polarization | SBFD antenna configuration option-2 : = (8,8,2,1,1;2,8) = (0.5, 0.8)λ, +45°/-45° polarization |  |
| BS Tx power | 24dBm (38.901) | 49 dBm (38.901) |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |  |
| Traffic model | FTP traffic model 3 Each UE is assigned both UL traffic and DL traffic Asymmetric packet size: 0.5Mbyte for DL and 0.125 Mbytes for UL UL Traffic load: low UL RU (<10%), medium UL RU (20%-30%), and high UL RU ([≥50%]). DL Traffic load: low DL RU (<10%), medium DL RU (20%-30%), and high DL RU (≥50%). Note: Type-2 RU definition (calculated per link direction) is used |  |  |
| UE-to-UE Channel model | Large-scale channel parameters: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |  | Large-scale channel parameters: UMi-Street canyon in TR 38.901(hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802, ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| Transmission mode | SU-MIMO |  |  |
| Channel estimation | Realistic |  |  |
| Overhead | No extra overhead |  |  |